3GPP TSG RAN WG1 Meeting #84-bis                                          R1-162775

Busan, Korea, 11th - 15th April 2016

Source:                    	Ericsson

Title:  	NB-IoT- NSSS Design

Document for:        	Discussion

Agenda Item:         	7.2.1.1.4

# Introduction

At RAN#69, a new work item named NarrowBand IOT (NB-IoT) was approved, see [1]. The objective is to specify a radio access for cellular internet of things that addresses improved indoor coverage, support for massive number of low throughput devices, low delay sensitivity, ultra-low device cost, low device power consumption and (optimized) network architecture. At RAN#70, a revised work item description was approved, see [2].

NB-IOT should support 3 different modes of operation:

1.	“Stand-alone operation” utilizing for example the spectrum currently being used by GERAN systems as a replacement of one or more GSM carriers, as well as scattered spectrum for potential IoT deployment.

2.	“Guard band operation” utilizing the unused resource blocks within a LTE carrier’s guard-band

3.	“In-band operation” utilizing resource blocks within a normal LTE carrier

NB-IoT will support 180 kHz UE RF bandwidth for both downlink and uplink. Furthermore according to [2], NB-IoT downlink is based on OFDMA using 15 kHz sub-carrier spacing for all the modes of operation (with normal or extended CP).

Furthermore according to [2], NB-IOT should have a single synchronization signal design for the different modes of operation, including techniques to handle overlap with legacy LTE signals.

The agreements on the NB-SSS design according to the latest RAN meeting are summarized below [3]:

- NB-SSS is transmitted in subframe 9

- Number of symbols for NB-SSS: 11

- NB-SSS base sequence is constructed from one or more ZC sequences

  - Length FFS

- FFS whether multiple root sequences are used or a binary scrambling code

In this contribution, we propose the design of sequences that would be used for NB-SSS.

# Structure of NB-SSS

The NB-SSS is transmitted in subframe 9 with a periodicity of 20 ms as shown in Figure 1.

Figure 1: Frame structure for NB-PSS and NB-SSS.

From the figure, the NB-SSS is transmitted in every odd numbered frame, where we assume that a frame is composed of 10 subframes.

Proposal 1: NB-SSS is transmitted with a periodicity of 20 ms.

In order to reduce the number of blind decoding attempts for the NB-PBCH, the NB-SSS can be used to provide the frame timing within a certain multiple of 20 ms. In order to have reasonable complexity for NB-PBCH decoding, we propose to use the NB-SSS to provide the timing within 80 ms.

Proposal 2: NB-SSS is used to provide the frame timing within 80 ms.

The NB-SSS design consists of a single Zadoff-Chu (ZC) sequence superimposed with a scrambling sequence. The Zadoff-Chu sequence is used to obtain the physical cell ID, and the scrambling sequence is used to obtain the frame timing within 80 ms.

Proposal 3: NB-SSS uses a hierarchical design, consisting of a base sequence to indicate the cell ID and a scrambling code on top to indicate the position within the 80 ms.

Proposal 4: Zad-off Chu sequences are used as the base sequence for NB-SSS, and binary sequences derived from m-sequences are used for the scrambling code.

The NB-SSS signal generator is shown in Figure 2.

Figure 2: NB-SSS Generator

Specifically, the NB-SSS is given as NB-SSSp,q(n)=apnbq(n), where p={0,1,…,503} denotes the cell identity and q={0,1,2,3} determines the location of the NB-SSS, i.e., the number of NB-SSS in the 80 ms block that have occurred before the current NB-SSS. We have

apn=exp-jπun-mNSSSn-m+1NSSSNSSS,                 n=0,1,… , NSSS-1

and

bqn=bn-lqNSSS      n=0,1,…,NSSS-1,  q=0,1,2,3,

l0=0,l1=3,l2=7,l3=13

bn+7=modbn+bn+1,2,        n=0,1,…119,

b0=1,bm=0,     m={1,2,3,4,5,6}

The operation kNSSSdenotes the modulo operation such that kNSSS=modk,NSSS. The length NSSS should be prime in order to enable support for upto NSSS-1NSSS different sequences which have good cross correlation properties. It should also satisfy the condition that (NSSS+1) should be close to a power of 2 in order to enable the design of suitable m-sequences that can be used as scrambling sequences. We choose NSSS=127, which is a prime number, and NSSS+1=128=27. The combination of the root and cyclic shift u,m is sufficient to encode the physical cell ID p. An example mapping is given as

u=1+modp-1,126, m=25 p-1127+1,                 p∈{1,2,…,504}

bq(n) is the scrambling sequence composed of cyclic shifts of a base sequence b(n) and is used to indicate the NB-SSS location within 80 ms in order to obtain the correct timing. The cyclic shift lq is dependent on q.

Proposal 5: NB-SSS is generated in the frequency domain and is of length-127.

# Conclusions

In this contribution, we provide the design of secondary synchronization signal (NB-SSS) to be used in NB-IoT. The proposals for NB-SSS design are listed below.

Proposal 1: NB-SSS is transmitted with a periodicity of 20 ms.

Proposal 2: NB-SSS is used to provide the frame timing within 80 ms.

Proposal 3: NB-SSS uses a hierarchical design, consisting of a base sequence to indicate the cell ID and a scrambling code on top to indicate the position within the 80 ms.

Proposal 4: Zad-off Chu sequences are used as the base sequence for NB-SSS, and binary sequences are used for the scrambling code.

Proposal 5: NB-SSS is generated in the frequency domain and is of length-127.

# References

- RP-151621, “New Work Item: Narrowband IOT (NB-IOT)”; Qualcomm; Sept. 2015; Phoenix, AZ, USA

- RP-152284, “New Work Item: Narrowband IOT (NB-IOT),” source Huawei and HiSilicon, RAN #70, Dec. 2015

- “Draft Report of 3GPP TSG RAN WG1 #84 v0.2.0”; RAN#84; Feb. 2016; Malta.