3GPP TSG RAN WG1 Meeting #84bis	R1-162846

Busan, Korea, 11th – 15th April 2016

Source:	Sharp

Title:	Shortened TTI for DL transmission

Agenda Item:	7.3.10.2

Document for:	Discussion and Decision

# Introduction

In RAN1#84, design assumptions for shortened TTI were discussed and the following agreements were made [1].

Agreements:

- Following design assumptions are considered:

  - No shortened TTI spans over subframe boundary

  - At least for SIBs and paging, PDCCH and legacy PDSCH are used for scheduling

- The potential specific impacts for the followings are studied

  - UE is expected to receive a sPDSCH at least for downlink unicast

    - sPDSCH refers PDSCH carrying data in a short TTI

  - UE is expected to receive PDSCH for downlink unicast

    - FFS whether a UE is expected to receive both sPDSCH and PDSCH for downlink unicast simultaneously

  - FFS: The number of supported short TTIs

  - If the number of supported short TTIs is more than one,

    - The length of short TTI can be variable

      - FFS whether a UE is expected to receive sPDSCHs with different lengths simultaneously

      - FFS how to change the length of short TTI for sPDSCH e.g., semi-static vs dynamic, cell-specific vs UE-specific, explicit vs implicit

  - TTI length for DL and UL can be different

In this contribution we provide our views on shortened TTI for downlink transmission.

# Reception of sPDSCH and PDSCH

In RAN1#84, it was agreed that Random access, SIB and Paging procedures are not modified. At the same time, it was concluded that FDM should be used for multiplexing of sTTI and non-sTTI, so that latency for sTTI based communication is minimized. Considering these principle, it is natural that UE should be able to receive sTTI based channels in the subframe where Random access, SIB or Paging procedures is going. If the UE has the capability to receive both of the sTTI based channels and PDSCH carrying broadcast information, the UE is also able to receive both of sPDSCH and PDSCH for downlink unicast. In addition, even if the UE is configured with the sTTI, at least one PDSCH HARQ process should be maintained for a fallback operation. Therefore, The sTTI capable UE should be able to operate sPDSCH and PDSCH HARQ processes simultaneously. On the other hand, simultaneous reception of sPDSCH and PDSCH for downlink unicast within a single downlink subframe may have more impacts on UE’s reception processing. Hence, it should be studied further.

Proposal 1:

- A sTTI capable UE should be able to simultaneously operate sPDSCH HARQ process(es) and PDSCH HARQ process(es) for downlink unicast.

  - FFS: simultaneous reception of sPDSCH and PDSCH for downlink unicast in a single downlink subframe.

# Supported shortened TTIs

For downlink, reduction of RTT may be dominant for latency reduction. A main motivation to introduce shortened TTI is realizing for reduction of processing time. In terms of receiving process, the processing time reduction at UE side is more challenging than that at eNB side. In this sense, the shorter TTI length (e.g. 1 or 2 OFDM symbols) requires less processing for sPDSCH decoding. In addition, sTTI with 2/3/4 OFDM symbols achieves almost the same BLER performance (or even better performance in high mobility cases) as the legacy PDSCH by using appropriate channel estimation schemes [2][3]. On the other hand, the slot based sTTI can be supported without much standardization effort, since the current RAN1 specification has slot-based initial partial subframe structure already.

Observation 1:

- Shorter TTI length brings more benefit with respect to UE’s processing time reduction, since it requires less processing for sPDSCH decoding.

# Co-existence of different sTTI lengths

From the network point of view, downlink resources should be able to be utilized as much as possible. On the other hand, if assuming a TCP slow start period, a suitable TTI length depends on the TPC packet size, which is different with respect to each UE. Therefore, different sTTI lengths should be able to co-exist within the same subframe.

From the UE perspective, however, there is no need to support reception of sPDSCHs with different TTI length configurations within a single downlink subframe, except for the simultaneous reception of legacy PDSCH and sPDSCH.

Proposal 2:

- A UE is not expected to receive sPDSCHs with different length configurations within a single subframe.

# CSI reporting

Although sPDSCH has shorter TTI length than the legacy PDSCH, the channel characteristics is exactly the same. Furthermore, an appropriate CSI reporting period depends only on time domain channel fluctuation and is independently of the sTTI length. Therefore, CSI reporting should rely on the legacy TTI basis, and there is no need to support sTTI based CSI reporting.

Proposal 3:

- There is no need to support sTTI based CSI reporting.

# Conclusion

In this contribution, we present our views on shortened TTI for downlink transmission and make the following observation and proposals:

Proposal 1:

- A sTTI capable UE should be able to simultaneously operate sPDSCH HARQ process(es) and PDSCH HARQ process(es) for downlink unicast.

  - FFS: simultaneous reception of sPDSCH and PDSCH for downlink unicast in a single downlink subframe.

Observation 1:

- Shorter TTI length brings more benefit with respect to UE’s processing time reduction, since it requires less processing for sPDSCH decoding.

Proposal 2:

- A UE is not expected to receive sPDSCHs with different length configurations within a single subframe.

Proposal 3:

- There is no need to support sTTI based CSI reporting.

# References

3GPP RAN1#84, Chairman’s note, February 2016.

R1-162842, “Link level evaluation results of shortened TTI for CRS based PDSCH”, Sharp, 3GPP RAN1#84bis, April 2016.

R1-162843, “Link level evaluation results of shortened TTI for DM-RS based PDSCH”, Sharp, 3GPP RAN1#84bis, April 2016.