3GPP TSG RAN WG1 Meeting #84bis	R1-162976

Busan, Korea 11th - 15th April 2016

Source: 	Intel Corporation

Title:                     	NB-IoT Secondary Synchronization Signal Design

Agenda item:	7.2.1.1.4

Document for:	Discussion and Decision

# 1 Introduction

At RAN WG1 Meeting #84 [1] and NB-IOT RAN WG1 Adhoc meeting [2], a number of agreements were made on the Secondary Synchronization Signal design. All agreements relevant to NB-SSS design are copied below:

Agreements

- NB-SSS is transmitted in subframe 9

- Number of symbols for NB-SSS: 11

- NB-SSS base sequence is constructed from one or more ZC sequences

  - Length FFS

  - FFS whether multiple root sequences are used or a binary scrambling code

- One transmission of NB-PSS, NB-SSS, NB-PBCH, and NB-PDSCH never overlaps between multiple LTE PRB bandwidths for in-band operation

- The number of subcarriers for NB-SSS is 12

- NB-SSS uses the last [11] or [9] OFDM symbols of subframes  in which NB-SSS occurs for normal CP

In this contribution, we discuss potential issues with the NB-SSS design discussed in [4], [5]. In addition, we also present our NB-SSS design from [6].

# NB-Secondary Synchronization Signal Design

In this section, we discuss various options of NB-SSS sequence design. A summary of all NB-SSS proposals is given in [3]. All the proposals use a combination of ZC sequence root indices and scrambling sequence or cyclic shifts to indicate the physical cell ID (PCID). However, the designs differ in the way the PCID information is encoded and/or in the way NB-SSS sequence is generated (time-domain or frequency-domain generation). The time-domain generation does not give any additional benefits, but requires extra processing at the base station for NB-SSS generation [8], [9]. Amongst the frequency domain generation options, the proposals primarily differ in either the choice of ZC sequence root index or the way the cell ID information is encoded (scrambling sequence or cyclic shift). [6], [10], [11] use a 131 length ZC sequence, whereas [12] uses a 127 length ZC sequence and cyclically extend it to length 132 (corresponding to the number of resources occupied). Additionally, these proposals differ in use of cyclic shifts or scrambling sequences to encode the additional information needed to represent the PCID (note that 127 or 131 root indices can only represent 127 or 131 PCIDs respectively). We prefer the option of using scrambling sequences to encode the additional information, as presented in Section 2.2 because it allows for the design of low-complexity detection, atleast for the UEs experiencing good SNR. Differentially decoding a ZC sequence gives a phase ramp sequence, which upon IFFT operation directly yields the root of the ZC sequence, thus eliminating the need to perform cross-correlations with all possible ZC sequence root indices. We next discuss the issues with using combination of ZC sequence root indices to represent the PCID.

## 2.1 Combination of ZC sequence root indices generated in frequency domain

A combination of 11 Zadoff Chu sequences and cyclic shifts are used to indicate the physical cell ID in [4]. The 11 Zadoff Chu sequences are generated in frequency domain and are then mapped to the 11 symbols. Each symbol carries independent information and needs to be correctly decoded in order to obtain the physical cell ID. If a user is at the cell edge, the user may see signals of similar strength from two or possibly three cells. In such a scenario, a user can decode certain ZC sequences from one cell and another set of ZC sequences from another cell, leading to a cell-ambiguity problem. The idea of carrying cell ID information using multiple sequences is the same as proposed earlier in LTE SSS, where a combination of two ZC sequences was used to represent cell ID. Such ambiguity and collision problems [7] are well known and that is why two types of scrambling sequences were proposed (PSS based scrambling and SSS1 based scrambling on SSS2 in order to resolve the issue). A similar sequence design is presented in [5], where a combination of two ZC sequences is used to encode the Cell ID information. Additionally, scrambling based on the PCID is proposed to resolve the issue. However, the problem is not completely resolved and there is still a 3dB sidelobe of the cross-correlation values when one of two roots is identical. Moreover, designing unique scrambling sequences with good cross correlation properties for each of the N = 504 cell IDs can be a challenge. In some scenarios, additional verification of the cell ID may be needed. Figure 1 shows such an ambiguous cell search scenario and the possible problematic scenarios are shown in Table 1 (copied from [7]).

Figure 1 Cell Search Scenario [7]

Table 1. 8 scenarios for PSC, SSC1, SSC2 triplet [7]

| No. | Pa & Pb | S1a & S1b | S2a & S2b | Problem |
| --- | --- | --- | --- | --- |
| 1 | Same | Same | Same | n/a |
| 2 | Same | Same | Different | Collision + Phase mismatch |
| 3 | Same | Different | Same | Collision + Phase mismatch |
| 4 | Same | Different | Different | Ambiguity + Phase mismatch |
| 5 | Different | Same | Same | Phase mismatch |
| 6 | Different | Same | Different | Collision + Phase mismatch |
| 7 | Different | Different | Same | Collision + Phase mismatch |
| 8 | Different | Different | Different | Ambiguity + Phase mismatch |

Thus, unless the additional scrambling (increasing UE complexity) is introduced, use of multiple ZC sequences to carry cell ID information should be avoided.

Proposal 1: A combination of ZC sequences is not used to represent cell ID information for NB-SSS design.

## 2.2 Proposed Secondary Synchronization Signal Design

The Secondary Synchronization Sequence (NB-SSS) is used in NB-IOT to achieve frame synchronization and detect one out of the N = 504 cell IDs. The NB-SSS design we discuss here is slightly different from the one mentioned in [2]. NB-SSS is generated in the frequency domain, occupies 12 subcarriers and is spread across 11 OFDM symbols. The NB-SSS symbols corresponding to the LTE RS are punctured. The NB-SSS is composed of a length NZC= 131 Zadoff-Chu (ZC) sequence cyclically extended to occupy 132 resources (other option would be the truncation of 5 last elements from length-137 ZC sequence). This 131 length ZC sequence can be generated using different roots and scrambled with unique sequences to provide 504 unique cell identities. To support N=504 cell IDs, we propose the use of different root indices and scrambling codes - four scrambling codes would be sufficient to cover 504 cell IDs (130*4=520>504). This NB-SSS sequence is transmitted every 20 ms. A minimum periodicity of 20 ms might be needed for acceptable NB-SSS detection performance for coverage-extension UEs. The time domain allocation is shown in Fig. 1. If additional information needs to be indicated (e.g. frame number information), (e.g. dmax=4) different time domain cyclic time shifts can be applied to the frequency domain SSS sequence.

Figure 2. Time domain location of NB-PSS and NB-SSS transmission for NB-IoT

We next provide the details of the sequence design for the NB-SSS sequence supporting 504 cell IDs, p=0,1,…,503. We use a combination of the root of the ZC sequence and a scrambling code to represent the cell ID of a cell. Specifically, the root index, say rp and scrambling sequence index, say sp uniquely represent a cell ID p, where

rp=1+modp,130 and

sp= p130.

To represent the location at ‘d’ different radio frame locations, we use time-domain cyclic shifts lq, where q =0, 1, 2, …, dmax-1.

Note that rp takes the values from {1,…,130}, while sp takes on the values {0, 1, 2, 3}. The transmitter is shown in Fig. 2.

Figure 3. Transmitter for NB-SSS generation

The NB-SSS sequence is given by

SSSn=e-jπrp(n'n'+1NZC cspn e-j2πlqndmax,       where n'=n mod NZc ,n=0,1,…,NZC,  lq={0, 1, 2, …, dmax-1}

where cspn is the scrambling sequence. If sending NB-SSS dmax=4 times like in Fig. 2, the following values of  can be used.

l0 =0, l1=33, l2=66, l3=99

The scrambling sequence can be a M-sequence or a PN sequence with the 4 different sequences chosen to have good autocorrelation and cross-correlation properties. We choose NZc=131 in the equation above, which is a prime number.

Proposal 2: It is proposed for NB-SSS:

- A combination of root index of Zadoff Chu sequence and scrambling sequence is used for carrying the cell ID information.

- NB-SSS generation uses ZC sequence of length 131.

- The periodicity of NB-PSS should be 20 ms.

- To embed the frame number information (for e.g., 2 bits of information corresponding to location within a 80 ms NB-frame), different cyclic time shifts can be applied to the frequency domain SSS sequence.

# Performance Evaluations

The simulation assumptions are tabulated in Table 1. For standalone mode, a transmit filter consisting of 19 taps is used in order that the NB-IoT signal fulfils the GSM PSD mask. This is essential to reduce the leakage outside the transmission bandwidth, thereby causing minimal interference to other systems, e.g., GSM. For guard-band and in-band operation, no such filter is required. Three levels of coverage enhancement for each operation mode (i.e. standalone, guard-band and in-band) are considered in the simulations, corresponding to coupling loss values of 164 dB, 154 dB and 144 dB. The correlation outputs are accumulated over multiple 20 ms windows. Rest of the simulation assumptions are summarized in Table 1.

Table 1: Simulation Parameters

| Parameter | Value |
| --- | --- |
| Carrier Frequency | 900 MHz |
| Channel Model | TU 1 Hz |
| Subcarrier Spacing | 15 kHz |
| Coupling loss | 144 dB, 154 dB, 164 dB |
| Sampling Frequency (Fs) | 1.92 MHz (A/D) |
| Carrier frequency offset | +/- 50 Hz |
| Cyclic Prefix | 10 samples for the 1st and 7th OFDM symbol within a subframe, 9 samples for the rest |
| Antenna Configuration | 1 Tx, 1 Rx for standalone 2 Tx, 1 Rx for guard-band/in-band |
| Number of realizations | 1000 |

We next report the time after timing and frequency offset estimation required to detect the Cell ID correctly. It is assumed that the timing is known correctly and the CFO is +/- 50 Hz post NB-PSS detection. Once the NB-IOT UE has timing and frequency information from NB-PSS, the UE tries look for NB-SSS in the next 20ms. If needed, the UE can accumulate over multiple 20ms subframes to detect NB-SSS. Correct NB-SSS detection leads to detection of cell ID as well as frame number information. We report the average, 50th percentile and 95th percentile for the three coupling losses (144 dB, 154 dB, 164 dB) in Tables 2, 3 and 4 for standalone, guard-band and in-band operation modes respectively.

Table 2: Time required for Cell ID detection (in ms) for standalone operation

| Statistic | 144 dB coupling loss | 154 dB coupling loss | 164 dB coupling loss |
| --- | --- | --- | --- |
| Average | 20 | 20.8 | 34 |
| 50th percentile | 20 | 20 | 20 |
| 95th percentile | 20 | 20 | 110 |

Table 3: Time required for Cell ID detection (in ms) for guard-band operation

| Statistic | 144 dB coupling loss | 154 dB coupling loss | 164 dB coupling loss |
| --- | --- | --- | --- |
| Average | 20 | 23.4 | 58.93 |
| 50th percentile | 20 | 20 | 40 |
| 95th percentile | 20 | 40 | 220 |

Table 4: Time required for Cell ID detection (in ms) for in-band operation

| Statistic | 144 dB coupling loss | 154 dB coupling loss | 164 dB coupling loss |
| --- | --- | --- | --- |
| Average | 20.2 | 25.8 | 65.84 |
| 50th percentile | 20 | 20 | 40 |
| 95th percentile | 20 | 60 | 240 |

# Conclusions

We described possible issues with the design in [3][4] leading to cell-ambiguity problem and proposed NB-SSS sequence design using a combination of Zadoff Chu sequence and sequence scrambling to indicate the cell ID. We also presented simulation results for the same. In conclusion, we propose the following:

Proposal 1: The combination of ZC sequences are not used to represent cell ID information for NB-SSS design.

Proposal 2: It is proposed for NB-SSS:

- A combination of root index of Zadoff Chu sequence and scrambling sequence is used for carrying the cell ID information.

- NB-SSS generation uses ZC sequence of length 131.

- The periodicity of NB-PSS should be 20 ms.

- To embed the frame number information (for e.g., 2 bits of information corresponding to location within a 80 ms NB-frame), different cyclic time shifts can be applied to the frequency domain SSS sequence.

# References

- “RAN 1 Chairman notes,” 3GPP TSG-RAN1 Meeting #84, St Julian’s, Malta, Feb 2016.

- “RAN 1 Chairman notes,” 3GPP TSG-RAN1 NB-IOT Ad Hoc, Budapest, Hungary, Jan 2016.

- R1-162047, “WF from offline discussion on NB-SS,’’ Intel Corporation, 3GPP TSG-RAN1 NB-IOT Ad Hoc#2, Sophia-Antipolis, France, Mar 2016.

- R1-161981, “NB-PSS and NB-SSS Design (Revised),’’ Qualcomm Incorporated, 3GPP TSG-RAN1 NB-IOT Ad Hoc#2, Sophia-Antipolis, France, Mar 2016.

- R1-161968, “Synchronization signal design for NB-IoT,’’ LG Electronics, 3GPP TSG-RAN1 NB-IOT Ad Hoc#2, Sophia-Antipolis, France, Mar 2016.

- R1-161896, “NB-IoT Secondary Synchronization Signal design,” Intel Corporation, 3GPP TSG-RAN1 NB-IOT Ad Hoc#2, Sophia-Antipolis, France, Mar 2016.

- R1-073434, “Secondary SCH Mapping and Scrambling,’’ Texas Instruments, RAN 1 # 50, Athens, Greece, Aug 2007.

- R1-161981, “NB-SSS Design,’’ Huawei, HiSilicon, 3GPP TSG-RAN1 NB-IOT Ad Hoc#2, Sophia-Antipolis, France, Mar 2016.

- R1-161863, “Remaining issues on PSS and SSS for NB-IoT,’’ ZTE, 3GPP TSG-RAN1 NB-IOT Ad Hoc#2, Sophia-Antipolis, France, Mar 2016.

- R1-161946, “Remaining issues on PSS and SSS for NB-IoT,’’ MediaTek, 3GPP TSG-RAN1 NB-IOT Ad Hoc#2, Sophia-Antipolis, France, Mar 2016.

- R1-161960, “Remaining details on synchronization signal design for NB-IoT,’’ Nokia Networks, Alcatel-Lucent, Alcatel-Lucent Shanghai Bell, 3GPP TSG-RAN1 NB-IOT Ad Hoc#2, Sophia-Antipolis, France, Mar 2016.

- R1-161829, “NB-IoT- NSSS Design,’’ Ericsson, 3GPP TSG-RAN1 NB-IOT Ad Hoc#2, Sophia-Antipolis, France, Mar 2016.