3GPP TSG RAN Meeting #84-bis	R1-162984

Busan, Korea, 11th – 15th April 2016

Agenda item:	7.2.1.2.4

Source:	NEC

Title:	Discussion on description of NPRACH frequency hopping generation

Document for:	Discussion

Introduction

Two explicit parameters for NPRACH configuration in frequency domain for NPRACH were agreed as in[2]:

- NPRACH resource subcarrier offset (nprach-SubcarrierOffset) ∊ {0, 12, 24, 36, 2, 18, 34}

- NPRACH resource number of subcarriers (nprach-NumSubcarriers) ∊ {12, 24, 36, 48}

In this contribution, we discuss about combination of those parameters.

Discussion

Table 1 summarizes the range of nscRAi, which describes frequency hopping of NPRACH in [1], for each combination of two NPRACH parameters, nprach-SubcarrierOffset and nprach-NumSubcarriers, or NscoffsetNPRACH and NscNPRACH.

A number of combinations of these parameters would be invalid because initial position of NPRACH in frequency domain, nscRA0, and frequency hopping could be outside of NB-IoT band. It seems there is no explicit agreement on UE behavior when it is received invalid combination. There would be some options, e.g. UE shall pick appropriate initial position so that hopping would be performed within NB-IoT band, or RRC signaling shall be rejected, or UE behavior is not specified, etc.

Proposal:

UE behavior should be clarified when the UE receives invalid combination of nprach-SubcarrierOffset and nprach-NumSubcarriers

Table 1: range of nscRAi for NPRACH configuration in frequency domain

| Range of nscRAi |  | nprach-SubcarrierOffset (NscoffsetNPRACH) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 12 | 24 | 36 | 2 | 18 | 34 |
| nprach-NumSubcarriers (NscNPRACH) | 12 | 0 – 11 | 12 – 23 | 24 – 35 | 36 – 47 | 2 – 13 | 18 – 29 | 34 – 45 |
|  | 24 | 0 – 23 | 12 – 35 | 24 – 47 | Invalid | 2 – 25 | 18 – 41 | Invalid |
|  | 36 | 0 – 35 | 12 – 47 | Invalid | Invalid | 2 – 37 | Invalid | Invalid |
|  | 48 | 0 – 47 | Invalid | Invalid | Invalid | Invalid | Invalid | Invalid |

Conclusion

Proposal: UE behavior should be clarified when the UE receives invalid combination of nprach-SubcarrierOffset and nprach-NumSubcarriers in Table 1.

Reference

[1] R1-163380	Introduction of NB-IoT	Ericsson

[2] RRC parameter list		Huawei