3GPP TSG RAN WG1 Meeting #84bis 	R1-163399

Busan, Korea, 11th - 15th April 2016

Source:	Sharp

Title:	Link level evaluation results of shortened TTI for CRS based PDSCH

Agenda Item:	7.3.10.1

Document for:	Discussion and Decision

Introduction

In the SID [1], the following objectives were captured on the latency reduction study item [1].

Agreements: - According to the SID, the following aspect should be studied in RAN1 - From RAN1#83: TTI shortening and reduced processing times [RAN1]: - Assess specification impact and study feasibility and performance of TTI lengths between 0.5ms and one OFDM symbol, taking into account impact on reference signals and physical layer control signaling - backwards compatibility shall be preserved (thus allowing normal operation of pre-Rel 13 UEs on the same carrier); RAN1 should perform evaluations in order to understand the feasibility and performance of TTI shortening

In this contribution, we show the evaluation results of CRS based PDSCH transmission.

BLER performance of sPDSCH

sPDSCH structure for CRS-based transmission

Figure 1 shows sPDSCH structure for slot, 3/4-symbol and 2-symbol TTI with 2 antenna ports and 2 PDCCH symbols in our evaluation.

Figure 1, sPDSCH structure for slot, 3/4-symbol and 2-symbol TTI

Evaluation results

We evaluated slot TTI, 3/4-symbol TTI and 2-symbol TTI with respect to BLER performance according to the agreed assumptions [2]. In this evaluation, we evaluated rank-1 transmission. The detailed simulation assumptions are listed in the section 6.1 in Annex.

Regarding the CRS assumptions for demodulation, 3 schemes are assumed in the section 7.1 in Appendix, and scheme 3 is applied in this evaluation. The performances of scheme 1 and 2 are shown in the section 7.1 in Appendix. Throughput performances are shown in Appendix to capture the TR.

Low mobility case

Figure 2 shows BLER performance for each sTTI in low mobility case, and also it shows BLER performance of the legacy PDSCH as a reference. Figure 2 (a) represents EPA 3 km/h, and (b) represents ETU 3 km/h. As shown in these figures, any TTI length of 2-symbol, 3/4-symbol, and slot TTI is similar performance of that of legacy TTI.

Observation 1:

- In low mobility case, short TTI lengths of 2-symbol, 3/4-symbol, and slot-TTI achieve comparable BLER performances to the legacy TTI length..

| (a) EPA, 3 km/h | (b) ETU, 3 km/h |
| --- | --- |
| Figure 2. BLER performances (low mobility case, rank 1) |  |

Table 1. SINR at 10% BLER (EPA 3 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 3.41 dB | 3.41 dB |  | 3.82 dB | 3.56 dB |
| 16QAM 3/4 |  | 14.46 dB | 14.43 dB |  | 14.85 dB | 14.31 dB |
| 64QAM 5/6 |  | 21.37 dB | 21.54 dB |  | 21.91 dB | 21.24 dB |

Table 2. SINR at 10% BLER (ETU 3 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 1.37 dB | 1.48 dB |  | 1.77 dB | 1.3 dB |
| 16QAM 3/4 |  | 12.67 dB | 12.64 dB |  | 13.02 dB | 12.9 dB |
| 64QAM 5/6 |  | 19.94 dB | 20.06 dB |  | 20.5 dB | 20.14 dB |

High mobility case

Figure 3 shows BLER performance for each sTTI in high mobility case. Figure3 (a) represents EVA 60 km/h, and (b) represents ETU 60 km/h.

As shown in these figures, short TTIs with length of 2-symbol, 3/4-symbol, and slot TTI have similar performances to that of legacy TTI in QPSK and 16QAM cases. On the other hand, the performances of 3/4-symbol and 2-symbol TTI are slightly degraded in 64QAM case. Because for some sPDSCHs (e.g. sPDSCH#0 in Figure 1(b)), the UE cannot observe sufficient time domain samples until the end of the sPDSCH reception for an accurate interpolation of the time-variant channel. However, some other sPDSCHs (e.g. sPDSCH#3 in Figure 1(b)) still achieve comparable BLER performance to the legacy TTI length. The performance in Figure 3 shows the average of all short TTIs with a given TTI length.

Observation 2:

- In high mobility case,

  - for QPSK and 16QAM, short TTIs with length of 2-symbol, 3/4-symbol, and slot TTI have similar performances to that of legacy TTI ,

  - for 64QAM, BLER performances of 3/4-symbol and 2-symbol TTI are slightly degraded (up to 2.5dB).

| (a) EVA, 60 km/h | (b) ETU, 60 km/h |
| --- | --- |
| Figure 3. BLER performances (high mobility case, Scheme 3) |  |

Table 3. SINR at 10% BLER (EVA 60 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 1.93 dB | 2.09 dB |  | 2.17 dB | 1.79 dB |
| 16QAM 3/4 |  | 13.12 dB | 13.65 dB |  | 13.65 dB | 13.03 dB |
| 64QAM 5/6 |  | 21.67 dB | 22.64 dB |  | 20.71 dB | 20.28 dB |

Table 4. SINR at 10% BLER (ETU 60 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 1.5 dB | 1.35 dB |  | 1.58 dB | 1.34 dB |
| 16QAM 3/4 |  | 12.81 dB | 13.06 dB |  | 13.03 dB | 12.62 dB |
| 64QAM 5/6 |  | 21.23 dB | 21.83 dB |  | 20.54 dB | 20.23 dB |

Conclusion

In this contribution, we have the following observations:

Observation 1:

- In low mobility case, short TTI lengths of 2-symbol, 3/4-symbol, and slot-TTI achieve comparable BLER performances to the legacy TTI length..

Observation 2:

- In high mobility case,

  - for QPSK and 16QAM, short TTIs with length of 2-symbol, 3/4-symbol, and slot TTI have similar performances to that of legacy TTI ,

  - for 64QAM, BLER performances of 3/4-symbol and 2-symbol TTI are slightly degraded (up to 2.5dB).

References

RAN1 chairman’s note in RAN1#84, February 2016.

R1-157806, “WF on evaluation methodology for latency reduction,” RAN1#83, November 2015.

R1-161422, “Way forward on simulation results and methodology”, RAN1#84, February 2016.

Annex

Evaluation assumptions

Table 5: Evaluation assumption

| Parameter | Value |
| --- | --- |
| Carrier frequency | 2 GHz |
| TTI length | 2, 3/4, 7 symbols |
| Allocated bandwidth | 50 PRBs (10 MHz) |
| Channel model | EPA, EVA, ETU |
| UE speed | 3km/h , 60km/h |
| Antenna configuration | 2Tx (eNB), 2Rx (UE) |
| Antenna correlation | Uncorrelated |
| Legacy PDCCH region | 2 OFDM symbols |
| CP length | Normal |
| Transmission mode | TM4 |
| RS configuration | 2 CRS ports |
| Receiver type | MMSE |
| Channel estimation | Practical |
| Rank adaptation | Fixed Rank 1 |
| Link adaptation | Disabled |
| Modulation and code rate | 64QAM 5/6, 16QAM 3/4, QPSK 1/3 |
| Precoding codebook | Fixed |
| HARQ retransmission | Disabled |
| Performance metrics | BLER, Throughput |
| Control channel overhead | 2CCE (72RE) reserved for short PDCCH transmission each short TTI |

Throughput performances to capture TR

Table 6 – 9 show the throughput performances at 10% BLER to capture TR.

Table 6. Throughput at 10% BLER (EPA 3 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 3.73 Mbps | 3.95 Mbps |  | 3.88 Mbps | 3.88 Mbps |
| 16QAM 3/4 |  | 17.42 Mbps | 18.00 Mbps |  | 17.44 Mbps | 17.58 Mbps |
| 64QAM 5/6 |  | 29.20 Mbps | 30.32 Mbps |  | 29.52 Mbps | 28.97 Mbps |

Table 7. Throughput at 10% BLER (ETU 3 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 3.53 Mbps | 3.78 Mbps |  | 3.79 Mbps | 3.64 Mbps |
| 16QAM 3/4 |  | 16.70 Mbps | 17.00 Mbps |  | 16.28 Mbps | 16.58 Mbps |
| 64QAM 5/6 |  | 29.30 Mbps | 30.33 Mbps |  | 28.08 Mbps | 29.14 Mbps |

Table 8. Throughput at 10% BLER (EVA 60 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 3.78 Mbps | 3.99 Mbps |  | 3.84 Mbps | 3.85 Mbps |
| 16QAM 3/4 |  | 16.85 Mbps | 17.26 Mbps |  | 17.26 Mbps | 16.78 Mbps |
| 64QAM 5/6 |  | 29.01 Mbps | 30.84 Mbps |  | 28.50 Mbps | 28.84 Mbps |

Table 9. Throughput at 10% BLER (ETU 60 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 4.16 Mbps | 3.68 Mbps |  | 3.64 Mbps | 3.38 Mbps |
| 16QAM 3/4 |  | 19.24 Mbps | 17.22 Mbps |  | 16.36 Mbps | 16.06 Mbps |
| 64QAM 5/6 |  | 27.81 Mbps | 30.60 Mbps |  | 28.45 Mbps | 28.84 Mbps |

Appendix

Channel estimation for demodulation in each sTTI

We can consider the following three channel estimation schemes:

- CRS included in OFDM symbols within 1 msec before the scheduled TTI is used for channel estimation.

- CRS from the first OFDM symbol the legacy subframe boundary including the scheduled sTTI to the scheduled TTI is used for channel estimation.

- If CRS is present within the scheduled TTI, only the CRS is used for channel estimation. Otherwise, CRS within one TTI before the scheduled TTI.

Figure A-1 shows Scheme 1 for UE#4 as an example. In this figure, when, UE#4 demodulates the received signal in the scheduled TTI, all the samples within 1 msec before the scheduled TTI can be used. This has a benefit for the UE at low speed condition from the channel estimation accuracy point of view.

Figure A-2 shows Scheme 2 and Scheme 3 for UE#4 as an example. The Scheme 2 is to use all the CRS samples in the subframe including the scheduled TTI and before the scheduled TTI. This assumption can be adopted in case that UE can only use the CRS samples on one subframe which includes the TTI scheduled for UE. The Scheme 3 is the simplest one based on the channel estimation only by CRS samples on scheduled TTI or nearest CRS samples among already received. However, in this regard, since there can be no CRS within a TTI, CRS in one TTI before the scheduled TTI is used for channel estimation for demodulation when CRS is not present in the scheduled TTI. In this evaluation, we evaluated the three assumptions are assumed for link level evaluations.

Figure A-1. Averaging window of channel estimation in Scheme 1

Figure A-2. Averaging window of channel estimation in Scheme 2 and 3

BLER performance for Scheme 3 is shown in Figure 2 and 3 in the section 2. Throughput performance for Scheme 3 is shown in Figure A-3 and A-4.

| (a) EPA, 3 km/h | (b) ETU, 3 km/h |
| --- | --- |
| Figure A-3. Throughput performances (low mobility case, Scheme 3) |  |

| (a) EVA, 60 km/h | (b) ETU, 60 km/h |
| --- | --- |
| Figure A-4. Throughput performances (high mobility case, Scheme 3) |  |

BLER performance results for Scheme 1 and Scheme 2 are shown in Figure A-5 to A-8. It is observed that Scheme 1 shows the best performance due to the linear interpolation in high mobility case (as shown in Figure A-6).

| (a) EPA, 3 km/h | (b) ETU, 3 km/h |
| --- | --- |
| Figure A-5. BLER performances (low mobility case, Scheme 1) |  |

| (a) EVA, 60 km/h | (b) ETU, 60 km/h |
| --- | --- |
| Figure A-6. BLER performances (high mobility case, Scheme 1) |  |

| (a) EPA, 3 km/h | (b) ETU, 3 km/h |
| --- | --- |
| Figure A-7. BLER performances (low mobility case, Scheme 2) |  |

| (a) EVA, 60 km/h | (b) ETU, 60 km/h |
| --- | --- |
| Figure A-8. BLER performances (high mobility case, Scheme 2) |  |

Throughput performance results are shown in Figure A-9 to A-12. Shorter TTI transmission is beneficial from the throughput performance point of view as shown below.

| (a) EPA, 3 km/h | (b) ETU, 3 km/h |
| --- | --- |
| Figure A-9. Throughput performances (low mobility case, Scheme 1) |  |

| (a) EVA, 60 km/h | (b) ETU, 60 km/h |
| --- | --- |
| Figure A-10. Throughput performances (high mobility case, Scheme 1) |  |

| (a) EPA, 3 km/h | (b) ETU, 3 km/h |
| --- | --- |
| Figure A-11. Throughput performances (low mobility case, Scheme 2) |  |

| (a) EVA, 60 km/h | (b) ETU, 60 km/h |
| --- | --- |
| Figure A-12. Throughput performances (high mobility case, Scheme 2) |  |

Table A-1 to A-4 show the SINR at 10% BLER for Scheme 1.

Table A-1. SINR at 10% BLER (EPA 3 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 4.59 dB | 5.18 dB |  | 3.8 dB | 3.56 dB |
| 16QAM 3/4 |  | 15.52 dB | 16.55 dB |  | 14.86 dB | 14.31 dB |
| 64QAM 5/6 |  | 22.94 dB | 23.59 dB |  | 22.03 dB | 21.24 dB |

Table A-2. SINR at 10% BLER (ETU 3 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 2.26 dB | 3.42 dB |  | 1.71 dB | 1.3 dB |
| 16QAM 3/4 |  | 14.46 dB | 15.13 dB |  | 13.08 dB | 12.9 dB |
| 64QAM 5/6 |  | 21.66 dB | 22.36 dB |  | 20.59 dB | 20.14 dB |

Table A-3. SINR at 10% BLER (EVA 60 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 3.79 dB | 4.03 dB |  | 2.35 dB | 1.79 dB |
| 16QAM 3/4 |  | 14.61 dB | 15.2 dB |  | 13.41 dB | 13.03 dB |
| 64QAM 5/6 |  | 21.95 dB | 22.79 dB |  | 20.65 dB | 20.28 dB |

Table A-4. SINR at 10% BLER (ETU 60 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 2.84 dB | 3.34 dB |  | 1.24 dB | 1.75 dB |
| 16QAM 3/4 |  | 14.42 dB | 14.97 dB |  | 13.1 dB | 12.85 dB |
| 64QAM 5/6 |  | 21.92 dB | 22.56 dB |  | 20.65 dB | 20.24 dB |

Table A-5 to A-8 show the SINR at 10% BLER for Scheme 2.

Table A-5. SINR at 10% BLER (EPA 3 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 4.05 dB | 4.53 dB |  | 3.93 dB | 3.56 dB |
| 16QAM 3/4 |  | 15.07 dB | 15.29 dB |  | 14.85 dB | 14.31 dB |
| 64QAM 5/6 |  | 21.97 dB | 22.73 dB |  | 21.91 dB | 21.24 dB |

Table A-6. SINR at 10% BLER (ETU 3 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 2.26 dB | 2.61 dB |  | 1.77 dB | 1.3 dB |
| 16QAM 3/4 |  | 13.66 dB | 14.16 dB |  | 13.02 dB | 12.9 dB |
| 64QAM 5/6 |  | 20.59 dB | 21.36 dB |  | 20.5 dB | 20.14 dB |

Table A-7. SINR at 10% BLER (EVA 60 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 2.72 dB | 3.08 dB |  | 2.17 dB | 1.79 dB |
| 16QAM 3/4 |  | 14.02 dB | 14.35 dB |  | 13.65 dB | 13.03 dB |
| 64QAM 5/6 |  | 21.78 dB | 22.76 dB |  | 20.71 dB | 20.28 dB |

Table A-8. SINR at 10% BLER (ETU 60 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 3.67 dB | 3.78 dB |  | 3.64 dB | 1.75 dB |
| 16QAM 3/4 |  | 16.85 dB | 17.59 dB |  | 16.36 dB | 12.85 dB |
| 64QAM 5/6 |  | 28.34 dB | 30.60 dB |  | 28.45 dB | 20.24 dB |

Table A-9 to A-12 show the throughput performances at 10% BLER for Scheme 1.

Table A-9. Throughput at 10% BLER (EPA 3 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 3.73 Mbps | 3.98 Mbps |  | 3.88 Mbps | 3.88 Mbps |
| 16QAM 3/4 |  | 17.28 Mbps | 18.30 Mbps |  | 17.49 Mbps | 17.58 Mbps |
| 64QAM 5/6 |  | 28.90 Mbps | 30.46 Mbps |  | 29.45 Mbps | 28.97 Mbps |

Table A-10. Throughput at 10% BLER (ETU 3 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 3.66 Mbps | 3.91 Mbps |  | 3.75 Mbps | 3.64 Mbps |
| 16QAM 3/4 |  | 17.02 Mbps | 17.82 Mbps |  | 16.40 Mbps | 16.58 Mbps |
| 64QAM 5/6 |  | 28.57 Mbps | 30.81 Mbps |  | 28.43 Mbps | 29.14 Mbps |

Table A-11. Throughput at 10% BLER (EVA 60 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 3.74 Mbps | 4.08 Mbps |  | 3.80 Mbps | 3.85 Mbps |
| 16QAM 3/4 |  | 17.09 Mbps | 17.75 Mbps |  | 16.88 Mbps | 16.78 Mbps |
| 64QAM 5/6 |  | 29.22 Mbps | 30.08 Mbps |  | 28.66 Mbps | 28.84 Mbps |

Table A-12. Throughput at 10% BLER (ETU 60 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 3.51 Mbps | 3.88 Mbps |  | 3.62 Mbps | 3.77 Mbps |
| 16QAM 3/4 |  | 17.06 Mbps | 17.63 Mbps |  | 16.47 Mbps | 16.57 Mbps |
| 64QAM 5/6 |  | 29.14 Mbps | 30.22 Mbps |  | 28.47 Mbps | 28.86 Mbps |

Table A-13 to A-16 show the throughput performances at 10% BLER for Scheme 2.

Table A-13. Throughput at 10% BLER (EPA 3 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 3.79 Mbps | 3.91 Mbps |  | 3.90 Mbps | 3.88 Mbps |
| 16QAM 3/4 |  | 17.32 Mbps | 17.77 Mbps |  | 17.44 Mbps | 17.58 Mbps |
| 64QAM 5/6 |  | 29.41 Mbps | 29.92 Mbps |  | 29.52 Mbps | 28.97 Mbps |

Table A-14. Throughput at 10% BLER (ETU 3 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 3.66 Mbps | 3.70 Mbps |  | 3.79 Mbps | 3.64 Mbps |
| 16QAM 3/4 |  | 17.09 Mbps | 17.79 Mbps |  | 16.28 Mbps | 16.58 Mbps |
| 64QAM 5/6 |  | 27.47 Mbps | 28.92 Mbps |  | 28.08 Mbps | 29.14 Mbps |

Table A-15. Throughput at 10% BLER (EVA 60 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 3.66 Mbps | 3.79 Mbps |  | 3.84 Mbps | 3.85 Mbps |
| 16QAM 3/4 |  | 17.56 Mbps | 17.51 Mbps |  | 17.26 Mbps | 16.78 Mbps |
| 64QAM 5/6 |  | 29.01 Mbps | 30.70 Mbps |  | 28.50 Mbps | 28.84 Mbps |

Table A-16. Throughput at 10% BLER (ETU 60 km/h, rank 1)

| sTTI length | 1 symbol | 2 symbols | 3 symbols | 4 symbols | 7 symbols | 14 symbols |
| --- | --- | --- | --- | --- | --- | --- |
| QPSK 1/3 |  | 3.67 Mbps | 3.78 Mbps |  | 3.64 Mbps | 3.77 Mbps |
| 16QAM 3/4 |  | 16.85 Mbps | 17.59 Mbps |  | 16.36 Mbps | 16.57 Mbps |
| 64QAM 5/6 |  | 28.34 Mbps | 30.60 Mbps |  | 28.45 Mbps | 28.86 Mbps |