3GPP TSG RAN WG1 Meeting #85 	R1- 164388

Nanjing, China, May 23-27, 2016

Agenda Item:	7.2.2

Source:	Huawei, HiSilicon, AT&T, Ericsson, Intel, KT Corporation, Nokia, NTT DOCOMO, Qualcomm, Samsung

Title:	Discussion on removal of square bracket for cluster level SSP

Document for:	Discussion and decision

Introduction

In the RAN1 #84bis meeting in Busan, the SSP for indoor office scenario has been agreed in [1]. Since there were some comments on the cluster level parameters, some SSP were put into the square bracket.

In this contribution, based on further investigation on related topics, it is proposing to remove the square bracket and confirm the agreed SSP in [1].

Summary on the cluster level parameters study

As shown in appendix from contribution [2], the number of cluster is less than exitsting model for indoor scenario in . Based on the data in Table 1, the average number of clusters is 4 in [1] for both LOS and NLOS. And the subpath number of 20 is reasonable taking consideration of contribution [6] for UMi scenario and [3] for UMa scenario.

Table 1. Preliminary Indoor office small large-scale channel characteristics

| Parameters | # 73GHz1 | # 28GHz2 | # 3.5GHz3 |  | # 8GHz3 |  | # 28GHz3 |  | # 73GHz3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Hybrid | LOS | LOS | NLOS | LOS | NLOS | LOS | NLOS | LOS | NLOS |
| Number of clusters | 6 | 7 | 4 | 3 | 4 | 3 | 3.271 | 3.5 | 3.9 | 4.3 |
| Number of rays per cluster | 10 | 4 | 20 | 20 | 20 | 20 | 52.9 | 41.82 | 38.47 | 35.75 |

- From Nokia/NYU based on ray-tracing

- From KT based on measurement

- From Huawei based on measurement

Proposal1: the square bracket on the cluster level SSP in [1] can be removed.

In [4], only 12, 19, and 20 are defined as the number of clusters for ZOA/ZOD generation, however, the numbers of clusters for indoor office is 4 in [1]. Hence, it seems necessary to define scaling factors for ZOA/ZOD generation since scaling factor for the number of clusters 4 is not included in [4].

For scaling factor generation, not only the cluster number is taken into consideration, but also other parameters such as delay spread, angular spread and K factor are related. In the case of situation that some LSP and SSP are modified for frequencies above 6 GHz, the scaling factor for angular generation should be recalculated also based on the method in [5] for verification.

In contribution [5], the value of C for NLOS EoD/EoA is proposed by solving the following problem.

|  | CN=argminCElog10σθ-log10σθ2, | (1) |
| --- | --- | --- |

where σθ=n=1NPnθn-μθ2 is the ESD/ESA of the generated clusters,  μθ=n=1NPnθn is the power-weighted mean elevation, σθ is the target ESD/ESA which may follow a distance/UE height-dependent log-normal distribution.  Note that the C depends on neither the target ESD/ESA nor the median offset.

For LOS, the constant C is obtained based on the NLOS version with the same total number of clusters and further tuned by a cubic function of the Ricean-K factor, i.e.

|  | CLOS(N,K)=C(N)⋅r=03arKr, | (2) |
| --- | --- | --- |

where K is the Ricean-K factor in dB, a=a0, a1,a2,a3T is the coefficient vector.  One can first solve the problem in (1) to get K-dependent CLOS(N,K).  Then, a can be obtained by curve fitting.

Note the expectation in (1) can be approximated by sample average over Pn.  Fortunately, Pn can be determined without any assumption on EoD/EoA.  We obtain the constant scaling factor as following.

Scaling factor for AOA, AOD generation

Although scaling factors the AOA, AOD generation is defined when cluster number is 4 in [4], the scaling factors need to be recalculated for verification as angular and delay spread is also modified in [1].

The value of C for NLOS in (1) and LOS case in (1)  is shown in Table 2 for different frequencies.

Table 2 .AOA, AOD Scaling factor for InH parameters

| Scaling factors for AOA/AOD for Indoor-office |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| frequency | asa/asd | C | a0 | a1 | a2 | a3 |
| 6G | asa | 0.82052 | 1.1022 | -0.009273 | -0.0021996 | 0.000010912 |
|  | asd | 0.82223 | 1.1008 | -0.0090269 | -0.002203 | 0.000009623 |
| 30G | asa | 0.82353 | 1.1046 | -0.0086365 | -0.0025224 | 0.000022636 |
|  | asd | 0.82311 | 1.1009 | -0.0088046 | -0.0025086 | 0.000023748 |
| 70G | asa | 0.8243 | 1.1113 | -0.008628 | -0.0028932 | 0.000049604 |
|  | asd | 0.82374 | 1.1138 | -0.0090179 | -0.0028868 | 0.000050137 |
| New equation |  | 0.822905 | 1.1056 | -0.0088978 | -0.0025356 | 2.77767E-05 |
| below 6 GHz |  | 0.779 | 1.1035 | -0.028 | -0.002 | 0.0001 |

There is slightly difference for scaling parameters between different frequencies. Same scaling factors could be adopted for angular generation for frequencies up to 100 GHz. Difference between expectation and AOA generation above 6 GHz are given in Figure 1 and Table 3. Both scaling factor below 6 GHz and values calculated by new parameters above 6 GHz are applied for comparison and verification. The fitting result is similar by these two methods in LOS and NLOS case. the results in TR 36.873 could be reused for  above 6 GHz.

Figure 1. error between expectation and AOA generation by different equation for LOS

Table 3 . error between expectation and AOA generation by different equation for NLOS

|  | frequency | 6G | 30G | 70G |
| --- | --- | --- | --- | --- |
| aoa spread error (log10([°])) | 0.779 (below 6 GHz) | 0.01 | 0.01 | 0.01 |
|  | 0.824 (new equation) | 0.01 | 0.01 | 0.01 |
| aod spread error (log10([°])) | 0.779 ( below 6 GHz) | 0.01 | 0.01 | 0.01 |
|  | 0.824 (new equation) | 0.01 | 0.01 | 0.01 |

Proposal 2: the scaling factors for AOA, AOD generation in TR 36.837 can be reused for InH.

Scaling factor for ZOA, ZOD generation

Taking consideration of parameters in [1], the scaling factors for cluster number 4 could be calculated as following.

Table 4 .ZOA, ZOD Scaling factor for InH parameters

| Scaling factors for ZOA/ZOD for Indoor-office |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| frequency | zsa/zsd | C | a0 | a1 | a2 | a3 |
| 6G | zsa | 0.6596 | 1.3681 | 0.027495 | -0.0020779 | -0.000090063 |
|  | zsd | 0.65948 | 1.3706 | 0.027505 | -0.0021235 | -0.000087993 |
| 30G | zsa | 0.66267 | 1.3695 | 0.028862 | -0.0024438 | -0.000080017 |
|  | zsd | 0.66266 | 1.3692 | 0.028808 | -0.0024288 | -0.000080619 |
| 70G | zsa | 0.66205 | 1.3784 | 0.029057 | -0.0027491 | -0.000063451 |
|  | zsd | 0.66194 | 1.3778 | 0.02914 | -0.0027202 | -0.000065675 |
| New results |  | 0.6614 | 1.372267 | 0.02847783 | -0.002423883 | -7.79697E-05 |
| 36.873 |  | N/A | 1.3086 | 0.0339 | -0.0077 | 0.0002 |

There is slightly difference for scaling parameters between different frequencies. Same scaling factors for LOS case could be adopted for angular generation for frequencies up to 100 GHz. For LOS case, both scaling factor in TR 36.873 and values calculated by new parameters above 6 GHz are compared and verified. The fitting result is similar as shown in Figure 2. Same values for LOS case could be reused for InH.

Figure 2 . error between expectation and ZOA generation by different equation for LOS

Table 5 .ZOA, ZOD Scaling factor for InH

| Scaling factors for ZOA/ZOD for Indoor-office |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| frequency | asa/asd | C | a0 | a1 | a2 | a3 |
| Above 6 GHz |  | 0.661 | 1.3086 | 0.0339 | -0.0077 | 0.0002 |

Proposal 3: the scaling factors for ZOA, ZOD generation in InH can be adopted as Table 5.

Conclusions

In this contribution, based on further investigation on related topics, it is proposing to remove the square bracket and confirm the agreed SSP in [1].

Proposal 1: the square bracket on the cluster level SSP in [1] can be removed.

Proposal 2: the scaling factors for AOA, AOD generation in TR 36.837 can be reused for InH.

Proposal 3: the scaling factors for ZOA, ZOD generation in InH can be adopted as in Table 5.

- References

R1-163472, “Large-scale and small-scale channel modeling for indoor hotspot”, Huawei, HiSilicon, AT&T, CMCC, Ericsson, Intel, KT Corporation, Nokia, NTT DOCOMO, Qualcomm, Samsung, ETRI, CATT, Busan, April. 11-15, 2016.

R1-162246, “Large-scale and small-scale channel modeling for indoor hotspot”, Huawei, HiSilicon, Busan, April. 11-15, 2016.

R1-162572, “Refinement on UMa LSP”, Nokia, AT&T, CMCC, Ericsson, ETRI,  Huawei, HiSilicon, Intel, KT Corporation, NTT DOCOMO, Qualcomm, Samsung, Busan, April. 11-15, 2016.

3GPP TR 36.873 V12.2.0 , June, 2015.

R1-135895, “Remaining issues of EoD/EoA modeling”, Qualcomm, San Francisco, United States,  November 11-15, 2013.

R1-161706, “Joint proposal on large-scale and small-scale parameters for >6GHz channel model”, Samsung, AT&T, CMCC, Ericsson, Huawei, HiSilicon, Intel, KT Corporation, Nokia, NTT DOCOMO, Qualcomm, ETRI, Ljubljana, March. 14-16, 2016.