3GPP TSG-RAN WG1 #85 	R1-164685

23rd – 27th May 2016

Nanjing, China

Agenda item:	7.1.3.1

Source: 	Qualcomm Incorporated

Title: 	OFDM based waveform single user evaluation

Document for:	Discussion/Decision

# Introduction

In RAN1 #84b, several OFDM-based waveform candidates have been proposed for further evaluation in NR [1]. It was further agreed that a few representative test cases should be used as common platform for further comparison across these candidates [2][3]. Specifically, test case 1 and 2 are targeted for RAN1 #85.

In this contribution, we provide further link level simulation and comparisons of a subset of waveform candidates using test case 1, targeting single user (numerology) scenario.

# Test description

Two single user (or numerology) test cases were defined in RAN1 #84b, for the purpose evaluating impact from ISI due to various waveform design.

- Case 1A: wide band tone assignment.

- Case 1B: narrow band tone assignment.

Block diagrams of these two modes are illustrated in Figure 1. More detailed simulation descriptions (as agreed in RAN1 #84b) are also included in

Figure 1 Test case 1A: single user wide band

Figure 2 Test case 1B: single user narrow band

Table 1 Detailed test descriptions agreed in RAN1 #84b

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| Duplex | FDD/TDD |
| System Bandwidth | 10 MHz |
| TTI length | 1 ms as baseline, other TTI length is FFS (e.g. short TTI ) |
| Subcarrier spacing | Single numerology case: 15KHz as baseline, Mixed numerology case: one is 15KHz, and the other subcarrier spacing should be selected by companies from the agreed numerologies. |
| Guard time interval | 4.7us (interval of LTE normal CP) as baseline, other interval is FFS |
| FFT size | e.g. 1024 for 15KHz subcarrier spacing |
| Data transmission bandwidth | Single numerology case: 4 and/or 1 PRBs for the bandwidth of target UE in case 1b Mixed numerology case: - At least two candidate BWs for target UE. - At least two candidate BWs for interfering subband - At least two numerologies. The values are FFS |
| Guard tone number | [0~12] subcarriers for the mixed numerology case |
| Antenna configuration | 1T1R or 2T2R or 4T4R, other is not precluded |
| MIMO mode | TM3, other non-LTE TM model is not precluded |
| Rank per UE | Fixed rank |
| MCS | Fixed. 16QAM: 1/2 or 2/3; 64QAM: 1/2 or 3/4; 256 QAM: 1/2 or 3/4 |
| Control Overhead | Zero |
| Channel estimation * | Ideal NOTE: *Non-ideal effects to be considered in a second stage (e.g., channel estimation, frequency offset, details are FFS) |
| Channel Model ** | TDL (1T1R), CDL (MIMO) in TR38.900 is mandatory, ETU/EPA/EVA are optional. Mobility: 3km/h or 30 km/h or 120 km/h, higher speed is not precluded. NOTE: **For RAN1#85: CDL in TR38.900 or ETU/EPA/EVA can be used |

In this contribution, we try to cover 3 waveform candidates using the above two single user test cases:

- Tx-WOLA (without assuming any WOLA knowledge at the receiver side, and without using Rx-WOLA), as discussed in [1].

- F-OFDM [4].

- UFMC [5], [6].

## Tx-WOLA

Figure 3 Tx-WOLA with CP-OFDM

Tx-WOLA waveform is synthesized by a conventional CP-OFDM waveform followed by weighting and overlap-and-add operation. As described in Figure 3, CP-OFDM is first extended by a cyclic extension in time domain, and both edges are shaped by a weighting function. The resulting symbol is overlap-and-added over time with the next symbol. Raised cosine function is chosen for the weighting function. Therefore, the additional computation complexity compared to the conventional CP-OFDM is equal to WOLA length.

Observation 1: Compared to CP-OFDM, the additional computation required for Tx-WOLA is (WOLA length) per symbol.

## F-OFDM

Figure 4 F-OFDM Transmitter block diagram

F-OFDM waveform is synthesized by a conventional CP-OFDM waveform followed by transmission filtering. The transmission filter is a bandpass filter whose passband matches with the used tones. Especially in [4], the time domain impulse response of the filter is chosen as the Sinc function followed by the Hamming window truncation. Therefore, compared to CP-OFDM, F-OFDM requires additional tx filtering. When the filter is applied in the time domain, the computation complexity for the tx filtering scales as (filter length) per sample. When the tx filtering is applied in the frequency domain, IFFT size in CP-OFDM has to be doubled for an oversampling. Given that the computation complexity of IFFT scales as ϴ(n logn), the filtering results in doubling the computational complexity of CP-OFDM.

Observation 2: If tx filtering is applied in time domain, F-OFDM requires (filter length)×(symbol length) number of more computations per symbol compared to CP-OFDM. If tx filtering is applied in frequency domain, the computation complexity of F-OFDM is doubled compared to that of CP-OFDM.

## UFMC

Figure 5 UFMC Transmitter block diagram

In UFMC waveform transmitter, each RB (resource block)’s waveform is synthesized separately. First, Zero-Padded OFDM waveform corresponding each RB is synthesized in a conventional way, then each Zero-Padded OFDM waveform is filtered by the corresponding bandpass filter. Since the size of each RB are equivalent, the same bandpass filter can be universally used with a frequency shift. Finally, all the waveforms for the RBs are added for the transmission. Especially, in [5], Chebyshev window is used for the bandpass filter.

The main drawback of this naive UFMC waveform synthesis is its computational complexity. Since each RB’s waveform is synthesized separately, the number of IFFT operation has to be equal to the number of RBs. For the wide band users, this complexity issue can be significant.

Observation 3: The naive implementation of UFMC transmitter has (Number of RBs) times more computational complexity compared to CP-OFDM.

Figure 6 Low Complexity Approximate UFMC Transmitter block diagram

An approximated but low-complexity UFMC transmitter is proposed in [6]. In the low-complexity UFMC transmitter, the tones in a RB are first converted to a time-domain signal by smaller size IFFT with size n. Then, the time domain signal is doubled by padding zeros, and converted to the frequency domain by FFT with size 2n. The resulting frequency domain signal is shaped by the bandpass filter which is converted to the frequency domain. This procedure is repeated for each RB, and the shaped frequency domain signals are overlap-and-added. The overlap-and-added frequency domain signal is converted to the time domain by IFFT whose length 2N is twice of the one in Figure 4. Finally, the time-domain signals is truncated to fit to the symbol length and transmitted.

When the smaller IFFT Size n is equal to the larger IFFT Size N, the low complexity UFMC transmitter in Figure 5 reduces to the naive UFMC transmitter in Figure 4. In [6], it is observed that the error due to the low complexity transmitter decreases as n increases.

Compared to CP-OFDM, the low complexity UFMC transmitter has at least twice more computationally complicated to CP-OFDM since the IFFT size is doubled. In addition to that, it also requires (Number of RBs) parallel IFFT and FFT operations with size n and 2n respectively.

Observation 4: The low complexity UFMC transmitter is at least twice more computationally complicated to CP-OFDM. Additionally, (Number of RBs) parallel small-size IFFT and FFT operations are required.

# Simulation Results

## PA Model

The selected PA model is based on the clipping PA model with 6 dB PAPR clipping threshold. This value is chosen to guarantee -30 dBc ACLR at least for the three waveform as shown in the figure below. The corresponding EVM for the three waveforms at different clipping is shown at Figure 6.

Fig 6 ACLR and EVM using the clipping PA model

Observation 5: WOLA achieves the best ACLR and EVM performance, while Filtered-OFDM has the worst ACLR and EVM performance for all clipping thresholds. 6 dB Clipping threshold guarantees at least -30 dBc of ACLR and -20 dB of EVM.

## Test case 1A

### Simulation parameter settings

In this scenario, wide band case with 9Mhz bandwidth is considered. Both the transmitter and the receiver have single antenna.  The summary of the simulation setup is summarized in Table 2.

Table 2 Simulation settings for case 1A

| FFT Length | 4096 |
| --- | --- |
| Tone spacing | 15kHz |
| Usable Tones | 50*12 = 600 tones = 9Mhz |
| CP/ZP length | 288 [samples] (=7% symbol length) |
| MCS | 16 QAM with 1/2-code, 64 QAM with 1/2-code |
| WOLA length | 200 [samples] (=5% symbol length) |
| FOFDM Tx Filter length | 2048 [samples] (= 50% symbol) |
| UFMC Tx Filter length | 288 [samples] (= 7% symbol length) |

### Simulation results

For a fair comparison of three waveforms with different tail lengths, we introduce an effective throughput metric defined as

Effective Thoughput= N×TsN×Ts+Tt×(Thoughput)

N: Number of Symbols per Subframe

Ts: Symbol duration including CP and ZP

Tt: Wola Length or Filter Tails

In this simulation, each subframe duration is set to be 1 ms which contains 14 OFDM symbols. The parameters for the effective throughput computation is given as follows.

Table 3 Parameters for the effective throughput computation

|  | N | Ts | Tt |
| --- | --- | --- | --- |
| WOLA | 14 | 71 us | 3.2 us |
| FOFDM | 14 | 71 us | 33 us |
| UFMC | 14 | 71 us | 0 us |

Figure 7 and Figure 8 summarizes the effective throughput for the three waveforms  – WOLA, F-OFDM, UFMC – discussed earlier.

Figure 7 Effective Throughput of Case 1 (a) when MCS is 16 QAM and 1/2-Code (Left: EPA, Right: ETU)

Figure 8 Effective Throughput of Case 1 (a) when MCS is 64 QAM and 1/2-Code (Left: EPA, Right: ETU)

Observation 6: For the wide band case, all three waveforms show a similar performance across different channel models and MCSs. TxWOLA performance is slight better than the others in some combinations of channel models and MCSs. F-OFDM has the smallest maximum effective throughput due to its long filter tail length.

## Test Case 1B

### Simulation parameter settings

In this scenario, narrow band case with 1.08Mhz bandwidth is considered. Both the transmitter and the receiver have single antenna.  The summary of the simulation setup is summarized in Table 3.

Table 3 Simulation settings for case 1B

| FFT Length | 4096 |
| --- | --- |
| Tone spacing | 15kHz |
| Usable Tones | 6*12 = 72 tones = 1.08Mhz |
| CP/ZP length | 288 [samples] (=7% symbol length) |
| MCS | 16 QAM with 1/2-code, 64 QAM with 1/2-code |
| WOLA length | 200 [samples] (=5% symbol length) |
| FOFDM Tx Filter length | 2048 [samples] (= 50% symbol) |
| UFMC Tx Filter length | 288 [samples] (= 7% symbol length) |

### Simulation results

As we did in Section 3.2.2, the effective throughput is used for a fair comparison across the three waveforms. The parameters for the effective throughput remains the same as Section 3.2.2.

Figure 9 and Figure 10 summarizes the effective throughput for the three waveforms – WOLA, F-OFDM and UFMC.

Figure 9 Effective Throughput of Case 1 (b) when MCS is 16 QAM and 1/2-Code (Left: EPA, Right: ETU)

Figure 10 Effective Throughput of Case 1 (b) when MCS is 64 QAM and 1/2-Code (Left: EPA, Right: ETU)

Observation 7: For the narrow band case, all three waveforms show a similar performance across different channel models and MCSs. WOLA or UFMC shows the best performance depending on channel models and MCSs. F-OFDM has the smallest maximum effective throughput due to its long filter tail length.

# Conclusions

Three different OFDM-based waveform candidates – Tx-WOLA, F-OFDM, UFMC – are studied as OFDM-based waveform candidates for NR. Among Tx-WOLA, F-OFDM and UFMC waveforms, the Tx-WOLA waveform has the smallest computational complexity, and the best ACLR and EVM. Furthermore, for single numerology case, the Tx-WOLA waveform shows the best or comparative efficient throughput for various combinations of CINR, bandwidth, channel models and MCSs.

Observation 1: Compared to CP-OFDM, the additional computation required for Tx-WOLA is (WOLA length) per symbol.

Observation 2: If tx filtering is applied in time domain, F-OFDM requires (filter length)×(symbol length) number of more computations per symbol compared to CP-OFDM. If tx filtering is applied in frequency domain, the computation complexity of F-OFDM is doubled compared to that of CP-OFDM.

Observation 3: The naive implementation of UFMC transmitter has (Number of RBs) times more computational complexity compared to CP-OFDM.

Observation 4: The low complexity UFMC transmitter is at least twice more computationally complicated to CP-OFDM. Additionally, (Number of RBs) parallel small-size IFFT and FFT operations are required.

Observation 5: WOLA achieves the best ACLR and EVM performance, while Filtered-OFDM has the worst ACLR and EVM performance for all clipping thresholds. 6 dB Clipping threshold guarantees at least -30 dBc of ACLR and -20 dB of EVM.

Observation 6: For the wide band case, all three waveforms show a similar performance across different channel models and MCSs. With WOLA, the performance is slight better than the others in some combinations of channel models and MCSs. F-OFDM has the smallest maximum effective throughput due to its long filter tail length.

Observation 7: For the narrow band case, all three waveforms show a similar performance across different channel models and MCSs. WOLA or UFMC shows the best performance depending on channel models and MCSs. F-OFDM has the smallest maximum effective throughput due to its long filter tail length.

Based on the above observations, we have the following proposal:

Proposal 1: Based on the simulated scenarios from test case 1A/1B, Tx-WOLA has better balance between performance and transmit/receiver complexities than other candidates.

# References

- 3GPP R1-162199 “Waveform candidates”.

- 3GPP R1-163558 “Way forward on assumptions for waveform evaluation”.

- 3GPP R1-16xxxx “Way forward on evaluation assumptions for NR waveform [v10]”.

- Abdoli, Javad, Ming Jia, and Jianglei Ma. "Filtered OFDM: A new waveform for future wireless systems." Signal Processing Advances in Wireless Communications (SPAWC), 2015 IEEE 16th International Workshop on. IEEE, 2015.

- Vakilian, Vida, et al. "Universal-filtered multi-carrier technique for wireless systems beyond LTE." Globecom Workshops (GC Wkshps), 2013 IEEE. IEEE, 2013.

- Wild, Thorsten, and Frank Schaich. "A Reduced Complexity Transmitter for UF-OFDM." Vehicular Technology Conference (VTC Spring), 2015 IEEE 81st. IEEE, 2015.