3GPP TSG RAN WG1 Meeting #85	R1-164861

Nanjing, China, May 23-27, 2016

Agenda Item:	6.2.9.1.2

Source:	Huawei, HiSilicon

Title:	Evaluation results for PC5-based V2X

Document for:	Discussion and decision

# Introduction

In this contribution, we provide simulation results for V2V/I/P on Uu for the evaluation scenarios of TR36.885, based on the results shown in [1].

# Simulation results for PC5-based V2I

System performance of UE-type RSU is evaluated in this section. Detailed simulation settings are listed in Table 1 in Appendix. Figure 1 shows the performance of UE-type RSU. It shows that the performance of I2V is better than that of V2I in different scenarios. In the notation “X2Y”, “Y” is the receiver. For V2I, when two vehicles approach an intersection from perpendicular streets and use the same resource, their signals will interfere with each other at receiver ‘I’, therefore degrading performance. For I2V, if another ‘V’ in perpendicular street and ‘I’ at the intersection use the same resource, their signals will interfere with each other at receiver ‘V’. Since ‘I’ is at intersection, the channel from ‘I’ to receiver ‘V’ is LOS while the other transmitter ‘V’ to the receiver ‘V’ is NLOS for V2V. Thus, the interference of ‘V’ to ‘I’ is very small, which explains the reason why the performance of I2V is much better than V2I. These phenomena are illustrated graphically in Figure 1.

We note also from simulation results that the performance of both V2I and I2V are better than V2V in the same scenario. That is because the path between ‘V’ and ‘I’ is almost always LOS, while the path between ‘V’ and ‘V’ can be both LOS and NLOS depending on vehicle positions. Furthermore, it is worth noting that the enhancement schemes for V2V, such as randomization, variable transmission numbers, etc., can also be applied for V2I and I2V to further enhance their performances, respectively.

Figure 1. Performance of UE-type RSU

# Simulation results for PC5-based V2P

System performances of V2P and P2V in urban scenarios are evaluated in this section. Detailed simulation settings are listed in Table 2. It is important to note that in all the simulations, a background 10Hz V2V traffic is assumed to be always present. Figure 2(a) and Figure 2 (b) show four performance curves for urban 60km/h and urban 15km/h, respectively. The four curves are:

- V2V: This curve shows the PRR of “V” when receiving messages from “V” (without pedestrian’s presence), mainly plotted for comparison purpose.

- V2P (P not Tx): This curve shows the PRR of “P” when receiving messages from “V”. Note that “P” does not transmit messages in this case.

- V2P (P also Tx): This curve shows the PRR of “P” when receiving messages from “V”. Note that in addition to receiving messages from “V”, “P” also transmits messages in this case.

- P2V: This curve shows the PRR of “V” when receiving messages from “P”. Note that “V” also transmits messages in this case.

Several observations can be drawn from Figure 2 (a). In regards to V2P performances, letting additional “P” transmit only degrades V2P performance marginally, primarily due to lower “P” density compared with that of “V”. In addition, the performance curves of V2P (whether “P” transmits) are quite close to that of V2V. This is because the only difference from V2V evaluation is that there will be additional receivers (“P”) receiving the same traffic as the V2V evaluation from vehicles. Finally, we note that the performance of P2V is worse than that of V2P, partly due to lower antenna gain for pedestrians (0 dBi for “P” and 3 dBi for “V”) and thus lower received signal power at the vehicle side, and partly due to different “P” and “V” distributions and thus varied interfering environments. Similar trends can be observed for the urban 15km/h case in Figure 2 (b).

| (a) | (b) |
| --- | --- |

Figure 2. Performance V2P/P2V mode 2 for urban scenarios

# Reference

R1-160737, "Consideration on PC5 support for V2P," Huawei, HiSilicon, Malta, Feb 15th-19th, 2016.

# Appendix

Table 1. Parameter setting for simulation of PC5-based V2I

| Parameter | Assumptions |
| --- | --- |
| RSU type | UE-type RSU |
| Carrier frequency | 6GHz |
| Bandwidth | 10MHz |
| Transmission mode | Mode 2 |
| Traffic model | Traffic model 1 |
| RSU height | 5m |
| Scenarios | Urban 60km/h; Urban 15km/h 10Hz; Urban 15km/h 2Hz |

Table 2. Parameter setting for simulation of PC5-based V2P

| Parameter | Assumptions |
| --- | --- |
| Carrier frequency | 6 GHz |
| Bandwidth | 10 MHz |
| Inter-pedestrian distance | 36.344m (14 road grids) |
| Transmission mode | Mode 2 |
| V2P Traffic model | Same as V2V |
| P2V Traffic model | 300 Bytes @ 1Hz |
| Antenna height of Pedestrian | 1.5 m |
| Antenna gain of Pedestrian | 0 dBi |
| Scenarios | Urban 60 km/h; Urban 15 km/h; |

- Note: A background 10Hz V2V traffic is assumed to be always present.