3GPP TSG RAN WG1 Meeting #85	R1-164997

Nanjing, China, 23rd - 27th May 2016

Source:	Sharp

Title:	On transmission power ratios for MUST

Agenda Item:	6.2.4.1

Document for:	Discussion and Decision

Introduction

At the RAN1#84bis meeting, the following agreement was made for power ratio for MUST [1].

Agreement: - For Case 1 & 2, up to two co-scheduled UEs per spatial layer are supported - For Case 1 & 2, MUST category 2 with one or more transmission power ratios for co-scheduled MUST UEs in each constellation combination is supported - One or more transmission power ratios for each constellation combination are supported - FFS: The number of multiple power ratios is down-selected from 1 to 8 - The superposed constellation corresponding to one of transmission power ratios in each constellation combination is a legacy constellation - For (MOD_N, MOD_F) = (QPSK, QPSK), 16QAM legacy constellation - For (MOD_N, MOD_F) = (16QAM, QPSK), 64QAM legacy constellation - For (MOD_N, MOD_F) = (64QAM, QPSK), 256QAM legacy constellation - If 2 or more power ratios are supported, the other multiple transmission power ratios for a MUST-far UE in each constellation combination can be selected from the following value ranges: - For (MOD_N, MOD_F) = (QPSK, QPSK), the power ratio range as a starting point is [0.6, 0.95] - For (MOD_N, MOD_F) = (16QAM, QPSK), the power ratio range as a starting point is [0.6, 0.95] - For (MOD_N, MOD_F) = (64QAM, QPSK), the power ratio range as a starting point is [0.6, 0.95] - FFS the impact if 16QAM for MUST-Far UE is supported - For further down-selection on the set of transmission power ratios, companies are encouraged to provide the scheduling PDF of power ratios and the corresponding performance for different sets of power ratios

In this contribution, we show our evaluation results of PDF of the selected power ratio in each number of transmission power ratios by system level evaluation, and share our views on the design of the transmission power ratio.

Discussions

Considerations on down-selection of power ratios

At the RAN1#84bis meeting, it was agreed that legacy constellations are supported. Table 1 shows power ratios in which the superpose constellations are the legacy constellations [2]. However, only 4 legacy constellations can be used for MUST since the range of far UE is limited to [0.6, 0.95] at the RAN1#84bis meeting.

Table 1: power ratios for legacy constellation

| Legacy constellation | Far UE | Near UE |
| --- | --- | --- |
| 16QAM | 0.800 (QPSK) | 0.200 (QPSK) |
| 64QAM | 0.762 (QPSK) | 0.238 (16QAM) |
| 256QAM | 0.753 (QPSK) | 0.247 (64QAM) |
| 64QAM | 0.952 (16QAM) | 0.048 (QPSK) |
| 256QAM | 0.941 (16QAM) | 0.059 (16QAM) |
| 256QAM | 0.988 (64QAM) | 0.012 (QPSK) |

In addition, the maximum number of transmission power ratios is 8 and the range of far UE is limited to [0.6, 0.95]. This means 7 other power ratios other than the power ratios for legacy constellation can be supported with the agreed range at maximum.

Evaluation results of scheduling PDF

We evaluate the scheduling PDF by system level evaluation. In our evaluation, we assume that non-full buffer traffic model with packet size of 100 kB, packet arrival rate  = 10, maximum transfer time = 1.6 sec. We apply MUST category 2 as MUST scheme and R-ML as a near UE receiver. Moreover, the same precoder for co-scheduled UEs and wideband scheduling are assumed, and modulation orders for far UEs are restricted to QPSK in our evaluation. Other evaluation assumptions are listed in Annex.

In addition, in our evaluation, we use 36 candidates of power ratios with [0.6, 0.95] with 0.01 step for down selection of power ratios to 8 candidates at maximum.

| (a) QPSK (Far UE), QPSK (Near UE) | (b) QPSK (Far UE), 16QAM (Near UE) |
| --- | --- |
| (c) QPSK (Far UE), 64QAM (Near UE) |  |
| Fig.1: PDF of selected power ratio |  |

The scheduling PDF of power ratios are shown in Fig.1. Fig.1 (a), (b), and (c) shows each modulation combination of far UE and near UE. Dashed lines mean the power ratio to generate legacy constellation. As we can see these figures, we can observe the power ratio to realize legacy constellation is not the most frequently applied. On the other hand, the PDFs may change depending on transmission conditions, e.g., eNB scheduler, transmission mode, antenna configuration. In other words, optimum power ratios may depend on each cell.

Observation:

- One or more power ratio(s) in addition to the legacy constellation is needed.

- The PDF may depend on transmission conditions.

Regarding the power ratios for each modulation order of MUST near UE, the following observations can be made from Fig. 1:

Observation 2:

- The selected power ratio ranges and its distributions are different depending to the combination of modulation orders of MUST co-scheduled UE.

  - If modulation order of a MUST near UE is QPSK, the power ratios range is [0.6, 0.95], and power ratio of around 0.85 is tend to be selected.

  - If modulation order of a MUST near UE is 16QAM, the power ratios range is [0.65, 0.9], and power ratio of [0.75, 0.85] is tend to be selected.

  - If modulation order of a MUST near UE is 64QAM, the power ratios range is [0.7, 0.85], and power ratio of around 0.8 is tend to be selected.

Based on this consideration, we propose:

Proposal:

- At least one additional power ratio is needed.

- The set of possible power ratios can be different depending on the combination of modulation orders of MUST co-scheduled UEs.

Conclusion

In this contribution, we made the following observations and proposals:

Observation 1:

- One or more power ratio(s) in addition to the legacy constellation is needed.

- The PDF may depend on transmission conditions.

Observation 2:

- The selected power ratio ranges and its distributions are different depending to the combination of modulation orders of MUST co-scheduled UE.

  - If modulation order of a MUST near UE is QPSK, the power ratios range is [0.6, 0.95], and power ratio of around 0.85 is tend to be selected.

  - If modulation order of a MUST near UE is 16QAM, the power ratios range is [0.65, 0.9], and power ratio of [0.75, 0.85] is tend to be selected.

  - If modulation order of a MUST near UE is 64QAM, the power ratios range is [0.7, 0.85], and power ratio of around 0.8 is tend to be selected.

Proposal:

- At least one additional power ratio is needed.

- The set of possible power ratios can be different depending on the combination of modulation orders of MUST co-scheduled UEs.

References

- Chairman’s Notes, RAN1 #84bis, Busan, Korea, Apr. 2016.

- R1-151425, Qualcomm, “Multiuser superposition schemes,” Apr. 2015.

Annex

Table 2 shows the evaluation assumptions.

Table 2: system level simulation assumptions

| Parameters | Assumptions / Values |
| --- | --- |
| Scenario | MUST Scenario 1 |
| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites (ISD = 500 m) |
| Traffic model | FTP model 1, 100KB |
| System bandwidth per carrier | 10 MHz |
| Carrier frequency | 2.0 GHz |
| Total BS TX power (Ptotal per carrier) | 46 dBm |
| Distance-dependent path loss | ITU UMa |
| Penetration loss | For outdoor UEs:0dB For indoor UEs: 20dB+0.5din (din : independent uniform random value between [ 0, min(25,d) ] for each link) |
| Antenna pattern | 3D (referring to TR36.819) |
| Antenna Height: | 25 m |
| UE antenna Height | 1.5 m |
| Antenna gain + connector loss | 17 dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna configuration | BS: 2 Tx, cross-polarized (+45 degree / -45 degree) UE: 2 Rx, cross-polarized (0 degree / 90 degree) |
| UE dropping | 20% UEs are outdoor and 80% UEs are indoor. |
| minimum distance from macro-cell to UEs | 35 m |
| UE receiver | For inter-cell and inter-spatial-layer interference - MMSE-IRC - Non-ideal covariance matrix estimation modelled by Wishart distribution (degree of freedom: 16) |
| Transmission mode | TM4 (Fixed to rank-1) |
| UE noise figure | 9 dB |
| UE speed | 3 km/h |
| Handover margin | 3 dB |
| Overhead | 3 symbols for DL CCHs |
| Receiver impairment modelling for demodulation | Non-ideal CRS based channel estimation |
| CSI estimation | Non-ideal CRS based CSI calculation |
| Feedback | CSI reporting mode 1-1 Feedback periodicity: 5ms Feedback delay: 5ms |
| EVM | Tx: 8 %, Rx: 4 % |