3GPP TSG-RAN WG1 Meeting #85	                                   R1-165138

Nanjing, China, 23rd - 27th May 2016

Agenda item:		6.2.4.3

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	System-level Evaluation of Case 1 and 3 MUST

Document for:		Discussion and Decision

# 1	Introduction

In the previous 3GPP TSG RAN Meeting #71 a MUST WID [1] has been approved. According to the WID, a MUST UE receiver is assumed to be capable to cancel or suppress intra-cell interference between co-scheduled MUST users for the following cases:

CASE-1: Superposed PDSCHs are transmitted using the same transmission scheme and the same spatial precoding vector

CASE-2: Superposed PDSCHs are transmitted using the same transmit diversity scheme.

CASE-3: Superposed PDSCHs are transmitted using the same transmission scheme, but their spatial precoding vectors are different.

In this contribution we investigate the system level performance of CASE-1 with Category 2 MUST, a scheme based on Gray mapping and label-bit assignment. Furthermore, we investigate the CASE-3 with orthogonal and semi-orthogonal PMI pairing and with 2Tx and 4Tx antennas using two-layer RML for reception. Together with the results we provide corresponding L2S methodology.

# 2	L2S for single-layer ML

The L2S for MUST CASE-1 system simulations has not been specified by 3GPP. Instead, it has been agreed in study item that each company provides used L2S methodology together with the presented system results. In this section we describe our used methodology for L2S with single-layer RML, used as MUST receiver, when “same beam” serves both superposed users. This L2S modelling is similar to the solution presented in [2].

The following processing is assumed. The receiver is utilizing linear MMSE-IRC receiver and obtains signal-to-interference-and-noise ratio (SINR) estimate at each sub-carrier. The SINRs are mapped to BLER within two steps:

In the first step, we create with Monte-Carlo simulations a SNR to mutual information mappings for each bit in the constellation.  The curves are obtained using the formula from [3] modified to bit level:

Iy,x,mML=log2(Xb)-1Xb∈Xbx'∈Xxm=bEnlog2x''∈Xexp-1σn2n+Hx'-x''2x''∈Xx''(m) =x'(m)exp-1σn2n+Hx'-x''2,

where m is the position of a bit we are interested in, assuming Gray bit to symbol mapping. There are m=log2X bits required to describe constellation X with cardinalityX.  Xb=0,1 denotes a bit set and σn2 is the noise+interference variance. Mutual information (MI) of bits can be summed together.

Figure 1 shows SNR to bit-pair MI mapping curves for LTE constellations.

Figure 1 SNR to bit-pair MI mapping curves

In the second step, we perform “circle conversion” to obtain MI and effective SINR, which is then mapped to BLER according to legacy EESM or MIESM tables. The conversion is illustrated in the example from Figure 2, where we depict three MI per bit-pair curves: 1)MI for QPSK, 2)MI for first two bits of 16QAM constellation and 3) MI for the last two bits of 16 QAM constellation.

The “circle conversion” reading operates as follows: the Near UE is allocated with bits 3 and 4 (less 2 significant bits) out of 16QAM LTE uniform constellation. After obtaining effective SINR (10→3 dB reading in the example), the near UE is treated as regular QPSK UE with MCS N. The obtained effective SINRs at each sub-carrier are mapped to BLER according to EESM tables in our setup. This procedure allows utilizing any bit split combination, for given constellation,   by mapping MI per bit-pair into effective SINR, and finally into BLER.

Figure 2 Conversion of SINR to effective SINR

# 3 	L2S for two-layer ML

Similarly to L2S for CASE-1 MUST, the L2S for CASE-3 MUST system simulations has not been specified by 3GPP. Therefore, we will describe herein, the L2S we used for evaluation of CASE-3.

We start from definition of mutual information between transmitted symbol xown from a vector of transmitted symbols x=xownxcoT and received signal y.

Iy,xown,mML=log2(Xb)-1Xb∈Xbx'∈Xxownm=bEnlog2x''∈Xexp-1σn2n+Hx'-x''2x''∈Xxown''(m) =xown'(m)exp-1σn2n+Hx'-x''2,

where X=Xown×Xco is a joint constellation of symbols xown and xco,  m is the position of a bit within symbol xown assuming Gray bit to symbol mapping of Xown and Xcoconstellations.  Xb=0,1 is a bit set and σn2 is the noise-plus-interference variance, which is assumed to be white. Mutual information (MI) of bits m in constellation Xown can be summed together.

Unlike with single layer L2S, we deal with two layers, each transmitted over different equivalent channel H=hownhco. In this contribution we model MI as function of 1) Own layer SINR γown and modulation order 2) Co layer SINR γco and modulation order 2) correlation between channels ρ=hcoHhown|hown||hco|. By Monte-Carlo simulations we obtain  Iγown,γco,ρ for modulation combinations summarized in Table 1.

| Case | UE1 MOD | UE2 MOD |
| --- | --- | --- |
| TL1 | 2 | 2 |
| TL2 | 2 | 4 |
| TL3 | 4 | 2 |
| TL4 | 4 | 4 |

Table 1 Two-layer ML cases

Given mutual information and MCS N, we obtain BLER estimate similarly as with single-layer RML. In addition, we take into account channel estimation error, namely self-interference as well as interference from the co-layer due to erroneous channel estimate.

# 4	System performance results

This section presents the results obtained by LTE system-level simulator. We perform wideband scheduling and we restrict feedback to rank-1 only. The RML L2S for both single-layer and two-layer operation has been described in previous two sections. We simulate the full-buffer traffic with 10UEs per sector and Gray encoding is enforced to all super-constellation transmitted on a single layer, no overlapping constellations have been simulated. No Gray encoding is enforced between constellations of different layers. Detailed simulation assumptions can be found in Appendix.

The goal of these simulations is to compare performance of CASE-1 and CASE-3 MUST. With CASE-1 MUST we have simulated 8 constellation-cases summarized in Table 2. Cases 2, 6 and 8 are employing uniform constellations, the rest are non-uniform. With CASE-3 MUST we employ joint constellations according to Table 1. In CASE-3, we assume 0.2 and 0.5 power splits.

Table 2 Simulated super constellations

| Case | Near UE power | Super-constellation | Far-MOD | Near-MOD |
| --- | --- | --- | --- | --- |
| 1 | 0.10 | Non-uniform 16QAM | 2 | 2 |
| 2 | 0.20 | LTE 16QAM | 2 | 2 |
| 3 | 0.10 | Non-uniform 64QAM | 2 | 4 |
| 4 | 0.15 | Non-uniform 64QAM | 2 | 4 |
| 5 | 0.20 | Non-uniform 64QAM | 2 | 4 |
| 6 | 0.238 | LTE 64QAM | 2 | 4 |
| 7 | 0.15 | Non-uniform 256QAM | 2 | 6 |
| 8 | 0.247 | LTE 256QAM | 2 | 6 |

The results are summarized in Table 3 and Table 4. It can be observed that CASE-3 MUST in TM4 delivers 2.5 more average user TP compared to CASE-1 MUST with 2Tx, and almost 5 times more with 4Tx. This is due to limited applicability of CASE-1 MUST with higher sized codebooks. At the same time, it can be observed that CASE-3 MUST is not a cell-edge technique, and gains at the cell-edge are smaller compared to CASE-1 MUST.

| Case | User Throughput (bps) | Baseline |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MUST Case 3 | Gain | MUST Case 1 | Gain |
| 2Tx | Cell average | 1.272E+6 | 1.590E+6 | 25.0% | 1.392E+6 | 9.4% |
|  | Cell edge | 2.396E+5 | 2.477E+5 | 3.4% | 2.660E+5 | 11.0% |
|  | Note: rank1 only wideband feedback |  |  |  |  |  |

Table 3 MUST 2Tx with ML receiver

| Case | User Throughput (bps) | Baseline |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | MUST Case 3 | Gain | MUST Case 1 | Gain |
| 4Tx | Cell average | 1.525E+6 | 1.878E+6 | 23.2% | 1.611E+6 | 5.64% |
|  | Cell edge | 3.053E+5 | 3.198E+5 | 4.75% | 3.376E+5 | 10.6% |
|  | Note: rank1 only wideband feedback |  |  |  |  |  |

Table 4 MUST 4Tx with ML receiver

Observation 1: CASE-3 delivers 2.5x respective 5x more average TP compared to CASE-1 with 2Tx resp. 4Tx antennas.

Observation 2: CASE-1 delivers 3x respective 2x more cell-edge TP compared to CASE 3 with 2Tx resp. 4Tx antennas.

Observation 3: CASE-1 together with CASE-3 MUST can significantly improve both average as well as cell-edge user TP.

Proposal: Proceed with standardization of MUST Case 3.

# 5	Conclusions

In this contribution we have been presenting system-level results of MUST performance for CASE-1 and CASE-3. The following observations and proposals can be summarized:

Observation 1: CASE-3 delivers 2.5x respective 5x more average TP compared to CASE-1 with 2Tx resp. 4Tx antennas.

Observation 2: CASE-1 delivers 3x respective 2x more cell-edge TP compared to CASE 3 with 2Tx resp. 4Tx antennas.

Observation 3: CASE-1 together with CASE-3 MUST can significantly improve both average as well as cell-edge user TP.

Proposal: Proceed with standardization of MUST Case 3.

# References

- RP-160680, “DL Multiuser Superposition Transmission for LTE”, Mediatek, CMCC,  RAN #71, Gothenburg, Sweden, 7 - 10 March 2016

- R1-154458, “Link-abstraction Method for ML Receiver in MUST”, Mediatek.

- Eckhard Ohlmer, Udo Wachsmann, Gerhard Fettweis “Mutual Information of MIMO Transmission over Correlated Channels with Finite Symbol Alphabets and Link Adaptation”, Globecom 2010

# Appendix

Table 5 Simulation Assumptions

| Layout | Hexagonal grid, 3 sectors per site, 19 macro sites (ISD = 500 m) |
| --- | --- |
| System bandwidth per carrier | 10 MHz |
| Carrier frequency | 2.0 GHz |
| Total BS TX power (Ptotal per carrier) | 46 dBm |
| Distance-dependent path loss | ITU UMa |
| Penetration loss | For outdoor UEs:0dB For indoor UEs: 20dB+0.5din (din : independent uniform random value between [ 0, min(25,d) ] for each link) |
| Shadowing | ITU Uma |
| Antenna pattern | 3D (referring to TR36.819) |
| Antenna Height: | 25 m |
| UE antenna Height | 1.5 m |
| Antenna gain + connector loss | 17 dBi |
| Antenna gain of UE | 0 dBi |
| Fast fading channel between eNB and UE | ITU UMa |
| Antenna configuration | BS: 2Tx (0.5 lambda), cross-polarized UE: 2Rx (0.5 lambda), cross-polarized |
| Number of UEs per cell | 10 |
| UE dropping | 20% UEs are outdoor and 80% UEs are indoor. |
| Minimum distance from macro-cell to UEs | 35 m |
| Traffic model | Full buffer |
| UE receiver | MMSE-diagonal covariance knowledge + RML per layer |
| Transmission mode | 2(4)x2 TM4 (rank1 only) |
| UE noise figure | 9 dB |
| UE speed | 3 km/h |
| Cell selection criteria | RSRP |
| Handover margin | 3 dB |
| Scheduling algorithm | Proportional fairness maximization |
| HARQ | Redundancy Version |
| Feedback | WB rank1 only |
| CQI quantization | Yes |
| Codebook | 2Tx/4Tx LTE Rel. 8 |
| Power ratio sets | According to super-constellation |
| OLLA | Yes |
| Number of superposed signals in superposition transmission | 2 |
| Channel Estimation | Realistic |
| EVM | Tx/Rx 8/4% |