3GPP TSG-RAN WG1#85 	R1-165321

Nanjing, China 23rd - 27th May 2016

Source: 	Ericsson

Title:	Design Principles for New UL 256 QAM MCS Entries

Agenda Item:	6.2.8

Document for: 	Discussion and Decision

# Introduction

The Uplink Capacity Enhancements for LTE Work Item [1] specifies the introduction of 256QAM support for the PUSCH. The support of 256QAM shall be specified in a generic manner for both licensed and unlicensed spectrum. Focus of the work should be specific to the support of small cell deployments or deployments with eNB with many receive antennas. We provided our input in [2] and [3] toward the objective of

- Introduce new MCS table and signaling to support 256QAM in UL [RAN1]

The essential features of the design were agreed in RAN1 #84b [4]:

Agreements:

- The minimum TBS index for 16QAM is 11

- The minimum TBS index for 64QAM is 20

- The minimum TBS index for 256QAM is 30

- 3 entries are reserved for retransmissions

- The number of PUSCH MCS entries to support 256QAM is down-selected between

  - Alt 1: 6

  - Alt 2: 5

  - Alt 3: 4

Agreements:

- Reference number of REs available in PUSCH for UL 256QAM TBS design is 144 REs/PRB

- The maximum coding rate for PUSCH with 256QAM should not exceed 0.931

- FFS whether or not to introduce new TBS index(es) for PUSCH with 256QAM

  - Companies are encouraged to submit evaluations to RAN1#85

In this document, we provide further discussion on the design for Rel-14 PUSCH MCS table with 256QAM support.

# MCS Design Principles for UL 256 QAM

## Number of PUSCH MCS entries to support 256QAM

The Rel-8 PUSCH MCS table was designed via the following principle [5]:

The Rel-8 PUSCH MCS table was designed by adjusting the modulation switching points of the Rel-8 PDSCH MCS table and completely reusing the TBS designed for the PDSCH.

The Rel-12 PDSCH MCS table with 256QAM support was designed with the following guidelines:

- The MCS density for lower order modulations is reduced.

- Duplicates of spectral efficiency using different modulation orders are removed.

- Existing TBS rows are used as much as possible and new rows are created to support data rates not reachable by existing TBS.

Extensive efforts were contributed by many companies to reach the consensus design for Rel-12 MCS table for PDSCH with 256QAM support. It is our view that duplicating such efforts should be avoided for the design of the MCS table for Rel-14 PUSCH with 256QAM support.

The approach adopted for Rel-8 PUSCH MCS table design should be followed to the maximum extent possible. No changes should be made to the TBS indices supporting modulation orders lower than 256QAM. As shown in [2], this leaves 5 MCS entries for UL 256QAM support.

Proposal 1

For Rel-14 PUSCH MCS table with 256QAM support, 5 MCS entries are used for UL 256QAM.

Proposal 2

For Rel-14 PUSCH MCS table with 256QAM support, MCS #0-23 are given in the following:

| MCS Index | Modulation Order | TBS Index |  | r x 1024 | SE |
| --- | --- | --- | --- | --- | --- |
| 0 | 2 | 0 |  | 100.0000 | 0.1953 |
| 1 | 2 | 2 |  | 160.8333 | 0.3141 |
| 2 | 2 | 4 |  | 256.6667 | 0.5013 |
| 3 | 2 | 6 |  | 374.1667 | 0.7308 |
| 4 | 2 | 8 |  | 501.6667 | 0.9798 |
| 5 | 2 | 10 |  | 630.0000 | 1.2305 |
| 6 | 4 | 11 |  | 361.6667 | 1.4128 |
| 7 | 4 | 12 |  | 408.3333 | 1.5951 |
| 8 | 4 | 13 |  | 460.8333 | 1.8001 |
| 9 | 4 | 14 |  | 513.3333 | 2.0052 |
| 10 | 4 | 15 |  | 548.3333 | 2.1419 |
| 11 | 4 | 16 |  | 582.5000 | 2.2754 |
| 12 | 4 | 17 |  | 646.2500 | 2.5244 |
| 13 | 4 | 18 |  | 708.7500 | 2.7686 |
| 14 | 4 | 19 |  | 770.0000 | 3.0078 |
| 15 | 6 | 20 |  | 555.0000 | 3.2520 |
| 16 | 6 | 21 |  | 599.1667 | 3.5107 |
| 17 | 6 | 22 |  | 643.3333 | 3.7695 |
| 18 | 6 | 23 |  | 685.0000 | 4.0137 |
| 19 | 6 | 24 |  | 727.5000 | 4.2627 |
| 20 | 6 | 25 |  | 758.3333 | 4.4434 |
| 21 | 6 | 27 |  | 790.0000 | 4.6289 |
| 22 | 6 | 28 |  | 837.7778 | 4.9089 |
| 23 | 6 | 29 |  | 885.5556 | 5.1888 |

## New TBS target code rates for 256QAM UL support

The Rel-12 DL 256QAM MCS table is further designed based on the following principles:

- Existing TBS rows are used as much as possible and new rows are created to support data rates not reachable by existing TBS.

- The highest code rate is set to 948/1024.

These design principles should also be followed for the Rel-14 PUSCH MCS table with 256QAM support.

Proposal 3

The highest code rate for 256QAM UL is set to 948/1024. Existing TBS rows are reused as much as possible.

The existing TBS #30 supports a code rate of r=700.8333/1024 for UL 256QAM MCS #24. Given the target code rate of 948/1024 for MCS #28, preliminary equally-spaced target code rates for MCS #25-27 are given in the following:

| MCS Index | Modulation Order | TBS Index | r x 1024 |
| --- | --- | --- | --- |
| 24 | 8 | 30 | 700.8333 |
| 25 | 8 |  | 762.6250 |
| 26 | 8 |  | 824.4167 |
| 27 | 8 |  | 886.2083 |
| 28 | 8 |  | 948.0000 |

However, the following adjustments should be made to maximize reuse of existing TBS rows:

- TBS #32 supports a code rate of r=763.7500/1024 for UL 256QAM. This code rate is very close to the preliminary target for MCS #25. Therefore, the target code rate for MCS #25 should be set to 763.7500/1024 to reuse TBS #32.

- TBS #33 supports a code rate of r=895.3333/1024 for UL 256QAM. This code rate is close to the preliminary target for MCS #27. Therefore, the target code rate for MCS #27 should be set to 895.3333/1024 to reuse TBS #33.

- Given the new target code rates for MCS #25 and #27, the target code rate of MCS #26 should consequently be adjusted to the middle point of these two MCS entries: r= 829.5417/1024.

Based on the above analysis, the MCS entries for 256QAM support is given in the following proposals.

Proposal 4

For Rel-14 PUSCH MCS table with 256QAM support, two new TBS rows are introduced with the following target code rates for 256QAM:

| TBS Index | Modulation Order | r x 1024 | SE |
| --- | --- | --- | --- |
| 34 | 8 | 829.5417 | 6.4808 |
| 35 | 8 | 948.0000 | 7.4063 |

Proposal 5

For Rel-14 PUSCH MCS table with 256QAM support, MCS #24-28 are given in the following:

| MCS Index | Modulation Order | TBS Index |  | r x 1024 | SE |
| --- | --- | --- | --- | --- | --- |
| 24 | 8 | 30 |  | 700.8333 | 5.4753 |
| 25 | 8 | 32 |  | 763.7500 | 5.9668 |
| 26 | 8 | 34 |  | 829.5417 | 6.4808 |
| 27 | 8 | 33 |  | 895.3333 | 6.9948 |
| 28 | 8 | 35 |  | 948.0000 | 7.4063 |

# Conclusion

Based on extensive analysis presented in this document, we make the following proposals.

Proposal 1

For Rel-14 PUSCH MCS table with 256QAM support, 5 MCS entries are used for UL 256QAM.

Proposal 2

For Rel-14 PUSCH MCS table with 256QAM support, MCS #0-23 are given in the following:

| MCS Index | Modulation Order | TBS Index |  | r x 1024 | SE |
| --- | --- | --- | --- | --- | --- |
| 0 | 2 | 0 |  | 100.0000 | 0.1953 |
| 1 | 2 | 2 |  | 160.8333 | 0.3141 |
| 2 | 2 | 4 |  | 256.6667 | 0.5013 |
| 3 | 2 | 6 |  | 374.1667 | 0.7308 |
| 4 | 2 | 8 |  | 501.6667 | 0.9798 |
| 5 | 2 | 10 |  | 630.0000 | 1.2305 |
| 6 | 4 | 11 |  | 361.6667 | 1.4128 |
| 7 | 4 | 12 |  | 408.3333 | 1.5951 |
| 8 | 4 | 13 |  | 460.8333 | 1.8001 |
| 9 | 4 | 14 |  | 513.3333 | 2.0052 |
| 10 | 4 | 15 |  | 548.3333 | 2.1419 |
| 11 | 4 | 16 |  | 582.5000 | 2.2754 |
| 12 | 4 | 17 |  | 646.2500 | 2.5244 |
| 13 | 4 | 18 |  | 708.7500 | 2.7686 |
| 14 | 4 | 19 |  | 770.0000 | 3.0078 |
| 15 | 6 | 20 |  | 555.0000 | 3.2520 |
| 16 | 6 | 21 |  | 599.1667 | 3.5107 |
| 17 | 6 | 22 |  | 643.3333 | 3.7695 |
| 18 | 6 | 23 |  | 685.0000 | 4.0137 |
| 19 | 6 | 24 |  | 727.5000 | 4.2627 |
| 20 | 6 | 25 |  | 758.3333 | 4.4434 |
| 21 | 6 | 27 |  | 790.0000 | 4.6289 |
| 22 | 6 | 28 |  | 837.7778 | 4.9089 |
| 23 | 6 | 29 |  | 885.5556 | 5.1888 |

Proposal 3

The highest code rate for 256QAM UL is set to 948/1024. Existing TBS rows are reused as much as possible.

Proposal 4

For Rel-14 PUSCH MCS table with 256QAM support, two new TBS rows are introduced with the following target code rates for 256QAM:

| TBS Index | Modulation Order | r x 1024 | SE |
| --- | --- | --- | --- |
| 34 | 8 | 829.5417 | 6.4808 |
| 35 | 8 | 948.0000 | 7.4063 |

Proposal 5

For Rel-14 PUSCH MCS table with 256QAM support, MCS #24-28 are given in the following:

| MCS Index | Modulation Order | TBS Index |  | r x 1024 | SE |
| --- | --- | --- | --- | --- | --- |
| 24 | 8 | 30 |  | 700.8333 | 5.4753 |
| 25 | 8 | 32 |  | 763.7500 | 5.9668 |
| 26 | 8 | 34 |  | 829.5417 | 6.4808 |
| 27 | 8 | 33 |  | 895.3333 | 6.9948 |
| 28 | 8 | 35 |  | 948.0000 | 7.4063 |

# References

- RP-160664 New Work Item on Uplink Capacity Enhancements for LTE enhanced LAA for LTE, 3GPP RAN WG #71;  Ericsson, CMCC

- R1-163345 MCS Table Design for UL 256QAM, Ericsson

- R1-163346 On the Performance of UL 256QAM, Ericsson

- 3GPP RAN1 #84b Chairman’s Notes

- R1-082091 MCS and TBS Tables for PUSCH; Ericsson, Panasonic, Motorola