3GPP TSG-RAN WG1 #85	R1-165520

Nanjing, P.R. China, 23th – 27th May 2016

Source: 	Ericsson

Title:	Updates to System level results

Agenda Item:	6.2.10.3

Document for:	Discussion and Decision

# Introduction

During RAN1#84, RAN1#84bis and RAN1#85 several companies provided system level evaluation results for latency reduction based on shortened TTI. In this TP, the results provided according to the agreed format [1] are added to the latency reduction TR [2].

# Proposal

Incorporate the following in the text proposal in the Latency reduction TR.

---------------------------------- Start of Text Proposal ------------------------------------------------------

# Annex B:
System evaluation results

In this Annex the system evaluation results of sTTI are provided. The results are generated by the simulation assumptions available in Annex A unless otherwise mentioned.

## B.1 Evaluation results with DL traffic and non-ideal core network delay

### B.1.1 Results for FS1

Table B.1.1-1: Results with 100kbit file transfer

| Reported parameters |  |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] |  | 5% | 1.1298 (0) | 1.4383 (+27.3) | 1.6106 (+42.6) |  | 1.7401 (+54) | - | 1.0479 (0) | 1.2253 (+16.9) | 1.2905 (+23.2) | 1.2385 (+18.2) | - | 0.9948 (0) |  | 1.0918 (+9.8) |  | 1.0597 (+6.5) |  | 0.9431 (-5.2) |  | - |
|  |  | 50% | 1.2068 (0) | 1.5804 (+31) | 1.8125 (+50.2) |  | 2.0173 (+67.2) | - | 1.2019 (0) | 1.5513 (+29.1) | 1.7535 (+45.9) | 1.8892 (+57.2) | - | 1.1831 (0) |  | 1.5361 (+29.8) |  | 1.7344 (+46.6) |  | 1.8377 (+55.3) |  | - |
|  |  | 95% | 1.25 (0) | 1.6259 (+30) | 1.8681 (+49.5) |  | 2.0815 (+66.5) | - | 1.238 (0) | 1.622 (+31) | 1.8631 (+50.5) | 2.0695 (+67.2) | - | 1.2248 (0) |  | 1.616 (+32) |  | 1.857 (+51.6) |  | 2.0611 (+68.3) |  | - |
|  |  | Mean | 1.2008 (0) | 1.5618 (+30) | 1.7883 (+48.9) |  | 1.9741 (+64.4) | - | 1.1822 (0) | 1.5083 (+27.6) | 1.6954 (+43.4) | 1.8162 (+53.6) | - | 1.1582 (0) |  | 1.4701 (+26.9) |  | 1.6477 (+42.3) |  | 1.7348 (+49.8) |  | - |
| DL: Delay CDF [s] |  | 5% | 0.0823 (0) | 0.0631 (-23.3) | 0.0549 (-33.3) |  | 0.0491 (-40.3) | - | 0.0829 (0) | 0.0631 (-23.9) | 0.055 (-33.7) | 0.0495 (-40.3) | - | 0.0838 (0) |  | 0.0635 (-24.2) |  | 0.0551 (-34.3) |  | 0.0495 (-40.9) |  | - |
|  |  | 50% | 0.0854 (0) | 0.065 (-23.9) | 0.0566 (-33.7) |  | 0.0508 (-40.5) | - | 0.0858 (0) | 0.0662 (-22.8) | 0.0584 (-31.9) | 0.0544 (-36.6) | - | 0.0871 (0) |  | 0.0669 (-23.2) |  | 0.0591 (-32.2) |  | 0.0559 (-35.8) |  | - |
|  |  | 95% | 0.0895 (0) | 0.0702 (-21.6) | 0.0625 (-30.2) |  | 0.0585 (-34.6) | - | 0.098 (0) | 0.0841 (-14.2) | 0.0803 (-18.1) | 0.086 (-12.2) | - | 0.1029 (0) |  | 0.0955 (-7.2) |  | 0.0985 (-4.3) |  | 0.1165 (+13.2) |  | - |
|  |  | Mean | 0.0861 (0) | 0.0661 (-23.2) | 0.0577 (-33) |  | 0.0525 (-39) | - | 0.0877 (0) | 0.0693 (-21) | 0.0621 (-29.2) | 0.0597 (-31.9) | - | 0.0897 (0) |  | 0.072 (-19.7) |  | 0.0655 (-27) |  | 0.0657 (-26.8) |  | - |
| RU |  |  | 19.4 | 19.8 | 21.3 |  | 25.3 | - | 44 | 52.3 | 57.2 | 63.3 | - | 54.1 |  | 64 |  | 68.9 |  | 77.1 |  | - |
| 𝜆 |  |  | 5 |  |  |  |  |  | 15 |  |  |  |  | 25 |  |  |  |  |  |  |  |  |
| Notes: |  |  | Source 1, R1-162398, 6ms core network delay, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-21.43%, 7OS-23.14%, 3/4OS-26.57%, 2OS-31.71%, Maximum number of UEs that can be scheduled per sTTI: 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | 0.65 | 0.78 20% | 0.84 29% |  | 0.82 26% | 0.18 -72% | 0.59 | 0.66 12% | 0.68 15% | 0.63 7% | 0.04 -93% | 0.51 |  | 0.52 2% |  | 0.46 -10% |  | 0.38 -25% |  | 0.01 -98% |
|  | 50% |  | 0.77 | 0.96 25% | 1.11 44% |  | 1.17 52% | 1.16 51% | 0.74 | 0.93 26% | 1.06 43% | 1.11 50% | 1.03 39% | 0.74 |  | 0.90 22% |  | 1.00 35% |  | 1.03 39% |  | 0.87 18% |
|  | 95% |  | 0.84 | 1.05 25% | 1.19 42% |  | 1.26 50% | 1.32 57% | 0.83 | 1.03 24% | 1.17 41% | 1.24 49% | 1.30 57% | 0.83 |  | 1.03 24% |  | 1.17 41% |  | 1.23 48% |  | 1.29 55% |
|  | Mean |  | 0.76 | 0.95 25% | 1.07 41% |  | 1.12 47% | 1.01 33% | 0.73 | 0.90 23% | 1.01 38% | 1.05 44% | 0.88 21% | 0.71 |  | 0.86 21% |  | 0.94 32% |  | 0.94 32% |  | 0.76 7% |
| DL: Delay CDF [s] | 5% |  | 0.11 | 0.09 -18% | 0.08 -27% |  | 0.08 -31% | 0.07 -34% | 0.11 | 0.09 -18% | 0.08 -27% | 0.08 -31% | 0.08 -34% | 0.11 |  | 0.09 -18% |  | 0.08 -27% |  | 0.08 -30% |  | 0.08 -34% |
|  | 50% |  | 0.13 | 0.10 -21% | 0.09 -31% |  | 0.08 -35% | 0.08 -38% | 0.13 | 0.11 -20% | 0.09 -30% | 0.09 -34% | 0.08 -37% | 0.13 |  | 0.11 -20% |  | 0.10 -29% |  | 0.09 -31% |  | 0.09 -36% |
|  | 95% |  | 0.16 | 0.13 -19% | 0.12 -26% |  | 0.12 -27% | 0.27 67% | 0.18 | 0.15 -16% | 0.14 -20% | 0.15 -18% | 0.33 84% | 0.19 |  | 0.17 -9% |  | 0.17 -9% |  | 0.18 -3% |  | 0.35 82% |
|  | Mean |  | 0.13 | 0.11 -19% | 0.10 -28% |  | 0.09 -31% | 0.13 -3% | 0.14 | 0.11 -18% | 0.10 -27% | 0.10 -29% | 0.14 0% | 0.14 |  | 0.12 -17% |  | 0.11 -23% |  | 0.11 -23% |  | 0.15 3% |
| RU |  |  | 0.20 | 0.22 | 0.23 |  | 0.20 | 0.16 | 0.40 | 0.46 | 0.46 | 0.41 | 0.26 | 0.60 |  | 0.69 |  | 0.71 |  | 0.61 |  | 0.34 |
| 𝜆 |  |  | 3.23 |  |  |  |  |  | 8.00 |  |  |  |  | 27.03 |  |  |  |  |  |  |  |  |
| Notes: |  |  | Source 2, R1-162536, 10ms core network delay, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI), 4 sTTI UsL HARQ timing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  |  | 2OS |  |  | 1OS |  |  | 14 OS |  |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] |  | 5% |  |  |  | 0.866 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.519 |  |  |  |  |  |  |
|  |  | 50% |  |  |  | 1.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.01 |  |  |  |  |  |  |
|  |  | 95% |  |  |  | 1.18 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.17 |  |  |  |  |  |  |
|  |  | Mean |  |  |  | 1.06 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.95 |  |  |  |  |  |  |
| DL: Delay CDF [s] |  | 5% |  |  |  | 0.084 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.085 |  |  |  |  |  |  |
|  |  | 50% |  |  |  | 0.091 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.095 |  |  |  |  |  |  |
|  |  | 95% |  |  |  | 0.119 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.164 |  |  |  |  |  |  |
|  |  | Mean |  |  |  | 0.096 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.107 |  |  |  |  |  |  |
| RU |  |  |  |  |  | 23.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 72.72 |  |  |  |  |  |  |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  | Source 2, R1-164912, 10ms core network delay, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI), 4 sTTI DL HARQ timing and 7 sTTI TCP ACK latency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | 1.30 | 1.86 | 2.22 |  | 2.09 | - | 1.31 | 1.82 | 1.88 | 1.60 | - |  | - |  | - |  | - |  | - | - |
|  | 50% |  | 1.31 | 1.88 | 2.37 |  | 2.71 | - | 1.42 | 1.95 | 2.37 | 2.68 | - |  | - |  | - |  | - |  | - | - |
|  | 95% |  | 1.51 | 2.04 | 2.46 |  | 2.79 | - | 1.52 | 2.03 | 2.47 | 2.73 | - |  | - |  | - |  | - |  | - | - |
|  | Mean |  | 1.35 | 1.91 | 2.36 |  | 2.62 | - | 1.41 | 1.94 | 2.31 | 2.51 | - |  | - |  | - |  | - |  | - | - |
| DL: Delay CDF [s] | 5% |  | 0.076 | 0.053 | 0.042 |  | 0.037 | - | 0.076 | 0.053 | 0.042 | 0.037 | - |  | - |  | - |  | - |  | - | - |
|  | 50% |  | 0.076 | 0.053 | 0.042 |  | 0.037 | - | 0.076 | 0.053 | 0.043 | 0.037 | - |  | - |  | - |  | - |  | - | - |
|  | 95% |  | 0.077 | 0.054 | 0.045 |  | 0.048 | - | 0.078 | 0.056 | 0.054 | 0.062 | - |  | - |  | - |  | - |  | - | - |
|  | Mean |  | 0.076 | 0.053 | 0.043 |  | 0.039 | - | 0.077 | 0.054 | 0.045 | 0.043 | - |  | - |  | - |  | - |  | - | - |
| RU |  |  | 0.19 | 0.12 | 0.1 |  | 0.15 |  | 0.43 | 0.33 | 0.32 | 0.36 | - |  | - |  | - |  | - |  | - | - |
| 𝜆 |  |  | 6.2 |  |  |  |  |  | 95 |  |  |  |  |  | - |  |  |  |  |  |  |  |
| Notes: |  |  | Source 4, R1-162503, CN delay = 10ms, CQI period = 5TTI, CQI delay = 6TTI FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | 0.85 | 1.16 | 1.34 |  | 1.43 | 1.47 | 0.7 | 1.02 | 1.18 | 1.22 | 1.29 |  | 0.43 |  | 0.78 |  | 0.87 |  | 0.87 | 0.83 |
|  | 50% |  | 1.00 | 1.38 | 1.65 |  | 1.80 | 1.92 | 0.9 | 1.28 | 1.55 | 1.7 | 1.82 |  | 0.73 |  | 1.13 |  | 1.4 |  | 1.55 | 1.68 |
|  | 95% |  | 1.10 | 1.49 | 1.78 |  | 1.95 | 2.10 | 1.04 | 1.43 | 1.73 | 1.89 | 2.04 |  | 0.94 |  | 1.34 |  | 1.63 |  | 1.81 | 1.97 |
|  | Mean |  | 0.99 | 1.36 | 1.62 |  | 1.76 | 1.88 | 0.89 | 1.26 | 1.51 | 1.64 | 1.76 |  | 0.71 |  | 1.1 |  | 1.35 |  | 1.47 | 1.58 |
| DL: Delay CDF [s] | 5% |  | 0.091 | 0.067 | 0.056 |  | 0.051 | 0.048 | 0.099 | 0.071 | 0.058 | 0.053 | 0.049 |  | 0.113 |  | 0.077 |  | 0.063 |  | 0.057 | 0.052 |
|  | 50% |  | 0.102 | 0.074 | 0.062 |  | 0.056 | 0.053 | 0.122 | 0.083 | 0.068 | 0.061 | 0.057 |  | 0.165 |  | 0.101 |  | 0.079 |  | 0.071 | 0.065 |
|  | 95% |  | 0.130 | 0.090 | 0.078 |  | 0.073 | 0.070 | 0.171 | 0.112 | 0.095 | 0.090 | 0.087 |  | 0.323 |  | 0.165 |  | 0.145 |  | 0.149 | 0.157 |
|  | Mean |  | 0.105 | 0.076 | 0.064 |  | 0.059 | 0.055 | 0.127 | 0.086 | 0.071 | 0.066 | 0.062 |  | 0.185 |  | 0.108 |  | 0.090 |  | 0.084 | 0.080 |
| RU |  |  | 0.2 | 0.17 | 0.18 |  | 0.18 | 0.18 | 0.4 | 0.34 | 0.35 | 0.35 | 0.29 |  | 0.6 |  | 0.5 |  | 0.53 |  | 0.53 | 0.42 |
| 𝜆 |  |  | 1.9 |  |  |  |  |  | 3.2 |  |  |  |  |  | 4.3 |  |  |  |  |  |  |  |
| Notes: |  |  | Source 6, R1-162703, CN delay = 6ms, 8 TTIs HARQ RTT, 13-TTI UL access delay, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.3%, 7OS-28.0%, 3/4OS-35.9%, 2OS-38.3%, 1OS-38.3%, Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-4UEs, 3/4OS-3UEs, 2OS-2UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | 0.85 | 1.10 | 1.18 |  | 1.20 | 1.17 | 0.7 | 0.96 | 1.02 | 1.03 | 0.95 |  | 0.43 |  | 0.71 |  | 0.74 |  | 0.67 | 0.61 |
|  | 50% |  | 1.00 | 1.33 | 1.51 |  | 1.57 | 1.56 | 0.9 | 1.22 | 1.41 | 1.46 | 1.45 |  | 0.73 |  | 1.08 |  | 1.25 |  | 1.3 | 1.31 |
|  | 95% |  | 1.10 | 1.46 | 1.70 |  | 1.81 | 1.85 | 1.04 | 1.39 | 1.61 | 1.71 | 1.75 |  | 0.94 |  | 1.3 |  | 1.52 |  | 1.6 | 1.66 |
|  | Mean |  | 0.99 | 1.31 | 1.48 |  | 1.54 | 1.54 | 0.89 | 1.21 | 1.37 | 1.42 | 1.41 |  | 0.71 |  | 1.05 |  | 1.21 |  | 1.24 | 1.24 |
| DL: Delay CDF [s] | 5% |  | 0.091 | 0.069 | 0.059 |  | 0.056 | 0.055 | 0.099 | 0.074 | 0.064 | 0.060 | 0.059 |  | 0.113 |  | 0.081 |  | 0.070 |  | 0.066 | 0.064 |
|  | 50% |  | 0.102 | 0.077 | 0.068 |  | 0.065 | 0.066 | 0.122 | 0.088 | 0.076 | 0.073 | 0.074 |  | 0.165 |  | 0.106 |  | 0.091 |  | 0.088 | 0.086 |
|  | 95% |  | 0.130 | 0.097 | 0.089 |  | 0.088 | 0.093 | 0.171 | 0.118 | 0.112 | 0.109 | 0.122 |  | 0.323 |  | 0.183 |  | 0.176 |  | 0.204 | 0.220 |
|  | Mean |  | 0.105 | 0.079 | 0.071 |  | 0.068 | 0.069 | 0.127 | 0.091 | 0.081 | 0.078 | 0.080 |  | 0.185 |  | 0.116 |  | 0.104 |  | 0.110 | 0.110 |
| RU |  |  | 0.2 | 0.17 | 0.18 |  | 0.19 | 0.19 | 0.4 | 0.35 | 0.37 | 0.38 | 0.34 |  | 0.6 |  | 0.52 |  | 0.58 |  | 0.6 | 0.48 |
| 𝜆 |  |  | 1.9 |  |  |  |  |  | 3.2 |  |  |  |  |  | 4.3 |  |  |  |  |  |  |  |
| Notes: |  |  | Source 6, R1-162704, CN delay = 6ms, 8 ms HARQ RTT, 13-TTI UL access delay, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.3%, 7OS-28.0%, 3/4OS-35.9%, 2OS-38.3%, 1OS-38.3%, Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-4UEs, 3/4OS-3UEs, 2OS-2UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | 1.36 | 1.55 | 1.57 |  | 1.55 | 1.55 | 1.20 | 1.36 | 1.37 | 1.32 | 1.32 |  | 0.88 |  | 1.09 |  | 1.02 |  | 0.95 | 0.85 |
|  | 50% |  | 1.67 | 1.88 | 1.96 |  | 2.00 | 2.04 | 1.57 | 1.77 | 1.85 | 1.89 | 1.94 |  | 1.44 |  | 1.63 |  | 1.72 |  | 1.75 | 1.79 |
|  | 95% |  | 1.84 | 2.05 | 2.13 |  | 2.18 | 2.21 | 1.77 | 1.98 | 2.07 | 2.11 | 2.16 |  | 1.66 |  | 1.89 |  | 1.97 |  | 2.04 | 2.11 |
|  | Mean |  | 1.64 | 1.85 | 1.92 |  | 1.94 | 1.98 | 1.54 | 1.74 | 1.80 | 1.83 | 1.86 |  | 1.37 |  | 1.58 |  | 1.63 |  | 1.65 | 1.67 |
| DL: Delay CDF [s] | 5% |  | 0.055 | 0.049 | 0.047 |  | 0.046 | 0.045 | 0.058 | 0.051 | 0.049 | 0.048 | 0.046 |  | 0.063 |  | 0.055 |  | 0.052 |  | 0.050 | 0.048 |
|  | 50% |  | 0.061 | 0.054 | 0.052 |  | 0.051 | 0.050 | 0.068 | 0.059 | 0.057 | 0.055 | 0.054 |  | 0.078 |  | 0.067 |  | 0.064 |  | 0.062 | 0.061 |
|  | 95% |  | 0.077 | 0.067 | 0.067 |  | 0.068 | 0.067 | 0.092 | 0.081 | 0.081 | 0.083 | 0.085 |  | 0.144 |  | 0.108 |  | 0.120 |  | 0.135 | 0.148 |
|  | Mean |  | 0.063 | 0.056 | 0.054 |  | 0.053 | 0.052 | 0.071 | 0.062 | 0.060 | 0.060 | 0.059 |  | 0.089 |  | 0.073 |  | 0.074 |  | 0.075 | 0.076 |
| RU |  |  | 0.2 | 0.17 | 0.18 |  | 0.19 | 0.18 | 0.41 | 0.35 | 0.36 | 0.35 | 0.31 |  | 0.61 |  | 0.52 |  | 0.54 |  | 0.52 | 0.42 |
| 𝜆 |  |  | 1.9 |  |  |  |  |  | 3.2 |  |  |  |  |  | 4.3 |  |  |  |  |  |  |  |
| Notes: |  |  | Source 6, R1-162705, CN delay = 6ms, 8 TTIs HARQ RTT, 1-TTI UL access delay, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.3%, 7OS-28.0%, 3/4OS-35.9%, 2OS-38.3%, 1OS-38.3%, Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-4UEs, 3/4OS-3UEs, 2OS-2UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | 0.73 | 0.98 | 1.15 |  | 1.17 | - | 0.61 | 0.59 | 0.65 | 0.72 | - |  | 0.51 |  | 0.45 |  | 0.47 |  | 0.57 | - |
|  | 50% |  | 1.36 | 1.68 | 1.95 |  | 2.12 | - | 1.09 | 1.30 | 1.50 | 1.70 | - |  | 0.88 |  | 1.04 |  | 1.17 |  | 1.36 | - |
|  | 95% |  | 2.78 | 3.01 | 3.38 |  | 3.71 | - | 2.38 | 2.61 | 2.88 | 3.31 | - |  | 2.02 |  | 2.25 |  | 2.46 |  | 2.85 | - |
|  | Mean |  | 1.52 | 1.83 | 2.08 |  | 2.25 | - | 1.25 | 1.40 | 1.60 | 1.83 | - |  | 1.01 |  | 1.16 |  | 1.29 |  | 1.49 | - |
| DL: Delay CDF [s] | 5% |  | 0.06 | 0.05 | 0.04 |  | 0.04 | - | 0.06 | 0.05 | 0.05 | 0.04 | - |  | 0.07 |  | 0.06 |  | 0.05 |  | 0.04 | - |
|  | 50% |  | 0.09 | 0.07 | 0.06 |  | 0.05 | - | 0.10 | 0.08 | 0.07 | 0.06 | - |  | 0.12 |  | 0.10 |  | 0.09 |  | 0.08 | - |
|  | 95% |  | 0.14 | 0.11 | 0.09 |  | 0.10 | - | 0.17 | 0.19 | 0.16 | 0.15 | - |  | 0.21 |  | 0.24 |  | 0.23 |  | 0.19 | - |
|  | Mean |  | 0.09 | 0.07 | 0.06 |  | 0.06 | - | 0.11 | 0.10 | 0.08 | 0.08 | - |  | 0.13 |  | 0.12 |  | 0.10 |  | 0.09 | - |
| RU |  |  | 15.00 | 27.73 | 24.83 |  | 18.94 | - | 35.00 | 57.67 | 55.03 | 39.07 | - |  | 44.00 |  | 61.59 |  | 63.08 |  | 48.44 | - |
| 𝜆 |  |  | - |  |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |  |  |
| Notes: |  |  | Source 8, R1-165301, CN delay = 10ms, macro scenario, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | 1.64 | 2.26 | 2.41 |  | 2.58 |  | 0.60 | 0.78 | 1.01 | 1.11 |  |  | 0.32 |  | 0.35 |  | 0.63 |  | 0.58 | - |
|  | 50% |  | 2.42 | 3.21 | 3.63 |  | 4.03 |  | 2.27 | 3.02 | 3.45 | 3.70 |  |  | 1.98 |  | 2.58 |  | 3.08 |  | 3.21 | - |
|  | 95% |  | 2.73 | 3.98 | 4.97 |  | 5.10 |  | 2.69 | 3.93 | 4.92 | 4.97 |  |  | 2.68 |  | 3.88 |  | 4.80 |  | 4.94 | - |
|  | Mean |  | 2.32 | 3.16 | 3.69 |  | 3.94 |  | 2.04 | 2.86 | 3.43 | 3.57 |  |  | 1.71 |  | 2.40 |  | 3.02 |  | 3.09 | - |
| DL: Delay CDF [s] | 5% |  | 0.04 | 0.03 | 0.02 |  | 0.02 |  | 0.04 | 0.03 | 0.02 | 0.02 |  |  | 0.04 |  | 0.03 |  | 0.02 |  | 0.02 | - |
|  | 50% |  | 0.04 | 0.03 | 0.03 |  | 0.03 |  | 0.05 | 0.04 | 0.03 | 0.03 |  |  | 0.05 |  | 0.04 |  | 0.03 |  | 0.03 | - |
|  | 95% |  | 0.06 | 0.05 | 0.04 |  | 0.04 |  | 0.35 | 0.16 | 0.08 | 0.06 |  |  | 0.33 |  | 0.31 |  | 0.21 |  | 0.21 | - |
|  | Mean |  | 0.06 | 0.04 | 0.04 |  | 0.03 |  | 0.08 | 0.05 | 0.04 | 0.04 |  |  | 0.10 |  | 0.08 |  | 0.06 |  | 0.06 | - |
| RU |  |  | 12.4 | 10 | 11.4 |  | 9.8 |  | 35.8 | 27 | 25.2 | 25.8 |  |  | 57.8 |  | 52.8 |  | 45.2 |  | 45.6 |  |
| 𝜆 |  |  | 1.00 |  |  |  |  |  | 1.82 |  |  |  |  |  | 2.50 |  |  |  |  |  |  |  |
| Notes: |  |  | Source 7, R1-165418, File size: 100 Kb, Macro scenario, 6 ms core network delay, Dynamic DCI overhead, FTP model 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | 1.64 | 2.24 | 2.51 |  | 2.65 |  | 0.50 | 0.61 | 0.89 | 0.98 |  |  | 0.31 |  | 0.35 |  | 0.52 |  | 0.47 |  |
|  | 50% |  | 2.42 | 3.24 | 3.61 |  | 4.01 |  | 2.28 | 2.95 | 3.43 | 3.72 |  |  | 2.00 |  | 2.61 |  | 3.10 |  | 2.93 |  |
|  | 95% |  | 2.73 | 3.98 | 5.05 |  | 5.08 |  | 2.69 | 3.95 | 4.98 | 5.00 |  |  | 2.68 |  | 3.91 |  | 4.88 |  | 4.88 |  |
|  | Mean |  | 2.32 | 3.17 | 3.69 |  | 3.95 |  | 2.05 | 2.84 | 3.35 | 3.55 |  |  | 1.71 |  | 2.40 |  | 2.83 |  | 2.80 |  |
| DL: Delay CDF [s] | 5% |  | 0.04 | 0.03 | 0.02 |  | 0.02 |  | 0.04 | 0.03 | 0.02 | 0.02 |  |  | 0.04 |  | 0.03 |  | 0.02 |  | 0.02 |  |
|  | 50% |  | 0.04 | 0.03 | 0.03 |  | 0.03 |  | 0.05 | 0.04 | 0.03 | 0.03 |  |  | 0.05 |  | 0.04 |  | 0.03 |  | 0.04 |  |
|  | 95% |  | 0.06 | 0.05 | 0.04 |  | 0.04 |  | 0.34 | 0.21 | 0.14 | 0.10 |  |  | 0.34 |  | 0.30 |  | 0.24 |  | 0.27 |  |
|  | Mean |  | 0.06 | 0.04 | 0.03 |  | 0.03 |  | 0.08 | 0.06 | 0.05 | 0.04 |  |  | 0.11 |  | 0.08 |  | 0.07 |  | 0.07 |  |
| RU |  |  | 11.8 | 9.6 | 9 |  | 9.8 |  | 35 | 29.4 | 27.2 | 26.6 |  |  | 58 |  | 53.2 |  | 49.6 |  | 56 |  |
| 𝜆 |  |  | 1.00 |  |  |  |  |  | 1.82 |  |  |  |  |  | 2.50 |  |  |  |  |  |  |  |
| Notes: |  |  | Source 7, R1-165418, File size: 100 Kb, Macro scenario, 6 ms core network delay, static DCI overhead, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-15%, 7OS-18.0%, 3/4OS-19%, 2OS-21.5%, 1OS-28%; Maximum number of UEs that can be scheduled per sTTI: 14OS-5UEs, 7OS-2UEs, 3/4OS-1UEs, 2OS-1UEs, 1OS-1UE, FTP model 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | 1.52 | 1.88 | 2.02 |  | 2.03 |  | 1.15 | 1.25 | 1.33 | 1.3 |  |  | 0.86 |  | 0.74 |  | 0.78 |  | 0.77 |  |
|  | 50% |  | 2.29 | 2.94 | 3.37 |  | 3.59 |  | 2.08 | 2.62 | 2.96 | 2.98 |  |  | 1.94 |  | 2.16 |  | 2.55 |  | 2.44 |  |
|  | 95% |  | 2.67 | 3.88 | 4.87 |  | 4.9 |  | 2.63 | 3.77 | 4.75 | 4.71 |  |  | 2.59 |  | 3.72 |  | 4.65 |  | 4.62 |  |
|  | Mean |  | 2.23 | 2.95 | 3.37 |  | 3.56 |  | 2 | 2.61 | 2.97 | 3.03 |  |  | 1.86 |  | 2.25 |  | 2.59 |  | 2.58 |  |
| DL: Delay CDF [s] | 5% |  | 0.039 | 0.027 | 0.022 |  | 0.021 |  | 0.04 | 0.028 | 0.022 | 0.022 |  |  | 0.04 |  | 0.028 |  | 0.023 |  | 0.023 |  |
|  | 50% |  | 0.046 | 0.036 | 0.031 |  | 0.029 |  | 0.05 | 0.04 | 0.035 | 0.035 |  |  | 0.054 |  | 0.048 |  | 0.041 |  | 0.043 |  |
|  | 95% |  | 0.069 | 0.056 | 0.052 |  | 0.052 |  | 0.091 | 0.084 | 0.08 | 0.08 |  |  | 0.122 |  | 0.141 |  | 0.134 |  | 0.136 |  |
|  | Mean |  | 0.049 | 0.038 | 0.033 |  | 0.032 |  | 0.057 | 0.045 | 0.042 | 0.042 |  |  | 0.063 |  | 0.06 |  | 0.056 |  | 0.056 |  |
| RU |  |  | 22.5 | 23.5 | 24 |  | 25 |  | 45 | 43.5 | 45 | 48 |  |  | 60 |  | 63 |  | 65 |  | 66.5 |  |
| 𝜆 |  |  | 2.86 |  |  |  |  |  | 5.26 |  |  |  |  |  | 8.33 |  |  |  |  |  |  |  |
| Notes: |  |  | Source 7, R1-165418, File size: 100 Kb, Macro Scenario, 6 ms core network delay, static DCI overhead, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-15%, 7OS-18.0%, 3/4OS-19%, 2OS-21.5%, 1OS-28%; Maximum number of UEs that can be scheduled per sTTI: 14OS-5UEs, 7OS-2UEs, 3/4OS-1UEs, 2OS-1UEs, 1OS-1UE, FTP model 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | 1.54 | 1.87 | 2.1 |  | 2.11 |  | 1.25 | 1.39 | 1.45 | 1.56 |  |  | 0.93 |  | 0.97 |  | 1.02 |  | 0.98 |  |
|  | 50% |  | 2.27 | 2.96 | 3.34 |  | 3.56 |  | 2.1 | 2.64 | 2.91 | 3.02 |  |  | 1.91 |  | 2.31 |  | 2.45 |  | 2.41 |  |
|  | 95% |  | 2.67 | 3.83 | 4.81 |  | 4.88 |  | 2.63 | 3.75 | 4.6 | 4.71 |  |  | 2.6 |  | 3.66 |  | 4.43 |  | 4.5 |  |
|  | Mean |  | 2.21 | 2.9 | 3.42 |  | 3.54 |  | 2.04 | 2.61 | 2.99 | 3.08 |  |  | 1.86 |  | 2.33 |  | 2.61 |  | 2.57 |  |
| DL: Delay CDF [s] | 5% |  | 0.039 | 0.027 | 0.022 |  | 0.021 |  | 0.04 | 0.028 | 0.023 | 0.022 |  |  | 0.04 |  | 0.029 |  | 0.024 |  | 0.023 |  |
|  | 50% |  | 0.046 | 0.035 | 0.031 |  | 0.029 |  | 0.05 | 0.04 | 0.036 | 0.035 |  |  | 0.055 |  | 0.045 |  | 0.042 |  | 0.043 |  |
|  | 95% |  | 0.068 | 0.056 | 0.05 |  | 0.05 |  | 0.084 | 0.076 | 0.073 | 0.067 |  |  | 0.114 |  | 0.109 |  | 0.104 |  | 0.108 |  |
|  | Mean |  | 0.049 | 0.038 | 0.033 |  | 0.032 |  | 0.055 | 0.044 | 0.04 | 0.039 |  |  | 0.064 |  | 0.054 |  | 0.051 |  | 0.052 |  |
| RU |  |  | 23 | 24 | 23.4 |  | 24.4 |  | 42.4 | 43.4 | 44.4 | 45 |  |  | 50 |  | 40 |  | 41 |  | 41 |  |
| 𝜆 |  |  | 2.86 |  |  |  |  |  | 5.26 |  |  |  |  |  | 8.33 |  |  |  |  |  |  |  |
| Notes: |  |  | Source 7, R1-165418, File size: 100 Kb, Macro Scenario, 6 ms core network delay, Dynamic DCI overhead, FTP model 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.1.1-2: Results with 100kB file transfer

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 4.1369 (0) | 4.8056 (+16.2) | 4.9679 (+20.1) | 5.1222 (+23.8) | - | 2.9091 (0) | 3.3283 (+14.4) | 3.4316 (+18) | 3.4583 (+18.9) | - | 1.7492 (0) |  | 2.018 (+15.4) |  | 1.8616 (+6.4) |  | 1.621 (-7.3) |  | - |
|  | 50% | 5.0463 (0) | 6.7574 (+33.9) | 7.8039 (+54.7) | 8.5604 (+69.6) | - | 4.9248 (0) | 6.3506 (+29) | 7.1025 (+44.2) | 7.5828 (+53.9) | - | 4.3616 (0) |  | 5.3047 (+21.6) |  | 5.4931 (+25.9) |  | 5.44 (+24.7) |  | - |
|  | 95% | 5.2785 (0) | 7.0448 (+33.5) | 8.1752 (+54.9) | 9.0656 (+71.8) | - | 5.2069 (0) | 6.9475 (+33.4) | 8.0233 (+54.1) | 8.8758 (+76.6) | - | 5.194 (0) |  | 6.7762 (+30.5) |  | 7.5335 (+45) |  | 8.0286 (+54.6) |  | - |
|  | Mean | 4.9235 (0) | 6.4509 (+31) | 7.3843 (+50) | 7.9954 (+62.4) | - | 4.6202 (0) | 5.9253 (+28.3) | 6.6492 (+43.9) | 7.0905 (+53.5) | - | 4.0523 (0) |  | 4.9797 (+22.9) |  | 5.2773 (+30.2) |  | 5.2525 (+29.6) |  | - |
| DL: Delay CDF [s] | 5% | 0.1591 (0) | 0.1191 (-25.1) | 0.1026 (-35.5) | 0.0925 (-41.9) | - | 0.1612 (0) | 0.1204 (-25.3) | 0.1046 (-35.1) | 0.0944 (-41.4) | - | 0.1619 (0) |  | 0.1234 (-23.8) |  | 0.1093 (-32.5) |  | 0.1063 (-34.3) |  | - |
|  | 50% | 0.167 (0) | 0.1243 (-25.6) | 0.1077 (-35.5) | 0.0985 (-41) | - | 0.1711 (0) | 0.1332 (-22.2) | 0.1201 (-29.8) | 0.1132 (-33.8) | - | 0.1995 (0) |  | 0.1674 (-16.1) |  | 0.165 (-17.3) |  | 0.1688 (-15.4) |  | - |
|  | 95% | 0.1999 (0) | 0.1754 (-12.3) | 0.1609 (-19.5) | 0.1658 (-17.1) | - | 0.3019 (0) | 0.2536 (-16) | 0.2434 (-19.4) | 0.2405 (-20.3) | - | 0.5183 (0) |  | 0.4466 (-13.8) |  | 0.4901 (-5.4) |  | 0.5828 (+12.4) |  | - |
|  | Mean | 0.1749 (0) | 0.1361 (-22.2) | 0.1212 (-30.7) | 0.1144 (-34.6) | - | 0.1953 (0) | 0.1562 (-20) | 0.1432 (-26.7) | 0.139 (-28.8) | - | 0.2508 (0) |  | 0.2151 (-14.2) |  | 0.214 (-14.7) |  | 0.2365 (-5.7) |  | - |
| RU |  | 16.5 | 16.8 | 17.7 | 19.8 | - | 40.5 | 39.8 | 40.2 | 42.3 | - | 70.4 |  | 69.8 |  | 72.9 |  | 76.4 |  | - |
| 𝜆 |  | 0.5 |  |  |  |  | 1 |  |  |  |  | 2 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 1, R1-162398, 6ms core network delay, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-21.43%, 7OS-23.14%, 3/4OS-26.57%, 2OS-31.71%, Maximum number of UEs that can be scheduled per sTTI: 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.19 | 3.51 | 2.82 | 1.79 | - | 2.27 | 2.26 | 1.33 | 0.72 | - |  | 1.73 |  | 1.45 |  | 0.70 |  | 0.32 | - |
|  | 50% | 4.81 | 6.50 | 7.22 | 7.49 | - | 4.68 | 5.68 | 5.32 | 4.62 | - |  | 4.43 |  | 5.08 |  | 3.93 |  | 3.23 | - |
|  | 95% | 5.21 | 7.44 | 9.23 | 10.59 | - | 5.23 | 7.47 | 9.08 | 10.05 | - |  | 5.25 |  | 7.41 |  | 8.72 |  | 9.54 | - |
|  | Mean | 4.57 | 6.09 | 6.75 | 6.95 | - | 4.31 | 5.43 | 5.37 | 5.10 | - |  | 4.05 |  | 4.84 |  | 4.37 |  | 4.04 | - |
| DL: Delay CDF [s] | 5% | 0.154 | 0.108 | 0.087 | 0.076 | - | 0.153 | 0.107 | 0.088 | 0.080 | - |  | 0.153 |  | 0.108 |  | 0.092 |  | 0.084 | - |
|  | 50% | 0.167 | 0.123 | 0.111 | 0.107 | - | 0.173 | 0.141 | 0.152 | 0.173 | - |  | 0.181 |  | 0.159 |  | 0.203 |  | 0.242 | - |
|  | 95% | 0.258 | 0.230 | 0.284 | 0.454 | - | 0.353 | 0.354 | 0.618 | 1.000 | - |  | 0.462 |  | 0.558 |  | 1.058 |  | 1.502 | - |
|  | Mean | 0.182 | 0.143 | 0.139 | 0.170 | - | 0.203 | 0.178 | 0.248 | 0.361 | - |  | 0.230 |  | 0.237 |  | 0.410 |  | 0.560 | - |
| RU |  | 0.21 | 0.2 | 0.23 | 0.24 |  | 0.41 | 0.43 | 0.51 | 0.48 | - |  | 0.56 |  | 0.6 |  | 0.7 |  | 0.63 | - |
| 𝜆 |  | 0.4 |  |  |  |  | 0.8 |  |  |  |  |  | 1.2 |  |  |  |  |  |  |  |
| Notes: |  | Source 4, R1-162503, CN delay = 10ms, CQI period = 5TTI, CQI delay = 6TTI FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.74 | 5.97 | 9.08 | 14.85 | 14.27 | - | - | - | - | - | 0.61 |  | 1.28 |  | 2.71 |  | 3.45 |  | 2.37 |
|  | 50% | 12.90 | 20.78 | 25.23 | 28.14 | 29.17 | - | - | - | - | - | 6.20 |  | 15.09 |  | 18.32 |  | 19.86 |  | 19.05 |
|  | 95% | 21.62 | 27.59 | 31.48 | 33.14 | 34.04 | - | - | - | - | - | 18.18 |  | 25.81 |  | 29.34 |  | 31.46 |  | 32.28 |
|  | Mean | 12.08 | 19.45 | 23.74 | 26.70 | 27.30 | - | - | - | - | - | 7.55 |  | 14.24 |  | 17.19 |  | 18.84 |  | 18.40 |
| DL: Delay CDF [s] | 5% | 0.2140 | 0.1340 | 0.0905 | 0.0537 | 0.0561 | - | - | - | - | - | 1.3070 |  | 0.6240 |  | 0.3136 |  | 0.2319 |  | 0.3368 |
|  | 50% | 0.0620 | 0.0385 | 0.0317 | 0.0284 | 0.0274 | - | - | - | - | - | 0.1290 |  | 0.0530 |  | 0.0438 |  | 0.0403 |  | 0.0420 |
|  | 95% | 0.0370 | 0.0290 | 0.0254 | 0.0241 | 0.0235 | - | - | - | - | - | 0.0440 |  | 0.0310 |  | 0.0273 |  | 0.0254 |  | 0.0248 |
|  | Mean | 0.0997 | 0.0513 | 0.0391 | 0.0325 | 0.0330 | - | - | - | - | - | 0.3184 |  | 0.1617 |  | 0.1081 |  | 0.0800 |  | 0.1027 |
| RU |  | 0.23 | 0.21 | 0.20 | 0.18 | 0.20 | - | - | - | - | - | 0.77 |  | 0.72 |  | 0.66 |  | 0.61 |  | 0.65 |
| 𝜆 |  | 1.05 |  |  |  |  | - |  |  |  |  | 2.1 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162618, CN delay = 6ms, CSI reporting period = 5*TTI, CSI measurement reference resource is located in the (n- 4* TTI) FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 60km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 5.97 | 10.81 | 14.20 | 15.77 | 15.28 | - | - | - | - | - | 0.71 |  | 1.63 |  | 2.04 |  | 2.04 |  | 1.71 |
|  | 50% | 19.51 | 36.36 | 46.28 | 49.56 | 45.71 | - | - | - | - | - | 7.08 |  | 20.00 |  | 21.96 |  | 22.67 |  | 21.25 |
|  | 95% | 44.44 | 72.73 | 74.18 | 71.80 | 65.89 | - | - | - | - | - | 29.63 |  | 57.14 |  | 61.55 |  | 62.92 |  | 56.85 |
|  | Mean | 20.94 | 38.11 | 46.43 | 48.23 | 44.42 | - | - | - | - | - | 9.99 |  | 23.32 |  | 25.73 |  | 26.28 |  | 24.24 |
| DL: Delay CDF [s] | 5% | 0.1340 | 0.0740 | 0.0563 | 0.0504 | 0.0524 | - | - | - | - | - | 1.1300 |  | 0.4910 |  | 0.3991 |  | 0.3914 |  | 0.4671 |
|  | 50% | 0.0410 | 0.0220 | 0.0173 | 0.0161 | 0.0175 | - | - | - | - | - | 0.1130 |  | 0.0400 |  | 0.0364 |  | 0.0353 |  | 0.0376 |
|  | 95% | 0.0180 | 0.0110 | 0.0108 | 0.0111 | 0.0121 | - | - | - | - | - | 0.0270 |  | 0.0140 |  | 0.0130 |  | 0.0127 |  | 0.0141 |
|  | Mean | 0.0586 | 0.0305 | 0.0233 | 0.0219 | 0.0232 | - | - | - | - | - | 0.2880 |  | 0.1422 |  | 0.1103 |  | 0.1137 |  | 0.1292 |
| RU |  | 0.22 | 0.22 | 0.20 | 0.19 | 0.20 | - | - | - | - | - | 0.77 |  | 0.72 |  | 0.69 |  | 0.68 |  | 0.72 |
| 𝜆 |  | 1.05 |  |  |  |  | - |  |  |  |  | 2.1 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162618, CN delay = 6ms, CSI reporting period = 5ms, CSI measurement reference resource is located in the (n- 4* TTI). FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 60km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.74 | 5.99 | 8.29 | 11.48 | 11.05 | - | - | - | - | - | 0.61 |  | 0.98 |  | 1.38 |  | 1.18 |  | 1.20 |
|  | 50% | 12.90 | 20.51 | 24.56 | 27.18 | 28.28 | - | - | - | - | - | 6.20 |  | 16.59 |  | 17.67 |  | 16.44 |  | 14.41 |
|  | 95% | 21.62 | 27.59 | 31.48 | 33.14 | 34.04 | - | - | - | - | - | 18.18 |  | 31.55 |  | 30.77 |  | 28.96 |  | 25.81 |
|  | Mean | 12.08 | 19.14 | 23.01 | 25.45 | 25.94 | - | - | - | - | - | 7.55 |  | 16.47 |  | 16.83 |  | 15.76 |  | 13.75 |
| DL: Delay CDF [s] | 5% | 0.2140 | 0.1330 | 0.0979 | 0.0696 | 0.0724 | - | - | - | - | - | 1.3070 |  | 0.8187 |  | 0.5774 |  | 0.6815 |  | 0.6665 |
|  | 50% | 0.0620 | 0.0390 | 0.0326 | 0.0294 | 0.0283 | - | - | - | - | - | 0.1290 |  | 0.0482 |  | 0.0453 |  | 0.0487 |  | 0.0555 |
|  | 95% | 0.0370 | 0.0290 | 0.0254 | 0.0241 | 0.0235 | - | - | - | - | - | 0.0440 |  | 0.0253 |  | 0.0260 |  | 0.0276 |  | 0.0310 |
|  | Mean | 0.0997 | 0.0520 | 0.0414 | 0.0362 | 0.0365 | - | - | - | - | - | 0.3184 |  | 0.1903 |  | 0.1558 |  | 0.1798 |  | 0.1891 |
| RU |  | 0.23 | 0.22 | 0.22 | 0.21 | 0.23 | - | - | - | - | - | 0.77 |  | 0.77 |  | 0.73 |  | 0.75 |  | 0.77 |
| 𝜆 |  | 1.05 |  |  |  |  | - |  |  |  |  | 2.1 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162618, CN delay = 6ms, CSI reporting period = 5ms, CSI measurement reference resource is located in the (n- 4ms). FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 60km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.87 | 3.34 | 3.21 | 2.98 | 2.73 | 2.41 | 2.72 | 2.52 | 2.28 | 2.01 |  | 1.86 |  | 2.23 |  | 1.90 |  | 1.57 | 1.29 |
|  | 50% | 4.32 | 5.75 | 6.52 | 6.74 | 6.86 | 4.09 | 5.48 | 6.09 | 6.22 | 6.23 |  | 3.92 |  | 5.17 |  | 5.67 |  | 5.77 | 5.52 |
|  | 95% | 4.68 | 6.36 | 7.62 | 8.40 | 9.02 | 4.62 | 6.30 | 7.60 | 8.35 | 8.95 |  | 4.57 |  | 6.26 |  | 7.53 |  | 8.25 | 8.82 |
|  | Mean | 4.09 | 5.37 | 6.04 | 6.26 | 6.45 | 3.87 | 5.07 | 5.66 | 5.83 | 5.88 |  | 3.63 |  | 4.79 |  | 5.25 |  | 5.32 | 5.31 |
| DL: Delay CDF [s] | 5% | 0.171 | 0.126 | 0.105 | 0.095 | 0.089 | 0.173 | 0.127 | 0.105 | 0.096 | 0.089 |  | 0.175 |  | 0.128 |  | 0.106 |  | 0.097 | 0.091 |
|  | 50% | 0.185 | 0.139 | 0.124 | 0.119 | 0.117 | 0.198 | 0.149 | 0.135 | 0.134 | 0.135 |  | 0.211 |  | 0.160 |  | 0.148 |  | 0.148 | 0.162 |
|  | 95% | 0.287 | 0.247 | 0.262 | 0.281 | 0.308 | 0.359 | 0.321 | 0.348 | 0.396 | 0.442 |  | 0.494 |  | 0.420 |  | 0.496 |  | 0.631 | 0.747 |
|  | Mean | 0.204 | 0.160 | 0.148 | 0.150 | 0.151 | 0.226 | 0.179 | 0.172 | 0.177 | 0.189 |  | 0.262 |  | 0.203 |  | 0.212 |  | 0.235 | 0.257 |
| RU |  | 0.2 | 0.18 | 0.21 | 0.23 | 0.23 | 0.4 | 0.38 | 0.43 | 0.45 | 0.47 |  | 0.6 |  | 0.57 |  | 0.63 |  | 0.67 | 0.67 |
| 𝜆 |  | 0.31 |  |  |  |  | 0.53 |  |  |  |  |  | 0.64 |  |  |  |  |  |  |  |
| Notes: |  | Source 6, R1-162703, CN delay = 6ms, 8 TTIs HARQ RTT, 13-TTIs UL access delay, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.3%, 7OS-28.0%, 3/4OS-35.9%, 2OS-38.3%, 1OS-38.3%, Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-4UEs, 3/4OS-3UEs, 2OS-2UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.87 | 3.19 | 3.05 | 2.69 | 2.58 | 2.41 | 2.74 | 2.37 | 2.09 | 1.85 |  | 1.86 |  | 2.06 |  | 1.66 |  | 1.35 | 1.17 |
|  | 50% | 4.32 | 5.54 | 6.04 | 6.19 | 6.14 | 4.09 | 5.25 | 5.72 | 5.57 | 5.49 |  | 3.92 |  | 5.00 |  | 5.33 |  | 5.14 | 4.82 |
|  | 95% | 4.68 | 6.33 | 7.35 | 7.83 | 8.11 | 4.62 | 6.25 | 7.17 | 7.69 | 7.95 |  | 4.57 |  | 6.13 |  | 7.02 |  | 7.40 | 7.63 |
|  | Mean | 4.09 | 5.22 | 5.66 | 5.77 | 5.77 | 3.87 | 4.93 | 5.28 | 5.30 | 5.21 |  | 3.63 |  | 4.63 |  | 4.85 |  | 4.80 | 4.62 |
| DL: Delay CDF [s] | 5% | 0.171 | 0.126 | 0.109 | 0.102 | 0.099 | 0.173 | 0.128 | 0.112 | 0.104 | 0.101 |  | 0.175 |  | 0.131 |  | 0.114 |  | 0.109 | 0.106 |
|  | 50% | 0.185 | 0.144 | 0.133 | 0.131 | 0.132 | 0.198 | 0.155 | 0.143 | 0.149 | 0.152 |  | 0.211 |  | 0.166 |  | 0.158 |  | 0.168 | 0.182 |
|  | 95% | 0.287 | 0.256 | 0.268 | 0.303 | 0.313 | 0.359 | 0.315 | 0.379 | 0.419 | 0.482 |  | 0.494 |  | 0.446 |  | 0.575 |  | 0.703 | 0.871 |
|  | Mean | 0.204 | 0.164 | 0.157 | 0.161 | 0.165 | 0.226 | 0.182 | 0.183 | 0.195 | 0.211 |  | 0.262 |  | 0.213 |  | 0.235 |  | 0.261 | 0.296 |
| RU |  | 0.2 | 0.19 | 0.2 | 0.22 | 0.24 | 0.4 | 0.39 | 0.42 | 0.48 | 0.49 |  | 0.6 |  | 0.58 |  | 0.66 |  | 0.7 | 0.7 |
| 𝜆 |  | 0.31 |  |  |  |  | 0.53 |  |  |  |  |  | 0.64 |  |  |  |  |  |  |  |
| Notes: |  | Source 6, R1-162704, CN delay = 6ms, 8 ms HARQ RTT, 13-TTIs UL access delay, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.3%, 7OS-28.0%, 3/4OS-35.9%, 2OS-38.3%, 1OS-38.3%, Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-4UEs, 3/4OS-3UEs, 2OS-2UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.45 | 3.64 | 3.38 | 2.90 | 2.71 | 2.92 | 3.18 | 2.56 | 2.31 | 1.91 |  | 2.14 |  | 2.40 |  | 1.99 |  | 1.51 | 1.23 |
|  | 50% | 6.75 | 7.47 | 7.41 | 7.21 | 7.15 | 6.37 | 7.02 | 6.94 | 6.62 | 6.49 |  | 5.94 |  | 6.57 |  | 6.43 |  | 6.08 | 5.61 |
|  | 95% | 8.00 | 8.89 | 9.18 | 9.43 | 9.58 | 7.85 | 8.79 | 9.14 | 9.34 | 9.48 |  | 7.68 |  | 8.66 |  | 9.00 |  | 9.17 | 9.34 |
|  | Mean | 6.28 | 6.95 | 6.90 | 6.74 | 6.72 | 5.93 | 6.56 | 6.48 | 6.27 | 6.11 |  | 5.49 |  | 6.13 |  | 5.98 |  | 5.70 | 5.46 |
| DL: Delay CDF [s] | 5% | 0.100 | 0.090 | 0.087 | 0.085 | 0.084 | 0.102 | 0.091 | 0.088 | 0.086 | 0.084 |  | 0.104 |  | 0.092 |  | 0.089 |  | 0.087 | 0.086 |
|  | 50% | 0.119 | 0.108 | 0.109 | 0.112 | 0.113 | 0.128 | 0.117 | 0.118 | 0.124 | 0.129 |  | 0.141 |  | 0.128 |  | 0.133 |  | 0.142 | 0.161 |
|  | 95% | 0.241 | 0.228 | 0.247 | 0.281 | 0.302 | 0.308 | 0.271 | 0.344 | 0.369 | 0.478 |  | 0.444 |  | 0.399 |  | 0.501 |  | 0.645 | 0.785 |
|  | Mean | 0.142 | 0.129 | 0.134 | 0.143 | 0.147 | 0.160 | 0.145 | 0.158 | 0.170 | 0.188 |  | 0.195 |  | 0.174 |  | 0.197 |  | 0.231 | 0.262 |
| RU |  | 0.2 | 0.19 | 0.21 | 0.23 | 0.24 | 0.41 | 0.38 | 0.43 | 0.46 | 0.48 |  | 0.61 |  | 0.57 |  | 0.62 |  | 0.67 | 0.68 |
| 𝜆 |  | 0.31 |  |  |  |  | 0.53 |  |  |  |  |  | 0.64 |  |  |  |  |  |  |  |
| Notes: |  | Source 6, R1-162705, CN delay = 6ms, 8 TTIs HARQ RTT, 1-TTI UL access delay, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.3%, 7OS-28.0%, 3/4OS-35.9%, 2OS-38.3%, 1OS-38.3%, Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-4UEs, 3/4OS-3UEs, 2OS-2UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.2 | 2.6 | 2.67 | 2.5 | - | 1.7 | 1.2 | 1.36 | 1.5 | - |  | 1.1 |  | 0.6 |  | 0.84 |  | 0.8 | - |
|  | 50% | 2.9 | 3.9 | 4.38 | 4.8 | - | 2.9 | 3.3 | 3.73 | 4.1 | - |  | 2.6 |  | 2.3 |  | 2.88 |  | 2.9 | - |
|  | 95% | 7.8 | 8.1 | 9.53 | 10.1 | - | 8.0 | 7.5 | 8.66 | 9.6 |  |  | 7.4 |  | 6.4 |  | 7.40 |  | 8.0 | - |
|  | Mean | 3.9 | 4.6 | 5.18 | 5.6 | - | 3.8 | 3.7 | 4.17 | 4.6 | - |  | 3.3 |  | 2.8 |  | 3.31 |  | 3.4 | - |
| DL: Delay CDF [s] | 5% | 0.16 | 0.14 | 0.12 | 0.11 | - | 0.14 | 0.11 | 0.12 | 0.11 | - |  | 0.17 |  | 0.16 |  | 0.13 |  | 0.13 | - |
|  | 50% | 0.28 | 0.21 | 0.19 | 0.17 | - | 0.26 | 0.21 | 0.23 | 0.21 | - |  | 0.32 |  | 0.38 |  | 0.30 |  | 0.29 | - |
|  | 95% | 0.38 | 0.33 | 0.32 | 0.35 | - | 0.75 | 0.62 | 0.71 | 0.62 | - |  | 0.83 |  | 1.35 |  | 1.06 |  | 1.11 | - |
|  | Mean | 0.28 | 0.22 | 0.21 | 0.21 | - | 0.30 | 0.34 | 0.31 | 0.28 | - |  | 0.39 |  | 0.51 |  | 0.41 |  | 0.42 | - |
| RU |  |  |  |  | 20.0 |  |  | 26.8 |  |  | 25.23 |  | 19.7 |  | - | 40.0 | 52.9 | 50.38 | 39.2 | - |  | 60.0 |  | 70.2 |  | 63.22 |  | 53.7 | - |
| 𝜆 |  | - |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |  |  |
| Notes: |  | Source 8, R1-163312, CN delay = 10ms, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI), HARQ RTT of 8 TTI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.2 | 2.2 | - | 2.4 | - | 1.7 | 1.2 | - | 1.4 | - |  | 1.1 |  | 0.6 |  | - |  | 0.8 | - |
|  | 50% | 2.9 | 3.3 | - | 4.5 | - | 2.9 | 2.9 | - | 3.8 | - |  | 2.6 |  | 2.2 |  | - |  | 2.8 | - |
|  | 95% | 7.8 | 7.6 | - | 9.9 | - | 8.0 | 7.0 | - | 8.9 |  |  | 7.4 |  | 5.8 |  | - |  | 7.7 | - |
|  | Mean | 3.9 | 4.0 | - | 5.2 | - | 3.8 | 3.4 | - | 4.2 | - |  | 3.3 |  | 2.5 |  | - |  | 3.2 | - |
| DL: Delay CDF [s] | 5% | 0.16 | 0.15 | - | 0.11 | - | 0.16 | 0.15 | - | 0.12 | - |  | 0.17 |  | 0.18 |  | - |  | 0.13 | - |
|  | 50% | 0.28 | 0.25 | - | 0.18 | - | 0.29 | 0.28 | - | 0.23 | - |  | 0.32 |  | 0.40 |  | - |  | 0.31 | - |
|  | 95% | 0.38 | 0.38 | - | 0.38 | - | 0.49 | 0.77 | - | 0.72 | - |  | 0.83 |  | 1.49 |  | - |  | 1.17 | - |
|  | Mean | 0.28 | 0.26 | - | 0.22 | - | 0.30 | 0.36 | - | 0.30 | - |  | 0.39 |  | 0.56 |  | - |  | 0.45 | - |
| RU |  | 0.2 | 0.27 | - | 0.2 | - | 0.4 | 0.53 | - | 0.4 | - |  | 0.6 |  | 0.68 |  | - |  | 0.55 | - |
| 𝜆 |  | - |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |  |  |
| Notes: |  | Source 8, R1-163312, CN delay = 10ms, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI), HARQ RTT of 12 TTI for 2 OS TTI length and 14 TTI for 7 OS TTI length |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.2 | - | - | 2.0 | - | 1.7 | - | - | 1.1 | - |  | 1.1 |  | - |  | - |  | 0.6 | - |
|  | 50% | 2.9 | - | - | 3.9 | - | 2.9 | - | - | 3.3 | - |  | 2.6 |  | - |  | - |  | 2.5 | - |
|  | 95% | 7.8 | - | - | 8.2 | - | 8.0 | - | - | 8.0 |  |  | 7.4 |  | - |  | - |  | 7.2 | - |
|  | Mean | 3.9 | - | - | 4.5 | - | 3.8 | - | - | 3.8 | - |  | 3.3 |  | - |  | - |  | 3.0 | - |
| DL: Delay CDF [s] | 5% | 0.16 | - | - | 0.13 | - | 0.16 | - | - | 0.14 | - |  | 0.17 |  | - |  | - |  | 0.15 | - |
|  | 50% | 0.28 | - | - | 0.21 | - | 0.29 | - | - | 0.25 | - |  | 0.32 |  | - |  | - |  | 0.34 | - |
|  | 95% | 0.38 | - | - | 0.44 | - | 0.49 | - | - | 0.91 | - |  | 0.83 |  | - |  | - |  | 1.38 | - |
|  | Mean | 0.28 | - | - | 0.25 | - | 0.30 | - | - | 0.35 | - |  | 0.39 |  | - |  | - |  | 0.49 | - |
| RU |  | 0.2 | 0.27 | - | 0.22 | - | 0.4 | 0.53 | - | 0.41 | - |  | 0.6 |  | 0.68 |  | - |  | 0.54 | - |
| 𝜆 |  | - |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |  |  |
| Notes: |  | Source 8, R1-163312, CN delay = 10ms, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI), HARQ RTT of 22 TTI for 2 OS TTI length |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.02 | 4.21 | 4.91 | 5.37 | 5.64 | - | - | - | - | - | 1.57 |  | 2.77 |  | 3.68 |  | 3.65 |  | 2.57 |
|  | 50% | 10.53 | 16.33 | 18.30 | 19.65 | 19.93 | - | - | - | - | - | 7.27 |  | 13.11 |  | 14.71 |  | 15.38 |  | 14.95 |
|  | 95% | 16.67 | 19.51 | 21.08 | 22.13 | 22.54 | - | - | - | - | - | 15.38 |  | 19.05 |  | 20.36 |  | 21.37 |  | 21.71 |
|  | Mean | 9.42 | 14.19 | 16.14 | 17.55 | 17.88 | - | - | - | - | - | 7.64 |  | 11.89 |  | 13.33 |  | 14.00 |  | 13.66 |
| DL: Delay CDF [s] | 5% | 0.2650 | 0.1900 | 0.1629 | 0.1490 | 0.1419 | - | - | - | - | - | 0.5090 |  | 0.2885 |  | 0.2176 |  | 0.2186 |  | 0.3094 |
|  | 50% | 0.0760 | 0.0485 | 0.0437 | 0.0407 | 0.0401 | - | - | - | - | - | 0.1100 |  | 0.0610 |  | 0.0544 |  | 0.0520 |  | 0.0535 |
|  | 95% | 0.0480 | 0.0410 | 0.0380 | 0.0361 | 0.0355 | - | - | - | - | - | 0.0520 |  | 0.0420 |  | 0.0393 |  | 0.0374 |  | 0.0369 |
|  | Mean | 0.1325 | 0.0767 | 0.0632 | 0.0552 | 0.0548 | - | - | - | - | - | 0.1872 |  | 0.1081 |  | 0.0881 |  | 0.0868 |  | 0.1117 |
| RU |  | 0.21 | 0.20 | 0.20 | 0.19 | 0.20 | - | - | - | - | - | 0.68 |  | 0.63 |  | 0.59 |  | 0.59 |  | 0.64 |
| 𝜆 |  | 1.05 |  |  |  |  |  |  |  |  |  | 2.1 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 10ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 3km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.74 | 3.66 | 4.36 | 4.79 | 4.05 | - | - | - | - | - |  | 1.27 |  | 1.57 |  | 1.44 |  | 1.03 | 0.55 |
|  | 50% | 3.65 | 9.20 | 12.77 | 14.32 | 11.94 | - | - | - | - | - |  | 3.48 |  | 7.31 |  | 8.16 |  | 8.16 | 5.79 |
|  | 95% | 14.81 | 17.58 | 18.57 | 19.18 | 18.67 | - | - | - | - | - |  | 12.50 |  | 16.33 |  | 17.46 |  | 17.95 | 16.26 |
|  | Mean | 6.50 | 9.62 | 11.64 | 12.92 | 11.52 | - | - | - | - | - |  | 5.03 |  | 8.21 |  | 8.85 |  | 8.83 | 7.07 |
| DL: Delay CDF [s] | 5% | 0.2910 | 0.2185 | 0.1834 | 0.1669 | 0.1971 | - | - | - | - | - |  | 0.6270 |  | 0.5070 |  | 0.5554 |  | 0.7644 | 1.4449 |
|  | 50% | 0.2190 | 0.0870 | 0.0631 | 0.0557 | 0.0669 | - | - | - | - | - |  | 0.2300 |  | 0.1095 |  | 0.0982 |  | 0.0980 | 0.1382 |
|  | 95% | 0.0540 | 0.0455 | 0.0431 | 0.0417 | 0.0429 | - | - | - | - | - |  | 0.0640 |  | 0.0490 |  | 0.0459 |  | 0.0446 | 0.0492 |
|  | Mean | 0.1783 | 0.1199 | 0.0955 | 0.0833 | 0.0963 | - | - | - | - | - |  | 0.2646 |  | 0.1777 |  | 0.1768 |  | 0.2210 | 0.3483 |
| RU |  | 0.23 | 0.22 | 0.21 | 0.21 | 0.26 | - | - | - | - | - |  | 0.66 |  | 0.62 |  | 0.64 |  | 0.69 | 0.82 |
| 𝜆 |  | 1.05 |  |  |  |  |  |  |  |  |  |  | 2.1 |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 10ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 3km/h, 10M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-21.43%, 7OS-23.14%, 3/4OS-26.57%, 2OS-31.71%, 1OS-42%Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | - | - | - | - | - | 0.45 |  | 1.23 |  | 1.51 |  | 2.21 |  | 1.64 |
|  | 50% | - | - | - | - | - | - | - | - | - | - | 4.55 |  | 11.43 |  | 12.87 |  | 14.07 |  | 13.43 |
|  | 95% | - | - | - | - | - | - | - | - | - | - | 13.79 |  | 17.98 |  | 19.63 |  | 20.90 |  | 21.17 |
|  | Mean | - | - | - | - | - | - | - | - | - | - | 5.77 |  | 10.48 |  | 11.68 |  | 12.90 |  | 12.58 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | - | - | - | - | - | 1.7610 |  | 0.6500 |  | 0.5360 |  | 0.3616 |  | 0.4844 |
|  | 50% | - | - | - | - | - | - | - | - | - | - | 0.1760 |  | 0.0700 |  | 0.0623 |  | 0.0569 |  | 0.0596 |
|  | 95% | - | - | - | - | - | - | - | - | - | - | 0.0580 |  | 0.0445 |  | 0.0408 |  | 0.0383 |  | 0.0378 |
|  | Mean | - | - | - | - | - | - | - | - | - | - | 0.4036 |  | 0.1945 |  | 0.1692 |  | 0.1327 |  | 0.1598 |
| RU |  |  | - | - | - | - | - | - | - | - | - | 0.80 |  | 0.76 |  | 0.73 |  | 0.68 |  | 0.73 |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  | 2.1 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 10ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 120km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | - | - | - | - | - | 0.41 |  | 0.54 |  | 0.64 |  | 0.58 |  | 0.41 |
|  | 50% | - | - | - | - | - | - | - | - | - | - | 2.68 |  | 5.32 |  | 6.31 |  | 6.44 |  | 5.32 |
|  | 95% | - | - | - | - | - | - | - | - | - | - | 10.13 |  | 15.09 |  | 16.41 |  | 17.28 |  | 15.36 |
|  | Mean | - | - | - | - | - | - | - | - | - | - | 3.46 |  | 6.80 |  | 7.45 |  | 7.62 |  | 6.36 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | - | - | - | - | - | 1.9160 |  | 1.4705 |  | 1.2508 |  | 1.3707 |  | 1.9516 |
|  | 50% | - | - | - | - | - | - | - | - | - | - | 0.2980 |  | 0.1500 |  | 0.1268 |  | 0.1241 |  | 0.1505 |
|  | 95% | - | - | - | - | - | - | - | - | - | - | 0.0790 |  | 0.0525 |  | 0.0488 |  | 0.0461 |  | 0.0520 |
|  | Mean | - | - | - | - | - | - | - | - | - | - | 0.5525 |  | 0.3455 |  | 0.3215 |  | 0.3322 |  | 0.4287 |
| RU |  |  | - | - | - | - | - | - | - | - | - | 0.84 |  | 0.80 |  | 0.81 |  | 0.81 |  | 0.89 |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  | 2.1 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 10ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 120km/h, 10M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-21.43%, 7OS-23.14%, 3/4OS-26.57%, 2OS-31.71%, 1OS-42%Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 12.004 | 15.697 | 17.780 | - | - | 9.543 | 11.298 | 12.037 | - | - | 0.905 |  | 1.068 |  | 1.231 |  | - |  | - |
|  | 50% | 12.812 | 17.220 | 19.441 | - | - | 12.407 | 15.306 | 17.103 | - | - | 9.098 |  | 10.957 |  | 11.536 |  | - |  | - |
|  | 95% | 13.302 | 18.385 | 20.620 | - | - | 13.732 | 18.967 | 21.064 | - | - | 13.715 |  | 18.318 |  | 20.289 |  | - |  | - |
|  | Mean | 12.779 | 17.061 | 19.419 | - | - | 12.112 | 15.304 | 16.891 | - | - | 8.123 |  | 9.787 |  | 10.506 |  | - |  | - |
| DL: Delay CDF [s] | 5% | 0.067 | 0.050 | 0.043 | - | - | 0.065 | 0.047 | 0.042 | - | - | 0.064 |  | 0.047 |  | 0.043 |  | - |  | - |
|  | 50% | 0.072 | 0.054 | 0.046 | - | - | 0.075 | 0.061 | 0.054 | - | - | 0.122 |  | 0.106 |  | 0.110 |  | - |  | - |
|  | 95% | 0.079 | 0.060 | 0.051 | - | - | 0.105 | 0.091 | 0.084 | - | - | 1.537 |  | 1.297 |  | 1.294 |  | - |  | - |
|  | Mean | 0.073 | 0.054 | 0.047 | - | - | 0.079 | 0.064 | 0.059 | - | - | 0.333 |  | 0.329 |  | 0.336 |  | - |  | - |
| RU |  | 19.702 | 19.603 | 19.824 | - | - | 42.987 | 43.410 | 43.998 | - | - | 65.017 |  | 65.609 |  | 66.183 |  | - |  | - |
| 𝜆 |  | 5 |  |  |  |  | 10 |  |  |  |  | 15 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 12, R1-163798, CN delay=6ms, UL access delay = 13TTI, FTP traffic model 3 Total control overhead per subframe compared to all available REs in the sTTIs: 14 OS:15%, 7 OS: 17%, 3/4 OS: 19% Maximum number of UEs that can be scheduled per sTTI: not modelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] |  |  |  | 5% |  |  |  | 2.16 |  |  |  | 2.39 |  |  |  | 2.27 |  |  | 1.40 |  |  |  |  | 1.60 |  | 1.29 |  | 0.86 |  | 0.57 |  |  |  | 1.13 |  | 0.66 |  | 0.54 |  | 0.43 |  |  |
|  |  |  |  | 50% |  |  |  | 2.91 |  |  |  | 3.80 |  |  |  | 4.15 |  |  | 3.84 |  |  |  |  | 2.84 |  | 3.33 |  | 3.09 |  | 2.37 |  |  |  | 2.71 |  | 2.55 |  | 2.29 |  | 1.77 |  |  |
|  |  |  |  | 95% |  |  |  | 7.37 |  |  |  | 7.87 |  |  |  | 8.34 |  |  | 8.37 |  |  |  |  | 7.64 |  | 7.32 |  | 7.22 |  | 6.57 |  |  |  | 6.93 |  | 6.52 |  | 6.77 |  | 5.65 |  |  |
|  |  |  |  | Mean |  |  |  | 3.82 |  |  |  | 4.42 |  |  |  | 4.59 |  |  | 4.11 |  |  |  |  | 3.67 |  | 3.63 |  | 3.40 |  | 2.74 |  |  |  | 3.31 |  | 2.88 |  | 2.74 |  | 2.20 |  |  |
| DL: Delay CDF [s] |  |  |  | 5% |  |  |  | 0.17 |  |  |  | 0.14 |  |  |  | 0.13 |  |  | 0.13 |  |  |  |  | 0.16 |  | 0.14 |  | 0.14 |  | 0.15 |  |  |  | 0.17 |  | 0.16 |  | 0.14 |  | 0.17 |  |  |
|  |  |  |  | 50% |  |  |  | 0.28 |  |  |  | 0.21 |  |  |  | 0.20 |  |  | 0.22 |  |  |  |  | 0.29 |  | 0.25 |  | 0.28 |  | 0.37 |  |  |  | 0.31 |  | 0.34 |  | 0.39 |  | 0.53 |  |  |
|  |  |  |  | 95% |  |  |  | 0.38 |  |  |  | 0.36 |  |  |  | 0.38 |  |  | 0.67 |  |  |  |  | 0.52 |  | 0.73 |  | 1.07 |  | 1.58 |  |  |  | 0.78 |  | 1.41 |  | 1.64 |  | 2.02 |  |  |
|  |  |  |  | Mean |  |  |  | 0.28 |  |  |  | 0.23 |  |  |  | 0.23 |  |  | 0.30 |  |  |  |  | 0.31 |  | 0.33 |  | 0.40 |  | 0.56 |  |  |  | 0.37 |  | 0.49 |  | 0.58 |  | 0.75 |  |  |
| RU |  |  |  |  |  |  |  | 20.00 |  |  |  | 28.87 |  |  |  | 30.01 |  |  | 31.18 |  |  |  |  | 40.00 |  | 54.28 |  | 57.28 |  | 54.55 |  |  |  | 55.00 |  | 65.87 |  | 68.30 |  | 62.00 |  |  |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 8, R1-165301, CN delay = 10ms, macro scenario, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI), DMRS with 6 REs/PRB and TTI for short TTI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] |  | 5% |  |  |  | 2.16 |  |  |  | 2.39 |  |  | 2.27 |  |  |  | 2.19 |  |  |  |  |  |  | 1.60 |  | 1.29 |  | 0.86 |  | 0.98 |  |  |  | 1.13 |  | 0.66 |  | 0.54 |  | 0.58 |  |  |  |
|  |  | 50% |  |  |  | 2.91 |  |  |  | 3.80 |  |  | 4.15 |  |  |  | 4.47 |  |  |  |  |  |  | 2.84 |  | 3.33 |  | 3.09 |  | 3.30 |  |  |  | 2.71 |  | 2.55 |  | 2.29 |  | 2.40 |  |  |  |
|  |  | 95% |  |  |  | 7.37 |  |  |  | 7.87 |  |  | 8.34 |  |  |  | 9.18 |  |  |  |  |  |  | 7.64 |  | 7.32 |  | 7.22 |  | 7.83 |  |  |  | 6.93 |  | 6.52 |  | 6.77 |  | 6.87 |  |  |  |
|  |  | Mean |  |  |  | 3.82 |  |  |  | 4.42 |  |  | 4.59 |  |  |  | 4.91 |  |  |  |  |  |  | 3.67 |  | 3.63 |  | 3.40 |  | 3.62 |  |  |  | 3.31 |  | 2.88 |  | 2.74 |  | 2.81 |  |  |  |
| DL: Delay CDF [s] |  | 5% |  |  |  | 0.17 |  |  |  | 0.14 |  |  | 0.13 |  |  |  | 0.12 |  |  |  |  |  |  | 0.16 |  | 0.14 |  | 0.14 |  | 0.13 |  |  |  | 0.17 |  | 0.16 |  | 0.14 |  | 0.15 |  |  |  |
|  |  | 50% |  |  |  | 0.28 |  |  |  | 0.21 |  |  | 0.20 |  |  |  | 0.19 |  |  |  |  |  |  | 0.29 |  | 0.25 |  | 0.28 |  | 0.26 |  |  |  | 0.31 |  | 0.34 |  | 0.39 |  | 0.37 |  |  |  |
|  |  | 95% |  |  |  | 0.38 |  |  |  | 0.36 |  |  | 0.38 |  |  |  | 0.41 |  |  |  |  |  |  | 0.52 |  | 0.73 |  | 1.07 |  | 0.90 |  |  |  | 0.78 |  | 1.41 |  | 1.64 |  | 1.54 |  |  |  |
|  |  | Mean |  |  |  | 0.28 |  |  |  | 0.23 |  |  | 0.23 |  |  |  | 0.23 |  |  |  |  |  |  | 0.31 |  | 0.33 |  | 0.40 |  | 0.37 |  |  |  | 0.37 |  | 0.49 |  | 0.58 |  | 0.55 |  |  |  |
| RU |  |  |  |  |  | 20.00 |  |  |  | 28.87 |  |  | 30.01 |  |  |  | 24.25 |  |  |  |  |  |  | 40.00 |  | 54.28 |  | 57.28 |  | 44.85 |  |  |  | 55.00 |  | 65.87 |  | 68.30 |  | 57.06 |  |  |  |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 8, R1-165301, CN delay = 10ms, macro scenario, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI), DMRS with 6 REs/PRB and TTI for short TTI, dynamic DMRS insertion for 2 symbol TTI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] |  | 5% |  |  |  | 2.16 |  |  |  | 2.39 |  |  | 2.34 |  |  |  | 1.77 |  |  |  |  |  |  | 1.60 |  | 1.29 |  | 1.15 |  | 0.82 |  |  |  | 1.13 |  | 0.66 |  | 0.63 |  | 0.55 |  |  |  |
|  |  | 50% |  |  |  | 2.91 |  |  |  | 3.80 |  |  | 4.27 |  |  |  | 4.37 |  |  |  |  |  |  | 2.84 |  | 3.33 |  | 3.54 |  | 3.15 |  |  |  | 2.71 |  | 2.55 |  | 2.48 |  | 2.33 |  |  |  |
|  |  | 95% |  |  |  | 7.37 |  |  |  | 7.87 |  |  | 8.73 |  |  |  | 9.02 |  |  |  |  |  |  | 7.64 |  | 7.32 |  | 7.44 |  | 7.62 |  |  |  | 6.93 |  | 6.52 |  | 7.14 |  | 6.69 |  |  |  |
|  |  | Mean |  |  |  | 3.82 |  |  |  | 4.42 |  |  | 4.87 |  |  |  | 4.76 |  |  |  |  |  |  | 3.67 |  | 3.63 |  | 3.76 |  | 3.45 |  |  |  | 3.31 |  | 2.88 |  | 2.93 |  | 2.74 |  |  |  |
| DL: Delay CDF [s] |  | 5% |  |  |  | 0.17 |  |  |  | 0.14 |  |  | 0.12 |  |  |  | 0.12 |  |  |  |  |  |  | 0.16 |  | 0.14 |  | 0.14 |  | 0.14 |  |  |  | 0.17 |  | 0.16 |  | 0.14 |  | 0.14 |  |  |  |
|  |  | 50% |  |  |  | 0.28 |  |  |  | 0.21 |  |  | 0.19 |  |  |  | 0.19 |  |  |  |  |  |  | 0.29 |  | 0.25 |  | 0.24 |  | 0.28 |  |  |  | 0.31 |  | 0.34 |  | 0.36 |  | 0.38 |  |  |  |
|  |  | 95% |  |  |  | 0.38 |  |  |  | 0.36 |  |  | 0.40 |  |  |  | 0.50 |  |  |  |  |  |  | 0.52 |  | 0.73 |  | 0.78 |  | 1.15 |  |  |  | 0.78 |  | 1.41 |  | 1.43 |  | 1.59 |  |  |  |
|  |  | Mean |  |  |  | 0.28 |  |  |  | 0.23 |  |  | 0.23 |  |  |  | 0.25 |  |  |  |  |  |  | 0.31 |  | 0.33 |  | 0.33 |  | 0.41 |  |  |  | 0.37 |  | 0.49 |  | 0.52 |  | 0.56 |  |  |  |
| RU |  |  |  |  |  | 20.00 |  |  |  | 28.87 |  |  | 27.78 |  |  |  | 25.12 |  |  |  |  |  |  | 40.00 |  | 54.28 |  | 51.72 |  | 46.55 |  |  |  | 55.00 |  | 65.87 |  | 66.24 |  | 56.88 |  |  |  |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 8, R1-165301, CN delay = 10ms, macro scenario, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI), DMRS with 6 REs/PRB and TTI for slot based TTI, , DMRS with 4 REs/PRB and TTI for 2 and 4 symbol TTI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] |  | 5% |  |  |  | 2.16 |  |  |  | 2.39 |  |  | 2.34 |  |  |  | 2.22 |  |  |  |  |  |  | 1.60 |  | 1.29 |  | 1.15 |  | 1.16 |  |  |  | 1.13 |  | 0.66 |  | 0.63 |  | 0.70 |  |  |  |
|  |  | 50% |  |  |  | 2.91 |  |  |  | 3.80 |  |  | 4.27 |  |  |  | 4.57 |  |  |  |  |  |  | 2.84 |  | 3.33 |  | 3.54 |  | 3.77 |  |  |  | 2.71 |  | 2.55 |  | 2.48 |  | 2.75 |  |  |  |
|  |  | 95% |  |  |  | 7.37 |  |  |  | 7.87 |  |  | 8.73 |  |  |  | 9.32 |  |  |  |  |  |  | 7.64 |  | 7.32 |  | 7.44 |  | 8.56 |  |  |  | 6.93 |  | 6.52 |  | 7.14 |  | 7.80 |  |  |  |
|  |  | Mean |  |  |  | 3.82 |  |  |  | 4.42 |  |  | 4.87 |  |  |  | 5.08 |  |  |  |  |  |  | 3.67 |  | 3.63 |  | 3.76 |  | 4.03 |  |  |  | 3.31 |  | 2.88 |  | 2.93 |  | 3.16 |  |  |  |
| DL: Delay CDF [s] |  | 5% |  |  |  | 0.17 |  |  |  | 0.14 |  |  | 0.12 |  |  |  | 0.12 |  |  |  |  |  |  | 0.16 |  | 0.14 |  | 0.14 |  | 0.13 |  |  |  | 0.17 |  | 0.16 |  | 0.14 |  | 0.13 |  |  |  |
|  |  | 50% |  |  |  | 0.28 |  |  |  | 0.21 |  |  | 0.19 |  |  |  | 0.18 |  |  |  |  |  |  | 0.29 |  | 0.25 |  | 0.24 |  | 0.23 |  |  |  | 0.31 |  | 0.34 |  | 0.36 |  | 0.32 |  |  |  |
|  |  | 95% |  |  |  | 0.38 |  |  |  | 0.36 |  |  | 0.40 |  |  |  | 0.39 |  |  |  |  |  |  | 0.52 |  | 0.73 |  | 0.78 |  | 0.79 |  |  |  | 0.78 |  | 1.41 |  | 1.43 |  | 1.28 |  |  |  |
|  |  | Mean |  |  |  | 0.28 |  |  |  | 0.23 |  |  | 0.23 |  |  |  | 0.22 |  |  |  |  |  |  | 0.31 |  | 0.33 |  | 0.33 |  | 0.33 |  |  |  | 0.37 |  | 0.49 |  | 0.52 |  | 0.47 |  |  |  |
| RU |  |  |  |  |  | 20.00 |  |  |  | 28.87 |  |  | 27.78 |  |  |  | 22.33 |  |  |  |  |  |  | 40.00 |  | 54.28 |  | 51.72 |  | 41.82 |  |  |  | 55.00 |  | 65.87 |  | 66.24 |  | 53.82 |  |  |  |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 8, R1-165301, CN delay = 10ms, macro scenario, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI), DMRS with 6 REs/PRB and TTI for slot based TTI, DMRS with 4 REs/PRB and TTI for 2 and 4 symbol TTI , dynamic DMRS insertion for 2 symbol TTI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] |  | 5% |  |  |  | 3.09 |  |  |  | 3.56 |  |  | 3.80 |  |  |  | 3.87 |  |  |  |  |  |  | 2.13 |  | 2.22 |  | 2.52 |  | 2.23 |  |  |  | 0.94 |  | 1.33 |  | 1.36 |  | 1.39 |  |  |  |
|  |  | 50% |  |  |  | 5.48 |  |  |  | 6.57 |  |  | 7.43 |  |  |  | 7.69 |  |  |  |  |  |  | 4.83 |  | 5.41 |  | 6.12 |  | 5.95 |  |  |  | 3.84 |  | 4.41 |  | 4.66 |  | 4.65 |  |  |  |
|  |  | 95% |  |  |  | 7.24 |  |  |  | 9.96 |  |  | 11.84 |  |  |  | 12.43 |  |  |  |  |  |  | 7.15 |  | 9.48 |  | 11.09 |  | 11.48 |  |  |  | 6.82 |  | 9.11 |  | 10.09 |  | 10.48 |  |  |  |
|  |  | Mean |  |  |  | 5.38 |  |  |  | 6.73 |  |  | 7.56 |  |  |  | 7.91 |  |  |  |  |  |  | 4.78 |  | 5.66 |  | 6.44 |  | 6.32 |  |  |  | 3.97 |  | 4.76 |  | 5.12 |  | 5.14 |  |  |  |
| DL: Delay CDF [s] |  | 5% |  |  |  | 0.12 |  |  |  | 0.08 |  |  | 0.07 |  |  |  | 0.07 |  |  |  |  |  |  | 0.12 |  | 0.09 |  | 0.08 |  | 0.07 |  |  |  | 0.12 |  | 0.09 |  | 0.08 |  | 0.08 |  |  |  |
|  |  | 50% |  |  |  | 0.15 |  |  |  | 0.13 |  |  | 0.11 |  |  |  | 0.11 |  |  |  |  |  |  | 0.17 |  | 0.16 |  | 0.14 |  | 0.14 |  |  |  | 0.22 |  | 0.19 |  | 0.18 |  | 0.18 |  |  |  |
|  |  | 95% |  |  |  | 0.27 |  |  |  | 0.24 |  |  | 0.22 |  |  |  | 0.22 |  |  |  |  |  |  | 0.40 |  | 0.38 |  | 0.33 |  | 0.40 |  |  |  | 0.92 |  | 0.67 |  | 0.63 |  | 0.66 |  |  |  |
|  |  | Mean |  |  |  | 0.17 |  |  |  | 0.14 |  |  | 0.13 |  |  |  | 0.12 |  |  |  |  |  |  | 0.22 |  | 0.20 |  | 0.17 |  | 0.18 |  |  |  | 0.31 |  | 0.26 |  | 0.25 |  | 0.25 |  |  |  |
| RU |  |  |  |  |  | 14.4 |  |  |  | 14.6 |  |  | 14.4 |  |  |  | 14 |  |  |  |  |  |  | 35.4 |  | 36.2 |  | 33.8 |  | 34.4 |  |  |  | 59.6 |  | 54.4 |  | 53.6 |  | 54.6 |  |  |  |
| 𝜆 |  | 0.20 |  |  |  |  | 0.40 |  |  |  |  | 0.56 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 7, R1-165478, File size: 100 KB, Macro scenario, 6 ms core network delay, Dynamic DCI overhead, FTP model 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.20 | 3.21 | 3.52 | 3.73 |  | 2.26 | 1.75 | 2.00 | 2.06 |  | 0.91 |  | 0.89 |  | 1.02 |  | 0.84 |  |  |
|  | 50% | 5.52 | 6.55 | 7.37 | 7.56 |  | 4.83 | 5.10 | 5.55 | 5.76 |  | 3.86 |  | 3.76 |  | 3.63 |  | 3.60 |  |  |
|  | 95% | 7.23 | 10.15 | 12.07 | 12.87 |  | 7.04 | 9.40 | 11.20 | 11.14 |  | 6.86 |  | 8.84 |  | 9.69 |  | 9.75 |  |  |
|  | Mean | 5.43 | 6.58 | 7.55 | 7.85 |  | 4.84 | 5.36 | 5.99 | 6.08 |  | 3.96 |  | 4.22 |  | 4.29 |  | 4.13 |  |  |
| DL: Delay CDF [s] | 5% | 0.12 | 0.08 | 0.07 | 0.07 |  | 0.12 | 0.09 | 0.08 | 0.08 |  | 0.12 |  | 0.10 |  | 0.09 |  | 0.09 |  |  |
|  | 50% | 0.15 | 0.13 | 0.11 | 0.11 |  | 0.17 | 0.17 | 0.15 | 0.15 |  | 0.22 |  | 0.22 |  | 0.24 |  | 0.24 |  |  |
|  | 95% | 0.26 | 0.26 | 0.24 | 0.23 |  | 0.38 | 0.50 | 0.43 | 0.42 |  | 0.93 |  | 1.01 |  | 0.86 |  | 1.03 |  |  |
|  | Mean | 0.17 | 0.15 | 0.13 | 0.12 |  | 0.21 | 0.22 | 0.20 | 0.20 |  | 0.31 |  | 0.34 |  | 0.33 |  | 0.36 |  |  |
| RU |  | 14 | 16 | 14.8 | 14 |  | 34 | 40.2 | 37.4 | 39 |  | 59.4 |  | 61.6 |  | 62.2 |  | 63.4 |  |  |
| 𝜆 |  | 0.20 |  |  |  |  | 0.40 |  |  |  |  | 0.56 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 7, R1-165478, File size: 100 KB, Macro scenario, 6 ms core network delay, static DCI overhead, FTP model 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.66 | 2.58 | 2.79 | 3.17 |  | 1.84 | 1.83 | 1.7 | 1.78 |  | 1.04 |  | 0.96 |  | 0.75 |  | 0.79 |  |  |
|  | 50% | 4.99 | 6.02 | 6.92 | 6.45 |  | 4.29 | 4.66 | 4.62 | 4.65 |  | 2.82 |  | 2.54 |  | 2.3 |  | 2.63 |  |  |
|  | 95% | 7.01 | 9.64 | 11.63 | 11.75 |  | 6.79 | 9.28 | 10.38 | 10.47 |  | 6.2 |  | 7.5 |  | 7.22 |  | 8.06 |  |  |
|  | Mean | 4.96 | 6.06 | 6.94 | 6.92 |  | 4.31 | 5.05 | 5.23 | 5.25 |  | 3.15 |  | 3.2 |  | 2.96 |  | 3.26 |  |  |
| DL: Delay CDF [s] | 5% | 0.12 | 0.087 | 0.072 | 0.071 |  | 0.124 | 0.091 | 0.081 | 0.08 |  | 0.135 |  | 0.112 |  | 0.116 |  | 0.105 |  |  |
|  | 50% | 0.168 | 0.139 | 0.121 | 0.13 |  | 0.195 | 0.179 | 0.181 | 0.179 |  | 0.297 |  | 0.328 |  | 0.358 |  | 0.318 |  |  |
|  | 95% | 0.316 | 0.325 | 0.307 | 0.268 |  | 0.457 | 0.461 | 0.494 | 0.472 |  | 0.803 |  | 0.878 |  | 1.114 |  | 1.062 |  |  |
|  | Mean | 0.191 | 0.165 | 0.147 | 0.144 |  | 0.237 | 0.218 | 0.22 | 0.212 |  | 0.361 |  | 0.386 |  | 0.467 |  | 0.418 |  |  |
| RU |  | 23 | 24 | 23.5 | 24.5 |  | 43 | 44 | 47.5 | 46.5 |  | 76.5 |  | 80 |  | 82 |  | 79 |  |  |
| 𝜆 |  | 0.33 |  |  |  |  | 0.59 |  |  |  |  | 1.25 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 7, R1-165478, File size: 100 KB, Macro Scenario, 6 ms core network delay, static DCI overhead, FTP model 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.79 | 2.96 | 3.33 | 3.34 |  | 2.07 | 2.16 | 2.25 | 2.22 |  | 1.05 |  | 1.04 |  | 1.01 |  | 0.88 |  |  |
|  | 50% | 5.09 | 6.02 | 6.75 | 7 |  | 4.37 | 4.89 | 5.35 | 5.27 |  | 2.95 |  | 2.88 |  | 3.01 |  | 2.71 |  |  |
|  | 95% | 7.03 | 9.74 | 11.52 | 11.85 |  | 6.84 | 9.2 | 10.45 | 10.69 |  | 6.18 |  | 7.67 |  | 8.13 |  | 7.59 |  |  |
|  | Mean | 5.07 | 6.2 | 7.06 | 7.28 |  | 4.44 | 5.23 | 5.78 | 5.73 |  | 3.22 |  | 3.39 |  | 3.56 |  | 3.4 |  |  |
| DL: Delay CDF [s] | 5% | 0.119 | 0.086 | 0.073 | 0.071 |  | 0.123 | 0.091 | 0.08 | 0.078 |  | 0.136 |  | 0.109 |  | 0.103 |  | 0.111 |  |  |
|  | 50% | 0.164 | 0.139 | 0.124 | 0.119 |  | 0.191 | 0.171 | 0.157 | 0.159 |  | 0.283 |  | 0.29 |  | 0.278 |  | 0.307 |  |  |
|  | 95% | 0.303 | 0.287 | 0.253 | 0.252 |  | 0.408 | 0.39 | 0.376 | 0.382 |  | 0.803 |  | 0.807 |  | 0.833 |  | 0.951 |  |  |
|  | Mean | 0.183 | 0.156 | 0.138 | 0.135 |  | 0.221 | 0.198 | 0.183 | 0.187 |  | 0.357 |  | 0.36 |  | 0.354 |  | 0.391 |  |  |
| RU |  | 22.4 | 22.6 | 23 | 22.6 |  | 39.6 | 41 | 40.4 | 42.8 |  | 72.4 |  | 76.2 |  | 76 |  | 78.4 |  |  |
| 𝜆 |  | 0.33 |  |  |  |  | 0.59 |  |  |  |  | 1.25 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 7, R1-165478, File size: 100 KB, Macro Scenario, 6 ms core network delay, Dynamic DCI overhead, FTP model 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.1.1-3: Results with 500kB file transfer

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 6.1755 (0) | 6.6431 (+7.6) | 6.4134 (+3.9) | 6.2805 (+1.7) | - | 4.9637 (0) | 4.7906 (-3.5) | 4.6274 (-6.8) | 4.5995 (-7.3) | - | 4.0796 (0) |  | 4.0747 (-0.1) |  | 3.6656 (-10.1) |  | 3.611 (-11.5) |  | - |
|  | 50% | 12.3475 (0) | 15.973 +29.4 | 17.1728 (+39.1) | 17.9773 (+45.6) | - | 11.8287 (0) | 14.4381 (+22.1) | 14.9479 (+26.4) | 15.4826 (+30.9) | - | 11.006 (0) |  | 13.3863 (+21.6) |  | 13.2407 (+20.3) |  | 13.1736 (+19.7) |  | - |
|  | 95% | 13.367 (0) | 17.900 +33.9 | 20.799 (+55.6) | 23.055 (+72.5) | - | 13.2732 (0) | 17.6868 (+33.3) | 20.11 (+51.5) | 22.6524 (+70.7) | - | 13.146 (0) |  | 17.5797 (+33.7) |  | 18.6976 (+42.2) |  | 20.4244 (+55.4) |  | - |
|  | Mean | 11.5033 (0) | 14.521 +26.2 | 15.7811 (+37.2) | 16.4798 (+43.3) | - | 10.7787 (0) | 13.2551 (+23) | 14.1738 (+31.5) | 14.7304 (+36.7) | - | 10.1084 (0) |  | 12.3852 (+22.5) |  | 12.5285 (+23.9) |  | 12.7193 (+25.8) |  | - |
| DL: Delay CDF [s] | 5% | 0.314 (0) | 0.2343 (-25.4) | 0.2016 (-35.8) | 0.1812 (-42.3) | - | 0.3153 (0) | 0.2369 (-24.9) | 0.2026 (-35.7) | 0.1852 (-42.2) | - | 0.3193 (0) |  | 0.238 (-25.5) |  | 0.2133 (-33.2) |  | 0.2081 (-34.8) |  | - |
|  | 50% | 0.3422 (0) | 0.2726 (-20.3) | 0.2505 (-26.8) | 0.2484 (-27.4) | - | 0.3613 (0) | 0.3015 (-16.6) | 0.2992 (-17.2) | 0.2986 (-17.4) | - | 0.3953 (0) |  | 0.3295 (-16.7) |  | 0.3395 (-14.1) |  | 0.3689 (-6.7) |  | - |
|  | 95% | 0.6318 (0) | 0.5902 (-6.6) | 0.6232 (-1.4) | 0.6504 (+2.9) | - | 0.845 (0) | 0.9672 (+14.5) | 0.9798 (+16) | 0.9248 (+9.4) | - | 1.131 (0) |  | 1.184 (+4.7) |  | 1.1743 (+3.8) |  | 1.2159 (+7.5) |  | - |
|  | Mean | 0.4121 (0) | 0.3416 (-17.1) | 0.3239 (-21.4) | 0.341 (-17.3) | - | 0.4647 (0) | 0.415 (-10.7) | 0.4126 (-11.2) | 0.4101 (-11.8) | - | 0.5235 (0) |  | 0.466 (-11) |  | 0.4844 (-7.5) |  | 0.5195 (-0.8) |  | - |
| RU |  | 24.7 | 24.5 | 25.2 | 27.7 | - | 34.6 | 36.5 | 37.1 | 38.7 | - | 47.3 |  | 44.4 |  | 48.6 |  | 50.4 |  |  |
| 𝜆 |  | 0.14 |  |  |  |  | 0.2 |  |  |  |  | 0.25 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 1, R1-162398, 6ms core network delay, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-21.43%, 7OS-23.14%, 3/4OS-26.57%, 2OS-31.71%, Maximum number of UEs that can be scheduled per sTTI: 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.68 | 2.70 1% | 2.55 -5% | 2.28 -15% | 0.46 -83% | 2.22 | 2.21 -0.5% | 1.97 -11% | 1.53 -31% | 0.11 -95% | 1.53 |  | 1.55 1% |  | 1.23 -20% |  | 0.90 -41% |  | 0.03 -98% |
|  | 50% | 6.99 | 8.39 20% | 9.16 31% | 9.25 32% | 7.86 12% | 5.94 | 7.08 19% | 7.58 28% | 7.22 22% | 5.51 -7% | 4.88 |  | 5.62 15% |  | 5.68 16% |  | 5.26 8% |  | 3.41 -30% |
|  | 95% | 9.57 | 11.69 22% | 12.91 35% | 13.44 40% | 13.61 42% | 8.78 | 10.65 21% | 11.80 34% | 11.93 36% | 12.06 37% | 8.02 |  | 9.61 20% |  | 10.50 31% |  | 10.52 31% |  | 10.71 34% |
|  | Mean | 6.68 | 7.95 19% | 8.53 28% | 8.63 29% | 7.47 12% | 5.77 | 6.83 18% | 7.25 26% | 7.02 22% | 5.74 -1% | 4.82 |  | 5.57 16% |  | 5.71 18% |  | 5.42 12% |  | 4.19 -13% |
| DL: Delay CDF [s] | 5% | 0.40 | 0.33 -18% | 0.30 -25% | 0.29 -28% | 0.29 -29% | 0.43 | 0.36 -18% | 0.32 -25% | 0.32 -27% | 0.31 -29% | 0.47 |  | 0.39 -17% |  | 0.35 -25% |  | 0.34 -26% |  | 0.32 -31% |
|  | 50% | 0.57 | 0.47 -17% | 0.44 -23% | 0.43 -23% | 0.47 -18% | 0.66 | 0.55 -16% | 0.52 -21% | 0.54 -18% | 0.59 -11% | 0.78 |  | 0.68 -13% |  | 0.66 -15% |  | 0.70 -11% |  | 0.74 -5% |
|  | 95% | 1.38 | 1.31 -5% | 1.42 2% | 1.58 14% | 2.98 116% | 1.75 | 1.69 -3% | 1.86 6% | 2.21 26% | 4.32 146% | 2.38 |  | 2.34 -2% |  | 2.64 11% |  | 3.15 32% |  | 5.93 149% |
|  | Mean | 0.70 | 0.61 -12% | 0.61 -13% | 0.63 -9% | 0.92 32% | 0.84 | 0.74 -11% | 0.75 -11% | 0.83 -1% | 1.25 49% | 1.04 |  | 0.95 -9% |  | 1.00 -4% |  | 1.11 7% |  | 1.64 58% |
| RU |  | 0.2 | 0.20 | 0.21 | 0.20 | 0.20 | 0.40 | 0.39 | 0.40 | 0.39 | 0.32 | 0.61 |  | 0.61 |  | 0.62 |  | 0.56 |  | 0.40 |
| 𝜆 |  | 0.08 |  |  |  |  | 0.15 |  |  |  |  | 0.23 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 2, R1-162536, 10ms core network delay, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI), 8 sTTI DL HARQ timing |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] |  | 5% |  |  |  | 2.893 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 1.563 |  |  |  |  |  |  |
|  |  | 50% |  |  |  | 9.33 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6.06 |  |  |  |  |  |  |
|  |  | 95% |  |  |  | 13.01 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.69 |  |  |  |  |  |  |
|  |  | Mean |  |  |  | 8.76 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 6.04 |  |  |  |  |  |  |
| DL: Delay CDF [s] |  | 5% |  |  |  | 0.301 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.347 |  |  |  |  |  |  |
|  |  | 50% |  |  |  | 0.420 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.632 |  |  |  |  |  |  |
|  |  | 95% |  |  |  | 1.270 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2.235 |  |  |  |  |  |  |
|  |  | Mean |  |  |  | 0.569 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.893 |  |  |  |  |  |  |
| RU |  |  |  |  |  | 19.72 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 61.26 |  |  |  |  |  |  |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 2, R1-164912, 10ms core network delay, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI), 4 sTTI DL HARQ timing and 7 sTTI TCP ACK latency |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 4.03 | 4.92 | 3.67 | 2.20 | - | 2.79 | 2.55 | 2.07 | 0.92 | - | 1.66 |  | 1.49 |  | 0.80 |  | 0.46 |  | - |
|  | 50% | 11.03 | 13.50 | 12.25 | 11.89 | - | 8.38 | 9.50 | 8.62 | 6.10 | - | 6.39 |  | 6.43 |  | 4.42 |  | 3.21 |  | - |
|  | 95% | 15.92 | 22.58 | 23.92 | 27.63 | - | 15.81 | 20.65 | 21.64 | 21.70 | - | 14.99 |  | 18.50 |  | 16.31 |  | 13.91 |  | - |
|  | Mean | 10.66 | 13.53 | 13.11 | 12.85 | - | 8.98 | 10.33 | 9.95 | 8.09 | - | 7.22 |  | 7.69 |  | 5.85 |  | 4.60 |  | - |
| DL: Delay CDF [s] | 5% | 0.251 | 0.177 | 0.167 | 0.145 | - | 0.253 | 0.193 | 0.184 | 0.180 | - | 0.267 |  | 0.219 |  | 0.241 |  | 0.287 |  | - |
|  | 50% | 0.360 | 0.295 | 0.324 | 0.331 | - | 0.483 | 0.429 | 0.464 | 0.619 | - | 0.632 |  | 0.638 |  | 0.902 |  | 1.178 |  | - |
|  | 95% | 0.991 | 0.752 | 1.002 | 1.728 | - | 1.506 | 1.692 | 1.965 | 3.753 | - | 2.558 |  | 2.833 |  | 4.857 |  | 7.750 |  | - |
|  | Mean | 0.454 | 0.371 | 0.424 | 0.576 | - | 0.611 | 0.620 | 0.711 | 1.080 | - | 0.929 |  | 0.997 |  | 1.546 |  | 2.130 |  | - |
| RU |  | 0.2 | 0.19 | 0.22 | 0.23 |  | 0.41 | 0.41 | 0.44 | 0.46 | - | 0.62 |  | 0.61 |  | 0.7 |  | 0.64 |  | - |
| 𝜆 |  | 0.08 |  |  |  |  | 0.15 |  |  |  |  | 0.24 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 4, R1-162503, CN delay = 10ms, CQI period = 5TTI, CQI delay = 6TTI FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.79 | 4.01 | 3.71 | 3.38 | 3.35 | 3.31 | 3.5 | 3.04 | 2.68 | 2.41 |  | 2.54 |  | 2.78 |  | 2.19 |  | 1.98 | 1.82 |
|  | 50% | 10.26 | 12.16 | 11.78 | 11.14 | 10.96 | 9.57 | 11.43 | 10.79 | 10 | 9.22 |  | 8.73 |  | 10.24 |  | 9.25 |  | 8.07 | 7.04 |
|  | 95% | 12.27 | 16.60 | 20.07 | 21.98 | 23.20 | 12.23 | 16.56 | 19.87 | 21.42 | 22.41 |  | 12.09 |  | 16.43 |  | 19.68 |  | 20.73 | 21.53 |
|  | Mean | 9.29 | 11.51 | 11.88 | 11.65 | 11.72 | 8.75 | 10.93 | 11.11 | 10.76 | 10.59 |  | 8.16 |  | 10.09 |  | 9.91 |  | 9.40 | 8.83 |
| DL: Delay CDF [s] | 5% | 0.325 | 0.240 | 0.199 | 0.182 | 0.172 | 0.326 | 0.241 | 0.201 | 0.187 | 0.178 |  | 0.331 |  | 0.243 |  | 0.203 |  | 0.192 | 0.186 |
|  | 50% | 0.389 | 0.329 | 0.343 | 0.363 | 0.365 | 0.424 | 0.354 | 0.377 | 0.411 | 0.441 |  | 0.470 |  | 0.406 |  | 0.456 |  | 0.519 | 0.605 |
|  | 95% | 1.051 | 0.998 | 1.079 | 1.182 | 1.223 | 1.243 | 1.156 | 1.318 | 1.538 | 1.664 |  | 1.664 |  | 1.500 |  | 1.920 |  | 2.026 | 2.274 |
|  | Mean | 0.507 | 0.448 | 0.458 | 0.486 | 0.494 | 0.57 | 0.492 | 0.526 | 0.583 | 0.615 |  | 0.664 |  | 0.585 |  | 0.667 |  | 0.742 | 0.816 |
| RU |  | 0.2 | 0.19 | 0.21 | 0.23 | 0.24 | 0.4 | 0.39 | 0.42 | 0.46 | 0.47 |  | 0.59 |  | 0.57 |  | 0.62 |  | 0.66 | 0.68 |
| 𝜆 |  | 0.07 |  |  |  |  | 0.13 |  |  |  |  |  | 0.14 |  |  |  |  |  |  |  |
| Notes: |  | Source 6, R1-162703, CN delay = 6ms, 8 TTIs HARQ RTT, 13-TTI UL access delay, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.3%, 7OS-28.0%, 3/4OS-35.9%, 2OS-38.3%, 1OS-38.3%, Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-4UEs, 3/4OS-3UEs, 2OS-2UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.79 | 3.99 | 3.64 | 3.29 | 3.06 | 3.31 | 3.54 | 2.96 | 2.68 | 2.4 |  | 2.54 |  | 2.61 |  | 2.19 |  | 1.96 | 1.69 |
|  | 50% | 10.26 | 12.03 | 11.42 | 10.65 | 10.54 | 9.57 | 11.16 | 10.46 | 9.81 | 9.09 |  | 8.73 |  | 10.02 |  | 8.99 |  | 7.93 | 7.09 |
|  | 95% | 12.27 | 16.39 | 19.23 | 21.02 | 21.75 | 12.23 | 16.36 | 19.17 | 20.22 | 20.6 |  | 12.09 |  | 16.12 |  | 18.67 |  | 19.72 | 19.79 |
|  | Mean | 9.29 | 11.31 | 11.45 | 11.20 | 11.21 | 8.75 | 10.75 | 10.71 | 10.33 | 9.96 |  | 8.16 |  | 9.86 |  | 9.54 |  | 9.06 | 8.41 |
| DL: Delay CDF [s] | 5% | 0.325 | 0.243 | 0.207 | 0.190 | 0.184 | 0.326 | 0.244 | 0.208 | 0.198 | 0.194 |  | 0.331 |  | 0.248 |  | 0.214 |  | 0.203 | 0.203 |
|  | 50% | 0.389 | 0.335 | 0.355 | 0.373 | 0.380 | 0.424 | 0.361 | 0.39 | 0.415 | 0.45 |  | 0.470 |  | 0.411 |  | 0.465 |  | 0.527 | 0.597 |
|  | 95% | 1.051 | 1.004 | 1.115 | 1.214 | 1.305 | 1.243 | 1.139 | 1.36 | 1.571 | 1.677 |  | 1.664 |  | 1.579 |  | 1.863 |  | 2.045 | 2.437 |
|  | Mean | 0.507 | 0.453 | 0.476 | 0.504 | 0.512 | 0.57 | 0.497 | 0.544 | 0.594 | 0.647 |  | 0.664 |  | 0.594 |  | 0.687 |  | 0.754 | 0.852 |
| RU |  | 0.2 | 0.19 | 0.21 | 0.23 | 0.23 | 0.4 | 0.38 | 0.42 | 0.46 | 0.47 |  | 0.59 |  | 0.57 |  | 0.63 |  | 0.66 | 0.68 |
| 𝜆 |  | 0.07 |  |  |  |  | 0.13 |  |  |  |  |  | 0.14 |  |  |  |  |  |  |  |
| Notes: |  | Source 6, R1-162704, CN delay = 6ms, 8 ms HARQ RTT, 13-TTI UL access delay, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.3%, 7OS-28.0%, 3/4OS-35.9%, 2OS-38.3%, 1OS-38.3%, Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-4UEs, 3/4OS-3UEs, 2OS-2UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.94 | 4.01 | 3.72 | 3.42 | 3.11 | 3.41 | 3.56 | 3.05 | 2.64 | 2.43 |  | 2.56 |  | 2.76 |  | 2.22 |  | 2.06 | 1.73 |
|  | 50% | 13.03 | 13.70 | 12.38 | 11.33 | 11.09 | 11.70 | 12.56 | 11.26 | 10.16 | 9.69 |  | 10.42 |  | 11.24 |  | 9.51 |  | 8.29 | 7.26 |
|  | 95% | 20.89 | 23.19 | 24.17 | 24.06 | 24.05 | 20.51 | 22.79 | 23.61 | 22.76 | 22.91 |  | 20.16 |  | 22.54 |  | 23.24 |  | 22.28 | 21.63 |
|  | Mean | 12.65 | 13.68 | 12.88 | 12.11 | 11.87 | 11.85 | 12.81 | 12.02 | 11.09 | 10.70 |  | 10.85 |  | 11.85 |  | 10.78 |  | 9.75 | 8.96 |
| DL: Delay CDF [s] | 5% | 0.191 | 0.172 | 0.165 | 0.166 | 0.166 | 0.193 | 0.175 | 0.169 | 0.175 | 0.174 |  | 0.198 |  | 0.177 |  | 0.172 |  | 0.179 | 0.185 |
|  | 50% | 0.308 | 0.292 | 0.327 | 0.351 | 0.365 | 0.345 | 0.327 | 0.367 | 0.401 | 0.421 |  | 0.403 |  | 0.374 |  | 0.435 |  | 0.509 | 0.594 |
|  | 95% | 1.006 | 0.995 | 1.070 | 1.168 | 1.244 | 1.200 | 1.142 | 1.340 | 1.515 | 1.661 |  | 1.643 |  | 1.513 |  | 1.870 |  | 2.012 | 2.332 |
|  | Mean | 0.425 | 0.404 | 0.441 | 0.480 | 0.500 | 0.490 | 0.465 | 0.511 | 0.578 | 0.617 |  | 0.585 |  | 0.547 |  | 0.645 |  | 0.743 | 0.820 |
| RU |  | 0.2 | 0.19 | 0.21 | 0.23 | 0.24 | 0.41 | 0.39 | 0.42 | 0.46 | 0.48 |  | 0.59 |  | 0.58 |  | 0.64 |  | 0.67 | 0.68 |
| 𝜆 |  | 0.07 |  |  |  |  | 0.13 |  |  |  |  |  | 0.14 |  |  |  |  |  |  |  |
| Notes: |  | Source 6, R1-162705, CN delay = 6ms, 8 TTIs HARQ RTT, 1-TTI UL access delay, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.3%, 7OS-28.0%, 3/4OS-35.9%, 2OS-38.3%, 1OS-38.3%, Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-4UEs, 3/4OS-3UEs, 2OS-2UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 4.51 | 4.30 | 4.27 | 3.74 | - | 2.36 | 1.75 | 1.42 | 1.37 | - |  | 1.42 |  | 1.12 |  | 1.02 |  | 1.03 | - |
|  | 50% | 7.24 | 8.67 | 9.98 | 10.25 | - | 6.41 | 6.17 | 6.03 | 6.65 | - |  | 5.03 |  | 4.61 |  | 4.70 |  | 4.86 | - |
|  | 95% | 11.46 | 11.82 | 13.71 | 15.48 | - | 10.49 | 11.17 | 12.61 | 14.04 | - |  | 9.31 |  | 10.47 |  | 11.93 |  | 12.78 | - |
|  | Mean | 7.40 | 8.64 | 9.84 | 10.34 | - | 6.34 | 6.28 | 6.41 | 7.09 | - |  | 5.20 |  | 5.08 |  | 5.36 |  | 5.66 | - |
| DL: Delay CDF [s] | 5% | 0.42 | 0.34 | 0.29 | 0.26 | - | 0.44 | 0.36 | 0.32 | 0.29 | - |  | 0.47 |  | 0.39 |  | 0.34 |  | 0.32 | - |
|  | 50% | 0.56 | 0.47 | 0.41 | 0.41 | - | 0.64 | 0.70 | 0.73 | 0.65 | - |  | 0.84 |  | 0.98 |  | 0.98 |  | 0.98 | - |
|  | 95% | 0.97 | 1.01 | 1.00 | 1.11 | - | 1.97 | 2.43 | 3.05 | 3.01 | - |  | 3.11 |  | 3.91 |  | 3.99 |  | 4.03 | - |
|  | Mean | 0.62 | 0.57 | 0.53 | 0.55 | - | 0.84 | 1.01 | 1.13 | 1.09 | - |  | 1.22 |  | 1.44 |  | 1.42 |  | 1.46 | - |
| RU |  | 20.00 | 24.14 | 22.92 | 18.34 | - | 40.00 | 48.78 | 49.90 | 38.86 | - |  | 55.00 |  | 61.83 |  | 60.23 |  | 48.16 | - |
| 𝜆 |  | - |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |  |  |
| Notes: |  | Source 8, R1-163312, CN delay = 10ms, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 6.50 | 9.46 | 10.90 | 12.18 | 12.33 | - | - | - | - | - | 3.22 |  | 5.04 |  | 4.86 |  | 4.91 |  | 3.74 |
|  | 50% | 8.81 | 11.51 | 13.18 | 15.32 | 16.81 | - | - | - | - | - | 8.06 |  | 11.25 |  | 13.00 |  | 14.67 |  | 14.73 |
|  | 95% | 14.39 | 19.09 | 21.86 | 23.45 | 24.37 | - | - | - | - | - | 14.04 |  | 18.56 |  | 20.93 |  | 22.58 |  | 23.49 |
|  | Mean | 9.20 | 12.91 | 15.37 | 17.56 | 18.44 | - | - | - | - | - | 8.06 |  | 12.00 |  | 13.95 |  | 15.47 |  | 15.25 |
| DL: Delay CDF [s] | 5% | 0.6120 | 0.4225 | 0.3640 | 0.3267 | 0.3241 | - | - | - | - | - | 1.2370 |  | 0.7900 |  | 0.8237 |  | 0.8087 |  | 1.0669 |
|  | 50% | 0.4540 | 0.3475 | 0.3038 | 0.2606 | 0.2371 | - | - | - | - | - | 0.4950 |  | 0.3555 |  | 0.3080 |  | 0.2720 |  | 0.2715 |
|  | 95% | 0.2760 | 0.2090 | 0.1830 | 0.1706 | 0.1641 | - | - | - | - | - | 0.2850 |  | 0.2155 |  | 0.1913 |  | 0.1771 |  | 0.1702 |
|  | Mean | 0.4559 | 0.3284 | 0.2780 | 0.2446 | 0.2337 | - | - | - | - | - | 0.6020 |  | 0.4021 |  | 0.3677 |  | 0.3406 |  | 0.3831 |
| RU |  | 0.18 | 0.16 | 0.17 | 0.18 | 0.19 |  |  |  |  |  | 0.56 |  | 0.51 |  | 0.52 |  | 0.54 |  | 0.60 |
| 𝜆 |  | 0.21 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 10ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 3km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 6.33 | 7.67 | 7.58 | 7.35 | 5.49 | - | - | - | - | - | 2.82 |  | 3.00 |  | 2.74 |  | 2.16 |  | 1.84 |
|  | 50% | 8.73 | 11.19 | 12.63 | 13.67 | 13.17 | - | - | - | - | - | 7.14 |  | 9.90 |  | 10.94 |  | 10.91 |  | 9.29 |
|  | 95% | 12.54 | 18.60 | 20.48 | 21.91 | 22.64 | - | - | - | - | - | 11.94 |  | 17.39 |  | 19.36 |  | 20.88 |  | 20.17 |
|  | Mean | 8.77 | 11.56 | 12.94 | 14.09 | 13.08 | - | - | - | - | - | 7.00 |  | 9.53 |  | 10.59 |  | 10.81 |  | 9.67 |
| DL: Delay CDF [s] | 5% | 0.6270 | 0.5185 | 0.5234 | 0.5349 | 0.7204 | - | - | - | - | - | 1.4040 |  | 1.3075 |  | 1.4290 |  | 1.7776 |  | 2.1434 |
|  | 50% | 0.4580 | 0.3575 | 0.3169 | 0.2927 | 0.3035 | - | - | - | - | - | 0.5600 |  | 0.4035 |  | 0.3638 |  | 0.3654 |  | 0.4299 |
|  | 95% | 0.3040 | 0.2120 | 0.1946 | 0.1821 | 0.1766 | - | - | - | - | - | 0.3290 |  | 0.2300 |  | 0.2063 |  | 0.1910 |  | 0.1980 |
|  | Mean | 0.4733 | 0.3662 | 0.3334 | 0.3168 | 0.3650 | - | - | - | - | - | 0.6858 |  | 0.5641 |  | 0.5323 |  | 0.5967 |  | 0.7067 |
| RU |  | 0.17 | 0.17 | 0.17 | 0.19 | 0.24 |  |  |  |  |  | 0.49 |  | 0.51 |  | 0.51 |  | 0.59 |  | 0.67 |
| 𝜆 |  | 0.21 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 10ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 3km/h, 10M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-21.43%, 7OS-23.14%, 3/4OS-26.57%, 2OS-31.71%, 1OS-42%Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | - | - | - | - | - |  | 1.68 |  | 2.85 |  | 3.17 |  | 3.23 | 2.88 |
|  | 50% | - | - | - | - | - | - | - | - | - | - |  | 6.93 |  | 10.47 |  | 12.43 |  | 13.89 | 14.40 |
|  | 95% | - | - | - | - | - | - | - | - | - | - |  | 12.66 |  | 18.14 |  | 20.46 |  | 22.29 | 23.26 |
|  | Mean | - | - | - | - | - | - | - | - | - | - |  | 6.71 |  | 10.60 |  | 12.60 |  | 14.33 | 14.50 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | - | - | - | - | - |  | 2.3490 |  | 1.3890 |  | 1.2711 |  | 1.2301 | 1.3813 |
|  | 50% | - | - | - | - | - | - | - | - | - | - |  | 0.5770 |  | 0.3815 |  | 0.3225 |  | 0.2879 | 0.2776 |
|  | 95% | - | - | - | - | - | - | - | - | - | - |  | 0.3150 |  | 0.2205 |  | 0.1957 |  | 0.1794 | 0.1719 |
|  | Mean | - | - | - | - | - | - | - | - | - | - |  | 0.8577 |  | 0.5318 |  | 0.4622 |  | 0.4195 | 0.4442 |
| RU |  | - | - | - | - | - | - | - | - | - | - |  | 0.69 |  | 0.65 |  | 0.63 |  | 0.59 | 0.66 |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 10ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 120km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | - | - | - | - | - |  | 1.37 |  | 1.90 |  | 1.79 |  | 1.57 | 1.61 |
|  | 50% | - | - | - | - | - | - | - | - | - | - |  | 5.82 |  | 8.76 |  | 9.37 |  | 9.76 | 7.94 |
|  | 95% | - | - | - | - | - | - | - | - | - | - |  | 10.39 |  | 15.97 |  | 18.36 |  | 20.44 | 18.00 |
|  | Mean | - | - | - | - | - | - | - | - | - | - |  | 5.82 |  | 8.41 |  | 9.26 |  | 9.79 | 8.52 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | - | - | - | - | - |  | 2.8340 |  | 2.0445 |  | 2.1633 |  | 2.3919 | 2.4828 |
|  | 50% | - | - | - | - | - | - | - | - | - | - |  | 0.6830 |  | 0.4565 |  | 0.4269 |  | 0.4093 | 0.5013 |
|  | 95% | - | - | - | - | - | - | - | - | - | - |  | 0.3790 |  | 0.2495 |  | 0.2172 |  | 0.1949 | 0.2175 |
|  | Mean | - | - | - | - | - | - | - | - | - | - |  | 1.0040 |  | 0.7363 |  | 0.6973 |  | 0.7211 | 0.8431 |
| RU |  | - | - | - | - | - | - | - | - | - | - |  | 0.63 |  | 0.60 |  | 0.64 |  | 0.65 | 0.76 |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 10ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 120km/h, 10M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-21.43%, 7OS-23.14%, 3/4OS-26.57%, 2OS-31.71%, 1OS-42%Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  | 14 OS |  | 7OS |  |  |  | 3/4 OS |  |  |  | 2OS |  |  |  | 1OS |  |  |  | 14 OS |  |  | 7OS |  |  |  |  |  | 3/4 OS |  |  |  | 2OS |  |  | 1OS |  |  | 14 OS |  |  | 7OS |  |  | 3/4 OS |  |  | 2OS |  | 1OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL: UPT CDF [Mbps] |  | 5% |  | 9.864 |  | 11.201 (+0.136) |  |  |  | N/A |  |  |  | 10.526 (+0.067) |  |  |  | N/A |  |  |  | 4.934 |  |  | 5.06 (+0.026) |  |  |  |  |  | N/A |  |  |  | 4.857 (-0.016) |  |  | N/A |  |  | 3.662 |  |  | 3.909 (+0.067) |  |  | N/A |  |  | 3.709 (+0.013) |  | N/A |
|  |  | 50% |  | 25.581 |  | 32.619 (+0.275) |  |  |  | N/A |  |  |  | 37.53 (+0.467) |  |  |  | N/A |  |  |  | 20.892 |  |  | 24.56 (+0.176) |  |  |  |  |  | N/A |  |  |  | 23.267 (+0.114) |  |  | N/A |  |  | 17.781 |  |  | 19.15 (+0.077) |  |  | N/A |  |  | 16.192 (-0.089) |  | N/A |
|  |  | 95% |  | 46.673 |  | 62.795 (+0.345) |  |  |  | N/A |  |  |  | 69.716 (+0.494) |  |  |  | N/A |  |  |  | 42.23 |  |  | 57.007 (+0.350) |  |  |  |  |  | N/A |  |  |  | 61.465 (+0.455) |  |  | N/A |  |  | 39.888 |  |  | 52.3275 (+0.312) |  |  | N/A |  |  | 51.495 (+0.291) |  | N/A |
|  |  | Mean |  | 26.901 |  | 34.51 (+0.283) |  |  |  | N/A |  |  |  | 37.553 (+0.396) |  |  |  | N/A |  |  |  | 21.146 |  |  | 26.126 (0.236) |  |  |  |  |  | N/A |  |  |  | 26.581 (+0.257) |  |  | N/A |  |  | 18.527 |  |  | 22.189 (+0.198) |  |  | N/A |  |  | 21.033 (+0.135) |  | N/A |
| DL: Delay CDF [ms] |  | 5% |  | 85.728 |  | 63.705 (-0.257) |  |  |  | N/A |  |  |  | 57.384 (-0.331) |  |  |  | N/A |  |  |  | 94.77 |  |  | 70.18 (-0.259) |  |  |  |  |  | N/A |  |  |  | 65.135 (-0.313) |  |  | N/A |  |  | 100.513 |  |  | 76.445 (-0.239) |  |  | N/A |  |  | 77.705 (-0.227) |  | N/A |
|  |  | 50% |  | 156.417 |  | 122.588 (-0.216) |  |  |  | N/A |  |  |  | 106.55 (-0.319) |  |  |  | N/A |  |  |  | 191.5 |  |  | 162.95 (-0.149) |  |  |  |  |  | N/A |  |  |  | 172.1 (-0.101) |  |  | N/A |  |  | 224.775 |  |  | 208.95 (-0.070) |  |  | N/A |  |  | 247.15 (+0.100) |  | N/A |
|  |  | 95% |  | 405.445 |  | 357.09 (-0.119) |  |  |  | N/A |  |  |  | 380.115 (-0.062) |  |  |  | N/A |  |  |  | 807.03 |  |  | 794.62 (-0.015) |  |  |  |  |  | N/A |  |  |  | 822.43 (+0.019) |  |  | N/A |  |  | 1089.68 |  |  | 1013.46 (-0.070) |  |  | N/A |  |  | 1079.79 (-0.009) |  | N/A |
|  |  | Mean |  | 184.509 |  | 151.71 (-0.178) |  |  |  | N/A |  |  |  | 150.596 (-0.184) |  |  |  | N/A |  |  |  | 291.302 |  |  | 259.019 (-0.111) |  |  |  |  |  | N/A |  |  |  | 270.751 (-0.071) |  |  | N/A |  |  | 353.799 |  |  | 331.678 (-0.063) |  |  | N/A |  |  | 357.548 (+0.011) |  | N/A |
| RU |  |  |  | 25.947 |  | 24.147 (-0.069) |  |  |  | N/A |  |  |  | 24.704 (-0.048) |  |  |  | N/A |  |  |  | 55.910 |  |  | 52.707 (-0.057) |  |  |  |  |  | N/A |  |  |  | 55.023 (-0.016) |  |  | N/A |  |  | 68.893 |  |  | 67.759 (-0.016) |  |  | N/A |  |  | 70.552 (+0.024) |  | N/A |
| 𝜆 |  |  |  | 0.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.75 |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  |  | Source 11, R1-163265, CN Delay=2ms; max scheduled users=10; Total overhead is 23.1% with 7s-TTI, 31.7% with 2s-TTI( CCE number=4); for legacy TTI length: control region total overhead is always 21.4% regardless CCE number; CSI feedback period 5ms, SR to grant 8 (s)TTI, HARQ RTT 8 s(TTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 14 OS |  | 7OS |  |  |  | 3/4 OS |  |  |  | 2OS |  |  |  | 1OS |  |  |  | 14 OS |  |  | 7OS |  |  |  |  |  | 3/4 OS |  |  |  | 2OS |  |  | 1OS |  |  | 14 OS |  |  | 7OS |  |  | 3/4 OS |  |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] |  | 5% |  | 9.864 |  | 10.901 (+0.105) |  |  |  | N/A |  |  |  | 7.2433 (-0.266) |  |  |  | N/A |  |  |  | 4.934 |  |  | 4.96375 (+0.006) |  |  |  |  |  | N/A |  |  |  | 3.271 (-0.337) |  |  | N/A |  |  | 3.663 |  |  | 3.84143 (+0.047) |  |  | N/A |  |  | 3.406 (-0.070) |  | N/A |
|  |  | 50% |  | 25.581 |  | 32.575 (+0.273) |  |  |  | N/A |  |  |  | 29.1 (0.138) |  |  |  | N/A |  |  |  | 20.892 |  |  | 24.0917 (+0.153) |  |  |  |  |  | N/A |  |  |  | 15.788 (-0.244) |  |  | N/A |  |  | 17.781 |  |  | 18.7 (+0.049) |  |  | N/A |  |  | 12.225 (-0.312) |  | N/A |
|  |  | 95% |  | 46.6723 |  | 63.265 (+0.356) |  |  |  | N/A |  |  |  | 64.085 (0.373) |  |  |  | N/A |  |  |  | 42.23 |  |  | 56.8225 (+0.346) |  |  |  |  |  | N/A |  |  |  | 47.923 (+0.135) |  |  | N/A |  |  | 39.888 |  |  | 52.7025 (+0.243) |  |  | N/A |  |  | 43.622 (+0.094) |  | N/A |
|  |  | Mean |  | 26.908 |  | 34.338 (+0.276) |  |  |  | N/A |  |  |  | 30.908 (0.149) |  |  |  | N/A |  |  |  | 21.146 |  |  | 25.781 (+0.219) |  |  |  |  |  | N/A |  |  |  | 19.89 (-0.059) |  |  | N/A |  |  | 18.527 |  |  | 21.9594 (+0.156) |  |  | N/A |  |  | 16.535 (-0.108) |  | N/A |
| DL: Delay CDF [ms] |  | 5% |  | 85.728 |  | 63.235 (-0.262) |  |  |  | N/A |  |  |  | 62.408 (-0.272) |  |  |  | N/A |  |  |  | 94.77 |  |  | 70.3775 (-0.257) |  |  |  |  |  | N/A |  |  |  | 83.51 (-0.119) |  |  | N/A |  |  | 100.513 |  |  | 75.89 (-0.324) |  |  | N/A |  |  | 91.635 (-0.088) |  | N/A |
|  |  | 50% |  | 156.417 |  | 122.85 (-0.215) |  |  |  | N/A |  |  |  | 137.45 (-0.121) |  |  |  | N/A |  |  |  | 191.5 |  |  | 166.25 (-0.132) |  |  |  |  |  | N/A |  |  |  | 253.35 (+0.323) |  |  | N/A |  |  | 224.775 |  |  | 213.85 (-0.051) |  |  | N/A |  |  | 326.95 (+0.455) |  | N/A |
|  |  | 95% |  | 405.445 |  | 366.595 (-0.096) |  |  |  | N/A |  |  |  | 553.27 (0.365) |  |  |  | N/A |  |  |  | 807.03 |  |  | 802.445 (-0.006) |  |  |  |  |  | N/A |  |  |  | 1218.59 (+0.510) |  |  | N/A |  |  | 1089.68 |  |  | 1033.21 (-0.055) |  |  | N/A |  |  | 1175.76 (+0.079) |  | N/A |
|  |  | Mean |  | 184.509 |  | 153.729 (-0.167) |  |  |  | N/A |  |  |  | 201.637 (0.093) |  |  |  | N/A |  |  |  | 291.302 |  |  | 264.129 (-0.093) |  |  |  |  |  | N/A |  |  |  | 393.5 (+0.351) |  |  | N/A |  |  | 353.799 |  |  | 337.174 (-0.049) |  |  | N/A |  |  | 446.497 (+0.262) |  | N/A |
| RU |  |  |  | 25.947 |  | 24.431 (-0.058) |  |  |  | N/A |  |  |  | 31.182 (0.202) |  |  |  | N/A |  |  |  | 55.910 |  |  | 53.506 (-0.043) |  |  |  |  |  | N/A |  |  |  | 65.365 (0.169) |  |  | N/A |  |  | 68.893 |  |  | 68.243 (-0.010) |  |  | N/A |  |  | 75.785 (0.100) |  | N/A |
| 𝜆 |  |  |  | 0.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.75 |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  |  | Source 11, R1-163265,CN Delay=2ms; max scheduled users=10; Total overhead is 24.9% with 7s-TTI, 42.0% with 2s-TTI(CCE number=8); For legacy TTI length, total overhead is always 21.4% regardless CCE number; CSI feedback period 5ms, SR to grant 8 (s)TTI, HARQ RTT 8 s(TTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 14 OS |  | 7OS |  |  |  | 3/4 OS |  |  |  | 2OS |  |  |  | 1OS |  |  |  | 14 OS |  |  | 7OS |  |  |  |  |  | 3/4 OS |  |  |  | 2OS |  |  | 1OS |  |  | 14 OS |  |  | 7OS |  |  | 3/4 OS |  |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] |  | 5% |  | 9.864 |  | 9.982 (+0.012) |  |  |  | N/A |  |  |  | 2.280 (-0.769) |  |  |  | N/A |  |  |  | 4.934 |  |  | 4.656 (-0.056) |  |  |  |  |  | N/A |  |  |  | 1.763 (-0.643) |  |  | N/A |  |  | 3.663 |  |  | 3.675 (-0.056) |  |  | N/A |  |  | 2.111 (-0.643) |  | N/A |
|  |  | 50% |  | 25.581 |  | 31.3714 (+0.226) |  |  |  | N/A |  |  |  | 9.941 (-0.611) |  |  |  | N/A |  |  |  | 20.892 |  |  | 22.85 (+0.094) |  |  |  |  |  | N/A |  |  |  | 6.003 (-0.713) |  |  | N/A |  |  | 17.7812 |  |  | 17.5167 (+0.094) |  |  | N/A |  |  | 5.4 (-0.713) |  | N/A |
|  |  | 95% |  | 46.673 |  | 61.655 (+0.321) |  |  |  | N/A |  |  |  | 40.555 (-0.131) |  |  |  | N/A |  |  |  | 42.23 |  |  | 54.42 (+0.289) |  |  |  |  |  | N/A |  |  |  | 25.798 (-0.389) |  |  | N/A |  |  | 39.888 |  |  | 51.425 (+0.289) |  |  | N/A |  |  | 28.84 (-0.389) |  | N/A |
|  |  | Mean |  | 26.901 |  | 33.1968 (+0.234) |  |  |  | N/A |  |  |  | 13.781 (-0.488) |  |  |  | N/A |  |  |  | 21.146 |  |  | 24.725 (+0.169) |  |  |  |  |  | N/A |  |  |  | 8.679 (-0.590) |  |  | N/A |  |  | 18.527 |  |  | 21.1374 (+0.169) |  |  | N/A |  |  | 8.822 (-0.590) |  | N/A |
| DL: Delay CDF [ms] |  | 5% |  | 85.728 |  | 64.89 (-0.243) |  |  |  | N/A |  |  |  | 98.573 (+0.150) |  |  |  | N/A |  |  |  | 94.77 |  |  | 73.48 (-0.225) |  |  |  |  |  | N/A |  |  |  | 155.205 (+0.638) |  |  | N/A |  |  | 100.513 |  |  | 77.775 (-0.225) |  |  | N/A |  |  | 138.76 (0.638) |  | N/A |
|  |  | 50% |  | 156.417 |  | 127.5 (-0.185) |  |  |  | N/A |  |  |  | 402.45 (+1.573) |  |  |  | N/A |  |  |  | 191.5 |  |  | 175.05 (-0.086) |  |  |  |  |  | N/A |  |  |  | 666.35 (+2.480) |  |  | N/A |  |  | 224.775 |  |  | 228.25 (-0.086) |  |  | N/A |  |  | 740.4 (+2.480) |  | N/A |
|  |  | 95% |  | 405.445 |  | 401.405 (-0.010) |  |  |  | N/A |  |  |  | 1750.36 (+3.317) |  |  |  | N/A |  |  |  | 807.03 |  |  | 858.92 (+0.064) |  |  |  |  |  | N/A |  |  |  | 2284.3 (+1.831) |  |  | N/A |  |  | 1089.68 |  |  | 1090.22 (+0.064) |  |  | N/A |  |  | 1895.44 (+1.831) |  | N/A |
|  |  | Mean |  | 184.509 |  | 161.805 (-0.123) |  |  |  | N/A |  |  |  | 605.673 (+2.283) |  |  |  | N/A |  |  |  | 291.302 |  |  | 281.614 (-0.033) |  |  |  |  |  | N/A |  |  |  | 869.306 (+1.984) |  |  | N/A |  |  | 353.799 |  |  | 353.27 (-0.033) |  |  | N/A |  |  | 829.17 (+1.984) |  | N/A |
| RU |  |  |  | 25.947 |  | 25.685 (-0.010) |  |  |  | N/A |  |  |  | 55.234 (+1.129) |  |  |  | N/A |  |  |  | 55.910 |  |  | 55.494 (-0.007) |  |  |  |  |  | N/A |  |  |  | 79.895 (+0.429) |  |  | N/A |  |  | 68.893 |  |  | 69.630 (-0.007) |  |  | N/A |  |  | 83.859 (+0.429) |  | N/A |
| 𝜆 |  |  |  | 0.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.75 |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  |  | Source 11, R1-163265, CN Delay=2ms; max scheduled users=10; Total overhead is 28.3% with 7s-TTI, 62.6% with 2s-TTI(CCE number=16); For legacy TTI length, total overhead is always 21.4% regardless CCE number; CSI feedback period 5ms, SR to grant 8 (s)TTI, HARQ RTT 8 s(TTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 14 OS |  | 7OS |  |  |  | 3/4 OS |  |  |  | 2OS |  |  |  | 1OS |  |  |  | 14 OS |  |  | 7OS |  |  |  |  |  | 3/4 OS |  |  |  | 2OS |  |  | 1OS |  |  | 14 OS |  |  | 7OS |  |  | 3/4 OS |  |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% |  |  | 9.864 |  | 11.201 (+0.136) |  |  |  | N/A |  |  |  | 10.541 (+0.069) |  |  |  | N/A |  |  |  | 4.954 |  |  | 5.138 (+0.037) |  |  |  |  |  | N/A |  |  |  | 4.998 (+0.009) |  |  | N/A |  |  | 3.792 |  |  | 4.135 (+0.091) |  |  | N/A |  |  | 3.738 (-0.014) |  | N/A |
|  | 50% |  |  | 25.573 |  | 32.645 (+0.277) |  |  |  | N/A |  |  |  | 37.563 (+0.469) |  |  |  | N/A |  |  |  | 20.917 |  |  | 24.783 (+0.185) |  |  |  |  |  | N/A |  |  |  | 23.242 (+0.111) |  |  | N/A |  |  | 17.720 |  |  | 19.100 (+0.078) |  |  | N/A |  |  | 16.325 (-0.079) |  | N/A |
|  | 95% |  |  | 46.673 |  | 62.498 (+0.339) |  |  |  | N/A |  |  |  | 69.615 (+0.492) |  |  |  | N/A |  |  |  | 42.073 |  |  | 56.913 (+0.353) |  |  |  |  |  | N/A |  |  |  | 61.405 (+0.459) |  |  | N/A |  |  | 39.670 |  |  | 52.670 (+0.328) |  |  | N/A |  |  | 52.255 (+0.317) |  | N/A |
|  | Mean |  |  | 26.910 |  | 34.512 (+0.283) |  |  |  | N/A |  |  |  | 37.553 (+0.395) |  |  |  | N/A |  |  |  | 21.171 |  |  | 26.194 (+0.237) |  |  |  |  |  | N/A |  |  |  | 26.581 (+0.256) |  |  | N/A |  |  | 18.531 |  |  | 22.233 (+0.200) |  |  | N/A |  |  | 21.018 (+0.134) |  | N/A |
| DL: Delay CDF [ms] | 5% |  |  | 85.728 |  | 64.003 (-0.253) |  |  |  | N/A |  |  |  | 57.428 (-0.330) |  |  |  | N/A |  |  |  | 95.090 |  |  | 70.288 (-0.261) |  |  |  |  |  | N/A |  |  |  | 65.195 (-0.314) |  |  | N/A |  |  | 100.930 |  |  | 75.930 (-0.248) |  |  | N/A |  |  | 76.545 (-0.242) |  | N/A |
|  | 50% |  |  | 156.450 |  | 122.538 (-0.217) |  |  |  | N/A |  |  |  | 106.475 (-0.319) |  |  |  | N/A |  |  |  | 191.300 |  |  | 161.325 (-0.157) |  |  |  |  |  | N/A |  |  |  | 172.150 (-0.100) |  |  | N/A |  |  | 225.800 |  |  | 207.950 (-0.079) |  |  | N/A |  |  | 244.850 (+0.084) |  | N/A |
|  | 95% |  |  | 405.645 |  | 357.095 (-0.120) |  |  |  | N/A |  |  |  | 379.215 (-0.065) |  |  |  | N/A |  |  |  | 803.320 |  |  | 781.425 (-0.027) |  |  |  |  |  | N/A |  |  |  | 804.005 (+0.001) |  |  | N/A |  |  | 1053.940 |  |  | 963.210 (-0.086) |  |  | N/A |  |  | 1071.750 (+0.017) |  | N/A |
|  | Mean |  |  | 184.515 |  | 151.708 (-0.178) |  |  |  | N/A |  |  |  | 150.561 (-0.184) |  |  |  | N/A |  |  |  | 290.705 |  |  | 256.814 (-0.117) |  |  |  |  |  | N/A |  |  |  | 269.923 (-0.071) |  |  | N/A |  |  | 351.380 |  |  | 328.409 (-0.065) |  |  | N/A |  |  | 356.848 (+0.016) |  | N/A |
| RU |  |  |  | 25.955 |  | 24.147 (-0.070) |  |  |  | N/A |  |  |  | 24.704 (-0.048) |  |  |  | N/A |  |  |  | 55.978 |  |  | 52.802 (-0.057) |  |  |  |  |  | N/A |  |  |  | 55.313 (-0.012) |  |  | N/A |  |  | 69.782 |  |  | 68.905 (-0.013) |  |  | N/A |  |  | 72.128 (+0.034) |  | N/A |
| 𝜆 |  |  |  | 0.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.75 |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  |  | Source 11, R1-163265 , CN Delay=2ms; max scheduled users=4; Total overhead is 23.1% with 7s-TTI, 31.7% with 2s-TTI(CCE number=4); For legacy TTI length, total overhead is always 21.4% regardless CCE number; CSI feedback period 5ms, SR to grant 8 (s)TTI, HARQ RTT 8 s(TTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 14 OS |  |  | 7OS |  |  |  | 3/4 OS |  |  |  | 2OS |  |  |  | 1OS |  |  |  | 14 OS |  |  | 7OS |  |  |  |  |  | 3/4 OS |  |  |  | 2OS |  |  | 1OS |  |  | 14 OS |  |  | 7OS |  |  | 3/4 OS |  |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  |  | 9.864 |  |  | 10.853 (+0.100) |  |  |  | N/A |  |  |  | 7.268 (-0.263) |  |  |  | N/A |  |  |  | 4.954 |  |  | 5.140 (+0.037) |  |  |  |  |  | N/A |  |  |  | 3.304 (-0.333) |  |  | N/A |  |  | 3.792 |  |  | 4.014 (+0.059) |  |  | N/A |  |  | 3.369 (-0.112) | N/A |
|  | 50% |  |  | 25.573 |  |  | 32.579 (+0.274) |  |  |  | N/A |  |  |  | 29.200 (+0.142) |  |  |  | N/A |  |  |  | 20.917 |  |  | 24.309 (+0.162) |  |  |  |  |  | N/A |  |  |  | 15.833 (-0.243) |  |  | N/A |  |  | 17.720 |  |  | 18.840 (+0.063) |  |  | N/A |  |  | 12.350 (-0.303) | N/A |
|  | 95% |  |  | 46.673 |  |  | 63.232 (+0.355) |  |  |  | N/A |  |  |  | 64.187 (+0.375) |  |  |  | N/A |  |  |  | 42.073 |  |  | 56.690 (+0.347) |  |  |  |  |  | N/A |  |  |  | 47.580 (+0.131) |  |  | N/A |  |  | 39.670 |  |  | 52.470 (+0.323) |  |  | N/A |  |  | 43.185 (+0.089) | N/A |
|  | Mean |  |  | 26.910 |  |  | 34.334 (+0.276) |  |  |  | N/A |  |  |  | 30.948 (+0.150) |  |  |  | N/A |  |  |  | 21.171 |  |  | 25.899 (+0.223) |  |  |  |  |  | N/A |  |  |  | 19.857 (-0.062) |  |  | N/A |  |  | 18.531 |  |  | 22.013 (+0.188) |  |  | N/A |  |  | 16.548 (-0.107) | N/A |
| DL: Delay CDF [ms] | 5% |  |  | 85.728 |  |  | 63.268 (-0.262) |  |  |  | N/A |  |  |  | 62.313 (-0.273) |  |  |  | N/A |  |  |  | 95.090 |  |  | 70.513 (-0.258) |  |  |  |  |  | N/A |  |  |  | 84.020 (-0.116) |  |  | N/A |  |  | 100.930 |  |  | 76.230 (-0.245) |  |  | N/A |  |  | 92.645 (-0.082) | N/A |
|  | 50% |  |  | 156.450 |  |  | 122.850 (-0.215) |  |  |  | N/A |  |  |  | 136.950 (-0.125) |  |  |  | N/A |  |  |  | 191.300 |  |  | 164.567 (-0.140) |  |  |  |  |  | N/A |  |  |  | 252.350 (+0.319) |  |  | N/A |  |  | 225.800 |  |  | 212.450 (-0.059) |  |  | N/A |  |  | 323.850 (+0.434) | N/A |
|  | 95% |  |  | 405.645 |  |  | 367.195 (-0.095) |  |  |  | N/A |  |  |  | 550.960 (+0.358) |  |  |  | N/A |  |  |  | 803.320 |  |  | 770.660 (-0.041) |  |  |  |  |  | N/A |  |  |  | 1211.560 (+0.508) |  |  | N/A |  |  | 1053.940 |  |  | 993.370 (-0.057) |  |  | N/A |  |  | 1189.750 (+0.129) | N/A |
|  | Mean |  |  | 184.515 |  |  | 153.696 (-0.167) |  |  |  | N/A |  |  |  | 200.832 (+0.088) |  |  |  | N/A |  |  |  | 290.705 |  |  | 261.423 (-0.101) |  |  |  |  |  | N/A |  |  |  | 394.525 (+0.357) |  |  | N/A |  |  | 351.380 |  |  | 333.649 (-0.050) |  |  | N/A |  |  | 446.289 (+0.270) | N/A |
| RU |  |  |  | 25.955 |  |  | 24.429 (-0.059) |  |  |  | N/A |  |  |  | 31.213 (+0.203) |  |  |  | N/A |  |  |  | 55.978 |  |  | 53.622 (-0.042) |  |  |  |  |  | N/A |  |  |  | 66.148 (+0.182) |  |  | N/A |  |  | 69.782 |  |  | 69.311 (-0.007) |  |  | N/A |  |  | 78.264 (+0.122) | N/A |
| 𝜆 |  |  |  | 0.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.75 |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  |  | Source 11, R1-163265, CN Delay=2ms; max scheduled users=4; Total overhead is 24.9% with 7s-TTI, 42.0% with 2s-TTI(CCE number=8); For legacy TTI length, total overhead is always 21.4% regardless CCE number; CSI feedback period 5ms, SR to grant 8 (s)TTI, HARQ RTT 8 s(TTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 14 OS | 7OS |  |  | 3/4 OS |  |  |  | 2OS |  |  |  |  | 1OS |  |  |  | 14 OS |  |  | 7OS |  |  | 3/4 OS |  | 2OS |  |  |  | 1OS |  |  |  | 14 OS |  |  | 7OS |  |  | 3/4 OS |  |  | 2OS |  |  | 1OS |  |  |  |
| DL: UPT CDF [Mbps] | 5% |  |  | 9.864 | 9.958 (+0.010) |  |  | N/A |  |  |  | 2.344 (-0.762) |  |  |  |  | N/A |  |  |  | 4.954 |  |  | 4.654 (-0.061) |  |  | N/A |  | 1.868 (-0.623) |  |  |  | N/A |  |  |  | 3.792 |  |  | 3.668 (-0.033) |  |  | N/A |  |  | 2.204 (-0.419) |  |  | N/A |  |  |  |
|  | 50% |  |  | 25.573 | 31.343 (+0.226) |  |  | N/A |  |  |  | 9.982 (-0.610) |  |  |  |  | N/A |  |  |  | 20.917 |  |  | 22.808 (+0.090) |  |  | N/A |  | 6.013 (-0.713) |  |  |  | N/A |  |  |  | 17.720 |  |  | 17.600 (-0.007) |  |  | N/A |  |  | 5.450 (-0.692) |  |  | N/A |  |  |  |
|  | 95% |  |  | 46.673 | 61.610 (+0.320) |  |  | N/A |  |  |  | 40.170 (-0.139) |  |  |  |  | N/A |  |  |  | 42.073 |  |  | 54.593 (+0.298) |  |  | N/A |  | 25.910 (-0.384) |  |  |  | N/A |  |  |  | 39.670 |  |  | 52.545 (0.325) |  |  | N/A |  |  | 27.683 (-0.302) |  |  | N/A |  |  |  |
|  | Mean |  |  | 26.910 | 33.143 (+0.232) |  |  | N/A |  |  |  | 13.818 (-0.487) |  |  |  |  | N/A |  |  |  | 21.171 |  |  | 24.721 (+0.168) |  |  | N/A |  | 8.661 (-0.591) |  |  |  | N/A |  |  |  | 18.531 |  |  | 21.165 (+0.142) |  |  | N/A |  |  | 8.669 (-0.532) |  |  | N/A |  |  |  |
| DL: Delay CDF [ms] | 5% |  |  | 85.728 | 64.890 (-0.243) |  |  | N/A |  |  |  | 99.550 (+0.161) |  |  |  |  | N/A |  |  |  | 95.090 |  |  | 73.308 (-0.229) |  |  | N/A |  | 154.190 (+0.622) |  |  |  | N/A |  |  |  | 100.930 |  |  | 76.155 (-0.245) |  |  | N/A |  |  | 144.435 (+0.431) |  |  | N/A |  |  |  |
|  | 50% |  |  | 156.450 | 127.567 (-0.185) |  |  | N/A |  |  |  | 400.850 (+1.562) |  |  |  |  | N/A |  |  |  | 191.300 |  |  | 175.450 (-0.083) |  |  | N/A |  | 662.400 (+2.463) |  |  |  | N/A |  |  |  | 225.800 |  |  | 227.050 (+0.006) |  |  | N/A |  |  | 733.150 (+2.247) |  |  | N/A |  |  |  |
|  | 95% |  |  | 405.645 | 401.455 (-0.010) |  |  | N/A |  |  |  | 1718.250 (+3.236) |  |  |  |  | N/A |  |  |  | 803.320 |  |  | 864.985 (+0.077) |  |  | N/A |  | 2157.510 (+1.686) |  |  |  | N/A |  |  |  | 1053.940 |  |  | 1093.290 (+0.037) |  |  | N/A |  |  | 1804.070 (+0.712) |  |  | N/A |  |  |  |
|  | Mean |  |  | 184.515 | 162.138 (-0.121) |  |  | N/A |  |  |  | 588.220 (+2.188) |  |  |  |  | N/A |  |  |  | 290.705 |  |  | 282.509 (-0.028) |  |  | N/A |  | 840.543 (+1.891) |  |  |  | N/A |  |  |  | 351.380 |  |  | 352.834 (+0.004) |  |  | N/A |  |  | 802.510 (+1.284) |  |  | N/A |  |  |  |
| RU |  |  |  | 25.955 | 25.765 (-0.007) |  |  | N/A |  |  |  | 55.438 (+1.136) |  |  |  |  | N/A |  |  |  | 55.978 |  |  | 55.792 (-0.003) |  |  | N/A |  | 81.425 (+0.455) |  |  |  | N/A |  |  |  | 69.782 |  |  | 70.919 (+0.016) |  |  | N/A |  |  | 85.892 (+0.231) |  |  | N/A |  |  |  |
| 𝜆 |  |  |  | 0.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  |  | Source 11, R1-163265, CN Delay=2ms; max scheduled users=4; Total overhead is 28.3% with 7s-TTI, 62.6% with 2s-TTI(CCE number=16); For legacy TTI length, total overhead is always 21.4% regardless CCE number; CSI feedback period 5ms, SR to grant 8 (s)TTI, HARQ RTT 8 s(TTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS |  | 7OS |  |  |  | 3/4 OS |  |  |  | 2OS |  |  | 1OS |  |  |  | 14 OS |  |  |  | 7OS |  |  |  | 3/4 OS |  | 2OS |  |  |  | 1OS |  |  | 14 OS |  |  | 7OS |  |  | 3/4 OS |  |  | 2OS |  |  | 1OS |  |  |  |
| DL: UPT CDF [Mbps] | 5% |  | 8.213 |  | 9.597 (+0.169) |  |  |  | N/A |  |  |  | 9.075 (+0.105) |  |  | N/A |  |  |  | 3.493 |  |  |  | 3.761 (+0.077) |  |  |  | N/A |  | 3.369 (-0.035) |  |  |  | N/A |  |  | 3.280 |  |  | 3.135 (-0.044) |  |  | N/A |  |  | 3.162 (-0.036) |  |  | N/A |  |  |  |
|  | 50% |  | 24.938 |  | 31.691 (+0.271) |  |  |  | N/A |  |  |  | 36.080 (+0.447) |  |  | N/A |  |  |  | 19.492 |  |  |  | 21.580 (+0.107) |  |  |  | N/A |  | 18.717 (-0.040) |  |  |  | N/A |  |  | 14.883 |  |  | 15.875 (+0.067) |  |  | N/A |  |  | 13.100 (-0.120) |  |  | N/A |  |  |  |
|  | 95% |  | 46.550 |  | 62.970 (+0.353) |  |  |  | N/A |  |  |  | 69.225 (+0.487) |  |  | N/A |  |  |  | 41.432 |  |  |  | 56.093 (+0.354) |  |  |  | N/A |  | 56.285 (+0.359) |  |  |  | N/A |  |  | 39.020 |  |  | 51.663 (+0.324) |  |  | N/A |  |  | 46.743 (+0.198) |  |  | N/A |  |  |  |
|  | Mean |  | 25.992 |  | 33.503 (+0.289) |  |  |  | N/A |  |  |  | 36.337 (+0.398) |  |  | N/A |  |  |  | 19.591 |  |  |  | 23.740 (+0.212) |  |  |  | N/A |  | 22.927 (0.170) |  |  |  | N/A |  |  | 17.016 |  |  | 20.226 (+0.189) |  |  | N/A |  |  | 18.144 (+0.066) |  |  | N/A |  |  |  |
| DL: Delay CDF [ms] | 5% |  | 85.925 |  | 63.530 (-0.261) |  |  |  | N/A |  |  |  | 57.783 (-0.328) |  |  | N/A |  |  |  | 96.568 |  |  |  | 71.310 (-0.262) |  |  |  | N/A |  | 71.115 (-0.264) |  |  |  | N/A |  |  | 102.380 |  |  | 77.438 (-0.244) |  |  | N/A |  |  | 85.585 (-0.164) |  |  | N/A |  |  |  |
|  | 50% |  | 160.425 |  | 126.250 (-0.213) |  |  |  | N/A |  |  |  | 110.850 (-0.309) |  |  | N/A |  |  |  | 205.250 |  |  |  | 185.300 (-0.097) |  |  |  | N/A |  | 213.350 (+0.039) |  |  |  | N/A |  |  | 268.650 |  |  | 252.450 (-0.060) |  |  | N/A |  |  | 305.500 (+0.137) |  |  | N/A |  |  |  |
|  | 95% |  | 486.650 |  | 418.410 (-0.140) |  |  |  | N/A |  |  |  | 442.750 (-0.090) |  |  | N/A |  |  |  | 1142.990 |  |  |  | 1064.970 (-0.068) |  |  |  | N/A |  | 1178.090 (+0.031) |  |  |  | N/A |  |  | 1227.720 |  |  | 1265.830 (+0.031) |  |  | N/A |  |  | 1264.330 (+0.030) |  |  | N/A |  |  |  |
|  | Mean |  | 201.676 |  | 165.144 (-0.181) |  |  |  | N/A |  |  |  | 162.706 (-0.193) |  |  | N/A |  |  |  | 354.412 |  |  |  | 329.893 (-0.069) |  |  |  | N/A |  | 360.392 (+0.017) |  |  |  | N/A |  |  | 415.602 |  |  | 397.413 (-0.044) |  |  | N/A |  |  | 440.231 (+0.059) |  |  | N/A |  |  |  |
| RU |  |  | 27.161 |  | 25.162 (-0.074) |  |  |  | N/A |  |  |  | 25.814 (-0.050) |  |  | N/A |  |  |  | 59.856 |  |  |  | 58.120 (-0.029) |  |  |  | N/A |  | 61.879 (+0.034) |  |  |  | N/A |  |  | 72.560 |  |  | 72.843 (+0.004) |  |  | N/A |  |  | 76.382 (+0.053) |  |  | N/A |  |  |  |
| 𝜆 |  |  | 0.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  | Source 11, R1-163265, CN Delay=2ms; max scheduled users=1; Total overhead is 23.1% with 7s-TTI, 31.7% with 2s-TTI(CCE number=4); For legacy TTI length, total overhead is always 21.4% regardless CCE number; CSI feedback period 5ms, SR to grant 8 (s)TTI, HARQ RTT 8 s(TTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS |  | 7OS |  |  |  | 3/4 OS |  |  |  | 2OS |  |  | 1OS |  |  |  | 14 OS |  |  |  | 7OS |  |  |  | 3/4 OS |  | 2OS |  |  |  | 1OS |  |  | 14 OS |  |  | 7OS |  |  | 3/4 OS |  |  | 2OS |  |  | 1OS |  |  |  |
| DL: UPT CDF [Mbps] | 5% |  | 8.213 |  | 9.398 (+0.144) |  |  |  | N/A |  |  |  | 5.820 (-0.291) |  |  | N/A |  |  |  | 3.493 |  |  |  | 3.728 (+0.067) |  |  |  | N/A |  | 2.542 (-0.272) |  |  |  | N/A |  |  | 3.280 |  |  | 3.232 (-0.015) |  |  | N/A |  |  | 2.681 (-0.183) |  |  | N/A |  |  |  |
|  | 50% |  | 24.938 |  | 31.600 (+0.267) |  |  |  | N/A |  |  |  | 26.740 (+0.072) |  |  | N/A |  |  |  | 19.492 |  |  |  | 21.286 (+0.092) |  |  |  | N/A |  | 12.492 (-0.359) |  |  |  | N/A |  |  | 14.883 |  |  | 15.900 (+0.068) |  |  | N/A |  |  | 9.917 (-0.334) |  |  | N/A |  |  |  |
|  | 95% |  | 46.550 |  | 62.705 (+0.347) |  |  |  | N/A |  |  |  | 62.953 (+0.352) |  |  | N/A |  |  |  | 41.432 |  |  |  | 54.730 (+0.321) |  |  |  | N/A |  | 45.525 (+0.099) |  |  |  | N/A |  |  | 39.020 |  |  | 50.855 (+0.303) |  |  | N/A |  |  | 42.470 (+0.088) |  |  | N/A |  |  |  |
|  | Mean |  | 25.992 |  | 33.201 (+0.277) |  |  |  | N/A |  |  |  | 29.061 (+0.118) |  |  | N/A |  |  |  | 19.591 |  |  |  | 23.449 (+0.197) |  |  |  | N/A |  | 16.964 (-0.134) |  |  |  | N/A |  |  | 17.016 |  |  | 19.930 (+0.171) |  |  | N/A |  |  | 14.611 (-0.141) |  |  | N/A |  |  |  |
| DL: Delay CDF [ms] | 5% |  | 85.925 |  | 63.795 (-0.258) |  |  |  | N/A |  |  |  | 63.520 (-0.261) |  |  | N/A |  |  |  | 96.568 |  |  |  | 72.970 (-0.244) |  |  |  | N/A |  | 87.775 (-0.091) |  |  |  | N/A |  |  | 102.380 |  |  | 78.645 (-0.232) |  |  | N/A |  |  | 94.230 (-0.080) |  |  | N/A |  |  |  |
|  | 50% |  | 160.425 |  | 126.600 (-0.211) |  |  |  | N/A |  |  |  | 149.700 (-0.067) |  |  | N/A |  |  |  | 205.250 |  |  |  | 187.950 (-0.084) |  |  |  | N/A |  | 320.050 (+0.559) |  |  |  | N/A |  |  | 268.650 |  |  | 252.150 (-0.061) |  |  | N/A |  |  | 403.100 (+0.500) |  |  | N/A |  |  |  |
|  | 95% |  | 486.650 |  | 427.010 (-0.123) |  |  |  | N/A |  |  |  | 686.260 (+0.410) |  |  | N/A |  |  |  | 1142.990 |  |  |  | 1080.910 (-0.054) |  |  |  | N/A |  | 1568.030 (+0.372) |  |  |  | N/A |  |  | 1227.720 |  |  | 1242.810 (+0.012) |  |  | N/A |  |  | 1487.770 (+0.212) |  |  | N/A |  |  |  |
|  | Mean |  | 201.676 |  | 167.287 (-0.171) |  |  |  | N/A |  |  |  | 233.058 (+0.156) |  |  | N/A |  |  |  | 354.412 |  |  |  | 333.100 (-0.060) |  |  |  | N/A |  | 501.927 (+0.416) |  |  |  | N/A |  |  | 415.602 |  |  | 400.992 (-0.035) |  |  | N/A |  |  | 543.892 (+0.309) |  |  | N/A |  |  |  |
| RU |  |  | 27.161 |  | 25.577 (-0.058) |  |  |  | N/A |  |  |  | 33.304 (+0.226) |  |  | N/A |  |  |  | 59.856 |  |  |  | 58.862 (-0.017) |  |  |  | N/A |  | 70.990 (+0.186) |  |  |  | N/A |  |  | 72.560 |  |  | 73.374 (+0.011) |  |  | N/A |  |  | 81.636 (+0.125) |  |  | N/A |  |  |  |
| 𝜆 |  |  | 0.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  | Source 11, R1-163265, CN Delay=2ms; max scheduled users=1; Total overhead is 24.9% with 7s-TTI, 42.0% with 2s-TTI(CCE number=8); For legacy TTI length, total overhead is always 21.4% regardless CCE number; CCE number=8; for legacy TTI length, control region is always 21.4% regardless CCE number, CSI feedback period 5ms, SR to grant 8 (s)TTI, HARQ RTT 8 s(TTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS |  | 7OS |  |  |  | 3/4 OS |  |  |  | 2OS |  |  | 1OS |  |  |  | 14 OS |  |  |  | 7OS |  |  |  | 3/4 OS |  | 2OS |  |  |  | 1OS |  |  | 14 OS |  |  | 7OS |  |  | 3/4 OS |  |  | 2OS |  |  | 1OS |  |  |  |
| DL: UPT CDF [Mbps] | 5% |  | 8.213 |  | 8.656 (+0.054ss) |  |  |  | N/A |  |  |  | 1.781 (-0.783) |  |  | N/A |  |  |  | 3.493 |  |  |  | 3.359 (-0.038) |  |  |  | N/A |  | 1.665 (-0.523) |  |  |  | N/A |  |  | 3.280 |  |  | 3.118 (-0.049) |  |  | N/A |  |  | 2.226 (-0.321) |  |  | N/A |  |  |  |
|  | 50% |  | 24.938 |  | 30.600 (+0.227) |  |  |  | N/A |  |  |  | 9.750 (-0.609) |  |  | N/A |  |  |  | 19.492 |  |  |  | 19.783 (+0.015) |  |  |  | N/A |  | 6.271 (-0.678) |  |  |  | N/A |  |  | 14.883 |  |  | 14.290 (-0.040) |  |  | N/A |  |  | 6.300 (-0.577) |  |  | N/A |  |  |  |
|  | 95% |  | 46.550 |  | 62.005 (+0.332) |  |  |  | N/A |  |  |  | 39.755 (-0.146) |  |  | N/A |  |  |  | 41.432 |  |  |  | 54.010 (+0.304) |  |  |  | N/A |  | 27.330 (-0.340) |  |  |  | N/A |  |  | 39.020 |  |  | 48.948 (+0.254) |  |  | N/A |  |  | 30.970 (-0.206) |  |  | N/A |  |  |  |
|  | Mean |  | 25.992 |  | 32.069 (+0.234) |  |  |  | N/A |  |  |  | 13.655 (-0.475) |  |  | N/A |  |  |  | 19.591 |  |  |  | 22.312 (+0.139) |  |  |  | N/A |  | 9.039 (-0.539) |  |  |  | N/A |  |  | 17.016 |  |  | 18.998 (+0.116) |  |  | N/A |  |  | 9.779 (-0.425) |  |  | N/A |  |  |  |
| DL: Delay CDF [ms] | 5% |  | 85.925 |  | 64.495 (-0.249) |  |  |  | N/A |  |  |  | 100.590 (+0.171) |  |  | N/A |  |  |  | 96.568 |  |  |  | 74.070 (-0.233) |  |  |  | N/A |  | 146.155 (+0.513) |  |  |  | N/A |  |  | 102.380 |  |  | 81.678 (-0.202) |  |  | N/A |  |  | 129.330 (+0.263) |  |  | N/A |  |  |  |
|  | 50% |  | 160.425 |  | 130.750 (-0.185) |  |  |  | N/A |  |  |  | 410.000 (+1.556) |  |  | N/A |  |  |  | 205.250 |  |  |  | 202.300 (-0.014) |  |  |  | N/A |  | 636.550 (+2.101) |  |  |  | N/A |  |  | 268.650 |  |  | 279.925 (+0.042) |  |  | N/A |  |  | 634.150 (+1.361) |  |  | N/A |  |  |  |
|  | 95% |  | 486.650 |  | 464.120 (-0.046) |  |  |  | N/A |  |  |  | 2244.410 (+3.612) |  |  | N/A |  |  |  | 1142.990 |  |  |  | 1190.730 (+0.042) |  |  |  | N/A |  | 2399.690 (+1.099) |  |  |  | N/A |  |  | 1227.720 |  |  | 1288.950 (+0.050) |  |  | N/A |  |  | 1782.270 (+0.452) |  |  | N/A |  |  |  |
|  | Mean |  | 201.676 |  | 177.635 (-0.119) |  |  |  | N/A |  |  |  | 685.752 (+2.400) |  |  | N/A |  |  |  | 354.412 |  |  |  | 364.250 (+0.028) |  |  |  | N/A |  | 878.732 (+1.479) |  |  |  | N/A |  |  | 415.602 |  |  | 424.268 (+0.021) |  |  | N/A |  |  | 744.913 (+0.792) |  |  | N/A |  |  |  |
| RU |  |  | 27.161 |  | 27.002 (-0.006) |  |  |  | N/A |  |  |  | 59.501 (+1.191) |  |  | N/A |  |  |  | 59.856 |  |  |  | 61.444 (+0.027) |  |  |  | N/A |  | 84.417 (+0.410) |  |  |  | N/A |  |  | 72.560 |  |  | 74.819 (+0.031) |  |  | N/A |  |  | 88.312 (+0.217) |  |  | N/A |  |  |  |
| 𝜆 |  |  | 0.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  | Source 11, R1-163265, CN Delay=2ms; max scheduled users=1; Total overhead is 28.3% with 7s-TTI, 62.6% with 2s-TTI(CCE number=16); For legacy TTI length, total overhead is always 21.4% regardless CCE number; CSI feedback period 5ms, SR to grant 8 (s)TTI, HARQ RTT 8 s(TTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  |  |  |  |  |  | 14 OS |  |  | 7OS |  |  | 3/4 OS |  |  |  | 2OS |  |  | 1OS |  |  |  | 14 OS |  |  | 7OS |  |  |  | 3/4 OS |  |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  |  | 2OS |  | 1OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL: UPT CDF [Mbps] |  |  | 5% |  |  |  | 14.946 |  |  | 19.763 |  |  | 23.154 |  |  |  | - |  |  | - |  |  |  | 13.156 |  |  | 15.885 |  |  |  | 16.749 |  |  | - |  | - |  | 3.338 |  | 3.577 |  | 3.984 |  |  | - |  | - |
|  |  |  | 50% |  |  |  | 19.141 |  |  | 24.506 |  |  | 28.856 |  |  |  | - |  |  | - |  |  |  | 18.869 |  |  | 22.556 |  |  |  | 25.094 |  |  | - |  | - |  | 12.830 |  | 14.579 |  | 14.992 |  |  | - |  | - |
|  |  |  | 95% |  |  |  | 21.942 |  |  | 27.875 |  |  | 32.711 |  |  |  | - |  |  | - |  |  |  | 22.464 |  |  | 28.234 |  |  |  | 32.791 |  |  | - |  | - |  | 21.615 |  | 26.905 |  | 30.059 |  |  | - |  | - |
|  |  |  | Mean |  |  |  | 18.974 |  |  | 24.485 |  |  | 28.593 |  |  |  | - |  |  | - |  |  |  | 18.429 |  |  | 22.529 |  |  |  | 25.269 |  |  | - |  | - |  | 13.140 |  | 15.479 |  | 16.741 |  |  | - |  | - |
| DL: Delay CDF [s] |  |  | 5% |  |  |  | 0.199 |  |  | 0.154 |  |  | 0.131 |  |  |  | - |  |  | - |  |  |  | 0.196 |  |  | 0.154 |  |  |  | 0.133 |  |  | - |  | - |  | 0.207 |  | 0.157 |  | 0.134 |  |  | - |  | - |
|  |  |  | 50% |  |  |  | 0.231 |  |  | 0.179 |  |  | 0.153 |  |  |  | - |  |  | - |  |  |  | 0.242 |  |  | 0.199 |  |  |  | 0.182 |  |  | - |  | - |  | 0.423 |  | 0.371 |  | 0.355 |  |  | - |  | - |
|  |  |  | 95% |  |  |  | 0.285 |  |  | 0.218 |  |  | 0.185 |  |  |  | - |  |  | - |  |  |  | 0.394 |  |  | 0.333 |  |  |  | 0.347 |  |  | - |  | - |  | 1.573 |  | 1.578 |  | 1.499 |  |  | - |  | - |
|  |  |  | Mean |  |  |  | 0.235 |  |  | 0.181 |  |  | 0.155 |  |  |  | - |  |  | - |  |  |  | 0.260 |  |  | 0.218 |  |  |  | 0.204 |  |  | - |  | - |  | 0.599 |  | 0.569 |  | 0.563 |  |  | - |  | - |
| RU |  |  |  |  |  |  | 19.901 |  |  | 20.004 |  |  | 20.336 |  |  |  | - |  |  | - |  |  |  | 41.500 |  |  | 41.596 |  |  |  | 42.071 |  |  | - |  | - |  | 62.487 |  | 62.860 |  | 63.484 |  |  | - |  | - |
| 𝜆 |  |  |  |  |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  | 3 |  |  |  |  |  |  |  |  |  |
| Notes: |  |  |  |  |  |  | Source 12, R1-163798, CN delay=6ms, UL access delay = 13TTI, FTP traffic model 3 Total control overhead per subframe compared to all available REs in the sTTIs: 14 OS:15%, 7 OS: 17%, 3/4 OS: 19% Maximum number of UEs that can be scheduled per sTTI: not modelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 14 OS |  |  | 7OS |  |  | 3/4 OS |  |  |  | 2OS |  |  | 1OS |  |  |  | 14 OS |  |  | 7OS |  |  |  | 3/4 OS |  |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 13.795 | 17.953 (30.14%) |  |  |  |  | 8.937 | 10.47 (17.16%) |  |  |  |  | 5.798 | 6.595 (13.75%) |  |  |  |  |
|  | 50% | 26.787 | 41.436 (54.69%) |  |  |  |  | 23.271 | 35.275 (51.58%) |  |  |  |  | 20.555 | 30.35 (47.66%) |  |  |  |  |
|  | 95% | 47.939 | 74.295 (54.98%) |  |  |  |  | 44.29 | 69.773 (57.54%) |  |  |  |  | 42.41 | 65.655 (54.81%) |  |  |  |  |
|  | Mean | 29.09 | 43.704 (50.24%) |  |  |  |  | 24.688 | 36.945 (49.65%) |  |  |  |  | 21.512 | 31.07 (44.43%) |  |  |  |  |
| DL: Delay CDF [s] | 5% | 83.397 | 53.805 (-35.48%) |  |  |  |  | 90.41 | 57.327 (-36.59%) |  |  |  |  | 94.29 | 60.945 (-35.36%) |  |  |  |  |
|  | 50% | 149.288 | 96.47 (-35.38%) |  |  |  |  | 171.8 | 113.4 (-33.99%) |  |  |  |  | 194.567 | 131.767 (-32.28%) |  |  |  |  |
|  | 95% | 289.915 | 222.895 (-23.12%) |  |  |  |  | 446.09 | 382.32 (-14.3%) |  |  |  |  | 690.81 | 610.71 (-11.6%) |  |  |  |  |
|  | Mean | 163.475 | 127.66 (-21.91%) |  |  |  |  | 206.952 | 152.708 (-26.21%) |  |  |  |  | 263.808 | 207.076 (-21.51%) |  |  |  |  |
| RU |  | 0.151 | 0.141 (-6.53%) |  |  |  |  | 0.381 | 0.349 (-8.41%) |  |  |  |  | 0.584 | 0.569 (-2.56%) |  |  |  |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.5 |  |  |  |  |  | 0.75 |  |  |  |  |  |
| Notes: |  | Source 11, R1-165238, CN Delay=2ms; max scheduled users=10; For slot TTI and legacy TTI: UL access delay: 12 (s)TTI, RTT: 8 (s)TTI. InitSSThresholdInMSS: 45. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  |  | 5% |  |  |  | 17.272 |  |  | 18.043 (4.46%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.413 |  |  | 11.52 (10.64%) |  |  |  |  |  |  |  |  |  |  | 6.054 |  | 6.763 (11.72%) |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  | 37.183 |  |  | 43.463 (16.89%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 31.51 |  |  | 37.55 (19.17%) |  |  |  |  |  |  |  |  |  |  | 27.367 |  | 32.067 (17.17%) |  |  |  |  |  |  |  |
|  |  |  | 95% |  |  |  | 66.146 |  |  | 78.715 (19%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 60.05 |  |  | 75.06 (25%) |  |  |  |  |  |  |  |  |  |  | 58.006 |  | 70.27 (21.14%) |  |  |  |  |  |  |  |
|  |  |  | Mean |  |  |  | 39.755 |  |  | 45.384 (14.16%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 32.87 |  |  | 39.205 (19.28%) |  |  |  |  |  |  |  |  |  |  | 28.194 |  | 33.104 (17.41%) |  |  |  |  |  |  |  |
| DL: Delay CDF [s] |  |  | 5% |  |  |  | 60.472 |  |  | 50.785 (-16.02%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 66.575 |  |  | 53.28 (-19.97%) |  |  |  |  |  |  |  |  |  |  | 68.968 |  | 56.923 (-17.46%) |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  | 107.55 |  |  | 91.963 (-14.49%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 126.9 |  |  | 106.567 (-16.02%) |  |  |  |  |  |  |  |  |  |  | 146.05 |  | 124.8 (-14.55%) |  |  |  |  |  |  |  |
|  |  |  | 95% |  |  |  | 231.985 |  |  | 221.115 (-4.69%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 382.15 |  |  | 347.26 (-9.13%) |  |  |  |  |  |  |  |  |  |  | 658.73 |  | 595.21 (-9.64%) |  |  |  |  |  |  |  |
|  |  |  | Mean |  |  |  | 129.275 |  |  | 119.229 (-7.77%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 161.821 |  |  | 141.984 (-12.26%) |  |  |  |  |  |  |  |  |  |  | 223.855 |  | 198.802 (-11.19%) |  |  |  |  |  |  |  |
| RU |  |  |  |  |  |  | 0.143 |  |  | 0.145 (1.64%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.372 |  |  | 0.326 (-12.17%) |  |  |  |  |  |  |  |  |  |  | 0.578 |  | 0.54 (-6.46%) |  |  |  |  |  |  |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.5 |  |  |  |  |  | 0.75 |  |  |  |  |  |
| Notes: |  | Source 11, R1-165238, CN Delay=2ms; max scheduled users=10; for UL access delay and RTT settings please see the second row of this Table; InitSSThresholdInMSS：45, UL access delay/RTT in (s)TTI: 5/5 for 14os and 8/8 for 7os |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  |  | 5% |  |  |  | 16.706 |  |  | 17.953 (7.46%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 9.988 |  |  | 10.47 (4.82%) |  |  |  |  |  |  |  |  |  |  | 5.733 |  | 6.595 (15.03%) |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  | 32.846 |  |  | 41.436 (26.15%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 28.975 |  |  | 35.275 (21.74%) |  |  |  |  |  |  |  |  |  |  | 25.338 |  | 30.35 (19.78%) |  |  |  |  |  |  |  |
|  |  |  | 95% |  |  |  | 59.394 |  |  | 74.295 (25.09%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 55.212 |  |  | 69.773 (26.37%) |  |  |  |  |  |  |  |  |  |  | 52.1 |  | 65.655 (26.02%) |  |  |  |  |  |  |  |
|  |  |  | Mean |  |  |  | 36.121 |  |  | 43.704 (20.99%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 30.387 |  |  | 36.945 (21.58%) |  |  |  |  |  |  |  |  |  |  | 25.913 |  | 31.07 (19.9%) |  |  |  |  |  |  |  |
| DL: Delay CDF [s] |  |  | 5% |  |  |  | 67.313 |  |  | 53.805 (-20.07%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 72.466 |  |  | 57.327 (-20.89%) |  |  |  |  |  |  |  |  |  |  | 76.767 |  | 60.945 (-20.61%) |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  | 121.725 |  |  | 96.47 (-20.75%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 138.125 |  |  | 113.4 (-17.9%) |  |  |  |  |  |  |  |  |  |  | 157.75 |  | 131.767 (-16.47%) |  |  |  |  |  |  |  |
|  |  |  | 95% |  |  |  | 239.238 |  |  | 222.895 (-6.83%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 401.535 |  |  | 382.32 (-4.79%) |  |  |  |  |  |  |  |  |  |  | 696.35 |  | 610.71 (-12.3%) |  |  |  |  |  |  |  |
|  |  |  | Mean |  |  |  | 130.815 |  |  | 127.66 (-2.41%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 174.987 |  |  | 152.708 (-12.73%) |  |  |  |  |  |  |  |  |  |  | 239.18 |  | 207.076 (-13.42%) |  |  |  |  |  |  |  |
| RU |  |  |  |  |  |  | 0.147 |  |  | 0.141 (-3.88%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.375 |  |  | 0.349 (-6.79%) |  |  |  |  |  |  |  |  |  |  | 0.583 |  | 0.569 (-2.44%) |  |  |  |  |  |  |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.5 |  |  |  |  |  | 0.75 |  |  |  |  |  |
| Notes: |  | Source 11, R1-165238, CN Delay=2ms; max scheduled users=10; for UL access delay and RTT settings please see the second row of this Table; InitSSThresholdInMSS:45, UL access delay/RTT in (s)TTI: 9/5 for 14os and 12/8 for 7os |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  |  | 5% |  |  |  | 19.464 |  |  | 20.975 (7.76%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 11.127 |  |  | 13.075 (17.51%) |  |  |  |  |  |  |  |  |  |  | 6.727 |  | 7.605 (13.05%) |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  | 46.657 |  |  | 56.613 (21.34%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 39.233 |  |  | 48.48 (23.57%) |  |  |  |  |  |  |  |  |  |  | 31.813 |  | 38.467 (20.92%) |  |  |  |  |  |  |  |
|  |  |  | 95% |  |  |  | 83.727 |  |  | 101.669 (21.43%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 77.41 |  |  | 95.425 (23.27%) |  |  |  |  |  |  |  |  |  |  | 72.983 |  | 90.193 (23.58%) |  |  |  |  |  |  |  |
|  |  |  | Mean |  |  |  | 49.466 |  |  | 58.696 (18.66%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 40.367 |  |  | 49.251 (22.01%) |  |  |  |  |  |  |  |  |  |  | 33.954 |  | 40.506 (19.3%) |  |  |  |  |  |  |  |
| DL: Delay CDF [s] |  |  | 5% |  |  |  | 47.787 |  |  | 39.331 (-17.69%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 51.69 |  |  | 41.975 (-18.79%) |  |  |  |  |  |  |  |  |  |  | 54.818 |  | 44.307 (-19.17%) |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  | 85.743 |  |  | 70.688 (-17.56%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 101.933 |  |  | 82.5 (-19.06%) |  |  |  |  |  |  |  |  |  |  | 125.775 |  | 103.933 (-17.37%) |  |  |  |  |  |  |  |
|  |  |  | 95% |  |  |  | 205.48 |  |  | 190.975 (-7.06%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 359.02 |  |  | 307.05 (-14.48%) |  |  |  |  |  |  |  |  |  |  | 592.765 |  | 525.88 (-11.28%) |  |  |  |  |  |  |  |
|  |  |  | Mean |  |  |  | 101.735 |  |  | 90.064 (-11.47%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 145.23 |  |  | 118.373 (-18.49%) |  |  |  |  |  |  |  |  |  |  | 201.374 |  | 173.454 (-13.86%) |  |  |  |  |  |  |  |
| RU |  |  |  |  |  |  | 0.139 |  |  | 0.134 (-3.41%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.351 |  |  | 0.308 (-12.31%) |  |  |  |  |  |  |  |  |  |  | 0.564 |  | 0.532 (-5.61%) |  |  |  |  |  |  |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.5 |  |  |  |  |  | 0.75 |  |  |  |  |  |
| Notes: |  | Source 11, R1-165238, CN Delay=2ms; max scheduled users=10; for UL access delay and RTT settings please see the second row of this Table; InitSSThresholdInMSS:45, UL access delay/RTT in (s)TTI: 3/3 for 14os and 4/4 for 7os |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  |  | 5% |  |  |  | 17.087 |  |  | 20.448 (19.67%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 10.559 |  |  | 11.92 (12.89%) |  |  |  |  |  |  |  |  |  |  | 6.143 |  | 7.276 (18.45%) |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  | 37.956 |  |  | 55.108 (45.19%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 31.75 |  |  | 44.825 (41.18%) |  |  |  |  |  |  |  |  |  |  | 27.033 |  | 34.888 (29.05%) |  |  |  |  |  |  |  |
|  |  |  | 95% |  |  |  | 126.48 |  |  | 161.394 (27.6%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 113.565 |  |  | 159.06 (40.06%) |  |  |  |  |  |  |  |  |  |  | 101.25 |  | 138.295 (36.59%) |  |  |  |  |  |  |  |
|  |  |  | Mean |  |  |  | 53.976 |  |  | 70.378 (30.39%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 42.089 |  |  | 55.386 (31.59%) |  |  |  |  |  |  |  |  |  |  | 34.17 |  | 43.941 (28.6%) |  |  |  |  |  |  |  |
| DL: Delay CDF [s] |  |  | 5% |  |  |  | 31.62 |  |  | 24.761 (-21.69%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 35.218 |  |  | 25.14 (-28.62%) |  |  |  |  |  |  |  |  |  |  | 39.55 |  | 28.905 (-26.92%) |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  | 105.44 |  |  | 72.55 (-31.19%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 125.975 |  |  | 89.05 (-29.31%) |  |  |  |  |  |  |  |  |  |  | 148.033 |  | 114.725 (-22.5%) |  |  |  |  |  |  |  |
|  |  |  | 95% |  |  |  | 234.14 |  |  | 195.053 (-16.69%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 380.065 |  |  | 333.48 (-12.26%) |  |  |  |  |  |  |  |  |  |  | 653.3 |  | 552.548 (-15.42%) |  |  |  |  |  |  |  |
|  |  |  | Mean |  |  |  | 111.902 |  |  | 98.004 (-12.42%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 155.288 |  |  | 126.006 (-18.86%) |  |  |  |  |  |  |  |  |  |  | 218.613 |  | 182.045 (-16.73%) |  |  |  |  |  |  |  |
| RU |  |  |  |  |  |  | 0.137 |  |  | 0.13 (-5.51%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.348 |  |  | 0.322 (-7.42%) |  |  |  |  |  |  |  |  |  |  | 0.56 |  | 0.546 (-2.47%) |  |  |  |  |  |  |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.5 |  |  |  |  |  | 0.75 |  |  |  |  |  |
| Notes: |  | Source 11, R1-165238, CN Delay=2ms; max scheduled users=10; For slot TTI and legacy TTI: UL access delay: 12 (s)TTI, RTT: 8 (s)TTI. InitSSThresholdInMSS: 150. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  |  | 5% |  |  |  | 19.485 |  |  | 20.075 (3.03%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 12.553 |  |  | 12.755 (1.61%) |  |  |  |  |  |  |  |  |  |  | 6.655 |  | 7.27 (9.24%) |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  | 50.395 |  |  | 57.438 (13.97%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 40.78 |  |  | 46.925 (15.07%) |  |  |  |  |  |  |  |  |  |  | 32.05 |  | 36 (12.32%) |  |  |  |  |  |  |  |
|  |  |  | 95% |  |  |  | 152.11 |  |  | 162.361 (6.74%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 146.968 |  |  | 161.367 (9.8%) |  |  |  |  |  |  |  |  |  |  | 130.808 |  | 138.99 (6.25%) |  |  |  |  |  |  |  |
|  |  |  | Mean |  |  |  | 66.726 |  |  | 70.003 (4.91%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 51.681 |  |  | 57.962 (12.15%) |  |  |  |  |  |  |  |  |  |  | 41.247 |  | 44.396 (7.64%) |  |  |  |  |  |  |  |
| DL: Delay CDF [s] |  |  | 5% |  |  |  | 26.295 |  |  | 24.639 (-6.3%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 27.218 |  |  | 24.759 (-9.03%) |  |  |  |  |  |  |  |  |  |  | 30.593 |  | 28.755 (-6.01%) |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  | 79.406 |  |  | 69.625 (-12.32%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 98.1 |  |  | 85.225 (-13.12%) |  |  |  |  |  |  |  |  |  |  | 124.775 |  | 111 (-11.04%) |  |  |  |  |  |  |  |
|  |  |  | 95% |  |  |  | 205.915 |  |  | 197.825 (-3.93%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 318.74 |  |  | 313.445 (-1.66%) |  |  |  |  |  |  |  |  |  |  | 602.215 |  | 547.19 (-9.14%) |  |  |  |  |  |  |  |
|  |  |  | Mean |  |  |  | 95.349 |  |  | 101.24 (6.18%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 128.862 |  |  | 120.934 (-6.15%) |  |  |  |  |  |  |  |  |  |  | 192.924 |  | 179.02 (-7.21%) |  |  |  |  |  |  |  |
| RU |  |  |  |  |  |  | 0.133 |  |  | 0.131 (-1.44%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.333 |  |  | 0.304 (-8.83%) |  |  |  |  |  |  |  |  |  |  | 0.555 |  | 0.527 (-5.07%) |  |  |  |  |  |  |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.5 |  |  |  |  |  | 0.75 |  |  |  |  |  |
| Notes: |  | Source 11, R1-165238, CN Delay=2ms; max scheduled users=10; for UL access delay and RTT settings please see the second row of this Table; InitSSThresholdInMSS:150, UL access delay/RTT in (s)TTI: 5/5 for 14os and 8/8 for 7os |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  |  | 5% |  |  |  | 18.287 |  |  | 20.448 (11.82%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 11.128 |  |  | 11.92 (7.11%) |  |  |  |  |  |  |  |  |  |  | 6.628 |  | 7.276 (9.79%) |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  | 45.16 |  |  | 55.108 (22.03%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 37.95 |  |  | 44.825 (18.12%) |  |  |  |  |  |  |  |  |  |  | 30.36 |  | 34.888 (14.91%) |  |  |  |  |  |  |  |
|  |  |  | 95% |  |  |  | 141.793 |  |  | 161.394 (13.82%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 134.929 |  |  | 159.06 (17.88%) |  |  |  |  |  |  |  |  |  |  | 121.795 |  | 138.295 (13.55%) |  |  |  |  |  |  |  |
|  |  |  | Mean |  |  |  | 62.54 |  |  | 70.378 (12.53%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 48.317 |  |  | 55.386 (14.63%) |  |  |  |  |  |  |  |  |  |  | 38.867 |  | 43.941 (13.05%) |  |  |  |  |  |  |  |
| DL: Delay CDF [s] |  |  | 5% |  |  |  | 28.205 |  |  | 24.761 (-12.21%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 29.671 |  |  | 25.14 (-15.27%) |  |  |  |  |  |  |  |  |  |  | 32.805 |  | 28.905 (-11.89%) |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  | 88.48 |  |  | 72.55 (-18%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 105.45 |  |  | 89.05 (-15.55%) |  |  |  |  |  |  |  |  |  |  | 131.8 |  | 114.725 (-12.96%) |  |  |  |  |  |  |  |
|  |  |  | 95% |  |  |  | 219.14 |  |  | 195.053 (-10.99%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 354.015 |  |  | 333.48 (-5.8%) |  |  |  |  |  |  |  |  |  |  | 599.49 |  | 552.548 (-7.83%) |  |  |  |  |  |  |  |
|  |  |  | Mean |  |  |  | 104.422 |  |  | 98.004 (-6.15%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 140.766 |  |  | 126.006 (-10.49%) |  |  |  |  |  |  |  |  |  |  | 199.616 |  | 182.045 (-8.8%) |  |  |  |  |  |  |  |
| RU |  |  |  |  |  |  | 0.134 |  |  | 0.13 (-3.54%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.334 |  |  | 0.322 (-3.64%) |  |  |  |  |  |  |  |  |  |  | 0.556 |  | 0.546 (-1.79%) |  |  |  |  |  |  |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.5 |  |  |  |  |  | 0.75 |  |  |  |  |  |
| Notes: |  | Source 11, R1-165238, CN Delay=2ms; max scheduled users=10; for UL access delay and RTT settings please see the second row of this Table; InitSSThresholdInMSS:150, UL access delay/RTT in (s)TTI: 9/5 for 14os and 12/8 for 7os |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS | 14 OS | 7OS |  | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  |  | 5% |  |  |  | 20.28 |  |  | 21.5 (6.02%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 12.745 |  |  | 13.91 (9.14%) |  |  |  |  |  |  |  |  |  |  | 6.983 |  | 7.838 (12.24%) |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  | 60.814 |  |  | 68.367 (12.42%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 46.05 |  |  | 54.267 (17.84%) |  |  |  |  |  |  |  |  |  |  | 35.233 |  | 40.617 (15.28%) |  |  |  |  |  |  |  |
|  |  |  | 95% |  |  |  | 158.669 |  |  | 162.704 (2.54%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 156.482 |  |  | 161.835 (3.42%) |  |  |  |  |  |  |  |  |  |  | 145.41 |  | 152.525 (4.89%) |  |  |  |  |  |  |  |
|  |  |  | Mean |  |  |  | 73.057 |  |  | 77.088 (5.52%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 56.671 |  |  | 63.249 (11.61%) |  |  |  |  |  |  |  |  |  |  | 44.93 |  | 49.449 (10.06%) |  |  |  |  |  |  |  |
| DL: Delay CDF [s] |  |  | 5% |  |  |  | 25.217 |  |  | 24.596 (-2.46%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 25.549 |  |  | 24.723 (-3.24%) |  |  |  |  |  |  |  |  |  |  | 27.58 |  | 26.275 (-4.73%) |  |  |  |  |  |  |  |
|  |  |  | 50% |  |  |  | 65.786 |  |  | 58.475 (-11.11%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 86.78 |  |  | 73.7 (-15.07%) |  |  |  |  |  |  |  |  |  |  | 113.55 |  | 98.525 (-13.23%) |  |  |  |  |  |  |  |
|  |  |  | 95% |  |  |  | 197.34 |  |  | 186.05 (-5.72%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 314.32 |  |  | 287.14 (-8.65%) |  |  |  |  |  |  |  |  |  |  | 574.42 |  | 511.325 (-10.98%) |  |  |  |  |  |  |  |
|  |  |  | Mean |  |  |  | 84.869 |  |  | 83.8 (-1.26%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 120.54 |  |  | 108.705 (-9.82%) |  |  |  |  |  |  |  |  |  |  | 181.446 |  | 162.607 (-10.38%) |  |  |  |  |  |  |  |
| RU |  |  |  |  |  |  | 0.13 |  |  | 0.13 (0.03%) |  |  |  |  |  |  |  |  |  |  |  |  |  | 0.327 |  |  | 0.293 (-10.48%) |  |  |  |  |  |  |  |  |  |  | 0.547 |  | 0.517 (-5.41%) |  |  |  |  |  |  |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.5 |  |  |  |  |  | 0.75 |  |  |  |  |  |
| Notes: |  | Source 11, R1-165238, CN Delay=2ms; max scheduled users=10; for UL access delay and RTT settings please see the second row of this Table; InitSSThresholdInMSS:150, UL access delay/RTT in (s)TTI: 3/3 for 14os and 4/4 for 7os |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  | 5% |  |  | 3.81 |  |  |  | 4.11 |  |  | 4.19 |  |  | 3.86 |  |  |  |  |  |  | 3.16 |  |  |  | 3.06 |  |  | 3.04 |  |  | 2.77 |  |  |  |  |  | 1.77 |  | 1.68 |  | 1.68 |  | 1.55 |  |  |  |  |
|  |  | 50% |  |  | 7.91 |  |  |  | 9.14 |  |  | 9.88 |  |  | 9.60 |  |  |  |  |  |  | 6.78 |  |  |  | 7.23 |  |  | 7.65 |  |  | 7.15 |  |  |  |  |  | 4.73 |  | 4.66 |  | 4.99 |  | 4.77 |  |  |  |  |
|  |  | 95% |  |  | 12.33 |  |  |  | 16.08 |  |  | 18.20 |  |  | 19.08 |  |  |  |  |  |  | 11.97 |  |  |  | 15.17 |  |  | 16.67 |  |  | 17.77 |  |  |  |  |  | 10.34 |  | 12.16 |  | 13.72 |  | 13.96 |  |  |  |  |
|  |  | Mean |  |  | 8.05 |  |  |  | 9.55 |  |  | 10.47 |  |  | 10.27 |  |  |  |  |  |  | 7.18 |  |  |  | 8.01 |  |  | 8.64 |  |  | 8.48 |  |  |  |  |  | 5.20 |  | 5.58 |  | 6.11 |  | 5.96 |  |  |  |  |
| DL: Delay CDF [s] |  | 5% |  |  | 0.34 |  |  |  | 0.26 |  |  | 0.23 |  |  | 0.22 |  |  |  |  |  |  | 0.35 |  |  |  | 0.28 |  |  | 0.25 |  |  | 0.25 |  |  |  |  |  | 0.41 |  | 0.35 |  | 0.31 |  | 0.30 |  |  |  |  |
|  |  | 50% |  |  | 0.53 |  |  |  | 0.46 |  |  | 0.42 |  |  | 0.44 |  |  |  |  |  |  | 0.62 |  |  |  | 0.58 |  |  | 0.55 |  |  | 0.59 |  |  |  |  |  | 0.89 |  | 0.89 |  | 0.85 |  | 0.89 |  |  |  |  |
|  |  | 95% |  |  | 1.12 |  |  |  | 1.03 |  |  | 1.03 |  |  | 1.11 |  |  |  |  |  |  | 1.33 |  |  |  | 1.39 |  |  | 1.40 |  |  | 1.53 |  |  |  |  |  | 2.38 |  | 2.50 |  | 2.51 |  | 2.72 |  |  |  |  |
|  |  | Mean |  |  | 0.60 |  |  |  | 0.53 |  |  | 0.50 |  |  | 0.52 |  |  |  |  |  |  | 0.70 |  |  |  | 0.68 |  |  | 0.64 |  |  | 0.70 |  |  |  |  |  | 1.09 |  | 1.08 |  | 1.04 |  | 1.09 |  |  |  |  |
| RU |  |  |  |  | 19.8 |  |  |  | 19.2 |  |  | 20.2 |  |  | 19.8 |  |  |  |  |  |  | 35.8 |  |  |  | 36.2 |  |  | 36.6 |  |  | 39.2 |  |  |  |  |  | 63.2 |  | 63.8 |  | 62.8 |  | 63.8 |  |  |  |  |
| 𝜆 |  | 0.06 |  |  |  |  |  | 0.10 |  |  |  |  |  | 0.17 |  |  |  |  |  |
| Notes: |  | Source 7, R1-165418, File size: 500 KB, Macro scenario, 6 ms core network delay, Dynamic DCI overhead, FTP model 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.85 | 3.95 | 4.03 |  | 4.01 |  | 2.96 | 2.87 | 2.73 |  | 2.47 |  | 1.75 | 1.67 | 1.47 |  | 1.45 |  |
|  | 50% | 7.90 | 8.87 | 9.42 |  | 9.42 |  | 6.64 | 6.95 | 7.26 |  | 6.73 |  | 4.69 | 4.55 | 4.19 |  | 4.21 |  |
|  | 95% | 12.25 | 16.01 | 18.30 |  | 18.66 |  | 11.95 | 14.91 | 16.75 |  | 16.81 |  | 10.32 | 12.30 | 12.70 |  | 12.57 |  |
|  | Mean | 8.02 | 9.30 | 10.02 |  | 10.17 |  | 7.10 | 7.67 | 8.13 |  | 8.02 |  | 5.21 | 5.49 | 5.38 |  | 5.33 |  |
| DL: Delay CDF [s] | 5% | 0.34 | 0.26 | 0.23 |  | 0.23 |  | 0.35 | 0.28 | 0.25 |  | 0.25 |  | 0.41 | 0.34 | 0.33 |  | 0.33 |  |
|  | 50% | 0.53 | 0.47 | 0.44 |  | 0.45 |  | 0.63 | 0.61 | 0.58 |  | 0.63 |  | 0.89 | 0.92 | 1.00 |  | 1.00 |  |
|  | 95% | 1.10 | 1.07 | 1.05 |  | 1.06 |  | 1.42 | 1.49 | 1.56 |  | 1.79 |  | 2.40 | 2.52 | 3.09 |  | 2.90 |  |
|  | Mean | 0.61 | 0.54 | 0.53 |  | 0.52 |  | 0.71 | 0.72 | 0.70 |  | 0.75 |  | 1.08 | 1.10 | 1.23 |  | 1.26 |  |
| RU |  | 19.8 | 19.8 | 21 |  | 21.2 |  | 35.4 | 38 | 37.6 |  | 39.4 |  | 64.4 | 65.4 | 68 |  | 68.8 |  |
| 𝜆 |  | 0.06 |  |  |  |  |  | 0.10 |  |  |  |  |  | 0.17 |  |  |  |  |  |
| Notes: |  | Source 7, R1-165418, File size: 500 KB, Macro scenario, 6 ms core network delay, static DCI overhead, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-15%, 7OS-18.0%, 3/4OS-19%, 2OS-21.5%, 1OS-28%; Maximum number of UEs that can be scheduled per sTTI: 14OS-5UEs, 7OS-2UEs, 3/4OS-1UEs, 2OS-1UEs, 1OS-1UE, FTP model 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.49 | 3.65 | 3.33 |  | 3.58 |  | 2.18 | 2.31 | 2.22 |  | 2.04 |  | 1.4 | 1.15 | 1.19 |  | 0.97 |  |
|  | 50% | 7.86 | 8.6 | 8.86 |  | 9.7 |  | 5.91 | 6.11 | 6.81 |  | 6.26 |  | 3.68 | 3.53 | 3.88 |  | 3.05 |  |
|  | 95% | 12.08 | 15.66 | 18.31 |  | 18.12 |  | 10.81 | 13.85 | 15.36 |  | 15.18 |  | 8.69 | 10.29 | 11.47 |  | 10.87 |  |
|  | Mean | 7.84 | 9.27 | 9.55 |  | 10.04 |  | 6.11 | 6.9 | 7.64 |  | 7.11 |  | 4.22 | 4.22 | 4.85 |  | 4.07 |  |
| DL: Delay CDF [s] | 5% | 0.347 | 0.269 | 0.23 |  | 0.232 |  | 0.389 | 0.303 | 0.268 |  | 0.277 |  | 0.485 | 0.408 | 0.368 |  | 0.386 |  |
|  | 50% | 0.535 | 0.481 | 0.467 |  | 0.433 |  | 0.706 | 0.682 | 0.623 |  | 0.671 |  | 1.141 | 1.17 | 1.089 |  | 1.357 |  |
|  | 95% | 1.208 | 1.161 | 1.283 |  | 1.178 |  | 1.934 | 1.796 | 1.925 |  | 2.086 |  | 2.996 | 3.668 | 3.603 |  | 4.397 |  |
|  | Mean | 0.629 | 0.556 | 0.576 |  | 0.529 |  | 0.87 | 0.818 | 0.789 |  | 0.844 |  | 1.339 | 1.527 | 1.432 |  | 1.748 |  |
| RU |  | 22 | 22 | 25 |  | 22.5 |  | 49.5 | 49 | 50.5 |  | 50 |  | 74 | 78.5 | 77 |  | 79 |  |
| 𝜆 |  | 0.07 |  |  |  |  |  | 0.14 |  |  |  |  |  | 0.25 |  |  |  |  |  |
| Notes: |  | Source 7, R1-165418, File size: 500 KB, Macro Scenario, 6 ms core network delay, static DCI overhead, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-15%, 7OS-18.0%, 3/4OS-19%, 2OS-21.5%, 1OS-28%; Maximum number of UEs that can be scheduled per sTTI: 14OS-5UEs, 7OS-2UEs, 3/4OS-1UEs, 2OS-1UEs, 1OS-1UE, FTP model 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.89 | 3.88 | 3.91 |  | 3.77 |  | 2.36 | 2.28 | 2.42 |  | 2.02 |  | 1.36 | 1.25 | 1.2 |  | 1.13 |  |
|  | 50% | 7.82 | 8.43 | 9.12 |  | 9.16 |  | 5.57 | 6.04 | 6.45 |  | 5.85 |  | 3.77 | 3.79 | 3.83 |  | 3.33 |  |
|  | 95% | 12.21 | 15.97 | 17.79 |  | 18.32 |  | 11.2 | 13.67 | 15.42 |  | 15.09 |  | 8.94 | 10.8 | 10.99 |  | 10.5 |  |
|  | Mean | 8 | 9.15 | 9.86 |  | 9.94 |  | 6.08 | 6.78 | 7.35 |  | 6.89 |  | 4.34 | 4.62 | 4.62 |  | 4.6 |  |
| DL: Delay CDF [s] | 5% | 0.344 | 0.263 | 0.236 |  | 0.228 |  | 0.375 | 0.308 | 0.272 |  | 0.279 |  | 0.47 | 0.388 | 0.382 |  | 0.4 |  |
|  | 50% | 0.535 | 0.496 | 0.46 |  | 0.456 |  | 0.753 | 0.693 | 0.65 |  | 0.711 |  | 1.109 | 1.107 | 1.104 |  | 1.261 |  |
|  | 95% | 1.082 | 1.087 | 1.094 |  | 1.124 |  | 1.786 | 1.867 | 1.76 |  | 2.126 |  | 3.102 | 3.365 | 3.522 |  | 4.136 |  |
|  | Mean | 0.607 | 0.557 | 0.536 |  | 0.534 |  | 0.867 | 0.846 | 0.779 |  | 0.874 |  | 1.355 | 1.384 | 1.434 |  | 1.657 |  |
| RU |  | 20 | 21 | 22 |  | 22.2 |  | 45.8 | 48.4 | 48 |  | 51 |  | 72.4 | 73.4 | 74.6 |  | 77.2 |  |
| 𝜆 |  | 0.07 |  |  |  |  |  | 0.14 |  |  |  |  |  | 0.25 |  |  |  |  |  |
| Notes: |  | Source 7, R1-165418, File size: 500 KB, Macro Scenario, 6 ms core network delay, dynamic DCI overhead, FTP model 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.1.1-4: Results with 1MB file transfer

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 4.20 | 4.42 | 3.87 | 3.48 | 3.35 | 3.68 | 4.01 | 3.51 | 3.01 | 2.95 | 2.99 |  | 3.32 |  | 2.83 |  | 2.68 |  | 2.64 |
|  | 50% | 12.88 | 14.93 | 13.96 | 12.43 | 11.97 | 11.41 | 12.83 | 12.11 | 11.22 | 10.52 | 10.45 |  | 12.17 |  | 10.99 |  | 9.63 |  | 8.84 |
|  | 95% | 15.87 | 21.56 | 26.00 | 27.26 | 28.13 | 15.87 | 21.56 | 25.85 | 27.50 | 28.34 | 15.79 |  | 21.42 |  | 25.6 |  | 26.26 |  | 26.07 |
|  | Mean | 11.56 | 14.10 | 14.20 | 13.52 | 13.36 | 10.80 | 13.15 | 13.10 | 12.66 | 12.26 | 10.13 |  | 12.37 |  | 12.33 |  | 11.47 |  | 10.88 |
| DL: Delay CDF [s] | 5% | 0.503 | 0.371 | 0.308 | 0.292 | 0.284 | 0.503 | 0.371 | 0.310 | 0.290 | 0.282 | 0.506 |  | 0.373 |  | 0.312 |  | 0.304 |  | 0.306 |
|  | 50% | 0.617 | 0.534 | 0.573 | 0.644 | 0.668 | 0.705 | 0.635 | 0.668 | 0.713 | 0.772 | 0.780 |  | 0.672 |  | 0.737 |  | 0.862 |  | 0.930 |
|  | 95% | 1.880 | 1.811 | 2.000 | 2.275 | 2.364 | 2.160 | 1.990 | 2.230 | 2.636 | 2.715 | 2.674 |  | 2.413 |  | 2.813 |  | 2.986 |  | 3.017 |
|  | Mean | 0.846 | 0.743 | 0.796 | 0.872 | 0.907 | 0.956 | 0.843 | 0.923 | 0.992 | 1.055 | 1.079 |  | 0.964 |  | 1.056 |  | 1.136 |  | 1.199 |
| RU |  | 0.2 | 0.19 | 0.21 | 0.23 | 0.24 | 0.41 | 0.39 | 0.42 | 0.45 | 0.46 | 0.6 |  | 0.58 |  | 0.62 |  | 0.64 |  | 0.66 |
| 𝜆 |  | 0.04 |  |  |  |  | 0.07 |  |  |  |  | 0.08 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 6, R1-162703, CN delay = 6ms, 8 TTIs HARQ RTT, 13-TTI UL access delay, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.3%, 7OS-28.0%, 3/4OS-35.9%, 2OS-38.3%, 1OS-38.3%, Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-4UEs, 3/4OS-3UEs, 2OS-2UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 4.20 | 4.41 | 3.79 | 3.43 | 3.32 | 3.68 | 3.83 | 3.25 | 2.91 | 2.98 |  | 2.99 |  | 3.16 |  | 2.86 |  | 2.75 | 2.56 |
|  | 50% | 12.88 | 14.77 | 12.96 | 11.97 | 11.81 | 11.41 | 12.75 | 11.76 | 10.82 | 10.46 |  | 10.45 |  | 12 |  | 10.84 |  | 9.53 | 8.87 |
|  | 95% | 15.87 | 21.19 | 25.26 | 26.58 | 27.38 | 15.87 | 21.28 | 25.15 | 26.63 | 27.43 |  | 15.79 |  | 21.11 |  | 24.73 |  | 25.13 | 25.31 |
|  | Mean | 11.56 | 13.91 | 13.80 | 13.17 | 13.05 | 10.80 | 12.97 | 12.73 | 12.26 | 11.93 |  | 10.13 |  | 12.15 |  | 11.93 |  | 11.11 | 10.62 |
| DL: Delay CDF [s] | 5% | 0.503 | 0.376 | 0.316 | 0.300 | 0.290 | 0.503 | 0.376 | 0.318 | 0.300 | 0.290 |  | 0.506 |  | 0.379 |  | 0.323 |  | 0.318 | 0.315 |
|  | 50% | 0.617 | 0.541 | 0.617 | 0.662 | 0.677 | 0.705 | 0.639 | 0.686 | 0.741 | 0.777 |  | 0.780 |  | 0.693 |  | 0.744 |  | 0.865 | 0.924 |
|  | 95% | 1.880 | 1.816 | 2.071 | 2.330 | 2.390 | 2.160 | 2.088 | 2.450 | 2.693 | 2.642 |  | 2.674 |  | 2.480 |  | 2.799 |  | 2.907 | 3.096 |
|  | Mean | 0.846 | 0.748 | 0.813 | 0.889 | 0.919 | 0.956 | 0.850 | 0.951 | 1.020 | 1.081 |  | 1.079 |  | 0.971 |  | 1.074 |  | 1.158 | 1.215 |
| RU |  | 0.2 | 0.19 | 0.21 | 0.23 | 0.24 | 0.41 | 0.39 | 0.43 | 0.45 | 0.46 |  | 0.6 |  | 0.57 |  | 0.62 |  | 0.64 | 0.65 |
| 𝜆 |  | 0.04 |  |  |  |  | 0.07 |  |  |  |  |  | 0.08 |  |  |  |  |  |  |  |
| Notes: |  | Source 6, R1-162704, CN delay = 6ms, 8 ms HARQ RTT, 13-TTI UL access delay, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.3%, 7OS-28.0%, 3/4OS-35.9%, 2OS-38.3%, 1OS-38.3%, Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-4UEs, 3/4OS-3UEs, 2OS-2UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 4.32 | 4.51 | 3.89 | 3.48 | 3.35 | 3.68 | 3.77 | 3.32 | 3.02 | 2.99 |  | 2.99 |  | 3.22 |  | 2.87 |  | 2.64 | 2.66 |
|  | 50% | 14.93 | 15.86 | 13.72 | 12.58 | 12.07 | 12.94 | 13.57 | 12.29 | 11.16 | 10.33 |  | 12.05 |  | 12.57 |  | 11.03 |  | 9.83 | 8.62 |
|  | 95% | 26.40 | 29.80 | 30.16 | 28.97 | 28.72 | 26.58 | 29.85 | 30.51 | 29.30 | 29.99 |  | 26.23 |  | 29.25 |  | 29.09 |  | 27.5 | 27.42 |
|  | Mean | 14.98 | 16.17 | 15.10 | 14.03 | 13.59 | 14.08 | 15.14 | 13.95 | 12.91 | 12.29 |  | 13.1 |  | 14.22 |  | 12.9 |  | 11.8 | 11.06 |
| DL: Delay CDF [s] | 5% | 0.299 | 0.267 | 0.264 | 0.276 | 0.277 | 0.294 | 0.268 | 0.261 | 0.272 | 0.266 |  | 0.304 |  | 0.273 |  | 0.275 |  | 0.291 | 0.292 |
|  | 50% | 0.534 | 0.504 | 0.583 | 0.636 | 0.662 | 0.622 | 0.591 | 0.660 | 0.720 | 0.779 |  | 0.679 |  | 0.657 |  | 0.743 |  | 0.821 | 0.943 |
|  | 95% | 1.824 | 1.747 | 1.922 | 2.301 | 2.357 | 2.160 | 2.100 | 2.317 | 2.630 | 2.654 |  | 2.652 |  | 2.463 |  | 2.775 |  | 3.018 | 2.999 |
|  | Mean | 0.739 | 0.694 | 0.768 | 0.860 | 0.905 | 0.846 | 0.805 | 0.906 | 0.994 | 1.067 |  | 0.978 |  | 0.918 |  | 1.039 |  | 1.142 | 1.185 |
| RU |  | 0.2 | 0.19 | 0.21 | 0.23 | 0.24 | 0.4 | 0.39 | 0.42 | 0.46 | 0.46 |  | 0.59 |  | 0.58 |  | 0.62 |  | 0.64 | 0.65 |
| 𝜆 |  | 0.04 |  |  |  |  | 0.07 |  |  |  |  |  | 0.08 |  |  |  |  |  |  |  |
| Notes: |  | Source 6, R1-162705, CN delay = 6ms, 8 TTIs HARQ RTT, 1-TTI UL access delay, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.3%, 7OS-28.0%, 3/4OS-35.9%, 2OS-38.3%, 1OS-38.3%, Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-4UEs, 3/4OS-3UEs, 2OS-2UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 5.21 | 4.82 | 3.99 | 2.83 | - | 2.51 | 2.10 | 1.77 | 1.45 | - |  | 1.44 |  | 1.29 |  | 1.24 |  | 1.08 | - |
|  | 50% | 10.59 | 11.07 | 10.68 | 10.46 | - | 8.15 | 6.86 | 6.70 | 6.65 | - |  | 5.95 |  | 4.96 |  | 4.76 |  | 4.85 | - |
|  | 95% | 14.87 | 17.53 | 19.34 | 19.46 | - | 13.83 | 15.41 | 14.38 | 16.53 | - |  | 12.78 |  | 13.41 |  | 13.02 |  | 14.81 | - |
|  | Mean | 10.63 | 11.16 | 10.75 | 10.64 | - | 8.22 | 7.60 | 7.35 | 7.64 | - |  | 6.42 |  | 5.99 |  | 5.79 |  | 6.16 | - |
| DL: Delay CDF [s] | 5% | 0.55 | 0.46 | 0.42 | 0.41 | - | 0.59 | 0.52 | 0.57 | 0.49 | - |  | 0.63 |  | 0.60 |  | 0.64 |  | 0.56 | - |
|  | 50% | 0.77 | 0.75 | 0.88 | 0.91 | - | 1.06 | 1.34 | 1.35 | 1.42 | - |  | 1.55 |  | 1.82 |  | 1.87 |  | 1.91 | - |
|  | 95% | 1.73 | 1.74 | 2.15 | 2.88 | - | 3.74 | 4.31 | 4.84 | 5.84 | - |  | 5.70 |  | 6.72 |  | 6.93 |  | 7.59 | - |
|  | Mean | 0.92 | 0.94 | 1.12 | 1.25 | - | 1.47 | 1.82 | 1.92 | 2.07 | - |  | 2.13 |  | 2.50 |  | 2.59 |  | 2.60 | - |
| RU |  | 20.00 | 24.89 | 25.32 | 23.06 | - | 40.00 | 48.11 | 47.86 | 39.73 | - |  | 55.00 |  | 60.32 |  | 61.45 |  | 48.62 | - |
| 𝜆 |  | - |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |  |  |
| Notes: |  | Source 8, R1-165301, CN delay = 10ms, macro scenario, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL: UPT CDF [Mbps] | 5% | 13.824 | 18.191 | 20.425 | - | - | 9.341 | 10.827 | 11.188 | - | - | 3.357 | 3.680 | 3.890 | - | - |
|  | 50% | 20.387 | 26.671 | 30.857 | - | - | 20.531 | 24.567 | 27.425 | - | - | 16.160 | 17.640 | 18.193 | - | - |
|  | 95% | 25.062 | 32.626 | 37.538 | - | - | 25.562 | 32.873 | 38.263 | - | - | 26.209 | 32.953 | 37.276 | - | - |
|  | Mean | 20.497 | 26.388 | 30.467 | - | - | 19.779 | 24.266 | 27.060 | - | - | 15.546 | 18.152 | 19.447 | - | - |
| DL: Delay CDF [s] | 5% | 0.340 | 0.257 | 0.223 | - | - | 0.347 | 0.274 | 0.222 | - | - | 0.336 | 0.265 | 0.227 | - | - |
|  | 50% | 0.420 | 0.321 | 0.278 | - | - | 0.434 | 0.366 | 0.313 | - | - | 0.622 | 0.549 | 0.542 | - | - |
|  | 95% | 0.577 | 0.440 | 0.372 | - | - | 0.972 | 0.898 | 0.733 | - | - | 2.602 | 2.356 | 2.464 | - | - |
|  | Mean | 0.431 | 0.333 | 0.290 | - | - | 0.484 | 0.409 | 0.352 | - | - | 0.850 | 0.785 | 0.791 | - | - |
| RU |  | 20.687 | 20.761 | 21.018 | - | - | 42.209 | 42.222 | 42.709 | - | - | 61.198 | 61.639 | 62.513 | - | - |
| 𝜆 |  | 0.5 |  |  |  |  | 1.0 |  |  |  |  | 1.5 |  |  |  |  |
| Notes: |  | Source 12, R1-163798, CN delay=6ms, UL access delay = 13TTI, FTP traffic model 3 Total control overhead per subframe compared to all available REs in the sTTIs: 14 OS:15%, 7 OS: 17%, 3/4 OS: 19% Maximum number of UEs that can be scheduled per sTTI: not modelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  | 5% |  | 3.81 |  | 4.36 |  | 4.50 |  | 4.31 |  |  |  | 3.19 |  | 3.38 |  | 3.19 |  | 2.93 |  |  | 2.44 | 2.17 | 2.32 | 2.08 |  |
|  |  | 50% |  | 8.41 |  | 9.49 |  | 10.40 |  | 10.23 |  |  |  | 7.33 |  | 8.22 |  | 8.24 |  | 8.28 |  |  | 5.86 | 6.12 | 6.38 | 6.15 |  |
|  |  | 95% |  | 13.46 |  | 17.97 |  | 19.60 |  | 20.37 |  |  |  | 12.96 |  | 16.47 |  | 18.64 |  | 19.33 |  |  | 12.58 | 15.30 | 16.95 | 17.78 |  |
|  |  | Mean |  | 8.65 |  | 10.18 |  | 11.12 |  | 11.06 |  |  |  | 7.65 |  | 8.97 |  | 9.39 |  | 9.35 |  |  | 6.48 | 7.11 | 7.73 | 7.43 |  |
| DL: Delay CDF [s] |  | 5% |  | 0.62 |  | 0.47 |  | 0.43 |  | 0.41 |  |  |  | 0.65 |  | 0.51 |  | 0.45 |  | 0.44 |  |  | 0.67 | 0.55 | 0.50 | 0.47 |  |
|  |  | 50% |  | 0.99 |  | 0.88 |  | 0.81 |  | 0.82 |  |  |  | 1.14 |  | 1.02 |  | 1.02 |  | 1.02 |  |  | 1.42 | 1.38 | 1.30 | 1.37 |  |
|  |  | 95% |  | 2.24 |  | 1.94 |  | 1.87 |  | 2.00 |  |  |  | 2.72 |  | 2.51 |  | 2.67 |  | 2.93 |  |  | 3.57 | 3.88 | 3.80 | 4.19 |  |
|  |  | Mean |  | 1.15 |  | 1.01 |  | 0.93 |  | 0.97 |  |  |  | 1.34 |  | 1.20 |  | 1.21 |  | 1.26 |  |  | 1.68 | 1.68 | 1.60 | 1.73 |  |
| RU |  |  |  | 22.2 |  | 21.6 |  | 22.2 |  | 22.6 |  |  |  | 33.6 |  | 33.8 |  | 35.8 |  | 34.8 |  |  | 55.4 | 54.4 | 54.6 | 56 |  |
| 𝜆 |  | 0.03 |  |  |  |  | 0.05 |  |  |  |  | 0.08 |  |  |  |  |
| Notes: |  | Source 7, R1-165418, File size: 1MB, Macro scenario, 6 ms core network delay, Dynamic DCI overhead, FTP model 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  | 5% |  | 3.79 |  | 3.86 |  | 4.10 |  | 3.95 |  |  |  | 3.22 |  | 3.16 |  | 2.89 |  | 2.81 |  |  | 2.37 | 2.07 | 2.17 | 1.88 |  |
|  |  | 50% |  | 8.49 |  | 9.01 |  | 9.99 |  | 9.69 |  |  |  | 7.39 |  | 7.65 |  | 7.69 |  | 8.01 |  |  | 6.06 | 5.59 | 6.82 | 5.89 |  |
|  |  | 95% |  | 13.51 |  | 17.41 |  | 19.21 |  | 21.11 |  |  |  | 13.09 |  | 16.71 |  | 18.53 |  | 18.59 |  |  | 12.43 | 14.25 | 16.50 | 16.80 |  |
|  |  | Mean |  | 8.66 |  | 9.86 |  | 10.77 |  | 11.04 |  |  |  | 7.68 |  | 8.56 |  | 9.01 |  | 9.04 |  |  | 6.59 | 6.68 | 7.31 | 7.03 |  |
| DL: Delay CDF [s] |  | 5% |  | 0.62 |  | 0.48 |  | 0.44 |  | 0.40 |  |  |  | 0.64 |  | 0.50 |  | 0.45 |  | 0.45 |  |  | 0.68 | 0.59 | 0.51 | 0.50 |  |
|  |  | 50% |  | 0.99 |  | 0.93 |  | 0.85 |  | 0.87 |  |  |  | 1.14 |  | 1.10 |  | 1.10 |  | 1.05 |  |  | 1.38 | 1.50 | 1.27 | 1.42 |  |
|  |  | 95% |  | 2.26 |  | 2.24 |  | 2.10 |  | 2.15 |  |  |  | 2.65 |  | 2.74 |  | 3.03 |  | 3.05 |  |  | 3.57 | 4.07 | 3.98 | 4.54 |  |
|  |  | Mean |  | 1.15 |  | 1.07 |  | 0.98 |  | 0.99 |  |  |  | 1.34 |  | 1.30 |  | 1.32 |  | 1.29 |  |  | 1.66 | 1.78 | 1.63 | 1.86 |  |
| RU |  |  |  | 21.8 |  | 23.6 |  | 22 |  | 21.4 |  |  |  | 34.4 |  | 35.2 |  | 37.6 |  | 37.4 |  |  | 52 | 57.4 | 55.2 | 57.2 |  |
| 𝜆 |  | 0.03 |  |  |  |  | 0.05 |  |  |  |  | 0.08 |  |  |  |  |
| Notes: |  | Source 7, R1-165418, File size: 1MB, Macro scenario, 6 ms core network delay, static DCI overhead, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-15%, 7OS-18.0%, 3/4OS-19%, 2OS-21.5%, 1OS-28%; Maximum number of UEs that can be scheduled per sTTI: 14OS-5UEs, 7OS-2UEs, 3/4OS-1UEs, 2OS-1UEs, 1OS-1UE, FTP model 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  | 5% |  | 3.69 |  | 3.81 |  | 3.78 |  | 3.8 |  |  |  | 2.55 |  | 2.16 |  | 2.76 |  | 1.97 |  |  | 1.64 | 1.46 | 1.6 | 1.07 |  |
|  |  | 50% |  | 8.09 |  | 8.95 |  | 11.07 |  | 9.68 |  |  |  | 6.52 |  | 6.68 |  | 7.18 |  | 7.22 |  |  | 4.39 | 4.38 | 4.65 | 3.06 |  |
|  |  | 95% |  | 12.97 |  | 16.84 |  | 18.66 |  | 18.12 |  |  |  | 12.56 |  | 14.36 |  | 16.38 |  | 17.05 |  |  | 10.89 | 12.61 | 12.83 | 12.22 |  |
|  |  | Mean |  | 8.37 |  | 9.73 |  | 10.93 |  | 10.32 |  |  |  | 6.7 |  | 7.65 |  | 8.66 |  | 8 |  |  | 5.03 | 5.26 | 5.62 | 4.29 |  |
| DL: Delay CDF [s] |  | 5% |  | 2.276 |  | 2.2 |  | 2.217 |  | 2.21 |  |  |  | 3.285 |  | 3.879 |  | 2.424 |  | 4.263 |  |  | 5.12 | 5.749 | 3.008 | 7.855 |  |
|  |  | 50% |  | 1.034 |  | 0.937 |  | 0.758 |  | 0.858 |  |  |  | 1.269 |  | 1.256 |  | 1.025 |  | 1.161 |  |  | 1.873 | 1.878 | 1.262 | 2.741 |  |
|  |  | 95% |  | 0.647 |  | 0.498 |  | 0.449 |  | 0.463 |  |  |  | 0.668 |  | 0.584 |  | 0.433 |  | 0.492 |  |  | 0.771 | 0.665 | 0.566 | 0.686 |  |
|  |  | Mean |  | 1.171 |  | 1.084 |  | 0.934 |  | 1.043 |  |  |  | 1.572 |  | 1.564 |  | 1.142 |  | 1.596 |  |  | 2.314 | 2.355 | 1.451 | 3.315 |  |
| RU |  |  |  | 24 |  | 26 |  | 27 |  | 24 |  |  |  | 49 |  | 48 |  | 48 |  | 48 |  |  | 73 | 73 | 70 | 80 |  |
| 𝜆 |  | 0.03 |  |  |  |  | 0.07 |  |  |  |  | 0.13 |  |  |  |  |
| Notes: |  | Source 7, R1-165418, File size: 1MB, Macro Scenario, 6 ms core network delay, static DCI overhead, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-15%, 7OS-18.0%, 3/4OS-19%, 2OS-21.5%, 1OS-28%; Maximum number of UEs that can be scheduled per sTTI: 14OS-5UEs, 7OS-2UEs, 3/4OS-1UEs, 2OS-1UEs, 1OS-1UE, FTP model 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 4.1 | 4.56 | 4.15 | 4.04 |  | 2.69 | 2.58 | 2.81 | 2.51 |  | 1.54 | 1.4 | 1.43 | 1.27 |  |
|  | 50% | 8.31 | 9.67 | 9.79 | 9.96 |  | 6.52 | 7.21 | 7.29 | 7.03 |  | 4.33 | 4.22 | 4.45 | 4.11 |  |
|  | 95% | 13.45 | 17.46 | 18.77 | 20.49 |  | 12.62 | 16.11 | 17.53 | 18.08 |  | 10.44 | 11.87 | 12.97 | 12.7 |  |
|  | Mean | 8.7 | 10.18 | 10.56 | 10.88 |  | 6.91 | 8.01 | 8.48 | 8.4 |  | 4.96 | 5.16 | 5.42 | 5.4 |  |
| DL: Delay CDF [s] | 5% | 0.624 | 0.482 | 0.449 | 0.411 |  | 0.666 | 0.522 | 0.479 | 0.469 |  | 0.806 | 0.712 | 0.653 | 0.616 |  |
|  | 50% | 1.005 | 0.867 | 0.857 | 0.843 |  | 1.299 | 1.152 | 1.15 | 1.189 |  | 1.932 | 1.981 | 1.894 | 2.049 |  |
|  | 95% | 2.074 | 1.877 | 2.142 | 2.137 |  | 3.112 | 3.26 | 3 | 3.399 |  | 5.512 | 6.042 | 5.909 | 6.749 |  |
|  | Mean | 1.122 | 0.997 | 1.013 | 1.004 |  | 1.525 | 1.43 | 1.341 | 1.449 |  | 2.406 | 2.474 | 2.421 | 2.621 |  |
| RU |  | 21.4 | 21 | 23.2 | 22.4 |  | 43.2 | 42.2 | 42.4 | 44.2 |  | 71.4 | 72.6 | 72 | 73.6 |  |
| 𝜆 |  | 0.03 |  |  |  |  | 0.07 |  |  |  |  | 0.13 |  |  |  |  |
| Notes: |  | Source 7, R1-165418, File size: 1MB, Macro Scenario, 6 ms core network delay, Dynamic DCI overhead, FTP model 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### B.1.2 Results for FS2

Table B.1.2-1: Results with 100kbit file transfer

| Reported parameters |  |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  | 5% | 3.7273 (0) | 4.3272 (+16.1) |  | 4.5044 (+20.8) |  | 3.2014 (0) | 3.7982 (+18.6) |  | 3.5345 (+10.4) |  | 2.881 (0) | 3.1407 (+9) |  | 2.5885 (-10.2) |  |
|  |  | 50% | 4.1588 (0) | 5.1108 (+23) |  | 6.3673 (+53.1) |  | 4.1387 (0) | 5.0572 (+22.2) |  | 6.0686 (+46.6) |  | 4.0938 (0) | 4.9064 (+19.8) |  | 5.4057 (+32) |  |
|  |  | 95% | 4.4846 (0) | 5.3799 (+20) |  | 6.6079 (+47.3) |  | 4.4799 (0) | 5.3674 (+19.8) |  | 6.5565 (+46.4) |  | 4.4788 (0) | 5.3559 (+19.6) |  | 6.4604 (+44.2) |  |
|  |  | Mean | 4.1519 (0) | 5.0448 (+21.5) |  | 6.1278 (+47.5) |  | 4.0839 (0) | 4.908 (+20.2) |  | 5.7346 (+40.4) |  | 3.9482 (0) | 4.6714 (+18.3) |  | 5.0658 (+28.3) |  |
| DL: Delay CDF [s] |  | 5% | 0.1871 (0) | 0.1559 (-16.7) |  | 0.1269 (-32.2) |  | 0.1872 (0) | 0.1562 (-16.6) |  | 0.128 (-31.7) |  | 0.1872 (0) | 0.1564 (-16.5) |  | 0.1299 (-30.6) |  |
|  |  | 50% | 0.202 (0) | 0.1643 (-18.7) |  | 0.132 (-34.7) |  | 0.2033 (0) | 0.1661 (-18.3) |  | 0.1393 (-31.5) |  | 0.2056 (0) | 0.1722 (-16.2) |  | 0.158 (-23.2) |  |
|  |  | 95% | 0.2261 (0) | 0.1925 (-14.9) |  | 0.1803 (-20.3) |  | 0.2571 (0) | 0.218 (-15.2) |  | 0.2292 (-10.9) |  | 0.286 (0) | 0.2656 (-7.1) |  | 0.3324 (+16.2) |  |
|  |  | Mean | 0.2035 (0) | 0.1677 (-17.6) |  | 0.1399 (-31.3) |  | 0.2084 (0) | 0.1746 (-16.2) |  | 0.1556 (-25.3) |  | 0.2199 (0) | 0.1892 (-14) |  | 0.191 (-13.3) |  |
| RU |  |  | 23 | 24.3 |  | 28.1 |  | 41 | 42.6 |  | 51.3 |  | 62 | 65.5 |  | 75.7 |  |
| 𝜆 |  |  | 1 |  |  |  |  | 2 |  |  |  |  | 4 |  |  |  |  |
| Notes: |  |  | Source 1, R1-164637, FS2 with existing UL/DL configuration , 6ms core network delay, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-16.05%, 7OS-17.83%, 2OS-26.27%, Maximum number of UEs that can be scheduled per sTTI: 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  |  | 4.2936 (-0.8) |  | 4.8637 (+8) |  |  | 3.4995 (-7.9) |  | 3.8172 (+8) |  |  | 2.772 (-11.7) |  | 2.363 (-8.7) |  |
|  | 50% |  |  | 5.5549 (+8.7) |  | 7.4219 (+16.6) |  |  | 5.4519 (+7.8) |  | 7.0183 (+15.9) |  |  | 5.1022 (+4) |  | 5.9849 (+10.7) |  |
|  | 95% |  |  | 5.827 (+8.3) |  | 7.8379 (+18.6) |  |  | 5.8067 (+8.2) |  | 7.8094 (+19.1) |  |  | 5.7776 (+7.9) |  | 7.6468 (+18.4) |  |
|  | Mean |  |  | 5.4087 (+7.2) |  | 7.067 (+15.3) |  |  | 5.1873 (+5.7) |  | 6.5576 (+14.4) |  |  | 4.8309 (+3.4) |  | 5.6769 (+12.1) |  |
| DL: Delay CDF [s] | 5% |  |  | 0.144 (-7.6) |  | 0.1069 (-15.8) |  |  | 0.1443 (-7.6) |  | 0.107 (-16.2) |  |  | 0.1452 (-7.2) |  | 0.1098 (-15.5) |  |
|  | 50% |  |  | 0.1511 (-8.03) |  | 0.1128 (-14.5) |  |  | 0.1538 (-7.4) |  | 0.1209 (-5.5) |  |  | 0.1643 (-4.6) |  | 0.1451 (-8.2) |  |
|  | 95% |  |  | 0.1967 (+2.2) |  | 0.1711 (-5.1) |  |  | 0.2391 (+9.7) |  | 0.2197 (-4.1) |  |  | 0.305 (+14.8) |  | 0.3416 (+2.8) |  |
|  | Mean |  |  | 0.1573 (-6.2) |  | 0.123 (-12.1) |  |  | 0.1679 (-3.8) |  | 0.1395 (-10.3) |  |  | 0.1893 (+0.05) |  | 0.1776 (-6.8) |  |
| RU |  |  |  | 25.3 |  | 30 |  |  | 49 |  | 52.5 |  |  | 71.2 |  | 78.2 |  |
| 𝜆 |  |  | 1 |  |  |  |  | 2 |  |  |  |  | 4 |  |  |  |  |
| Notes: |  |  | Source 1, R1-164637, FS2 with new subframe type, 6ms core network delay, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-16.05%, 7OS-15.23%, 2OS-25.09%, Maximum number of UEs that can be scheduled per sTTI: 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.1.2-2: Results with 100kB file transfer

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mean |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL: Delay CDF [s] | 5% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mean |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RU |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.1.2-3: Results with 500kB file transfer

| Reported parameters |  |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  | 5% | 7.0814 (0) | 7.3624 (+4) |  | 6.1689 (-12.9) |  | 6.3052 (0) | 6.1219 (-2.9) |  | 5.1637 (-18.1) |  | 5.0804 (0) | 5.0075 (-1.4) |  | 4.0992 (-19.3) |  |
|  |  | 50% | 10.3584 (0) | 12.8161 (+23.7) |  | 15.2424 (+47.2) |  | 10.1512 (0) | 12.4954 (+23.1) |  | 12.9008 (+27.1) |  | 10.0251 (0) | 11.5342 (+15.1) |  | 11.5781 (+15.5) |  |
|  |  | 95% | 11.3794 (0) | 13.6825 (+20.2) |  | 16.9325 (+48.8) |  | 11.3632 (0) | 13.6735 (+20.3) |  | 16.8127 (+48) |  | 11.3593 (0) | 13.6124 (+19.8) |  | 16.2753 (+43.3) |  |
|  |  | Mean | 10.0803 (0) | 12.1552 (+20.6) |  | 13.7741 (+36.6) |  | 9.7447 (0) | 11.703 (+20.1) |  | 12.445 (+27.7) |  | 9.2832 (0) | 10.86 (+17) |  | 11.0248 (+18.8) |  |
| DL: Delay CDF [s] |  | 5% | 0.3684 (0) | 0.3065 (-16.8) |  | 0.2476 (-32.8) |  | 0.3686 (0) | 0.3067 (-16.8) |  | 0.2493 (-32.4) |  | 0.3689 (0) | 0.3079 (-16.5) |  | 0.2569 (-30.4) |  |
|  |  | 50% | 0.4052 (0) | 0.3271 (-19.3) |  | 0.28 (-31) |  | 0.4137 (0) | 0.3371 (-18.5) |  | 0.3361 (-18.8) |  | 0.4213 (0) | 0.3692 (-12.4) |  | 0.3855 (-8.5) |  |
|  |  | 95% | 0.576 (0) | 0.5207 (-9.6) |  | 0.6915 (+20.1) |  | 0.6587 (0) | 0.6598 (+0.2) |  | 0.8025 (+21.8) |  | 0.7916 (0) | 0.808 (+2.1) |  | 1.0624 (+34.2) |  |
|  |  | Mean | 0.4283 (0) | 0.3616 (-15.6) |  | 0.3436 (-19.8) |  | 0.4549 (0) | 0.3884 (-14.6) |  | 0.4112 (-9.6) |  | 0.5055 (0) | 0.4467 (-11.6) |  | 0.5032 (-0.5) |  |
| RU |  |  | 24.9 | 26 |  | 30.2 |  | 39 | 37.9 |  | 47 |  | 55.6 | 57.6 |  | 65 |  |
| 𝜆 |  |  | 1/5 |  |  |  |  | 1/3 |  |  |  |  | 1/2 |  |  |  |  |
| Notes: |  |  | Source 1, R1-164637, FS2 with existing UL/DL configuration , 6ms core network delay, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-16.05%, 7OS-17.83%, 2OS-26.27%, Maximum number of UEs that can be scheduled per sTTI: 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  |  | 7.0403 (-4.4) |  | 6.3707 (+3.3) |  |  | 5.9283 (-3.2) |  | 4.903 (-5.1) |  |  | 4.0092 (-19.9) |  | 3.8225 (-6.8) |  |
|  | 50% |  |  | 13.5486 (+5.7) |  | 17.1841 (+12.7) |  |  | 12.8387 (+2.7) |  | 14.6503 (+13.6) |  |  | 11.6998 (+1.4) |  | 12.5073 (+8) |  |
|  | 95% |  |  | 14.831 (+8.4) |  | 20.11 (+18.8) |  |  | 14.775 (+8.1) |  | 20.0442 (+19.2) |  |  | 14.6516 (+7.6) |  | 19.9025 (+22.3) |  |
|  | Mean |  |  | 12.6118 (+3.8) |  | 15.5882 (+13.2) |  |  | 11.9836 (+2.4) |  | 13.8772 (+11.5) |  |  | 10.6747 (-1.7) |  | 12.5639 (+14) |  |
| DL: Delay CDF [s] | 5% |  |  | 0.2825 (-7.8) |  | 0.2085 (-15.8) |  |  | 0.2833 (-8) |  | 0.2091 (-16.1) |  |  | 0.2839 (-7.8) |  | 0.2098 (-18.3) |  |
|  | 50% |  |  | 0.3114 (-4.8) |  | 0.2462 (-12.1) |  |  | 0.3318 (-1.6) |  | 0.2996 (-10.9) |  |  | 0.3851 (+4.3) |  | 0.3469 (-10) |  |
|  | 95% |  |  | 0.574 (+10.3) |  | 0.6312 (-8.7) |  |  | 0.6709 (+1.7) |  | 0.815 (+1.6) |  |  | 1.0732 (+32.8) |  | 1.1379 (+7.1) |  |
|  | Mean |  |  | 0.3575 (-1.1) |  | 0.3203 (-6.7) |  |  | 0.3953 (+1.8) |  | 0.3875 (-5.8) |  |  | 0.4965 (+11.1) |  | 0.4712 (-6.4) |  |
| RU |  |  |  | 29 |  | 28.5 |  |  | 43.3 |  | 50 |  |  | 63.5 |  | 65 |  |
| 𝜆 |  |  | 1/5 |  |  |  |  | 1/3 |  |  |  |  | 1/2 |  |  |  |  |
| Notes: |  |  | Source 1, R1-164637, FS2 with new subframe type, 6ms core network delay, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-16.05%, 7OS-15.23%, 2OS-25.09%, Maximum number of UEs that can be scheduled per sTTI: 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | - | - | - | - | - | 3.5 | - | - | 3.2 | - | 1.8 | - | - | 1.3 | - |
|  | 50% |  | - | - | - | - | - | 8.0 | - | - | 13.7 | - | 7.0 | - | - | 11.6 | - |
|  | 95% |  | - | - | - | - | - | 11.2 | - | - | 20.4 | - | 10.9 | - | - | 19.8 | - |
|  | Mean |  | - | - | - | - | - | 7.7 | - | - | 13.1 | - | 6.2 | - | - | 11.3 | - |
| DL: Delay CDF [s] | 5% |  | - | - | - | - | - | 1.158 | - | - | 1.256 | - | 2.854 | - | - | 3.074 | - |
|  | 50% |  | - | - | - | - | - | 0.498 | - | - | 0.291 | - | 0.572 | - | - | 0.346 | - |
|  | 95% |  | - | - | - | - | - | 0.356 | - | - | 0.196 | - | 0.367 | - | - | 0.201 | - |
|  | Mean |  | - | - | - | - | - | 0.600 | - | - | 0.453 | - | 0.985 | - | - | 0.821 | - |
| RU |  |  | - | - | - | - | - | 0.48 | - | - | 0.54 | - | 0.61 | - | - | 0.71 | - |
| 𝜆 |  |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  |  | Source 5, R1-164824 & R1-164825, CN delay = 6ms, TDD configuration 2, UE speed = 3km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:14OS-16.05%, 3/4OS-18.49%, 2OS-21.16% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | - | - | - | - | - | - | - | - | 4.5 | - | - | - | - | 2.2 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 16.9 | - | - | - | - | 14.3 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 23.6 | - | - | - | - | 23.1 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 15.9 | - | - | - | - | 14.0 | - |
| DL: Delay CDF [s] | 5% |  | - | - | - | - | - | - | - | - | 0.891 | - | - | - | - | 1.803 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 0.237 | - | - | - | - | 0.280 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 0.169 | - | - | - | - | 0.173 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 0.345 | - | - | - | - | 0.559 | - |
| RU |  |  | - | - | - | - | - | - | - | - | 0.47 | - | - | - | - | 0.65 | - |
| 𝜆 |  |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  |  | Source 5, R1-164825, CN delay = 6ms, Enhanced frame structure set 1 (example 1), UE speed = 3km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-21.94% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | - | - | - | - | - | 1.6 | - | - | 1.9 | - | 0.8 | - | - | 0.8 | - |
|  | 50% |  | - | - | - | - | - | 6.2 | - | - | 11.9 | - | 4.9 | - | - | 10.9 | - |
|  | 95% |  | - | - | - | - | - | 10.6 | - | - | 19.7 | - | 10.0 | - | - | 19.2 | - |
|  | Mean |  | - | - | - | - | - | 6.2 | - | - | 11.5 | - | 5.1 | - | - | 10.5 | - |
| DL: Delay CDF [s] | 5% |  | - | - | - | - | - | 2.550 | - | - | 2.059 | - | 5.070 | - | - | 5.007 | - |
|  | 50% |  | - | - | - | - | - | 0.640 | - | - | 0.336 | - | 0.808 | - | - | 0.367 | - |
|  | 95% |  | - | - | - | - | - | 0.376 | - | - | 0.203 | - | 0.401 | - | - | 0.208 | - |
|  | Mean |  | - | - | - | - | - | 0.997 | - | - | 0.646 | - | 1.521 | - | - | 1.166 | - |
| RU |  |  | - | - | - | - | - | 0.61 | - | - | 0.63 | - | 0.76 | - | - | 0.79 | - |
| 𝜆 |  |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  |  | Source 5, R1-164824 & R1-164825, CN delay = 6ms, TDD configuration 2, UE speed = 60km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:14OS-16.05%, 3/4OS-18.49%, 2OS-21.16% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | - | - | - | - | - | - | - | - | 2.9 | - | - | - | - | 1.2 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 14.9 | - | - | - | - | 13.1 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 23.1 | - | - | - | - | 22.5 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 14.4 | - | - | - | - | 12.6 | - |
| DL: Delay CDF [s] | 5% |  | - | - | - | - | - | - | - | - | 1.394 | - | - | - | - | 3.409 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 0.269 | - | - | - | - | 0.306 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 0.174 | - | - | - | - | 0.177 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 0.455 | - | - | - | - | 0.900 | - |
| RU |  |  | - | - | - | - | - | - | - | - | 0.54 | - | - | - | - | 0.74 | - |
| 𝜆 |  |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  |  | Source 5, R1-164825, CN delay = 6ms, Enhanced frame structure set 1 (example 1), UE speed = 60km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-21.94% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | - | - | - | - | - | 1.5 | - | - | 1.4 | - | 0.7 | - | - | 0.9 | - |
|  | 50% |  | - | - | - | - | - | 6.2 | - | - | 11.4 | - | 4.6 | - | - | 10.6 | - |
|  | 95% |  | - | - | - | - | - | 10.7 | - | - | 19.4 | - | 10.0 | - | - | 19.2 | - |
|  | Mean |  | - | - | - | - | - | 6.2 | - | - | 11.1 | - | 4.9 | - | - | 10.4 | - |
| DL: Delay CDF [s] | 5% |  | - | - | - | - | - | 2.581 | - | - | 2.742 | - | 5.568 | - | - | 4.661 | - |
|  | 50% |  | - | - | - | - | - | 0.647 | - | - | 0.351 | - | 0.867 | - | - | 0.376 | - |
|  | 95% |  | - | - | - | - | - | 0.375 | - | - | 0.206 | - | 0.401 | - | - | 0.208 | - |
|  | Mean |  | - | - | - | - | - | 1.046 | - | - | 0.788 | - | 1.673 | - | - | 1.153 | - |
| RU |  |  | - | - | - | - | - | 0.61 | - | - | 0.68 | - | 0.77 | - | - | 0.82 | - |
| 𝜆 |  |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  |  | Source 5, R1-164824 & R1-164825, CN delay = 6ms, TDD configuration 2, UE speed = 120km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:14OS-16.05%, 3/4OS-18.49%, 2OS-21.16% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 3/4OS-10UEs, 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | - | - | - | - | - | - | - | - | 2.8 | - | - | - | - | 1.2 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 14.7 | - | - | - | - | 12.9 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 23.0 | - | - | - | - | 22.5 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 14.2 | - | - | - | - | 12.5 | - |
| DL: Delay CDF [s] | 5% |  | - | - | - | - | - | - | - | - | 1.428 | - | - | - | - | 3.340 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 0.272 | - | - | - | - | 0.310 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 0.174 | - | - | - | - | 0.178 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 0.465 | - | - | - | - | 0.945 | - |
| RU |  |  | - | - | - | - | - | - | - | - | 0.55 | - | - | - | - | 0.77 | - |
| 𝜆 |  |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  |  | Source 5, R1-164825, CN delay = 6ms, Enhanced frame structure set 1 (example 1), UE speed = 120km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-21.94% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | - | - | - | - | - | 2.6 | - | - | 2.7 | - | 1.6 | - | - | 1.2 | - |
|  | 50% |  | - | - | - | - | - | 5.9 | - | - | 11.0 | - | 4.9 | - | - | 9.5 | - |
|  | 95% |  | - | - | - | - | - | 8.3 | - | - | 15.2 | - | 8.0 | - | - | 14.8 | - |
|  | Mean |  | - | - | - | - | - | 5.7 | - | - | 10.4 | - | 4.9 | - | - | 9.3 | - |
| DL: Delay CDF [s] | 5% |  | - | - | - | - | - | 1.524 | - | - | 1.502 | - | 2.559 | - | - | 3.210 | - |
|  | 50% |  | - | - | - | - | - | 0.681 | - | - | 0.385 | - | 0.823 | - | - | 0.448 | - |
|  | 95% |  | - | - | - | - | - | 0.480 | - | - | 0.264 | - | 0.502 | - | - | 0.270 | - |
|  | Mean |  | - | - | - | - | - | 0.811 | - | - | 0.585 | - | 1.096 | - | - | 0.884 | - |
| RU |  |  | - | - | - | - | - | 0.47 | - | - | 0.52 | - | 0.62 | - | - | 0.68 | - |
| 𝜆 |  |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  |  | Source 5, R1-164824 & R1-164825, CN delay = 6ms, TDD configuration 1, UE speed = 3km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:14OS-16.67%, 3/4OS-19.07%, 2OS-21.77% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | - | - | - | - | - | - | - | - | 3.9 | - | - | - | - | 2.0 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 13.1 | - | - | - | - | 11.1 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 17.9 | - | - | - | - | 17.4 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 11.9 | - | - | - | - | 10.8 | - |
| DL: Delay CDF [s] | 5% |  | - | - | - | - | - | - | - | - | 1.036 | - | - | - | - | 2.024 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 0.305 | - | - | - | - | 0.361 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 0.223 | - | - | - | - | 0.229 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 0.435 | - | - | - | - | 0.644 | - |
| RU |  |  | - | - | - | - | - | - | - | - | 0.45 | - | - | - | - | 0.62 | - |
| 𝜆 |  |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  |  | Source 5, R1-164825, CN delay = 6ms, Enhanced frame structure set 1 (example 2), UE speed = 3km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-23.35% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | - | - | - | - | - | 1.3 | - | - | 1.4 | - | 0.6 | - | - | 0.7 | - |
|  | 50% |  | - | - | - | - | - | 4.7 | - | - | 9.2 | - | 3.7 | - | - | 8.6 | - |
|  | 95% |  | - | - | - | - | - | 7.9 | - | - | 14.5 | - | 7.4 | - | - | 14.3 | - |
|  | Mean |  | - | - | - | - | - | 4.7 | - | - | 9.0 | - | 3.8 | - | - | 8.4 | - |
| DL: Delay CDF [s] | 5% |  | - | - | - | - | - | 3.045 | - | - | 2.948 | - | 6.228 | - | - | 5.609 | - |
|  | 50% |  | - | - | - | - | - | 0.849 | - | - | 0.464 | - | 1.078 | - | - | 0.498 | - |
|  | 95% |  | - | - | - | - | - | 0.508 | - | - | 0.276 | - | 0.541 | - | - | 0.279 | - |
|  | Mean |  | - | - | - | - | - | 1.201 | - | - | 0.802 | - | 1.924 | - | - | 1.358 | - |
| RU |  |  | - | - | - | - | - | 0.60 | - | - | 0.63 | - | 0.74 | - | - | 0.76 | - |
| 𝜆 |  |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  |  | Source 5, R1-164824 & R1-164825, CN delay = 6ms , TDD configuration 1, UE speed = 60km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:14OS-16.67%, 3/4OS-19.07%, 2OS-21.77% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 3/4OS-10UEs, 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | - | - | - | - | - | - | - | - | 2.5 | - | - | - | - | 1.1 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 11.5 | - | - | - | - | 10.1 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 17.5 | - | - | - | - | 16.9 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 10.6 | - | - | - | - | 9.7 | - |
| DL: Delay CDF [s] | 5% |  | - | - | - | - | - | - | - | - | 1.564 | - | - | - | - | 3.523 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 0.347 | - | - | - | - | 0.397 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 0.228 | - | - | - | - | 0.236 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 0.565 | - | - | - | - | 1.000 | - |
| RU |  |  | - | - | - | - | - | - | - | - | 0.50 | - | - | - | - | 0.72 | - |
| 𝜆 |  |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  |  | Source 5, R1-164825, CN delay = 6ms, Enhanced frame structure set 1 (example 2), UE speed = 60km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-23.35% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | - | - | - | - | - | 0.9 | - | - | 1.1 | - | 0.5 | - | - | 0.6 | - |
|  | 50% |  | - | - | - | - | - | 4.5 | - | - | 8.6 | - | 3.4 | - | - | 8.5 | - |
|  | 95% |  | - | - | - | - | - | 7.8 | - | - | 14.5 | - | 7.3 | - | - | 14.0 | - |
|  | Mean |  | - | - | - | - | - | 4.5 | - | - | 8.4 | - | 3.6 | - | - | 8.3 | - |
| DL: Delay CDF [s] | 5% |  | - | - | - | - | - | 4.272 | - | - | 3.600 | - | 7.793 | - | - | 6.528 | - |
|  | 50% |  | - | - | - | - | - | 0.880 | - | - | 0.464 | - | 1.173 | - | - | 0.507 | - |
|  | 95% |  | - | - | - | - | - | 0.515 | - | - | 0.277 | - | 0.551 | - | - | 0.285 | - |
|  | Mean |  | - | - | - | - | - | 1.488 | - | - | 1.020 | - | 2.224 | - | - | 1.596 | - |
| RU |  |  | - | - | - | - | - | 0.61 | - | - | 0.67 | - | 0.78 | - | - | 0.82 | - |
| 𝜆 |  |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  |  | Source 5, R1-164824 & R1-164825, CN delay = 6ms , TDD configuration 1, UE speed = 120km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:14OS-16.67%, 3/4OS-19.07%, 2OS-21.77% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | - | - | - | - | - | - | - | - | 1.7 | - | - | - | - | 0.8 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 10.7 | - | - | - | - | 9.7 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 17.3 | - | - | - | - | 16.8 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 10.0 | - | - | - | - | 9.3 | - |
| DL: Delay CDF [s] | 5% |  | - | - | - | - | - | - | - | - | 2.314 | - | - | - | - | 4.909 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 0.372 | - | - | - | - | 0.412 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 0.232 | - | - | - | - | 0.239 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 0.727 | - | - | - | - | 1.246 | - |
| RU |  |  | - | - | - | - | - | - | - | - | 0.59 | - | - | - | - | 0.77 | - |
| 𝜆 |  |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  |  | Source 5, R1-164825, CN delay = 6ms, Enhanced frame structure set 1 (example 2), UE speed = 120km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-23.35% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | - | - | - | - | - | - | - | - | 4.2 | - | - | - | - | 1.7 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 16.0 | - | - | - | - | 12.9 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 21.3 | - | - | - | - | 21.1 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 14.7 | - | - | - | - | 12.7 | - |
| DL: Delay CDF [s] | 5% |  | - | - | - | - | - | - | - | - | 0.958 | - | - | - | - | 2.311 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 0.250 | - | - | - | - | 0.309 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 0.188 | - | - | - | - | 0.190 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 0.384 | - | - | - | - | 0.658 | - |
| RU |  |  | - | - | - | - | - | - | - | - | 0.49 | - | - | - | - | 0.68 | - |
| 𝜆 |  |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  |  | Source 5, R1-164826, CN delay = 6ms, Enhanced frame structure set 2, UE speed = 3km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-22.94% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | - | - | - | - | - | - | - | - | 2.3 | - | - | - | - | 1.1 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 13.2 | - | - | - | - | 12.2 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 20.9 | - | - | - | - | 20.7 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 12.8 | - | - | - | - | 11.7 | - |
| DL: Delay CDF [s] | 5% |  | - | - | - | - | - | - | - | - | 1.734 | - | - | - | - | 3.659 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 0.302 | - | - | - | - | 0.328 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 0.191 | - | - | - | - | 0.193 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 0.582 | - | - | - | - | 0.925 | - |
| RU |  |  | - | - | - | - | - | - | - | - | 0.60 | - | - | - | - | 0.75 | - |
| 𝜆 |  |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  |  | Source 5, R1-164826, CN delay = 6ms, Enhanced frame structure set 2, UE speed = 60km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-22.94% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | - | - | - | - | - | - | - | - | 2.0 | - | - | - | - | 1.1 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 12.8 | - | - | - | - | 12.1 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 20.8 | - | - | - | - | 20.5 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 12.6 | - | - | - | - | 11.5 | - |
| DL: Delay CDF [s] | 5% |  | - | - | - | - | - | - | - | - | 2.007 | - | - | - | - | 3.683 | - |
|  | 50% |  | - | - | - | - | - | - | - | - | 0.312 | - | - | - | - | 0.331 | - |
|  | 95% |  | - | - | - | - | - | - | - | - | 0.192 | - | - | - | - | 0.195 | - |
|  | Mean |  | - | - | - | - | - | - | - | - | 0.627 | - | - | - | - | 0.988 | - |
| RU |  |  | - | - | - | - | - | - | - | - | 0.63 | - | - | - | - | 0.79 | - |
| 𝜆 |  |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  |  | Source 5, R1-164826, CN delay = 6ms, Enhanced frame structure set 2, UE speed = 120km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-22.94% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.1.2-4: Results with 1MB file transfer

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mean |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL: Delay CDF [s] | 5% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mean |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RU |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## B.2 Evaluation results with DL traffic and 0ms core network delay

### B.2.1 Results for FS1

Table B.2.1-1: Results with 100kbit file transfer, 0ms core network delay

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.04 | 2.66 | - | 3.48 | 3.73 | 1.61 | 1.88 | - | 2.16 | 2.22 | 1.27 | 1.26 | - |  | 1.26 | 1.16 |
|  | 50% | 2.78 | 4.64 | - | 8.41 | 10.14 | 2.56 | 3.99 | - | 6.35 | 7.18 | 2.38 | 3.27 | - |  | 4.14 | 4.05 |
|  | 95% | 3.22 | 6.23 | - | 13.93 | 20.27 | 3.12 | 5.86 | - | 12.90 | 17.95 | 3.12 | 5.54 | - |  | 11.62 | 14.89 |
|  | Mean | 2.74 | 4.56 | - | 8.55 | 10.83 | 2.52 | 3.98 | - | 6.79 | 8.27 | 2.30 | 3.34 | - |  | 5.05 | 5.56 |
| DL: Delay CDF [s] | 5% | 0.031 | 0.016 | - | 0.007 | 0.005 | 0.032 | 0.017 | - | 0.008 | 0.006 | 0.032 | 0.018 | - |  | 0.009 | 0.007 |
|  | 50% | 0.036 | 0.022 | - | 0.011 | 0.010 | 0.039 | 0.025 | - | 0.016 | 0.014 | 0.042 | 0.031 | - |  | 0.024 | 0.025 |
|  | 95% | 0.049 | 0.037 | - | 0.025 | 0.026 | 0.062 | 0.053 | - | 0.046 | 0.045 | 0.079 | 0.080 | - |  | 0.079 | 0.086 |
|  | Mean | 0.037 | 0.023 | - | 0.013 | 0.012 | 0.042 | 0.029 | - | 0.020 | 0.018 | 0.047 | 0.037 | - |  | 0.032 | 0.033 |
| RU |  | 18.19% | 15.51% |  | 12.54% | 11.77% | 46.81% | 42.86% |  | 36.65% | 34.52% | 77.41% | 72.99% |  |  | 65.12% | 62.37% |
| 𝜆 |  | - |  |  |  |  | - |  |  |  |  | - |  |  |  |  |  |
| Notes: |  | Source 3, R1-165446, macro scenario, Static PDCCH overhead Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-28%,2OS-29.71%, 1OS-34.86% Total Reference signal overhead (CRS + DMRS) per subframe [%]: 14OS-16.67%, 7OS-15.48%, 2OS-23.81%, 1OS-23.81% Maximum number of UEs that can be scheduled per sTTI: Low RU-4UEs, Medium RU-4UEs, High RU-8UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 3.883 | - | - | - | - |  | - | - | - | - | 1.343 |  | - |
|  | 50% | - | - | - | 9.302 | - | - | - | - |  | - | - | - | - | 4.686 |  | - |
|  | 95% | - | - | - | 14.810 | - | - | - | - |  | - | - | - | - | 12.664 |  | - |
|  | Mean | - | - | - | 9.292 | - | - | - | - |  | - | - | - | - | 5.597 |  | - |
| DL: Delay CDF [s] | 5% | - | - | - | 0.0068 | - | - | - | - |  | - | - | - | - | 0.0079 |  | - |
|  | 50% | - | - | - | 0.0107 | - | - | - | - |  | - | - | - | - | 0.0213 |  | - |
|  | 95% | - | - | - | 0.0253 | - | - | - | - |  | - | - | - | - | 0.0743 |  | - |
|  | Mean | - | - | - | 0.0126 | - | - | - | - |  | - | - | - | - | 0.0288 |  | - |
| RU |  |  | - |  | - |  | - |  | 12.34% |  |  |  | - |  | - |  | - |  | - |  | - |  | - |  |  | - |  | - |  | 65.4% |  | - |
| 𝜆 |  |  | - |  |  |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  |  |  | - |  |  |  |  |  |  |  |  |
| Notes: |  | Source 3, R1-165446, macro scenario, Dynamic PDCCH overhead for each TTI based on the number of schedule UE per TTI and the aggregation level required per UE Total Reference signal overhead (CRS + DMRS) per subframe [%]:2OS-23.81% Probability of each aggregatio level [%]: AL1-59.54%, AL2-31.89%, AL4-8.09%, AL8-0.48% Average number of scheduled UE per TTI: Low RU-1.1106, High RU-1.9783 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.12 | 4.11 | 5.55 | 2.65 | - | 2.11 | 3.81 | 3.71 | 1.93 | - | 2.08 | 3.32 | 2.48 |  | 0.54 | - |
|  | 50% | 2.15 | 4.29 | 8.09 | 13.43 | - | 2.16 | 4.30 | 7.76 | 12.49 | - | 2.28 | 4.23 | 7.26 |  | 12.14 | - |
|  | 95% | 2.46 | 4.54 | 8.29 | 15.09 | - | 2.46 | 4.62 | 8.44 | 15.10 | - | 2.44 | 4.32 | 8.23 |  | 14.79 | - |
|  | Mean | 2.20 | 4.29 | 7.70 | 11.45 | - | 2.23 | 4.27 | 7.15 | 10.87 | - | 2.26 | 4.08 | 6.53 |  | 10.60 | - |
| DL: Delay CDF [s] | 5% | 0.046 | 0.023 | 0.012 | 0.007 | - | 0.046 | 0.023 | 0.012 | 0.007 | - | 0.046 | 0.023 | 0.012 |  | 0.007 | - |
|  | 50% | 0.047 | 0.023 | 0.012 | 0.007 | - | 0.046 | 0.023 | 0.013 | 0.008 | - | 0.047 | 0.024 | 0.014 |  | 0.008 | - |
|  | 95% | 0.047 | 0.025 | 0.018 | 0.038 | - | 0.048 | 0.026 | 0.027 | 0.052 | - | 0.049 | 0.030 | 0.039 |  | 0.184 | - |
|  | Mean | 0.047 | 0.024 | 0.013 | 0.014 | - | 0.047 | 0.024 | 0.015 | 0017 | - | 0.047 | 0.025 | 0.029 |  | 0.077 | - |
| RU |  | 0.21 | 0.14 | 0.13 | 0.21 | - | 0.39 | 0.3 | 0.31 | 0.4 |  | - | 0.54 | 0.61 |  | 0.62 |  |
| 𝜆 |  | 6.2 |  |  |  |  | 20 |  |  |  |  | 95 |  |  |  |  |  |
| Notes: |  | Source 4, R1-162503, CN delay = 0ms, CQI period = 5TTI, CQI delay = 6TTI FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.12 | 4.06 | 4.22 | 1.28 | - | 2.11 | 3.73 | 3.11 | 0.95 | - | 2.08 | 3.33 | 2.43 |  | 0.46 | - |
|  | 50% | 2.15 | 4.28 | 7.92 | 10.73 | - | 2.16 | 4.29 | 7.49 | 10.56 | - | 2.28 | 4.23 | 7.12 |  | 11.84 | - |
|  | 95% | 2.46 | 4.57 | 8.25 | 14.86 | - | 2.46 | 4.59 | 8.44 | 15.00 | - | 2.44 | 4.31 | 8.23 |  | 14.67 | - |
|  | Mean | 2.20 | 4.28 | 7.33 | 9.51 | - | 2.23 | 4.24 | 6.82 | 9.52 | - | 2.26 | 4.07 | 6.44 |  | 10.18 | - |
| DL: Delay CDF [s] | 5% | 0.046 | 0.023 | 0.012 | 0.007 | - | 0.046 | 0.023 | 0.012 | 0.007 | - | 0.046 | 0.023 | 0.012 |  | 0.007 | - |
|  | 50% | 0.047 | 0.023 | 0.013 | 0.009 | - | 0.046 | 0.023 | 0.013 | 0.009 | - | 0.047 | 0.024 | 0.014 |  | 0.008 | - |
|  | 95% | 0.047 | 0.025 | 0.024 | 0.078 | - | 0.048 | 0.027 | 0.032 | 0.099 | - | 0.049 | 0.030 | 0.040 |  | 0.202 | - |
|  | Mean | 0.047 | 0.024 | 0.015 | 0.043 | - | 0.047 | 0.024 | 0.017 | 0.033 | - | 0.047 | 0.025 | 0.051 |  | 0.080 | - |
| RU |  | 0.21 | 0.14 | 0.17 | 0.29 | - | 0.39 | 0.31 | 0.35 | 0.46 | - | 0.57 | 0.54 | 0.62 |  | 0.64 | - |
| 𝜆 |  | 6.2 |  |  |  |  | 20 |  |  |  |  | 95 |  |  |  |  |  |
| Notes: |  | Source 4, R1-162503, CN delay = 0ms, CQI period = 5ms, CQI delay = 6ms FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.12 | 4.08 | 4.68 | 1.49 | - | 2.11 | 3.77 | 3.16 | 1.13 | - | 2.08 | 3.32 | 2.42 |  | 0.48 | - |
|  | 50% | 2.15 | 4.28 | 8.04 | 11.33 | - | 2.16 | 4.29 | 7.55 | 11.42 | - | 2.28 | 4.23 | 7.04 |  | 11.96 | - |
|  | 95% | 2.46 | 4.53 | 8.26 | 14.91 | - | 2.46 | 4.63 | 8.29 | 15.03 | - | 2.44 | 4.32 | 8.24 |  | 14.71 | - |
|  | Mean | 2.20 | 4.28 | 7.50 | 9.92 | - | 2.23 | 4.26 | 6.90 | 9.91 | - | 2.26 | 4.08 | 6.39 |  | 10.32 | - |
| DL: Delay CDF [s] | 5% | 0.046 | 0.023 | 0.012 | 0.007 | - | 0.046 | 0.023 | 0.012 | 0.007 | - | 0.046 | 0.023 | 0.012 |  | 0.007 | - |
|  | 50% | 0.047 | 0.023 | 0.012 | 0.009 | - | 0.046 | 0.023 | 0.013 | 0.009 | - | 0.047 | 0.024 | 0.014 |  | 0.008 | - |
|  | 95% | 0.047 | 0.025 | 0.022 | 0.067 | - | 0.048 | 0.027 | 0.032 | 0.088 | - | 0.049 | 0.030 | 0.040 |  | 0.186 | - |
|  | Mean | 0.047 | 0.024 | 0.014 | 0.026 | - | 0.047 | 0.024 | 0.020 | 0.030 | - | 0.047 | 0.025 | 0.033 |  | 0.050 | - |
| RU |  | 0.21 | 0.14 | 0.14 | 0.28 | - | 0.39 | 0.3 | 0.32 | 0.46 | - | 0.57 | 0.54 | 0.58 |  | 0.63 | - |
| 𝜆 |  | 6.2 |  |  |  |  | 20 |  |  |  |  | 95 |  |  |  |  |  |
| Notes: |  | Source 4, R1-162503, CN delay = 0ms, CQI period = 5TTI, CQI delay = 6ms FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.12 | 4.12 | 5.41 | 2.06 | - | 2.11 | 3.78 | 3.67 | 1.43 | - | 2.08 | 3.31 | 2.49 |  | 0.47 | - |
|  | 50% | 2.15 | 4.29 | 8.10 | 12.87 | - | 2.16 | 4.30 | 7.72 | 11.98 | - | 2.28 | 4.23 | 7.01 |  | 11.97 | - |
|  | 95% | 2.46 | 4.56 | 8.30 | 15.07 | - | 2.46 | 4.60 | 8.43 | 15.07 | - | 2.44 | 4.32 | 8.24 |  | 14.77 | - |
|  | Mean | 2.20 | 4.29 | 7.70 | 10.96 | - | 2.23 | 4.26 | 7.06 | 7.97 | - | 2.26 | 4.08 | 6.39 |  | 10.47 | - |
| DL: Delay CDF [s] | 5% | 0.046 | 0.023 | 0.012 | 0.007 | - | 0.046 | 0.023 | 0.012 | 0.007 | - | 0.046 | 0.023 | 0.012 |  | 0.007 | - |
|  | 50% | 0.047 | 0.023 | 0.012 | 0.008 | - | 0.046 | 0.023 | 0.013 | 0.008 | - | 0.047 | 0.024 | 0.014 |  | 0.008 | - |
|  | 95% | 0.047 | 0.024 | 0.018 | 0.049 | - | 0.048 | 0.027 | 0.027 | 0.068 | - | 0.049 | 0.030 | 0.037 |  | 0.189 | - |
|  | Mean | 0.047 | 0.024 | 0.013 | 0.017 | - | 0.047 | 0.024 | 0.016 | 0.026 | - | 0.047 | 0.025 | 0.033 |  | 0.076 | - |
| RU |  | 0.21 | 0.13 | 0.13 | 0.24 | - | 0.39 | 0.3 | 0.3 | 0.43 | - | 0.57 | 0.54 | 0.57 |  | 0.63 | - |
| 𝜆 |  | 6.2 |  |  |  |  | 20 |  |  |  |  | 95 |  |  |  |  |  |
| Notes: |  | Source 4, R1-162503, CN delay = 0ms, CQI period = 5ms, CQI delay = 6TTI FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 1.96 | 2.41 | 2.37 | 2.38 | 2.43 | 1.66 | 1.58 | 1.85 | 1.90 | 1.72 | 1.22 | 1.25 | 1.27 |  | 1.34 | 1.22 |
|  | 50% | 3.38 | 4.97 | 6.83 | 7.03 | 7.12 | 3.15 | 4.28 | 5.23 | 5.20 | 5.08 | 2.93 | 3.76 | 3.81 |  | 4.03 | 3.63 |
|  | 95% | 3.86 | 7.14 | 11.57 | 15.82 | 18.74 | 3.83 | 7.02 | 11.22 | 13.95 | 16.74 | 3.79 | 6.87 | 11.19 |  | 12.96 | 14.85 |
|  | Mean | 3.22 | 4.97 | 6.94 | 7.93 | 8.50 | 3.04 | 4.34 | 5.82 | 6.42 | 6.70 | 2.83 | 3.89 | 4.96 |  | 5.25 | 5.12 |
| DL: Delay CDF [s] | 5% | 0.05 | 0.04 | 0.04 | 0.04 | 0.04 | 0.06 | 0.07 | 0.06 | 0.06 | 0.06 | 0.09 | 0.08 | 0.08 |  | 0.08 | 0.09 |
|  | 50% | 0.03 | 0.02 | 0.02 | 0.02 | 0.02 | 0.03 | 0.02 | 0.02 | 0.02 | 0.02 | 0.04 | 0.03 | 0.03 |  | 0.03 | 0.03 |
|  | 95% | 0.03 | 0.02 | 0.01 | 0.01 | 0.01 | 0.03 | 0.02 | 0.01 | 0.01 | 0.01 | 0.03 | 0.02 | 0.01 |  | 0.01 | 0.01 |
|  | Mean | 0.04 | 0.02 | 0.02 | 0.02 | 0.02 | 0.04 | 0.03 | 0.03 | 0.02 | 0.02 | 0.04 | 0.04 | 0.03 |  | 0.03 | 0.04 |
| RU |  | 23 | 26 | 27 | 28 | 31 | 37 | 42 | 44 | 46 | 50 | 55 | 59 | 62 |  | 65 | 72 |
| 𝜆 |  | 3.33 |  |  |  |  | 5.26 |  |  |  |  | 7.14 |  |  |  |  |  |
| Notes: |  | Source 7, R1-163403, CN delay = 0ms, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-15%, 7OS-18.0%, 3/4OS-19%, 2OS-21.5%, 1OS-28%; Maximum number of UEs that can be scheduled per sTTI: 14OS-5UEs, 7OS-2UEs, 3/4OS-1UEs, 2OS-1UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 5.88 | 8.70 | 8.70 | 9.33 | 8.70 | - | - | - | - | - | 0.48 | 0.89 | 1.40 | 1.45 | 1.17 |
|  | 50% | 33.33 | 33.33 | 34.17 | 35.00 | 35.00 | - | - | - | - | - | 7.14 | 15.38 | 17.73 | 18.92 | 18.42 |
|  | 95% | 50.00 | 66.67 | 68.35 | 70.03 | 63.65 | - | - | - | - | - | 33.33 | 50.00 | 58.33 | 53.85 | 53.85 |
|  | Mean | 31.03 | 36.37 | 36.70 | 38.11 | 36.71 | - | - | - | - | - | 11.18 | 19.44 | 21.00 | 21.86 | 21.34 |
| DL: Delay CDF [s] | 5% | 0.0170 | 0.0115 | 0.0115 | 0.0107 | 0.0115 | - | - | - | - | - | 0.2090 | 0.1120 | 0.0717 | 0.0690 | 0.0856 |
|  | 50% | 0.0030 | 0.0030 | 0.0029 | 0.0029 | 0.0029 | - | - | - | - | - | 0.0140 | 0.0065 | 0.0056 | 0.0053 | 0.0054 |
|  | 95% | 0.0020 | 0.0015 | 0.0015 | 0.0014 | 0.0016 | - | - | - | - | - | 0.0030 | 0.0020 | 0.0017 | 0.0019 | 0.0019 |
|  | Mean | 0.0076 | 0.0048 | 0.0043 | 0.0042 | 0.0043 | - | - | - | - | - | 0.0645 | 0.0522 | 0.0402 | 0.0399 | 0.0514 |
| RU |  | 0.23 | 0.23 | 0.25 | 0.25 | 0.27 | - | - | - | - | - | 0.76 | 0.74 | 0.73 | 0.73 | 0.76 |
| 𝜆 |  | 8.4 |  |  |  |  | - |  |  |  |  | 16.8 |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 0ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 3km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.33 | 3.08 | 3.52 | 3.52 | 2.83 | - | - | - | - | - | 0.28 | 0.32 | 0.39 | 0.31 | 0.10 |
|  | 50% | 12.50 | 13.33 | 15.07 | 15.22 | 12.85 | - | - | - | - | - | 3.57 | 7.41 | 8.14 | 7.87 | 7.00 |
|  | 95% | 33.33 | 33.33 | 31.47 | 29.17 | 25.00 | - | - | - | - | - | 16.67 | 25.00 | 23.95 | 23.34 | 19.72 |
|  | Mean | 14.78 | 15.79 | 16.14 | 15.98 | 13.45 | - | - | - | - | - | 5.21 | 9.00 | 9.53 | 9.22 | 7.96 |
| DL: Delay CDF [s] | 5% | 0.0430 | 0.0325 | 0.0285 | 0.0284 | 0.0353 | - | - | - | - | - | 0.3590 | 0.3160 | 0.2620 | 0.3240 | 1.0105 |
|  | 50% | 0.0080 | 0.0075 | 0.0066 | 0.0066 | 0.0078 | - | - | - | - | - | 0.0280 | 0.0135 | 0.0123 | 0.0127 | 0.0143 |
|  | 95% | 0.0030 | 0.0030 | 0.0032 | 0.0034 | 0.0040 | - | - | - | - | - | 0.0060 | 0.0040 | 0.0042 | 0.0043 | 0.0051 |
|  | Mean | 0.0126 | 0.0111 | 0.0106 | 0.0106 | 0.0145 | - | - | - | - | - | 0.0931 | 0.1010 | 0.0937 | 0.1073 | 0.1890 |
| RU |  | 0.27 | 0.30 | 0.31 | 0.30 | 0.37 | - | - | - | - | - | 0.79 | 0.81 | 0.80 | 0.82 | 0.91 |
| 𝜆 |  | 8.4 |  |  |  |  |  |  |  |  |  | 16.8 |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 0ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 3km/h, 10M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-21.43%, 7OS-23.14%, 3/4OS-26.57%, 2OS-31.71%, 1OS-42%Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | - | - | - | - | - | 0.08 | 0.28 | 0.36 | 0.62 | 0.45 |
|  | 50% | - | - | - | - | - | - | - | - | - | - | 4.55 | 14.29 | 15.66 | 16.67 | 16.67 |
|  | 95% | - | - | - | - | - | - | - | - | - | - | 33.33 | 50.00 | 50.93 | 53.85 | 50.00 |
|  | Mean | - | - | - | - | - | - | - | - | - | - | 8.00 | 17.88 | 18.89 | 19.97 | 19.64 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | - | - | - | - | - | 1.3130 | 0.3555 | 0.2819 | 0.1617 | 0.2208 |
|  | 50% | - | - | - | - | - | - | - | - | - | - | 0.0220 | 0.0070 | 0.0064 | 0.0060 | 0.0060 |
|  | 95% | - | - | - | - | - | - | - | - | - | - | 0.0030 | 0.0020 | 0.0020 | 0.0019 | 0.0020 |
|  | Mean | - | - | - | - | - | - | - | - | - | - | 0.2148 | 0.1152 | 0.0995 | 0.0743 | 0.0913 |
| RU |  | - | - | - | - | - | - | - | - | - | - | 0.84 | 0.83 | 0.82 | 0.80 | 0.82 |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  | 16.8 |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 0ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 120km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | - | - | - | - | - | 0.06 | 0.16 | 0.17 | 0.16 | 0.06 |
|  | 50% | - | - | - | - | - | - | - | - | - | - | 1.92 | 6.67 | 7.09 | 7.07 | 6.45 |
|  | 95% | - | - | - | - | - | - | - | - | - | - | 12.50 | 22.22 | 22.05 | 21.21 | 18.42 |
|  | Mean | - | - | - | - | - | - | - | - | - | - | 3.33 | 8.21 | 8.52 | 8.36 | 7.39 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | - | - | - | - | - | 1.7960 | 0.6185 | 0.5875 | 0.6207 | 1.6781 |
|  | 50% | - | - | - | - | - | - | - | - | - | - | 0.0520 | 0.0150 | 0.0141 | 0.0141 | 0.0155 |
|  | 95% | - | - | - | - | - | - | - | - | - | - | 0.0080 | 0.0045 | 0.0045 | 0.0047 | 0.0054 |
|  | Mean | - | - | - | - | - | - | - | - | - | - | 0.3222 | 0.1656 | 0.1531 | 0.1595 | 0.2424 |
| RU |  | - | - | - | - | - | - | - | - | - | - | 0.88 | 0.88 | 0.88 | 0.89 | 0.95 |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  | 16.8 |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 0ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 120km/h, 10M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-21.43%, 7OS-23.14%, 3/4OS-26.57%, 2OS-31.71%, 1OS-42%Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | 1.61 | 2.90 | - | 5.55 | 7.07 | 1.43 | 2.22 | - | 3.44 | 4.43 |
|  | 50% | - | - | - | - | - | 2.33 | 4.55 | - | 12.3 | 19.2 | 2.33 | 4.35 | - | 10.9 | 16.5 |
|  | 95% | - | - | - | - | - | 2.94 | 5.71 | - | 18.4 | 34.2 | 2.94 | 5.71 | - | 17.5 | 31.1 |
|  | Mean | - | - | - | - | - | 2.27 | 4.25 | - | 10.6 | 15.5 | 2.17 | 3.80 | - | 8.18 | 11.5 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | 34.0 | 17.5 | - | 5.4 | 2.9 | 34.0 | 17.5 | - | 5.7 | 3.2 |
|  | 50% | - | - | - | - | - | 43.0 | 22.0 | - | 8.2 | 5.2 | 43.0 | 23.0 | - | 9.2 | 6.1 |
|  | 95% | - | - | - | - | - | 62.0 | 34.5 | - | 18.0 | 14.1 | 70.0 | 45.0 | - | 29.0 | 22.6 |
|  | Mean | - | - | - | - | - | 44.0 | 23.5 | - | 9.4 | 6.4 | 46.2 | 26.3 | - | 12.2 | 8.7 |
| RU |  | - | - | - | - | - | 0.41 | 0.37 | - | 0.34 | 0.31 | 0.68 | 0.64 | - | 0.59 | 0.54 |
| 𝜆 |  |  |  |  |  |  | 1/0.12 |  |  |  |  | 1/0.08 |  |  |  |  |
| Notes: |  | Source 10, R1-163065, CN delay = 0ms, TCP slow start apply to each new traffic burst, 5-ms CSI period, 6-sTTI CSI delay, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-0.6%, 7OS-1.2%, 2OS-1.2%, 1OS-1.2% Maximum number of UEs that can be scheduled per sTTI: 8UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | 3.54 | 4.92 | - | 6.65 | 7.98 | 2.25 | 2.85 | - | 3.71 | 4.76 |
|  | 50% | - | - | - | - | - | 7.49 | 11.6 | - | 21.2 | 27.0 | 6.32 | 10.2 | - | 17.4 | 21.8 |
|  | 95% | - | - | - | - | - | 12.1 | 21.1 | - | 46.1 | 65.1 | 11.7 | 19.8 | - | 41.0 | 56.3 |
|  | Mean | - | - | - | - | - | 6.60 | 10.1 | - | 16.2 | 20.3 | 4.72 | 6.89 | - | 10.3 | 13.7 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | 8.3 | 4.7 | - | 2.2 | 1.5 | 8.5 | 5.0 | - | 2.4 | 1.8 |
|  | 50% | - | - | - | - | - | 13.4 | 8.6 | - | 4.7 | 3.7 | 15.8 | 9.8 | - | 5.7 | 4.6 |
|  | 95% | - | - | - | - | - | 28.3 | 20.3 | - | 15.0 | 12.5 | 44.5 | 35.1 | - | 27.0 | 21.0 |
|  | Mean | - | - | - | - | - | 15.2 | 9.9 | - | 6.2 | 4.9 | 21.2 | 14.5 | - | 9.7 | 7.3 |
| RU |  | - | - | - | - | - | 0.34 | 0.33 | - | 0.31 | 0.28 | 0.60 | 0.58 | - | 0.54 | 0.49 |
| 𝜆 |  |  |  |  |  |  | 1/0.12 |  |  |  |  | 1/0.08 |  |  |  |  |
| Notes: |  | Source 10, R1-163065, CN delay = 0ms, UDP, 5-ms CSI period, 6-sTTI CSI delay Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-0.6%, 7OS-1.2%, 2OS-1.2%, 1OS-1.2% Maximum number of UEs that can be scheduled per sTTI: 8UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | 1.61 | 2.86 | - | 6.79 | 8.65 | 1.43 | 2.50 | - | 4.79 | 5.86 |
|  | 50% | - | - | - | - | - | 2.33 | 4.35 | - | 12.7 | 20.6 | 2.33 | 3.92 | - | 11.5 | 17.7 |
|  | 95% | - | - | - | - | - | 2.94 | 5.71 | - | 19.4 | 35.0 | 2.94 | 5.71 | - | 17.9 | 32.6 |
|  | Mean | - | - | - | - | - | 2.27 | 4.09 | - | 11.5 | 17.5 | 2.17 | 3.78 | - | 9.57 | 13.7 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | 34.0 | 17.5 | - | 5.1 | 2.9 | 34.0 | 17.5 | - | 5.6 | 3.1 |
|  | 50% | - | - | - | - | - | 43.0 | 23.0 | - | 7.9 | 4.9 | 43.0 | 25.5 | - | 8.7 | 5.6 |
|  | 95% | - | - | - | - | - | 62.0 | 35.0 | - | 14.7 | 11.6 | 70.0 | 40.0 | - | 20.9 | 17.1 |
|  | Mean | - | - | - | - | - | 44.0 | 24.4 | - | 8.7 | 5.7 | 46.2 | 26.5 | - | 10.4 | 7.3 |
| RU |  | - | - | - | - | - | 0.41 | 0.31 | - | 0.28 | 0.26 | 0.68 | 0.55 | - | 0.48 | 0.45 |
| 𝜆 |  |  |  |  |  |  | 1/0.12 |  |  |  |  | 1/0.08 |  |  |  |  |
| Notes: |  | Source 10, R1-163072, CN delay = 0ms, TCP slow start apply to each new traffic burst, 5-ms CSI period, 6-sTTI CSI delay, fast CQI per sTTI Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-0.6%, 7OS-1.2%, 2OS-1.2%, 1OS-1.2% Maximum number of UEs that can be scheduled per sTTI: 8UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | 3.54 | 5.69 | - | 9.17 | 10.5 | 2.25 | 3.95 | - | 6.11 | 7.05 |
|  | 50% | - | - | - | - | - | 7.49 | 11.7 | - | 25.2 | 32.8 | 6.32 | 10.5 | - | 21.0 | 26.7 |
|  | 95% | - | - | - | - | - | 12.1 | 22.7 | - | 51.3 | 70.8 | 11.7 | 21.0 | - | 47.4 | 65.8 |
|  | Mean | - | - | - | - | - | 6.60 | 10.7 | - | 20.5 | 25.2 | 4.72 | 8.50 | - | 15.4 | 18.9 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | 8.3 | 4.4 | - | 1.9 | 1.4 | 8.5 | 4.8 | - | 2.1 | 1.5 |
|  | 50% | - | - | - | - | - | 13.4 | 8.5 | - | 4.0 | 3.0 | 15.8 | 9.5 | - | 4.8 | 3.7 |
|  | 95% | - | - | - | - | - | 28.3 | 17.6 | - | 10.9 | 9.6 | 44.5 | 25.3 | - | 16.4 | 14.2 |
|  | Mean | - | - | - | - | - | 15.2 | 9.3 | - | 4.9 | 4.0 | 21.2 | 11.8 | - | 6.5 | 5.3 |
| RU |  | - | - | - | - | - | 0.34 | 0.27 | - | 0.24 | 0.23 | 0.60 | 0.48 | - | 0.42 | 0.40 |
| 𝜆 |  |  |  |  |  |  | 1/0.12 |  |  |  |  | 1/0.08 |  |  |  |  |
| Notes: |  | Source 10, R1-163072, CN delay = 0ms, UDP, 5-ms CSI period, 6-sTTI CSI delay, fast CQI per sTTI Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-0.6%, 7OS-1.2%, 2OS-1.2%, 1OS-1.2% Maximum number of UEs that can be scheduled per sTTI: 8UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL: UPT CDF [Mbps] | 5% | 4.6917 | 7.3425 | 10.1940 | - | - | 4.0583 | 5.8846 | 7.4901 | - | - | 0.1475 | 0.1652 | 0.2076 | - | - |
|  | 50% | 4.8027 | 7.7573 | 10.9814 | - | - | 4.6009 | 7.0707 | 9.5314 | - | - | 4.0702 | 5.9189 | 7.4772 | - | - |
|  | 95% | 4.8772 | 8.1235 | 11.7935 | - | - | 4.8952 | 7.9934 | 11.5251 | - | - | 4.8560 | 7.8225 | 11.2063 | - | - |
|  | Mean | 4.7953 | 7.7386 | 10.9836 | - | - | 4.5667 | 7.0567 | 9.6131 | - | - | 3.2073 | 4.7547 | 6.1723 | - | - |
| DL: Delay CDF [s] | 5% | 0.0222 | 0.0134 | 0.0092 | - | - | 0.0222 | 0.0135 | 0.0093 | - | - | 0.0223 | 0.0137 | 0.0095 | - | - |
|  | 50% | 0.0229 | 0.0141 | 0.0100 | - | - | 0.0243 | 0.0158 | 0.0118 | - | - | 0.0290 | 0.0204 | 0.0161 | - | - |
|  | 95% | 0.0237 | 0.0151 | 0.0108 | - | - | 0.0287 | 0.0196 | 0.0158 | - | - | 1.2285 | 1.2766 | 1.3693 | - | - |
|  | Mean | 0.0229 | 0.0142 | 0.0100 | - | - | 0.0247 | 0.0161 | 0.0120 | - | - | 0.2191 | 0.2204 | 0.2339 | - | - |
| RU |  | 20.6245 | 20.5881 | 20.6413 | - | - | 45.4310 | 45.8360 | 41.6793 | - | - | 66.1491 | 66.6248 | 67.3148 | - | - |
| 𝜆 |  | 40 |  |  |  |  | 80 |  |  |  |  | 120 |  |  |  |  |
| Notes: |  | Source 12, R1-163798, CN delay=0ms, UL access delay = 13TTI, FTP traffic model 3 Total control overhead per subframe compared to all available REs in the sTTIs: 14 OS:15%, 7 OS: 17%, 3/4 OS: 19% Maximum number of UEs that can be scheduled per sTTI: not modelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 2.85 | 4.11 |  |  |  | 2.72 | 3.81 |  |  |  | 2.59 | 3.32 |  |  |  |
|  | 50% | 2.95 | 4.29 |  |  |  | 2.94 | 4.30 |  |  |  | 2.93 | 4.23 |  |  |  |
|  | 95% | 2.97 | 4.54 |  |  |  | 2.97 | 4.62 |  |  |  | 2.97 | 4.32 |  |  |  |
|  | Mean | 2.93 | 4.29 |  |  |  | 2.91 | 4.27 |  |  |  | 2.87 | 4.08 |  |  |  |
| DL: Delay CDF [s] | 5% | 0.034 | 0.023 |  |  |  | 0.034 | 0.023 |  |  |  | 0.034 | 0.023 |  |  |  |
|  | 50% | 0.034 | 0.023 |  |  |  | 0.034 | 0.023 |  |  |  | 0.034 | 0.024 |  |  |  |
|  | 95% | 0.035 | 0.025 |  |  |  | 0.037 | 0.026 |  |  |  | 0.039 | 0.030 |  |  |  |
|  | Mean | 0.034 | 0.024 |  |  |  | 0.034 | 0.024 |  |  |  | 0.035 | 0.025 |  |  |  |
| RU |  | 0.23 | 0.14 |  |  |  | 0.43 | 0.3 |  |  |  | 0.63 | 0.54 |  |  |  |
| 𝜆 |  | 6.2 |  |  |  |  | 20 |  |  |  |  | 95 |  |  |  |  |
| Notes: |  | Source 4, R1-165429, FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Assumptions on processing time and mas TBS scaling for 14OS follow that of 7OS, CN delay = 0ms, CQI period = 5TTI, CQI delay = 6TTI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.2.1-2: Results with 100kB file transfer, 0ms core network delay

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 14OS | 7OS | 3/4OS | 2OS | - | 2.52 | 2.49 | 1.45 | 0.85 | - | 1.72 |  | 1.42 |  | 0.66 |  | 0.24 |  | - |
|  | 50% | 3.70 | 4.49 | 3.65 | 2.07 | - | 6.45 | 8.36 | 7.28 | 5.64 | - | 5.89 |  | 6.55 |  | 4.48 |  | 3.42 |  | - |
|  | 95% | 6.96 | 10.82 | 11.99 | 11.28 | - | 8.45 | 15.89 | 21.47 | 24.37 | - | 8.55 |  | 15.17 |  | 19.56 |  | 19.34 |  | - |
|  | Mean | 8.40 | 16.06 | 24.40 | 31.71 | - | 6.09 | 8.70 | 9.03 | 8.36 | - | 5.61 |  | 7.39 |  | 6.64 |  | 5.96 |  | - |
| DL: Delay CDF [s] | 5% | 6.59 | 10.64 | 12.88 | 13.56 | - | 0.095 | 0.050 | 0.037 | 0.033 | - | 0.094 |  | 0.053 |  | 0.041 |  | 0.041 |  | - |
|  | 50% | 0.095 | 0.050 | 0.033 | 0.025 | - | 0.125 | 0.095 | 0.110 | 0.140 | - | 0.136 |  | 0.122 |  | 0.178 |  | 0.230 |  | - |
|  | 95% | 0.115 | 0.074 | 0.067 | 0.071 | - | 0.318 | 0.332 | 0.554 | 0.941 | - | 0.464 |  | 0.581 |  | 1.185 |  | 2.079 |  | - |
|  | Mean | 0.216 | 0.180 | 0.219 | 0.386 | - | 0.155 | 0.131 | 0.206 | 0.310 | - | 0.191 |  | 0.195 |  | 0.412 |  | 0.588 |  | - |
| RU |  | 0.21 | 0.19 | 0.22 | 0.24 | - | 0.43 | 0.43 | 0.51 | 0.5 |  | 1.72 |  | 1.42 |  | 0.66 |  | 0.24 |  |  |
| 𝜆 |  | 0.4 |  |  |  |  | 0.8 |  |  |  |  | 1.2 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 4, R1-162503, CN delay = 0ms, CQI period = 5TTI, CQI delay = 6TTI FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.70 | 4.00 | 2.58 | 1.07 | - | 2.52 | 2.49 | 1.45 | 0.85 | - | 1.72 |  | 1.18 |  | 0.31 |  | 0.02 |  | - |
|  | 50% | 6.96 | 10.25 | 9.73 | 7.31 | - | 6.45 | 8.36 | 7.28 | 5.64 | - | 5.89 |  | 6.37 |  | 3.57 |  | 2.47 |  | - |
|  | 95% | 8.40 | 15.60 | 22.14 | 24.41 | - | 8.45 | 15.89 | 21.47 | 24.37 | - | 8.55 |  | 14.99 |  | 16.46 |  | 14.10 |  | - |
|  | Mean | 6.59 | 10.21 | 10.70 | 9.43 | - | 6.09 | 8.70 | 9.03 | 8.36 | - | 5.61 |  | 7.02 |  | 5.53 |  | 4.26 |  | - |
| DL: Delay CDF [s] | 5% | 0.095 | 0.051 | 0.036 | 0.033 | - | 0.095 | 0.050 | 0.037 | 0.033 | - | 0.094 |  | 0.053 |  | 0.048 |  | 0.056 |  | - |
|  | 50% | 0.115 | 0.078 | 0.082 | 0.109 | - | 0.125 | 0.095 | 0.110 | 0.140 | - | 0.136 |  | 0.125 |  | 0.221 |  | 0.290 |  | - |
|  | 95% | 0.216 | 0.200 | 0.320 | 0.742 | - | 0.318 | 0.332 | 0.554 | 0.941 | - | 0.464 |  | 0.699 |  | 1.508 |  | 2.891 |  | - |
|  | Mean | 0.133 | 0.095 | 0.119 | 0.258 | - | 0.155 | 0.131 | 0.206 | 0.310 | - | 0.191 |  | 0.230 |  | 0.514 |  | 0.728 |  | - |
| RU |  | 0.21 | 0.21 | 0.26 | 0.31 | - | 0.43 | 0.46 | 0.58 | 0.55 | - | 0.59 |  | 0.65 |  | 0.76 |  | 0.68 |  | - |
| 𝜆 |  | 0.4 |  |  |  |  | 0.8 |  |  |  |  | 1.2 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 4, R1-162503, CN delay = 0ms, CQI period = 5ms, CQI delay = 6ms FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.70 | 4.20 | 3.14 | 1.45 | - | 2.52 | 2.38 | 1.15 | 0.46 | - | 1.72 |  | 1.23 |  | 0.42 |  | 0.10 |  | - |
|  | 50% | 6.96 | 10.64 | 10.97 | 8.54 | - | 6.45 | 8.16 | 6.69 | 4.32 | - | 5.89 |  | 6.56 |  | 4.29 |  | 2.66 |  | - |
|  | 95% | 8.40 | 15.87 | 22.97 | 26.37 | - | 8.45 | 15.34 | 19.65 | 19.91 | - | 8.55 |  | 15.06 |  | 17.72 |  | 15.04 |  | - |
|  | Mean | 6.59 | 10.49 | 11.66 | 10.47 | - | 6.09 | 8.44 | 8.20 | 6.59 | - | 5.61 |  | 7.24 |  | 6.10 |  | 4.71 |  | - |
| DL: Delay CDF [s] | 5% | 0.095 | 0.050 | 0.035 | 0.030 | - | 0.095 | 0.052 | 0.041 | 0.040 | - | 0.094 |  | 0.053 |  | 0.045 |  | 0.053 |  | - |
|  | 50% | 0.115 | 0.075 | 0.073 | 0.094 | - | 0.125 | 0.098 | 0.119 | 0.182 | - | 0.136 |  | 0.121 |  | 0.184 |  | 0.286 |  | - |
|  | 95% | 0.216 | 0.190 | 0.255 | 0.552 | - | 0.318 | 0.343 | 0.675 | 1.305 | - | 0.464 |  | 0.649 |  | 1.624 |  | 2.213 |  | - |
|  | Mean | 0.133 | 0.091 | 0.101 | 0.186 | - | 0.155 | 0.139 | 0.234 | 0.381 | - | 0.191 |  | 0.216 |  | 0.449 |  | 0.687 |  | - |
| RU |  | 0.21 | 0.2 | 0.24 | 0.29 | - | 0.43 | 0.45 | 0.52 | 0.54 | - | 0.59 |  | 0.64 |  | 0.71 |  | 0.68 |  | - |
| 𝜆 |  | 0.4 |  |  |  |  | 0.8 |  |  |  |  | 1.2 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 4, R1-162503, CN delay = 0ms, CQI period = 5TTI, CQI delay = 6ms FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.70 | 4.48 | 3.36 | 1.82 | - | 2.52 | 2.47 | 1.48 | 0.72 | - | 1.72 |  | 1.49 |  | 0.63 |  | 0.18 |  | - |
|  | 50% | 6.96 | 10.76 | 12.12 | 10.49 | - | 6.45 | 8.27 | 6.92 | 4.96 | - | 5.89 |  | 6.51 |  | 4.65 |  | 3.09 |  | - |
|  | 95% | 8.40 | 16.04 | 23.94 | 29.40 | - | 8.45 | 15.78 | 21.23 | 21.25 | - | 8.55 |  | 15.18 |  | 18.13 |  | 18.23 |  | - |
|  | Mean | 6.59 | 10.57 | 12.69 | 12.24 | - | 6.09 | 8.67 | 8.78 | 7.45 | - | 5.61 |  | 7.36 |  | 6.56 |  | 5.44 |  | - |
| DL: Delay CDF [s] | 5% | 0.095 | 0.050 | 0.033 | 0.027 | - | 0.095 | 0.051 | 0.038 | 0.038 | - | 0.094 |  | 0.053 |  | 0.044 |  | 0.042 |  | - |
|  | 50% | 0.115 | 0.074 | 0.066 | 0.076 | - | 0.125 | 0.096 | 0.115 | 0.161 | - | 0.136 |  | 0.123 |  | 0.170 |  | 0.246 |  | - |
|  | 95% | 0.216 | 0.179 | 0.245 | 0.439 | - | 0.318 | 0.326 | 0.539 | 1.091 | - | 0.464 |  | 0.558 |  | 1.075 |  | 1.809 |  | - |
|  | Mean | 0.133 | 0.091 | 0.092 | 0.143 | - | 0.155 | 0.135 | 0.200 | 0.364 | - | 0.191 |  | 0.195 |  | 0.399 |  | 0.557 |  | - |
| RU |  | 0.21 | 0.2 | 0.22 | 0.26 | - | 0.43 | 0.43 | 0.51 | 0.52 | - | 0.59 |  | 0.63 |  | 0.7 |  | 0.67 |  | - |
| 𝜆 |  | 0.4 |  |  |  |  | 0.8 |  |  |  |  | 1.2 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 4, R1-162503, CN delay = 0ms, CQI period = 5ms, CQI delay = 6TTI FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 5.97 | 11.85 | 15.74 | 18.73 | 17.72 | - | - | - | - | - | 0.71 |  | 1.81 |  | 3.31 |  | 4.63 |  | 2.86 |
|  | 50% | 19.51 | 38.10 | 50.02 | 53.85 | 49.34 | - | - | - | - | - | 7.08 |  | 20.78 |  | 24.53 |  | 28.14 |  | 23.78 |
|  | 95% | 44.44 | 72.73 | 74.18 | 71.80 | 65.89 | - | - | - | - | - | 29.63 |  | 57.14 |  | 66.33 |  | 70.00 |  | 63.28 |
|  | Mean | 20.94 | 39.62 | 48.61 | 51.08 | 47.11 | - | - | - | - | - | 9.99 |  | 23.80 |  | 28.16 |  | 31.19 |  | 27.06 |
| DL: Delay CDF [s] | 5% | 0.1340 | 0.0675 | 0.0509 | 0.0427 | 0.0451 | - | - | - | - | - | 1.1300 |  | 0.4400 |  | 0.2595 |  | 0.1721 |  | 0.2801 |
|  | 50% | 0.0410 | 0.0210 | 0.0160 | 0.0149 | 0.0162 | - | - | - | - | - | 0.1130 |  | 0.0385 |  | 0.0328 |  | 0.0284 |  | 0.0336 |
|  | 95% | 0.0180 | 0.0110 | 0.0108 | 0.0111 | 0.0121 | - | - | - | - | - | 0.0270 |  | 0.0140 |  | 0.0121 |  | 0.0114 |  | 0.0126 |
|  | Mean | 0.0586 | 0.0283 | 0.0217 | 0.0198 | 0.0214 | - | - | - | - | - | 0.2880 |  | 0.1240 |  | 0.0815 |  | 0.0576 |  | 0.0872 |
| RU |  | 0.22 | 0.20 | 0.18 | 0.17 | 0.19 | - | - | - | - | - | 0.77 |  | 0.70 |  | 0.63 |  | 0.56 |  | 0.63 |
| 𝜆 |  | 1.05 |  |  |  |  | - |  |  |  |  | 2.1 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162618, CN delay = 0ms, CSI reporting period = 5*TTI, CSI measurement reference resource is located in the (n- 4* TTI) FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 60km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 5.97 | 10.81 | 14.20 | 15.77 | 15.28 | - | - | - | - | - | 0.71 |  | 1.63 |  | 2.04 |  | 2.04 |  | 1.71 |
|  | 50% | 19.51 | 36.36 | 46.28 | 49.56 | 45.71 | - | - | - | - | - | 7.08 |  | 20.00 |  | 21.96 |  | 22.67 |  | 21.25 |
|  | 95% | 44.44 | 72.73 | 74.18 | 71.80 | 65.89 | - | - | - | - | - | 29.63 |  | 57.14 |  | 61.55 |  | 62.92 |  | 56.85 |
|  | Mean | 20.94 | 38.11 | 46.43 | 48.23 | 44.42 | - | - | - | - | - | 9.99 |  | 23.32 |  | 25.73 |  | 26.28 |  | 24.24 |
| DL: Delay CDF [s] | 5% | 0.1340 | 0.0740 | 0.0563 | 0.0504 | 0.0524 | - | - | - | - | - | 1.1300 |  | 0.4910 |  | 0.3991 |  | 0.3914 |  | 0.4671 |
|  | 50% | 0.0410 | 0.0220 | 0.0173 | 0.0161 | 0.0175 | - | - | - | - | - | 0.1130 |  | 0.0400 |  | 0.0364 |  | 0.0353 |  | 0.0376 |
|  | 95% | 0.0180 | 0.0110 | 0.0108 | 0.0111 | 0.0121 | - | - | - | - | - | 0.0270 |  | 0.0140 |  | 0.0130 |  | 0.0127 |  | 0.0141 |
|  | Mean | 0.0586 | 0.0305 | 0.0233 | 0.0219 | 0.0232 | - | - | - | - | - | 0.2880 |  | 0.1422 |  | 0.1103 |  | 0.1137 |  | 0.1292 |
| RU |  | 0.22 | 0.22 | 0.20 | 0.19 | 0.20 | - | - | - | - | - | 0.77 |  | 0.72 |  | 0.69 |  | 0.68 |  | 0.72 |
| 𝜆 |  | 1.05 |  |  |  |  | - |  |  |  |  | 2.1 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162618, CN delay = 0ms, CSI reporting period = 5ms, CSI measurement reference resource is located in the (n- 4* TTI). FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 60km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 5.97 | 10.60 | 13.19 | 13.19 | 14.41 | - | - | - | - | - | 0.71 |  | 1.21 |  | 1.41 |  | 1.64 |  | 1.08 |
|  | 50% | 19.51 | 35.56 | 43.98 | 43.98 | 45.16 | - | - | - | - | - | 7.08 |  | 19.51 |  | 20.56 |  | 20.82 |  | 20.55 |
|  | 95% | 44.44 | 72.73 | 74.18 | 74.18 | 65.89 | - | - | - | - | - | 29.63 |  | 57.14 |  | 59.90 |  | 58.95 |  | 55.72 |
|  | Mean | 20.94 | 37.96 | 44.72 | 44.72 | 43.93 | - | - | - | - | - | 9.99 |  | 22.79 |  | 24.45 |  | 24.70 |  | 23.52 |
| DL: Delay CDF [s] | 5% | 0.1340 | 0.0750 | 0.0607 | 0.0607 | 0.0554 | - | - | - | - | - | 1.1300 |  | 0.6575 |  | 0.5705 |  | 0.4881 |  | 0.7361 |
|  | 50% | 0.0410 | 0.0225 | 0.0182 | 0.0182 | 0.0177 | - | - | - | - | - | 0.1130 |  | 0.0410 |  | 0.0389 |  | 0.0384 |  | 0.0389 |
|  | 95% | 0.0180 | 0.0110 | 0.0108 | 0.0108 | 0.0121 | - | - | - | - | - | 0.0270 |  | 0.0140 |  | 0.0134 |  | 0.0136 |  | 0.0144 |
|  | Mean | 0.0586 | 0.0308 | 0.0247 | 0.0247 | 0.0244 | - | - | - | - | - | 0.2880 |  | 0.1650 |  | 0.1503 |  | 0.1342 |  | 0.1799 |
| RU |  | 0.22 | 0.22 | 0.21 | 0.20 | 0.21 | - | - | - | - | - | 0.77 |  | 0.75 |  | 0.74 |  | 0.73 |  | 0.75 |
| 𝜆 |  | 1.05 |  |  |  |  | - |  |  |  |  | 2.1 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162618, CN delay = 0ms, CSI reporting period = 5ms, CSI measurement reference resource is located in the (n- 4ms). FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 60km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 3.38 | 3.74 | 3.68 | 3.72 | 3.35 | 2.43 | 2.73 | 2.88 | 2.90 | 2.70 |  | 1.66 |  | 1.76 |  | 1.63 |  | 1.44 | 1.04 |
|  | 50% | 7.43 | 8.64 | 9.75 | 10.68 | 9.58 | 6.23 | 6.95 | 7.80 | 7.81 | 6.23 |  | 4.60 |  | 4.77 |  | 5.17 |  | 4.99 | 3.83 |
|  | 95% | 9.84 | 15.73 | 20.87 | 24.16 | 24.67 | 9.47 | 15.09 | 19.72 | 18.89 | 18.27 |  | 9.17 |  | 13.25 |  | 14.13 |  | 14.09 | 11.01 |
|  | Mean | 7.18 | 9.31 | 11.25 | 12.02 | 11.29 | 6.30 | 7.86 | 9.16 | 8.96 | 7.98 |  | 5.11 |  | 5.76 |  | 6.13 |  | 5.80 | 4.96 |
| DL: Delay CDF [s] | 5% | 0.25 | 0.22 | 0.23 | 0.23 | 0.25 | 0.35 | 0.31 | 0.29 | 0.29 | 0.31 |  | 0.51 |  | 0.48 |  | 0.51 |  | 0.58 | 0.81 |
|  | 50% | 0.11 | 0.10 | 0.09 | 0.08 | 0.09 | 0.13 | 0.12 | 0.11 | 0.10 | 0.13 |  | 0.18 |  | 0.18 |  | 0.16 |  | 0.17 | 0.21 |
|  | 95% | 0.09 | 0.05 | 0.04 | 0.04 | 0.03 | 0.09 | 0.06 | 0.04 | 0.04 | 0.05 |  | 0.09 |  | 0.06 |  | 0.06 |  | 0.06 | 0.08 |
|  | Mean | 0.13 | 0.11 | 0.10 | 0.09 | 0.10 | 0.16 | 0.14 | 0.13 | 0.14 | 0.15 |  | 0.22 |  | 0.21 |  | 0.21 |  | 0.22 | 0.31 |
| RU |  | 18 | 21 | 22 | 21 | 24 | 36 | 39 | 42 | 43 | 48 |  | 59 |  | 66 |  | 69 |  | 69 | 77 |
| 𝜆 |  | 0.33 |  |  |  |  | 0.59 |  |  |  |  |  | 1.00 |  |  |  |  |  |  |  |
| Notes: |  | Source 7, R1-163403, CN delay = 0ms, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-15%, 7OS-18.0%, 3/4OS-19%, 2OS-21.5%, 1OS-28%; Maximum number of UEs that can be scheduled per sTTI: 14OS-5UEs, 7OS-2UEs, 3/4OS-1UEs, 2OS-1UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 5.88 | 11.51 | 16.21 | 18.60 | 19.48 | - | - | - | - | - | 2.96 |  | 3.61 |  | 4.94 |  | 6.18 |  | 3.61 |
|  | 50% | 18.18 | 39.02 | 48.15 | 50.91 | 49.12 | - | - | - | - | - | 11.94 |  | 22.86 |  | 26.77 |  | 29.17 |  | 25.75 |
|  | 95% | 44.44 | 72.73 | 74.18 | 71.80 | 65.89 | - | - | - | - | - | 38.10 |  | 64.00 |  | 70.05 |  | 71.80 |  | 64.74 |
|  | Mean | 21.05 | 40.21 | 47.63 | 49.57 | 47.28 | - | - | - | - | - | 14.67 |  | 26.90 |  | 30.88 |  | 32.74 |  | 28.79 |
| DL: Delay CDF [s] | 5% | 0.1360 | 0.0695 | 0.0495 | 0.0430 | 0.0411 | - | - | - | - | - | 0.2700 |  | 0.2200 |  | 0.1620 |  | 0.1291 |  | 0.2209 |
|  | 50% | 0.0440 | 0.0205 | 0.0167 | 0.0157 | 0.0163 | - | - | - | - | - | 0.0670 |  | 0.0350 |  | 0.0299 |  | 0.0274 |  | 0.0311 |
|  | 95% | 0.0180 | 0.0110 | 0.0108 | 0.0111 | 0.0121 | - | - | - | - | - | 0.0210 |  | 0.0125 |  | 0.0113 |  | 0.0111 |  | 0.0124 |
|  | Mean | 0.0613 | 0.0296 | 0.0224 | 0.0200 | 0.0203 | - | - | - | - | - | 0.1007 |  | 0.0671 |  | 0.0516 |  | 0.0448 |  | 0.0656 |
| RU |  | 0.21 | 0.20 | 0.18 | 0.17 | 0.18 |  | - | - | - | - | 0.64 |  | 0.60 |  | 0.56 |  | 0.52 |  | 0.61 |
| 𝜆 |  | 1.05 |  |  |  |  |  |  |  |  |  | 2.1 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 0ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 3km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 4.71 | 6.48 | 8.52 | 8.27 | 5.54 | - | - | - | - | - | 1.38 |  | 1.99 |  | 2.17 |  | 1.26 |  | 0.70 |
|  | 50% | 8.60 | 15.38 | 21.31 | 21.71 | 17.13 | - | - | - | - | - | 5.80 |  | 10.81 |  | 12.27 |  | 10.81 |  | 8.41 |
|  | 95% | 32.00 | 34.78 | 32.85 | 30.27 | 25.11 | - | - | - | - | - | 22.22 |  | 29.63 |  | 31.79 |  | 28.87 |  | 22.31 |
|  | Mean | 12.51 | 18.52 | 21.42 | 20.89 | 16.59 | - | - | - | - | - | 7.71 |  | 12.60 |  | 13.79 |  | 12.23 |  | 9.58 |
| DL: Delay CDF [s] | 5% | 0.1690 | 0.1225 | 0.0942 | 0.0967 | 0.1436 | - | - | - | - | - | 0.5760 |  | 0.4000 |  | 0.3732 |  | 0.6319 |  | 1.1362 |
|  | 50% | 0.0930 | 0.0520 | 0.0377 | 0.0369 | 0.0467 | - | - | - | - | - | 0.1380 |  | 0.0740 |  | 0.0655 |  | 0.0740 |  | 0.0951 |
|  | 95% | 0.0250 | 0.0230 | 0.0244 | 0.0264 | 0.0319 | - | - | - | - | - | 0.0350 |  | 0.0270 |  | 0.0252 |  | 0.0277 |  | 0.0358 |
|  | Mean | 0.0901 | 0.0569 | 0.0457 | 0.0480 | 0.0656 | - | - | - | - | - | 0.2049 |  | 0.1268 |  | 0.1211 |  | 0.1785 |  | 0.2681 |
| RU |  | 0.21 | 0.20 | 0.19 | 0.20 | 0.26 |  | - | - | - | - | 0.64 |  | 0.61 |  | 0.59 |  | 0.67 |  | 0.79 |
| 𝜆 |  | 1.05 |  |  |  |  |  |  |  |  |  | 2.1 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 0ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 3km/h, 10M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-21.43%, 7OS-23.14%, 3/4OS-26.57%, 2OS-31.71%, 1OS-42%Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | - | - | - | - | - |  | 0.62 |  | 1.39 |  | 1.97 |  | 2.87 | 1.94 |
|  | 50% | - | - | - | - | - | - | - | - | - | - |  | 6.35 |  | 18.18 |  | 21.47 |  | 24.35 | 22.05 |
|  | 95% | - | - | - | - | - | - | - | - | - | - |  | 28.57 |  | 55.17 |  | 62.07 |  | 64.37 | 59.58 |
|  | Mean | - | - | - | - | - | - | - | - | - | - |  | 9.18 |  | 22.23 |  | 25.33 |  | 27.75 | 25.01 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | - | - | - | - | - |  | 1.2790 |  | 0.5720 |  | 0.4057 |  | 0.2787 | 0.4091 |
|  | 50% | - | - | - | - | - | - | - | - | - | - |  | 0.1260 |  | 0.0440 |  | 0.0373 |  | 0.0329 | 0.0363 |
|  | 95% | - | - | - | - | - | - | - | - | - | - |  | 0.0280 |  | 0.0145 |  | 0.0129 |  | 0.0124 | 0.0134 |
|  | Mean | - | - | - | - | - | - | - | - | - | - |  | 0.3243 |  | 0.1498 |  | 0.1154 |  | 0.0872 | 0.1201 |
| RU |  |  | - | - | - | - | - | - | - | - | - |  | 0.79 |  | 0.74 |  | 0.70 |  | 0.64 | 0.69 |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  | 2.1 |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 0ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 120km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | - | - | - | - | - |  | 0.43 |  | 0.85 |  | 0.63 |  | 0.65 | 0.38 |
|  | 50% | - | - | - | - | - | - | - | - | - | - |  | 3.59 |  | 8.25 |  | 8.74 |  | 8.33 | 6.83 |
|  | 95% | - | - | - | - | - | - | - | - | - | - |  | 15.09 |  | 25.81 |  | 25.60 |  | 23.93 | 19.86 |
|  | Mean | - | - | - | - | - | - | - | - | - | - |  | 4.96 |  | 10.04 |  | 10.30 |  | 9.82 | 8.06 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | - | - | - | - | - |  | 1.8660 |  | 0.9370 |  | 1.2736 |  | 1.2343 | 2.1024 |
|  | 50% | - | - | - | - | - | - | - | - | - | - |  | 0.2230 |  | 0.0970 |  | 0.0914 |  | 0.0959 | 0.1171 |
|  | 95% | - | - | - | - | - | - | - | - | - | - |  | 0.0530 |  | 0.0310 |  | 0.0311 |  | 0.0334 | 0.0402 |
|  | Mean | - | - | - | - | - | - | - | - | - | - |  | 0.4838 |  | 0.2603 |  | 0.2878 |  | 0.2898 | 0.4245 |
| RU |  |  | - | - | - | - | - | - | - | - | - |  | 0.80 |  | 0.78 |  | 0.80 |  | 0.81 | 0.88 |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  | 2.1 |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 0ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 120km/h, 10M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-21.43%, 7OS-23.14%, 3/4OS-26.57%, 2OS-31.71%, 1OS-42%Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL: UPT CDF [Mbps] | 5% | 6.50 | 9.25 | - | 11.8 | 13.2 | 5.23 | 7.14 | - | 9.14 | 10.3 | 1.67 | 2.21 | - | 2.98 | 3.60 |
|  | 50% | 10.4 | 18.6 | - | 36.3 | 45.2 | 9.76 | 17.0 | - | 31.1 | 37.3 | 8.16 | 12.5 | - | 19.1 | 22.3 |
|  | 95% | 12.9 | 25.4 | - | 65.8 | 86.2 | 12.9 | 25.4 | - | 64.3 | 84.9 | 12.7 | 22.9 | - | 48.7 | 62.2 |
|  | Mean | 9.70 | 16.3 | - | 27.9 | 33.1 | 8.74 | 14.2 | - | 22.9 | 27.0 | 4.98 | 7.03 | - | 9.95 | 11.9 |
| DL: Delay CDF [s] | 5% | 0.062 | 0.032 | - | 0.012 | 0.009 | 0.062 | 0.032 | - | 0.012 | 0.009 | 0.063 | 0.035 | - | 0.016 | 0.013 |
|  | 50% | 0.077 | 0.043 | - | 0.022 | 0.018 | 0.082 | 0.047 | - | 0.026 | 0.214 | 0.098 | 0.064 | - | 0.042 | 0.036 |
|  | 95% | 0.123 | 0.087 | - | 0.068 | 0.061 | 0.153 | 0.112 | - | 0.088 | 0.078 | 0.479 | 0.363 | - | 0.268 | 0.222 |
|  | Mean | 0.083 | 0.049 | - | 0.029 | 0.024 | 0.092 | 0.056 | - | 0.035 | 0.030 | 0.161 | 0.114 | - | 0.080 | 0.068 |
| RU |  | 0.21 | 0.19 | - | 0.18 | 0.16 | 0.38 | 0.34 | - | 0.30 | 0.28 | 0.88 | 0.82 | - | 0.77 | 0.73 |
| 𝜆 |  | 1/1.2 |  |  |  |  | 1/0.8 |  |  |  |  | 1/0.4 |  |  |  |  |
| Notes: |  | Source 10, R1-163065, CN delay = 0ms, TCP slow start apply to each new traffic burst, 5-ms CSI period, 6-sTTI CSI delay Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-0.6%, 7OS-1.2%, 2OS-1.2%, 1OS-1.2% Maximum number of UEs that can be scheduled per sTTI: 8UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 9.55 | 11.3 | - | 12.3 | 13.3 | 4.59 | 5.91 | - | 6.72 | 7.20 | - | - | - | - | - |
|  | 50% | 28.2 | 35.8 | - | 45.5 | 48.9 | 20.1 | 25.7 | - | 31.1 | 33.5 | - | - | - | - | - |
|  | 95% | 54.8 | 73.5 | - | 93.8 | 102 | 47.8 | 65.3 | - | 82.6 | 91.9 | - | - | - | - | - |
|  | Mean | 22.2 | 27.5 | - | 32.2 | 34.7 | 13.0 | 16.8 | - | 19.8 | 21.2 | - | - | - | - | - |
| DL: Delay CDF [s] | 5% | 14.6 | 10.9 | - | 8.5 | 7.8 | 16.7 | 12.2 | - | 9.7 | 8.7 | - | - | - | - | - |
|  | 50% | 28.4 | 22.3 | - | 17.6 | 16.4 | 39.8 | 31.2 | - | 25.8 | 23.9 | - | - | - | - | - |
|  | 95% | 83.8 | 70.9 | - | 64.8 | 60.0 | 174 | 135 | - | 119 | 111 | - | - | - | - | - |
|  | Mean | 36.0 | 29.1 | - | 24.9 | 23.0 | 61.6 | 47.4 | - | 40.5 | 37.7 | - | - | - | - | - |
| RU |  | 0.19 | 0.18 | - | 0.17 | 0.16 | 0.50 | 0.45 | - | 0.43 | 0.41 | - | - | - | - | - |
| 𝜆 |  | 1/1.2 |  |  |  |  | 1/0.6 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 10, R1-163065, CN delay = 0ms, UDP, 5-ms CSI period, 6-sTTI CSI delay Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-0.6%, 7OS-1.2%, 2OS-1.2%, 1OS-1.2% Maximum number of UEs that can be scheduled per sTTI: 8UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 6.50 | 10.4 | - | 13.4 | 14.6 | 5.23 | 8.33 | - | 10.9 | 11.6 | 1.67 | 3.05 | - | 4.30 | 4.55 |
|  | 50% | 10.4 | 19.5 | - | 40.8 | 48.8 | 9.76 | 18.0 | - | 36.1 | 42.3 | 8.16 | 13.9 | - | 23.2 | 26.1 |
|  | 95% | 12.9 | 25.4 | - | 68.2 | 86.9 | 12.9 | 25.4 | - | 67.4 | 86.2 | 12.7 | 23.9 | - | 55.9 | 70.9 |
|  | Mean | 9.70 | 17.5 | - | 31.4 | 35.9 | 8.74 | 15.5 | - | 26.8 | 30.3 | 4.98 | 8.74 | - | 13.4 | 14.6 |
| DL: Delay CDF [s] | 5% | 0.062 | 0.032 | - | 0.012 | 0.009 | 0.062 | 0.032 | - | 0.012 | 0.009 | 0.063 | 0.034 | - | 0.014 | 0.011 |
|  | 50% | 0.077 | 0.041 | - | 0.020 | 0.016 | 0.082 | 0.045 | - | 0.022 | 0.019 | 0.098 | 0.058 | - | 0.035 | 0.031 |
|  | 95% | 0.123 | 0.077 | - | 0.060 | 0.055 | 0.153 | 0.096 | - | 0.073 | 0.069 | 0.479 | 0.262 | - | 0.186 | 0.176 |
|  | Mean | 0.083 | 0.046 | - | 0.025 | 0.022 | 0.092 | 0.052 | - | 0.030 | 0.026 | 0.161 | 0.092 | - | 0.060 | 0.055 |
| RU |  | 0.21 | 0.17 | - | 0.16 | 0.15 | 0.38 | 0.30 | - | 0.26 | 0.26 | 0.88 | 0.75 | - | 0.68 | 0.67 |
| 𝜆 |  | 1/1.2 |  |  |  |  | 1/0.8 |  |  |  |  | 1/0.4 |  |  |  |  |
| Notes: |  | Source 10, R1-163072, CN delay = 0ms, TCP slow start apply to each new traffic burst, 5-ms CSI period, 6-sTTI CSI delay, fast CQI per sTTI Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-0.6%, 7OS-1.2%, 2OS-1.2%, 1OS-1.2% Maximum number of UEs that can be scheduled per sTTI: 8UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 9.55 | 12.9 | - | 14.2 | 14.5 | 4.59 | 7.45 | - | 8.76 | 8.93 | - | - | - | - | - |
|  | 50% | 28.2 | 40.7 | - | 50.8 | 53.2 | 20.1 | 29.6 | - | 37.3 | 39.6 | - | - | - | - | - |
|  | 95% | 54.8 | 74.6 | - | 97.2 | 104 | 47.8 | 69.7 | - | 93.8 | 100.3 | - | - | - | - | - |
|  | Mean | 22.2 | 30.7 | - | 36.7 | 38.0 | 13.0 | 20.4 | - | 24.8 | 25.7 | - | - | - | - | - |
| DL: Delay CDF [s] | 5% | 14.6 | 10.7 | - | 8.2 | 7.7 | 16.7 | 11.5 | - | 8.5 | 8.0 | - | - | - | - | - |
|  | 50% | 28.4 | 19.7 | - | 15.8 | 15.0 | 39.8 | 27.0 | - | 21.5 | 20.2 | - | - | - | - | - |
|  | 95% | 83.8 | 61.9 | - | 56.3 | 55.3 | 174 | 107 | - | 91.4 | 89.5 | - | - | - | - | - |
|  | Mean | 36.0 | 26.0 | - | 21.8 | 21.0 | 61.6 | 39.2 | - | 32.3 | 31.2 | - | - | - | - | - |
| RU |  | 0.19 | 0.16 | - | 0.15 | 0.15 | 0.50 | 0.39 | - | 0.37 | 0.36 | - | - | - | - | - |
| 𝜆 |  | 1/1.2 |  |  |  |  | 1/0.6 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 10, R1-163072, CN delay = 0ms, UDP, 5-ms CSI period, 6-sTTI CSI delay, fast CQI per sTTI Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-0.6%, 7OS-1.2%, 2OS-1.2%, 1OS-1.2% Maximum number of UEs that can be scheduled per sTTI: 8UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL: UPT CDF [Mbps] | 5% | 17.6469 | 25.0747 | 29.3540 | - | - | 12.4772 | 15.0259 | 16.1946 | - | - | 1.0630 | 1.2719 | 1.5864 | - | - |
|  | 50% | 18.7934 | 28.6268 | 33.6199 | - | - | 16.7899 | 22.6137 | 26.0684 | - | - | 12.0753 | 14.1659 | 15.1013 | - | - |
|  | 95% | 19.6400 | 31.7421 | 37.5620 | - | - | 19.6025 | 30.9845 | 35.9645 | - | - | 19.3781 | 28.9365 | 33.2371 | - | - |
|  | Mean | 18.7281 | 28.4343 | 33.5681 | - | - | 16.6618 | 23.1218 | 26.3110 | - | - | 10.8497 | 14.1127 | 15.6010 | - | - |
| DL: Delay CDF [s] | 5% | 0.0447 | 0.0279 | 0.0228 | - | - | 0.0438 | 0.0280 | 0.0240 | - | - | 0.0435 | 0.0301 | 0.0259 | - | - |
|  | 50% | 0.0476 | 0.0316 | 0.0264 | - | - | 0.0549 | 0.0407 | 0.0358 | - | - | 0.0922 | 0.0823 | 0.0885 | - | - |
|  | 95% | 0.0520 | 0.0364 | 0.0310 | - | - | 0.0839 | 0.0685 | 0.0646 | - | - | 1.3742 | 1.4880 | 1.3034 | - | - |
|  | Mean | 0.0480 | 0.0319 | 0.0266 | - | - | 0.0581 | 0.0442 | 0.0405 | - | - | 0.2953 | 0.3004 | 0.3052 | - | - |
| RU |  | 19.7010 | 19.6027 | 19.8235 | - | - | 42.9873 | 43.4095 | 43.9975 | - | - | 65.0172 | 65.6092 | 66.1828 | - | - |
| 𝜆 |  | 5 |  |  |  |  | 10 |  |  |  |  | 15 |  |  |  |  |
| Notes: |  | Source 12, R1-163798, CN delay=0ms, UL access delay = 13TTI, FTP traffic model 3 Total control overhead per subframe compared to all available REs in the sTTIs: 14 OS:15%, 7 OS: 17%, 3/4 OS: 19% Maximum number of UEs that can be scheduled per sTTI: not modelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  | 5% |  | 3.72 |  | 4.49 |  |  |  |  |  |  |  | 2.15 |  | 2.49 |  |  |  |  |  |  |  | 1.26 |  |  | 1.42 |  |  |  |
|  |  | 50% |  | 8.63 |  | 10.82 |  |  |  |  |  |  |  | 7.02 |  | 8.36 |  |  |  |  |  |  |  | 5.95 |  |  | 6.55 |  |  |  |
|  |  | 95% |  | 11.66 |  | 16.06 |  |  |  |  |  |  |  | 11.61 |  | 15.89 |  |  |  |  |  |  |  | 11.59 |  |  | 15.17 |  |  |  |
|  |  | Mean |  | 8.27 |  | 10.64 |  |  |  |  |  |  |  | 7.07 |  | 8.70 |  |  |  |  |  |  |  | 6.19 |  |  | 7.39 |  |  |  |
| DL: Delay CDF [s] |  | 5% |  | 0.069 |  | 0.050 |  |  |  |  |  |  |  | 0.069 |  | 0.050 |  |  |  |  |  |  |  | 0.069 |  |  | 0.053 |  |  |  |
|  |  | 50% |  | 0.092 |  | 0.074 |  |  |  |  |  |  |  | 0.113 |  | 0.095 |  |  |  |  |  |  |  | 0.134 |  |  | 0.122 |  |  |  |
|  |  | 95% |  | 0.215 |  | 0.180 |  |  |  |  |  |  |  | 0.373 |  | 0.332 |  |  |  |  |  |  |  | 0.637 |  |  | 0.581 |  |  |  |
|  |  | Mean |  | 0.112 |  | 0.089 |  |  |  |  |  |  |  | 0.154 |  | 0.131 |  |  |  |  |  |  |  | 0.219 |  |  | 0.195 |  |  |  |
| RU |  | 0.22 | 0.19 |  |  |  | 0.47 | 0.43 |  |  |  | 0.64 | 0.63 |  |  |  |
| 𝜆 |  | 0.4 |  |  |  |  | 0.8 |  |  |  |  | 1.2 |  |  |  |  |
| Notes: |  | Source 4, R1-165429, FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Assumptions on processing time and mas TBS scaling for 14OS follow that of 7OS, CN delay = 0ms, CQI period = 5TTI, CQI delay = 6TTI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.2.1-3: Results with 500kB file transfer in DL, 0ms core network delay

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 5.16 | 4.69 | - | 4.42 | 3.55 | 3.67 | 3.16 | - | 2.66 | 2.11 |  | 2.06 |  | 1.65 |  | - |  | 1.25 | 0.51 |
|  | 50% | 12.01 | 12.48 | - | 13.14 | 11.78 | 10.13 | 9.83 | - | 9.36 | 7.81 |  | 6.19 |  | 5.58 |  | - |  | 4.46 | 1.77 |
|  | 95% | 21.97 | 30.06 | - | 38.98 | 39.33 | 21.27 | 27.39 | - | 33.01 | 29.24 |  | 17.94 |  | 21.44 |  | - |  | 19.33 | 8.55 |
|  | Mean | 12.52 | 14.34 | - | 16.05 | 14.98 | 11.03 | 11.79 | - | 12.33 | 10.48 |  | 7.64 |  | 7.61 |  | - |  | 6.48 | 2.71 |
| DL: Delay CDF [s] | 5% | 0.182 | 0.133 | - | 0.103 | 0.102 | 0.188 | 0.146 | - | 0.121 | 0.137 |  | 0.223 |  | 0.187 |  | - |  | 0.207 | 0.468 |
|  | 50% | 0.333 | 0.320 | - | 0.305 | 0.337 | 0.395 | 0.407 | - | 0.427 | 0.512 |  | 0.645 |  | 0.717 |  | - |  | 0.897 | 2.254 |
|  | 95% | 0.775 | 0.848 | - | 0.904 | 1.122 | 1.085 | 1.266 | - | 1.502 | 1.892 |  | 1.942 |  | 2.422 |  | - |  | 3.192 | 7.911 |
|  | Mean | 0.390 | 0.379 | - | 0.378 | 0.444 | 0.485 | 0.516 | - | 0.567 | 0.697 |  | 0.803 |  | 0.950 |  | - |  | 1.195 | 2.896 |
| RU |  | 24.22% | 22.88% | - | 20.64% | 21.94% | 45.30% | 43.67% | - | 39.72% | 41.00% |  | 88.92% |  | 81.44% |  | - |  | 72.71% | 44.50% |
| 𝜆 |  | - |  |  |  |  | - |  |  |  |  |  | - |  |  |  |  |  |  |  |
| Notes: |  | Source 3, R1-165446, macro scenario, Static PDCCH overhead Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-28%,2OS-29.71%, 1OS-34.86% Total Reference signal overhead (CRS + DMRS) per subframe [%]: 14OS-16.67%, 7OS-15.48%, 2OS-23.81%, 1OS-23.81% Maximum number of UEs that can be scheduled per sTTI: Low RU-4UEs, Medium RU-4UEs, High RU-9UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 4.52 | 5.53 | 4.01 | 2.26 | - | 2.96 | 3.03 | 1.88 | 0.96 | - |  | 1.65 |  | 1.50 |  | 0.71 |  | 0.47 | - |
|  | 50% | 12.84 | 17.34 | 15.39 | 13.53 | - | 9.95 | 11.45 | 8.74 | 6.91 | - |  | 7.10 |  | 6.98 |  | 4.36 |  | 3.14 | - |
|  | 95% | 22.08 | 35.07 | 36.65 | 42.55 | - | 20.89 | 30.30 | 30.31 | 28.31 | - |  | 19.28 |  | 24.08 |  | 19.36 |  | 15.68 | - |
|  | Mean | 13.10 | 18.50 | 17.20 | 16.74 | - | 10.73 | 13.53 | 11.71 | 9.50 | - |  | 8.41 |  | 9.19 |  | 6.50 |  | 5.12 | - |
| DL: Delay CDF [s] | 5% | 0.181 | 0.114 | 0.109 | 0.093 | - | 0.191 | 0.132 | 0.134 | 0.141 | - |  | 0.207 |  | 0.166 |  | 0.207 |  | 0.252 | - |
|  | 50% | 0.311 | 0.230 | 0.259 | 0.288 | - | 0.406 | 0.351 | 0.457 | 0.575 | - |  | 0.569 |  | 0.573 |  | 0.911 |  | 1.193 | - |
|  | 95% | 0.793 | 0.707 | 0.959 | 1.681 | - | 1.507 | 1.358 | 2.255 | 3.722 | - |  | 2.688 |  | 2.872 |  | 5.072 |  | 6.571 | - |
|  | Mean | 0.381 | 0.290 | 0.368 | 0.521 | - | 0.558 | 0.514 | 0.731 | 1.106 | - |  | 0.876 |  | 0.899 |  | 1.520 |  | 1.976 | - |
| RU |  | 0.19 | 0.17 | 0.21 | 0.23 | - | 0.4 | 0.38 | 0.45 | 0.47 | - |  | 0.61 |  | 0.61 |  | 0.71 |  | 0.65 | - |
| 𝜆 |  | 0.08 |  |  |  |  | 0.15 |  |  |  |  |  | 0.24 |  |  |  |  |  |  |  |
| Notes: |  | Source 4, R1-162503, CN delay = 0ms, CQI period = 5TTI, CQI delay = 6TTI FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |
| DL: UPT CDF [Mbps] | 5% | 4.52 | 5.55 | 2.96 | 1.48 | - | 2.96 | 2.67 | 1.47 | 0.61 | - | 1.65 |  | 1.23 |  | 0.59 |  | 0.05 |  | - |
|  | 50% | 12.84 | 15.99 | 12.77 | 7.98 | - | 9.95 | 11.20 | 7.37 | 4.58 | - | 7.10 |  | 6.32 |  | 3.51 |  | 2.58 |  | - |
|  | 95% | 22.08 | 33.98 | 33.23 | 31.03 | - | 20.89 | 29.88 | 26.54 | 21.29 | - | 19.28 |  | 24.77 |  | 16.92 |  | 12.75 |  | - |
|  | Mean | 13.10 | 17.34 | 14.64 | 11.27 | - | 10.73 | 13.06 | 9.77 | 6.98 | - | 8.41 |  | 8.77 |  | 5.57 |  | 4.04 |  | - |
| DL: Delay CDF [s] | 5% | 0.181 | 0.118 | 0.119 | 0.126 | - | 0.191 | 0.137 | 0.147 | 0.184 | - | 0.207 |  | 0.161 |  | 0.236 |  | 0.296 |  | - |
|  | 50% | 0.311 | 0.250 | 0.311 | 0.486 | - | 0.406 | 0.357 | 0.537 | 0.824 | - | 0.569 |  | 0.638 |  | 1.131 |  | 1.408 |  | - |
|  | 95% | 0.793 | 0.750 | 1.317 | 2.579 | - | 1.507 | 1.537 | 2.735 | 4.724 | - | 2.688 |  | 3.605 |  | 6.044 |  | 7.626 |  | - |
|  | Mean | 0.381 | 0.316 | 0.483 | 0.819 | - | 0.558 | 0.566 | 0.899 | 1.458 | - | 0.876 |  | 1.077 |  | 1.820 |  | 2.259 |  | - |
| RU |  | 0.19 | 0.19 | 0.24 | 0.31 | - | 0.4 | 0.4 | 0.5 | 0.52 | - | 0.61 |  | 0.62 |  | 0.74 |  | 0.67 |  | - |
| 𝜆 |  | 0.08 |  |  |  |  | 0.15 |  |  |  |  | 0.24 |  |  |  |  |  |  |  |  |
| Notes: |  | Source 4, R1-162503, CN delay = 0ms, CQI period = 5ms, CQI delay = 6ms FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 4.28 | 4.02 | 4.21 | 4.25 | 3.71 | 3.21 | 2.99 | 2.11 | 2.50 | 2.31 |  | 2.32 |  | 2.06 |  | 1.78 |  | 1.42 | 1.31 |
|  | 50% | 10.56 | 10.87 | 12.17 | 13.01 | 11.34 | 7.99 | 8.39 | 9.00 | 7.55 | 8.09 |  | 6.60 |  | 5.90 |  | 5.87 |  | 5.19 | 5.12 |
|  | 95% | 16.32 | 22.89 | 27.28 | 28.52 | 27.25 | 15.95 | 20.97 | 25.12 | 22.96 | 24.59 |  | 14.29 |  | 17.61 |  | 20.71 |  | 17.24 | 16.16 |
|  | Mean | 10.81 | 12.02 | 14.04 | 14.56 | 13.45 | 8.80 | 9.56 | 10.50 | 9.30 | 9.73 |  | 7.41 |  | 7.42 |  | 7.82 |  | 6.85 | 6.45 |
| DL: Delay CDF [s] | 5% | 0.98 | 1.04 | 1.00 | 0.99 | 1.13 | 1.31 | 1.40 | 1.98 | 1.68 | 1.81 |  | 1.81 |  | 2.04 |  | 2.36 |  | 2.96 | 3.21 |
|  | 50% | 0.40 | 0.39 | 0.33 | 0.32 | 0.37 | 0.52 | 0.50 | 0.46 | 0.55 | 0.52 |  | 0.64 |  | 0.71 |  | 0.72 |  | 0.81 | 0.82 |
|  | 95% | 0.26 | 0.18 | 0.15 | 0.15 | 0.15 | 0.26 | 0.20 | 0.17 | 0.18 | 0.17 |  | 0.29 |  | 0.24 |  | 0.20 |  | 0.24 | 0.26 |
|  | Mean | 0.46 | 0.46 | 0.41 | 0.40 | 0.46 | 0.59 | 0.60 | 0.65 | 0.70 | 0.70 |  | 0.77 |  | 0.86 |  | 0.91 |  | 1.12 | 1.14 |
| RU |  | 18 | 21 | 22 | 21 | 22 | 39 | 44 | 47 | 50 | 51 |  | 53 |  | 61 |  | 64 |  | 63 | 69 |
| 𝜆 |  | 0.07 |  |  |  |  | 0.14 |  |  |  |  |  | 0.20 |  |  |  |  |  |  |  |
| Notes: |  | Source 7, R1-163403, CN delay = 0ms, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-15%, 7OS-18.0%, 3/4OS-19%, 2OS-21.5%, 1OS-28%; Maximum number of UEs that can be scheduled per sTTI: 14OS-5UEs, 7OS-2UEs, 3/4OS-1UEs, 2OS-1UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 11.63 | 19.32 | 21.72 | 23.55 | 19.60 | - | - | - | - | - | 5.56 | 7.84 | 9.00 | 8.99 | 5.58 |
|  | 50% | 20.83 | 40.40 | 52.73 | 56.80 | 52.19 | - | - | - | - | - | 14.71 | 29.96 | 36.65 | 37.58 | 31.22 |
|  | 95% | 37.74 | 75.47 | 75.12 | 72.35 | 66.04 | - | - | - | - | - | 32.26 | 68.97 | 74.72 | 72.17 | 65.81 |
|  | Mean | 21.89 | 40.96 | 50.63 | 53.37 | 48.55 | - | - | - | - | - | 15.99 | 32.35 | 38.23 | 38.70 | 33.54 |
| DL: Delay CDF [s] | 5% | 0.3430 | 0.2050 | 0.1844 | 0.1694 | 0.2011 | - | - | - | - | - | 0.7150 | 0.5095 | 0.4440 | 0.4317 | 0.7051 |
|  | 50% | 0.1920 | 0.0990 | 0.0758 | 0.0704 | 0.0766 | - | - | - | - | - | 0.2720 | 0.1330 | 0.1093 | 0.1063 | 0.1279 |
|  | 95% | 0.1050 | 0.0530 | 0.0533 | 0.0553 | 0.0606 | - | - | - | - | - | 0.1240 | 0.0580 | 0.0535 | 0.0554 | 0.0608 |
|  | Mean | 0.2058 | 0.1120 | 0.0914 | 0.0862 | 0.0971 | - | - | - | - | - | 0.3327 | 0.1881 | 0.1632 | 0.1631 | 0.2196 |
| RU |  | 0.17 | 0.15 | 0.15 | 0.16 | 0.17 |  | - | - | - | - | 0.50 | 0.43 | 0.42 | 0.44 | 0.52 |
| 𝜆 |  | 0.21 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 0ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 3km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 9.15 | 11.53 | 10.79 | 10.15 | 6.27 | - | - | - | - | - | 2.91 | 3.13 | 3.70 | 2.95 | 1.70 |
|  | 50% | 17.09 | 24.39 | 25.22 | 24.96 | 19.00 | - | - | - | - | - | 10.31 | 15.30 | 15.92 | 14.35 | 10.18 |
|  | 95% | 25.32 | 33.76 | 32.88 | 30.40 | 25.12 | - | - | - | - | - | 22.35 | 29.20 | 30.47 | 30.11 | 24.58 |
|  | Mean | 17.37 | 23.04 | 23.73 | 23.02 | 17.81 | - | - | - | - | - | 11.38 | 15.98 | 16.63 | 15.38 | 11.41 |
| DL: Delay CDF [s] | 5% | 0.4320 | 0.3425 | 0.3592 | 0.3846 | 0.6220 | - | - | - | - | - | 1.3500 | 1.2640 | 1.0677 | 1.3234 | 2.3445 |
|  | 50% | 0.2330 | 0.1635 | 0.1584 | 0.1603 | 0.2105 | - | - | - | - | - | 0.3880 | 0.2610 | 0.2509 | 0.2767 | 0.3929 |
|  | 95% | 0.1540 | 0.1175 | 0.1215 | 0.1316 | 0.1592 | - | - | - | - | - | 0.1780 | 0.1355 | 0.1309 | 0.1324 | 0.1627 |
|  | Mean | 0.2560 | 0.1937 | 0.1919 | 0.1991 | 0.2828 | - | - | - | - | - | 0.5220 | 0.4191 | 0.3800 | 0.4462 | 0.7062 |
| RU |  | 0.16 | 0.15 | 0.16 | 0.17 | 0.24 |  | - | - | - | - | 0.49 | 0.48 | 0.47 | 0.53 | 0.67 |
| 𝜆 |  | 0.21 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 0ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 3km/h, 10M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-21.43%, 7OS-23.14%, 3/4OS-26.57%, 2OS-31.71%, 1OS-42%Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | - | - | - | - | - | 2.08 | 3.70 | 3.91 | 5.87 | 3.83 |
|  | 50% | - | - | - | - | - | - | - | - | - | - | 10.26 | 23.12 | 26.39 | 31.32 | 26.88 |
|  | 95% | - | - | - | - | - | - | - | - | - | - | 25.32 | 55.94 | 66.04 | 71.25 | 64.15 |
|  | Mean | - | - | - | - | - | - | - | - | - | - | 11.62 | 25.37 | 29.56 | 33.89 | 29.76 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | - | - | - | - | - | 1.9060 | 1.0635 | 1.0262 | 0.6747 | 1.0344 |
|  | 50% | - | - | - | - | - | - | - | - | - | - | 0.3890 | 0.1725 | 0.1515 | 0.1270 | 0.1488 |
|  | 95% | - | - | - | - | - | - | - | - | - | - | 0.1580 | 0.0710 | 0.0605 | 0.0561 | 0.0621 |
|  | Mean | - | - | - | - | - | - | - | - | - | - | 0.6057 | 0.3298 | 0.2970 | 0.2159 | 0.2978 |
| RU |  |  |  | - | - | - | - |  | - | - | - | 0.65 | 0.57 | 0.60 | 0.50 | 0.60 |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 0ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 120km/h, 20M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-15.14%, 3/4OS-16.86%, 2OS-19.43%, 1OS-25.43% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | - | - | - | - | - | 1.65 | 1.84 | 1.98 | 2.07 | 1.37 |
|  | 50% | - | - | - | - | - | - | - | - | - | - | 7.23 | 11.49 | 11.80 | 11.68 | 9.00 |
|  | 95% | - | - | - | - | - | - | - | - | - | - | 19.90 | 27.21 | 29.51 | 28.78 | 23.14 |
|  | Mean | - | - | - | - | - | - | - | - | - | - | 8.56 | 12.62 | 13.43 | 13.42 | 10.29 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | - | - | - | - | - | 2.3770 | 2.1490 | 1.9592 | 1.8801 | 2.8921 |
|  | 50% | - | - | - | - | - | - | - | - | - | - | 0.5510 | 0.3480 | 0.3387 | 0.3409 | 0.4440 |
|  | 95% | - | - | - | - | - | - | - | - | - | - | 0.1980 | 0.1465 | 0.1355 | 0.1390 | 0.1709 |
|  | Mean | - | - | - | - | - | - | - | - | - | - | 0.8224 | 0.6353 | 0.5911 | 0.6170 | 0.8235 |
| RU |  |  |  | - | - | - | - |  | - | - | - | 0.61 | 0.64 | 0.63 | 0.61 | 0.74 |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |
| Notes: |  | Source 5, R1-162107, 0ms core network delay, CQI period = 5TTI, CQI delay = 6TTI FTP model 3 is assumed. TCP connection is maintained for 1 second at least, UE speed = 120km/h, 10M bandwidth, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-21.43%, 7OS-23.14%, 3/4OS-26.57%, 2OS-31.71%, 1OS-42%Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 14.129 | 15.231 |  |  |  | 7.0051 | 7.5102 |  |  |  | 2.1339 | 2.2535 |  |  |  |
|  | 50% | 28.362 | 31.344 |  |  |  | 17.067 | 18.135 |  |  |  | 8.7618 | 9.1898 |  |  |  |
|  | 95% | 46.983 | 55.284 |  |  |  | 37.224 | 40.293 |  |  |  | 25.654 | 27.125 |  |  |  |
|  | Mean | 29.155 | 32.872 |  |  |  | 19.114 | 20.511 |  |  |  | 10.644 | 11.247 |  |  |  |
| DL: Delay CDF [s] | 5% | 0.0574 | 0.0566 |  |  |  | 0.0583 | 0.0568 |  |  |  | 0.0861 | 0.0792 |  |  |  |
|  | 50% | 0.1204 | 0.1075 |  |  |  | 0.2039 | 0.1907 |  |  |  | 0.4024 | 0.3725 |  |  |  |
|  | 95% | 0.3972 | 0.3734 |  |  |  | 0.7387 | 0.7040 |  |  |  | 2.1858 | 1.9150 |  |  |  |
|  | Mean | 0.1598 | 0.1459 |  |  |  | 0.2767 | 0.2615 |  |  |  | 0.7379 | 0.6513 |  |  |  |
| RU |  | 12.914 | 11.898 |  |  |  | 28.332 | 29.627 |  |  |  | 52.223 | 49.29 |  |  |  |
| 𝜆 |  | 0.1 |  |  |  |  | 0.15 |  |  |  |  | 0.2 |  |  |  |  |
| Notes: |  | Source 9, R1- 163543, CN delay = 0ms, macro only scenario, simulation time=120sec, TCP connection maintained for the duration of the simulation, TCP-ACK delay=12TTI Control overhead: 1 OFDM symbol per TTI for 0.5ms TTI (~12% of all REs in TTI), 2 OFDM symbols per TTI for 1ms TTI (~12% of all REs in TTI) Number of UEs scheduled per TTI was measured and at most 5 DL UEs were scheduled per TTI for the highest simulated RU (details in Annex D of R1-163543) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 22.315 | 22.818 |  |  |  | 16.667 | 17.142 |  |  |  | 10.357 | 10.716 |  |  |  |
|  | 50% | 50.37 | 53.63 |  |  |  | 45.621 | 47.699 |  |  |  | 40.665 | 42.243 |  |  |  |
|  | 95% | 63.752 | 68.987 |  |  |  | 63.793 | 67.595 |  |  |  | 62.809 | 65.902 |  |  |  |
|  | Mean | 47.401 | 50.561 |  |  |  | 43.314 | 45.422 |  |  |  | 38.609 | 40.215 |  |  |  |
| DL: Delay CDF [s] | 5% | 0.057 | 0.0566 |  |  |  | 0.057 | 0.0566 |  |  |  | 0.0571 | 0.0566 |  |  |  |
|  | 50% | 0.0723 | 0.0665 |  |  |  | 0.0807 | 0.0768 |  |  |  | 0.094 | 0.0911 |  |  |  |
|  | 95% | 0.2183 | 0.2096 |  |  |  | 0.2842 | 0.2764 |  |  |  | 0.4452 | 0.4282 |  |  |  |
|  | Mean | 0.0981 | 0.0932 |  |  |  | 0.1147 | 0.1104 |  |  |  | 0.1568 | 0.1498 |  |  |  |
| RU |  | 16.749 | 16.336 |  |  |  | 30.014 | 29.401 |  |  |  | 49.832 | 48.765 |  |  |  |
| 𝜆 |  | 0.3 |  |  |  |  | 0.45 |  |  |  |  | 0.6 |  |  |  |  |
| Notes: |  | Source 9, R1- 163543, CN delay = 0ms, Small cell scenario 2a, simulation time=120sec, TCP connection maintained for the duration of the simulation, TCP-ACK delay=12TTI Control overhead: 1 OFDM symbol per TTI for 0.5ms TTI (~12% of all REs in TTI), 2 OFDM symbols per TTI for 1ms TTI (~12% of all REs in TTI) Number of UEs scheduled per TTI was measured and at most 5 DL UEs were scheduled per TTI for the highest simulated RU (details in Annex D of R1-163543) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | 6.85 | 7.91 | - | 8.57 | 8.95 | 3.02 | 3.82 | - | 3.88 | 4.65 |
|  | 50% | - | - | - | - | - | 21.5 | 29.3 | - | 34.7 | 37.2 | 16.5 | 21.3 | - | 23.5 | 26.5 |
|  | 95% | - | - | - | - | - | 33.6 | 58.4 | - | 86.9 | 95.4 | 31.5 | 50.6 | - | 68.6 | 77.5 |
|  | Mean | - | - | - | - | - | 16.11 | 20.8 | - | 23.6 | 25.2 | 9.70 | 12.4 | - | 13.1 | 15.3 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | 0.119 | 0.069 | - | 0.046 | 0.042 | 0.127 | 0.079 | - | 0.058 | 0.052 |
|  | 50% | - | - | - | - | - | 0.186 | 0.137 | - | 0.115 | 0.108 | 0.243 | 0.188 | - | 0.170 | 0.151 |
|  | 95% | - | - | - | - | - | 0.584 | 0.506 | - | 0.467 | 0.447 | 1.32 | 1.05 | - | 1.03 | 0.860 |
|  | Mean | - | - | - | - | - | 0.248 | 0.193 | - | 0.170 | 0.159 | 0.412 | 0.324 | - | 0.305 | 0.262 |
| RU |  | - | - | - | - | - | 0.44 | 0.40 | - | 0.39 | 0.38 | 0.77 | 0.72 | - | 0.71 | 0.67 |
| 𝜆 |  |  |  |  |  |  | 1/3.0 |  |  |  |  | 1/2.0 |  |  |  |  |
| Notes: |  | Source 10, R1-163065, CN delay = 0ms, TCP slow start apply to each new traffic burst, 5-ms CSI period, 6-sTTI CSI delay Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-0.6%, 7OS-1.2%, 2OS-1.2%, 1OS-1.2% Maximum number of UEs that can be scheduled per sTTI: 8UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | 6.85 | 9.38 | - | 9.85 | 10.2 | 3.02 | 4.92 | - | 5.47 | 5.56 |
|  | 50% | - | - | - | - | - | 21.5 | 33.3 | - | 40.0 | 42.1 | 16.5 | 24.3 | - | 29.0 | 29.7 |
|  | 95% | - | - | - | - | - | 33.6 | 64.0 | - | 95.5 | 95.4 | 31.5 | 57.1 | - | 81.6 | 86.9 |
|  | Mean | - | - | - | - | - | 16.11 | 23.9 | - | 27.2 | 28.3 | 9.70 | 14.9 | - | 17.2 | 17.6 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | 0.119 | 0.063 | - | 0.042 | 0.042 | 0.127 | 0.070 | - | 0.049 | 0.046 |
|  | 50% | - | - | - | - | - | 0.186 | 0.120 | - | 0.100 | 0.095 | 0.243 | 0.164 | - | 0.138 | 0.135 |
|  | 95% | - | - | - | - | - | 0.584 | 0.427 | - | 0.406 | 0.392 | 1.32 | 0.814 | - | 0.731 | 0.719 |
|  | Mean | - | - | - | - | - | 0.248 | 0.168 | - | 0.147 | 0.141 | 0.412 | 0.269 | - | 0.232 | 0.228 |
| RU |  | - | - | - | - | - | 0.44 | 0.37 | - | 0.36 | 0.35 | 0.77 | 0.65 | - | 0.63 | 0.63 |
| 𝜆 |  |  |  |  |  |  | 1/3.0 |  |  |  |  | 1/2.0 |  |  |  |  |
| Notes: |  | Source 10, R1-163072, CN delay = 0ms, TCP slow start apply to each new traffic burst, 5-ms CSI period, 6-sTTI CSI delay, fast CQI per sTTI Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-0.6%, 7OS-1.2%, 2OS-1.2%, 1OS-1.2% Maximum number of UEs that can be scheduled per sTTI: 8UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL: UPT CDF [Mbps] | 5% | 10.272 | 11.282 (+0.098) | N/A | 10.833 (+0.055) | N/A | 5.1067 | 5.366 (+0.051) | N/A | 4.889 (-0.043) | N/A |  | 3.73 | 4.033 (+0.081) | N/A | 3.807 (+0.021) | N/A |
|  | 50% | 30.706 | 39.64 (+0.291) | N/A | 42.39 (+0.380) | N/A | 23.6 | 27.6333 (+0.171) | N/A | 24.975 (+0.058) | N/A |  | 19.84 | 21.1 (+0.064) | N/A | 17.058 (-0.140) | N/A |
|  | 95% | 61.096 | 91.085 (+0.491) | N/A | 91.947 (+0.505) | N/A | 54.66 | 76.67 (+0.403) | N/A | 78.48 (+0.436) | N/A |  | 50.89 | 68.7 (0.350) | N/A | 68.683 (+0.350) | N/A |
|  | Mean | 32.458 | 43.1058 (+0.328) | N/A | 45.336 (+0.397) | N/A | 25.133 | 32.1955 (+0.281) | N/A | 30.978 (+0.233) | N/A |  | 21.8795 | 27.3994 (+0.252) | N/A | 24.305 (+0.111) | N/A |
| DL: Delay CDF [ms] | 5% | 65.504 | 43.915 (-0.330) | N/A | 43.516 (-0.336) | N/A | 73.24 | 52.23 (-0.287) | N/A | 51.01 (-0.304) | N/A |  | 78.7067 | 58.267 (-0.260) | N/A | 58.218 (-0.260) | N/A |
|  | 50% | 130.15 | 100.867 (-0.225) | N/A | 94.325 (-0.275) | N/A | 169.35 | 144.75 (-0.145) | N/A | 160.05 (-0.055) | N/A |  | 201.8 | 189.65 (-0.060) | N/A | 234.85 (+0.164) | N/A |
|  | 95% | 390.285 | 354.07 (-0.093) | N/A | 368.835 (-0.055) | N/A | 781.66 | 745.57 (-0.046) | N/A | 817.48 (+0.046) | N/A |  | 1065.78 | 995.75 (-0.066) | N/A | 1050.76 (-0.014) | N/A |
|  | Mean | 164.022 | 138.937 (-0.153) | N/A | 135.103 (-0.176) | N/A | 267.545 | 237.741 (-0.111) | N/A | 259.564 (-0.030) | N/A |  | 332.149 | 309.586 (-0.068) | N/A | 341.95 (+0.030) | N/A |
| RU |  | 25.670 | 23.859 (-0.071) | N/A | 24.379 (-0.050) | N/A | 55.681 | 52.256 (-0.062) | N/A | 54.832 (-0.015) | N/A |  | 68.921 | 67.824 (-0.016) | N/A | 70.265 (+0.020) | N/A |
| 𝜆 |  | 0.25 |  |  |  |  | 0.5 |  |  |  |  |  | 0.75 |  |  |  |  |
| Notes: |  | Source 11, R1-163265, CN Delay=0ms; max scheduled users=10; Total overhead is 23.1% with 7s-TTI, 31.7% with 2s-TTI(CCE number=4); For legacy TTI length, total overhead is always 21.4% regardless CCE number;CCE number=4; for legacy TTI length, control region is always 21.4% regardless CCE number, CSI feedback period 5ms, SR to grant 8 (s)TTI, HARQ RTT 8 s(TTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 10.272 | 11.1033 (+0.081) | N/A | 7.441 (-0.276) | N/A | 5.107 | 5.344 (+0.047) | N/A | 3.305 (-0.353) | N/A |  | 3.73 | 3.935 (+0.055) | N/A | 3.454 (-0.074) | N/A |
|  | 50% | 30.706 | 39.28 (+0.279) | N/A | 31.6083 (+0.029) | N/A | 23.6 | 27.07 (+0.147) | N/A | 16.667 (-0.294) | N/A |  | 19.84 | 20.55 (+0.036) | N/A | 12.675 (-0.361) | N/A |
|  | 95% | 61.096 | 90.99 (+0.489) | N/A | 78.1975 (+0.280) | N/A | 54.66 | 77.618 (+0.420) | N/A | 63.735 (+0.166) | N/A |  | 50.89 | 67.5533 (+0.327) | N/A | 56.065 (+0.102) | N/A |
|  | Mean | 32.458 | 42.7291 (0.316) | N/A | 35.6 (+0.097) | N/A | 25.1326 | 31.81 (+0.266) | N/A | 22.274 (-0.114) | N/A |  | 21.8795 | 26.9341 (+0.231) | N/A | 18.5508 (-0.152) | N/A |
| DL: Delay CDF [ms] | 5% | 65.504 | 44.005 (-0.328) | N/A | 51.2013 (-0.218) | N/A | 73.24 | 51.483 (-0.297) | N/A | 62.765 (-0.143) | N/A |  | 78.7067 | 59.2467 (-0.247) | N/A | 71.435 (-0.092) | N/A |
|  | 50% | 130.15 | 101.767 (-0.218) | N/A | 126.55 (-0.028) | N/A | 169.35 | 147.85 (-0.127) | N/A | 240.65 (+0.421) | N/A |  | 201.8 | 194.6 (-0.036) | N/A | 315.35 (+0.563) | N/A |
|  | 95% | 390.285 | 359.39 (-0.079) | N/A | 538.095 (+0.379) | N/A | 781.66 | 747.735 (-0.043) | N/A | 1210.87 (+0.549) | N/A |  | 1065.78 | 1018.26 (-0.045) | N/A | 1158.46 (+0.087) | N/A |
|  | Mean | 164.022 | 138.381 (-0.156) | N/A | 186.558 (+0.137) | N/A | 267.545 | 239.901 (-0.103) | N/A | 381.24 (+0.425) | N/A |  | 332.149 | 315.572 (-0.050) | N/A | 431.275 (+0.298) | N/A |
| RU |  | 25.670 | 24.126 (-0.060) | N/A | 30.873 (+0.203) | N/A | 55.681 | 52.787 (-0.052) | N/A | 65.005 (+0.167) | N/A |  | 68.921 | 68.293 (-0.009) | N/A | 75.697 (+0.098) | N/A |
| 𝜆 |  | 0.25 |  |  |  |  | 0.5 |  |  |  |  |  | 0.75 |  |  |  |  |
| Notes: |  | Source 11, R1-163265, CN Delay=0ms; max scheduled users=10; Total overhead is 24.9% with 7s-TTI, 42.0% with 2s-TTI(CCE number=8); For legacy TTI length, total overhead is always 21.4% regardless CCE number;CCE number=8; for legacy TTI length, control region is always 21.4% regardless CCE number, CSI feedback period 5ms, SR to grant 8 (s)TTI, HARQ RTT 8 s(TTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS |  | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 10.272 | 10.241 (-0.003) | N/A | 2.2867 (-0.777) | N/A | 5.107 | 4.738 (-0.072) | N/A | 2.101 (-0.589) |  | N/A | 3.73 | 3.746 (+0.004) | N/A | 2.119 (-0.432) | N/A |
|  | 50% | 30.706 | 36.988 (+0.205) | N/A | 10.29 (-0.665) | N/A | 23.6 | 25.2 (+0.068) | N/A | 7.283 (-0.691) |  | N/A | 19.84 | 18.96 (-0.044) | N/A | 5.462 (-0.725) | N/A |
|  | 95% | 61.096 | 89.398 (0.463) | N/A | 45.98 (-0.247) | N/A | 54.66 | 71.25 (+0.304) | N/A | 34.895 (-0.362) |  | N/A | 50.89 | 65.885 (+0.295) | N/A | 32.31 (-0.365) | N/A |
|  | Mean | 32.458 | 40.860 (+0.259) | N/A | 14.5858 (-0.551) | N/A | 25.133 | 29.991 (+0.193) | N/A | 10.767 (-0.572) |  | N/A | 21.8795 | 25.621 (+0.171) | N/A | 9.398 (-0.570) | N/A |
| DL: Delay CDF [ms] | 5% | 65.504 | 44.703 (-0.318) | N/A | 87.04 (+0.329) | N/A | 73.24 | 56.15 (-0.233) | N/A | 114.605 (+0.565) |  | N/A | 78.7067 | 60.715 (-0.229) | N/A | 123.79 (+0.573) | N/A |
|  | 50% | 130.15 | 108.15 (-0.169) | N/A | 388.9 (+1.988) | N/A | 169.35 | 158.767 (-0.062) | N/A | 548.050 (+2.236) |  | N/A | 201.8 | 211.0 (+0.046) | N/A | 733.65 (+2.636) | N/A |
|  | 95% | 390.285 | 389.895 (-0.001) | N/A | 1743.26 (+3.467) | N/A | 781.66 | 836.95 (+0.071) | N/A | 1890.800 (+1.419) |  | N/A | 1065.78 | 1073.77 (+0.007) | N/A | 1890.41 (+0.774) | N/A |
|  | Mean | 164.022 | 147.576 (-0.100) | N/A | 596.269 (+2.635) | N/A | 267.545 | 264.553 (-0.011) | N/A | 749.923 (+1.803) |  | N/A | 332.149 | 334.699 (+0.008) | N/A | 820.819 (+1.471) | N/A |
| RU |  | 25.670 | 25.495 (-0.007) | N/A | 54.994 (+1.142) | N/A | 55.681 | 55.106 (-0.010) | N/A | 75.869 (+0.363) |  | N/A | 68.921 | 69.675 (+0.011) | N/A | 84.132 (+0.221) | N/A |
| 𝜆 |  | 0.25 |  |  |  |  | 0.5 |  |  |  |  |  | 0.75 |  |  |  |  |
| Notes: |  | Source 11, R1-163265, CN Delay=0ms; max scheduled users=10; Total overhead is 28.3% with 7s-TTI, 62.6% with 2s-TTI(CCE number=16); For legacy TTI length, total overhead is always 21.4% regardless CCE number; CSI feedback period 5ms, SR to grant 8 (s)TTI, HARQ RTT 8 s(TTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL: UPT CDF [Mbps] | 5% | 23.7288 | 30.7905 | 32.7829 | - | - | 18.0848 | 19.7121 | 19.5472 | - | - | 3.8573 | 4.4975 | 4.6217 | - | - |
|  | 50% | 28.3768 | 42.2248 | 47.3298 | - | - | 25.2785 | 32.5592 | 34.5528 | - | - | 16.4967 | 18.9736 | 19.0183 | - | - |
|  | 95% | 31.7293 | 46.4316 | 56.7143 | - | - | 31.2697 | 45.9729 | 52.2405 | - | - | 30.5819 | 39.6148 | 43.2715 | - | - |
|  | Mean | 28.1258 | 40.1698 | 47.0089 | - | - | 25.3755 | 32.2747 | 35.1944 | - | - | 17.1468 | 20.7985 | 21.6559 | - | - |
| DL: Delay CDF [s] | 5% | 0.1333 | 0.0874 | 0.0723 | - | - | 0.1348 | 0.0914 | 0.0799 | - | - | 0.1401 | 0.1031 | 0.0973 | - | - |
|  | 50% | 0.1542 | 0.1061 | 0.0918 | - | - | 0.1787 | 0.1466 | 0.1392 | - | - | 0.3340 | 0.3208 | 0.3524 | - | - |
|  | 95% | 0.1849 | 0.1367 | 0.1327 | - | - | 0.3166 | 0.2950 | 0.3255 | - | - | 1.4486 | 1.5412 | 1.4870 | - | - |
|  | Mean | 0.1562 | 0.1110 | 0.0959 | - | - | 0.1966 | 0.1657 | 0.1594 | - | - | 0.5255 | 0.5131 | 0.5240 | - | - |
| RU |  | 19.9007 | 20.0035 | 20.3360 | - | - | 41.5001 | 41.5959 | 42.0710 | - | - | 62.4866 | 62.8597 | 63.4842 | - | - |
| 𝜆 |  | 1 |  |  |  |  | 2 |  |  |  |  | 3 |  |  |  |  |
| Notes: |  | Source 12, R1-163798, CN delay=0ms, UL access delay = 13TTI, FTP traffic model 3 Total control overhead per subframe compared to all available REs in the sTTIs: 14 OS:15%, 7 OS: 17%, 3/4 OS: 19% Maximum number of UEs that can be scheduled per sTTI: not modelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 4.32 | 5.53 |  |  |  | 2.57 | 3.03 |  |  |  | 1.40 | 1.50 |  |  |  |
|  | 50% | 13.59 | 17.34 |  |  |  | 9.35 | 11.45 |  |  |  | 6.30 | 6.98 |  |  |  |
|  | 95% | 24.52 | 35.07 |  |  |  | 24.16 | 30.30 |  |  |  | 20.92 | 24.08 |  |  |  |
|  | Mean | 14.32 | 18.50 |  |  |  | 11.24 | 13.53 |  |  |  | 8.26 | 9.19 |  |  |  |
| DL: Delay CDF [s] | 5% | 0.163 | 0.114 |  |  |  | 0.166 | 0.132 |  |  |  | 0.191 | 0.166 |  |  |  |
|  | 50% | 0.294 | 0.230 |  |  |  | 0.422 | 0.351 |  |  |  | 0.632 | 0.573 |  |  |  |
|  | 95% | 0.925 | 0.707 |  |  |  | 1.559 | 1.358 |  |  |  | 2.856 | 2.872 |  |  |  |
|  | Mean | 0.384 | 0.290 |  |  |  | 0.588 | 0.514 |  |  |  | 0.956 | 0.899 |  |  |  |
| RU |  | 0.21 | 0.17 |  |  |  | 0.44 | 0.38 |  |  |  | 0.69 | 0.61 |  |  |  |
| 𝜆 |  | 0.08 |  |  |  |  | 0.15 |  |  |  |  | 0.24 |  |  |  |  |
| Notes: |  | Source 4, R1-165429, FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Assumptions on processing time and mas TBS scaling for 14OS follow that of 7OS, CN delay = 0ms, CQI period = 5TTI, CQI delay = 6TTI |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.2.1-4: Results with 1MB file transfer, 0ms core network delay

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | 4.99 | 5.08 | 5.10 | 5.49 | 4.15 | 3.03 | 3.34 | 2.66 | 2.47 | 2.16 | 2.75 | 2.20 | 1.80 | 1.55 | 1.67 |
|  | 50% | 11.71 | 12.75 | 13.19 | 13.80 | 12.20 | 8.77 | 9.22 | 9.97 | 8.94 | 8.21 | 7.21 | 6.94 | 6.71 | 5.51 | 5.11 |
|  | 95% | 17.62 | 22.92 | 28.13 | 28.34 | 26.51 | 17.02 | 21.64 | 23.86 | 28.08 | 25.06 | 16.46 | 19.66 | 22.47 | 20.95 | 20.45 |
|  | Mean | 11.64 | 13.56 | 14.39 | 15.07 | 13.62 | 9.37 | 10.40 | 11.29 | 10.91 | 10.45 | 7.95 | 8.34 | 8.99 | 7.83 | 7.40 |
| DL: Delay CDF [s] | 5% | 1.68 | 1.65 | 1.64 | 1.53 | 2.02 | 2.77 | 2.52 | 3.16 | 3.40 | 3.75 | 3.05 | 3.82 | 4.67 | 5.43 | 5.01 |
|  | 50% | 0.72 | 0.66 | 0.64 | 0.61 | 0.69 | 0.96 | 0.91 | 0.83 | 0.94 | 1.00 | 1.16 | 1.21 | 1.21 | 1.52 | 1.63 |
|  | 95% | 0.48 | 0.37 | 0.30 | 0.30 | 0.32 | 0.49 | 0.39 | 0.35 | 0.30 | 0.33 | 0.51 | 0.43 | 0.37 | 0.40 | 0.41 |
|  | Mean | 0.84 | 0.78 | 0.79 | 0.74 | 0.85 | 1.16 | 1.12 | 1.18 | 1.27 | 1.34 | 1.41 | 1.50 | 1.54 | 1.92 | 1.95 |
| RU |  | 19 | 20 | 23 | 22 | 22 | 40 | 41 | 44 | 44 | 51 | 55 | 59 | 60 | 63 | 63 |
| 𝜆 |  | 0.03 |  |  |  |  | 0.07 |  |  |  |  | 0.10 |  |  |  |  |
| Notes: |  | Source 7, R1-163403, CN delay = 0ms, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]:14OS-15%, 7OS-18.0%, 3/4OS-19%, 2OS-21.5%, 1OS-28%; Maximum number of UEs that can be scheduled per sTTI: 14OS-5UEs, 7OS-2UEs, 3/4OS-1UEs, 2OS-1UEs, 1OS-1UE |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | 7.50 | 8.62 | - | 8.81 | 9.52 | 4.32 | 4.73 | - | b | 5.38 |
|  | 50% | - | - | - | - | - | 26.4 | 33.4 | - | 36.1 | 39.1 | 20.1 | 24.0 | - | 24.5 | 27.6 |
|  | 95% | - | - | - | - | - | 42.8 | 71.4 | - | 91.4 | 99.7 | 40.0 | 60.4 | - | 72.7 | 80.3 |
|  | Mean | - | - | - | - | - | 19.0 | 23.2 | - | 24.7 | 26.6 | 12.9 | 15.0 | - | 15.3 | 17.0 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | 0.187 | 0.112 | - | 0.088 | 0.080 | 0.200 | 0.133 | - | 0.110 | 0.100 |
|  | 50% | - | - | - | - | - | 0.303 | 0.240 | - | 0.221 | 0.205 | 0.399 | 0.334 | - | 0.327 | 0.290 |
|  | 95% | - | - | - | - | - | 1.07 | 0.929 | - | 0.909 | 0.840 | 1.85 | 1.69 | - | 1.68 | 1.49 |
|  | Mean | - | - | - | - | - | 0.422 | 0.335 | - | 0.324 | 0.300 | 0.619 | 0.534 | - | 0.524 | 0.470 |
| RU |  | - | - | - | - | - | 0.42 | 0.39 | - | 0.39 | 0.37 | 0.73 | 0.69 | - | 0.69 | 0.65 |
| 𝜆 |  |  |  |  |  |  | 1/6.0 |  |  |  |  | 1/4.0 |  |  |  |  |
| Notes: |  | Source 10, R1-163065, CN delay = 0ms, TCP slow start apply to each new traffic burst, full CSI in A/N Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-0.6%, 7OS-1.2%, 2OS-1.2%, 1OS-1.2% Maximum number of UEs that can be scheduled per sTTI: 8UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | 7.73 | 8.26 | - | 7.82 | 8.82 | 2.43 | 2.90 | - | 3.05 | 3.45 |
|  | 50% | - | - | - | - | - | 34.3 | 36.7 | - | 36.1 | 38.3 | 21.9 | 23.7 | - | 24.0 | 25.8 |
|  | 95% | - | - | - | - | - | 89.4 | 95.1 | - | 94.4 | 101 | 69.4 | 74.2 | - | 75.2 | 80.3 |
|  | Mean | - | - | - | - | - | 22.3 | 23.8 | - | 23.0 | 25.3 | 5.11 | 6.77 | - | 6.44 | 8.27 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | 0.089 | 0.084 | - | 0.085 | 0.080 | 0.115 | 0.108 | - | 0.106 | 99.6 |
|  | 50% | - | - | - | - | - | 0.233 | 0.218 | - | 0.221 | 0.206 | 0.365 | 0.338 | - | 0.334 | 0.310 |
|  | 95% | - | - | - | - | - | 1.04 | 0.969 | - | 1.02 | 0.907 | 3.29 | 2.75 | - | 2.62 | 2.32 |
|  | Mean | - | - | - | - | - | 0.359 | 0.336 | - | 0.349 | 0.316 | 1.56 | 1.18 | - | 1.24 | 0.967 |
| RU |  | - | - | - | - | - | 0.40 | 0.38 | - | 0.39 | 0.37 | 0.72 | 0.69 | - | 0.69 | 0.67 |
| 𝜆 |  |  |  |  |  |  | 1/6.0 |  |  |  |  | 1/4.0 |  |  |  |  |
| Notes: |  | Source 10, R1-163065, CN delay = 0ms, UDP, 5-ms CSI period, 6-sTTI CSI delay Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-0.6%, 7OS-1.2%, 2OS-1.2%, 1OS-1.2% Maximum number of UEs that can be scheduled per sTTI: 8UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | 7.50 | 9.96 | - | 10.3 | 10.8 | 4.32 | 5.77 | - | 6.08 | 6.10 |
|  | 50% | - | - | - | - | - | 26.4 | 38.0 | - | 42.6 | 44.0 | 20.1 | 27.4 | - | 31.0 | 31.0 |
|  | 95% | - | - | - | - | - | 42.8 | 80.0 | - | 102 | 105 | 40.0 | 69.9 | - | 88.2 | 89.6 |
|  | Mean | - | - | - | - | - | 19.0 | 26.6 | - | 28.9 | 29.7 | 12.9 | 17.5 | - | 19.3 | 19.2 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | 0.187 | 0.100 | - | 0.079 | 0.076 | 0.200 | 0.115 | - | 0.091 | 0.089 |
|  | 50% | - | - | - | - | - | 0.303 | 0.211 | - | 0.188 | 0.182 | 0.399 | 0.292 | - | 0.258 | 0.259 |
|  | 95% | - | - | - | - | - | 1.07 | 0.804 | - | 0.777 | 0.741 | 1.85 | 1.39 | - | 1.32 | 1.31 |
|  | Mean | - | - | - | - | - | 0.422 | 0.300 | - | 0.276 | 0.269 | 0.619 | 0.457 | - | 0.415 | 0.418 |
| RU |  | - | - | - | - | - | 0.42 | 0.36 | - | 0.35 | 0.34 | 0.73 | 0.63 | - | 0.60 | 0.61 |
| 𝜆 |  |  |  |  |  |  | 1/6.0 |  |  |  |  | 1/4.0 |  |  |  |  |
| Notes: |  | Source 10, R1-163072, CN delay = 0ms, TCP slow start apply to each new traffic burst, 5-ms CSI period, 6-sTTI CSI delay, fast CQI per sTTI Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-0.6%, 7OS-1.2%, 2OS-1.2%, 1OS-1.2% Maximum number of UEs that can be scheduled per sTTI: 8UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | 7.73 | 9.54 | - | 9.70 | 9.87 | 2.43 | 4.29 | - | 4.48 | 4.42 |
|  | 50% | - | - | - | - | - | 34.3 | 41.4 | - | 43.1 | 43.0 | 21.9 | 28.1 | - | 29.2 | 29.4 |
|  | 95% | - | - | - | - | - | 89.4 | 101 | - | 107 | 108 | 69.4 | 85.2 | - | 89.0 | 89.2 |
|  | Mean | - | - | - | - | - | 22.3 | 27.2 | - | 27.9 | 28.6 | 5.11 | 12.4 | - | 12.5 | 12.5 |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | 0.089 | 0.079 | - | 0.075 | 0.074 | 0.115 | 0.094 | - | 0.090 | 0.090 |
|  | 50% | - | - | - | - | - | 0.233 | 0.193 | - | 0.186 | 0.186 | 0.365 | 0.285 | - | 0.274 | 0.272 |
|  | 95% | - | - | - | - | - | 1.04 | 0.839 | - | 0.824 | 0.810 | 3.29 | 1.86 | - | 1.79 | 1.80 |
|  | Mean | - | - | - | - | - | 0.359 | 0.294 | - | 0.288 | 0.286 | 1.56 | 0.648 | - | 0.640 | 0.640 |
| RU |  | - | - | - | - | - | 0.40 | 0.35 | - | 0.35 | 0.34 | 0.72 | 0.62 | - | 0.62 | 0.62 |
| 𝜆 |  |  |  |  |  |  | 1/6.0 |  |  |  |  | 1/4.0 |  |  |  |  |
| Notes: |  | Source 10, R1-163072, CN delay = 0ms, UDP, 5-ms CSI period, 6-sTTI CSI delay, fast CQI per sTTI Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-0.6%, 7OS-1.2%, 2OS-1.2%, 1OS-1.2% Maximum number of UEs that can be scheduled per sTTI: 8UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL: UPT CDF [Mbps] | 5% | 20.6713 | 29.5131 | 33.2234 | - | - | 12.0626 | 14.0944 | 14.7406 | - | - | 4.5606 | 5.8520 | 5.2458 | - | - |
|  | 50% | 30.9070 | 42.3016 | 48.5211 | - | - | 27.8022 | 35.1595 | 37.5010 | - | - | 19.4586 | 20.4785 | 21.1384 | - | - |
|  | 95% | 36.4907 | 52.3080 | 61.1163 | - | - | 36.9131 | 52.8467 | 57.3839 | - | - | 36.0908 | 47.6759 | 49.0287 | - | - |
|  | Mean | 30.0849 | 42.4208 | 48.2050 | - | - | 27.0395 | 34.7495 | 36.9441 | - | - | 20.0266 | 23.8729 | 24.2925 | - | - |
| DL: Delay CDF [s] | 5% | 0.2306 | 0.1596 | 0.1321 | - | - | 0.2373 | 0.1596 | 0.1417 | - | - | 0.2338 | 0.1689 | 0.1571 | - | - |
|  | 50% | 0.2768 | 0.1997 | 0.1744 | - | - | 0.3289 | 0.2657 | 0.2669 | - | - | 0.5111 | 0.4897 | 0.5150 | - | - |
|  | 95% | 0.3690 | 0.2995 | 0.2612 | - | - | 0.8191 | 0.6874 | 0.6937 | - | - | 1.9169 | 1.7183 | 1.8455 | - | - |
|  | Mean | 0.2900 | 0.2097 | 0.1845 | - | - | 0.3740 | 0.3131 | 0.3091 | - | - | 0.7076 | 0.6714 | 0.7056 | - | - |
| RU |  | 20.6873 | 20.7612 | 21.0177 | - | - | 42.2087 | 42.2224 | 42.7088 | - | - | 61.1982 | 61.6394 | 62.5134 | - | - |
| 𝜆 |  | 0.5 |  |  |  |  | 1.0 |  |  |  |  | 1.5 |  |  |  |  |
| Notes: |  | Source 12, R1-163798, CN delay=0ms, UL access delay = 13TTI, FTP traffic model 3 Total control overhead per subframe compared to all available REs in the sTTIs: 14 OS:15%, 7 OS: 17%, 3/4 OS: 19% Maximum number of UEs that can be scheduled per sTTI: not modelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

### B.2.2 Results for FS2

Table B.2.2-1: Results with 100kbit file transfer

| Reported parameters |  |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  | 5% | 1.234 | 1.549 |  | 1.527 |  |  |  |  |  |  | 0.485 | 0.426 |  | 0.391 |  |
|  |  | 50% | 1.999 | 3.221 |  | 3.758 |  |  |  |  |  |  | 1.250 | 1.398 |  | 1.221 |  |
|  |  | 95% | 2.379 | 4.867 |  | 6.128 |  |  |  |  |  |  | 2.220 | 3.839 |  | 4.480 |  |
|  |  | Mean | 1.916 | 3.285 |  | 3.760 |  |  |  |  |  |  | 1.282 | 1.687 |  | 1.625 |  |
| DL: Delay CDF [s] |  | 5% | 0.042 | 0.021 |  | 0.016 |  |  |  |  |  |  | 0.045 | 0.026 |  | 0.022 |  |
|  |  | 50% | 0.050 | 0.031 |  | 0.027 |  |  |  |  |  |  | 0.080 | 0.072 |  | 0.082 |  |
|  |  | 95% | 0.080 | 0.063 |  | 0.065 |  |  |  |  |  |  | 0.206 | 0.235 |  | 0.256 |  |
|  |  | Mean | 0.054 | 0.035 |  | 0.032 |  |  |  |  |  |  | 0.097 | 0.094 |  | 0.104 |  |
| RU |  |  | 16.53% | 14.49% |  | 11.56% |  |  |  |  |  |  | 92.49% | 80.87% |  | 65.65% |  |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  | Source 3, R1-164237, SIB-1 TDD UL/DL configuration 1, for shoort TTI special subframes #1/6 are changed to new special subframe type, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-28%, 2OS-29.71% Total Reference signal overhead (CRS + DMRS) per subframe [%]: 14OS-16.67%, 7OS-15.48%, 2OS-23.81% Maximum number of UEs that can be scheduled per sTTI: Low RU-4UEs, Medium RU-5UEs, High RU-9UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  |  | 1.549 |  | 1.723 |  |  |  |  |  |  |  | 0.424 |  |  | 0.396 |
|  | 50% |  |  | 3.220 |  | 4.680 |  |  |  |  |  |  |  | 1.398 |  |  | 1.308 |
|  | 95% |  |  | 4.867 |  | 9.426 |  |  |  |  |  |  |  | 3.838 |  |  | 5.634 |
|  | Mean |  |  | 3.254 |  | 4.976 |  |  |  |  |  |  |  | 1.680 |  |  | 1.894 |
| DL: Delay CDF [s] | 5% |  |  | 0.021 |  | 0.011 |  |  |  |  |  |  |  | 0.026 |  |  | 0.018 |
|  | 50% |  |  | 0.033 |  | 0.021 |  |  |  |  |  |  |  | 0.072 |  |  | 0.077 |
|  | 95% |  |  | 0.071 |  | 0.057 |  |  |  |  |  |  |  | 0.236 |  |  | 0.253 |
|  | Mean |  |  | 0.038 |  | 0.026 |  |  |  |  |  |  |  | 0.094 |  |  | 0.099 |
| RU |  |  |  | 14.54% |  | 12.27% |  |  |  |  |  |  |  | 80.83% |  |  | 70.01% |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  | Source 3, R1-164237, SIB-1 TDD UL/DL configuration 0, for short TTI UL subframe #3/4/7/8 and special subframe #1/6 are changed to new special subframe type, macro scenario, Total control overhead per subframe compared to all available REs in the sTTIs [%]: 14OS-14.29%, 7OS-28%, 2OS-29.71% Total Reference signal overhead (CRS + DMRS) per subframe [%]: 14OS-16.67%, 7OS-15.48%, 2OS-23.81% Maximum number of UEs that can be scheduled per sTTI: Low RU-4UEs, Medium RU-5UEs, High RU-9UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | 1.51 | 2.70 | 2.46 | 0.95 |  | 1.38 | 2.25 | 1.77 | 0.45 |  | 1.21 | 1.79 | 0.87 | 0.12 |  |
|  | 50% |  | 1.63 | 3.21 | 4.39 | 5.69 |  | 1.62 | 3.15 | 4.24 | 5.25 |  | 1.61 | 3.02 | 3.97 | 5.28 |  |
|  | 95% |  | 1.65 | 3.27 | 4.56 | 7.25 |  | 1.65 | 3.26 | 4.57 | 7.27 |  | 1.65 | 3.23 | 4.53 | 7.11 |  |
|  | Mean |  | 1.61 | 3.13 | 4.07 | 4.94 |  | 1.58 | 3.00 | 3.82 | 4.61 |  | 1.54 | 2.82 | 3.51 | 4.55 |  |
| DL: Delay CDF [s] | 5% |  | 0.061 | 0.031 | 0.022 | 0.014 |  | 0.061 | 0.031 | 0.022 | 0.014 |  | 0.061 | 0.031 | 0.022 | 0.014 |  |
|  | 50% |  | 0.062 | 0.031 | 0.023 | 0.018 |  | 0.062 | 0.032 | 0.024 | 0.019 |  | 0.062 | 0.033 | 0.025 | 0.019 |  |
|  | 95% |  | 0.066 | 0.037 | 0.041 | 0.105 |  | 0.073 | 0.044 | 0.057 | 0.217 |  | 0.083 | 0.056 | 0.100 | 0.522 |  |
|  | Mean |  | 0.062 | 0.032 | 0.026 | 0.045 |  | 0.064 | 0.034 | 0.030 | 0.077 |  | 0.070 | 0.044 | 0.097 | 0.223 |  |
| RU |  |  | 0.2 | 0.16 | 0.18 | 0.30 |  | 0.42 | 0.42 | 0.44 | 0.58 |  | 0.60 | 0.73 | 0.74 | 0.74 |  |
| 𝜆 |  |  | 3 |  |  |  |  | 12 |  |  |  |  | 52 |  |  |  |  |
| Notes: |  |  | Source 4, R1-165430, TDD simulations assuming set 1, FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI). TDD UL-DL configuration #1 is used. 14OS assumes Reference Set. RU is counted only on DL subframe. The average TCP ACK delay [ms]: 14OS – 19ms, 7OS – 9.1ms, 3/4OS – 6.91ms, 2OS – 4ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | 1.51 | 2.91 | 2.70 | 1.30 |  | 1.38 | 2.55 | 2.10 | 0.62 |  | 1.21 | 2.30 | 1.47 | 0.24 |  |
|  | 50% |  | 1.63 | 3.30 | 4.22 | 5.55 |  | 1.62 | 3.26 | 4.15 | 5.06 |  | 1.61 | 3.16 | 3.91 | 5.01 |  |
|  | 95% |  | 1.65 | 3.37 | 4.35 | 6.12 |  | 1.65 | 3.36 | 4.36 | 6.15 |  | 1.65 | 3.32 | 4.33 | 6.11 |  |
|  | Mean |  | 1.61 | 3.26 | 3.97 | 4.68 |  | 1.58 | 3.15 | 3.79 | 4.34 |  | 1.54 | 3.02 | 3.56 | 4.25 |  |
| DL: Delay CDF [s] | 5% |  | 0.061 | 0.030 | 0.023 | 0.016 |  | 0.061 | 0.030 | 0.023 | 0.016 |  | 0.061 | 0.030 | 0.023 | 0.016 |  |
|  | 50% |  | 0.062 | 0.030 | 0.024 | 0.018 |  | 0.062 | 0.031 | 0.024 | 0.020 |  | 0.062 | 0.032 | 0.026 | 0.020 |  |
|  | 95% |  | 0.066 | 0.034 | 0.037 | 0.077 |  | 0.073 | 0.039 | 0.048 | 0.161 |  | 0.083 | 0.043 | 0.068 | 0.346 |  |
|  | Mean |  | 0.062 | 0.031 | 0.027 | 0.033 |  | 0.064 | 0.032 | 0.028 | 0.053 |  | 0.070 | 0.034 | 0.037 | 0.122 |  |
| RU |  |  | 0.20 | 0.13 | 0.14 | 0.14 |  | 0.42 | 0.37 | 0.35 | 0.3 |  | 0.60 | 0.65 | 0.60 | 0.41 |  |
| 𝜆 |  |  | 3 |  |  |  |  | 12 |  |  |  |  | 52 |  |  |  |  |
| Notes: |  |  | Source 4, R1-165430, TDD simulations assuming set 2, FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI). TDD UL-DL configuration #1 is used. 14OS assumes Reference Set. RU is counted only on DL subframe. The average TCP ACK delay [ms]: 14OS – 19ms, 7OS – 8.83ms, 3/4OS – 7.09ms, 2OS – 5.09ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | 1.51 | 2.72 | 1.74 | 0.81 |  | 1.38 | 2.22 | 1.28 | 0.37 |  | 1.21 | 2.00 | 0.97 | 0.20 |  |
|  | 50% |  | 1.63 | 3.19 | 3.17 | 3.03 |  | 1.62 | 3.13 | 3.03 | 2.87 |  | 1.61 | 3.01 | 2.86 | 2.83 |  |
|  | 95% |  | 1.65 | 3.28 | 3.31 | 3.31 |  | 1.65 | 3.25 | 3.32 | 3.31 |  | 1.65 | 3.23 | 3.30 | 3.30 |  |
|  | Mean |  | 1.61 | 3.11 | 2.92 | 2.60 |  | 1.58 | 2.98 | 2.74 | 2.42 |  | 1.54 | 2.84 | 2.61 | 2.39 |  |
| DL: Delay CDF [s] | 5% |  | 0.061 | 0.031 | 0.030 | 0.030 |  | 0.061 | 0.031 | 0.030 | 0.030 |  | 0.061 | 0.031 | 0.030 | 0.030 |  |
|  | 50% |  | 0.062 | 0.031 | 0.032 | 0.033 |  | 0.062 | 0.032 | 0.033 | 0.035 |  | 0.062 | 0.033 | 0.035 | 0.035 |  |
|  | 95% |  | 0.066 | 0.037 | 0.058 | 0.124 |  | 0.073 | 0.045 | 0.078 | 0.254 |  | 0.083 | 0.050 | 0.103 | 0.432 |  |
|  | Mean |  | 0.062 | 0.032 | 0.038 | 0.057 |  | 0.064 | 0.034 | 0.041 | 0.100 |  | 0.070 | 0.037 | 0.051 | 0.195 |  |
| RU |  |  | 0.20 | 0.18 | 0.24 | 0.30 |  | 0.42 | 0.42 | 0.48 | 0.56 |  | 0.60 | 0.72 | 0.68 | 0.68 |  |
| 𝜆 |  |  | 3 |  |  |  |  | 12 |  |  |  |  | 52 |  |  |  |  |
| Notes: |  |  | Source 4, R1-165430, TDD simulations assuming set 3, FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI). TDD UL-DL configuration #1 is used. 14OS assumes Reference Set. RU is counted only on DL subframe. The average TCP ACK delay [ms]: 14OS – 19ms, 7OS – 8.7ms, 3/4OS – 7.8ms, 2OS – 7.55ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.2.2-2: Results with 100kB file transfer

| Reported parameters |  |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] |  | 5% | 1.88 | 2.95 | 2.58 | 1.27 |  | 1.16 | 1.76 | 1.36 | 0.43 |  | 0.50 | 0.88 | 0.53 | 0.10 |  |
|  |  | 50% | 4.30 | 7.49 | 7.75 | 6.73 |  | 3.54 | 5.77 | 5.46 | 3.61 |  | 2.84 | 4.13 | 3.43 | 2.13 |  |
|  |  | 95% | 6.09 | 11.65 | 13.59 | 16.61 |  | 6.00 | 11.26 | 13.17 | 14.22 |  | 5.95 | 10.56 | 11.28 | 10.18 |  |
|  |  | Mean | 4.14 | 7.49 | 7.83 | 7.64 |  | 3.56 | 6.07 | 6.13 | 4.77 |  | 3.04 | 4.78 | 4.49 | 3.33 |  |
| DL: Delay CDF [s] |  | 5% | 0.131 | 0.069 | 0.059 | 0.048 |  | 0.133 | 0.071 | 0.061 | 0.056 |  | 0.134 | 0.076 | 0.071 | 0.077 |  |
|  |  | 50% | 0.186 | 0.107 | 0.103 | 0.118 |  | 0.226 | 0.139 | 0.146 | 0.217 |  | 0.280 | 0.193 | 0.230 | 0.342 |  |
|  |  | 95% | 0.426 | 0.272 | 0.310 | 0.633 |  | 0.673 | 0.456 | 0.577 | 1.348 |  | 1.393 | 0.909 | 1.376 | 3.230 |  |
|  |  | Mean | 0.222 | 0.129 | 0.135 | 0.200 |  | 0.315 | 0.187 | 0.216 | 0.395 |  | 0.449 | 0.328 | 0.426 | 0.745 |  |
| RU |  |  | 0.18 | 0.13 | 0.16 | 0.18 |  | 0.42 | 0.33 | 0.40 | 0.46 |  | 0.63 | 0.58 | 0.66 | 0.66 |  |
| 𝜆 |  |  | 0.15 |  |  |  |  | 0.35 |  |  |  |  | 0.55 |  |  |  |  |
| Notes: |  |  | Source 4, R1-165430, TDD simulations assuming set 1, FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI). TDD UL-DL configuration #1 is used. 14OS assumes Reference Set. RU is counted only on DL subframe. The average TCP ACK delay [ms]: 14OS – 19ms, 7OS – 9.1ms, 3/4OS – 6.91ms, 2OS – 4ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | 1.88 | 3.58 | 2.64 | 1.73 |  | 1.16 | 2.43 | 1.57 | 0.76 |  | 0.50 | 1.40 | 0.77 | 0.13 |  |
|  | 50% |  | 4.30 | 8.48 | 7.98 | 8.50 |  | 3.54 | 6.78 | 5.89 | 4.62 |  | 2.84 | 5.25 | 4.13 | 2.75 |  |
|  | 95% |  | 6.09 | 11.86 | 13.72 | 17.65 |  | 6.00 | 11.96 | 12.84 | 14.81 |  | 5.95 | 11.47 | 12.22 | 11.34 |  |
|  | Mean |  | 4.14 | 8.22 | 8.15 | 8.75 |  | 3.56 | 6.97 | 6.53 | 5.88 |  | 3.04 | 5.72 | 5.14 | 4.02 |  |
| DL: Delay CDF [s] | 5% |  | 0.131 | 0.067 | 0.058 | 0.045 |  | 0.133 | 0.067 | 0.062 | 0.054 |  | 0.134 | 0.070 | 0.065 | 0.071 |  |
|  | 50% |  | 0.186 | 0.094 | 0.100 | 0.094 |  | 0.226 | 0.118 | 0.136 | 0.170 |  | 0.280 | 0.152 | 0.193 | 0.278 |  |
|  | 95% |  | 0.426 | 0.224 | 0.303 | 0.461 |  | 0.673 | 0.329 | 0.509 | 0.936 |  | 1.393 | 0.570 | 1.016 | 2.078 |  |
|  | Mean |  | 0.222 | 0.113 | 0.128 | 0.153 |  | 0.315 | 0.149 | 0.194 | 0.295 |  | 0.449 | 0.222 | 0.338 | 0.609 |  |
| RU |  |  | 0.18 | 0.12 | 0.13 | 0.09 |  | 0.42 | 0.28 | 0.35 | 0.23 |  | 0.63 | 0.47 | 0.56 | 0.35 | 0.35 |
| 𝜆 |  |  | 0.15 |  |  |  |  | 0.35 |  |  |  |  | 0.55 |  |  |  |  |
| Notes: |  |  | Source 4, R1-165430, TDD simulations assuming set 2, FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI). TDD UL-DL configuration #1 is used. 14OS assumes Reference Set. RU is counted only on DL subframe. The average TCP ACK delay [ms]: 14OS – 19ms, 7OS – 8.83ms, 3/4OS – 7.09ms, 2OS – 5.09ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  | 1.88 | 2.76 | 2.18 | 1.20 |  | 1.16 | 1.70 | 1.06 | 0.39 |  | 0.50 | 0.86 | 0.27 | 0.00 |  |
|  | 50% |  | 4.30 | 7.50 | 6.67 | 5.72 |  | 3.54 | 5.59 | 4.27 | 3.28 |  | 2.84 | 3.82 | 2.67 | 1.89 |  |
|  | 95% |  | 6.09 | 11.44 | 10.85 | 11.14 |  | 6.00 | 10.95 | 10.42 | 9.85 |  | 5.95 | 10.19 | 9.11 | 7.50 |  |
|  | Mean |  | 4.14 | 7.31 | 6.60 | 5.82 |  | 3.56 | 5.99 | 4.84 | 3.92 |  | 3.04 | 4.54 | 3.54 | 2.67 |  |
| DL: Delay CDF [s] | 5% |  | 0.131 | 0.070 | 0.074 | 0.072 |  | 0.133 | 0.073 | 0.077 | 0.081 |  | 0.134 | 0.079 | 0.087 | 0.105 |  |
|  | 50% |  | 0.186 | 0.106 | 0.120 | 0.140 |  | 0.226 | 0.143 | 0.186 | 0.238 |  | 0.280 | 0.209 | 0.293 | 0.385 |  |
|  | 95% |  | 0.426 | 0.290 | 0.368 | 0.644 |  | 0.673 | 0.470 | 0.756 | 1.331 |  | 1.393 | 0.916 | 1.759 | 2.969 |  |
|  | Mean |  | 0.222 | 0.134 | 0.157 | 0.224 |  | 0.315 | 0.191 | 0.279 | 0.461 |  | 0.449 | 0.334 | 0.534 | 0.754 |  |
| RU |  |  | 0.18 | 0.14 | 0.16 | 0.18 |  | 0.42 | 0.34 | 0.46 | 0.46 |  | 0.63 | 0.58 | 0.7 | 0.64 |  |
| 𝜆 |  |  | 0.15 |  |  |  |  | 0.35 |  |  |  |  | 0.55 |  |  |  |  |
| Notes: |  |  | Source 4, R1-165430, TDD simulations assuming set 3, FTP model 2 is assumed. TCP connection is maintained for each FTP packet. Dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI). TDD UL-DL configuration #1 is used. 14OS assumes Reference Set. RU is counted only on DL subframe. The average TCP ACK delay [ms]: 14OS – 19ms, 7OS – 8.7ms, 3/4OS – 7.8ms, 2OS – 7.55ms |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.2.2-3: Results with 500kB file transfer

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  |  |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 12.31 | - |  | - |  | - |  | - |  | 5.12 |  | - |  | - |  | - |  | - |  | 2.77 |  | - |  |
|  | 50% | - | - | - | 39.49 | - |  | - |  | - |  | - |  | 26.24 |  | - |  | - |  | - |  | - |  | 21.69 |  | - |  |
|  | 95% | - | - | - | 70.53 | - |  | - |  | - |  | - |  | 69.48 |  | - |  | - |  | - |  | - |  | 65.42 |  | - |  |
|  | Mean | - | - | - | 41.09 | - |  | - |  | - |  | - |  | 30.48 |  | - |  | - |  | - |  | - |  | 25.77 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.32 | - |  | - |  | - |  | - |  | 0.78 |  | - |  | - |  | - |  | - |  | 1.44 |  | - |  |
|  | 50% | - | - | - | 0.10 | - |  | - |  | - |  | - |  | 0.25 |  | - |  | - |  | - |  | - |  | 0.18 |  | - |  |
|  | 95% | - | - | - | 0.06 | - |  | - |  | - |  | - |  | 0.06 |  | - |  | - |  | - |  | - |  | 0.06 |  | - |  |
|  | Mean | - | - | - | 0.13 | - |  | - |  | - |  | - |  | 0.15 |  | - |  | - |  | - |  | - |  | 0.42 |  | - |  |
| RU |  | - | - | - | 25.6% | - |  | - |  | - |  | - |  | 50.44% |  | - |  | - |  | - |  | - |  | 65.76% |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162117, CN delay = 0ms, TDD configuration 2, UE speed = 3km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection is maintained for each FTP packet Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-21.16% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 15.82 | - |  | - |  | - |  | - |  | 8.69 |  | - |  | - |  | - |  | - |  | 5.30 |  | - |  |
|  | 50% | - | - | - | 48.36 | - |  | - |  | - |  | - |  | 36.84 |  | - |  | - |  | - |  | - |  | 29.60 |  | - |  |
|  | 95% | - | - | - | 68.83 | - |  | - |  | - |  | - |  | 69.83 |  | - |  | - |  | - |  | - |  | 69.83 |  | - |  |
|  | Mean | - | - | - | 46.62 | - |  | - |  | - |  | - |  | 38.38 |  | - |  | - |  | - |  | - |  | 32.93 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.25 | - |  | - |  | - |  | - |  | 0.46 |  | - |  | - |  | - |  | - |  | 0.75 |  | - |  |
|  | 50% | - | - | - | 0.08 | - |  | - |  | - |  | - |  | 0.11 |  | - |  | - |  | - |  | - |  | 0.15 |  | - |  |
|  | 95% | - | - | - | 0.06 | - |  | - |  | - |  | - |  | 0.06 |  | - |  | - |  | - |  | - |  | 0.06 |  | - |  |
|  | Mean | - | - | - | 0.11 | - |  | - |  | - |  | - |  | 0.17 |  | - |  | - |  | - |  | - |  | 0.25 |  | - |  |
| RU |  | - | - | - | 22.44% | - |  | - |  | - |  | - |  | 39.67% |  | - |  | - |  | - |  | - |  | 53.39% |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162117, CN delay = 0ms, Enhanced frame structure example 1, UE speed = 3km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection is maintained for each FTP packet Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-21.94% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 9.74 | - |  | - |  | - |  | - |  | 3.26 |  | - |  | - |  | - |  | - |  | 1.42 |  | - |  |
|  | 50% | - | - | - | 35.35 | - |  | - |  | - |  | - |  | 23.29 |  | - |  | - |  | - |  | - |  | 19.40 |  | - |  |
|  | 95% | - | - | - | 70.35 | - |  | - |  | - |  | - |  | 66.19 |  | - |  | - |  | - |  | - |  | 56.34 |  | - |  |
|  | Mean | - | - | - | 37.36 | - |  | - |  | - |  | - |  | 27.01 |  | - |  | - |  | - |  | - |  | 22.67 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.41 | - |  | - |  | - |  | - |  | 1.22 |  | - |  | - |  | - |  | - |  | 2.80 |  | - |  |
|  | 50% | - | - | - | 0.11 | - |  | - |  | - |  | - |  | 0.17 |  | - |  | - |  | - |  | - |  | 0.21 |  | - |  |
|  | 95% | - | - | - | 0.06 | - |  | - |  | - |  | - |  | 0.06 |  | - |  | - |  | - |  | - |  | 0.07 |  | - |  |
|  | Mean | - | - | - | 0.17 | - |  | - |  | - |  | - |  | 0.38 |  | - |  | - |  | - |  | - |  | 0.66 |  | - |  |
| RU |  | - | - | - | 28.09% | - |  | - |  | - |  | - |  | 56.94% |  | - |  | - |  | - |  | - |  | 73.96% |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162117, CN delay = 0ms, TDD configuration 2, UE speed = 60km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection is maintained for each FTP packet Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-21.16% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 14.27 | - |  | - |  | - |  | - |  | 7.04 |  | - |  | - |  | - |  | - |  | 2.69 |  | - |  |
|  | 50% | - | - | - | 44.44 | - |  | - |  | - |  | - |  | 32.22 |  | - |  | - |  | - |  | - |  | 23.41 |  | - |  |
|  | 95% | - | - | - | 69.83 | - |  | - |  | - |  | - |  | 69.65 |  | - |  | - |  | - |  | - |  | 66.51 |  | - |  |
|  | Mean | - | - | - | 44.03 | - |  | - |  | - |  | - |  | 35.03 |  | - |  | - |  | - |  | - |  | 27.40 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.28 | - |  | - |  | - |  | - |  | 0.57 |  | - |  | - |  | - |  | - |  | 1.47 |  | - |  |
|  | 50% | - | - | - | 0.09 | - |  | - |  | - |  | - |  | 0.12 |  | - |  | - |  | - |  | - |  | 0.17 |  | - |  |
|  | 95% | - | - | - | 0.06 | - |  | - |  | - |  | - |  | 0.06 |  | - |  | - |  | - |  | - |  | 0.06 |  | - |  |
|  | Mean | - | - | - | 0.12 | - |  | - |  | - |  | - |  | 0.20 |  | - |  | - |  | - |  | - |  | 0.43 |  | - |  |
| RU |  | - | - | - | 23.51% | - |  | - |  | - |  | - |  | 43.63% |  | - |  | - |  | - |  | - |  | 64.72% |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162117, CN delay = 0ms, Enhanced frame structure example 1, UE speed = 60km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection is maintained for each FTP packet1 second at least Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-21.94% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 9.09 | - |  | - |  | - |  | - |  | 1.94 |  | - |  | - |  | - |  | - |  | 1.05 |  | - |  |
|  | 50% | - | - | - | 33.45 | - |  | - |  | - |  | - |  | 20.25 |  | - |  | - |  | - |  | - |  | 17.34 |  | - |  |
|  | 95% | - | - | - | 70.00 | - |  | - |  | - |  | - |  | 58.58 |  | - |  | - |  | - |  | - |  | 51.19 |  | - |  |
|  | Mean | - | - | - | 35.92 | - |  | - |  | - |  | - |  | 23.44 |  | - |  | - |  | - |  | - |  | 20.84 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.44 | - |  | - |  | - |  | - |  | 2.04 |  | - |  | - |  | - |  | - |  | 3.78 |  | - |  |
|  | 50% | - | - | - | 0.12 | - |  | - |  | - |  | - |  | 0.20 |  | - |  | - |  | - |  | - |  | 0.23 |  | - |  |
|  | 95% | - | - | - | 0.06 | - |  | - |  | - |  | - |  | 0.07 |  | - |  | - |  | - |  | - |  | 0.08 |  | - |  |
|  | Mean | - | - | - | 0.17 | - |  | - |  | - |  | - |  | 0.51 |  | - |  | - |  | - |  | - |  | 0.83 |  | - |  |
| RU |  | - | - | - | 29.7% | - |  | - |  | - |  | - |  | 66.95% |  | - |  | - |  | - |  | - |  | 81.97% |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-162117, CN delay = 0ms, TDD configuration 2, UE speed = 120km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection is maintained for each FTP packet1 second at least Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-21.16% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 11.89 | - |  | - |  | - |  | - |  | 4.53 |  | - |  | - |  | - |  | - |  | 1.76 |  | - |  |
|  | 50% | - | - | - | 43.14 | - |  | - |  | - |  | - |  | 28.34 |  | - |  | - |  | - |  | - |  | 21.84 |  | - |  |
|  | 95% | - | - | - | 69.83 | - |  | - |  | - |  | - |  | 68.80 |  | - |  | - |  | - |  | - |  | 63.49 |  | - |  |
|  | Mean | - | - | - | 42.78 | - |  | - |  | - |  | - |  | 31.31 |  | - |  | - |  | - |  | - |  | 25.49 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.33 | - |  | - |  | - |  | - |  | 0.87 |  | - |  | - |  | - |  | - |  | 2.27 |  | - |  |
|  | 50% | - | - | - | 0.09 | - |  | - |  | - |  | - |  | 0.14 |  | - |  | - |  | - |  | - |  | 0.18 |  | - |  |
|  | 95% | - | - | - | 0.06 | - |  | - |  | - |  | - |  | 0.06 |  | - |  | - |  | - |  | - |  | 0.06 |  | - |  |
|  | Mean | - | - | - | 0.13 | - |  | - |  | - |  | - |  | 0.27 |  | - |  | - |  | - |  | - |  | 0.58 |  | - |  |
| RU |  | - | - | - | 24.95% | - |  | - |  | - |  | - |  | 50.87% |  | - |  | - |  | - |  | - |  | 71.50% |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5,R1-162117, CN delay = 0ms, Enhanced frame structure example 1, UE speed = 120km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection is maintained for each FTP packet1 second at least Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-21.94% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 7OS-10UEs, 3/4OS-10UEs, 2OS-10UEs, 1OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 9.5 | - |  | 4.3 |  | - |  | 3.3 |  | 3.9 |  | - |  | 2.5 |  | - |  | 1.4 |  | 2.1 |  | - |  |
|  | 50% | - | - | - | 30.5 | - |  | 10.5 |  | - |  | 17.9 |  | 20.2 |  | - |  | 8.6 |  | - |  | 14.3 |  | 16.7 |  | - |  |
|  | 95% | - | - | - | 54.4 | - |  | 17.9 |  | - |  | 44.2 |  | 53.6 |  | - |  | 16.9 |  | - |  | 39.7 |  | 50.5 |  | - |  |
|  | Mean | - | - | - | 31.7 | - |  | 10.9 |  | - |  | 20.0 |  | 23.5 |  | - |  | 9.0 |  | - |  | 16.4 |  | 19.9 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.415 | - |  | 0.916 |  | - |  | 1.202 |  | 1.011 |  | - |  | 1.548 |  | - |  | 2.747 |  | 1.867 |  | - |  |
|  | 50% | - | - | - | 0.130 | - |  | 0.380 |  | - |  | 0.224 |  | 0.324 |  | - |  | 0.467 |  | - |  | 0.281 |  | 0.233 |  | - |  |
|  | 95% | - | - | - | 0.078 | - |  | 0.223 |  | - |  | 0.091 |  | 0.078 |  | - |  | 0.237 |  | - |  | 0.102 |  | 0.078 |  | - |  |
|  | Mean | - | - | - | 0.169 | - |  | 0.441 |  | - |  | 0.390 |  | 0.331 |  | - |  | 0.638 |  | - |  | 0.556 |  | 0.544 |  | - |  |
| RU |  | - | - | - | 0.26 | - |  | 0.49 |  | - |  | 0.54 |  | 0.50 |  | - |  | 0.64 |  | - |  | 0.72 |  | 0.66 |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-164824, CN delay = 0ms, TDD configuration 2, UE speed = 3km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:14OS-16.05%, 3/4OS-18.49%, 2OS-21.16% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 3/4OS-10UEs, 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 12.2 | - |  | - |  | - |  | - |  | 6.7 |  | - |  | - |  | - |  | - |  | 4.1 |  | - |  |
|  | 50% | - | - | - | 37.3 | - |  | - |  | - |  | - |  | 28.4 |  | - |  | - |  | - |  | - |  | 22.8 |  | - |  |
|  | 95% | - | - | - | 53.9 | - |  | - |  | - |  | - |  | 53.9 |  | - |  | - |  | - |  | - |  | 53.9 |  | - |  |
|  | Mean | - | - | - | 36.0 | - |  | - |  | - |  | - |  | 29.6 |  | - |  | - |  | - |  | - |  | 25.4 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.326 | - |  | - |  | - |  | - |  | 0.593 |  | - |  | - |  | - |  | - |  | 0.974 |  | - |  |
|  | 50% | - | - | - | 0.107 | - |  | - |  | - |  | - |  | 0.141 |  | - |  | - |  | - |  | - |  | 0.175 |  | - |  |
|  | 95% | - | - | - | 0.074 | - |  | - |  | - |  | - |  | 0.074 |  | - |  | - |  | - |  | - |  | 0.074 |  | - |  |
|  | Mean | - | - | - | 0.146 | - |  | - |  | - |  | - |  | 0.215 |  | - |  | - |  | - |  | - |  | 0.318 |  | - |  |
| RU |  | - | - | - | 0.22 | - |  | - |  | - |  | - |  | 0.40 |  | - |  | - |  | - |  | - |  | 0.53 |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-164825, CN delay = 0ms, Enhanced frame structure set 1 (example 1), UE speed = 3km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-21.94% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 7.5 | - |  | 2.1 |  | - |  | 1.4 |  | 2.5 |  | - |  | 0.8 |  | - |  | 0.8 |  | 1.1 |  | - |  |
|  | 50% | - | - | - | 27.3 | - |  | 8.2 |  | - |  | 13.1 |  | 18.0 |  | - |  | 5.4 |  | - |  | 11.9 |  | 15.0 |  | - |  |
|  | 95% | - | - | - | 54.3 | - |  | 16.4 |  | - |  | 35.6 |  | 51.1 |  | - |  | 14.3 |  | - |  | 32.3 |  | 43.5 |  | - |  |
|  | Mean | - | - | - | 28.8 | - |  | 8.6 |  | - |  | 15.0 |  | 20.8 |  | - |  | 6.2 |  | - |  | 13.7 |  | 17.5 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.531 | - |  | 1.925 |  | - |  | 3.416 |  | 1.581 |  | - |  | 5.044 |  | - |  | 4.800 |  | 3.630 |  | - |  |
|  | 50% | - | - | - | 0.143 | - |  | 0.490 |  | - |  | 0.304 |  | 0.220 |  | - |  | 0.745 |  | - |  | 0.335 |  | 0.272 |  | - |  |
|  | 95% | - | - | - | 0.078 | - |  | 0.242 |  | - |  | 0.113 |  | 0.078 |  | - |  | 0.280 |  | - |  | 0.124 |  | 0.091 |  | - |  |
|  | Mean | - | - | - | 0.220 | - |  | 0.751 |  | - |  | 0.695 |  | 0.493 |  | - |  | 1.451 |  | - |  | 1.174 |  | 0.856 |  | - |  |
| RU |  | - | - | - | 0.30 | - |  | 0.58 |  | - |  | 0.69 |  | 0.57 |  | - |  | 0.76 |  | - |  | 0.84 |  | 0.74 |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-164824, CN delay = 0ms, TDD configuration 2, UE speed = 60km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:14OS-16.05%, 3/4OS-18.49%, 2OS-21.16% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 3/4OS-10UEs, 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 11.0 | - |  | - |  | - |  | - |  | 5.4 |  | - |  | - |  | - |  | - |  | 2.1 |  | - |  |
|  | 50% | - | - | - | 34.3 | - |  | - |  | - |  | - |  | 24.9 |  | - |  | - |  | - |  | - |  | 18.1 |  | - |  |
|  | 95% | - | - | - | 53.9 | - |  | - |  | - |  | - |  | 53.7 |  | - |  | - |  | - |  | - |  | 51.3 |  | - |  |
|  | Mean | - | - | - | 34.0 | - |  | - |  | - |  | - |  | 27.0 |  | - |  | - |  | - |  | - |  | 21.1 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.362 | - |  | - |  | - |  | - |  | 0.736 |  | - |  | - |  | - |  | - |  | 1.908 |  | - |  |
|  | 50% | - | - | - | 0.116 | - |  | - |  | - |  | - |  | 0.161 |  | - |  | - |  | - |  | - |  | 0.221 |  | - |  |
|  | 95% | - | - | - | 0.074 | - |  | - |  | - |  | - |  | 0.074 |  | - |  | - |  | - |  | - |  | 0.078 |  | - |  |
|  | Mean | - | - | - | 0.156 | - |  | - |  | - |  | - |  | 0.255 |  | - |  | - |  | - |  | - |  | 0.557 |  | - |  |
| RU |  | - | - | - | 0.24 | - |  | - |  | - |  | - |  | 0.44 |  | - |  | - |  | - |  | - |  | 0.65 |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-164825, CN delay = 0ms, Enhanced frame structure set 1 (example 1), UE speed = 60km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-21.94% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 7.0 | - |  | 1.7 |  | - |  | 1.2 |  | 1.5 |  | - |  | 0.8 |  | - |  | 0.7 |  | 0.8 |  | - |  |
|  | 50% | - | - | - | 25.8 | - |  | 8.0 |  | - |  | 13.1 |  | 15.6 |  | - |  | 5.9 |  | - |  | 11.4 |  | 13.4 |  | - |  |
|  | 95% | - | - | - | 54.0 | - |  | 16.4 |  | - |  | 34.7 |  | 45.2 |  | - |  | 14.8 |  | - |  | 31.6 |  | 39.5 |  | - |  |
|  | Mean | - | - | - | 27.7 | - |  | 8.4 |  | - |  | 14.7 |  | 18.1 |  | - |  | 6.5 |  | - |  | 13.2 |  | 16.1 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.570 | - |  | 2.274 |  | - |  | 3.221 |  | 2.644 |  | - |  | 4.825 |  | - |  | 5.598 |  | 4.900 |  | - |  |
|  | 50% | - | - | - | 0.156 | - |  | 0.503 |  | - |  | 0.305 |  | 0.259 |  | - |  | 0.679 |  | - |  | 0.352 |  | 0.298 |  | - |  |
|  | 95% | - | - | - | 0.078 | - |  | 0.244 |  | - |  | 0.116 |  | 0.091 |  | - |  | 0.271 |  | - |  | 0.127 |  | 0.104 |  | - |  |
|  | Mean | - | - | - | 0.220 | - |  | 0.808 |  | - |  | 0.759 |  | 0.665 |  | - |  | 1.387 |  | - |  | 1.268 |  | 1.076 |  | - |  |
| RU |  | - | - | - | 0.34 | - |  | 0.59 |  | - |  | 0.78 |  | 0.67 |  | - |  | 0.75 |  | - |  | 0.85 |  | 0.82 |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-164824, CN delay = 0ms, TDD configuration 2, UE speed = 120km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:14OS-16.05%, 3/4OS-18.49%, 2OS-21.16% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 3/4OS-10UEs, 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 9.2 | - |  | - |  | - |  | - |  | 3.5 |  | - |  | - |  | - |  | - |  | 1.4 |  | - |  |
|  | 50% | - | - | - | 33.3 | - |  | - |  | - |  | - |  | 21.9 |  | - |  | - |  | - |  | - |  | 16.8 |  | - |  |
|  | 95% | - | - | - | 53.9 | - |  | - |  | - |  | - |  | 53.1 |  | - |  | - |  | - |  | - |  | 49.0 |  | - |  |
|  | Mean | - | - | - | 33.0 | - |  | - |  | - |  | - |  | 24.2 |  | - |  | - |  | - |  | - |  | 19.7 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.433 | - |  | - |  | - |  | - |  | 1.133 |  | - |  | - |  | - |  | - |  | 2.943 |  | - |  |
|  | 50% | - | - | - | 0.120 | - |  | - |  | - |  | - |  | 0.183 |  | - |  | - |  | - |  | - |  | 0.237 |  | - |  |
|  | 95% | - | - | - | 0.074 | - |  | - |  | - |  | - |  | 0.075 |  | - |  | - |  | - |  | - |  | 0.082 |  | - |  |
|  | Mean | - | - | - | 0.172 | - |  | - |  | - |  | - |  | 0.352 |  | - |  | - |  | - |  | - |  | 0.753 |  | - |  |
| RU |  | - | - | - | 0.25 | - |  | - |  | - |  | - |  | 0.51 |  | - |  | - |  | - |  | - |  | 0.72 |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-164825, CN delay = 0ms, Enhanced frame structure set 1 (example 1), UE speed = 120km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-21.94% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 7.7 | - | 3.2 |  | - |  | 2.5 |  | 3.9 |  | - |  | 1.9 |  | - |  | 1.1 |  | 1.7 |  | - |  |
|  | 50% | - | - | - | 24.5 | - | 8.0 |  | - |  | 13.3 |  | 16.6 |  | - |  | 6.3 |  | - |  | 10.6 |  | 12.7 |  | - |  |
|  | 95% | - | - | - | 39.8 | - | 15.8 |  | - |  | 32.8 |  | 39.6 |  | - |  | 13.4 |  | - |  | 29.4 |  | 37.1 |  | - |  |
|  | Mean | - | - | - | 24.4 | - | 8.5 |  | - |  | 14.8 |  | 18.6 |  | - |  | 6.8 |  | - |  | 12.1 |  | 15.0 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.516 | - | 1.258 |  | - |  | 1.623 |  | 1.016 |  | - |  | 2.100 |  | - |  | 3.709 |  | 2.414 |  | - |  |
|  | 50% | - | - | - | 0.163 | - | 0.501 |  | - |  | 0.302 |  | 0.241 |  | - |  | 0.632 |  | - |  | 0.379 |  | 0.314 |  | - |  |
|  | 95% | - | - | - | 0.100 | - | 0.252 |  | - |  | 0.122 |  | 0.101 |  | - |  | 0.298 |  | - |  | 0.137 |  | 0.107 |  | - |  |
|  | Mean | - | - | - | 0.221 | - | 0.595 |  | - |  | 0.527 |  | 0.365 |  | - |  | 0.861 |  | - |  | 0.750 |  | 0.702 |  | - |  |
| RU |  | - | - | - | 0.23 | - | 0.45 |  | - |  | 0.53 |  | 0.44 |  | - |  | 0.62 |  | - |  | 0.72 |  | 0.64 |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-164824, CN delay = 0ms, TDD configuration 1, UE speed = 3km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:14OS-16.67%, 3/4OS-19.07%, 2OS-21.77% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 3/4OS-10UEs, 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 8.8 | - | - |  | - |  | - |  | 4.8 |  | - |  | - |  | - |  | - |  | 2.3 |  | - |  |
|  | 50% | - | - | - | 26.5 | - | - |  | - |  | - |  | 20.1 |  | - |  | - |  | - |  | - |  | 15.4 |  | - |  |
|  | 95% | - | - | - | 39.1 | - | - |  | - |  | - |  | 39.1 |  | - |  | - |  | - |  | - |  | 38.7 |  | - |  |
|  | Mean | - | - | - | 25.6 | - | - |  | - |  | - |  | 21.2 |  | - |  | - |  | - |  | - |  | 17.3 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.457 | - | - |  | - |  | - |  | 0.827 |  | - |  | - |  | - |  | - |  | 1.774 |  | - |  |
|  | 50% | - | - | - | 0.151 | - | - |  | - |  | - |  | 0.199 |  | - |  | - |  | - |  | - |  | 0.260 |  | - |  |
|  | 95% | - | - | - | 0.102 | - | - |  | - |  | - |  | 0.102 |  | - |  | - |  | - |  | - |  | 0.103 |  | - |  |
|  | Mean | - | - | - | 0.202 | - | - |  | - |  | - |  | 0.300 |  | - |  | - |  | - |  | - |  | 0.561 |  | - |  |
| RU |  | - | - | - | 0.23 | - | - |  | - |  | - |  | 0.40 |  | - |  | - |  | - |  | - |  | 0.58 |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-164825, CN delay = 0ms, Enhanced frame structure set 1 (example 2), UE speed = 3km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-23.35% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | - |  | - |  | - |  | 5.7 |  | - |  | - |  | - |  | - |  | 2.3 |  | - |  |
|  | 50% | - | - | - | - | - | - |  | - |  | - |  | 24.8 |  | - |  | - |  | - |  | - |  | 18.4 |  | - |  |
|  | 95% | - | - | - | - | - | - |  | - |  | - |  | 53.2 |  | - |  | - |  | - |  | - |  | 51.8 |  | - |  |
|  | Mean | - | - | - | - | - | - |  | - |  | - |  | 26.9 |  | - |  | - |  | - |  | - |  | 21.4 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | - |  | - |  | - |  | 0.699 |  | - |  | - |  | - |  | - |  | 1.736 |  | - |  |
|  | 50% | - | - | - | - | - | - |  | - |  | - |  | 0.161 |  | - |  | - |  | - |  | - |  | 0.218 |  | - |  |
|  | 95% | - | - | - | - | - | - |  | - |  | - |  | 0.075 |  | - |  | - |  | - |  | - |  | 0.077 |  | - |  |
|  | Mean | - | - | - | - | - | - |  | - |  | - |  | 0.254 |  | - |  | - |  | - |  | - |  | 0.488 |  | - |  |
| RU |  | - | - | - | - | - | - |  | - |  | - |  | 0.42 |  | - |  | - |  | - |  | - |  | 0.63 |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-164826, CN delay = 0ms, Enhanced frame structure set 2, UE speed = 3km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-22.94% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 6.1 | - | 1.3 |  | - |  | 1.0 |  | 1.9 |  | - |  | 0.5 |  | - |  | 0.6 |  | 0.8 |  | - |  |
|  | 50% | - | - | - | 19.9 | - | 6.0 |  | - |  | 9.7 |  | 13.4 |  | - |  | 3.9 |  | - |  | 8.8 |  | 10.6 |  | - |  |
|  | 95% | - | - | - | 39.7 | - | 12.8 |  | - |  | 26.4 |  | 36.7 |  | - |  | 10.5 |  | - |  | 23.9 |  | 30.4 |  | - |  |
|  | Mean | - | - | - | 21.1 | - | 6.4 |  | - |  | 11.1 |  | 15.2 |  | - |  | 4.5 |  | - |  | 10.1 |  | 12.6 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.653 | - | 3.063 |  | - |  | 4.612 |  | 2.055 |  | - |  | 7.562 |  | - |  | 6.480 |  | 4.926 |  | - |  |
|  | 50% | - | - | - | 0.201 | - | 0.669 |  | - |  | 0.411 |  | 0.299 |  | - |  | 1.034 |  | - |  | 0.452 |  | 0.378 |  | - |  |
|  | 95% | - | - | - | 0.101 | - | 0.312 |  | - |  | 0.152 |  | 0.109 |  | - |  | 0.380 |  | - |  | 0.168 |  | 0.132 |  | - |  |
|  | Mean | - | - | - | 0.270 | - | 1.034 |  | - |  | 0.938 |  | 0.648 |  | - |  | 2.060 |  | - |  | 1.585 |  | 1.186 |  | - |  |
| RU |  | - | - | - | 0.29 | - | 0.58 |  | - |  | 0.70 |  | 0.57 |  | - |  | 0.77 |  | - |  | 0.84 |  | 0.77 |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-164824, CN delay = 0ms, TDD configuration 1, UE speed = 60km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:14OS-16.67%, 3/4OS-19.07%, 2OS-21.77% Maximum number of UEs that can be scheduled per sTTI: 14OS-10UEs, 3/4OS-10UEs, 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 7.3 | - | - |  | - |  | - |  | 2.9 |  | - |  | - |  | - |  | - |  | 1.3 |  | - |  |
|  | 50% | - | - | - | 24.7 | - | - |  | - |  | - |  | 16.3 |  | - |  | - |  | - |  | - |  | 12.8 |  | - |  |
|  | 95% | - | - | - | 39.1 | - | - |  | - |  | - |  | 38.7 |  | - |  | - |  | - |  | - |  | 36.3 |  | - |  |
|  | Mean | - | - | - | 24.3 | - | - |  | - |  | - |  | 18.0 |  | - |  | - |  | - |  | - |  | 14.8 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.548 | - | - |  | - |  | - |  | 1.385 |  | - |  | - |  | - |  | - |  | 3.157 |  | - |  |
|  | 50% | - | - | - | 0.162 | - | - |  | - |  | - |  | 0.245 |  | - |  | - |  | - |  | - |  | 0.313 |  | - |  |
|  | 95% | - | - | - | 0.102 | - | - |  | - |  | - |  | 0.103 |  | - |  | - |  | - |  | - |  | 0.110 |  | - |  |
|  | Mean | - | - | - | 0.223 | - | - |  | - |  | - |  | 0.436 |  | - |  | - |  | - |  | - |  | 0.877 |  | - |  |
| RU |  | - | - | - | 0.25 | - | - |  | - |  | - |  | 0.50 |  | - |  | - |  | - |  | - |  | 0.69 |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-164825, CN delay = 0ms, Enhanced frame structure set 1 (example 2), UE speed = 60km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-23.35% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | - |  | - |  | - |  | 3.9 |  | - |  | - |  | - |  | - |  | 1.3 |  | - |  |
|  | 50% | - | - | - | - | - | - |  | - |  | - |  | 20.8 |  | - |  | - |  | - |  | - |  | 15.2 |  | - |  |
|  | 95% | - | - | - | - | - | - |  | - |  | - |  | 52.4 |  | - |  | - |  | - |  | - |  | 44.5 |  | - |  |
|  | Mean | - | - | - | - | - | - |  | - |  | - |  | 23.4 |  | - |  | - |  | - |  | - |  | 18.1 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | - |  | - |  | - |  | 1.031 |  | - |  | - |  | - |  | - |  | 3.147 |  | - |  |
|  | 50% | - | - | - | - | - | - |  | - |  | - |  | 0.192 |  | - |  | - |  | - |  | - |  | 0.263 |  | - |  |
|  | 95% | - | - | - | - | - | - |  | - |  | - |  | 0.076 |  | - |  | - |  | - |  | - |  | 0.089 |  | - |  |
|  | Mean | - | - | - | - | - | - |  | - |  | - |  | 0.428 |  | - |  | - |  | - |  | - |  | 0.810 |  | - |  |
| RU |  | - | - | - | - | - | - |  | - |  | - |  | 0.49 |  | - |  | - |  | - |  | - |  | 0.75 |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-164826, CN delay = 0ms, Enhanced frame structure set 2, UE speed = 60km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-22.94% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 5.1 | - | 1.2 |  | - |  | 0.9 |  | 1.1 |  | - |  | 0.5 |  | - |  | 0.5 |  | 0.6 |  | - |  |
|  | 50% | - | - | - | 18.4 | - | 5.8 |  | - |  | 9.7 |  | 11.8 |  | - |  | 3.7 |  | - |  | 8.4 |  | 9.9 |  | - |  |
|  | 95% | - | - | - | 39.6 | - | 12.8 |  | - |  | 25.7 |  | 33.5 |  | - |  | 10.4 |  | - |  | 23.4 |  | 28.7 |  | - |  |
|  | Mean | - | - | - | 19.9 | - | 6.2 |  | - |  | 10.9 |  | 13.7 |  | - |  | 4.3 |  | - |  | 9.8 |  | 11.7 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.787 | - | 3.449 |  | - |  | 4.348 |  | 3.635 |  | - |  | 8.258 |  | - |  | 7.558 |  | 6.251 |  | - |  |
|  | 50% | - | - | - | 0.218 | - | 0.691 |  | - |  | 0.412 |  | 0.338 |  | - |  | 1.076 |  | - |  | 0.475 |  | 0.404 |  | - |  |
|  | 95% | - | - | - | 0.101 | - | 0.312 |  | - |  | 0.156 |  | 0.119 |  | - |  | 0.383 |  | - |  | 0.172 |  | 0.139 |  | - |  |
|  | Mean | - | - | - | 0.300 | - | 1.116 |  | - |  | 1.025 |  | 0.924 |  | - |  | 2.188 |  | - |  | 1.711 |  | 1.425 |  | - |  |
| RU |  | - | - | - | 0.35 | - | 0.60 |  | - |  | 0.72 |  | 0.65 |  | - |  | 0.79 |  | - |  | 0.85 |  | 0.80 |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-164824, CN delay = 0ms, TDD configuration 1, UE speed = 120km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:14OS-16.67%, 3/4OS-19.07%, 2OS-21.77% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | 6.9 | - | - |  | - |  | - |  | 1.8 |  | - |  | - |  | - |  | - |  | 0.9 |  | - |  |
|  | 50% | - | - | - | 23.4 | - | - |  | - |  | - |  | 14.7 |  | - |  | - |  | - |  | - |  | 11.1 |  | - |  |
|  | 95% | - | - | - | 39.1 | - | - |  | - |  | - |  | 37.9 |  | - |  | - |  | - |  | - |  | 31.9 |  | - |  |
|  | Mean | - | - | - | 23.6 | - | - |  | - |  | - |  | 16.5 |  | - |  | - |  | - |  | - |  | 13.3 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | 0.578 | - | - |  | - |  | - |  | 2.207 |  | - |  | - |  | - |  | - |  | 4.626 |  | - |  |
|  | 50% | - | - | - | 0.171 | - | - |  | - |  | - |  | 0.271 |  | - |  | - |  | - |  | - |  | 0.360 |  | - |  |
|  | 95% | - | - | - | 0.102 | - | - |  | - |  | - |  | 0.105 |  | - |  | - |  | - |  | - |  | 0.125 |  | - |  |
|  | Mean | - | - | - | 0.238 | - | - |  | - |  | - |  | 0.609 |  | - |  | - |  | - |  | - |  | 1.169 |  | - |  |
| RU |  | - | - | - | 0.26 | - | - |  | - |  | - |  | 0.56 |  | - |  | - |  | - |  | - |  | 0.77 |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-164825, CN delay = 0ms, Enhanced frame structure set 1 (example 2), UE speed = 120km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-23.35% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  | 14 OS |  | 7OS |  | 3/4 OS |  | 2OS |  | 1OS |  |
| DL: UPT CDF [Mbps] | 5% | - | - | - | - | - | - |  | - |  | - |  | 2.6 |  | - |  | - |  | - |  | - |  | 1.0 |  | - |  |
|  | 50% | - | - | - | - | - | - |  | - |  | - |  | 18.8 |  | - |  | - |  | - |  | - |  | 14.3 |  | - |  |
|  | 95% | - | - | - | - | - | - |  | - |  | - |  | 51.4 |  | - |  | - |  | - |  | - |  | 42.1 |  | - |  |
|  | Mean | - | - | - | - | - | - |  | - |  | - |  | 21.6 |  | - |  | - |  | - |  | - |  | 17.1 |  | - |  |
| DL: Delay CDF [s] | 5% | - | - | - | - | - | - |  | - |  | - |  | 1.537 |  | - |  | - |  | - |  | - |  | 3.934 |  | - |  |
|  | 50% | - | - | - | - | - | - |  | - |  | - |  | 0.213 |  | - |  | - |  | - |  | - |  | 0.280 |  | - |  |
|  | 95% | - | - | - | - | - | - |  | - |  | - |  | 0.078 |  | - |  | - |  | - |  | - |  | 0.095 |  | - |  |
|  | Mean | - | - | - | - | - | - |  | - |  | - |  | 0.460 |  | - |  | - |  | - |  | - |  | 0.932 |  | - |  |
| RU |  | - | - | - | - | - | - |  | - |  | - |  | 0.56 |  | - |  | - |  | - |  | - |  | 0.79 |  | - |  |
| 𝜆 |  | 0.25 |  |  |  |  | 0.35 |  |  |  |  |  |  |  |  |  | 0.42 |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 5, R1-164826, CN delay = 0ms, Enhanced frame structure set 2, UE speed = 120km/h, 20M bandwidth, macro scenario FTP model 3 is assumed. TCP connection with slow start is not limited at the first FTP packet. Total control and GP overhead per subframe compared to all available REs in the sTTIs [%]:2OS-22.94% Maximum number of UEs that can be scheduled per sTTI: 2OS-10UEs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.2.2-4: Results with 1MB file transfer

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| DL: UPT CDF [Mbps] | 5% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mean |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DL: Delay CDF [s] | 5% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 50% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 95% |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Mean |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| RU |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## B.3 Evaluation results with UL traffic and non-ideal core network delay (FS1 only)

Table B.3-1: Results with 100kbit file transfer

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS |
| UL: UPT CDF [Mbps] | 5% | 0.28 | 0.49 | 0.41 | 0.34 | - | 0.18 | 0.32 | 0.29 | 0.19 | - | 0.27 | 0.29 | 0.27 | 0.24 | - |
|  | 50% | 0.76 | 1.12 | 1.27 | 1.25 | - | 0.74 | 1.08 | 1.24 | 1.03 | - | 0.49 | 0.61 | 0.65 | 0.66 | - |
|  | 95% | 1.38 | 1.81 | 2.25 | 2.23 | - | 1.46 | 1.89 | 2.21 | 2.04 | - | 1.12 | 1.48 | 1.73 | 1.80 | - |
|  | Mean | 0.80 | 1.12 | 1.27 | 1.24 | - | 0.76 | 1.08 | 1.22 | 1.04 | - | 0.55 | 0.71 | 0.80 | 0.80 | - |
| UL: Delay CDF [s] | 5% | 0.09 | 0.06 | 0.05 | 0.05 | - | 0.09 | 0.06 | 0.05 | 0.05 | - | 0.11 | 0.08 | 0.06 | 0.06 | - |
|  | 50% | 0.14 | 0.09 | 0.08 | 0.08 | - | 0.15 | 0.10 | 0.09 | 0.11 | - | 0.21 | 0.17 | 0.17 | 0.16 | - |
|  | 95% | 0.40 | 0.21 | 0.28 | 0.31 | - | 0.60 | 0.37 | 0.37 | 0.60 | - | 0.37 | 0.35 | 0.37 | 0.43 | - |
|  | Mean | 0.18 | 0.12 | 0.12 | 0.13 | - | 0.21 | 0.14 | 0.13 | 0.20 | - | 0.22 | 0.19 | 0.18 | 0.20 | - |
| RU |  | 20 | 20.00 | 25.41 | 34.13 |  | 40.00 | 30.00 | 42.15 | 54.08 |  | 60 | 40.00 | 61.11 | 66.18 |  |
| 𝜆 |  | - |  |  |  |  | - |  |  |  |  | - |  |  |  |  |
| Notes: |  | Source 8, R1-165302, CN delay = 10ms, macro scenario, DMRS in every sTTI, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS | 14 OS | 7OS | 4 OS | 2OS | 1OS |
| UL: UPT CDF [Mbps] | 5% | 0.28 | 0.49 | 0.41 | 0.53 | - | 0.18 | 0.32 | 0.29 | 0.36 | - | 0.27 | 0.29 | 0.27 | 0.30 | - |
|  | 50% | 0.76 | 1.12 | 1.27 | 1.50 | - | 0.74 | 1.08 | 1.24 | 1.43 | - | 0.49 | 0.61 | 0.65 | 0.78 | - |
|  | 95% | 1.38 | 1.81 | 2.25 | 2.62 | - | 1.46 | 1.89 | 2.21 | 2.65 | - | 1.12 | 1.48 | 1.73 | 2.15 | - |
|  | Mean | 0.80 | 1.12 | 1.27 | 1.52 | - | 0.76 | 1.08 | 1.22 | 1.47 | - | 0.55 | 0.71 | 0.80 | 0.96 | - |
| UL: Delay CDF [s] | 5% | 0.09 | 0.06 | 0.05 | 0.05 | - | 0.09 | 0.06 | 0.05 | 0.05 | - | 0.11 | 0.08 | 0.06 | 0.05 | - |
|  | 50% | 0.14 | 0.09 | 0.08 | 0.07 | - | 0.15 | 0.10 | 0.09 | 0.07 | - | 0.21 | 0.17 | 0.17 | 0.14 | - |
|  | 95% | 0.40 | 0.21 | 0.28 | 0.22 | - | 0.60 | 0.37 | 0.37 | 0.31 | - | 0.37 | 0.35 | 0.37 | 0.35 | - |
|  | Mean | 0.18 | 0.12 | 0.12 | 0.10 | - | 0.21 | 0.14 | 0.13 | 0.11 | - | 0.22 | 0.19 | 0.18 | 0.17 | - |
| RU |  | 20.00 | 20.00 | 25.41 | 23.32 | - | 40.00 | 30.00 | 42.15 | 39.91 | - | 50.00 | 40.00 | 61.11 | 60.01 | - |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 8, R1-165302, CN delay = 10ms, macro scenario, DMRS in every second sTTI for 2 OS TTI, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.3-1: Results with 100kB file transfer

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| UL: UPT CDF [Mbps] | 5% | 0.49 | 0.80 | 0.60 | 0.52 | - | 0.35 | 0.49 | 0.42 | 0.32 | - | 0.38 | 0.41 | 0.37 | 0.29 | - |
|  | 50% | 2.07 | 3.01 | 3.41 | 3.10 | - | 1.72 | 2.30 | 2.22 | 1.71 | - | 1.19 | 1.53 | 1.53 | 1.21 | - |
|  | 95% | 3.54 | 4.92 | 5.08 | 4.79 | - | 4.17 | 4.87 | 5.24 | 4.27 | - | 4.45 | 4.53 | 4.02 | 3.91 | - |
|  | Mean | 2.03 | 2.86 | 3.10 | 2.87 | - | 1.84 | 2.34 | 2.44 | 2.03 | - | 1.59 | 1.82 | 1.77 | 1.56 | - |
| UL: Delay CDF [s] | 5% | 0.29 | 0.20 | 0.18 | 0.17 | - | 0.24 | 0.20 | 0.18 | 0.19 | - | 0.23 | 0.20 | 0.22 | 0.20 | - |
|  | 50% | 0.39 | 0.27 | 0.24 | 0.27 | - | 0.48 | 0.37 | 0.39 | 0.54 | - | 0.75 | 0.62 | 0.59 | 0.73 | - |
|  | 95% | 1.74 | 1.03 | 1.26 | 1.64 | - | 2.44 | 1.62 | 1.91 | 2.52 | - | 2.07 | 1.92 | 2.09 | 2.78 | - |
|  | Mean | 0.58 | 0.41 | 0.41 | 0.54 | - | 0.77 | 0.60 | 0.64 | 0.93 | - | 0.89 | 0.83 | 0.85 | 1.02 | - |
| RU |  | 20 | 20.00 | 20.55 | 31.02 | - | 40.00 | 47.94 | 51.13 | 57.41 | - | 60 | 50.00 | 61.68 | 70.23 |  |
| 𝜆 |  | - |  |  |  |  | - |  |  |  |  | - |  |  |  |  |
| Notes: |  | Source 8, R1-165302, CN delay = 10ms, macro scenario, DMRS in every sTTI, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| UL: UPT CDF [Mbps] | 5% | 0.49 | 0.80 | 0.60 | 0.83 | - | 0.35 | 0.49 | 0.42 | 0.47 | - | 0.38 | 0.41 | 0.37 | 0.38 | - |
|  | 50% | 2.07 | 3.01 | 3.41 | 4.00 | - | 1.72 | 2.30 | 2.22 | 2.69 | - | 1.19 | 1.53 | 1.53 | 2.18 | - |
|  | 95% | 3.54 | 4.92 | 5.08 | 6.29 | - | 4.17 | 4.87 | 5.24 | 6.31 | - | 4.45 | 4.53 | 4.02 | 5.43 | - |
|  | Mean | 2.03 | 2.86 | 3.10 | 3.63 | - | 1.84 | 2.34 | 2.44 | 2.87 | - | 1.59 | 1.82 | 1.77 | 2.35 | - |
| UL: Delay CDF [s] | 5% | 0.29 | 0.20 | 0.18 | 0.15 | - | 0.24 | 0.20 | 0.18 | 0.15 | - | 0.23 | 0.20 | 0.22 | 0.16 | - |
|  | 50% | 0.39 | 0.27 | 0.24 | 0.20 | - | 0.48 | 0.37 | 0.39 | 0.32 | - | 0.75 | 0.62 | 0.59 | 0.41 | - |
|  | 95% | 1.74 | 1.03 | 1.26 | 0.96 | - | 2.44 | 1.62 | 1.91 | 1.71 | - | 2.07 | 1.92 | 2.09 | 2.07 | - |
|  | Mean | 0.58 | 0.41 | 0.41 | 0.38 | - | 0.77 | 0.60 | 0.64 | 0.56 | - | 0.89 | 0.83 | 0.85 | 0.76 | - |
| RU |  | 20.00 | 20.55 | 23.06 | 21.15 | - | 40.00 | 47.94 | 51.13 | 46.29 | - | 50.00 | 61.68 | 65.96 | 60.95 | - |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 8, R1-165302, CN delay = 10ms, macro scenario, DMRS in every second sTTI for 2 OS TTI, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.3-1: Results with 500kB file transfer

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| UL: UPT CDF [Mbps] | 5% | 0.54 | 0.85 | 0.81 | 0.51 | - | 0.47 | 0.59 | 0.59 | 0.40 | - | 0.50 | 0.48 | 0.43 | 0.36 | - |
|  | 50% | 5.34 | 6.10 | 5.74 | 4.22 | - | 3.27 | 3.43 | 3.15 | 2.01 | - | 2.27 | 2.17 | 1.92 | 1.53 | - |
|  | 95% | 6.89 | 8.98 | 10.07 | 8.34 | - | 6.65 | 8.46 | 8.92 | 6.76 | - | 6.74 | 7.77 | 7.26 | 5.58 | - |
|  | Mean | 4.67 | 5.58 | 5.48 | 4.30 | - | 3.58 | 3.99 | 3.86 | 2.75 | - | 2.98 | 3.04 | 2.78 | 2.07 | - |
| UL: Delay CDF [s] | 5% | 0.58 | 0.45 | 0.40 | 0.47 | - | 0.60 | 0.47 | 0.45 | 0.60 | - | 0.60 | 0.52 | 0.55 | 0.68 | - |
|  | 50% | 0.76 | 0.67 | 0.75 | 1.07 | - | 1.36 | 1.29 | 1.56 | 2.33 | - | 2.32 | 2.38 | 2.54 | 3.10 | - |
|  | 95% | 6.42 | 4.72 | 4.78 | 8.18 | - | 8.47 | 6.53 | 6.61 | 9.95 | - | 8.10 | 8.10 | 9.04 | 11.14 | - |
|  | Mean | 1.84 | 1.32 | 1.62 | 2.21 | - | 2.55 | 2.15 | 2.35 | 3.46 | - | 3.25 | 3.06 | 3.52 | 4.46 | - |
| RU |  | 20.00 | 20.83 | 23.36 | 32.26 |  | 40.00 | 45.78 | 48.65 | 57.19 | 58.93 | 50.00 | 58.93 | 63.43 | 68.26 | - |
| 𝜆 |  | - |  |  |  |  | - |  |  |  |  | - |  |  |  |  |
| Notes: |  | Source 8, R1-165302, CN delay = 10ms, macro scenario, DMRS in every sTTI, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| UL: UPT CDF [Mbps] | 5% | 0.54 | 0.85 | 0.81 | 0.78 | - | 0.47 | 0.59 | 0.59 | 0.63 | - | 0.46 | 0.44 | 0.42 | 0.46 | - |
|  | 50% | 5.34 | 6.10 | 5.74 | 6.90 | - | 3.27 | 3.43 | 3.15 | 3.63 | - | 1.76 | 1.75 | 1.48 | 1.97 | - |
|  | 95% | 6.89 | 8.98 | 10.07 | 11.10 | - | 6.65 | 8.46 | 8.92 | 9.90 | - | 6.53 | 6.74 | 6.59 | 8.19 | - |
|  | Mean | 4.67 | 5.58 | 5.48 | 6.33 | - | 3.58 | 3.99 | 3.86 | 4.34 | - | 2.55 | 2.50 | 2.37 | 2.88 | - |
| UL: Delay CDF [s] | 5% | 0.58 | 0.45 | 0.40 | 0.36 | - | 0.60 | 0.47 | 0.45 | 0.40 | - | 0.61 | 0.60 | 0.60 | 0.48 | - |
|  | 50% | 0.76 | 0.67 | 0.75 | 0.60 | - | 1.36 | 1.29 | 1.56 | 1.34 | - | 2.98 | 2.81 | 3.33 | 2.44 | - |
|  | 95% | 6.42 | 4.72 | 4.78 | 5.00 | - | 8.47 | 6.53 | 6.61 | 6.34 | - | 8.64 | 8.99 | 9.40 | 8.65 | - |
|  | Mean | 1.84 | 1.32 | 1.62 | 1.66 | - | 2.55 | 2.15 | 2.35 | 2.12 | - | 3.69 | 3.58 | 3.97 | 3.40 | - |
| RU |  | 20.00 | 20.83 | 23.36 | 21.37 | - | 40.00 | 45.78 | 48.65 | 45.42 | - | 55.00 | 67.10 | 69.51 | 67.63 | - |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 8, R1-165302, CN delay = 10ms, macro scenario, DMRS in every second sTTI for 2 OS TTI, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table B.3-1: Results with 1MB file transfer

| Reported parameters |  | Low load RU range for legacy TTI: 10%~25% |  |  |  |  | Medium load RU range for legacy TTI: 35%~50% |  |  |  |  | High load RU range for legacy TTI: above 55% |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| UL: UPT CDF [Mbps] | 5% | 0.56 | 0.86 | 0.59 | 0.52 | - | 0.51 | 0.61 | 0.59 | 0.35 | - | 0.51 | 0.54 | 0.55 | 0.38 | - |
|  | 50% | 6.57 | 6.89 | 6.15 | 4.18 | - | 4.28 | 3.51 | 3.12 | 2.12 | - | 2.87 | 2.44 | 2.24 | 1.60 | - |
|  | 95% | 10.22 | 12.11 | 12.74 | 9.70 | - | 9.60 | 10.80 | 10.90 | 7.53 | - | 9.03 | 9.15 | 8.84 | 5.42 | - |
|  | Mean | 5.91 | 6.76 | 6.41 | 4.58 | - | 4.79 | 4.58 | 4.29 | 2.81 | - | 3.78 | 3.44 | 3.15 | 2.06 | - |
| UL: Delay CDF [s] | 5% | 0.78 | 0.66 | 0.62 | 0.82 | - | 0.82 | 0.74 | 0.73 | 1.05 | - | 0.89 | 0.88 | 0.91 | 1.52 | - |
|  | 50% | 1.31 | 1.26 | 1.37 | 2.22 | - | 2.25 | 2.49 | 2.79 | 4.68 | - | 3.96 | 3.73 | 4.29 | 5.47 | - |
|  | 95% | 12.12 | 7.91 | 12.02 | 15.03 | - | 15.15 | 12.76 | 13.18 | 22.66 | - | 15.40 | 14.72 | 14.62 | 20.16 | - |
|  | Mean | 3.41 | 2.50 | 3.16 | 4.27 | - | 4.37 | 3.87 | 4.54 | 7.44 | - | 5.57 | 5.40 | 5.91 | 7.86 | - |
| RU |  | 20.00 | 19.56 | 22.54 | 31.41 | - | 40.00 | 45.22 | 47.91 | 57.31 | - | 50.00 | 59.64 | 62.98 | 70.02 | - |
| 𝜆 |  | - |  |  |  |  | - |  |  |  |  | - |  |  |  |  |
| Notes: |  | Source 8, R1-165302, CN delay = 10ms, macro scenario, DMRS in every sTTI, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS | 14 OS | 7OS | 3/4 OS | 2OS | 1OS |
| UL: UPT CDF [Mbps] | 5% | 0.56 | 0.86 | 0.59 | 0.76 | - | 0.51 | 0.61 | 0.59 | 0.60 | - | 0.51 | 0.54 | 0.55 | 0.52 | - |
|  | 50% | 6.57 | 6.89 | 6.15 | 6.84 | - | 4.28 | 3.51 | 3.12 | 3.59 | - | 2.87 | 2.44 | 2.24 | 2.53 | - |
|  | 95% | 10.22 | 12.11 | 12.74 | 13.77 | - | 9.60 | 10.80 | 10.90 | 11.50 | - | 9.03 | 9.15 | 8.84 | 9.76 | - |
|  | Mean | 5.91 | 6.76 | 6.41 | 7.01 | - | 4.79 | 4.58 | 4.29 | 4.69 | - | 3.78 | 3.44 | 3.15 | 3.59 | - |
| UL: Delay CDF [s] | 5% | 0.78 | 0.66 | 0.62 | 0.57 | - | 0.82 | 0.74 | 0.73 | 0.71 | - | 0.89 | 0.88 | 0.91 | 0.82 | - |
|  | 50% | 1.31 | 1.26 | 1.37 | 1.26 | - | 2.25 | 2.49 | 2.79 | 2.56 | - | 3.96 | 3.73 | 4.29 | 4.00 | - |
|  | 95% | 12.12 | 7.91 | 12.02 | 9.72 | - | 15.15 | 12.76 | 13.18 | 12.15 | - | 15.40 | 14.72 | 14.62 | 15.39 | - |
|  | Mean | 3.41 | 2.50 | 3.16 | 3.53 | - | 4.37 | 3.87 | 4.54 | 4.06 | - | 5.57 | 5.40 | 5.91 | 5.61 | - |
| RU |  | 20.00 | 19.56 | 22.54 | 21.55 | - | 40.00 | 45.22 | 47.91 | 46.18 | - | 50.00 | 59.64 | 62.98 | 59.28 | - |
| 𝜆 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Notes: |  | Source 8, R1-165302, CN delay = 10ms, macro scenario, DMRS in every second sTTI for 2 OS TTI, dynamic control region per sTTI assumed (dependent of the scheduled UEs in a sTTI) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------- End of Text Proposal ------------------------------------------------

# References

- R1-161422, Way forward on simulation results and methodology, Ericsson, RAN1#84, February 2016

- TR36.881v0.6.0, Study on latency reduction techniques for LTE