3GPP TSG RAN1 Meeting #85 	R1-165531

# Nanjing, China, May 23-27, 2016

| CR-Form-v11.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.213 | CR | 669 | rev | - | Current version: | 13.1.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the usage of CEModeA and CEModeB for eMTC in TS 36.213 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_MTCe2_L1 |  |  |  |  |  |  | Date: |  | 2016-05-24 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) |  |
| Reason for change: |  | CEModeA and CEModeB are only used for BL/CE UE in RRC_CONNECTED state with configured CEMode. For BL/CE UE in RRC_IDLE state, the CEMode has not been configured, and PRACH CE level should be used. In current specification, the description related to "If a UE is configured with CEModeA" or "If a UE is configured with CEModeB" does not cover the case that UE has not been configured with a CEMode. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For BL/CE UE without a configured CEMode, “CEModeA” means UE in PRACH CE level 0 or 1, and “CEModeB” means UE in PRACH CE level 2 or 3. Change CEMode_A to CEModeA to make unified description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description related to "If a UE is configured with CEModeA" or "If a UE is configured with CEModeB" does not cover the case that UE does not have avaialble CEMode. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.6.4A, 7.3.2.1, 9.1.5.1, 12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

< Unchanged parts are omitted >

7.1.6.4A	PDSCH starting position for BL/CE UEs

The starting OFDM symbol for PDSCH is given by index  in the first slot in a subframe  and is determined as follows

-	if subframe  is a subframe used for receiving SIB1-BR

-	if for the cell on which PDSCH is received

-	if for the cell on which PDSCH is received

-	else

-	is given by the higher layer parameter startSymbolLC

-	if subframe  is configured as an MBSFN subframe and if the BL/CE UE is configured in CEModeA

-

-	else

-	.

< Unchanged parts are omitted >

If the BL/CE UE is configured in CEModeA, and if the PDSCH is assigned by or semi-statically scheduled by a MPDCCH with DCI format 6-1A, the UE shall assume no PDSCH repetition if the higher layer parameter csi-NumRepetitionCE-r13 indicates one subframe.

7.3.2.2	TDD HARQ-ACK reporting procedure for different UL/DL configurations

< Unchanged parts are omitted >

9.1.5.1	MPDCCH starting position

The starting OFDM symbol for MPDCCH given by index  in the first slot in a subframe  and is determined as follows

-	if subframe  is a subframe used for receiving SIB1-BR

-	if for the cell on which PDSCH is received

-	if for the cell on which PDSCH is received

-	else

-	is given by the higher layer parameter startSymbolLC

-	if subframe  is configured as an MBSFN subframe and if the BL/CE UE is configured in CEModeA

-

-	else

-	.

< Unchanged parts are omitted >

12	Assumptions independent of physical channel

A UE shall not assume that two antenna ports are quasi co-located unless specified otherwise.

A UE may assume the antenna ports 0 – 3 of a serving cell are quasi co-located (as defined in [3]) with respect to delay spread, Doppler spread, Doppler shift, average gain, and average delay.

For the purpose of discovery-signal-based measurements, a UE shall not assume any other signals or physical channels are present other than the discovery signal.

If a UE supports discoverySignalsInDeactSCell-r12, and if the UE is configured with discovery-signal-based RRM measurements on a carrier frequency applicable for a secondary cell on the same carrier frequency, and if the secondary cell is deactivated, and if the UE is not configured by higher layers to receive MBMS on the secondary cell, the UE shall, except for discovery-signal transmissions, assume that PSS, SSS, PBCH, CRS, PCFICH, PDSCH, PDCCH, EPDCCH, PHICH, DMRS and CSI-RS may be not transmitted by the secondary cell until the subframe where an activation command is received for the secondary cell.

For BL/CE UE, if CEModeA or CEModeB is not configured, UE shall assume the following configuration:

- For a BL/CE UE with the PRACH coverage enhancement level 0/1, UE shall assume CEModeA.

- For a BL/CE UE with the PRACH coverage enhancement level 2/3, UE shall assume CEModeB.

< Unchanged parts are omitted >