3GPP TSG RAN WG1 #86	R1-166281

August 22nd – 27th, 2016

Gothenburg, Sweden

Agenda item:    7.2.6.1

Source:             Qualcomm Incorporated

Title:                  Design options for longer cyclic prefix and link level simulation results

Document for:  Discussion and Decision

- Introduction

In RAN#71 a new work item (WI) named eMBMS enhancement for LTE [1] was introduced. The objectives of the WI include the support of longer CP, additional MBSFN subframes, and study standalone carrier. In a previous contribution [2], we presented simulation results showing 95% SINR coverage for both outdoor and rooftop simulation scenarios for various CP lengths and showed that a CP length in the range of 100us-200us resulted in SINR coverage exceeding ~15dB, which corresponds to a spectral efficiency goal of 2 b/s/Hz. In a companion contribution [3], we proposed various design options for the reference signal design for the longer CP cases. In this document, we show link level simulation results for a subset of these configurations with given large delay spread channel model.

- Simulation Assumptions

For the link level simulation assumption, we recommend leveraging portions of the RAN#4 PMCH performance specifications defined in 36.101 [4]. A table of proposed link level simulation parameters for the extended CP PMCH channel is given in Table 1.

| Extended CP PMCH Simulation Parameters |  |
| --- | --- |
| CP Lengths | 100us, 200us |
| Bandwidth | 10MHz |
| Carrier Freq | 670MHz |
| Symbols for Control | 0 |
| Channel Model | 18 tap, 3 cluster TU6 Cluster delays: (0,90,130us) for 200us CP (0,30,50us) for 100us CP |
| Speed | 5, 60, 100 Km/Hr |
| RS Tone Separation | 3 |
| RS Stagger Period (syms) | 2 for 200us CP 2,3 for 100us CP |
| Modulation | 64 QAM |
| Payload Sizes (bits) | 18336 (MCS12) 19848 (MCS13) 21384 (MCS14) 22920 (MCS15) |

Table 1: Proposed Parameters for Extended CP Link Level Simulation

For purposes of performance evaluation, there is no gating of the broadcast signal with any unicast data and no insertion of any control symbols. The reference signals repeat with a periodicity equal to the stagger period; we consider two stagger periods of 2 and 3 symbols in order to examine the performance trade-off of Doppler performance with reference signal overhead. The reference signal configuration parameters are defined in frequency and time by their tone separation and stagger period, respectively, as shown in Figure 1.

Figure 1: Reference Signal Configuration Parameters

For the channel delay spread, we propose an 18-path channel that is composed of three sets of a TU6 channel with cluster delays of 0us, 90us, and 130us for the 200us CP and 0us, 30us, and 50us for the 100us CP case. The relative mean power for the three clusters are 0dB, -5dB, and -10dB. The exact path configuration for the 200us CP case is given in Table 1. Additionally,  speeds of 5, 60, and 100Km/Hr are considered with the carrier frequency configured at 670MHz.

| Extended Delay Spread |  |  |
| --- | --- | --- |
| Path Number | Relative Delay [us] | Relative Mean Power [dB] |
| 1 | 0 | -3.0 |
| 2 | .2 | 0.0 |
| 3 | .5 | -2.0 |
| 4 | 1.6 | -6.0 |
| 5 | 2.3 | -8.0 |
| 6 | 5.0 | -10.0 |
| 7 | 90.0 | -8.0 |
| 8 | 90.2 | -5.0 |
| 9 | 90.5 | -7.0 |
| 10 | 91.6 | -11.0 |
| 11 | 92.3 | -13.0 |
| 12 | 95.0 | -15.0 |
| 13 | 130.0 | -13.0 |
| 14 | 130.2 | -10.0 |
| 15 | 130.5 | -12.0 |
| 16 | 131.6 | -16.0 |
| 17 | 132.3 | -18.0 |
| 18 | 135 | -20.0 |

Table 2: Specification of Simulation Channel Delay Spread

- Summary of Results

For the 100us and 200us CP cases, we show the simulation results for payload sizes of 18363, 19848, 21384, and 22920 bits corresponding to approximate spectral efficiencies of 1.84, 1.98, 2.14, and 2.29 bits/sec/Hz.

In Figures 2-4, the simulations results for a 100us CP with a RS stagger period of 2 symbols is shown for speeds of 5, 60, and 100 Km/Hr, respectively. In Figures 5-7, the simulations results for a 100us CP with a RS stagger period of 3 symbols is shown for speeds of 5, 60, and 100 Km/Hr, respectively. Finally, in figures 8-10, the simulations results for a 200us CP with a RS stagger period of 2 symbols is shown for speeds of 5, 60, and 100 Km/Hr, respectively.

Figure 2: eMBMS, 100us CP, RS Stagger Period 2, 3 Cluster TU, 5Km/Hr

Figure 3: eMBMS, 100us CP, RS Stagger Period 2, 3 Cluster TU, 60Km/Hr

Figure 4: eMBMS, 100us CP, RS Stagger Period 2, 3 Cluster TU, 100Km/Hr

Figure 5: eMBMS, 100us CP, RS Stagger Period 3, 3 Cluster TU, 5Km/Hr

Figure 6: eMBMS, 100us CP, RS Stagger Period 3, 3 Cluster TU, 60Km/Hr

Figure 7: eMBMS, 100us CP, RS Stagger Period 3, 3 Cluster TU, 100Km/Hr

Figure 8: eMBMS, 200us CP, RS Stagger Period 2, 3 Cluster TU, 5 Km/Hr

Figure 9: eMBMS, 200us CP, RS Stagger Period 2, 3 Cluster TU, 60 Km/Hr

Figure 10: eMBMS, 200us CP, RS Stagger Period 2, 3 Cluster TU, 100Km/Hr

From the graphs, we can approximate the SNR required to obtain a spectral efficiency of 2.0 b/s/Hz and these are listed in Table 3. A comparison of the 100us CP case for RS stagger period of 2 and 3 symbols shows that the 3 symbol RS stagger configuration outperforms at the lower speeds. For the case of 5Km/Hr, there is a .5dB improvement due to the lower density of RS tones providing increased coding gain for the same payload size. For 60Km/Hr, the perormances are comparable, but for the 100 Km/Hr case, the 2 symbol RS stagger period provides a more robust channel estimation due to the higher density of RS tones. Additionally, note that for the 200us CP case provides suitable performance of 16.1dB at the lowest speed of 5Km/Hr. As the speed increases, channel estimation and demodulation suffer due to the larger symbol duration. Note also that the path delay profiles are not the same between the 100us and 200us CP case, so a direct comparison between the two is not entirely useful.

|  | 100us CP, RS Stagger 2 | 100us CP, RS Stagger 3 | 200us CP, RS Stagger 2 |
| --- | --- | --- | --- |
| 5 Km/Hr | 14.5dB | 14.0dB | 15.9dB |
| 60 Km/Hr | 14.7dB | 14.6dB | 17.3dB |
| 100 Km/Hr | 15.0dB | 16.4dB | 20.1dB |

Table 3: SNR operating point to achieve 2 b/s/Hz for 100us and 200us CP cases

These results indicate that both the 100us and 200us CP cases should be included for further consideration in the eMBMS enhancement study.  Both provide reasonable performance at the lower speeds and each offers a trade-off between performance in high Doppler scenarios versus accommodating larger cell size deployments.

Proposal #1: Based on both link level and system level simulation results, both the 100us and 200us CP cases should be considered further for standardization in the eMBMS enhancement study.

- Conclusions

This contribution proposes link level simulation parameters to be used for simulating the extended CP scenario considered in this WI.  A set of eMBMS simulation and channel parameters is proposed. Link level simulation results are provided for both the 100us and 200us CP configurations with the following offered proposal:

Proposal #1: Based on both link level and system level simulation results, both the 100us and 200us CP cases should be considered further for standardization in the eMBMS enhancement study.

5. References

[1] RP-160675, “New WID: eMBMS enhancements for LTE“, Ericsson, Qualcomm Inc. Nokia Networks, EBU;  3GPP RAN #71, Goteborg, Sweden, March 2016

[2] R1-164438 Evaluation Results for LTE eMBMS Enhancements Study

[3] R1-164439 Design Options for Longer Cyclic Prefix for MBSFN Subframes