3GPP TSG RAN WG1 Meeting #86		R1-166283

Gothenburg, Sweden 22nd - 26th August 2016

Agenda item:    7.2.6.3

Source:             Qualcomm Incorporated

Title:                  MBSFN subframes without a unicast control region and cell-specific reference signals

Document for:  Discussion and Decision

- Introduction

In RAN#71 a new work item (WI) named eMBMS enhancement for LTE [1] was introduced. The objectives of the WI include the support of longer CP, additional MBSFN subframes, and study standalone carrier. In this document we provide our views on the support of longer MBSFN subframes without unicast control region and cell-specific reference signals.

- Signaling details of presence of control region

Similarly to the MBSFN definition, we can include additional signalling to indicate the presence or absence of control region in a given subframe. With this, for a given subframe, we have the following possibilities:

- Normal subframe, control region is present

- MBSFN subframe, control region is present

- MBSFN subframe, control region is not present

In general, there is no need to allow for flexibility to signal ON/OFF in every single subframe, since in most cases it is only interesting to turn on/off the control region in all the MBSFN subframes. For example, if a cell is operated with increased number of MBSFN subframes and/or without control region, it can be cross-carrier scheduled from the PCell, if desired. Thus, we propose to signal with a single bit the presence or absence of control region in MBSFN subframes

Proposal 1: A single bit is used to signal the presence or absence of control region in all the MBSFN subframes in a carrier.

- Interaction of longer CP with control region

In current LTE specification the MBSFN numerology is defined regardless of the presence of control symbols. If the control region and MBSFN numerologies are different, then there is a fraction of the transmitted waveform that is not defined by the specification. This approach is not efficient anymore if we want to support longer CP.

In the case with 1 control symbol, for example, the basic 200us numerology without CP can be replaced by a 200us numerology where the CP is punctured by the control region. The specification should allow for this type of operation (e.g. puncturing) for the new CP sizes if a legacy control region is needed.

Figure 2 Different CP options and numerologies for control and control-less MBSFN subframes

Additionally, there may be limited use for control region in subframes with longer CP for standalone and SCell MBSFN cases. Thus, RAN1 should prioritize the definition of longer CP for the case of no control region.

Observation 1: Supporting longer CP and control region simultaneously is possible by redefining the interaction between control and MBSFN region (e.g. by puncturing the CP by the control region for long CP values).

Observation 2: There may be limited use for control region in subframes with longer CP in the standalone and SCell MBSFN cases.

Proposal 2: Prioritize the definition of new numerologies with longer CP for the case of no control region. If control region is desired, it can puncture the CP.

- Control region in non-MBSFN subframes

With the removal of the control region from MBSFN subframes, it could be argued that the control region can be removed from non-MBSFN subframes with additional (but small) effort. In the current WID

c.	Specify means of configuring MBSFN subframes without a unicast control region and cell-specific reference signals. (RAN1, RAN2)

Thus, changing the control region in non-MBSFN subframes is out of the scope of the work item, and can be considered in future LTE work items.

Proposal 3: Do not introduce any change for the control region in non-MBSFN subframes.

- Summary

Proposal 1: The presence of control region is jointly encoded with the MBSFN subframe configuration.

Observation 1: Supporting longer CP and control region simultaneously is possible by redefining the interaction between control and MBSFN region (e.g. by puncturing the CP by the control region for long CP values).

Observation 2: There may be limited use for control region in subframes with longer CP in the standalone and SCell MBSFN cases.

Proposal 2: Prioritize the definition of new numerologies with longer CP for the case of no control region. If control region is desired, it can puncture the CP.

Proposal 3: Do not introduce any change for the control region in non-MBSFN subframes.

References

[1] RP-160675, “New WID: eMBMS enhancements for LTE“, Ericsson, Qualcomm Inc. Nokia Networks, EBU;  3GPP RAN #71, Goteborg, Sweden, March 2016