3GPP TSG RAN WG1 Meeting #86                                                    R1-166470

Gothenburg, Sweden, August 22nd - 26th 2016

Source:	CATT

Title:	Initial SLS results of PDMA

Agenda Item:	8.1.2.2

Document for: 	Discussion and Decision

Introduction

During RAN1#85 meeting, system-level evaluation assumptions were agreed for MA in mMTC scenario – urban coverage for massive connection [1]. In this contribution, we discuss link-to-system (L2S) interface evaluation methodology for non-orthogonal MA. And SLS simulation results are also presented.

Discussions on PHY abstraction methodology of non-orthogonal MA schemes

Abstracting the link performance plays an important role in simple and accurate performance evaluation of wireless communication system. This L2S interface is often referred to as a PHY abstraction model and is based on an effective signal-to-noise ratio (SNR) mapping (ESM) concept. A number of PHY abstraction methods have been introduced for linear receiver, which can be classified as exponential, or Shannon capacity ESM (EESM), mapping function method with mutual information, such as received-bit information rate (RBIR) and mutual information per bit (MMIB). However, for non-orthogonal multiple access system, non-linear receiver is applied and accurate link estimation becomes challenging due to no direct derivation for post-processing SINR on every subcarrier.

PHY abstraction is implemented with two processes: effective SNR calculation and curve fitting for , introduced in detail as follows. In this contribution, only the post-processing SINR upper bound is calculated and by adjusting the factor , BLER corresponding to the effective SNR is made to approaching to link level simulation results.

- Effective SNR calculation

The ultimate goal of L2S interface is to obtain a single estimate, the effective SNR, for a user. To achieve this goal, the effective SNR is calculated with the following three steps:

- Step1: For user k, calculate the post-processing SINR upper bound on subcarrier n. PIC (Perfect Interference Cancellation) is usually taken as the upper bound for SINR estimation [2]. With PIC, this SINR upper bound for nth subcarrier is calculated as

,

where ,  is the effective channel for user k in subcarrier n, and  is the covariance matrix of thermal noise plus the inter-cell interference.

- Step2: With the post-processing SINR, calculate the detected capacity for nth subcarrier

or .

where is the fitting factor and predetermined system parameter. With both of the above formulas, almost the same performance can be achieved with different value of

- Step3: Calculate the effective SNR of user k

,

where is the mean capacity on the simulated bandwidth, .

After the post-processing SINR upper bound is calculated in Step 1, the effective SNR can also be calculated with received-bit information rate (RBIR) metrics [3]. The step 2 and step 3 should be modified correspondingly.

- Curve fitting for

The effective SNR is calculated under different fitting factor . The optimal  should be selected with reference to the simulation results of link level. The process of curve fitting for is as follows:

- Step1: Under given simulation conditions, run the link level simulation and get for the ith SINR, which is the average BLER among users with the same diversity

- Step2: For every diversity degree and under same simulation conditions as link level simulation, calculate the post-effective SNR with a given and search the corresponding BLER by looking up the AWGN SNR-BLER table. Get the mean for BLER on the ith SNR, , among subframes and among the users with the same diversity degree.

- Step3: Calculate the mean square error of BLER

In the searching space of , find the optimal ,, to meet minimum MSE, that is

is the fitting results and used in the system level simulation.

Simulation results for L2S

The simulation parameters are shown in Table I.

Table I  L2S Simulation parameters

| Parameters | value |
| --- | --- |
| Channel type | ITU NLOS |
| Number for seed | 1 seed and 10000 realization |
| Number PRB | 4 |
| MCS | QPSK, 1/2 |
| Antenna configuration | 1 Tx 2Rx |
| Power allocation | No pathloss difference |
| Receiver | Turbo-MPA (outer / inner iteration number: 3 / 3) |
| PDMA pattern matrix |  |
| Metric | MSE between link level simulation BLER and L2S calculation BLER |
| Number of UEs | 8 |

In the simulation, PDMA pattern matrix  is used for the simulation, by which maximal 8 users can share one PRB groups with 4 PRBs. The first 4 users use the patterns with the diversity degree of 2, and the other 4 users have diversity degree of 1. The L2S processes for these two kinds of users are implemented separately. The minimal mean square error (MSE) and optimal  are shown in Table II.

Table II L2S simulation results

|  | User1-4 | User5-8 |
| --- | --- | --- |
| Minimal MSE | 0.0039 | 0.0017 |
| Optimal | 0.87 | 0.92 |

In this simulation, the detected capacity for nth subcarrier is calculated by . Figure 1 and Figure 2 give the results for curve fitting with optimal for user 1~4 with diversity degree 2 and user 5~8 with diversity degree 1, respectively.

Figure 1. Curve fitting results with optimal for user 1~4

Figure 2. Curve fitting results with optimal for user 5~8

Simulation results for UL mMTC

In RAN1#85, system-level evaluation assumptions were agreed for MA in mMTC scenario. In this part, the simulation results are shown. Table III gives the simulation parameters.

Table III System level simulation parameters.

| Attributes | Values or assumptions |
| --- | --- |
| Layout | Single layer --Macro layer: Hex. Grid |
| Inter-BS distance | 1732m |
| Carrier frequency | 700MHz |
| Simulation bandwidth | 4 PRB |
| Channel model | 3D UMa |
| Tx power | UE: Max 23dBm |
| BS antenna configuration | Rx: 2 |
| BS antenna pattern | Follow the modeling of TR36.873 |
| BS antenna height | 25m |
| BS antenna tilt | 100 |
| BS antenna element gain + connector loss | 8 dBi, including 3dB cable loss |
| BS receiver noise figure | 5 dB |
| UE antenna elements | 1Tx |
| UE antenna height | 1.5m |
| UE antenna gain | -4dBi |
| Traffic model | Non-full buffer small packet. Packet size=20 bytes |
| UE distribution | 20% of users are outdoors (3km/h) 80% of users are indoor (3km/h) Users dropped uniformly in entire cell |
| BS receiver | Turbo-MPA for PDMA, Single user MMSE receiver for OFDMA |
| UL power control | Open loop power control with Po=-95dBm and alpha = 1 |
| Channel estimation | Ideal |

The methodology for the simulation is described in [4] and . The users in the simulation are assumed to adopt the grant-free transmission scheme, and collide when they occupy the same RE groups and pattern. For OFDMA, the signal of collided users is modelled as interference. For PDMA, the base station tries to detect those collided users by Turbo-MPA algorithm.  The results are shown in Figure 3.

Figure 3. System level simulation results comparing with OFDMA

In the results, at 1% packet dropping rate, the packet arrival rate is about 300 packet/s/cell/MHz for OFDMA and about 980 packet/s/cell/MHz for PDMA, i.e. PDMA provides 3.26 times packet arrival rate compared with OFDMA.

Conclusion

In this contribution, we discuss the PHY abstraction methodology of non-orthogonal MA schemes. Because the non-linear detector in non-orthogonal MA brings challenges to PHY abstraction, the PHY abstaction methodology should be discussed for the alignment of system level simulation. The L2S results and mMTC SLS results are also given. From simulation results, it is observed that PDMA can support more connections than OFDMA under the scenario of mMTC.

References

- RAN1 #85meeting chairman notes.

- TR 36.866, “Study on network-assisted interference cancellation and suppression (NAICS) for LTE (Release 12)” v12.0.1

- IEEE 802.16m, “Evaluation methodology document (EMD)”

- R1-166468, “ Remaining issues on evaluation assumption and methodology of MA”, CATT