3GPP TSG RAN WG1 Meeting #86					R1-166893

Gothenburg, Sweden 22nd - 26th August 2016

Agenda Item:	8.1.4.1

Source: 	LG Electronics

Title: 	Polar code design

Document for:	Discussion and decision

- Introduction

In last RAN1#84bis meeting, the study on several channel coding schemes were agreed [1] as follows:

Coding candidates:

- Identified channel coding schemes for each usage scenario

| eMBB | mMTC | URLLC |
| --- | --- | --- |
|  | Convolutional codes | Convolutional codes |
| LDPC | LDPC | LDPC |
| Polar | Polar | Polar |
| Turbo | Turbo | Turbo |

- Common simulation assumptions are required to evaluate theoretical performance of proposed coding schemes

- Selection of the coding scheme should also consider various other aspects

Initial Simulation Assumptions:

- Focus mainly on the BLER performance of candidate coding schemes.

- Evaluate performance of coding schemes with similar code rates and block sizes.

- Exact code constructions should be provided.

  - Example: Parity check matrices, polar code construction, ..

- Encoding/decoding complexity of the adopted algorithms should be described.

In this contribution, we discuss polar code design.

Polar code design

Polar codes employ channel polarization property that channel can be classified into two categories as codeword length goes to infinity, so called, noise free and noisy bit channels [2]. When the data would be assigned to noise free channel, the channel capacity can be achieved. The codeword length of polar codes is restricted to N=2x, where x is a non-zero integer number. Therefore, the rate matching (e.g., puncturing/repetition pattern) should be carefully designed to support various codeword lengths depending on MCS and the resource allocation. For example, assuming TBS+CRC of 400 bits, a (800, 400) polar codeword having coding rate of 1/2 can be achieved by puncturing 224 bits of (1024, 400) polar code and by repeating 288 bits of (512, 400) polar code, respectively, where (n, k) denotes a codeword length of n and information block size (e.g., TBS+CRC) of k. A puncturing pattern, so called, quasi-uniform puncturing (QUP) was mentioned in [3][4]. We evaluate the performance of polar code when QUP is assumed in rate matching. The evaluation assumptions are summarized in table 1. For comparison, we also evaluate the performance of polar code when random puncturing is assumed in rate matching.

Table 1 Evaluation assumptions.

| Channel | AWGN |
| --- | --- |
| Information block size including CRC bits | 170 |
| Modulation | QPSK |
| Codeword length | 384 punctured from 512 |
| Decoding algorithm | List decoding |
| Puncturing pattern | QUP, random puncturing |

Figure 1 Performance of polar code considering rate matching (QUP and random puncturing).

The evaluation results are shown in figure 1. As can be seen in figure 1, the performance difference between QUP and random puncturing is not so significant. However, performance of QUP becomes poor when the amount of puncturing is large and list size is 1.

Observation 1: Random puncturing and QUP shows similar performance except list size of 1. QUP performance becomes poor when the amount of puncturing is large and list size is 1.

In addition, the puncturing may impact the channel polarization property and may result in performance degradation when comparing with performance without puncturing. In this evaluation, (384, 170) polar code obtained by random puncturing (512, 170). In addition (512, 227) and (256, 113) without puncturing are considered for comparison. Note that 384 is the median number between 256 and 512, which implies the amount of puncturing and repetition is same. Two polar codes have the same code rate, 227/512=113/256=170/384=0.443. We use the same assumptions in table 1 except information size and codeword length. The performance comparison is shown in figure 2. Unlike the performance of turbo code, the performance of polar code may be sensitive to the amount of puncturing. As can be seen in figure 2, the performance degradation by puncturing is about 0.6 dB assuming list size of 32 even for the same code rate. The performance gap becomes larger when list size increses. That is, polar code obtained without puncturing shows better performance than that obtained by puncturing for a given code rate.

Observation 2: The polar code performance is dependent on the amount of puncturing for a given code rate.

Figure 2 Performance comparison of polar codes with and without puncturing.

:

- Conclusions

We discuss the puncturing aspects of polar code. We have following observations:

Observation 1: Random puncturing and QUP shows similar performance except list size of 1. QUP performance becomes poor when the amount of puncturing is large and list size is 1.

Observation 2: The polar code performance is dependent on the amount of puncturing for a given code rate.

- Reference

[1] RAN1#84 bis Chairman’s notes.

[2] E. Arikan, “Channel polarization: A method for constructing capacity achieving codes for symmetric binary-input memoryless channels,” IEEE Trans. Inform. Theory, vol. 55, pp. 3051-3073, July 2009.

[3] R1-164039, “Polar codes - encoding and decoding,” Huawei, HiSilicon.

[4] R1-164699, “Polar code design overview,” Qualcomm Incorporated.