3GPP TSG RAN WG1 Meeting #86	R1-166930

Gothenburg, Sweden, 22-26 August 2016

Source:	Ericsson

Title:	Turbo Code for NR Scenarios

Agenda Item:	8.1.4.1

Document for:	Discussion and Decision

# Introduction

In RAN#71, a new study item, “Study on New Radio Access Technology,” has been approved. The initial work of the study item is expected to focus on fundamental physical layer signal structure for new RAT, of which channel coding schemes is listed as an area to investigate.

In RAN1#84bis meeting, simulation assumptions were agreed for the eMBB, URLLC, and mMTC scenarios. In RAN1#85 meeting, the following observations were made:

Observations:

- At least in AWGN channels:

  - For large information block sizes, all candidate channel coding schemes show comparable link performance

In this paper, we further investigate the performance of candidate channel coding schemes for relatively short information block sizes.  The information block sizes and code rates agreed for URLLC and mMTC are used in the study. That is:

- Code rate:  1/12, 1/6, 1/3

- Info. block length*** (bits w/o CRC): 20, 40, 200, 600, 1000

Clearly the observation obtained in this study applies whenever short information block lengths and low code rate are used, including those in eMBB scenario.

Our study shows that turbo codes, LDPC codes, and Polar codes demonstrate comparable link performance for short information block sizes as well.

# Performance Comparison of Turbo Code vs. Polar Code for Short Block Lengths

In this section, we present a comparison between Turbo codes and Polar Codes short block lengths such as those used for URLLC, MMTC scenarios as well as certain eMBB conditions.

The LTE turbo mother code rate Rm=1/3 is investigated here, and it is called “LTE turbo code”. The mother code rate Rm = 1/5 enhanced LTE as discussed in [8], and repeated in the appendix, are also used in the study; and it is called “Enhanced Turbo code”. For the rate 1/3 simulation, the Rm =1/5 Enhanced Turbo is reduced to the LTE turbo code.

In the comparison, Turbo code does not use CRC whereas Polar code uses Successive Cancelation (SC) List decoding of list size 32, aided with CRC decoding. The LTE 16-bit CRC is used.

## R=1/3

Figure 1 shows the performance comparison of Turbo and Polar codes for the case R=1/3. We observe that the two codes have comparable performance. Turbo code outperforms Polar codes for k=20 due to the high CRC overhead attached to the Polar code.

The comparable performance is observed despite of the larger codeword used by Polar code. Larger codeword means that larger memory is required to store the soft channel bits. Larger codeword also means the decoding is done with relatively higher complexity. As shown in Table 1, the polar decoder requires 1.1 – 1.7 times as much soft channel bits storage, and decoding of a correspondingly large codeword.

Table 1. Code sizes for rate 1/3.

| Info block size (bits) | 20 | 40 | 200 | 600 | 1000 |
| --- | --- | --- | --- | --- | --- |
| Codeword size (bits) of Turbo | 60 | 120 | 600 | 1800 | 3000 |
| Codeword size (bits) of Polar | 64 | 128 | 1024 | 2048 | 4096 |
| (Codeword size of Polar ) / (Codeword size of turbo) | 1.1 | 1.1 | 1.7 | 1.1 | 1.4 |

Figure 1. Turbo code vs. Polar code performance comparison for rate R=1/3 and QPSK.

## R=1/6

In Figure 2, performance curves are shown for Rm=1/3 LTE Turbo code and Polar L=32 SCL decoding. Figure 2 shows the case of R=1/6. For k=20, LTE Turbo code outperforms Polar code due to the CRC overhead in Polar code. For larger k, Polar code with list size =32 is slightly better than the LTE Turbo code.

Figure 2. Turbo code vs. Polar code performance comparison for rate R=1/6 and QPSK

Figure 3 shows the performance comparison between Rm=1/5 Enhanced Turbo code and Polar code for the case R=1/6. We observe comparable performance of the two codes.

The comparable performance is observed despite of the repetition used in turbo code to achieve R=1/6, as compared Polar codes where no repetition is used. As shown in Table 2, the polar decoder requires 1.3 – 2 times as much soft channel bits storage, and decoding of a correspondingly large codeword.

Table 2. Code sizes for rate 1/6.

| Info block size (bits) | 20 | 40 | 200 | 600 | 1000 |
| --- | --- | --- | --- | --- | --- |
| Codeword size (bits) of Rm=1/5 Enhanced Turbo | 100 | 200 | 1000 | 3000 | 5000 |
| Codeword size (bits) of Polar | 128 | 256 | 2048 | 4096 | 8192 |
| (Codeword size of Polar ) / (Codeword size of Rm=1/5 Enhanced Turbo) | 1.3 | 1.3 | 2.0 | 1.4 | 1.6 |

Figure 3. Enhanced Turbo code vs. Polar code performance comparison for rate R=1/6 and QPSK. For Turbo code, mother code of rate 1/5 is used.

## R=1/12

In Figure 4, performance curves are shown for Rm=1/3 LTE Turbo code and Polar L=32 SCL decoding. Figure 4 shows the performance comparison for rate R=1/12. For k=20, the two codes have comparable performance whereas for larger k, Polar code outperforms LTE Turbo code.

Since the mother code rate of LTE Turbo code is Rm = 1/3, the code rate 1/12 is reached by 4 times repetition of the LTE codeword. For polar code, a codeword of dedicated rate 1/12 is used without repetition. This leads to the performance difference between LTE Turbo code and Polar code.

Figure 4. Turbo code vs. Polar code performance comparison for rate R=1/12 and QPSK

Figure 6 shows the performance comparison between Rm=1/5 Enhanced Turbo code vs Polar code for rate R=1/12. Again, the two codes show comparable performance for smaller k. Since the mother code rate of Enhanced Turbo code is 1/5, hence the code rate 1/12 is reached by repetition beyond 1/5.

Repetition leads to degraded code performance of turbo, as compared non-degraded performance of Polar codes since no repetition is used. On the other hand, repetition allows much less soft channel bits storage and simpler decoder. As shown in Table 3, the polar decoder requires 2.6 – 4.1 times as much soft channel bits storage, and decoding of a correspondingly large codeword.

Table 3. Code sizes for rate 1/12.

| Info block size (bits) | 20 | 40 | 200 | 600 | 1000 |
| --- | --- | --- | --- | --- | --- |
| Codeword size (bits) of Rm=1/5 Enhanced Turbo | 100 | 200 | 1000 | 3000 | 5000 |
| Codeword size (bits) of Polar | 256 | 512 | 4096 | 8192 | 16384 |
| (Codeword size of Polar ) / (Codeword size of Rm=1/5 Enhanced turbo) | 2.6 | 2.6 | 4.1 | 2.7 | 3.3 |

Figure 5. Enhanced Turbo code vs. Polar code performance comparison for rate R=1/12 and QPSK.

# Performance Comparison of Turbo Code vs. LDPC Code for Short Block Lengths

Next two plots show the comparison between Rm=1/5 Enhanced Turbo code and LDPC code. The turbo decoding is max-log-MAP with scaling factor = 0.75. The LDPC decoder is sum-product with 30 iterations. It should be noted that practical LDPC decoder based on min-sum will experience a performance degradation compared to what’s shown here.

Figure 6.  Turbo code vs. LDPC performance comparison for rate R=1/3 and QPSK.

Figure 7.  Turbo code vs. LDPC performance comparison for rate R=1/5 and QPSK

# Conclusion

In this contribution, we discussed potential enhancements to the LTE turbo code. We also present performance comparison between Rm=1/3 Turbo code vs Polar codes and Rm=1/5 Enhanced Enhanced Turbo code vs Polar code, for short block lengths.  Based on the discussion, we have the following observation and proposal:

- For short information block lengths and rate between 1/3 and 1/12, Turbo code with mother code rate 1/5 achieve comparable performance as Polar codes

- For short information block lengths and rate between 1/3 and 1/12, Turbo code with mother code rate 1/5 achieve better performance than LDPC codes.

- Adopt Rm=1/5 Enhanced Turbo codes for error protection of short block lengths for eMBB, URLLC, and MMTC.

# References

RP-160671, “New SID Proposal: Study on New Radio Access Technology,” NTT DOCOMO, 3GPP TSG RAN Meeting #71, Göteborg, Sweden, 7.-10. March, 2016.

3GPP TS 36.212 v13.0.0.

R1-164356, “Performance Evaluation of TBCC and Polar Codes,” Ericsson.

R1-164357, “Analysis of Candidate Code Types for Short Block Length,” Ericsson.

R1-164358, “Performance Evaluation of Turbo Codes and LDPC Codes at Lower Code Rates,” Ericsson.

R1-164359, “Performance Evaluation of Turbo Codes and LDPC Codes at Higher Code Rates,” Ericsson.

R1-164360, “Analysis of Candidate Code Types for Long Block Length,” Ericsson.

R1-164361, “Turbo Code Enhancements,” Ericsson.

R1-165036, “Impact of Number of Iterations in Decoding Turbo and LDPC codes,” Ericsson.

# Appendix. Enhanced Turbo Code with Lower Code Rate R=1/5

In LTE, the turbo code has mother code rate 1/3 with transfer function of

G(D) =

To lower the mother code rate to 1/5, a second parity bit is added for each of the two constituent codes, so that 5 bits are generated for each info bit (here the tail effect is ignored).  The transfer function of the 8-state constituent code for the PCCC is modified to:

G(D) =

where

g0(D) = 1 + D2 + D3,

g1(D) = 1 + D + D3,

g2(D) = 1 + D+ D2 + D3.

Note that this is the same transfer function adopted by 3GPP2. The rate 1/5 turbo code structure is illustrated in Error! Reference source not found., where yk and y k correspond to g2(D).

A simple and backward compatible extension to the circular buffer to support the rate 1/5 code can be adopted correspondingly. That is, simply append the existing circular buffer with the multiplexed bits of g2(D) from the two component codes. More specifically, the composition of the extended circular buffer is as follows:

- The first part contains 32 columns of systematic bits xk;

- The second part contains 64 columns of bit-multiplexed coded bits from g1(D) of the two component codes, zk and z k.

- The third part contains 64 columns of bit-multiplexed coded bits from g2(D) of the two component codes, yk and y k.

Figure 6. Structure of rate 1/5 turbo encoder (dotted lines apply for trellis termination only).