3GPP TSG RAN WG1 Meeting #86	 	R1-167007

Gothenburg, Sweden, 22-26 August 2016

Source: 	Ericsson

Title:	PSBCH Contents for V2V

Agenda Item:	7.2.2.7

Document for:	Discussion and Decision

# Introduction

This paper discusses the PSBCH contents for V2V operation. A companion contribution provides the details of the synchronization protocol [1].

# PSBCH Contents

We propose the following contents for the V2X PSBCH:

- DFN: 14 bits = 10 bits counter + 4 bits offset

- TDD UL-DL config: 3 bits:

  - In case of FDD, this field is set to 000, purely for the purpose of decoding of PD2DSCH and does not imply any other UE behaviour

  - The UE is assumed to know a priori the duplex mode of the carrier

- In-coverage indicator: 1 bit

- Sidelink system bandwidth: 3 bits

- Reserved field: FFS bits set to a SIB-signalled or preconfigured value in V2X PSBCH.

The contents are identical to Rel-12 except for the length of the reserved field. Due to the reduction in REs available to PSBCH in V2X compared to Rel-12 we suggest to re-evaluate the length of the reserved field in order to meet a reasonable link budget requirement.

Proposal:

- V2X PSBCH content is the same as Rel-12 V2X.

- The length of the reserved field should be adjusted according to the link budget target.

# Conclusions

In this contribution we discuss PSBCH contents:

Proposal:

- V2X PSBCH content is the same as Rel-12 V2X.

- The length of the reserved field should be adjusted according to the link budget target.

# References

[1] R1-166960,	Distributed Synchronization Procedure for V2X over PC5,	Ericsson