3GPP TSG-RAN WG1 #86 	R1-167604

22nd – 26th Aug 2016

Gothenburg, Sweden

Agenda item:	8.1.2.1

Source: 	Qualcomm Incorporated

Title: 	Evaluation of OFDM with Companding

Document for:	Discussion/Decision

# Introduction

It was approved in [1] that NR will study the following waveforms:

- Waveform based on OFDM, with potential support of non-orthogonal waveform and multiple access

  - FFS: other waveforms if they demonstrate justifiable gain

It was also proposed [2][3] that optional DFT spreading technique (i.e. SC-FDM waveform), as used in 4G LTE uplink, be also supported in NR uplink when there is link budget or coverage limitation. Further, it was agreed in RAN1-85 [4]

- The following OFDM-based waveforms should be used as RAN1 NR waveform performance reference:

  - OFDM with CP

  - DFT-s-OFDM with CP

- All waveform in RAN1 #84bis/#85 meeting can be evaluated based on agreed assumptions

  - Note: Each company should provide details on the DFT-spreading, guard interval, Tx/Rx filtering and/or windowing applied to OFDM waveform for evaluation

Notice that the selection of SC-FDM or OFDM waveform is independent of the spectral containment techniques, such as windowing or filtering, that have been discussed in [2]~[7].

We present detailed OFDM and SC-FDM waveform comparison in [10], where we show a significant (~2dB) performance gain of SC-FDM vs. OFDM for link budget limited users. This is mainly due to the large PAPR reduction from SC-FDM compared with OFDM. One proposal to improve OFDM PAPR is to use companding.

In this contribution, we evaluate OFDM waveform with implementation-based PAPR reduction techniques (such as companding), and demonstrate the performance advantage of using SC-FDM waveform in terms of better PA efficiency and better coverage, while achieving similar ACLR and demodulation performance. Based on the detailed analysis, we propose to adopt SC-FDM for eMBB uplink at least for coverage limited scenarios.

# Evaluation of PAPR Reduction Techniques

In this section, we discuss a few PAPR reduction techniques for OFDM waveform.

## Companding Techniques

We study the OFDM system with the following companding techniques:

### Airy function based compander

The companding function is given by [8]:

The decompanding/expander function is given by

### Piecewise-linear

Here we choose a piecewise-linear companding function with 2 points of non-linearity.

Figure 1: Piecewise linear compander

The expander function will be another piecewise-linear function with 2 points of non-linearity.

### µ-law compander

Here the companding function [9] is given by:

And the expander is given by:

## ACLR performance

We study the ACLR performance of the companding schemes and compare against the performance of SC-FDM. The setup for simulation study is as follows for the OFDM system.

Figure 2: Setup for ACLR measurement

In this study we optimised the compander parameters so that the input power (Pin) to PA is maximized while achieving 30 dB ACLR.  The piecewise linear model was not optimized due to the large number of parameters to be optimized. The PA model used in the study is a polynomial PA model. The results are tabulated below:

Table 1: Power back-off for 30 dB ACLR without WOLA

| Scheme | Input power (dBm) to PA to attain 30 dB ACLR | Power Backoff (dB) from SCFDM |
| --- | --- | --- |
| SC-FDM | -1.30 | 0 |
| OFDM without companding | -3.75 | 2.45 |
| ### OFDM with airy compander | -3.55 | 2.25 |
| ### OFDM with piece-wise linear compander | -4.20 | 2.90 |
| ### OFDM with µ-law compander | -3.50 | 2.20 |

Table 2: Power back-off for 30 dB ACLR with WOLA (wolalength 2% of FFTSize)

| Scheme | Input power (dBm) to PA to attain 30 dB ACLR | Power Backoff (dB) from SC-FDM |
| --- | --- | --- |
| SCFDM | -1.10 | 0 |
| OFDM without companding | -3.50 | 2.40 |
| ### OFDM with airy compander | -3.40 | 2.30 |
| ### OFDM with piece-wise linear compander | -3.90 | 2.80 |
| ### OFDM with µ-law compander | -3.30 | 2.20 |

## Demodulation loss

The simulation setup for the demodulation loss is illustrated in the following figures. We use a SISO system. The transmitter side setup is as follows:

Figure 3: Demodulation study, transmitter side

The receiver side setup is as:

Figure 4: Demodulation study, receiver side

Table 3: Simulation parameters

| Tx antennas | 1 |
| --- | --- |
| Rx antennas | 1 |
| Code | Turbo, Rate ½ , block length 1944 |
| Modulation | QPSK |
| Systemtones | 600 |
| FFT Size | 2048 |
| Wolalength | 0 |

Note that the power amplifier is not included in the simple simulation setup, we offset the SNR axis of the codeblock error rate (CBER) curves by the power back off obtained in [Table 1: Power back-off for 30 dB ACLR], to take into account the effect of ACLR. The simulations also include the cases with using compander at the transmitter, but not using expander at the receiver.

Figure 5: BLER for SinglePath channel

Figure 11: BLER for EPA channel

Figure 12: BLER for EVA channel

Figure 13: BLER for ETU channel

# Conclusions

Based on the above observations, we have the following observation and proposal:

Observation 1: SC-FDM provides 2 dB improvement in link budget for coverage limited users compared to OFDM with companding.

Proposal 1: NR should support SC-FDM for uplink in order to meet the same coverage as current LTE deployment.

More detailed comparison of OFDM vs. SC-FDM is provided in [10].

# References

- 3GPP RP-160351 “New SID proposal: study on next generation new radio access technology”, NTT Docomo, CMCC, Ericsson, Huawei, Intel, Nokia Networks, Qualcomm, Samsung. Sweden, March 7-11, 2016

- 3GPP R1-162199 “Waveform candidates”, Qualcomm.

- 3GPP R1-162177 “Waveform and multiple access for 5G new radio interface”, Samsung.

- 3GPP RAN1 “Chairman’s notes RAN1_85”, Nanjing, May, 2016.

- Abdoli, Javad, Ming Jia, and Jianglei Ma. "Filtered OFDM: A new waveform for future wireless systems." Signal Processing Advances in Wireless Communications (SPAWC), 2015 IEEE 16th International Workshop on. IEEE, 2015.

- Vakilian, Vida, et al. "Universal-filtered multi-carrier technique for wireless systems beyond LTE." Globecom Workshops (GC Wkshps), 2013 IEEE. IEEE, 2013.

- Wild, Thorsten, and Frank Schaich. "A Reduced Complexity Transmitter for UF-OFDM." Vehicular Technology Conference (VTC Spring), 2015 IEEE 81st. IEEE, 2015.

- Y. Jiang, "New companding transform for PAPR reduction in OFDM," IEEE Communications Letters, vol. 14, no. 4, pp. 282-284, April 2010.

- H. Sakran, M. Shokair and A. Abou Elazm, "A new peak-to-average power reduction technique in the OFDM system using μ-law compander," 2008 IEEE International Symposium on Wireless Communication Systems, Reykjavik, 2008, pp. 386-390.

- 3GPP R1-166354, “OFDM and SC-FDM support for Uplink”, Qualcomm.