3GPP TSG RAN WG1 Meeting #86bis	R1-1609057

Lisbon, Portugal, 10th – 14th October 2016

Agenda item:	8.1.2.2

Source: 	Samsung

Title: 	Evaluation results of eMBB with HARQ retransmission in case of multiplexing eMBB and URLLC

Document for:	Discussion and Decision

Introduction

In last RAN1#86 meeting, there was discussion on multiplexing of eMBB and URLLC and following agreements were achieved [2]:

Agreements:

- At least the following potential options should be considered

  - At least for shorter transmission UL, semi-static resource sharing between URLLC and eMBB

    - FDM and/or TDM manner

      - UL grant-free transmission for URLLC

      - Other schemes are not precluded

  - Dynamic resource sharing between URLLC and eMBB

    - For DL, mechanisms to schedule a transmission where the resources of it can overlap with resources of ongoing/scheduled longer transmission at least from network perspective

      - FFS: A similar or same mechanism applicability to UL

      - Preemption or superposition

      - Other schemes are not precluded

    - Scheduling based approaches (e.g., by adapting transmission duration or by using different subbands) to allow multiplexing of different durations of transmission

    - UL grant-free transmission for URLLC

    - Other schemes are not precluded

  - Other mechanisms are not precluded

In this contribution, we provide error performance of eMBB with HARQ retransmission in DL in case of preemption-based dynamic resource sharing between eMBB and URLLC.

Discussions

In this section, preemption-based dynamic resource sharing between eMBB and URLLC is briefly described and then, block error rate (BLER) performance of eMBB with HARQ retransmission is provided.

  - Preemption-based dynamic resource sharing

In last RAN1 #86, preemption-based multiplexing between eMBB and URLLC was introduced as one of alternatives for dynamic resource sharing. As discussed in [3], when URLLC traffic is occurred during eMBB transmission, this scheme allows puncturing eMBB data transmitted in the overlapped resource region with URLLC data and URLLC data is transmitted as shown in Figure 1.

Figure 1: Example of puncturing of eMBB data for URLLC

In this case, gNB cannot always provide information on which region should be punctured (e.g., puncturing information) to the eMBB receiver because control information for eMBB data can be transmitted before URLLC traffic is occurred. If the eMBB receiver does not know the puncturing information, eMBB receiver has to perform channel decoding using wrong received signals incurred by URLLC data. So, eMBB could suffer from drastic degradation in error performance. In this case, HARQ retransmission and combining method can be considered as one of alternatives for improving eMBB error performance. If eMBB code block error is occurred, gNB can perform the HARQ retransmission and the eMBB receiver can perform combining between previous received signals and new ones. However, if the eMBB receiver has no puncturing information for previous received signals, this HARQ retransmission and combining scheme may not be able to effectively compensate eMBB performance degradations incurred by multiplexing eMBB and URLLC because wrong received signals incurred by URLLC in the previous reception can affect the HARQ combining.

  - Impact on eMBB BLER

In this subsection, BLER performance of eMBB with HARQ retransmission is evaluated in case of preemption-based multiplexing between eMBB and URLLC. For evaluating eMBB performance, it is assumed that eMBB and URLLC employ same subcarrier spacing (15 kHz) and eMBB’s TTI consists of 14 OFDM symbols and URLLC TTI spans 2 OFDM symbol. Here, gNB punctures 2 OFDM symbols of eMBB which is overlapped with URLLC. If eMBB receiver fails to receive eMBB code blocks, gNB performs the HARQ retransmission, and the eMBB receiver can perform combining between previous received signals and new ones. Here, it is assumed that retransmitted eMBB signals are not punctured. Detailed simulation parameters are shown in Appendix.

Figure 2: BLER performance of eMBB

Figure 2 shows BLER performance of eMBB with HARQ retransmission for QPSK and 16QAM modulation. Here, it is assumed that the eMBB receiver has no pu;ncturing information for the first transmission. As shown in Figure 2, if the eMBB receiver has no puncturing information for the first transmission, eMBB BLER performance can be quite degraded compared with no URLLC case even though retransmitted eMBB signals are not punctured. For example, for the case of SNR=3dB in Figure 2(a) which is 10% BLER of no URLLC case, the eMBB receiver without puncturing information can provide only 40% BLER even though the HARQ retransmission and combining is performed two times and retransmitted eMBB signals are not punctured.

Observation 1: In case of preemption-based multiplexing between eMBB and URLLC, BLER performance of eMBB with HARQ retransmission can be quite degraded compared with no URLLC case if the eMBB receiver has no puncturing information for the first transmission.

Also, Figure 3 shows eMBB BLER performance for the case when the eMBB receiver can aware of wrong received signals incurred by URLLC in the previous received signals and these received signals are not used in HARQ combining process. The results demonstrate that the eMBB receiver with HARQ combining based on puncturing information for the first transmission can provide better performance compared with no URLLC case. These results indicate that wrong received signals from the punctured eMBB resource regions in the previous reception can adversely affect HARQ retransmission and combining. Therefore, for preemption-based dynamic resource sharing between eMBB and URLLC, methods on how to inform the eMBB receiver its puncturing regions in previous received signals should be studied.

Figure 3: BLER performance of eMBB

Observation 2: If an eMBB receiver can aware of punctured regions in previous received signals at the current reception, BLER performance of eMBB with HARQ can be dramatically improved.

Proposal 1: Study methods on how to inform the eMBB receiver its puncturing regions in previous received signals in case of preemption-based dynamic resource sharing between eMBB and URLLC.

Conclusions

In this contribution, error performance of eMBB with HARQ retransmission and combining in DL is evaluated in case of preemption-based dynamic resource sharing between eMBB and URLLC. It can be summarized as below.

Observation 1: In case of preemption-based multiplexing between eMBB and URLLC, BLER performance of eMBB with HARQ retransmission can be quite degraded compared with no URLLC case if the eMBB receiver has no puncturing information for the first transmission.

Observation 2: If an eMBB receiver can aware of punctured regions in previous received signals at the current reception, BLER performance of eMBB with HARQ can be dramatically improved.

Proposal 1: Study methods on how to inform the eMBB receiver its puncturing regions in previous received signals in case of preemption-based dynamic resource sharing between eMBB and URLLC.

- References

- RP-160671, New SID Proposal: Study on New Radio Access Technology

- Draft Report of 3GPP TSG RAN WG1 #86 v0.1.0.

- R1-166759, Discussion on URLLC support in NR, Samsung

- Appendix – Link level simulation setup

# Table.1. LLS Evaluation parameters

| Parameters | Values or assumptions |
| --- | --- |
| Carrier Frequency | 2 GHz |
| Waveform | OFDM |
| Channel coding | LTE Turbo code |
| Numerology | Same as Release 13 |
| System Bandwidth | 20 MHz |
| Modulation of eMBB | QPSK, 16QAM |
| Code rate of eMBB | 0.5 |
| Modulation of URLLC | QPSK |
| BS antenna configuration | 1Tx |
| UE antenna configuration | 2Rx |
| Channel estimation | Perfect channel estimation |
| Propagation channel & UE velocity | TDL-C with DS{300}ns & 3km/h in TR38.900 |
| Number of HARQ retransmissions | 1 or 2 |
| Combining method for HARQ retransmission | Chase combining |
| TTI of eMBB | 14 OFDM symbols duration |
| TTI of URLLC | 2 OFDM symbols duration |
| Number of OFDM symbols for PDCCH | 3 OFDM symbols |