3GPP TSG RAN WG1 #86bis	R1-1609073

# Lisbon, Portugal, 10th – 14th Oct. 2016

Agenda item:	8.1.3.1

Source: 	Samsung

Title: 	Discussion on latency of channel codes for NR

Document for:	Discussion and Decision

Introduction

One of the requirements for NR channel coding scheme(s) is latency [1].

In this contribution, we discuss the latency requirement on channel decoder for data channel and control channel based on the special subframe/slot, so-called self-contained subframe/slot structure.

Latency Requirement

  - Data Channel Coding

It has been discussed the feasibility of the special subframe type with fast HARQ-ACK, as known as “self-contained structure” where HARQ ACK/NACK is transmitted over a guard period in the same subframe/slot as shown in Figure 1 [2]. Even if the self-contained structure is introduced in the specification, the actual support of it may depend on UE capability and so on. However, from the decoding latency perspective, this structure is the worst case. So, we discuss the latency requirements based on this frame structure.

Figure 1: Self-contained structure for downlink

As shown in Figure 2, self-contained subframe/slot consists of DL data part (yellow), guard period (white) and UL control part (blue). In order to support fast HARQ ACK/NACK, the time required for decoding NR PDSCH (1) and the time required for encoding NR PUCCH (2) including round trip delay should be less than guard period (GP). Here, the decoding time of NR PDSCH (1) contains processing time needed for channel estimation, demodulation and channel decoding.

Figure 2: Timing analysis for self-contained approach

We use some notations and assumptions as follows:

- TRTD : Round trip delay (6.67 µs with the assumption of 1km cell radius)

- TGP : Guard period (one OFDM symbol)

- TD : Decoding time

- TDM : Demodulation time

- TCE : Channel estimation time

- TEN : Encoding time

If it is assumed that the channel decoding time (TD) is half of the time needed for NR PDSCH decoding (=TD +TMD + TCE) and NR PUCCH encoding (TEN), then decoding time (TD) should satisfy the condition below:

TD ≤ (TGP - TRTD )/2

With the above assumptions, the maximum allowable decoding time (TD) for each subcarrier spacing fsc is shown in Table1.

Table 1:  Decoding latency requirements

| fsc (kHz) | Guard Period (µs) | Decoding time (µs) |
| --- | --- | --- |
| 15 | 67 | 30.16 |
| 30 | 33 | 13.16 |
| 60 | 16 | 4.66 |
| 120 | 8 | 0.665 |

  - Control Channel Coding

In “self-contained structure”, UL data is transmitted in the same subframe/slot with the corresponding DL control transmissions, as illustrated Figure 3.

Figure 3: Self-contained structure for uplink

As shown in Figure 4, self-contained subframe/slot consists of DL control part (blue), guard time (white) and UL data part (yellow). In order to support fast transmission of UL data, the time required for decoding NR PDCCH (1) and encoding NR PUSCH (2) including round trip delay should be less than the guard period (GP). Here, the decoding time of NR PDCCH (1) includes processing time needed for channel estimation, demodulation and channel decoding.

Figure 4: Timing analysis for self-contained approach

In order to drive the latency requirement for control channel coding, the number of blind decoding (TBD) for the control channel should be considered. Here, TBD = 44 is assumed. Suppose that channel decoding time (TD) is half of the time use for NR PDCCH decoding (=TD + TMD + TCE) and NR PUSCH encoding (TEN), then decoding time (TD) should satisfy the condition below:

TD ≤ (TGP - TRTD )/( TBD+1)

With the above assumptions, the maximum allowable decoding time (TD) for each subcarrier spacing fsc are given in Table 2.

Table 2:   Decoding latency requirements

| fsc (kHz) | Guard Period (µs) | Decoding Time (µs) |
| --- | --- | --- |
| 15 | 67 | 1.34 |
| 30 | 33 | 0.58 |
| 60 | 16 | 0.2 |
| 120 | 8 | 0.03 |

Decoding Latency of Channel Coding

  - Data Channel Coding

In this section, we summarize the decoding latency of turbo, LDPC, and polar decoders introduced in [3].

Turbo Code

Notation for our calculation is given below.

- I: iteration number

- K: number of information bits

- r: radix realization of the recursion units

- P: number of APP decoders

Since there are P APP (a posteriori probability) decoders in one turbo decoder, one APP decoder handles K/P bits. If we adopt radix-2 and radix-4 decoding algorithms, 1-bit and 2-bit can be processed in one clock cycle, respectively. When employing a non-sliding window (NSW) decoding, the decoding latency of the turbo decoder is given as follows [4]:

Turbo decoding latency ~ 2∙I∙(K/P)/log2r [clock cycles]

LDPC Code

The row-parallel architecture [5], [6], [7], [8] provides a high throughput up to tens of Gbps, while its routing complexity can still be kept low, permitting a high energy and area efficiency. To enhance the throughput, we reduce the number of effective row blocks by using row-merging technique and apply dual frame processing to improve the efficiency [5], [9]. When the number of effective row blocks is L (= the number of layers in layered decoding) and the number of decoding iteration is I, the latency can be obtained as follows [5], [9]:

LDPC decoding latency ~ L∙I∙Ns clock cycles,

where Ns denotes the number of processing clocks at each layered decoding step.

Polar Code

Polar codes can be decoded by successive cancellation (SC) decoding or improved by SC list (SCL) decoding. Notation for our calculation is given below.

- N: block length of mother polar code (the power of two)

- P: parallel processors

The minimized latency achievable by P parallel processors of SC decoder (a simple two input check/variable node processor) is

SC decoding latency=2N+NPlogN4P≥2N-2 [clock cycles]

In the SCL decoding, the decoder selects L best paths from 2L paths forking from the previous L most-likely paths where L is the list size. So the latency due to path metric sorting should be taken into account as

SCL decoding latency=2N+NPlogN4P+Lsort≥2N-2+Lsort [clock cycles]

We assume the parallel sorter such as radix 2L sorter [10] that can complete a single sorting in one clock cycle whatsoever at the price of complexity increase. Then the decoding latency is given as

SCL decoding Latency≥2N-2+K [clock cycles].

The decoding latency of Turbo/LDPC codes and polar can be reduced by less number of iterations or list size. However, the decoding performance becomes worse for fewer iteration or smaller list size. Thus, both the performance and the latency should be simultaneously considered. Fig. 5-7 show the required SNR at BLER 1% depending on latency.

Considering both the decoding latency and performance, only the LDPC decoder can meet the latency requirements derived in Section 2 for all subcarrier spaces from 15kHz to 120kHz. LTE turbo decoder satisfies the latency requirements only if subcarrier space is less than or equal to 60kHz.

Table 3: Decoder configuration and simulation condition

| Channel |  | AWGN |
| --- | --- | --- |
| Modulation |  | QPSK |
| Decoder Configurations | LTE Turbo Code | Scaled Max Log Map (scaling = 0.75) K = 6144, r = 2, P = 64 |
|  | LDPC code [13] | Offset Min-Sum (Layered; Offset = 0.5) Ns =4, 6-layer for R=8/9, 16-layer for R=1/3 |
|  | Polar code | Sub-optimal, CA-SCL |
| Info. block length (bits w/o CRC) |  | 6000, 1000 |

Figure 5: Performance vs Latency (R=1/3, K=6000)

Figure 6: Performance vs Latency (R=8/9, K=6000)

Figure 7: Performance vs Latency (R=1/3, K=1000)

Observation 1: Summary of condition to meet the decoding latency requirements for eMBB data channel

|  |  |  | LDPC | Turbo | Polar |
| --- | --- | --- | --- | --- | --- |
| K=6000 | R=1/3 | 120 Hz | Marginally satisfy | * | * |
|  |  | 60 Hz | Satisfy | Satisfy |  |
|  |  | 30 Hz |  |  |  |
|  |  | 15 Hz |  |  |  |
|  | R=8/9 | 120 Hz | Satisfy | Marginally satisfy | * |
|  |  | 60 Hz |  | Satisfy |  |
|  |  | 30 Hz |  |  |  |
|  |  | 15 Hz |  |  | Marginally satisfy |
| K=1000 | R=1/3 | 120 Hz | Satisfy | Marginally satisfy | * |
|  |  | 60 Hz |  | Satisfy |  |
|  |  | 30 Hz |  |  |  |
|  |  | 15 Hz |  |  |  |

* could be unsatisfied for some cases

  - Control Channel Coding

The candidate channel coding schemes for eMBB control channel are LDPC code, turbo code, Polar code, and TBCC.

LDPC Code

Since block size of control channel is small on the contrary to data channel, the LDPC decoder can be implemented by fully-parallel architectures [11]. Then,

LDPC Full Parallel Decoding Latency ∼ Ns∙I  [clock cycles]

where I and Ns is the maximum number of iterations and the processing clock cycles for single iteration, respectively.

TBCC

Notation for our calculation is given below.

- I: the number of wrap-rounds

- P: the parallelism level

- L: the length of code block without CRC;

- D : the overlapping length

The minimized latency achievable by P parallel processors of TBCC decoder is

TBCC decoding latency=I∙LP+2∙D[clock cycles].

It is well known that D=30 is sufficient for short/medium code length [12].

Table 4 shows decoding latency for given implementation configuration. It is assumed that the decoder clock frequency is 600 MHz.

Table 4: Decoding latency of channel coding schemes

| Channel Codes | Implementation Configurations | Decoding Latency (clock cycle) | Decoding Latency (µs) |
| --- | --- | --- | --- |
| Turbo | I = 6, K = 100, r = 2, P = 1 | 1200 | 2 |
| LDPC | Full Parallel, I = 40, Ns=2 | 80 | 0.13 |
| Polar | SC, K=100, N = 512, R=1/3 | 1022 | 1.7 |
|  | CA-SCL w/ L=32, K=100, N = 512, R=1/3 | 1122 | 1.88 |
| TBCC | D=30, I=2, L=100, P=4 | 170 | 0.28 |
|  | D=30, I=4, L=100, P=4 | 340 | 0.56 |

Observation 2: Summary of condition to meet the decoding latency requirements for control channel

|  |  |  | LDPC | Turbo | Polar | TBCC |
| --- | --- | --- | --- | --- | --- | --- |
| K=100 | R=1/3 | 120 Hz | * | * | * | * |
|  |  | 60 Hz | Satisfy |  |  |  |
|  |  | 30 Hz |  |  |  | Satisfy |
|  |  | 15 Hz |  |  |  |  |

* could be unsatisfied for some cases

Observations and Proposals

Observation 1: Summary of condition to meet the decoding latency requirements for eMBB data channel

|  |  |  | LDPC | Turbo | Polar |
| --- | --- | --- | --- | --- | --- |
| K=6000 | R=1/3 | 120 Hz | Marginally satisfy | * | * |
|  |  | 60 Hz | Satisfy | Satisfy |  |
|  |  | 30 Hz |  |  |  |
|  |  | 15 Hz |  |  |  |
|  | R=8/9 | 120 Hz | Satisfy | Marginally satisfy | * |
|  |  | 60 Hz |  | Satisfy |  |
|  |  | 30 Hz |  |  |  |
|  |  | 15 Hz |  |  | Marginally satisfy |
| K=1000 | R=1/3 | 120 Hz | Satisfy | Marginally satisfy | * |
|  |  | 60 Hz |  | Satisfy |  |
|  |  | 30 Hz |  |  |  |
|  |  | 15 Hz |  |  |  |

* could be unsatisfied for some cases

Observation 2: Summary of condition to meet the decoding latency requirements for control channel

|  |  |  | LDPC | Turbo | Polar | TBCC |
| --- | --- | --- | --- | --- | --- | --- |
| K=100 | R=1/3 | 120 Hz | * | * | * | * |
|  |  | 60 Hz | Satisfy |  |  |  |
|  |  | 30 Hz |  |  |  | Satisfy |
|  |  | 15 Hz |  |  |  |  |

* could be unsatisfied for some cases

Proposal 1: LDPC codes should be selected for eMBB data channel

Proposal 2: The decoding latency requirement should be considered for control channel coding

- References

- 3GPP RAN1 #84bis, Chairman note

- “Number of HARQ processes,” 3GPP TSG RAN WG1 #86bis, Lisbon, Portugal, 10-14 Oct. 2016.

- R1-166757, Samsung, " Discussion on frame structure and HARQ timing," 3GPP TSG RAN WG1 #86, Gothenburg, Sweden, 22-26 Aug. 2016.

- Y. Sun, J.R. Cavallaro, "Efficient hardware implementation of a highly-parallel 3GPP LTE/LTE-advance turbo decoder," INTEGRATION, the VLSI journal, 2011.

- Y. Park, D. Blaauw, D. Sylvester, Z. Zhang, "Low-Power High-Throughput LDPC Decoder Using Non-Refresh Embedded DRAM," IEEE Journal of Solid-State Circuits, Feb. 2014

- S.-W. Yen, S.-Y. Hung, C.-H. Chen, H.-C. Chang, S.-J. Jou, and C.-Y. Lee, "A 5.79-Gb/s energy-efficient multirate LDPC codec chip for IEEE 802.15.3c applications," IEEE J. Solid-State Circuits, vol. 47, no. 9, pp. 2246–2256, Sep. 2012.

- Z. Zhang, V. Anantharam, M. J. Wainwright, and B. Nikolić, "An efficient 10GBASE-T wthernet LDPC decoder design with low error floors, " IEEE J. Solid-State Circuits, vol. 45, no. 4, pp. 843–855, Apr. 2010.

- A. Cevrero, Y. Leblebici, P. Ienne, and A. Burg, "A 5.35 mm 10GBASE-T Ethernet LDPC decoder chip in 90 nm CMOS, " in Proc. 2010 IEEE Asian Solid-State Circuits Conf., Beijing, China, Nov. 2010, pp. 317–320.

- M. Weiner, B. Nikolic, and Z. Zhang, "LDPC decoder architecture for high-data rate personal-area networks, " in Proc. IEEE Int. Symp. Circuits and Systems, May 2011, pp. 1784–1787.

- A. Balatsoukas-Stimming, M. Parizi, and A. Burg, “LLR-based successive cancellation list decoding of polar codes,” IEEE Trans. Signal Process., vol. 63, no. 19, pp. 5165–5179, Oct. 2015.

- N. Onizawa, T. Hanyu, and V. C. Gaudet, “Design of high throughput fully parallel LDPC decoders based on wire partitioning,” IEEE Trans. VLSI Syst., vol. 18, no. 3, pp. 482–489, Mar. 2010

- R1-164279, ZTE, “Throughput and latency analysis on New channel coding candidates for NR,” 3GPP TSG RAN WG1 #85, Nanjing, China, 23-27 May. 2016.

- R1-167889, Samsung, “Design of Flexible LDPC Codes”, 3GPP TSG RAN WG1 #86, Gothenburg, Sweden, 22th – 26th Aug. 2016