3GPP TSG-RAN WG1 #86-bis  													                           R1-1609540

# Lisbon, Portugal 10th - 14th October 2016

Source: 	Intel Corporation

Title:	Reference signal design for NR downlink control channel

Agenda item:	    8.1.7.1

Document for:	Discussion and Decision

# Introduction

General design requirements for NR physical downlink control channel (PDCCH) are discussed in [1]. Accordingly, required transmission schemes are also discussed in [2]. Associated with these possible transmission schemes, this contribution presents our view on the preferred characteristics of reference signals used for NR PDCCH coherent demodulation. And the reference signal design for NR data channel is presented in [3].

# Discussion

We envision that different reference signals for control and data channels are needed in some scenarios. In LTE, in case of PDCCH scheduling DMRS based PDSCH transmission, CRS is used for PDCCH demodulation while DMRS is used for PDSCH demodulation so that different reference signals are used for control and data demodulation. Such situation may also be needed in NR. For example, to achieve better reliability, distributed NR PDCCH transmission employs certain transmit diversity scheme which may use different beamforming, e.g., broader beam, than that for the scheduled data channel [4], in this case, different DMRS ports for control and data channels would be needed.

Proposal 1: Different DMRS ports between control and data should be supported.

As described in [2], in some cases, localized control and data transmission is preferred in NR for better support of forward compatibility. Moreover, localized NR PDCCH has the advantage to apply DMRS based beamforming transmission. This is of particular importance for NR PDCCH transmitted in high frequency band where beamforming gain has the essential impact on the achievable coverage. In case of UE dedicated control and data transmission, it is plausible that same beamforming can be applied to control and data channels by using same antenna port. Such shared DMRS for data and control channel has the advantage of reducing DMRS overhead, which can be also quite beneficial in those situations where DMRS overhead would have significant impact on the achievable throughput.

Proposal 2: Shared DMRS between control and data channels should be supported.

In some scenarios, UE specific beamforming transmission may not be feasible for some NR PDCCH. For example, when NR PDCCH schedules a common data message intended to multiple UEs, beamformed transmission towards to a particular UE is obviously not preferred to be employed for the NR PDCCH. In this case, to ensure certain coverage requirement from the deployment scenario, some robust transmission schemes to achieve sufficient diversity should be supported for NR PDCCH as well. For such common NR PDCCH which needs to be decoded by multiple UEs, the associated reference signal to support coherent demodulation shall be designed and transmitted in a manner that the associated PDCCH targeting multiple UEs should meet the coverage requirement.

Proposal 3: Shared DMRS among different UEs should be supported.

# Conclusions

Our proposals in this contribution are summarized as follows.

Proposal 1: Different DMRS ports between control and data should be supported.

Proposal 2: Shared DMRS between control and data channels should be supported.

Proposal 3: Shared DMRS among different UEs should be supported.

# References

- R1-1609539, On NR physical downlink control channel, Intel Corporation

- R1-1609541, On transmission schemes of NR physical downlink control channel, Intel Corporation

- R1-1609527, Discussion on DM-RS design for NR, Intel Corporation

- R1-1609519, Discussion on beamforming for NR PDCCH, Intel Corporation