3GPP TSG-RAN WG1 Meeting #86bis	R1-1609673

Lisbon, Portugal, October 10-14, 2016

Agenda Item:	8.1.5.2

Source:	Ericsson

Title:	NR random-access response design

Document for:	Discussion, Decision

# Introduction

A proposed design for the NR random access procedure is described in [1], where the physical design of the PRACH preamble is described in [2]. In this contribution, we discuss Random Access Response (RAR) design. More details about the physical channel format for RAR are given in [3].

# Discussion

The Random Access Response typically has the following payload:

- Detected PRACH preamble index such as to acknowledge of PRACH preamble

- Timing Advance (TA) command to UE

- Uplink scheduling grant to UE

- Temporary identity (e.g. TC-RNTI)

- Configuration of additional synchronization signals if needed (further study needed)

- Back off Indicator (BI) (further study needed)

A UE might have to be able to receive RAR in a somewhat large timing uncertainty interval. Reasons for large timing offset between received SS and RAR (using UE perspective) may be e.g.:

- Narrowband and low SNR for SS

  - only enough downlink timing accuracy for small broadcast

  - Without beamforming of SS, it might be received with low SNR

- Different nodes for SS and RAR

  - Propagation: 1 km corresponds to a round trip time of 3.3 𝜇𝑠. This should be compared to a cyclic prefix of approximately 4 𝜇𝑠

- Non-synchronized nodes

  - ”Loosely” synchronized nodes

  - SFN (Single Frequency Network) type of SS transmission

The RAR might arrive to the UE with a large timing uncertainty

An initial access procedure is illustrated in Figure 1. Here, synchronization signals are transmitted from several nodes (node A and B), and received with a relative timing difference. The UE selects a downlink timing reference based on received SS from node A and transmits a PRACH preamble based in this downlink timing. Due to propagation delay, this PRACH preamble is received with a timing offset relative to SS in node B, which estimates this timing offset and prepares a RAR containing a timing advance (TA) command. This RAR is constructed by a reference signal (RS) and one or several OFDM symbols with data (DS). When the UE receives RAR, it adjusts its uplink timing based upon the TA command. It can also adjust its downlink timing from a DL timing estimated by using the RS included in the RAR.

Figure 1. Uplink and downlink synchronization in initial access procedure

Design RAR with a self-contained reference signal, designed both for DL synchronization and demodulation of the RAR payload

# Conclusion

In section 2 we made the following observations:

Observation 1	The RAR might arrive to the UE with a large timing uncertainty

Based on the discussion in section 2 we propose the following:

Proposal 1	Design RAR with a self-contained reference signal, designed both for DL synchronization and demodulation of the RAR payload

# References

R1-1609670, “NR random access procedure”, 3GPP TSG-RAN WG1 #86bis, Lisbon, Portugal, October 10-14, 2016

R1-1609671, “NR PRACH preamble design”, 3GPP TSG-RAN WG1 #86bis, Lisbon, Portugal, October 10-14, 2016

R1-1609674, “NR random-access response signal format”, 3GPP TSG-RAN WG1 #86bis, Lisbon, Portugal, October 10-14, 2016