3GPP TSG-RAN WG1#86bis		R1- 1609686

Lisbon, Portugal, 10-14 October, 2016

Source:	Ericsson

Title:	General principles for handling interruptions with SRS carrier based switching

Agenda Item:	7.2.5.3

Document for:	Discussion and Decision

# Introduction

Since the beginning of the SRS work item during RAN1#84b, it has been acknowledged that SRS carrier based switching will introduce an interruption time of a certain duration, depending on the terminal properties. From a RAN4 LS it was concluded that this range can be quite wide and varied from as little as 30usec and up to 900usec. In a separate contribution [1], we call for the use of UE classes to categorize the interruption time on a per-band basis. In this contribution, we discuss the general principle for handling interruption time while minimizing uplink capacity loss. In further contribution, the discussion is detailed for the cases of PUCCH [2], PUSCH [3] and the special subframe [4].

# SRS switching symbol locations

# Special subframes

For TDD CCs, the special subframe UPPTS part allows to send SRS. Up to 4 symbols can be allocated to SRS as detailed in 36.213 section 8.2.   and 36.211 section 4.1.  Depending on the special subframe configuration (table 4.2.1 in 36.211), a smaller or larger guard time can be exploited to manage the interruption time due to switching.  In [4], a proposal to handle SRS switching in the special subframe is detailed.   The maximum interruption time that can be handled by the special subframe without losses to UCI or data is 5 symbols “one-way” interruption. That is to say, the SRS switching operation would take 11 symbols, which can be handled with the special subframe formats 1 or 5.

# Last symbol in the subframe

SRS carrier based switching is also beneficial for systems not using TDD exclusively. Therefore, it was decided to not only support SRS switching in the special subframe but also for the legacy position of SRS.  For FDD, TDD as well as joint TDD FDD cases, the last symbol in the subframe can be used to transmit SRS. In that case, the SRS subframe as well as the following subframe may be affected by the interruption time. it is therefore desirable to design solution to minimize impact for these cases.

Observation 1: the “from” CC can be either TDD or FDD.

# On the interruption time impact on capacity

The capacity loss due to dropping one symbol or one slot per subframe has been simulated as a function of the SRS periodicity.   In the following, the impact of carrier switching on throughput of PUCCH/PUSCH has been studied for different interruption times and SRS switching periodicities. The periodicities have been taken from 36.213 and reproduced below for convenience.

Table 8.2-2: UE Specific SRS Periodicity    for trigger type 0, TDD

| SRS Configuration Index ISRS | SRS Periodicity (ms) |
| --- | --- |
| 0 to 9 | 2 |
| 10 – 14 | 5 |
| 15 – 24 | 10 |
| 25 – 44 | 20 |
| 45 – 84 | 40 |
| 85 – 164 | 80 |
| 165 – 324 | 160 |
| 325 – 644 | 320 |

The study took into account the amount of switching within one SRS period, and whether the dropping algorithm allowed dropping on the slot or subframe level. Figure 1 shows the results for slot dropping with TDD CA with 5DL and 2DL and figure 4 for 2DL+1UL.

Figure 1a, b:  Percentage of available PUSCH/PUCCH for 5DL+2UL CA and SRS switching for the remaining CC with slot dropping and subframe dropping (unseen cases are overlapping)

Figure 2 a,b:  Percentage of available PUSCH/PUCCH for 2DL+1UL CA and SRS switching for the remaining CC with slot dropping and subframe dropping (unseen cases are overlapping)

From figure 3 and 4 the following observation can be made:

Observation 3: SRS transmission with more that 10ms spacing will cost less than 10% of capacity.

Observation 4:  Slot level dropping can provide up to 90% of capacity for SRS transmission spacing of 5ms and above

# Interruption time and subframe structure impact

# Interruption time classes

As mentioned in [1], interruption classes of interest in meaningful design include:

class 1: under 350usec interruption (under 5 symbols)

class 2: under 500usec interruption (under a slot)

class 3: under 1000usec interruption (under a subframe)

class 4: over 1000usec interruption (over a subframe)

class 1 to 3 can be useful to decide to drop symbols or slot and maintain as much UCI and data capacity as possible. Class 4 can be interesting to decide on the parameterization of the carrier switching for that UE and band pair.

## Special subframe structure and interruption

The special subframe allows up to 4 SRS symbols and a guard period ranging from 1 to 10 symbols.

Table 1: configuration of the special subframe

| Configuration | DwPTS (Ts) | UpPTS (Ts) | Number of symbols/ subframe |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Dwpts | GP | Uppts |
| 0 | 6592 | 2192 | 3 | 10-X | 1+X |
| 1 | 19760 | 2192 | 9 | 4-X | 1+X |
| 2 | 21952 | 2192 | 10 | 3-X | 1+X |
| 3 | 24144 | 2192 | 11 | 2-X | 1+X |
| 4 | 26336 | 2192 | 12 | 1-X | 1+X |
| 5 | 6592 | 4384 | 3 | 9-X | 2+X |
| 6 | 19760 | 4384 | 9 | 3-X | 2+X |
| 7 | 21952 | 4384 | 10 | 2-X | 2+X |
| 8 | 24144 | 4384 | 11 | 1-X | 2+X |
| 9 | 13168 | 4384 | 6 | 6-X | 2+X |

As can ben seen from table 1, the total retuning time of SRS may be absorbed by the special subframe GP and UPPTS in some configurations.  From knowledge of the interruption time, the enodeB scheduler may be able to parameterize SRS transmission in the special subframe to minimize loss due to dropped slots.  The largest time that can be handled in the special subframe is GP+UPPTS =11 symbols. This means that an interruption time of 5 symbols or less can be handled by the special subframe. How to handle the interruption is detailed in [4]

For interruption times of less than five symbol (class 1), the total “round trip” time of SRS transmission is less than 11 symbols and thus can be handled within the special subframe. For larger interruption time (class 2 and above). the returning time to the uplink CC with PUSCH will spill over the next subframe and cause droppings.

Slot 2

Slot 1

Guard period + UPPTS

Slot 1

Slot 2

Subframe n

From CC

SRS

Interrupt

Interrupt

DWPTS

Subframe n+1

To CC

figure 3: SRS switching for Class 1 interruption with SRS in the special subframe.

Proposal: for class 1 interruption and under, adopt the principles of  SRS transmission in UPPTS when possible

## Subframe structure with class 2 interruption

The from and to carriers subframes are shown in figure 2, using a 500usec interruption time. Clearly the slot before and after SRS are affected and must be dropped.

Slot 0

Inter-rupt

Inter-rupt

Slot 1

From CC

Subframe n

Subframe n+1

SRS

To CC

figure 4: symbol dropping for Class 2 interruption.

We end up with two possible new additional subframe formats for Class 2 interruption. One with the last slot in the subframe lost to interruption and SRS and one with the leading slot lost to interruption. Figure 2 lists the two new formats based on slot dropping. The exact details of the slot formats for PUSCH and PUCCH are detailed in [4,5].

Slot 0

Inter-rupt

subframe format for class 2, SRS subframe

Inter-rupt

Slot 1

Subframe format for class 2, following SRSslot

figure 5: slot formats for Class 2 interruption. Green is for transmission, red for interruption. In the SRS subframe the transmission lasts 1 slot, i.e. 7 symbols and is then followed by an interruption of 1 slot. In the subframe following the SRS transmission, the first slot, or 500usec is interrupted, and the next slot is transmitted in full, i.e 7 symbols.

Proposal: for the purpose of handling class 1 or 2 interruption, adopt the principles of slot dropping for the SRS slot and its following slots as shown in figure 8.

## Subframe structure with class 3 and class 4 interruption

For interruption time above 500usec, i.e. class 3 or 4, the interruption time is such that the whole subframe will be dropped.  Therefore using legacy rules will suffice. It should be noted that the information of large interruption time can also be used to prioritize or down prioritize SRS transmission

Proposal: for class 3-4 interruption, use the legacy principles for subframe dropping. The interruption class can also be used to down-prioritize such carrier switching

# Conclusions

This contribution presented the general principle of handling SRS via the special subframe or slot dropping of PUCCH / PUSCH. The following assumption have been made.

Observation 1: the “from” CC can be either TDD or FDD.

Observation 2:  class 1 interruption time with UPPTS srs transmission allows to preserve the capacity

Observation 3: with slot dropping and class 2 interruption SRS transmission with more that 10ms spacing will cost less than 10% of capacity.

Observation 4:  Slot level dropping can provide up to 90% of capacity for SRS transmission spacing of 5ms and above

Proposal: for class 1 interruption and under, adopt the principles of SRS transmission in UPPTS when possible

Proposal: for the purpose of handling class 1 or 2 interruption, adopt the principles of slot dropping for the SRS slot and its following slots as shown in figure 5.

Proposal: for class 3-4 interruption, use the legacy principles for subframe dropping. The interruption class can also be used to down-prioritize such carrier switching.

# References

[1] R1-1609684 UE reporting of interruption time for SRS in carrier based switching, Ericsson, RAN1#86b, Lisbon, Portugal,2016

[2] R1-1609687 Slot format for PUCCH in SRS carrier switching, Ericsson, RAN1#86b, Lisbon, Portugal,2016

[3] R1-1609688 Slot format for PUSCH in SRS carrier switching, Ericsson, RAN1#86b, Lisbon, Portugal,2016

[4] R1-1609689 SRS switching within the Special subframe, Ericsson, RAN1#86b, Lisbon, Portugal,2016