3GPP TSG RAN WG1 Meeting #86bis	     R1-1609779

Lisbon, Portugal, 10th - 14th October 2016

# Source: 	ZTE, ZTE Microelectronics

# Title:	Further evaluations on FB-OFDM

# Agenda Item:	8.1.1.1

# Document for:	Discussion and Decision

# Introduction

This contribution gives the simulation results of evaluation case 1 to case 4 based on the FB-OFDM (2nd option) as described in [1], together with the comparison to W-OFDM [3] in each case. All evaluations in this contribution follow the simulation parameters agreed in earlier RAN1 meetings, including the PA modelling.

# PSD

Fig. 1 shows the PSD for W-OFDM and FB-OFDM with RAPP PA model, given the data transmission bandwidth equal to 50 RBs. Fig. 2 shows the PSD for W-OFDM and FB-OFDM with agreed UL PA model, given the data transmission bandwidth equal to 4 RBs.

(a) Full view                                                                   (b) Amplified region

Fig 1 PSD of W-OFDM and FB-OFDM with RAPP PA model (50RBs 46dBm)

(a) Full view                                                                   (b) Amplified region

Fig 2 PSD of W-OFDM and FB-OFDM with UL PA model (4RB)

According to the simulation, in order to meet the eNB emission mask requirement:

- For 10MHz data transmission in case 1a, the system guard band Gsys is 0.594 MHz for W-OFDM and 0.219 MHz for FB-OFDM (2nd option).

- For 4RB data transmission bandwidth in case 1b, the system guard band Gsys is 0.2675MHz (≈18 subcarriers) for W-OFDM and 0.093MHz (≈6 subcarriers) for FB-OFDM (2nd option).

Observation 1: The OOB energy of FB-OFDM falls much more rapidly than W-OFDM, which results in narrower system guard band and less number of guard-tones between adjacent subbands.

# Spectrum efficiency

This section gives the BLER and spectrum efficiency performances of evaluation case 1 to case 4. The spectrum efficiency is calculated as , where  denotes the number of correctly received bits by target user, T is the transmission time of the target user. The detailed descriptions of the evaluation cases are given in Appendix 1, with the corresponding simulation parameters given in Appendix 2. The waveform transmission and reception in the simulation are described in [4] for W-OFDM, and in [1] for FB-OFDM. The PA models applied in the simulations are described in Appendix 3.

## Case 1a and case 1b

For cases 1a and 1b,  is determined as following:

- Case 1a:  =BWcarrier is the whole bandwidth (50RBs) including system guard band.

- For W-OFDM,  = BWcarrier;= 15K*600+15K*1 (DC)+594K = 9.609 MHz

- For FB-OFDM (2nd option),  = BWcarrier;=15K*600+15K*1 (DC)+219K = 9.234 MHz

- Case 1b:  is the bandwidth of 4RB plus guard tones.

- For W-OFDM,  = (48+18)*15K = 990KHz

- For FB-OFDM (2nd option),  = (48+6)*15K = 810KHz

The spectrum efficiency and BLER performance are collected in Fig. 3 for case 1a and in Fig. 4 for case 1b, under the assumption of ideal channel estimation. It can be observed that,

- In case 1a and case 1b, the BLER performances of W-OFDM and FB-OFDM are very close to each other; then due to narrower guard band, FB-OFDM has higher spectrum efficiency than W-OFDM.

- Because the ratio of guard band to data bandwidth in case 1b is much higher than in case 1a, the larger differences of spectrum efficiencies between two waveforms are observed in case 1b than in case 1a.

Fig 3 BLER & SE for case 1a (TDL-C 300ns, 3km/h)

Fig 4 BLER & SE for case 1b (TDL-C 300ns, 3km/h)

Observation 2: With the RAN1-agreed evaluation assumptions, the BLER performances of FB-OFDM are very close to that of W-OFDM in case 1. Besides, FB-OFDM can achieve higher spectrum due to its smaller system guard band.

## Case 2

Fig. 5 shows the spectrum efficiency and BLER performance for case 2. With 16QAM modulation and 1/2 code rate in TDL-C channel model of 1000ns delay spread, the BLER performance and spectrum efficiency of FB-OFDM very close to those of W-OFDM.

Fig. 5 BLER & SE for case 2 (16QAM 1/2, TDL-C 1000ns, 3 km/h)

Observation 3: With the RAN1-agreed evaluation assumptions, the BLER performances and spectrum efficiency of FB-OFDM are very close to those of W-OFDM in case 2.

## Case 3

Fig. 6 shows the spectrum efficiency and BLER performance for case 3. With 64QAM modulation, 1/2 code rate in TDL-C channel model of 300ns delay spread and 10dB power offset between adjacent subbands, the BLER performance and spectrum efficiency of FB-OFDM are much better than those of W-OFDM in case 3, especially for the smaller number of guard tones (e.g., 0, 1 or 2). For the same width of guard band, the higher the power offset of interfering neighbour subband, the larger SE gain in FB-OFDM than in W-OFDM.

Fig 6 BLER & SE for case 3 (10 dB power offset, 128Ts time offset, 64QAM 1/2, TDL-C 300ns, 3km/h)

## Case 4

Fig. 7 shows the spectrum efficiency and BLER performance for case 4. With 64QAM modulation, 1/2 code rate in TDL-C channel model of 300ns delay spread and 10dB power offset between adjacent subbands, the BLER performance and spectrum efficiency of FB-OFDM are remarkably better than those of W-OFDM, especially when a small number (e.g., 0,1 or 2) of guard-tones are used between neighbouring subbands.

Fig 7 BLER & SE for case 3 (10 dB power offset, 128Ts time offset, 64QAM 1/2, TDL-C 300ns, 3km/h)

Observation 4: With the RAN1-agreed evaluation assumptions, the BLER performance and spectrum efficiency of FB-OFDM are much better than those of W-OFDM in case 3 and case 4, especially when a small number of guard tones are used.

# Conclusion

This contribution evaluates the FB-OFDM, as proposed in [1], in various evaluation cases, and concludes with the following observations:

Observation 1: The OOB energy of FB-OFDM falls much more rapidly than W-OFDM, which results in narrower system guard band and less number of guard-tones between adjacent subbands.

Observation 2: With the RAN1-agreed evaluation assumptions, the BLER performances of FB-OFDM are very close to that of W-OFDM in case 1. Besides, FB-OFDM can achieve higher spectrum due to its smaller system guard band.

Observation 3: With the RAN1-agreed evaluation assumptions, the BLER performances and spectrum efficiency of FB-OFDM are very close to those of W-OFDM in case 2.

Observation 4: With the RAN1-agreed evaluation assumptions, the BLER performance and spectrum efficiency of FB-OFDM are much better than those of W-OFDM in case 3 and case 4, especially when a small number of guard tones are used.

To summarize, similar to the FB-OFDM proposed in [2], the FB-OFDM (2nd option) given in [1] is suitable for the mixed numerology scenarios and asynchronous-subbands cases due to its highly localized power distribution in frequency domain.

# References

- R1-1609778, Further discussions on FB-OFDM, ZTE, ZTE Microelectronics

- R1-167915, Tx/Rx Structure of FB-OFDM, ZTE, ZTE Microelectronics

- R1-162199, Waveform Candidates, Qualcomm

- Datta, Rohit, et al. "FBMC and GFDM interference cancellation schemes for flexible digital radio PHY design." Digital System Design (DSD), 2011 14th Euromicro Conference on. IEEE, 2011.

# Appendix 1 Evaluation cases

- Case 1: single numerology case

    - 1a (as in Fig. 1): Downlink

    - 1b (as in Fig. 2): Uplink, only one UE with narrow bandwidth is located at the edge of wide frequency band. It is assumed that no wide-band filter upon the whole frequency band.

    - High speed scenario with speed up to 500 Km/h

In both Fig 1 and Fig 2, Gsys is the system guard band and depends on the OOB of new waveform.

In the simulation, the numerology FB-OFDM is cut and windowed as shown in [4]. Without loss of generality, in CP-OFDM with WOLA (W-OFDM), the shape of the edge in time domain is raised cosine window with length of 1/2 CP length. More simulation parameters are listed in Appendix 2.

Case 1a: Downlink single numerology case

Case 1b: Uplink single numerology case

- Case 2: DL mixed numerology case

Case 2: Downlink mixed numerology case

In case 2, the target user, which is either at the edge or within the full-band of the “target” subband (using numerology 1), is interfered by the interfering subband (using numerology 2). Gsubband is the width of guard band between neighbouring subbands and its value depends on the OOB performance of the waveform.

- Case  3: UL single numerology case (asynchronous reception between UEs)

Case 3: UL single numerology case (asynchronous reception between UEs)

In case 3, the target user, which is interfered by two asynchronous UEs located in neighbor subbands. GUE is the width of guard band between neighbouring subbands.

- Case  4: UL mixed numerology case (synchronous reception between UEs)

Case 4: UL mixed numerology case (synchronous transmission)

In case 4, the target user, which is interfered by two UEs (using numerology 2) located in neighbor subbands. GUE is the width of guard band between neighbouring subbands.

# Appendix 2 Simulation assumptions

Table A1 Parameters for case 1a/1b

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| Duplex | FDD/TDD (FDD and TDD have the same performance in these simulation results in this contribution) |
| System Bandwidth | 10 MHz |
| TTI length | 1 ms |
| Subcarrier spacing | 15KHz |
| Guard time interval | 4.7us (interval of LTE normal CP). The symbol interval of FB-OFDM is 1/14ms just the same as LTE |
| FFT size | 1024 |
| Data transmission bandwidth | 50 PRBs for the bandwidth of target UE in case 1a 4 PRBs for the bandwidth of target UE in case 1b |
| Antenna configuration | 1T1R |
| MIMO mode | SISO |
| Rank per UE | Fixed single rank |
| MCS | Fixed. 16QAM: 1/2 or 2/3; 64QAM: 1/2 |
| Control Overhead | Zero |
| Channel estimation | Ideal (without RS) |
| Channel Model | TDL-C for DS {300※,1000}ns |
| Others | The window process of CP-OFDM with WOLA in transmitter and receiver is the same as that described in [4] and window length at Tx is 36Ts. FB-OFDM: T1 = 18Ts, T2 = 126Ts, A = 10 |

Table A2 Parameters for case 3 and case 4

| Assumptions | Value |
| --- | --- |
| Carrier frequency | 4GHz |
| Duplex | FDD /TDD |
| Subframe duration | 1 ms as baseline, other duration is FFS (short duration could be considered) |
| Subcarrier spacing | Case 3: 15KHz as baseline, Case 4: Target UE: 15KHz; Interferer pair: {30KHz※, 30KHz※}, {7.5KHz, 7.5KHz} other value for interferers is not precluded. |
| Guard time interval | 6.7% overhead as baseline, other interval is FFS (depend on numerology progress ) |
| System bandwidth & FFT size | 10 MHz, 1024 for 15KHz subcarrier spacing |
| UE bandwidth (data transmission bandwidth plus guard tone bandwidth of the desired UE) | Case3: -720 KHz (48 Subcarriers per user allocated for both target UE and interferer UEs) Case4: Config1※: Target UE: - 720KHz (48 Subcarrier allocated) Interferer users: - 720KHz (per UE) |
| Bandwidth of guard tones between neighboring UEs | {0, 15, 30, 60※, 90, 180}KHz |
| Number of uplink users | Case 2: 2 (1 target user and 1 interferer users) Case 3 and case 4: 3 (1 target user and 2 interferer users) |
| Power offset of the interferer user | 10 dB※ |
| Antenna configuration | 1T1R※ |
| MIMO mode | SISO |
| MCS | Fixed. 16QAM: 1/2; 64QAM: 1/2※ |
| Control overhead | Zero |
| Time offset of interfering user | Case 3: fixed offset 128※samples (for 15 KHz subcarrier spacing with 1024 FFT size) Case 4: 0 |
| Channel estimation * | Ideal※ (without RS) |
| Channel model ** | TDL-C for DS {300※ }ns, 3km/h※ |
| Others | The window process of CP-OFDM with WOLA in transmitter and receiver is the same as that described in [4] and window length at Tx is 36Ts. FB-OFDM: T1 = 18Ts, T2 = 126Ts, A = 10 |

# Appendix 3 PA models

- Rapp PA model for downlink

The following modified Rapp model with AM/AM and AM/PM distortion models for downlink:

AM/AM:

AM/PM:

x is an amplitude of the input signal, and G=1.

| Parameter for Rapp model < 6GHz DL |  |  |
| --- | --- | --- |
| Target output power [dBm] | 46 |  |
| Saturation output power [dBm] | 57.6 |  |
| Smoothness factor p | 3 |  |
| Smoothness factor q | 5 |  |
| Fitting parameter A | -0.14 |  |
| Fitting parameter B | 1.2 |  |

- polynomial PA model for Uplink

The PA model using AM/AM and AM/PM polynomial approximation is shown in the next figure.

Figure  PA model (blue: measured data, red: polynomial approximation)

The polynomial data in dBm domain is also given below. The format can be directly copied to Matlab. The coefficients are organized as follows: [p9  p8  p7  …  p0] and the PA output y(t) can be computed from input x(t) using the formula
y(t) = p0 + p1∙x(t) + p2∙x(t)2 + p3∙x(t)3 + …

p_am = [7.9726e-12  1.2771e-9  8.2526e-8  2.6615e-6  3.9727e-5  2.7715e-5  -7.1100e-3  -7.9183e-2  8.2921e-1  27.3535];

p_pm = [9.8591e-11  1.3544e-8  7.2970e-7  1.8757e-5  1.9730e-4  -7.5352e-4  -3.6477e-2  -2.7752e-1  -1.6672e-2  79.1553];

The validity of the polynomial model deteriorates below input power -35 to -30 dBm, and it is suggested that fully linear model is applied below such input levels. Also extremely deep in compression, e.g. at input power above 9 dBm, the model does not provide realistic results. This should be fine since the signal peaks would anyway be limited to a reasonable value before entering the PA.