3GPP TSG RAN WG1 Meeting #86bis	     R1-1609780

Lisbon, Portugal, 10th - 14th October 2016

# Source: 	ZTE, ZTE Microelectronics

# Title:	Text Proposal on TR38.802 for NR waveform

# Agenda Item:	8.1.1.1

Document for:	Discussion and Decision

# Introduction

The FB-OFDM was proposed in [1]. In order to better support Tx-Rx transparency, the second formulation of FB-OFDM is provided in [2]. The relevant performance evaluations are updated in [3]. This contribution provides the corresponding text proposal, as given in Annex, to TR38.802, for both formulations of FB-OFDM.

# References

- R1-167915, Tx&Rx structure of FB-OFDM, ZTE, ZTE Microelectronics

- R1-1609778, Further discussions on FB-OFDM, ZTE, ZTE Microelectronics

- R1-1609779, Further evaluations on FB-OFDM, ZTE, ZTE Microelectronics

# Annex. Text Proposal to TR 38.802

<Start of changed section>

### 6.1.2 	Waveform

OFDM-based waveform is supported.

## The following waveform candidates are studied.

## 6.1.2.x FB-OFDM

In FB-OFDM, each subcarrier is independently filtered and therefore a bank of filters is needed to filter multiple subcarriers. The continuous baseband signal of FB-OFDM waveform can be formulated as

st=lkak,l∙gt-lT∙ej2πk∙∆f∙t-lT                                          (6.1.2.x-1)

where ak,l represents the resource element on k-th subcarrier and l-th time symbol. T is the time symbol interval. ∆f is the subcarrier spacing. gt is the pulse shaping function. Different pulse shaping functions can be selected for different use scenarios. For example, both CP-OFDM and W-OFDM are special cases of (6.1.2.x-1). Specifically, two options for FB-OFDM are studied.

## In Option-1 of FB-OFDM, the pulse shape function gt is defined by gt=gtt∙gft, where

- gtt is a time-domain raised cosine function defined by

gtt=10≤t<1-βTb121+cost-1-βTb2βTbπ1-βTb≤t<1+βTb0t≥1+βTb             (6.1.2.x-2)

Here β is the roll-off factor and Tb is 50% of the half-amplitude width of the function in time domain. The length of non-zero portion of gtt  is assumed to be 21+βTb=LT, where T is symbol interval.

- gft is derived from a frequency-domain root raised cosine function and is defined by gft=IDFTRf, where

Rf=10≤f<1-αf0cosf-1-αf04αf0π1-αf0≤f<1+αf00f≥1+αf0                    (6.1.2.x-3)

Here α is the roll-off factor in the range of [0,1]. f0 is 50% of the half-amplitude width of the raised cosine function in frequency domain.

gft aims to provide good frequency localization and orthogonality between symbols, while gtt is multiplied to avoid infinite pulse shape function in time domain and therefore to reach a good balance between time locality and frequency locality. The simulations based on <α=0.25, β=0.1, L=5, f0=1/T, T=1/14ms> exhibit superior out-of-band emission benefits, as well as better BLER performances and spectrum efficiencies in various evaluation cases.

In Option-2 of FB-OFDM, the pulse shape function gt is defined by

## gt=wt+T1;T2∙p(0;A,T0+T1)pt+T1;A,T0+T1-T1-T2≤t<-T1-T1≤t<T0wT0-t;T2∙p(0;A,T0+T1)0 T0≤t<T0+T2otherwise                   (6.1.2.x-4)

## where pt;A,L=cost-L2πA∙L for 0 ≤t<L, wt;x=121+costπx for -x≤t<0 and T0=1∆f. Accordingly, the pulse shape function is continuous and consists of three non-zero parts: the main middle part is the central symmetric cosine function whose length is T0+T1; the first and the third parts are raised cosine functions of length T2. Unlike W-OFDM, T0+T2 can be larger than time symbol interval T in FB-OFDM.

## It is observed in simulations that

- The Option-2 FB-OFDM offers good balance between out-of-band emission and inter-symbol interference, which makes it a better waveform candidate, especially in evaluation case 3 and case 4.

- A receiver designed for CP-OFDM can be used to receive the Option-2 FB-OFDM waveform.

The above observations are obtained based on parameters that are set to T1 = 18Ts, T2 = 126Ts and A = 10, given the subcarrier spacing equal to 15kHz and the system bandwidth equal to 10MHz. However, this choice on <T1, T2, A> should not be taken as a restriction on parameterization of FB-OFDM.

<End of changed section>