3GPP TSG-RAN WG1 #86bis	R1-1609852

Lisbon, Portugal, October 10th – 14th, 2016

Source:	Ericsson

Title:	Performance Comparison of Different Antenna Layouts

Agenda Item:	7.2.2.1.4

Document for:	Discussion & Decision

# Introduction

In RAN1#86, a way forward [2] on port layouts for Class A CSI reporting in Rel-14 was extensively discussed. One concern raised by some company was that evaluations were not done for all the proposed port layouts.  In this contribution, we present our system evaluation results for the port layouts included in the way forward.

# Discussion

The system performance of different antenna port layouts with 20, 24,28 and 32 antenna ports are presented under both the 3D UMi and the 3D UMa scenarios. They are compared with the performance of the corresponding 1D antenna array. More detailed simulation assumptions can be found in Appendix A.

The port layout details are summarized in Table 1 below, these are the set of port layouts in this table is the same as the set of port layouts proposed to be supported in rel-14 in the way forward [2]. The port layouts are grouped into “wide” and “tall” antenna array groups. In each group of port layouts, the corresponding 16 ports antenna is used as the baseline system for performance comparison.  2x1 virtualization is used in all the antenna configurations so a port layout of 1x8 corresponding to an antenna array of (M, N, P) = (2,8,2), where M, N and P are the number of antenna elements in vertical, horizontal and polarization domain, respectively.

Table 1: Port layouts for performance comparison of different number of antenna ports

|  | Port layout (VxH) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Antenna | 16 ports (baseline) | 20 ports | 24 ports | 28 ports | 32 ports |
| 1D “wide” array | 1x8 | 1x10 | 1x12 | 1x14 | 1x16 |
| 2D “wide” array | 2x4 | 2x5 | 2x6 | 2x7 | 2x8 |
| 2D “square” array | 4x2 |  | 4x3, 3x4 |  | 4x4 |
| 2D “tall” array | 4x2 | 5x2 | 6x2 | 7x2 | 8x2 |
| 1D “tall” array | 8x1 | 10x1 | 12x1 | 14x1 | 16x1 |

## 3D UMi

Figure 1 to Figure 5 shows the system performance gains with antennas of 20, 24,28 and 32 antenna ports over the baseline system with an antenna of 16 ports under 3D UMi scenario, all at 50% of resource utilization of the baseline system.  It can be seen the gain increases almost linearly as the antenna port number increases and for a given number of antenna ports, the gain is quite similar for different port layouts.  For 32 ports, mean throughput gains ranges from 18% to 23% and cell edge throughput gain ranges from 49% to 64% over 16 ports.

Figure 1: 1D “wide” array: 16 ports vs. 20, 24, 28 and 32 ports

Figure 2: 2D “wide” array: 16 ports vs. 20, 24, 28 and 32 ports

Figure 3: 2D “square” array: 16 ports vs. 24 and 32 ports

Figure 4: 2D “tall” array: 16 ports vs. 20, 24, 28 and 32 ports

Figure 5:1D “tall” array: 16 ports vs. 20, 24, 28 and 32 ports

## 3D UMa

Figure 6 to Figure 10Figure 5 shows the performance gains with antennas of 20, 24, 28 and 32 antenna ports over the baseline antenna with 16 ports under 3D UMa scenario.  like the 3D UMi scenario, the gain increases almost linearly as the antenna port number increases and for a given number of antenna ports, the gain is quite similar for different port layouts.  For 32 ports, 20% to 29% mean throughput gain and 54% to 81% cell edge throughput gain are achieved over 16 ports. The largest gain is observed with 1D wide antenna array.

Figure 6:1D “wide” array: 16 ports vs. 20, 24, 28 and 32 ports

Figure 7: 2D wide array: 2D “wide” array: 16 ports vs. 20, 24, 28 and 32 ports

Figure 8: 2D “square” array: 16 ports vs. 20, 24, 28 and 32 ports

Figure 9:2D tall array: 16 ports vs. 20, 24, 28 and 32 ports

Figure 10: 1D tall array: 16 ports vs. 20, 24, 28 and 32 ports

Large UE throughput gains are observed with all antenna port layouts with 20,24,28 and 32 ports as outlined in [2] over the ones with 16 antenna ports baseline.  The gains increase almost linearly with number of antenna ports

## Performance comparison of different baseline port layouts

Figure 11 shows the performance comparison of different baseline systems of 16 ports under both 3D UMi and 3D UMa. Under 3D UMi, the 1D “tall” array 8x1 performs the best, the mean throughputs are similar, between 0% to 6%.  While in 3D UMa, the 1D “wide” array performs best.  Thus different port layouts are good in different scenarios and  there is no “bad” port layout.

Figure 11: Performance comparison of different port layouts of the 16-port baseline systems

Different port layouts are good in difference scenarios, 1D wide array performs the best in 3D UMa.

Given the above observations, we have the following proposal:

Support all the port layouts outlined in the WF [2].

# Conclusion

In this contribution, we have provided system performance results of various antenna port layouts for 20,24,28 and 32 antenna ports as outlined in the WF [2].  Based on the results, we do not see a “bad” layout. Different layouts perform differently in different scenarios, they all provide large gains over the baseline 16 ports systems.  Based on the results, we  have the following observations and proposal:

Observation 1	Large UE throughput gains are observed with all antenna port layouts with 20,24,28 and 32 ports as outlined in [2] over the ones with 16 antenna ports baseline.  The gains increase almost linearly with number of antenna ports

Observation 2	Different port layouts are good in difference scenarios, 1D wide array performs the best in 3D UMa.

Proposal 1	Support all the port layouts outlined in the WF [2].

# References

Chairman’s note, RAN1#86

R1-168511, “Way Forward on Antenna Port Layouts”, Ericsson, QUALCOMM, Intel, CATT, ATT, DOCOMO, Verizon, RAN1#86

# Appendix: A Simulation Assumptions

| Carrier frequency | 2 GHz |
| --- | --- |
| Bandwidth | 10 MHz |
| Scenarios | 3D UMi 200m ISD, 3D UMa |
| Antenna Configurations | (M, N): 16 ports: 2x8, 4x4, 8x2, 16x1 20 ports: 2x10,4x5,10x2,20x1 24 ports: 2x12, 4x6, 6x4, 8x3, 12x2, 24x1 28 ports: 2x14, 4x7, 14x2, 28x1 32 ports: 2x16, 4x8, 8x4, 16x2, 32x1 2x1 virtualization, UMi (130° tilt) and UMa (122° tilt) |
| Cell layout | 57 homogeneous cells |
| Wrapping | Radio distance based |
| UE receiver | MMSE-IRC |
| CSI periodicity | 5 ms |
| CSI delay | 5 ms |
| CSI mode | PUSCH Mode 3-2 |
| Outer loop Link Adaptation | Yes, 10% BLER target |
| UE Rx antenna | Two cross polarized isotropic antennas |
| UE noise figure | 9 dB |
| eNB Tx power | 41 dBm (UMi) and 46dBm (UMa) |
| Traffic model | FTP Model 1, 500 kB packet size |
| UE speed | 3 km/h |
| Scheduling | Proportional fair in time and frequency |
| DMRS overhead | 2 DMRS ports |
| CSI-RS | Overhead accounted for Channel estimation error modeled. |
| Codebook | Rel-13 and extension, config 1, O1=O2=4 |
| HARQ | Max 5 retransmissions |
| Antenna spacing | 0.8 lambda in vertical, 0.5 lambda in horizontal |
| Handover margin | 3 dB |
| Transmission Mode | TM10, with non-shifted CRS |

# Appendix B: Simulation Results

## 3D UMi

Table 2: 1D “wide” antenna array

| Antenna | 2x8 | 2x10 | 2x12 | 2x14 | 2x16 |
| --- | --- | --- | --- | --- | --- |
| Number of antenna ports | 16 | 10 | 24 | 28 | 32 |
| Baseline RU = 20 % |  |  |  |  |  |
| Mean user throughput | 0% | 2% | 5% | 5% | 6% |
| 50% user throughput | 0% | 5% | 8% | 11% | 13% |
| Cell-edge user throughput | 0% | 5% | 14% | 17% | 20% |
| Baseline RU = 50 % |  |  |  |  |  |
| Mean user throughput | 0% | 7% | 12% | 14% | 18% |
| 50% user throughput | 0% | 10% | 18% | 21% | 27% |
| Cell-edge user throughput | 0% | 14% | 27% | 33% | 49% |
| Baseline RU = 70 % |  |  |  |  |  |
| Mean user throughput | 0% | 17% | 29% | 33% | 37% |
| 50% user throughput | 0% | 26% | 43% | 51% | 56% |
| Cell-edge user throughput | 0% | 37% | 69% | 77% | 93% |

Table 3: 2D “wide” antenna array

| Antenna | 4x4 | 4x5 | 4x6 | 4x8 |
| --- | --- | --- | --- | --- |
| Number of antenna ports | 16 | 20 | 24 | 32 |
| Baseline RU = 20 % |  |  |  |  |
| Mean user throughput | 0% | 3% | 5% | 6% |
| 50% user throughput | 0% | 6% | 10% | 13% |
| Cell-edge user throughput | 0% | 9% | 15% | 24% |
| Baseline RU = 50 % |  |  |  |  |
| Mean user throughput | 0% | 8% | 12% | 19% |
| 50% user throughput | 0% | 11% | 17% | 27% |
| Cell-edge user throughput | 0% | 19% | 31% | 52% |
| Baseline RU = 70 % |  |  |  |  |
| Mean user throughput | 0% | 17% | 27% | 38% |
| 50% user throughput | 0% | 24% | 40% | 59% |
| Cell-edge user throughput | 0% | 41% | 59% | 100% |

Table 4: 2D “square” antenna array

| Antenna | 8x2 | 6x4 | 8x3 | 8x4 |
| --- | --- | --- | --- | --- |
| Number of antenna ports | 16 | 24 | 24 | 32 |
| Baseline RU = 20 % |  |  |  |  |
| Mean user throughput | 0% | 5% | 5% | 7% |
| 50% user throughput | 0% | 9% | 9% | 14% |
| Cell-edge user throughput | 0% | 17% | 16% | 27% |
| Baseline RU = 50 % |  |  |  |  |
| Mean user throughput | 0% | 14% | 15% | 20% |
| 50% user throughput | 0% | 19% | 20% | 28% |
| Cell-edge user throughput | 0% | 33% | 35% | 57% |
| Baseline RU = 70 % |  |  |  |  |
| Mean user throughput | 0% | 26% | 29% | 39% |
| 50% user throughput | 0% | 37% | 42% | 58% |
| Cell-edge user throughput | 0% | 55% | 69% | 97% |

Table 5: 2D “tall” antenna array

| Antenna | 8x2 | 10x2 | 12x2 | 14x2 | 16x2 |
| --- | --- | --- | --- | --- | --- |
| Number of antenna ports | 16 | 20 | 24 | 28 | 32 |
| Baseline RU = 20 % |  |  |  |  |  |
| Mean user throughput | 0% | 3% | 5% | 6% | 7% |
| 50% user throughput | 0% | 5% | 9% | 13% | 15% |
| Cell-edge user throughput | 0% | 11% | 19% | 24% | 28% |
| Baseline RU = 50 % |  |  |  |  |  |
| Mean user throughput | 0% | 9% | 15% | 20% | 23% |
| 50% user throughput | 0% | 12% | 20% | 28% | 33% |
| Cell-edge user throughput | 0% | 23% | 37% | 53% | 64% |
| Baseline RU = 70 % |  |  |  |  |  |
| Mean user throughput | 0% | 18% | 32% | 35% | 43% |
| 50% user throughput | 0% | 26% | 47% | 52% | 63% |
| Cell-edge user throughput | 0% | 41% | 78% | 85% | 117% |

Table 6: 1D “tall” antenna array

| Antenna | 16x1 | 20x1 | 24x1 | 28x1 | 32x1 |
| --- | --- | --- | --- | --- | --- |
| Number of antenna ports | 16 | 20 | 24 | 28 | 32 |
| Baseline RU = 20 % |  |  |  |  |  |
| Mean user throughput | 0% | 3% | 6% | 6% | 7% |
| 50% user throughput | 0% | 6% | 10% | 13% | 15% |
| Cell-edge user throughput | 0% | 11% | 20% | 25% | 27% |
| Baseline RU = 50 % |  |  |  |  |  |
| Mean user throughput | 0% | 10% | 15% | 20% | 22% |
| 50% user throughput | 0% | 13% | 22% | 29% | 32% |
| Cell-edge user throughput | 0% | 22% | 34% | 49% | 59% |
| Baseline RU = 70 % |  |  |  |  |  |
| Mean user throughput | 0% | 19% | 32% | 40% | 46% |
| 50% user throughput | 0% | 28% | 48% | 58% | 69% |
| Cell-edge user throughput | 0% | 44% | 76% | 99% | 121% |

## 3D UMa

Table 7: 1D “wide” antenna array

| Antenna | 2x8 | 2x10 | 2x12 | 2x14 | 2x16 |
| --- | --- | --- | --- | --- | --- |
| Number of antenna ports | 16 | 20 | 24 | 28 | 32 |
| Baseline RU = 20 % |  |  |  |  |  |
| Mean user throughput | 0% | 3% | 5% | 6% | 7% |
| 50% user throughput | 0% | 5% | 9% | 12% | 15% |
| Cell-edge user throughput | 0% | 7% | 12% | 18% | 23% |
| Baseline RU = 50 % |  |  |  |  |  |
| Mean user throughput | 0% | 9% | 16% | 19% | 21% |
| 50% user throughput | 0% | 14% | 24% | 28% | 31% |
| Cell-edge user throughput | 0% | 20% | 38% | 45% | 54% |
| Baseline RU = 70 % |  |  |  |  |  |
| Mean user throughput | 0% | 18% | 26% | 35% | 41% |
| 50% user throughput | 0% | 25% | 39% | 54% | 63% |
| Cell-edge user throughput | 0% | 32% | 53% | 80% | 97% |

Table 8: 2D “wide” antenna array

| Antenna | 4x4 | 4x5 | 4x6 | 4x8 |
| --- | --- | --- | --- | --- |
| Number of ports | 16 | 20 | 24 | 32 |
| Baseline RU = 20 % |  |  |  |  |
| Mean user throughput | 0% | 3% | 6% | 8% |
| 50% user throughput | 0% | 5% | 10% | 16% |
| Cell-edge user throughput | 0% | 8% | 14% | 25% |
| Baseline RU = 50 % |  |  |  |  |
| Mean user throughput | 0% | 9% | 16% | 23% |
| 50% user throughput | 0% | 15% | 24% | 35% |
| Cell-edge user throughput | 0% | 21% | 39% | 63% |
| Baseline RU = 70 % |  |  |  |  |
| Mean user throughput | 0% | 19% | 32% | 47% |
| 50% user throughput | 0% | 28% | 48% | 72% |
| Cell-edge user throughput | 0% | 36% | 69% | 113% |

Table 9: 2D “square” antenna array

| Antenna | 8x2 | 6x4 | 8x3 | 8x4 |
| --- | --- | --- | --- | --- |
| Number of ports | 16 | 24 | 24 | 32 |
| Baseline RU = 20 % |  |  |  |  |
| Mean user throughput | 0% | 7% | 6% | 8% |
| 50% user throughput | 0% | 13% | 9% | 15% |
| Cell-edge user throughput | 0% | 19% | 16% | 29% |
| Baseline RU = 50 % |  |  |  |  |
| Mean user throughput | 0% | 24% | 19% | 28% |
| 50% user throughput | 0% | 35% | 27% | 42% |
| Cell-edge user throughput | 0% | 60% | 48% | 77% |
| Baseline RU = 70 % |  |  |  |  |
| Mean user throughput | 0% | 44% | 35% | 54% |
| 50% user throughput | 0% | 67% | 53% | 84% |
| Cell-edge user throughput | 0% | 103% | 79% | 131% |

Table 10: 2D “tall” antenna array

| Antenna | 8x2 | 10x2 | 12x2 | 14x2 | 16x2 |
| --- | --- | --- | --- | --- | --- |
| Number of ports | 16 | 20 | 24 | 28 | 32 |
| Baseline RU = 20 % |  |  |  |  |  |
| Mean user throughput | 0% | 2% | 4% | 5% | 7% |
| 50% user throughput | 0% | 4% | 7% | 10% | 14% |
| Cell-edge user throughput | 0% | 5% | 12% | 16% | 23% |
| Baseline RU = 50 % |  |  |  |  |  |
| Mean user throughput | 0% | 8% | 13% | 17% | 20% |
| 50% user throughput | 0% | 11% | 18% | 24% | 30% |
| Cell-edge user throughput | 0% | 19% | 35% | 43% | 54% |
| Baseline RU = 70 % |  |  |  |  |  |
| Mean user throughput | 0% | 13% | 23% | 31% | 47% |
| 50% user throughput | 0% | 21% | 35% | 50% | 75% |
| Cell-edge user throughput | 0% | 24% | 39% | 71% | 109% |

Table 11: 1D  “tall” antenna array

| Antenna | 16x1 | 20x1 | 24x1 | 28x1 | 32x1 |
| --- | --- | --- | --- | --- | --- |
| Number of ports | 16 | 20 | 24 | 28 | 32 |
| Baseline RU = 20 % |  |  |  |  |  |
| Mean user throughput | 0% | 3% | 6% | 8% | 9% |
| 50% user throughput | 0% | 4% | 10% | 13% | 15% |
| Cell-edge user throughput | 0% | 7% | 17% | 25% | 31% |
| Baseline RU = 50 % |  |  |  |  |  |
| Mean user throughput | 0% | 10% | 18% | 24% | 29% |
| 50% user throughput | 0% | 14% | 26% | 35% | 43% |
| Cell-edge user throughput | 0% | 24% | 42% | 62% | 81% |
| Baseline RU = 70 % |  |  |  |  |  |
| Mean user throughput | 0% | 21% | 37% | 46% | 57% |
| 50% user throughput | 0% | 33% | 57% | 75% | 92% |
| Cell-edge user throughput | 0% | 48% | 83% | 118% | 152% |