3GPP TSG RAN WG1 Meeting #86bis	R1-1609899

Lisbon, Portugal, 10th – 14th October 2016

Agenda Item:   8.1.3.1

Source:	  InterDigital Communications

Title:	          Interleaver for Polar Codes

Document for:	  Discussion and Decision

# 1	Introduction

In the RAN1 meeting #86, the BLER performance and the implementation complexity of major candidate channel codes (turbo, LDPC, polar) were extensively discussed [1]. Following these discussions, it was concluded that further channel coding simulation data sharing is needed [2].

When evaluating the BLER performance for turbo codes, many companies apply the same interleaving scheme (QPP interleaver and sub-block interleaver) as in LTE [3]. It is also observed that interleaver might not be required for LDPC codes [4].

In this contribution, we investigate the impact of interleaver on the BLER performance of polar codes. The performance evaluations of different interleaving schemes are provided.

# 2	Discussion

## 2.1 	Polar Codes

We consider a N,K polar code, where K is the information block length and N is the coded block length. Here, the value N is a set as a power of 2, i.e., N=2n for some integer n. The generator matrix of the polar code can be expressed by GN=BNF2⨂n, where BN is the bit-reversal permutation matrix, (∙)⨂n denotes the n-th Kronecker power and F2=1011.

With the well-defined polar encoder, the main design (or construction) of polar code is the mapping of the K information bits to the N input bits of polar encoder. In principle, the K information bits should be put on the K best (or most reliable) bit channels, and the remaining N-K input bits not mapped from the information bits are called frozen bits. There are several ways to determine the reliability of bit channels [5], e.g., the Bhattacharyya bounds, the Monte-Carlo estimation, the full transition probability matrices estimation, and the Gaussian approximation, etc. In this contribution, we use the Bhattacharyya bounds to construct the polar code with design-SNR 0 dB.

Also, we use the puncturing scheme in [6] to match the desired coding rates. When M bits are punctured, the effective coding rate of polar code is KN-M. In the remaining of this contribution, we use (N,K,M) to denote a (N, K) polar code with M bits punctured.

## 2.2 	Performance Evaluations of the Interleaver for Polar Codes

We examine the performance of different types of interleavers, which are applied to polar encoded bits. The interleaved bits are then passed to modulator. The receiver applies the same interleaver before polar decoding.

In our simulations, the rate 1/3 polar codes with different information block lengths are used on top of AWGN channels. We evaluate the cases of 1). no interleaver; 2). random interleaver; 3). LTE QPP interleaver; 4). LTE sub-block interleaver.

Figure 1 shows the simulation results of a (512, 170, 2) polar code with QPSK modulation, where the CRC-aided SCL-4 decoding algorithm or the CRC-aided SCL-32 decoding algorithm is used. It is seen from the figure that there is no performance difference among all these interleaving schemes. Specifically, the scheme without interleaver performs as well as other schemes.

Observation 1: For QPSK modulation, the intereleaver may not be needed for polar codes.

Figure 1: Interleaving schemes comparison for QPSK modulation

Figure 2 shows the simulation results of a (512, 170, 4) polar code with 16QAM modulation, where the CRC-aided SCL-4 decoding algorithm or the CRC-aided SCL-32 decoding algorithm is used. It is seen from the figure that sub-block interleaver performs worse than the other schemes and the scheme without interleaver performs as well as the random interleaving scheme.

Observation 2: For16QAM modulation, the intereleaver may not be needed for polar codes.

Figure 2: Interleaving schemes comparison for 16QAM modulation

Figure 3 shows the simulation results of a (4096, 1356, 28) polar code with 64QAM modulation, where the CRC-aided SCL-4 decoding algorithm or the CRC-aided SCL-32 decoding algorithm is used. It is seen from the figure that both QPP interleaver and random interleaver outperform the sub-block interleaver and the scheme without interleaver. The performance difference is about 0.5 dB at the BLER level of 1%, or more than 0.6 dB at the BLER level of 0.1%.

Observation 3: For 64QAM modulation, the QPP intereleaver achieves the same performance as random interleaver for polar codes.

Figure 3: Interleaving schemes comparison for 64QAM modulation

We only use AWGN channel in the above simulations. The performance impact of interleaver on polar codes in the case of fading channels may need to be further evaluated. Hence, we have the following proposals.

Proposal 1: Further study whether interleaver is needed for polar codes with low order modulation.

Proposal 2: Further study whether QPP interleaver is proper for polar codes with high order modulation.

# 3	Conclusion

In this contribution, we discussed different interleaving schemes for polar codes, and compared their performance. Our simulation results show that:

Observation 1: For QPSK modulation, the intereleaver may not be needed for polar codes.

Observation 2: For16QAM modulation, the intereleaver may not be needed for polar codes.

Observation 3: For 64QAM modulation, the QPP intereleaver achieves the same performance as random interleaver for polar codes.

We have the following proposals:

Proposal 1: Further study whether interleaver is needed for polar codes with low order modulation.

Proposal 2: Further study whether QPP interleaver is proper for polar codes with high order modulation.

# References

- Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #86, Gothenburg, Sweden, Aug. 2016.

- R1-168163, “WF on further channel coding simulation data sharing,” Aug. 2016.

- 3GPP TS 36.212, “Evolved universal terrestrial radio access (E-UTRA); multiplexing and channel coding,” Mar. 2016.

- R1-162927, “Design considerations on channel coding with very large blocklength,” Apr. 2016.

- H. Vangala, E. Viterbo and Y. Hong, “A comparative study of polar code constructions for the AWGN channel,” arXiv.1501.02473v1, Jan. 2015.

- R. Wang and R. Liu, “A novel puncturing scheme for polar codes,” IEEE Communications letters, Dec. 2014.