3GPP TSG RAN WG1 Meeting #86bis                              	       R1-1610001

Lisbon, Portugal 10th - 14th October 2016

Agenda item:    7.2.9.1.3

Source:             Qualcomm Incorporated

Title:                  Uplink-based narrowband positioning

Document for:  Discussion and Decision

- Introduction

In RAN#72 a new work item (WI) named enhancements of NB-IoT  [1] was introduced. The objectives of the WI include the support of positioning, multicast, non-anchor PRB enhancements, mobility and new power classes.

The following was agreed in context of uplink based positioning

- UTDOA positioning is supported under the following conditions:

  - It uses an existing NB-IoT transmission

  - It can be used by Rel-13 UEs

In this document we provide our views on the support of uplink based positioning.

- Design and procedures for uplink-based positioning

For narrowband positioning, we consider the reuse of NPRACH channel. NPRACH has the following characteristics that make it amenable for position location:

- Multiplexing of different UEs: NPRACH is designed to support a large number of UEs. Additionally, for Rel-14 NPRACH transmission will be supported outside the anchor PRB, which will also increase the NPRACH capacity of each cell. Multiplexing of a large number of UEs is important from a position location point of view, since the NPRACH resources have to be reserved not only in the serving cell, but in a number of cells of the neighbourhood.

- Support of single tone UEs: In Rel-13 it was decided that multitone support is mandatory, but there is the possibility of disabling it via FGI bit. If this FGI bit is decided to be maintained in Rel-14, the only single tone physical channel that would support position location (i.e., timing difference estimation) would be NPRACH.

- Designed for timing acquisition: One of the main objectives of NPRACH is to determine the initial timing error of a UE to issue the corresponding timing advance command for subsequent uplink transmissions. As uplink based positioning is based on observed time difference between different BS, so

Observation 1: NPRACH is a good candidate for uplink-based position location

For uplink-based positioning, the network has to indicate to the UE that it has to transmit NPRACH. This may result in network overloading, since the UEs can only monitor NPDCCH if they are in connected mode. Additionally, for uplink-based positioning it is not possible to determine if the UE has moved from the previous position, which may result in unnecessary NPRACH transmission when the UE is in a fixed location.

Observation 2: The NPRACH transmission for position location can be triggered by NPDCCH order. This requires the UE to be in connected mode.

For positioning the same NPRACH signal has to be monitored by multiple eNBs. Since open loop power control is used by the UE for NPRACH transmission, if the UE happens to be close to its serving cell, it will transmit the NPRACH with a low power. This then may not be hearable by other eNBs. To enable positioning the network would hence have to rely on UE transmitting the NPRACH multiple times with the power ramp up. Such an approach would lead to increased power consumption at UE and will be inefficient from a network resource usage point of view.

Observation 3: Due to use of open loop power control, a UE may have to transmit the NPRACH multiple times with power ramp up for hearability. This leads to increased power consumption at UE and introduces additional inefficiencies in the network. RAN1 should study and quantify the performance due to power control for PRACH, and the possible increase in power consumption.

In the next section we provide link-level simulation results for NPRACH based timing estimation.

- Link level simulation results

We evaluated the accuracy of NPRACH timing acquisition for different scenarios. The simulation assumptions are described in Table 1. The results are in Figures 1-6.

Table 1 Link level simulation assumptions

| Channel model | AWGN, TU1 |
| --- | --- |
| Number of eNB Rx antennas | 2 |
| UE Tx Power | 23dBm |
| Frequency error | 50Hz |
| Number of NPRACH repetitions | 2 – 1024 (depending on coverage level) |
| NPRACH CP | 266.7us |

In the following, we present results for 164, 154 and 144dB MCL. The results are shown in Figures 1-3.

us

Figure 1 CDF of timing error estimation for 164dB MCL for AWGN and TU1 channels

Figure 2 CDF of timing error estimation for 154dB MCL for AWGN and TU1 channels

Figure 3 CDF of timing error estimation for 144dB MCL for AWGN and TU1 channels

From these results, we make the following observations:

- To achieve a reasonable accuracy for 164dB MCL, the largest number of NPRACH repetitions is needed. The 90% accuracy is between [-0.4, 0.4]us.

- For AWGN channel, increasing the number of repetitions also increases the accuracy. For 144dB MCL, for example, 2 NPRACH repetitions achieve a 90% accuracy of [-0.7, 0.7]us, and 16 NPRACH repetitions achieve [-0.2, 0.2]us.

- For TU1 channel a bias of ~0.5us is observed. This is due to the fact that, for 180kHz bandwidth, paths that arrive too close are not possible to distinguish due to windowing. Thus, finding the first arrival path may be challenging in some cases (especially when the first path is not the strongest one). For 144dB MCL, for example, the 90% error for 16 repetitions is [-0.1us, 1.2us]. Similar observations can be made for the other coverage cases.

In Table 2 we provide tabulated results for the 90% accuracy for different coverage levels and channel models.

Table 2 90% accuracy for different CL (in us), repetition number and channel model

|  | AWGN | TU1 |
| --- | --- | --- |
|  | 164dB CL |  |
| 128 Repetitions | [-1.3, 1.3]us | [-1, 2]us |
| 1024 Repetitions | [-0.5, 0.5]us | [0.1, 1]us |
|  | 154dB CL |  |
| 32 repetitions | [ -0.5, 0.5]us | [-0.5, 1.5]us |
| 256 repetitions | [-0.25, 0.25]us | [0.3, 0.8]us |
|  | 144dB CL |  |
| 2 repetitions | [-0.7, 0.7]us | [-0.6, 1.7] us |
| 16 repetitions | [-0.25, 0.25]us | [-0.1, 1.2]us |

In view of these results, we make the following observation:

Observation 4: For AWGN channel, increasing the number of NPRACH repetitions also increases the timing accuracy. For TU1 channel, a large bias (around 0.5us) is observed due to reduced bandwidth.

- Comparison with DL-based positioning

For the sake of completeness, we provide in this contribution a comparison with the results in contribution [3] for DL narrowband positioning.

Figure 4 Comparison between UL and DL positioning for 164dB MCL

Figure 5 Comparison between UL and DL positioning for 154dB MCL

Figure 6 Comparison between UL and DL positioning for 144dB MCL

It is observed that, for the same MCL, DL based positioning achieves higher accuracy than UL based positioning for AWGN channel. For TU1 channel, the bias due to reduce bandwidth dominates the error, and both offer similar performance.

Observation 5: For the same transmission/reception time and same MCL, DL based positioning achieves higher accuracy than UL based positioning in AWGN channel. For TU1 channel, the bias dominates the error and both offer similar performance.

- Summary

In this contribution we presented a design based on NPRACH transmission for NB-IoT uplink-based position location. We make the following observations:

Observation 1: NPRACH is a good candidate for uplink-based position location

Observation 2: The NPRACH transmission for position location can be triggered by NPDCCH order. This requires the UE to be in connected mode.

Observation 3: Due to use of open loop power control, a UE may have to transmit the NPRACH multiple times with power ramp up for hearability. This leads to increased power consumption at UE and introduces additional inefficiencies in the network. RAN1 should study and quantify the performance due to power control for PRACH, and the possible increase in power consumption.

Observation 4: For AWGN channel, increasing the number of NPRACH repetitions also increases the timing accuracy. For TU1 channel, a large bias (around 0.5us) is observed due to reduced bandwidth.

Observation 5: For the same transmission/reception time and same MCL, DL based positioning achieves higher accuracy than UL based positioning in AWGN channel. For TU1 channel, the bias dominates the error and both offer similar performance.

References

[1] RP-161324, “New work item proposal: Enhancements of NB-IoT “,Vodafone, Huawei, HiSilicon, Ericsson, Qualcomm 3GPP TSG RAN Meeting #72, Busan, Korea, June 13 - 16, 2016

[2] RP-161901, “Revised work item proposal: Enhancements of NB-IoT “,Vodafone, Huawei, HiSilicon, Ericsson, Qualcomm 3GPP TSG RAN Meeting #73, New Orleans, USA, Sep. 19 - 22, 2016

[3] R1-1610000, “Downlink-based narrowband positioning”, Qualcomm, RAN1#86bis, Lisbon, Portugal 10th - 14th October 2016