3GPP TSG RAN WG1 Meeting #86bis							R1-1610047

Lisbon, Portugal 10th - 14th October 2016

Source:	NTT DOCOMO, INC.

Title:	sPUCCH for shortened TTI

Agenda Item:	7.2.10.2.1

Document for: 	Discussion and Decision

- Introduction

At the RAN1 #86 meeting, for shortened TTI length together with shortened processing time, some initial agreements on sPDCCH, UL DMRS position for sPUSCH and principles of shortened TTI lengths were achieved [1]. For sPUCCH, following agreements achieved during the study item should be the baseline [2].

Agreement: - The following sPUCCH formats are recommended to be supported - One sPUCCH format for HARQ-ACK and/or SR feedback for a serving cell - sPUCCH format(s) for multiple HARQ-ACK bits, e.g. as in CA and frame structure type 2 - The amount of sPUCCH formats to support is depending on the maximum identified payload size to support - sPUCCH format allows for multiplexing of HARQ-ACK and SR - FFS: sPUCCH format supports CSI feedback

In this contribution, we present our views on channel design for sPUCCH comprising 2-symbol, 4-symbol and 1-slot sTTI lengths. In addition, we provide our views on sPUCCH formats design.

- PUCCH in a short-TTI (sPUCCH)

Currently, the potential lengths of short-TTI for UL transmission are 2 symbols, 4 symbols, and 1 slot. In the following, sPUCCH design including the channel structure and sPUCCH formats based on above sTTI lengths are discussed.

  - sPUCCH for 4-symbol sTTI and 1-slot sTTI

For a given received SNR, as the number of symbols per sPUCCH reduces, the resulting UCI energy per bit-to-noise spectrum density ratio (Eb/N0) reduces linearly. Therefore, as TTI length becomes shorter, the required received SNR for achieving the target BER/BLER becomes higher (see Fig. 6 in appendix). In order to mitigate this impact, sPUCCH should support diversity mechanisms as much as possible. At least, intra-TTI frequency hopping should be the baseline for sPUCCH. Similarly, other means for SNR improvement should be considered, e.g., transmit antenna diversity.

In order to support intra-TTI frequency hopping, at least two DMRS symbols are necessary; one for the first half and the other for the second half of each sTTI. For 4-symbol sTTI, it is beneficial to apply DMRS sharing to make the number of sTTI per subframe being 4. Compared to DMRS sharing for sPUSCH [3], DMRS sharing for sPUCCH would be relatively easier, since the number of PRBs is not much flexible compared to sPUSCH/PUSCH. For 1-slot sPUCCH, considering SRS transmission, the first half of sPUCCH should be 3 symbols, and the latter half of sPUCCH should be 4 symbols at least for the second slot. As shown in Fig. 1, for 4-symbol sTTI, the structure per subframe is 4/4/4/4-symbol sTTI; for 1-slot sTTI, the structure per subframe is 7/7-symbol sTTI. Note that positions of DMRS and data symbol are just examples and need further investigation if agreed to be specified.

Fig. 1 sTTI structure per subframe for PUCCH transmission

(Left: 4/4/4/4-symbol sTTI per subframe structure; Right: 7/7-symbol sTTI per subframe structure)

Proposal 1:

- It is beneficial to apply intra-TTI frequency-hopping for sPUCCH.

  - Other mechanisms to improve the SINR such as transmit antenna diversity is beneficial.

Proposal 2:

- For sPUCCH transmission, it is preferred to support 4/4/4/4-symbol sTTI per subframe and 7/7-symbol sTTI per subframe structure.

  - For sPUCCH having 4 symbols, DMRS sharing can be supported to reduce the DMRS overhead.

In general, for a given TTI length, larger UCI payload requires higher SNR to decode, and for a given SNR, longer TTI length can convey larger UCI payload. For 4-symbol sPUCCH and 1-slot sPUCCH, following two alternatives for sPUCCH formats design can be considered.

Alt.1: At least two sPUCCH formats are defined for sTTI operation.

- First sPUCCH format targets small UCI payloads such as legacy PUCCH format 1/1a/1b.

Following UCI type can be supported by this sPUCCH format:

- 1-bit Scheduling request (SR);

- 1/2-bit HARQ-ACK for non-CA scenario and/or for CA scenario with HARQ-ACK bundling over spatial-domain and/or CC-domain.

- For simultaneous transmission of SR and HARQ-ACK, legacy mechanism can be re-used, e.g. if SR is positive, the sPUCCH resource configured for SR is used, otherwise the sPUCCH resource for HARQ-ACK is used.

Regarding the sPUCCH format structure, principles such as RE mapping, DMRS cyclic shift, OCC mechanisms and coding scheme e.g. block coding employed for legacy PUCCH format 1/1a/1b can be re-used. Fig. 2 is an example of the first sPUCCH format for 4- and 1-slot sTTI with intra-TTI frequency hopping.

Fig. 2 First sPUCCH format for small UCI payloads with intra-TTI frequency hopping

For both the 4-symbol and 1-slot sTTI, the maximum CDM capacity is limited by the number of DMRS symbols or data symbols based on the cyclic shift and OCC. If the DMRS and data symbols are mapped as in Fig. 2, the maximum multiplexing capacity in theory is 24 for both 4-symbol and 1-slot sTTI.

- Second sPUCCH format targeting large UCI payloads is based on legacy PUCCH format 4 or 5.

This sPUCCH format having larger payload size can support CSI feedback and/or multiple HARQ-ACK bits and SR if any.

Regarding the sPUCCH format structure, principles such as the rate matching, RE mapping and coding scheme e.g. TBCC of PUCCH format 4/5 can be directly re-used. For sPUCCH format based on PUCCH format 4, the sPUCCH resource without multiplexing can occupy multiple PRBs. For sPUCCH format based on PUCCH format 5, the supported number of PRB is one while it can support multiplexing of two UEs in the same PRB/sTTI by applying spreading factor in the frequency domain. Although legacy PUCCH format 4 and 5 do not support transmit diversity, it is applicable to the second sPUCCH format, in addition to intra-TTI frequency hopping. Fig. 3 and Fig. 4 give examples of the second sPUCCH format based on PUCCH format 4 and 5 respectively.

Fig. 3 Second sPUCCH format for large UCI payloads based on PUCCH format 4 with intra-TTI frequency hopping

(Left: 4-symbol sTTI; Right: 1-slot sTTI)

Fig. 4 Second sPUCCH format for large UCI payloads based on PUCCH format 5 with intra-TTI frequency hopping

(Left: 4-symbol sTTI; Right: 1-slot sTTI)

Alt.2 single sPUCCH format is defined to cover different UCI payloads.

Single sPUCCH format can be designed by combining and modifying PUCCH format 4 and PUCCH format 5. First, multiple PRBs can be used for sPUCCH resource similar to PUCCH format 4. This multi-PRB sPUCCH transmission is to increase the UCI payload for non-power-limited UEs. Besides, CDMA is considered for this sPUCCH format to mitigate the overhead increase. Variable spreading factor within symbol domain before DFT spreading can be considered which is similar to PUCCH format 5 so that sPUCCH having different sTTI length on the same PRB can still be orthogonal. Other design factors such as rate matching, RE mapping, interleaving structure, and intra-TTI frequency hopping for the sPUCCH format can be the same as those for PUCCH format 4 and 5. Through above analysis, it can be expected that alternative 2 can realize flexible trade-off among UCI payload, CDM capacity, and BLER performance.

Proposal 3:

- Different UCI payloads should be covered by a given sTTI length. Further study is needed for the following two alternatives.

  - Alt.1 at least two sPUCCH formats are defined for sTTI operation.

    - First sPUCCH format targeting small UCI payloads is based on legacy PUCCH format 1/1a/1b.

    - Second sPUCCH format targeting large UCI payloads is based on legacy PUCCH format 4 or 5.

  - Alt.2 single sPUCCH format is defined to cover different UCI payloads.

    - Variable spreading factor with single/multiple PRB(s) can be applied to the sPUCCH format.

  - sPUCCH for 2-symbol sTTI

For sPUCCH comprising 2 symbols, if single carrier property needs to be maintained, two alternatives can be considered. The first alternative is to multiplex DMRS and data into two SC-FDMA symbols in a sTTI. However, this way cannot realize intra-TTI frequency-hopping (FH) since one symbol must be DMRS and the other symbol must be data. Instead of applying intra-TTI FH, other means should be investigated, e.g., power boosting, transmit antenna diversity, etc. In case no intra-TTI FH, as discussed in [3], for efficient coexistence with legacy TTI PUCCH and/or legacy TTI PUSCH with intra-subframe hopping, it would be better not to allow 2-symbol sPUCCH spanning across slot boundary. This results in leaving one unpaired symbol within each slot, e.g. 2/2/2/1-symbol structure per slot. The position and use of the unpaired symbol need further discussion. The other alternative is sequence based sPUCCH design [4], which can maintain single carrier property and allows intra-TTI FH. For example, different information can be carried by DMRS using different cyclic shifts. By this way, intra-TTI frequency hopping can be applied and the 2-symbol sPUCCH can span cross slot boundary which simplifies the 2-symbol sTTI structure per subframe, e.g. 2/2/2/2/2/2/2-symbol structure per subframe. However, the multiplexing capacity is reduced.

If single carrier property is not necessary for 2-symbol sPUCCH, it is possible to multiplex DMRS and data within an OFDM symbol in a shortened TTI, which is similar to DL RS design. This way does not only reduce the DMRS overhead but also maintain the intra-TTI FH gain. In addition, the 2-symbol sPUCCH can be designed to across slot boundary for efficient co-existence with legacy TTI PUCCH and legacy TTI PUSCH with intra-subframe FH.

Proposal 4:

- For sPUCCH having 2 symbols,

  - It is preferable to multiplex DMRS and data within an OFDM symbol in a sPUCCH to reduce DMRS overhead and maintain intra-TTI frequency hopping.

- Conclusion

In this contribution, we discussed UL aspects of TTI shortening and reached following proposals.

Proposal 1:

- It is beneficial to apply intra-TTI frequency-hopping for sPUCCH.

  - Other mechanisms to improve the SINR such as transmit antenna diversity is beneficial.

Proposal 2:

- For sPUCCH transmission, it is preferred to support 4/4/4/4-symbol sTTI per subframe and 7/7-symbol sTTI per subframe structure.

  - For sPUCCH having 4 symbols, DMRS sharing can be supported to reduce the DMRS overhead.

Proposal 3:

- Different UCI payloads should be covered by a given sTTI length. Further study is needed for the following two alternatives.

  - Alt.1 at least two sPUCCH formats are defined for sTTI operation.

    - First sPUCCH format targeting small UCI payloads is based on legacy PUCCH format 1/1a/1b.

    - Second sPUCCH format targeting large UCI payloads is based on legacy PUCCH format 4 or 5.

  - Alt.2 single sPUCCH format is defined to cover different UCI payloads.

    - Variable spreading factor with single/multiple PRB(s) can be applied to the sPUCCH format.

Proposal 4:

- For sPUCCH having 2 symbols,

  - It is preferable to multiplex DMRS and data within an OFDM symbol in a sPUCCH to reduce DMRS overhead and maintain intra-TTI frequency hopping.

References

- 3GPP RAN1#86, Chairman’s note.

- 3GPP RAN1#85, Chairman’s note.

- R1-1610048, ‘sPUSCH for shortened TTI,’ NTT DOCOMO, INC., Oct. 2016

- R1-164545, ‘Discussion on sPUCCH for HARQ-ACK in latency reduction,’ LG Electronics, May 2016.

Appendix

Table 1. Link-level simulation assumptions for sPUCCH

| Parameter |  | Value |
| --- | --- | --- |
| Carrier frequency |  | 2 GHz |
| System bandwidth |  | 10 MHz |
| PUCCH baseline design |  | PUCCH format 1b for 14 symbols, see Fig. 5 (a) |
| Shortened TTI length | Without frequency hopping | - 7 symbols, see Fig. 5 (b) - 4 symbols, see Fig. 5 (c) |
|  | With frequency hopping | - 7 symbols, see Fig. 7 (a) - 4 symbols, see Fig. 7 (b) |
| Allocated bandwidth |  | 1 PRB |
| Frequency hopping |  | Enabled for 14 symbols; Enabled/Disabled for 7 symbols; Enabled/Disabled for 4 symbols. |
| Channel model |  | EPA, ETU |
| UE speed |  | 3km/h, 60km/h |
| Antenna configuration |  | 1Tx(UE), 2Rx(eNB) |
| CP length |  | Normal |
| Receiver type |  | Maximum likelihood (ML) detection |
| DTX detection threshold |  | DTX-to-ACK error probability < 1% |
| Channel estimation |  | Practical |
| Performance metrics |  | BLER |

Simulation results for ACK-to-NACK and NACK-to-ACK error rate of sPUCCH having 7 and 4 symbols without frequency hopping compared with existing PUCCH format 1b having 14 symbols.

(a) 1ms (existing PUCCH format 1b as a reference)

(b) 7 symbols (slot-level short-TTI)				(c) 4 symbols

Fig. 5	PUCCH structures for 1ms and short TTIs having 7 and 4 symbols.

Fig. 6	ACK-to-NACK and NACK-to-ACK error rates as a function of SNR per Rx antenna.

From Fig. 6, it is clearly observed that PUCCH on a shorter-TTI requires higher SNR to achieve the target error rates. In addition, with the short-TTIs, loss of frequency-diversity gain further degrades the performance. The required SNR difference between sPUCCH having 4 and 7 symbols is around 2.5dB, which is almost exactly same as the Eb/N0 loss: 10log10(4/7) = 2.4dB. However, the difference between 4/7 and 14 symbols is much more significant because of the loss of frequency-diversity gain.

Simulation results for ACK-to-NACK and NACK-to-ACK error rate of sPUCCH having 7 and 4 symbols with intra-TTI frequency hopping compared with existing PUCCH format 1b having 14 symbols.

(a) 7 symbols (slot-level short-TTI)		(b) 4 symbols	(1st and 2nd symbols replaced)

Fig. 7	PUCCH structures for short TTIs having 7 and 4 symbols with intra-TTI frequency-hopping.

Fig. 8	ACK-to-NACK and NACK-to-ACK error rates as a function of SNR per Rx antenna.

From Fig. 8, it is observed that the same diversity-order as with legacy PUCCH format 1b (i.e., 1ms TTI) is achieved for sPUCCH having 7 and 4 symbols. Compared to the case without intra-TTI frequency-hopping plotted in Fig. 6, the required SNR for achieving the target error rates can be reduced by roughly about 3dB for the same number of symbols per short-TTI. Even though higher SNR is still necessary for sPUCCH compared to 1ms TTI PUCCH, it is important to apply intra-TTI frequency-hopping so that the performance degradation is mitigated.