3GPP TSG RAN WG1 Meeting #86bis	R1-1608657

Lisbon, Portugal, October 10 – 14, 2016

Agenda Item:	7.2.1.5.3

Source:	Huawei, HiSilicon

Title:	Sidelink synchronization enhancement for V2X communication

Document for:	Discussion and decision

# Introduction

At the RAN1 #86 meeting, the following agreements on synchronization for PC5 interface based V2V communication were reached [1]:

- GNSS is at the highest priority of synchronization source for time and frequency when the following conditions are met:

  - The UE is out of coverage  on the V2X carrier (as defined by RAN2)

  - The UE does not receive any eNB synchronization configuration

  - The UE directly receives GNSS with sufficient reliability (as defined by RAN4)

  - The UE does not receive any SLSS_net signal  with in-coverage indicator 1

- The existing agreements apply when at least one of the above conditions is not met.

  - FFS UE behaviour when the UE is out-of-coverage and receives SLSS_net.

While significant progress was achieved during the WI on synchronization, synchronization for partial coverage was not discussed extensively. In this contribution, we discuss synchronization for this scenario.

# Discussion

## Synchronization issue for out of coverage of GNSS or eNB

There are areas, such as a tunnel or an underground parking lot, where UEs cannot receive either GNSS or eNB signals. Once a UE loses GNSS synchronization, it will attempt to find a synchronization reference UE that transmits synchronization information. However, the timing of the reference UE may be different than GNSS timing. Thus, UEs in coverage of GNSS may not be synchronized with nearby UEs without GNSS timing. An example is shown in Figure 1. UE 1 and UE 5 use GNSS timing. UE 2, UE 3 and UE 4 use OOC timing derived based on UE timing. UE 5 and UE 4 may not be able to communicate with each other due to different timing without reestablishing timing synchronization. The same issue happens for UE 2 and UE 1 as well. In addition, due to different timing, UEs that are out of GNSS coverage may cause interference to in-GNSS coverage UEs. A similar problem occurs for in-GNSS coverage UEs.

For D2D, a UE can be configured by network to transmit synchronization information irrespective of having data to transmit. Unless a UE is configured, it shall deliver eNB timing as long as the RSRP is good enough. For V2V, synchronization is more crucial since it is a safety service. To avoid switching timing during which vehicles may miss some messages, it is crucial for a UE to be able to find a timing quickly. Even when a UE is not transmitting V2V messages, it should transmit synchronization information to provide timing when it is close to being in partial coverage.

Observation 1: A procedure is needed to provide GNSS or eNB timing to UEs without GNSS timing or out of coverage.

Figure 1. Example scenario of out of GNSS or eNB coverage

## Sidelink synchronization enhancement in V2V

According to observations on GNSS timing maintenance [2], synchronization can be maintained at the UE for several minutes after GNSS synchronization is lost. A synchronization source that recently lost GNSS synchronization has more precise GNSS timing than a source that has lost GNSS for a longer time. In Figure 1, although UE 4, UE 3 and UE 2 all lost GNSS timing, UE4’s timing is closer to GNSS timing than UE 3 and UE 2. It is reasonable for UE 4 to transmit synchronization information rather than to synchronize with UE 3. If a UE that recently lost GNSS transmits synchronization information for a certain time duration, a still relatively accurate GNSS timing can be provided to other UEs.

Proposal 1: When a UE loses GNSS, the UE still transmits the synchronization signal based on its previous GNSS timing for a certain time duration (timer definition and duration up to RAN2/RAN4).

We need to look at the dual problem of a UE with no GNSS timing getting into satellite coverage. After the UE enters GNSS coverage, it should transmit synchronization information based on the new timing reference for at least as long as the timing is deemed accurate enough. This enables neighboring UEs to have an accurate time reference.

Proposal 2: When a UE acquires GNSS or eNB timing, it starts transmitting synchronization signals

# Conclusions

In this contribution, we discuss synchronization information transmission procedure in V2V and make the following observations and proposals:

Observation 1: A procedure is needed to provide GNSS or eNB timing to UEs without GNSS timing or out of coverage.

Proposal 1: When a UE loses GNSS, the UE still transmits the synchronization signal based on its previous GNSS timing for a certain time duration (timer definition and duration up to RAN2/RAN4).

Proposal 2: When a UE acquires GNSS or eNB timing, it starts transmitting synchronization signals.

# References

3GPP Chairman Notes, RAN1#86, Gothenburg, Sweden, August 2016

R1-162639, “Synchronization priority and timing consideration for V2V”, Huawei, HiSilicon, 3GPP RAN1 #84bis, April 11 - 15, 2016.