3GPP TSG-RAN WG1 Meeting #86-BIS                                                       R1-1608666

Lisbon, Portugal, 10th – 14th October, 2016

Source:	ZTE, ZTE Microelectronics

Title:	Discussion on beamforming procedure considering high frequency channel characteristics

Agenda Item:	8.1.4.1

Document for: 	Discussion and Decision

Introduction

NR DL beam management [1] was discussed in RAN1 #86 and the following agreements have been reached:

The following DL L1/L2 beam management procedures are supported within one or multiple TRPs:

  - P-1: is used to enable UE measurement on different TRP Tx beams to support selection of TRP Tx beams/UE Rx beam(s)

    - For beamforming at TRP, it typically includes a intra/inter-TRP Tx beam sweep from a set of different beams

    - For beamforming at UE, it typically includes a UE Rx beam sweep from a set of different beams

    - FFS: TRP Tx beam and UE Rx beam can be determined jointly or sequentially

  - P-2: is used to enable UE measurement on different TRP Tx beams to possibly change inter/intra-TRP Tx beam(s)

    - From  a possibly smaller set of beams for beam refinement than in P-1

    - Note: P-2 can be a special case of P-1

  - P-3: is used to enable UE measurement on the same TRP Tx beam to change UE Rx beam in the case UE uses beamforming

  - Strive for the same procedure design for Intra-TRP and inter-TRP beam management

    - Note: UE may not know whether it is intra-TRP or inter TRP beam

  - Note: Procedures P-2&P-3 can be performed jointly and/or multiple times to achieve e.g. TRP Tx/UE Rx beam change simultaneously

  - Note: Procedures P-3 may or may not have physical layer procedure spec. impact

As an additional task over the below 6GHz system, the object of beamforming procedure is to keep low overhead while still benefit from antenna array, which has been highlighted in several agreements [1][2][3]. In order to achieve this object, the design of beamforming procedure should thoroughly consider the impact of channel characteristics of above 6-GHz.

In light of this, this contribution elaborates the channel characteristics of above 6-GHz, according to high-frequency channel model TR 38.900 [4] and real-field measurements, and highlights some must-have/efficient beamforming procedure accordingly.

Discussion

In this contribution, some channel characteristics containing spatial consistency, blockage, channel-gain-related sparsity in space domain and cluster identification in time domain are discussed as follows for the NR beamforming design:

Spatial consistency

Spatial consistency, as an additional feature in TR 38.900, is to describe continuous evolutionary behaviours of the channels with regard to the topological and geographical variations/mobility of TX and RX on both large and small scale levels. For instance, regarding the indoor scenario, the evolutions of power arrival azimuth profiles have been shown in Figure 1 according to map-based hybrid model in TR 38.900, where the smooth evolutions of power delay profiles along the UE route can be observed clearly. This aforementioned analysis proves the effectiveness of priori information on beamforming alignment, and the subspace beam refinement instead of the whole-space beamforming sweeping can continuously maintain the TX/RX beam pairs aligning during UE mobility.

- (b)

Figure 1 (a) Indoor scenario and UE's moving trajectory; (b) Evolutions of power arrival azimuth profiles

Observation 1: The subspace beam refinement instead of the whole-space beamforming sweeping can continuously maintain the TX/RX beam pairs aligning during UE mobility.

Therefore, the beamforming-related reference signal is to be triggered by UEs/TRP with the explicitly configurable or fixed number of sweeping beams as aperiodic beam training, which also can be used for beam tracking against UE's movement. Distinguishing the UL/DL and TX/RX sweeping, beam refinements for DL-TX, DL-RX, UL-TX and UL-RX should be supported in NR, and one side of TX/RX beam remains during other’s beam sweeping. Joint TX and RX beam refinement due to inter-combination between RX and TX would raise the sweeping cost and is FFS.

Proposal 1:  The aperiodic beamforming refinement should be considered for addressing the issue of UE mobility according to spatial consistency, and the single-side sweeping procedures containing DL-TX, DL-RX, UL-TX and UL-RX sweeping should be supported in NR.

Figure 2  Beamforming refinements for (a) DL-TX beams; (b) DL-RX beams; (c) UL-TX beams; (d) UL-RX beams

Blockage

The high-frequency communications due to the poor diffraction are more vulnerable to the object blockage, e.g., blockage caused by human or vehicle. Therefore, the blockage in high frequency channel modelling has been drawn much attention from numerous companies and organizations. Blockage modelling is an add-on feature to the 3GPP above 6GHz channel model.

The original transmission link would be broken once the blockage for current propagation path occurs, taking into account that the blockage poses approximately about 20 dB attenuation. Therefore, some spatial diversity techniques for beamforming, such as generating multiple beams along the various propagation paths or switching to alternative beams if blockage occurs, are an essential way to address this blockage issue.

Observation 2: Spatial diversity techniques for beamforming procedure are feasible to address this severe channel blockage issue.

It is worthwhile to be noticed that there exists the decay time of around 100 ms or more from blockage start to the basically constant attenuation, which means that TRP/UEs might find this event, such as via neighboring beam measurement [5], and adjust own beamforming approach regardless of via PHY or MAC. To be more specific, the attenuation for the specific cluster can be approximated as a piecewise linear, which has a period with linear degradation, a period with constant Ls and a period with attenuation decrease, as shown in Figure 3.

Figure 3  Cluster-related power attenuation induced by blockage

Proposal 2: The TRP/UEs should be capable of finding this blockage event and adjusting their own beamforming, and the mechanism for rapidly detecting blockage and PHY/MAC scheduling delay should be studied in beam management.

Channel-gain-related sparsity in space domain

For above 6GHz channels, non-coherent paths consisting of diffuse scattering diffraction and high-order reflection only contribute very small portion of total received energy due to little scattering effects and severe path loss; on the contrary, the large portion of total propagation energy is carried on specular paths involving direct and low-order reflection, e.g., first-order/second-order ones, from limited propagation directions. Generally contributions from non-coherent paths would be negligible due to the fact that their powers are below noise floor of receiver, and therefore these dominantly paths that are used to support subsequent data transmissions are limited.

Considering the large-scale antenna array, the number of dominantly paths is much less than spatial resolution nrnt, where nr and nt denotes the number of TX and RX antenna elements. It is termed channel-gain-related sparsity in space domain. In TR 38.900, the numbers of clusters for various scenarios are summarized in Table 1. Nevertheless, in the typical case of 256 TX elements (for TRP) and 16 RX elements (for UEs), spatial resolution nrnt is up to 256*16=4096. This channel-gain-related sparsity also can be demonstrated with real-field measurement. In the indoor scenario, the results of our measurement campaign where the azimuth of TX/RX is partitioned by equally-spaced 36 beam directions with separation equal to 10 degree are described in Figure 4, which shows that the 6.02% TX/RX pairs occupy the 80% power of channel gain matrix.

Observation 3: Considering the large-scale antenna array, the number of dominantly paths is much less than spatial resolution nrnt, i.e., the channel-gain-related sparsity in space domain.

Table 1 The number of clusters in various scenarios

|  | UMi |  |  | UMa |  |  | RMa (up to 7GHz) |  |  |  | Indoor |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | LOS | NLOS | O-to-I | LOS | NLOS | O-to-I | LOS | NLOS | LOS O-to-I | NLOS O-to-I | LOS | NLOS |
| Number of clusters | 12 | 19 | 12 | 12 | 20 | 12 | 11 | 10 | 11 | 10 | 8 | 10 |

Figure 4   Channel gain measurement in the indoor scenario.

According to compressed sensing, the search for the best beam pair can be achieved while the sweeping beams are far less than the total number of candidate pairs, i.e., non-exhaustive searches. To be more specific, some advanced techniques for beam sweeping, such as multi-level-feedback and sweeping-precoding-based search, can be used to improve the efficiency of beamforming procedure via exploiting this sparsity in space domain. Some more details can be found in [6].

Proposal 3: To include the sparisity-based approaches, such as multi-level-feedback and sweeping-precoding-based search, into RAN1's further study on NR beamforming procedure.

Cluster identification in time domain

For above 6GHz NR systems, their signal bandwidths are ultra wide due to rich spectrum resources. The NR receiver using this ultra wide band would have a very high time-domain resolution in ns. Meanwhile the delay spread in TR 38.900 is about 30~100 ns for various scenarios or frequencies. That means that the NR receiver’s time-domain resolution can be obviously less than the delay spread, and therefore it can efficiently identify different paths/clusters in time domain. While taking into account of the limited dominant paths as analyzed in Section 2.3, the cluster identification in time domain should be considered for enhancing the beamforming performance, such as MIMO precoding and multi-path detection against unexpected blockage. Highlighted that the ability of cluster identification in time domain should be considered for design of NR beamforming-CSI feedback

This characteristic has also been demonstrated in our real-field measurement. Given the bandwidth (after carrier aggregation) is 500 MHz, the NR receiver has the time resolution of up to 2 ns. That means that the receiver can identify these arrival paths in time domain while the difference between their propagation distances is not less than 0.6 meter. In the indoor scenario, the results of our measurement campaign and ray tracing with the corresponding digital map are shown in Figure 5, which shows that most of clusters can be distinguished given the time resolution of 2 ns.

Figure 5  Channel measurement and ray-tracing results in the indoor scenario.

Observation 4: The NR transceiver’s time-domain resolution can be obviously less than the delay spread for above-6GHz channel, and therefore different paths/clusters identifying in time domain becomes feasible.

Proposal 4: The ability of cluster identification in time domain should be considered for design of NR beamforming-CSI feedback, in order to enhance the beamforming performance such as MIMO precoding and multi-path detection against unexpected blockage.

Conclusion

This contribution provides our observations and proposals as following for beamforming procedure considering high frequency channel characteristics:

Observation 1: The subspace beam refinement instead of the whole-space beamforming sweeping can continuously maintain the TX/RX beam pairs aligning during UE mobility.

Observation 2: Spatial diversity techniques for beamforming procedure are feasible to address this severe channel blockage issue.

Observation 3: Considering the large-scale antenna array, the number of dominantly paths is much less than spatial resolution nrnt, i.e., the channel-gain-related sparsity in space domain.

Observation 4: The NR transceiver’s time-domain resolution can be obviously less than the delay spread for above-6GHz channel, and therefore different paths/clusters identifying in time domain becomes feasible.

Proposal 1:  The aperiodic beamforming refinement should be considered for addressing the issue of UE mobility according to spatial consistency, and the single-side sweeping procedures containing DL-TX, DL-RX, UL-TX and UL-RX sweeping should be supported in NR.

Proposal 2: The TRP/UEs should be capable of finding this blockage event and adjusting their own beamforming, and the mechanism for rapidly detecting blockage and PHY/MAC scheduling delay should be studied in beam management.

Proposal 3: To include the sparisity-based approaches, such as multi-level-feedback and sweeping-precoding-based search, into RAN1's further study on NR beamforming procedure.

Proposal 4: The ability of cluster identification in time domain should be considered for design of NR beamforming-CSI feedback, in order to enhance the beamforming performance such as MIMO precoding and multi-path detection against unexpected blockage.

- References

- R1-168278, WF on DL beam management, Intel Corporation

- R1-165564, WF on UE beamforming and beam management, Nokia, Samsung, Intel, Interdigital, Alcatel-Lucent Shanghai Bell

- R1-166030, WF on beam management, Intel, Ericsson, Nokia, Qualcomm, Samsung, ZTE, NTT DoCoMo, Huawei, Xinwei, CATT, MTK

- 3GPP TR 38.900, Channel model for frequency spectrum above 6 GHz  (Release 14)

- Y.M. Tsang, and A.S.Y. Poon, Detecting Human Blockage and Device Movement in mmWave Communication System, Globecom 2011

- R1-1608667, Discussion on methods of determining Tx and Rx beams for NR MIMO, ZTE, ZTE Microelectronics