3GPP TSG RAN WG1 #86b	R1-1609090

# Lisbon, Portugal 10th - 14th October 2016

Agenda item:	8.1.4.3

Source: 	Samsung

Title: 	Simulation results for explicit CSI feedback for NR MIMO

Document for:	Discussion and Decision

# Introduction

It is expected that MIMO will be one of the key enablers to meet the high system throughput requirements for NR. The accurate CSI acquisition is an important step in any MIMO transmission scheme. In RAN1#86 [1], the following agreement about CSI acquisition was made.

Agreements:

- A simplified CSI acquisition framework should be studied in NR, which could support

  - CSI measurement based on CSI-RS (if supported)

  - Implicit and explicit CSI feedback

  - CSI acquisition based on different degree of reciprocity

  - Other features to be supported

- The implicit CSI feedback methods should be studied in NR

  - Codebook design

  - Interference measurement based on interference measurement resource which could be one or more of the following options

    - ZP CSI-RS (if supported)

    - NZP CSI-RS (if supported)

    - DMRS (if supported)

    - Other resources are not precluded

  - CSI feedback based on DMRS (if supported)

- The explicit CSI feedback methods should be studied in NR

  - Feedback of channel covariance matrix

  - Feedback of channel matrix

  - Feedback of  channel eigenvector

  - Both quantized and unquantized/analog feedback

  - Other methods are not precluded

- CSI measurement and reporting with the following components should be studied in NR

  - Wideband/long-term CSI

  - Subband/short-term CSI

  - Explicit CSI

  - Implicit CSI

  - Configuration of the above components individually or jointly is FFS

Samsung’s high-level view on CSI acquisition for DL and UL NR MIMO is presented in companion contributions [2-3]. The companion contribution [4] proposes the CSI acquisition based on a dual-resolution CSI reporting framework where the two CSI resolutions correspond to implicit and explicit CSIs. This contribution provides simulation results for the two resolutions (implicit and explicit) and shows that performance close to ideal CSI can be achieved with explicit CSI reporting, and hence makes the proposal that explicit CSI reporting should be supported in NR MIMO.

# Simulation Results for Explicit CSI Reporting

To illustrate the potential performance gains with explicit CSI reporting, simulation results are provided for the following CSI reporting schemes:

- Class A: CSI reporting is based on Rel. 13 Class A codebook (and its extension to > 16 antenna ports) with Codebook-Config = 1.

- LC Implicit: CSI reporting is based on linear combination (LC) codebook proposed in companion contributions [5-7], where Rel. 13 Class A W1 codebook corresponding to Codebook-Config = 3 is considered as the W1 codebook for LC codebook.

- LC Explicit: CSI reporting is based on LC based explicit CSI reporting proposed in [8], and illustrated in Figure 1. For each SB, the dominant eigenvector is represented as e≈l=07clblϕlbl, where ϕl belongs to {1,j,-1,-j}; b0, b1,…,b7 are L = 8 DFT beams forming (4,2) beam group (as shown in Figure 1); and the (un-quantized) coefficients cll=07=B*B-1B*e, where B=[b0, b1,…,b7].

Figure 1: LC based explicit CSI reporting

- Ideal: the dominant DL eigenvector(s) is (are) known at the eNB (performance upper bound).

The non-full-buffer system-level evaluation is carried out for UMa-200m and UMi-2GHz channel models in medium (50% target RU) traffic loading scenarios, and dynamic switching between SU-MIMO and MU-MIMO is considered in the simulation. For each scenario, three sets of results are provided for 16, 32, and 64 antenna ports with (N1,N2) = (2,4), (4,4), and (8,4), respectively, where we assume that the first dimension is horizontal and the second dimension is vertical. The detailed results are provided in Table 2 in the Appendix. The downtilt angles in the elevation domain are chosen according to [9]. In these simulations, partial-port non-precoded CSI-RS associated with 16 antenna ports is used for CSI estimation, and the corresponding CSI-RS overhead is taken into account in the final throughput calculation. Note that the CSI-RS overhead is the same for 16, 32, and 64 antenna ports. An illustration of partial port CSI-RS is shown in Figure 2 in which the partial antenna ports carrying CSI-RS are shown in black. Cell association antenna pattern is approximated by one-TXRU pattern, and proportional fair scheduling (max 4 layers per time-frequency resource) have been used. For MU-MIMO, SLNR precoding is considered. The relevant simulation parameters are enlisted in Table 1. The rest of the simulation assumption is according to [9].

Figure 2: Partial port CSI-RS

The performance gains for 16 antenna ports with “Class A” as reference are shown in Figure 3 and Figure 4 for UMa-200m and UMi-2GHz, respectively. Similar results for 32 and 64 antenna ports are shown in Figure 5 – Figure 8. From these results, we can make the following observation.

Observation 1:

- Large performance gap between Rel. 13 Class A codebook based implicit and ideal CSI schemes; the gap reduces with LC codebook [5-7] based implicit CSI scheme; however, the performance gap is still large.

- LC based explicit CSI scheme, on the other hand, can reduce this gap significantly.

Figure 3: Performance gain: 16 ports, UMa-200m

Figure 4: Performance gain: 16 ports, UMi-2GHz

Figure 5: Performance gain: 32 ports, UMa-200m

Figure 6: Performance gain: 32 ports, UMi-2GHz

Figure 7: Performance gain: 64 ports, UMa-200m

Figure 8: Performance gain: 64 ports, UMi-2GHz

We also compare the performance of increasing number of antenna ports from 16 to 64 with Class A for 16 ports as reference. The comparison is shown in Figure 9 and Figure 10. We can make the following observation.

Observation 2: The performance gain with LC based explicit CSI scheme is maintained as number of antenna ports increases from 16 to 64.

Figure 9: Performance gain: UMa-200m

Figure 10: Performance gain: UMi-2GHz

Since the gap between implicit and ideal CSI reporting schemes is large, which can be reduced significantly with LC based explicit CSI reporting, we propose that NR MIMO should support advanced CSI reporting schemes belonging to the category of explicit CSI reporting.

Proposal 1: NR MIMO supports explicit CSI reporting.

# Conclusions

In this contribution, simulation results comparing implicit and explicit CSI reporting schemes for NR MIMO are presented. The observations and proposals made are summarized as follows.

Observation:

- Large performance gap between Rel. 13 Class A codebook based implicit and ideal CSI schemes; the gap reduces with LC codebook [5-7] based implicit CSI scheme; however, the performance gap is still large.

- LC based explicit CSI scheme, on the other hand, can reduce this gap significantly.

- The performance gain with LC based explicit CSI scheme is maintained as number of antenna ports increases from 16 to 64.

Proposal: NR MIMO supports explicit CSI reporting.

# References

- RAN1#86, Chairman’s notes

- R1-1609087, “CSI acquisition for DL NR MIMO,” Samsung.

- R1-1609088, “CSI acquisition for DL NR MIMO,” Samsung.

- R1-1609089, “Advanced CSI feedback for NR,” Samsung.

- R1-1609012, “Linear combination W1 codebook,” Samsung.

- R1-1609013, “Linear combination W2 codebook,” Samsung.

- R1-1609014, “Rank 2 linear combination codebook,” Samsung.

- R1-166735, “LC based explicit CSI feedback and simulation results,” Samsung.

- 3GPP, TR36.897v1.0.1, “Study on Elevation Beamforming/Full-Dimension (FD) MIMO for LTE”

# Appendix: Simulation Assumptions

Table 1: Simulation Parameters

| Parameters | Values |
| --- | --- |
| Simulation Type | Non-full-buffer (Medium load 50% Target RU, Lambda = 4) |
| Channel model | UMi-2GHz, UMa-200m |
| Number of BS (H,V) antenna elements | (8,8), x-polarized, subarray partition |
| (N1,N2, P) | 16, 32, 64 ports: (4,2,2), (4,4,2), (8,4,2), |
| (O1,O2) | (8,8) |
| BS (H,V) antenna spacing | (0.5, 0.8)λ |
| BS and MS antenna polarizations | BS: (+45°,-45°); MS: (0°, 90°) |
| Number of UE antennas | 2 |
| SU/MU pre-coding | SLNR |
| Scheduling | MU, Proportional fair, up to 4 layers |
| Channel estimation | Ideal |
| Transmission rank | 1,2 |
| Receiver | MMSE-IRC |
| CSI feedback schemes | Reference: Rel. 13 Class A Codebook-Config = 1 LC codebook: proposed in [5-7], Codebook-Config = 3 LC Explicit: 8 beams, (4,2) beam group Co-phase: QPSK for each beam Coefficient: Unquantized Ideal: dominant eigenvectors are known to the eNB |

Table 2: Summary of non-full-buffer simulation results

| Channel | #ports | Scheme | Avg. UPT | 50% UPT | 5% UPT | Avg. UPT gain | 50% UPT gain | 5% UPT gain | RU |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UMa-200m | 16 | Class A | 18.84 | 16.00 | 5.56 | 100.0% | 100.0% | 100.0% | 59.6% |
|  |  | LC Implicit | 20.80 | 18.23 | 6.82 | 110.4% | 113.9% | 122.7% | 55.7% |
|  |  | LC Explicit | 25.87 | 22.73 | 9.47 | 137.3% | 142.0% | 170.3% | 48.9% |
|  |  | Ideal | 26.97 | 23.15 | 10.16 | 143.1% | 144.7% | 182.9% | 47.6% |
|  | 32 | Class A | 20.58 | 17.70 | 7.07 | 109.2% | 110.6% | 127.2% | 56.0% |
|  |  | LC Implicit | 22.30 | 20.10 | 8.27 | 118.3% | 125.6% | 148.7% | 52.8% |
|  |  | LC Explicit | 26.93 | 24.10 | 10.19 | 142.9% | 150.6% | 183.4% | 46.8% |
|  |  | Ideal | 29.56 | 27.03 | 11.45 | 156.8% | 168.9% | 206.0% | 43.9% |
|  | 64 | Class A | 20.83 | 17.89 | 6.76 | 110.5% | 111.8% | 121.7% | 55.4% |
|  |  | LC Implicit | 22.44 | 20.16 | 8.12 | 119.1% | 126.0% | 146.2% | 52.5% |
|  |  | LC Explicit | 26.79 | 24.26 | 10.15 | 142.2% | 151.6% | 182.6% | 47.2% |
|  |  | Ideal | 30.69 | 28.19 | 11.77 | 162.8% | 176.2% | 211.7% | 43.1% |
| UMi-2GHz | 16 | Class A | 18.69 | 15.81 | 5.91 | 100.0% | 100.0% | 100.0% | 60.6% |
|  |  | LC Implicit | 20.87 | 18.27 | 7.09 | 111.7% | 115.5% | 120.0% | 56.6% |
|  |  | LC Explicit | 27.41 | 24.67 | 9.46 | 146.7% | 156.1% | 160.1% | 48.6% |
|  |  | Ideal | 28.88 | 26.14 | 10.13 | 154.6% | 165.4% | 171.4% | 47.2% |
|  | 32 | Class A | 21.51 | 18.38 | 7.29 | 115.1% | 116.2% | 123.3% | 55.3% |
|  |  | LC Implicit | 23.33 | 21.02 | 8.54 | 124.8% | 132.9% | 144.5% | 52.6% |
|  |  | LC Explicit | 29.45 | 27.21 | 10.69 | 157.6% | 172.1% | 180.9% | 45.6% |
|  |  | Ideal | 32.67 | 32.13 | 12.08 | 174.8% | 203.2% | 204.3% | 42.6% |
|  | 64 | Class A | 21.74 | 18.86 | 7.20 | 116.3% | 119.3% | 121.9% | 54.7% |
|  |  | LC Implicit | 23.37 | 21.38 | 8.48 | 125.0% | 135.2% | 143.6% | 52.0% |
|  |  | LC Explicit | 29.23 | 27.21 | 10.80 | 156.4% | 172.1% | 182.8% | 45.2% |
|  |  | Ideal | 34.24 | 34.78 | 13.21 | 183.2% | 220.0% | 223.5% | 40.2% |