3GPP TSG RAN WG1 #86-bis	R1-1609098

# Lisbon, Portugal, 10th – 14th October 2016

Agenda item:	8.1.4.2

Source: 	Samsung

Title: 	Discussions on CSI-RS design for NR MIMO

Document for:	Discussion and Decision

Introduction

In RAN1#85 and RAN1#86, the following agreements on CSI-RS were made [1], [2]:

Agreements: - In NR multi-antenna schemes, studies on RS design and CSI acquisition considering following use cases - Non-UE-specific RS use case for CSI measurement - UE-specific RS use case for CSI measurement - Note: functionally may be analogous e.g.: to R13/14 UE-specific beamformed CSI-RS (with dynamic beamforming) - Note: maybe relevant to UE receiving beam sweeping - Use of a joint operation among multiple RS from the same or different use cases - RS for interference measurement use cases - RS for channel reciprocity use cases - Other use cases is not precluded - Strive to design a unified CSI framework, avoiding introducing multiple classes/subclasses and redundant (equally performing) configurations, while still covering a wide variety of use cases and frequency bands - Coupling/Decoupling (e.g. fixed timing relationships, joint configuration) between the following functions should be studied - RS transmission used for CSI acquisition (CSI-RS transmitted in DL and SRS transmitted in UL) - Use of other RS(s) is not precluded (e.g., DMRS) - Note that CSI-RS and SRS may or may not have the same physical signal design - Note that the reference signal naming can be revisited later - CSI measurement/reporting - Multi-antenna transmission method/scheme - Downlink control signaling - Study flexible scheduling/configuration of CSI-RS, CSI report and transmission method/scheme for data and control - At least one of following RS configurations for CSI measurement are supported in NR - Aperiodic RS - Semi-persistent RS - Periodic RS - Study RS pattern for CSI measurement in NR both for DL and UL - Study the density of RS - Study the position of RS in the [subframe] - Study the mapping of RS in one or several symbols - For NR, support DL CSI measurement with X antenna ports - Study the values of X = 1,2,4,8,12,16,20,24,28,32 - Study whether additional values of X are needed, including X > 32 (e.g., 64) - Other values of X are not precluded

This contribution provides Samsung’s views on design principles of CSI-RS for NR MIMO, focusing on the highlighted parts above.

Basic design principles

As we discussed in our companion contribution [3], modular CSI framework is desirable to support various use cases of NR within a unified CSI framework. CSI configuration of modular CSI framework comprises following three components: CSI reporting, RS, and CSI measurement. Among these three components, we will focus on the design principle of the RS setting/configuration in this contribution. Streamlining the framework for CSI-RS in LTE-A Pro is a good starting point of NR CSI-RS design. Based on this, we can avoid introducing multiple classes/subclasses and redundant configurations while keep its various features.

For the non-UE-specific RS use case for CSI measurement, functionalities analogous to LTE NP CSI-RS and cell-specific BF CSI-RS (both composed of one or more static macro-beams) can be combined into one type, termed the first-level CSI-RS which is composed of K≥1 CSI-RS resources. Each of the K cell-/group-specific beam(s) represented by each of the K CSI-RS resources can cover relatively wide coverage. As evident, the use cases of the first-level CSI-RS are the combination of use cases of Rel.13 NP CSI-RS and cell-specific BF CSI-RS. When used for virtual sectorization, a UE can be configured with CRI feedback. In addition, partial-port CSI-RS (CSI-RS subsampling in port-domain) can be implemented by configuring a UE to measure K > 1 CSI-RS resources, each resource representing a part of the larger port array. The second type of CSI-RS, analogous to Rel.13 UE-specific BF CSI-RS, is termed the second-level CSI-RS. Sharing the same characteristics, second-level CSI-RS can be composed of UE-specific beams and provides high penetration at relatively narrow coverage than the first level CSI-RS. Table 1 summarizes the comparison between these two CSI-RS types.

Table 1. Dual-level CSI-RS

| System aspect | First level CSI-RS | Second level CSI-RS |
| --- | --- | --- |
| CSI-RS resource usage | Non-UE-specific RS use cases for CSI measurement. E.g., one CSI-RS resource per “coverage beam”, each associated with nk (1,2,4,6,.., 32) CSI-RS ports, K ≥ 1 CSI-RS resources | UE-specific RS use cases for CSI measurement. E.g. one CSI-RS resource per UE with m (2,4,6,8) ports, m ≤ nk |
| CSI-RS “beams” | Cell-/group-specific beam direction(s) with wide coverage | UE-specific beam direction(s) with narrow coverage |
| Use cases | - Virtual sectorization - “Partial port” CSI-RS mapping | - UE-specific beamformed CSI-RS |

Figure 1 describes the above dual-level CSI-RS design. A gNB transmits two first-level CSI-RSs on two cell-specific beams to two UEs. These two cell-specific beams 0 and 1 (hence two first-level CSI-RSs) can be transmitted simultaneously or multiplexed in time or frequency. The gNB transmits two second-level CSI-RSs 0.0 and 0.1. The two UE-specific beams utilized for transmitting the two second-level CSI-RSs are derived relative to the first cell-specific beam and hence correspond to subspaces of the cell-specific beam. Since these two UE-specific beams are transmitted along a smaller range of directions, they have higher penetration due to additional beamforming gains. The second UE measures second-level CSI-RS and receives data transmission from the gNB along a data beam 0.1.0. Beamforming vector for the data beam can be formed either from a CSI reporting derived from the second-level CSI-RS (more relevant for FDD) or measuring UL SRS (more relevant for TDD). Multiple CSI-RS resource components per each CSI-RS level can be studied for ease of beam direction acquisition considering operation scenarios according to the various environments such as granularity of virtual sectorization, UE mobility, etc.

Figure 1. NR MIMO operation based on dual-level CSI-RS

Considering various aspects such as efficient resource utilization and interference management, on-demand transmission of CSI-RS seems to be a primary operational mode. On the other hand, on-demand transmission of CSI-RS may cause burdensome signaling overhead if the UE needs to report CSI only for link maintenance. In this case, periodic CSI-RS transmission might be beneficial for both of gNB and UE complexities. As a hybrid type of both configurations, semi-persistent CSI-RS configurations can be proper in case that moderate data rate and transmission periodicity are required, e.g. for voice service.

Support of forward compatibilities and co-existence with LTE/LTE-A is another important issue in NR. CSI-RS patterns based on small time/frequency granularity might be a good starting point to cover both of forward compatibilities and co-existence with LTE/LTE-A. For instance, given that NR may have large dynamic range in time domain from multiple TTIs and/or numerologies, short time duration of CSI-RS pattern can be good for ease of RS interference management. Therefore, small time/frequency granularity such as CSI-RS patterns within one OFDM symbol can be considered.

Based on above discussions, we propose following CSI-RS design principles for NR:

Proposal: Following CSI-RS design principles should be considered for NR

A dual-level CSI-RS structure includes:

A First level CSI-RS to covers ‘non-UE-specific RS use cases’ for CSI measurement.

The beam direction(s) for the CSI-RS can be cell-/group-specific; and the CSI-RS beam(s) can provides wide coverage

A Second level CSI-RS to covers ‘UE-specific RS use cases’ for CSI measurement.

The beam direction(s) for the CSI-RS can be UE-specific; and the CSI-RS beam(s) can provides narrow coverage

Aperiodic/periodic/semi-persistent type CSI-RS configurations

Support of high flexibility via small time/frequency granularities

E.g., CSI-RS can be transmitted within one OFDM symbol

Conclusions

In this contribution, Samsung’s view of CSI-RS for NR is presented. Based on above discussions, the following proposals are made:

Proposal: Following CSI-RS design principles should be considered for NR

A dual-level CSI-RS structure includes:

A First level CSI-RS to covers ‘non-UE-specific RS use cases’ for CSI measurement.

The beam direction(s) for the CSI-RS can be cell-/group-specific; and the CSI-RS beam(s) can provides wide coverage

A Second level CSI-RS to covers ‘UE-specific RS use cases’ for CSI measurement.

The beam direction(s) for the CSI-RS can be UE-specific; and the CSI-RS beam(s) can provides narrow coverage

Aperiodic/periodic/semi-persistent type CSI-RS configurations

Support of high flexibility via small time/frequency granularities

E.g., CSI-RS can be transmitted within one OFDM symbol

- References

- 3GPP, RAN1#85, Chairman’s Notes

- 3GPP, RAN1#86, Chairman’s Notes

- R1-16xxxx,	CSI acquisition for DL NR MIMO,	Samsung