3GPP TSG-RAN1#86bis	              R1-1609549

Lisbon, Portugal, 10 – 14 October 2016

Source: 	ETRI

Title: 	Comparison of OFDMA vs LSSA system level UL performance

Agenda Item:	8.1.1.2

Document for:	Discussion / Decision

1	Introduction

UL SLS baseline design for providng reference system level performance was agreed during RAN #86 meeting, and evaluation assumptions for new radio interface have been discussed.  For UL mMTC scenario using OFDMA multiple access scheme, following were the reached consensus so far [1]:

Agreements:

- For providing absolute system level evaluation results for UL mMTC, for calibration purpose only

  - CP-OFDM as the UL waveform

    - UL DMRS overhead, 1 OFDM symbol out of 7 OFDM symbols

  - A UE selects a MA physical resource randomly from a pool of orthogonal MA physical resources

    - There is no partial overlapping between the MA physical resources selected by more than one UE

    - All orthogonal MA physical resources are of same size

  - Rx MMSE receiver, assuming realistic channel estimation

    - No blind decoding assumed

  - Same MCS for all UEs,

    - MCS is reported by each company

  - Open loop power control

    - Alpha=1, P0= -90 dBm

  - Packet size is fixed by 20 bytes

  - FFS other parameters, if any

    - Email discussion on any other potential parameters until 9/2 – Yi Wang  (Huawei)

Note: The above assumptions only apply to the calibration purpose, i.e. other assumptions can be used for evaluation of proposed non-orthogonal multiple access scheme(s)

In relation to the contribution [2], the proposed non-orthogonal MA is evaluated via system level simulation performance under the agreed SLS parameters/environment, and it is compared with the baseline SLS.  First we explain how the PHY abstraction method was done, and then we list some findings from system level evaluation results according to the latest evaluation methodology and evaluation assumption agreement defined in [1].

2	PHY abstraction method	for	LSSA UL

PHY abstraction method for interference cancellation with linear MRC receiver type of non-orthogonal MA scheme is proposed for SLS evaluation.  LSSA employs parallel interference canceller, which assumes the overall cancellation effect to be reliable as SINR increases.  As verified in our contribution [2], this assumption is found generally true, and perfect interference cancellation is expected especially at high SINR operating region.  For LSSA, the proposed approach to calculate post processing SINR is to figure out the perfect interference cancellation lower bound.  As shown in [2], almost no performance loss, 200% overloaded and low-rate channel code case for example,  is observed as a result of employing parallel interference cancallation unless the overload factor exceeds a certain threshold.

The post-processing SINR of the orthogonal MA receiver with MRC for the n-th RB can be given as [3]

SINR0n=Ptx0Ploss0r=0NR-1Hr0n2σ2+j=1NjPtxjPlossjr=0NR-1Hr0nHHrjn2r=0NR-1Hr0n2,               (1)

where Ptx(0) and Ploss(0) are tramit power and path loss from a target user 0.   Hr(0) is the SIMO small scale fading channel in matrix from the same target user 0.  σ2 is the thermal noise plus noise figure at eNB.  The rest of the notations with j-th index indicates interfering users from other cells.  On the other hand, the post-processing SINR of the non-orthogonal LSSA MA receiver for the n-th RB can be given as

SINR0(n)=βγ,ε,δPtx0Ploss0r=0NR-1Hr0n2σ2+j=1NjPtxjPlossjr=0NR-1Hr0nHHrjn2 .     (2)

Unlike the baseline OFDMA scheme with MRC type receiver, the LSSA receiver usually has some strong interferences removed completely before LLR computation and/or channel decoding process is done.  The power of residual interference is close to the thermal noise level, and the SINR can be modelled as somewhat similar to interference cancelled SIMO channel.    βγ,ε,δ is the effective SINR adjustment value for particular input SINR range γ with particular channel coding rate ε and overloading conditions δ.  If the overload factor exceeds a certain threshold, a BLER performance gap is observed.  However, depending on the effective SINR range and channel coding rate, the performance gap between single user perfect cancellation lower bound case and overloaded transmission factor can be taken into account, as marked as γ,ε,δ in (2).

The performance gap can be converted into SINR loss, and this SINR loss can be a function of βγ,ε,δ.  However, the SINR adjustment factor βγ,ε,δ can be trained to have an approximate single value, when realistic channel estimation assumption is applied, regardless of overload and channel coding rate conditions.

3		Comparison of OFDMA vs	LSSA UL PDR performance

The parameters on system level simulation for UL mMTC scenario in Urban Macro coverage is defined as  following:

Table 1 Applied system level simulation parameters for UL mMTC scenario

| Attributes | Values or assumptions |
| --- | --- |
| Layout | Single layer - Macro layer: Hex. Grid |
| Inter-BS distance | 1732m |
| Carrier frequency | 700 MHz |
| Simulation bandwidth | Companies report simulation bandwidth used in evaluation |
| Channel model | 3D UMa Take 5GCM output into account if applicable. |
| Tx power | UE: Max 23dBm |
| BS antenna configuration | Rx: 2 and 4 ports |
| BS antenna pattern | Follow the modeling of TR36.873 |
| BS antenna height | 25m |
| BS antenna tilt | 102 degrees |
| BS antenna element gain + connector loss | 8 dBi, including 3dB cable loss |
| BS receiver noise figure | 5 dB |
| UE antenna elements | 1Tx |
| UE antenna height | 1.5m |
| UE antenna gain | -4dBi |
| Traffic model | Poisson with arrival rate Lamda of arrivals/ms/sector |
| UE distribution | 20% of users are outdoor (3km/h) 80% of users are indoor (3km/h) Users dropped uniformly in entire cell |
| BS receiver | MMSE-IRC as baseline, Advanced receiver is not precluded |
| UL power control | Alpha = 1, P0 = -90dBm |
| Channel estimation | Ideal |

Table 2 SLS assumptions for LSSA system

| Attributes | Values or assumptions |
| --- | --- |
| Waveform | - CP-OFDM as the UL waveform - UL DMRS overhead, 1 OFDM symbol out of 7 OFDM symbols |
| Resource allocation | - A UE selects a MA signature resource randomly from a pool scheduled by eNB - There is no partial overlapping between MA physical resources selected by more than one UE - - Total allocated bandwidth: 6RB |
| Receiver | - No blind decoding assumed - |
| MCS | - QPSK is assumed - TB size of 160bit including CRC |
| HARQ | - No retransmission |
| Carrier frequency | 700 MHz |
| Channel model | 3D UMa Take 5GCM output into account if applicable. |
| Tx power | UE: Max 23dBm |
| BS antenna configuration | Rx: 2 ports |
| BS antenna pattern | Follow the modeling of TR36.873 |
| BS antenna height | 25m |
| BS antenna tilt | 102 degrees |
| BS antenna element gain + connector loss | 8 dBi, including 3dB cable loss |

The downlink performance of LSSA system for Urban Macro scenario is considered.  For a comparison of system outage/connection efficiency result, packet drop rate (PDR) vs system loading (packet arrival rate: PAR) is evaluated.

For baseline OFDMA system, channel estimation is set to ideal case.  Thus, even though UL transmission collision in the same RB (belonging to the same sector) happens, collided packet is considered not as a direct packet drop event, but regarded as strong interference to each received target data.

- TB size of 160 occupying 1 subframe                     (b) TB size of 160 occupying 2 subframes

Figure 1. Fixed size TB PAR vs PDR comparison of OFDMA and LSSA

In both figure 1 (a) and (b), OFDMA has lower packet drop rate than LSSA in low traffic loading condition, while the situation is reversed as packet arrival rate is increased.   This For figure 1 (a), low PDR is observed for OFDMA if traffic loading condition is low.   On the other hand, wide-band transmission of LSSA effectively introduces larger noise power density per RB and smaller transmission power per RB than OFDMA.  This contributes to higher PDR at low PAR rates, which corresponds to worse SINR condition per RB compared to OFDMA.  In addition, on top of being able to transmit higher power per given bandwidth, the receiver for OFDMA could expect less UL interference as the UE randomly chooses a MA physical resource.  However, the merits of OFDMA fades away when PAR starts to increase, as number of other UL signals contributing to interference and collision (modelled as strong interference) among UL signals sharing the same RB increases.   However, the merit comes from the high overloaded conditions for LSSA.  As shown in figure 1, non-orthogonal scheme is more efficient than baseline OFDMA for packet arrival rates higher than 2.5.

Figure 2 Fixed size TB PAR vs PDR with time repeated transmission

On the other hand, the PDR is way too high to meet the target PDR for both 1 or 2 subframe transmission case.   Even though resource of 2 subframes are used, it is still far from reaching the target outage performance of PDR 1%.   For LSSA, roughly 10% reduction of PDR is observed for doubling the MA physical resource.  Given the bandwidth limitation, time diversity must be considered to achieve PDR of 10%.  From the figure 2 above, it is analyzed that at least more than 20ms of transmission time is required.   Also, increased capability of overloading must be considered to compensate the reduced time resource.

4	Discussion / conclusion

In this contribution, we presented the SLS evaluation results of LSSA non-orthogonal MA scheme.  A comparison of the performance metric between OFDMA and LSSA suggest that each candidate MA scheme has their strong and weak points in particular traffic condition scenarios.  In conclusion the following proposal is made:

Observation: Capacity of connection density depends on traffic loading conditions, and multiple access scheme must be selected based on traffic conditions.   If high traffic loading condition is the case, non-orthogonal MA with high overloading capability can be more efficient, in terms of connection efficiency.

5	References

- 3GPP Chairman Notes, RAN1 #86, Gothenburg, Sweden, August 22-26, 2016.

- R1-1609548, “DMRS design for non-orthogonal UL multiple access user channel estimation”, ETRI, October 2016.

- IEEE 802.16m-08/004r3, “IEEE 802.16m Evaluation Methodology Document (EMD)”, July 2008.