3GPP TSG-RAN WG1#86bis	R1- 1609684

Lisbon, Portugal, 10-14 October, 2016

Source:	Ericsson

Title:	UE reporting of interruption time for SRS in carrier based switching

Agenda Item:	7.2.5.1

Document for:	Discussion and Decision

# Introduction

In RAN1#86, the following agreements were reached on UL DMRS [1]:

Agreement:

- UE reports the switching time/interruption time for SRS switching

  - FFS: Per band-pair or Per CC-pair

  - FFS: set of potential values

In the following, interruption time classes are introduced. In-band and Intra band switching time reporting is discussed.

# Use case for interruption time

## Use of interruption classes

The most obvious use of a classification of Ues by interruption time is to have a different solution for handling interruption in each class and optimize that solution to minimize lost transmission opportunities. For example, one could have a class for interruption time less than the maximum interruption that can be handled by the special subframe, or 5 symbols  (class 1), less than a slot (class 2), less than a subframe (class 3) or more than a subframe (class 4).

Subframe level shortening (legacy)

Symbol level shortening

Slot level shortening

CLASS 4

CLASS 3

CLASS 2

CLASS 1

Interruption time

1 subframe

5 symbols

1 slot

Each class can then be associated with rules for SRS transmission in the special subframe and shortening / dropping of PUSCH/PUCCH and associate designs for UCI and PDCCH / PDSCH.

## SRS Prioritization

Another application is prioritization. Class could be given a priority in SRS switching transmission. Is a class 1 UE and a class 4 UE compete for SRS transmission, clearly class 1 Ues have a lower system penalty.

# Interruption time reporting type

## Type 1: Per CC pair

Each CC pair may have an associated interruption time, or class.  This type of report is very detail and should only be used if it clearly shows gains compared to other options.

## Type 2: Per band pair

Each CC in a band is assumed to have the same interruption time for intra-band switching. Intra-band switching is defined as a SRS switching where the source and the destination carrier share the same band. It is also assumed that the UE has different interruption times for different band pair in an inter-band switching. Inter-band switching is defined as a SRS switching where the source and the destination carrier are not in the same frequency band.  Therefore, each band pair as well as intra-band interruption time for each band is given a class of interruption time.

Proposal: introduce interruption time reporting in classes for each band pair and within each band.

Proposal: interruption time classes are as follow: class 1 0 to 350usec, class 2 350 to 500usec, class 3 500 to 1000usec, class 4 1000usec and above.

## Interruption time report

Interruption time should be reported along with other UE capability at connection. It is therefore a RAN2 issue.

Observation: RRC aspects of interruption time reports should be handled by RAN2

# Conclusions

This contribution discusses the reporting and classification of interruption times for carrier switching. The following proposals have been made:

Proposal: introduce interruption time reporting in classes for each band pair and within each band.

Proposal: interruption time classes are as follow: class 1 0 to 350usec, class 2 350 to 500usec, class 3 500 to 1000usec, class 4 1000usec and above.

# References

Chairman Notes RAN1 86, göteborg, Sweden, 2016.