# 3GPP TSG-RAN1#86bis	R1-1610399

# 10th – 14th October 2016

# Lisbon, Portugal

Agenda Item:	7.2.9.4

Source: 	Ericsson

Title:  	Power consumption and latency reduction by increasing the maximum NPUSCH transport block size

Document for:	Discussion

# Introduction

One of the objectives of the REL-14 NB-IoT enhancements work item is to reduce power consumption and latency. This can be achieved by supporting 2 HARQ processes in both downlink and uplink, and increasing the maximum transport block size (TBS) of NPDSCH and NPUSCH. [1]

In this contribution, we discuss the benefits of increasing the maximum NPUSCH TBS in terms of increases in average throughput. The increase in average throughput results in lower latency and improvement in UE power efficiency.

# Rel-13 limitation

In Rel-13, only a single HARQ process is supported for both downlink and uplink. For the uplink, the timing relationship, NPDCCH search space periodicity, and maximum TBS of NPUSCH altogether limit the throughput and latency in the uplink.

The following timing relationships are considered.

- The UE search space is configured to have Rmax=2, and G=4. And thus the search space period is T=8. See Figure 1. One search space in this case consists of 2 valid subframes.

- A 8 ms gap exists between the NPDCCH DCI and the scheduled NPUSCH.

- A 3 ms gap between NPUSCH and the NPDCCH DCI. When such time relationship is satisfied, but the subframe is taken by NPBCH/NPSS/NSSS, the NPDCCH DCI is postponed to the next available subframe as long as there is at least 4 ms gap before the next NPDCCH search space.

Figure 1: Search space definition. (Rmax=2, and G=4.)

An example of scheduling a series of uplink transport blocks of maximum size 1000 bits for a UE in good radio conditions is shown in Table 1. Each row represents 20 subframes (SFs) with the top row indicating the subframe numbers. The signals carried in the first 20 subframes are indicated in the 2nd row, and the subsequent rows indicate the signals carried in subsequent subframes. The subframes indicated with “C” are subframes with NPDCCH DCI for scheduling the NPUSCH. The subframes indicated with “U” are NPUSCH subframes. NPBCH, NPSS, and NSSS subframes are indicated by “B”, “P”, and “S”, respectively.

It is seen that in this case, NPBCH/NPSS/NSSS result in an increase in latency due to the postponement of DCI. The spacings between DCI’s are 16 ms, 16 ms, 18 ms, 21 ms, 16 ms, 16 ms, etc. The average latency for a large number of transport blocks is actually close to 17.2 ms, and thus the average uplink throughput is 58.3 kbps. Note that if collision with NPBCH/NPSS/NSSS has not been accounted for, the average latency and throughput for delivering a series of uplink transport blocks of maximum size 1000 bits would have been 16 ms and 62.5 kbps, respectively.

Observation 1: The maximum uplink throughput in Rel-13 with Rmax=2, and G=4 is limited to approximately 58.3 kbps.

Table 1: An example of delivering a series of uplink transport blocks of maximum size 1000 bits for a UE in good coverage. C: subframes carrying DCI, U: subframes carrying NPUSCH, B: NPBCH, P: NPSS, S: NSSS.

# Benefits of increasing maximum transport block sizes

When the maximum TBS increases, the number of resource units (NRU) that is needed to carry the transport block may also increase. In the below discussion, we consider 12-tone NPUSCH format, which has 1 ms resource unit. In one resource unit, there are 144 symbols, and 288 bits with QPSK modulation. Thus,

NRU=TBS288cmax,

where cmax is the maximum coding rate. We assume cmax=0.93. Note that in Rel-13, NRU can only be 1, 2, 3, 4, 5, 6, 8, or 10. To minimize the impact on specifications, e.g. allowing reusing the same Rel-13 DCI format, we propose that the same set of NRU is used in Rel-14. In this case, if TBS288cmax becomes 7 or 9, NRU will take value of 8 or 10, respectively.

Proposal 1: Rel-14 reuses the same mapping for determining the number of resource units (NRU∈{1,2,3,4,5,6,8,10}) according to the resource assignment field (IRU) in the DCI

The average UL throughput as a function of TBS is shown in Figure 2. It is seen that, increasing the maximum TBS may not always give rise to an increase in the UL throughput. This is because that an increase in NRU may result in needing to skip one extra period of NPDCCH search space in order to meet the required timing relationship.

Observation 2: Increasing the maximum TBS may not always give rise to an increase in UL throughput.

A good value is TBS=1576, which increases the maximum uplink throughput by 35% compared to Rel-13 and avoid extra latency caused by extra NPDCCH search period. As seen in Figure 2, increasing TBS beyond 1576 bits has diminishing return in improvement in uplink throughput.

The Rel-13 NPUSCH TBS table is shown in Table 2. Note that the highlighted elements are not specified in Rel-13. Allowing the larger TBS up to 1576, we could consider the revised TBS table shown in Table 3.

Proposal 2: The maximum TBS of NPUSCH is increased to 1576 bits.

Figure 2: The average UL throughput as a function of TBS.

Table 2: Rel-13 NPUSCH TBS table. (the highlighted elements are not specified in Rel-13)

Table 3: Proposed NPUSCH TBS table for Rel-14.

# Conclusion

In this contribution, we discuss the benefits of increasing the maximum NPUSCH TBS in terms of increases in average throughput. The increase in average throughput results in lower latency and improvement in UE power efficiency. We make the following observations and proposals.

Observation 1: The maximum uplink throughput in Rel-13 with Rmax=2, and G=4 is limited to approximately 58.3 kbps.

Observation 2: Increasing the maximum TBS may not always give rise to an increase in UL throughput.

Proposal 1: Rel-14 reuses the same mapping for determining the number of resource units (NRU∈{1,2,3,4,5,6,8,10}) according to the resource assignment field (IRU) in the DCI.

Proposal 2: The maximum TBS of NPUSCH is increased to 1576 bits.

# References

- RP-161901, “Revised work item proposal: Enhancements of NB-IoT,” RAN#73, source Huawei, HiSilicon, 19 – 22 September, 2016.