3GPP TSG RAN WG1 Meeting #88                                                                                  R1-1701708

Athens, Greece, 13th - 17th February 2017

Agenda Item:	8.1.4.1

Source:	Huawei, HiSilicon

Title:	Performance evaluation of LDPC codes

Document for:	Discussion and Decision

# Introduction

As it was highlighted in the previous RAN1 Ad Hoc meeting [5], the following methodology is agreed for comparison of the codes:

Working Assumption:

- For the purpose of H matrix design, minimum Code Block Size (including CRC) in the range 40-100 bits should be considered

Conclusion:

- Evaluations at BLER of a single code block = 1e-2 (for performance comparison between codes) and 1e-4 (for the purpose of comparing the error floor performance of the codes)

  - (Note that this does not preclude other comparison criteria)

In this contribution, we present the BLER performance evaluation of multiple LDPC schemes at levels 10-2 and 10-4 for all the code rate and codeword length of eMBB data channel.

# Performance evaluation

The simulation set-up is provided in Table 1. The following codes were selected for comparison (see Figure 1):

- Code A (only high family is used due to best performance) [1]

- Code B [2]

- Nested NC-QRO LDPC code [3]

In all 3 cases layered decoder processes the PCM from bottom to top order of rows.

Table 1 Simulation assumptions

| Channel | AWGN |
| --- | --- |
| Modulation | QPSK |
| Coding Scheme | LDPC |
| Code rate | 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9 |
| Decoding algorithm | Flooding BP (max_iter = 50), Layered offset MS (max_iter = 15, scale=1, offset=1/2) |
| Info. block length (bits w/o CRC) | 32, 48, 64, 80, …, 8192 |

Figure 1. Performance results, Es/N0(dB) at BLER=10-2, QPSK, BP decoder, 50 iterations.

From the figures above, it can be observed that the NC-QRO code has better performance than code A for any code rate and codeword length, particularly for code rate 0.2, where up to 0.2 dB gap can be observed. For code rate 8/9, the curve of the NC-QRO scheme is smoother than code A due to the finer granularity on the circulant values.

Figure 2. Performance results, Es/N0(dB) at BLER=10-2, QPSK, LOMS decoder, 15 iterations.

From the figures above, it can be observed that the NC-QRO code has comparable performance to code A over the code rates and codeword lengths, with the NC-QRO scheme having better performance for code rates smaller than 0.75 and code A having better performance for rates larger than 0.75. For code rate 8/9, the curve of the nested scheme is smoother than code A due to the finer granularity on the circulant values.

Figure 3. Performance results, Es/N0(dB) at BLER=10-4, QPSK.

From the figures above, it can observed that the proposed NC-QRO code has no error floor at BLER=10-4.  Code A shows an error floor for code rate ¾, and code B shows an error floor for code rate ¾ and below.

Implementation aspects for the three codes are discussed in [4], where it is shown that the NC-QRO code outperforms code A and B in terms of latency and hardware complexity trade-off.

Observation 1:  In terms of BLER, the NC-QRO LDPC scheme outperforms code A for low and high code rates, and outperforms code B for middle code rates.

Observation 2: The NC-QRO LDPC scheme with offset lifting method described in [3] has no error floor at BLER=1e-4.

# Conclusions

This contribution describes a design of QC LDPC code for eMBB. It is shown that the proposed LDPC code has good performance with fine-granularity for all code rates and codeword lengths of eMBB data channel.

In summary, the proposed design has the following characteristics:

Observation 1:  In terms of BLER, the NC-QRO LDPC scheme outperforms code A for low and high code rates, and outperforms code B for middle code rates.

Observation 2: The NC-QRO LDPC scheme with offset lifting method described in [3] has no error floor at BLER=1e-4.

# References

R1-166388, “LDPC rate compatible design”, Qualcomm Incorporated

R1-1612547, “Discussion on Channel Coding for eMBB Data Channel”, Samsung

R1-1701706, “LDPC design for eMBB data”, Huawei, HiSilicon

R1-1701707, “Implementation aspects of LDPC codes”, Huawei, HiSilicon

Chairman’s Notes, RAN1 AH, January 2017