3GPP TSG RAN WG1 Meeting #88	R1-1703347

Athens, Greece 13th – 17th February 2017

Agenda Item:	8.1.2.5

Source:	Huawei, HiSilicon

Title:	Discussion on evaluation assumption for beam management

Document for:	Discussion and decision

# Introduction

Beam management is a potential technology for NR MIMO enhancement. And the following aspects are identified as related to beam management study.

- Beam reporting (including CSI-RS design, beam reporting information design, etc.)

- Beam correspondence

- Beam recovery from beam failure (including control channel and data channel design on this aspect)

- UL beam management

- Etc.

In RAN1 NR AdHoc meeting in Jan 2017 in Spokane, beam management mechanisms are further discussed, and additional agreements are made to better support NR, e.g., [1]

Agreements: - (Beam recovery) - NR supports that UE can trigger mechanism to recover from beam failure - NR-PDCCH transmission supports robustness against beam pair link blocking - (Beam reporting) Beam reporting information at least include - Measurement quantities for N beam (s) - Information indicating N DL Tx beam(s), if N < K - (Beam correspondence) - Support capability indication of UE beam correspondence related information to TRP - (UL beam management) - For NR UL, support transmissions of SRS precoded with same and different UE Tx beams within a time duration

It is observed that many mechanisms are considered in beam management study, and different solutions may be proposed on this aspect. To study and evaluate the potential benefit that might be brought by those mechanisms, evaluation method and parameter need to be developed to enable the evaluation of different beam management mechanisms. This contribution provides the view on evaluation assumptions for NR beam management study.

# Discussion

It is observed that several aspects including beam reporting, beam correspondence design, beam recovery design, and UL beam management design are considered in beam management study. To evaluate the performance and potential gains of these mechanisms, link level simulation and system level simulation can be employed.

- Link level simulation could be conducted to evaluate the link performance, e.g.,

    - For beam reporting, the CSI-RS pattern and CSI-RS density design can be investigated with link level simulation.

  - In this case, the following evaluation metrics could be used

    - BLER vs SINR curve, and RSRP measurement margin

      - RSRP measurement margin is given by |RSRPideal – RSRPmea|, where RSRPideal indicates the RSRP measurement under ideal channel estimation, while RSRPmea indicates the RSRP measurement under real channel estimation according to CSI-RS.

      - “RSRP measurement margin” provides a measure of RSRP measurement error introduced by different CSI-RS pattern and density design.

- System level simulation could be conducted to evaluate the data transmission performance in multi-user multi-cell case.

  - In this case, the following evaluation metrics could be used

    - Spectral efficiency

    - Outage (as defined in Phase 2 calibration)

    - 5%-, 50% UPT

Furthermore, evaluation parameters need to be defined, which is further discussed as below.

## Consideration on SLS parameter

For system level evaluation, the following parameters are considered, which are based on the parameters employed in TR38.802 [2] and the simulation parameters employed in MIMO calibration. The red fonts are highlighted proposals which are different from that of [3].

Table 1 SLS parameter for NR beam management evaluation

| Attributes | Values |
| --- | --- |
| Scenarios (Carrier Frequency) | Indoor hotspot (carrier frequency 4GHz, 30GHz), Urban macro (carrier frequency 4GHz, 30GHz), Dense Urban (For 30GHz, evaluate micro layer only; for 4GHz, evaluate macro layer) - Macro layer: macro-only 10 users per TRP, all UEs are connected to macro layer. Users randomly and uniformly dropped throughout the macro geographical area - Micro layer: micro-only 3 micro BSs per macro BS and 10 users per TRP |
| Mode | SU-MIMO / MU-MIMO |
| System bandwidth | 4GHz: 20MHz (DL+UL) 30GHz: 80MHz (DL+UL) |
| Sub-carrier spacing | 4GHz: 15kHz 30GHz: 60kHz |
| Channel Model | Following related assumption in TR 38.802 |
| TXRU mapping to antenna elements | Companies explain the details of TXRU mapping to antenna elements (30GHz: 2D DFT based beam per polarization as a baseline 4GHz: 1D DFT per vertical dimension per polarization as baseline) |
| TXRU mapping weights | Companies explain the details of TXRU mapping weights. |
| Criteria for selection for serving TRP | Companies explain the details of criteria for selection for serving TRP. |
| Criteria for beam selection for serving TRP | Companies explain the details of criteria for beam selection for serving TRP. |
| Constraints for the range of selective beams per TRP sector | Companies report the details |
| Scheduling algorithm | PF scheduler |
| Traffic Model | FTP model 1/3 for UPT evaluation, with packet size 0.1 and 0.5Mbytes (other value is not precluded). Other traffic models including full buffer are not precluded. |
| BS antenna configurations | 30GHz: (M,N,P, Mg,Ng) = (4,8,2,2,2). (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (2.0, 4.0)λ 4GHz: (M,N,P, Mg,Ng) = (8,8,2,1,1) as baseline. (dV,dH) = (0.8, 0.5)λ. Note: important to consider also other antenna configurations to maintain flexibility |
| BS antenna element radiation pattern | According to TR36.873 and TR38.802 |
| UE antenna configurations | 30GHz: (M, N, P, Mg,Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. * Θmg,ng=90 deg; Ω0,1=Ω0,0+180 (deg); 4GHz: Up to 8Tx/Rx Note: important to consider also other antenna configurations to maintain flexibility |
| UE antenna element radiation pattern | 4GHz: Omni-directional with 5dBi gain 30GHz: See Table A.2.1-8 in TR 38.802 |
| Inter-panel calibration | Ideal, non-ideal following 38.802 (optional) |
| Beam correspondence calibration | Companies report details |
| Control and RS overhead | Reported by companies |
| UE receiver type | MMSE-IRC as baseline; other advanced receiver is not precluded. |
| BF scheme | Companies report details |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies report details. |
| UE mobility feature | Follow Phase 3 calibration; Add-on features including UE mobility, blockage, etc. could be considered. Note: Companies explain whether or which model is used in simulation evaluation. If used, the configuration details should be explained |
| Metric | - Spectral efficiency (evaluated under full buffer) - Outage - 5%-, 50% UPT (evaluated under FTP model) |

## Consideration on LLS parameter

For link level evaluation, the following parameters are considered, which are based on the parameters employed in TR38.802 and the simulation parameters employed in MIMO calibration. The red fonts are highlighted proposals which are different from that of [3].

It is noted that the link level channel model parameter proposal for delay spread and angular spread are according to the delay spread and corresponding angular spread in system level channel model as defined in Report ITU-R M.2135, 3GPP TR36.873, and TR38.900 [4~6] in Indoor Hotspot, Urban macro and Urban micro.

Table 2 LLS parameter for NR beam management evaluation

| Attributes | Values |
| --- | --- |
| Carrier Frequency | 30 GHz, 4GHz |
| Subcarrier Spacing | 30GHz: 60kHz 4GHz: 15kHz |
| Data allocation | 8 RBs - Note: Error free PDCCH decoding is assumed. - First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel |
| Channel Model | CDL-A /B/C model - delay spread =100ns - UE speed=3km/h. - The angles of BS, i.e., AoD, ZoD, are uniformly distributed within [-12.5, 12.5] degrees in azimuth domain and [103, 108] degrees in zenith domain, and those of UE, i.e., AoA, ZoA, are uniformly distributed within [-30, 30] degrees in azimuth domain and [80, 90] in zenith domain, via applying uniform-distribution desired mean angle in Section 7.7.5.1 in TR 38.900 accordingly. |
| TXRU mapping to antenna elements | Companies explain details of the using TXRU mapping to antenna elements. (30GHz:2D DFT based beam per polarization as a baseline 4GHz: 1D DFT per vertical dimension per polarization as baseline) |
| TXRU mapping weights | Companies explain details of the using TXRU mapping weights. |
| Procedure of beam sweeping | Companies explain details of procedure of beam sweeping. |
| Criteria for beam selection | Companies explain details of criteria for beam selection. |
| UE reporting | Companies explain details of criteria for UE reporting. |
| BS antenna configurations | 30GHz: (M,N,P, Mg,Ng) = (4,8,2,2,2). (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (2.0, 4.0)λ 4GHz: (M,N,P, Mg,Ng) = (8,8,2,1,1) as baseline. (dV,dH) = (0.8, 0.5)λ. Note: important to consider also other antenna configurations to maintain flexibility |
| BS antenna element radiation pattern | According to TR36.873 (for below 6GHz) and TR38.802 (for above 6 GHz) |
| UE antenna configurations | 30GHz: (M, N, P, Mg,Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. * Θmg,ng=90; Ω0,1=Ω0,0+180; 4GHz: Up to 8Tx/Rx Note: important to consider also other antenna configurations to maintain flexibility |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 0 degree, ΩUT,g = 0 degree |
| UE antenna element radiation pattern | 4GHz: Omni-directional with 5dBi gain 30GHz: See Table A.2.1-8 in TR 38.802 |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| MIMO mode | SU-MIMO / MU-MIMO |
| UE receiver type | MMSE-IRC as baseline; other advanced receiver is not precluded. |
| Metrics | - BLER vs SINR curve - RSRP measurement margin - RSRP measurement margin is given by ¦RSRPideal – RSRPmea¦, where RSRPideal indicates the RSRP measurement under ideal channel estimation, while RSRPmea indicates the RSRP measurement under real channel estimation according to CSI-RS |

# Conclusion

In the contribution the evaluation assumption including evaluation method, evaluation metrics, and evaluation parameters for NR beam management study are discussed. Based on the discussion, the following proposal is made.

Proposal: Adopt Table 1 and Table 2 for SLS and LLS evaluation for NR beam management study. Other SLS parameters are according to TR38.802.

# References

Chairman’s note RAN1-NR Adhoc, Jan 2017.

3GPP TR38.802 “Study on New Radio (NR) Access Technology Physical Layer Aspects”, Nov 2016.

R1-1701514, “Way Forward on Evaluation for Beam Management,” ZTE, etc., Jan 2017.

Report ITU-R M.2135, “Guidelines for evaluation of radio interface technologies for IMT-Advanced”.

3GPP TR36.873, “Study on 3D channel model for LTE”.

3GPP TR38.900 “Study on channel model for frequency spectrum above 6 GHz”.