3GPP TSG RAN WG1 Meeting #88		      R1-1703380

Athens, Greece 13th - 17th February 2017

Agenda Item:	8.1.2.5

Source: 	CATR

Title: 	Initial Calibration Results of NR MIMO

Document for:	Discussion and Decision

Introduction

In [86-20] email discussion, the following agreements on the phased approach for NR MIMO calibration have been made, Following the phased approach below for calibration:

- Phase 1: Calibration can be used to check the channel model and the basic beamforming behavior, e.g., by looking at the SNR/SINR distribution (aim to finish it in RAN1#86bis) 
Phase 2:  Start discussion on whether and how to establish the baseline.  Further discuss simulation assumptions for Phase 2 and Phase 3. Calibration can be used to check the link/system level performances, e.g., by looking at the BLER and spectrum efficiency (aim to finish it in RAN1#87) 
Phase 3: Calibration can be used to check the UE movement/rotation/blockage (aim to finish it after RAN1#87) 
NOTE: Parameters can be different in different phases and can be re-discussed in each phase if needed.

In RAN1#87 meeting, Proposal of link-level calibration is shown as below:

Calibrate both CDL-A and CDL-B with delay spread =100ns and UE speed=3km/h.  The angles of BS, i.e., AoD, ZoD, are fixed with [0] degree in azimuth domain and [90] degree in zenith domain, and those of UE, i.e., AoA, ZoA, are distributed within [-180, 180] degrees in azimuth domain and fixed with [90] in zenith domain, via applying desired mean angle in Section 7.7.5.1 in TR 38.900 accordingly.

Proposal of system-level calibration is shown as below:

1.       For system level calibration,  SNR CDF curves are generated with no interference in this step 
a.       Single TX/RX subelement - without TX/RX beamforming   
        	-  The first sub-element in a single fixed panel is used with single-pol. 
b.       With analog TX/RX beamforming, using a single digital TX/RX port 
       		 -  The first TXU/RXU in a single fixed panel is used with single-pol 
b'.       With fixed analog TX/RX beamforming, using a single digital TX/RX port 
        	-  The first TXU/RXU in a single fixed panel is used with single-pol 
c.       With analog TX/RX beamforming, and with SVD-based digital TX/RX beamforming 
        	 - The UE panel with the best receive SNR is chosen for output metric. i.e. no combining is done between panels.

In this contribution, we give our intial LLS and SLS calibration results of NR MIMO.

Initial link level calibraiton results of NR MIMO

For link level beam selection, there are two methods for calibration agreed as following:

- Beam selection method 1: The DFT beam directly is pointing to the strongest cluster.

- Beam selection method 2: Select a beam among the limited set of DFT beams with oversampling factor =1. Select the best beam pair at the same time based on the criteria of maximizing receive power after beamforming.

In our link level results, we adopt beam selection method 1, the channel models we used here are 3GHz CDL-A and CDL-B, and two transmission schemes are calibrated here, one is fixed angle with 1 panel and 1 polarization, the other is different angle with 1 panel and 1 polarization.

The detailed simulation assumptions are shown in the appendix.

Figure 1 30GHz CDL-B(1 fixed angle with 1 panel and 1 polarization)

Figure 2 30GHz CDL-B(1 panel and 1 polarization with different angle)

Figure 3 30GHz CDL-A(1 fixed angle with 1 panel and 1 polarization)

Figure 4 30GHz CDL-A(1 fixed angle with 1 panel and 1 polarization)

Initial system simulation results of NR MIMO

In this section, we give our calibration results of NR MIMO without beamforming with both 30GHz UMA and 30GHz UMI scenarios.

Figure 5: NR MIMO with beamforming in UMA scenario

Figure 5: NR MIMO with beamforming in UMI scenario

Conclusion

In this contribution, we give our initial link level and system level calibration results of NR MIMO based on the discussions and agreements of NR MIMO calibration.

References

[1] R1-168547, Offline discussion further updated summary of evaluation assumptions for NR, NTT DOCOMO

[2] R1-1613741 Way forward on Phase 1 and Phase 2 MIMO calibration for NR,   ZTE, ZTE Microelectronics, Ericsson, LG Electronics, Spreadtrum Communications

[3] R1-1608861 Summary of [86-20] email discussion on MIMO calibration for NR, ZTE

Appendix simulation assumptions

| Parameter | Values |
| --- | --- |
| BS antenna configurations | 30GHz: (M,N,P,Mg,Ng) = (4,8,2,2,2). (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (2.0, 4.0)λ |
| BS antenna pattern | Refer to [1] |
| UE antenna configurations | 30GHz: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90 Notes: introduce (Ωmg,ng, Θmg,ng) for orientation of the panel (mg, ng), 0≤mg<Mg, 0≤ng<Ng, where the orientation of the first panel (Ω0,0, Θ0,0) is the same as UE orientation, Ωmg,ng is the array bearing angle and Θmg,ng is the array downtilt angle defined in [TR 36.873]. Notes: The panel with the best receive SNR is chosen for output metric. i.e. no combining is done between panels. |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree Notes: Taking into account the definition of mechanic downtilt that “Considering a BS antenna element the x-axis of the GCS is aligned with the pointing direction of the sector.” in TR 38.900, the mechanic downtilt here is 20 0 degree that means that the downtilt angle of boresight of antenna array is 110 90 degree with regards to Z-axis. |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 90 degree, ΩUT, = 0 degree |
| UE antenna pattern | Refer to [1] |
| BF scheme | Analog BF based on beam selection |