3GPP TSG RAN WG1 Meeting #88	R1-1701698

Athens, Greece, February 13 – 17, 2017

Agenda Item:	8.1.2.4.3

Source:	Huawei, HiSilicon

Title:	Further details for PT-RS design

Document for:	Discussion and decision

Introduction

In RAN1 Ad Hoc meeting for NR in January 2017, the following were agreed [1][2]:

- Regarding PT-RS for CP-OFDM, the following is supported

  - For a given UE, the designated PT-RS is confined in scheduled resource as a baseline

    - Whether/how to share DL PT-RS among UEs is FFS

  - Presence of PT-RS in scheduled resource is UE-specifically configured/indicated

  - Multiple PT-RS densities defined in time/frequency domain are supported

  - UE can assume same precoding for a DM-RS port and a PT-RS port

    - Among which ports and mapping rules (fixed and/or configurable, etc) are FFS

  - Number of PT-RS ports can be fewer than number of DM-RS ports in scheduled resource

- Study the following for PT-RS, taking overhead and forward compatibility into account

  - Details on frequency domain patterns/densities

  - How to indicate presence/patterns of PT-RS

    - E.g., implicitly indicated based on association with numerology/MCS/number of allocated PRBs/UE category

    - E.g., explicit indication by L1/L2/L3 signaling

  - Port multiplexing methods

    - E.g., non-orthogonal multiplexing within PT-RS ports and with data

  - Using PT-RS for CFO/Doppler estimation

  - QCL relationship between PT-RS and DM-RS

  - Joint transmission of CSI-RS and PT-RS for improving CSI acquisition accuracy

  - Others are not precluded

- NR considers frequency offset and PN compensation for DFTsOFDM

  - FFS the exact method (e.g. pre-DFT /post-DFT insertion of PT-RS, blind detection, DM-RS)

  - Consider receiver complexity, PAPR, modulation order to be supported, etc

In this paper, further details for PT-RS design are discussed, addressing the study points listed above. The evaluation assumptions follow [3], while specific choices will be given along with the results. The phase noise (PHN) models in [4] are used and denoted as model 1 (UE) and model 2 (TRP).

Design aspects for PT-RS

In this section, we provide our views on port multiplexing, indication of presence/patterns, and the necessity of PT-RS for DFT-s-OFDM.

Port multiplexing

As discussed in previous contributions, PT-RS is needed only for high MCS in high SNR region. In addition, PT-RS mainly serves for CPE estimation, which is only a scalar, with which the interference from PT-RS/data mapped on same REs of neighbouring layers are much less influential. For these reasons, non-orthogonal port multiplexing, which can help to reduce overhead, appears promising for PT-RS. To evaluate non-orthogonal port multiplexing, three methods in Figure 1 are considered:

- Method A: Non-orthogonal multiplexing of one PT-RS port and data at the same REs in adjacent layer;

- Method B: Non-orthogonal multiplexing of two PT-RS ports at the same REs in adjacent layers;

- Method C: Orthogonal multiplexing of two PT-RS ports per layer.

Figure 1 Different port multiplexing methods for PT-RS

The numbers of TXRUs at TRP and UE are set as 2, same as the number of layers. The cases with the same and independent phase noise at TXRUs at UE are both considered. As the simplest case, identity matrix is used for precoding. The frequency density of PT-RS is set to one subcarrier per 4 RBs and the time density is every symbol. For method A, with the same phase noise at different TXRUs at UE, the phase noise estimate from one layer is used to compensate the other. The resulting spectral efficiency (SE) are depicted in Figure 2, for the case with 64QAM and a code rate of 3/4.

Figure 2 SE of different port multiplexing methods

As shown in Figure 2, Method A achieves the highest spectrum efficiency when the phase noise are the same at different TXRUs at UE. In addition, Method B performs better than Method C, due to reduced overhead when independent phase noise are experienced by different TXRUs at UE.

Proposal 1: Support non-orthogonal multiplexing within PT-RS ports, and between PT-RS and data.

Indication of presence/patterns

To reduce signaling overhead, implicit indication for the presence/patterns of PT-RS is preferred. Whether to schedule PT-RS or not can be associated with MCS and BW. One example is PT-RS is mapped automatically when the scheduled MCS and/or BW is/are above certain thresholds. Also, the time domain density can be associated with numerology. For example, for the 30 GHz band, {every symbol, every 2nd symbol} can be associated with {60, 120} kHz subcarrier spacing, respectively. In addition, the frequency domain density, more specifically, the exact number of subcarriers allocated for PT-RS can be implicitly indicated by predefined rules associated with scheduled BW, by using e.g. {4, 6, 8} subcarriers for above {8, 16, 32} RBs, respectively.

To provide forward compatibility, the association rules of implicit indication for the presence/patterns of PT-RS, e.g., MCS/BW thresholds, should not be hardcoded in Specs, and high-layer signalling to update such association rules should be supported.

Proposal 2: Support implicit indication for the presence/patterns of PT-RS, associated with numerology/MCS/BW, where the association rules can be configured by high-layer signalling, enabling forward compatibility.

PT-RS for DFT-s-OFDM

When DFT-s-OFDM is used, PT-RS may not be needed, as it targets at coverage-limited scenario with single stream transmission only, where low-order modulation and smaller BW are more applicable. To verify this, evaluations results of single-layer transmission are provided in Figure 3. Time-domain PT-RS is inserted before DFT with an overhead of 6%. As can be seen, such PT-RS and phase noise compensation (PNC) provides no gain for 16QAM 1/2 and 16QAM 2/3, and negative impacts are observed with even lower MCS such as QPSK and 16QAM 1/3, due the reason that phase tracking in low SNR is inaccurate.

Proposal 3: Consider to preclude PT-RS for DFT-s-OFDM for at least below 40 GHz.

Figure 3 BLER of DFT-s-OFDM

Summary of proposals

The proposals in this paper are summarized as follows:

Proposal 1: Support non-orthogonal multiplexing within PT-RS ports, and between PT-RS and data.

Proposal 2: Support implicit indication for the presence/patterns of PT-RS, associated with numerology/MCS/BW, where the association rules can be configured by high-layer signalling, enabling forward compatibility.

Proposal 3: Consider to preclude PT-RS for DFT-s-OFDM for at least below 40 GHz.

References

- 3GPP, “RAN1 Chairman’s Notes”, Spokane, USA, January 16 – 20, 2017.

Huawei, HiSilicon, Ericsson, National Instruments, Nokia, Alcatel-Lucent Shanghai Bell, ETRI, Intel, NTT DOCOMO, Panasonic, “WF on PT-RS”, R1-1701379, Spokane, USA, January 16 – 20, 2017.

- LG Electronics, Nokia, Huawei, Samsung, Qualcomm, InterDigital, KT, ZTE, “WF on evaluation assumptions for tracking of phase rotation and/or frequency offset”, R1-1611012, Lisbon, Portugal, October 10 – 14, 2016.

- Huawei, HiSilicon, “Phase noise model for above 6 GHz”, R1-164041, Nanjing, P. R. China, May 23 – 27, 2016.