3GPP TSG RAN WG1 Meeting	#88		   					 R1-1702333

Athens, Greece, 13th-17th February 2017

Agenda Item:	8.1.2.4.4

Source:	InterDigital Communications

Title:	Considerations on SRS for NR

Document for:	Discussion

# Introduction

In RAN1 #NR-Adhoc meeting, following has been agreed as a progress for SRS design in NR [1]:

Agreements:

- NR supports periodic and semi-persistent NR-SRS transmission.

  - Note aperiodic SRS transmission has been agreed

- Out of K ≥ 1 configured NR-SRS resources:

  - For aperiodic transmission, the UE can be configured to transmit a subset of or all K NR-SRS resources with no precoding, the same or different precoding

  - For periodic and semi-persistent transmission, the UE can be configured to transmit K NR-SRS resources with no precoding, the same or different precoding

- FFS details related position & timing of SRS transmission (including configurable)

- NR supports SRS transmission including

  - Number of SRS ports are 1, 2, 4, FFS 3, 8 (possibly other values)

  - Comb levels are 2 and 4

  - Configurable frequency hopping (details FFS)

- Clarify the following agreed configurations are applicable to each NR-SRS resource from K configured NR-SRS resources:

  - NR-SRS bandwidth

  - Number of CP-OFDM/DFT-S-OFDM symbols

  - Number of SRS ports

  - Comb level

In this contribution, we discuss further details on uplink sounding reference signal design for NR.

# Considerations on SRS for NR

It has been agreed to support aperiodic SRS transmission triggered by gNB although details of the triggering method are still FFS. Similar to LTE, using L1 control signalling seems to be the most flexible and dynamic way to handle the aperiodic SRS transmission. Since it is also agreed that a UE can be configured with multiple SRS resources, it seems to be straightforward to indicate the NR-SRS resource out of K configured SRS resources for aperiodic SRS transmission in the associated L1 control signalling.

Assuming that L1 control signalling is used for aperiodic SRS trigger, the control signalling overhead for triggering could be significant. For example, the gNB needs to send the L1 control signalling each time it needs the SRS, which may result in increasing blocking probability for the control channel. Therefore, on top of the aperiodic SRS transmission, other SRS transmission schemes with low control signalling overhead is needed to support continuous SRS transmission scenarios.

A semi-persistent SRS transmission which may be activated/deactivated by L1 control signalling or MAC CE has been agreed to address the control signalling overhead while keeping the scheduling flexibility. The semi-persistent reference signal transmission has been used for downlink reference signal transmission as it can efficiently reduce the reference signal overhead as compared with periodic reference signal transmission while the control signalling overhead is lower than that of aperiodic reference signal in some cases.

Unlike downlink semi-persistent reference signal transmission (e.g., semi-persistent CSI-RS transmission), the impact from misdetection of deactivation signal could be significant as a UE may keep sending an SRS which may result in unnecessary interference and UE battery consumption. Therefore, the semi-persistent SRS transmission should include automatic turn-off mechanism after a maximum time window (e.g., preconfigured or indicated) if there is no deactivation within the maximum time window. Therefore, regardless of signalling mechanism (e.g., DCI or MAC-CE), a maximum time window is introduced so that a UE automatically deactivate semi-persistent SRS transmission if the UE doesn’t receive deactivation within the maximum time window which started after activation.

Proposal-1: a maximum activation window is introduced for semi-persistent SRS transmission

LTE supports a periodic SRS transmission which is configured via higher layer signalling; the UE transmits the configured SRS periodically unless a collision occurs with a higher priority uplink signal. From the standpoint of flexibility, capacity, and UE battery consumption, the periodic SRS seems to provide no benefit as compared with aperiodic SRS. However, it doesn’t require L1 control signalling, thus reduces the blocking probability of the downlink control channel when downlink traffic is heavy. From this observation, supporting periodic SRS transmission similar to LTE seems to be beneficial for NR.

Proposal-2: support periodic SRS transmission in NR

It has been agreed that partial band SRS transmission is supported and the partial band size is configurable (e.g., N PRBs). The frequency resources used for SRS transmission in LTE has been consecutive in order to keep the single carrier property of the SC-FDMA. Therefore, if a UE is power limited, the partial frequency resource for SRS transmission should be localized, thus requiring multiple SRS transmissions to acquire CSI for multiple subbands when frequency selective scheduling is used.

Considering that OFDM is one of the waveforms used for uplink in NR, SRS transmission in multiple non-consecutive partial bands may be supported so that gNB may acquire the CSI for multiple subbands with lower latency.

Proposal-3: support SRS transmission in multiple non-consecutive partial bands in NR

In uplink, a UE may be configured to use either OFDM or DFT-s-OFDM waveform based on the UE uplink coverage level. Hence, multiplexing OFDM and DFT-s-OFDM in the same subframe should be supported at least in a FDM manner in order to avoid uplink scheduling restriction. Considering that a UE can be triggered to transmit SRS irrespective of the waveform configured, NR should consider a unified SRS design that supports multiplexing of SRSs from UEs configured with different waveforms.

Proposal-4: support a unified SRS design for OFDM and DFT-s-OFDM waveforms

In LTE, cell-specific and UE-specific SRS subframes have been used so that a UE is informed in which subframe the UE should avoid collision between SRS and other signals and the UE should transmit SRS if configured or triggered. Similar to LTE, cell-specific and/or UE-specific SRS subframe concept may be reused in NR. Alternatively, on-demand SRS subframe may be considered in NR for better flexibility of SRS transmission.

Proposal-5: study necessity of cell-specific and/or UE-specific SRS subframe configuration in NR

# Summary

This contribution discussed on the design considerations of SRS in NR. Based on the discussions, we propose followings:

Proposal-1: a maximum activation window is introduced for semi-persistent SRS transmission

Proposal-2: support periodic SRS transmission in NR

Proposal-3: support SRS transmission in multiple non-consecutive partial bands in NR

Proposal-4: support a unified SRS design for OFDM and DFT-s-OFDM waveforms

Proposal-5: study necessity of cell-specific and/or UE-specific SRS subframe configuration in NR

# References

RAN1 Chairman’s Notes, 3GPP TSG-RAN WG1 #NR-Adhoc