3GPP TSG RAN WG1 Meeting #88bis                                 R1-1704367

# Spokane, USA, 3rd - 7th April 2017

# Source:        ZTE, ZTE Microelectronics

Title:           Mobility procedure

Agenda Item:   8.1.1.5

Document for:  Discussion and Decision

## Introduction

This contribution discusses the high-level CONNECTED mode L3 mobility procedure, in particular RRM measurements. Also, the triggering of CSI-RS for L3 mobility is considered. Our companion contribution [1] discusses CSI-RS for L3 mobility in more details. In our RAN2 contribution [4], we discuss measurement configurations, events and reporting for L3 mobility in more detail.

## CONNECTED Mode L3 Mobility Procedure

Some of the steps in the L3 mobility procedure for a UE in CONNECTED active mode is considered below. Note that several of the steps, sub-steps or configurations may be optional.

- The UE obtains the serving cell minimum SI, which may include:

  - The serving cell SS burst set configuration, e.g. the position of actually transmitted SS blocks

  - The serving cell configuration of CSI-RS for L3 mobility.

- The UE may obtain configurations of neighbor cells from a serving cell, either in other SI or in a UE-specific configuration. Such configurations may include:

  - The neighbor cell SS burst set configuration, e.g. the positions of actually transmitted SS blocks

  - The neighbor cell configurations of CSI-RS for L3 mobility

- The UE may also obtain a UE-specific configuration of the serving cell CSI-RS for L3 mobility from a serving cell. Such a configuration may complement and partly replace the configuration obtained in the minimum SI (e.g. number of antenna ports, periodicity, etc).

- The UE is also configured with a reporting configuration, which defines reporting criteria, reporting quantities,etc [2].

- The UE searches for NR-SS.

- The UE detects NR-SSs of neighbor cells and performs measurements on IDLE mode RS, using information in the neighbor cell SS burst set configuration, if available to the UE.

  - The measurements on IDLE mode RS result in beam level quantities, i.e. SS-block-RSRPs, which are also combined into cell-level quantities [4].

- The UE may receive the minimum SI from a neighbor cell, e.g. in order to obtain configurations of the SS burst set or the CSI-RS for L3 mobility.

- In addition to measurements on IDLE mode RS, the UE may also perform neighbor cell measurements using CSI-RS for L3 mobility, e.g. if some criterion on IDLE mode RS power is met and if the CSI-RS for L3 mobility is configured for the neighbor cell.

  - The measurements on CSI-RS for L3 mobility result in beam level quantities, e.g. CSI-RSRP, which may also be combined into cell-level quantities [4].

- Based on the criteria in the reporting configuration and the measurement results [2], the UE reports measurement results to the serving cell.

  - Which quantities to include in the report may be configured in the reporting configuration, but may also depend on the measurement results.

- Based on the measurement reports from a UE, the serving gNB may initiate a handover procedure with a target cell. The handover request message may include detailed measurement results, such as beam-level measurement results. This may be helpful for the target gNB in the handover preparation, for instance in the allocation of preambles for contention-free random access [3].

Based on the procedure described above, we make the following proposals:

Proposal 1: A cell’s configurations of SS burst set and CSI-RS for L3 mobility can be included in the remaining minimum SI of the cell.

Proposal 2: Neighbor cells’ configurations of SS burst set and CSI-RS for L3 mobility can be included in a cell’s other SI.

## Triggering of CSI-RS for L3 Mobility

One of the important requirements of NR is to minimize the always-on signals comprising NR-SS and NR-PBCH to support forward compatibility and resource efficiency. This means that it is beneficial if CSI-RS for L3 mobility can be transmitted on demand. In other words, as long as at least one UE needs CSI-RS for L3 mobility, the gNB should transmit it. One way to solve this is to introduce a request to trigger gNB transmission of CSI-RS for L3 mobility, using a dedicated PRACH resource shared by all UEs. This could allow fine beam searching using CSI-RS before RRC connection is established.

Proposal 3: CSI-RS for L3 mobility can be transmitted on–demand. A request mechanism using PRACH may be needed for UEs to trigger transmission of CSI-RS for L3 mobility.

Alternatively, the MRS transmission can be triggered by UL signal measurements, even without standard support of UL mobility. Standalone cells that do not serve any CONNECTED mode UE should be able to transmit only the always-on signals, which do not include CSI-RS. Such cells will be discoverable and measurable by connected UEs in neighbor cells. However, these signals may not be sufficient for CONNECTED mode mobility, as discussed in [1]. Therefore, in addition to the UE triggering of CSI-RS, it would be useful for a serving gNB to be able to request a candidate cell to turn on the CSI-RS for L3 mobility, for example as part of handover preparation, in order to enable higher-accuracy RRM measurements for connected mode.

Proposal 4: A serving gNB can request a candidate cell to turn on CSI-RS for L3 mobility, for example as part of handover preparation.

The transmission of only always-on IDLE mode RS in some cells and additional transmission of CSI-RS for L3 mobility in other cells is illustrated in Figure 1, in which there is a single connected UE in a cell. The two closest candidate cells have turned on transmission of CSI-RS for L3 mobility. The other cells (dashed) only transmit the always-on signals, which enable support for idle mode UEs. It also makes the cells discoverable and measureable for the CONNECTED mode UE.

Figure 1

## Conclusions

The following was proposed above:

Proposal 1: A cell’s configurations of SS burst set and CSI-RS for L3 mobility can be included in the remaining minimum SI of the cell.

Proposal 2: Neighbor cells’ configurations of SS burst set and CSI-RS for L3 mobility can be included in a cell’s other SI.

Proposal 3: CSI-RS for L3 mobility can be transmitted on–demand. A request mechanism using PRACH may be needed for UEs to trigger transmission of CSI-RS for L3 mobility.

Proposal 4: A serving gNB can request a candidate cell to turn on CSI-RS for L3 mobility, for example as part of handover preparation.

## References

- R1-1704366, “CSI-RS for L3 mobility”, ZTE, ZTE Microelectronics

- R2-1702822, “Discussion on NR measurement”, ZTE, ZTE Microelectronics

- R1-1704365, “4-step RA procedure”, ZTE, ZTE Microelectronics

- R2-1702821, “Discussion on the cell level quality derivation”, ZTE, ZTE Microelectronics