3GPP TSG RAN WG1 Meeting #88bis		      	R1-1704905

Spokane, USA 3rd - 7th April 2017

Agenda Item:	8.1.3.1.5

Source: 	LG Electronics

Title: 	Considerations on PDCCH design for URLLC

Document for:	Discussion and decision

- Introduction

In RAN1#88 meetings, following agreements were made regarding NR-PDCCH design [1]:

Agreements: - NR-PDCCH can be mapped contiguously or non-contiguously in frequency - The following may be considered to achieve the above (in the physical domain) - Option 1: Localized or distributed mapping of REGs to a CCE. - Option 2: Localized mapping of REGs to a CCE. Localized or distributed mapping of CCEs when multiple CCEs are needed for an NR-PDCCH - Down-selection between Opt 1 and Opt 2 should be further discussed - Companies are encouraged to perform evaluations considering aspects such as channel estimation, frequency diversity, impact of resource reuse for NR-PDSCH, etc., especially for one CCE case Agreements: - FFS details of mapping of NR-PDCCH in time and frequency, considering the following options: - Frequency first mapping of REGs to CCEs, frequency first mapping of CCEs to search space candidate - Time first mapping of REGs to CCEs, time first mapping of CCEs to search space candidate - Frequency first mapping of REGs to CCEs, time first mapping of CCEs to search space candidate - Time first mapping of REGs to CCEs, frequency first mapping of CCEs to search space candidate - Down-selection should be discussed, including of the number of supported option(s) Agreements: - Multiple control resource sets can be overlapped in frequency and time for a UE. - A search space in NR is associated with a single control resource set - The search spaces in different control resources sets are defined independently. - The max number of BD candidates for a UE is defined independently of the number of control resource sets and the number of search spaces.

In this contribution, we provide our views on URLLC-specific aspects on NR-PDCCH design. Especially, we focus on how to define monitoring occasions, and how to perform physical resource mapping of NR-PDCCH considering characteristics of URLLC traffic.

Before discussion, in terms of UE capability on URLLC UEs, some clarifications are necessary. As agreed, from the network perspective, eMBB/URLLC traffic can be multiplexed using the same CP overhead with same subcarrier spacing or different subcarrier spacing. In other words, eMBB and URLLC UEs can use different numerologies. Also, dynamic FDM and/or TDM between different numerologies are supported. From a UE perspective, depending on use cases, it is also possible that a UE supports both eMBB and URLLC at the same time with same or different numerologies. Also, it is possible that a UE supports only URLLC. Depending on UE capability, a UE may be configured with only mini-slot based scheduling or both mini-slot and slot based scheduling.

Observation 1: A URLLC UE may be configured with only mini-slot based scheduling or both mini-slot and slot-based scheduling.

- UE monitoring behaviour

Considering the characteristics of URLLC traffic, NR needs to configure control monitoring interval at mini-slot level for URLLC UEs to reduce overall user plane latency. In this case, following consideration points needs to be discussed.

First of all, when UE perform BD attempts in every mini-slot, it is necessary to investigate whether or how much the overall number of BD attempts for a given time duration (e.g. slot) will increase compared to UE perform BD attempts in a unit of slot. To be specific, increasing BD attempts can increase UE complexity and/or decoding latency. If necessary, gNB could adjust the overall number of BD attempts for eMBB and/or URLLC PDCCH adequately based on UE capability and service requirements.

Next, considering URLLC traffic would arrive very rarely, it could be inefficient that UE try to detect PDCCH scheduling URLLC in every mini-slot. It can cause excessive battery consumption at UE side, but it is needed for low latency. In that point of view, it is necessary to carefully investigate how to configure monitoring occasions for URLLC traffic. One example is to consider separate DRX configuration for eMBB and URLLC or slot-based and mini-slot based DRX configuration.

Proposal 1: It is necessary to investigate how to configure BD attempts and monitoring occasions for URLLC scheduling considering UE complexity and latency requirements.

- Physical resource mapping of NR-PDCCH for URLLC

When URLLC DL transmission overlapping the resources scheduled for an eMBB UE in downlink occurs, parts of DL resources for eMBB DL transmission will be pre-empted by URLLC DL transmission. In case, information of impacted resources will include resources used for URLLC PDCCH transmission as well as URLLC PDSCH transmission. Considering on the contents of indication for impacted resources, it is necessary to design which resource mapping is supported for PDCCH for URLLC. For instance, if indication of impacted resources can be given in terms of time and frequency resources, and if distributed mapping is used for URLLC PDCCH, it is necessary to carefully design how to indicate impacted resources. On the other hands, if the indication of impacted resources is given by only time-domain resources, both localized and distributed mapping can be supported for URLLC PDCCH.

Proposal 2: It is necessary to investigate whether localized and/or distributed mapping methods are supported for URLLC PDCCH considering indication of impacted resources and detection performance of PDCCH.

- Control region sharing between eMBB and URLLC

At the beginning of eMBB TTI (e.g. slot), URLLC DL transmission can pre-empt parts of eMBB PDCCH which can lead eMBB PDCCH performance degradation. This may lead the scheduled PDSCH to be entirely dropped due to PDCCH missing. To avoid this case, if URLLC DL transmission always avoid overlapping with the eMBB control region to protect DL control channel, it may increase overall latency. To mitigate this problem, it can be considered that control regions for eMBB and URLLC are shared at the beginning of a slot, and URLLC PDSCH can be scheduled in regions not overlapped with slot PDCCH. For instance, URLLC PDCCH scheduling URLLC PDSCH can be mapped on a part of control region configured for eMBB at the beginning of a slot. Regarding Rel-14 latency reduction WI, it is discussed that PDCCH can convey either DCI (scheduling 1ms PDSCH or PUSCH) or sDCI (scheduling 2/7-OS PDSCH or PUSCH). In a similar manner, at least for the case where the same SCS is used for eMBB and URLLC, it can be considered to share control region between eMBB and URLLC. For simplicity, it is up to network decision, whether or not search spaces or control resource sets is overlapped between eMBB and URLLC.

Observation 2: When resources for PDCCH transmission is partially pre-empted by another DL transmission, the corresponding PDSCH could be entirely dropped due to PDCCH missing.

Proposal 3: NR supports control region sharing between DL transmissions with different service requirements and/or scheduling unit at least at the beginning of a slot.

Furthermore, when group common DCI is scheduled, if a UE supports only mini-slot based scheduling, handling of group common PDCCH needs to be clarified. One approach is to share group common PDCCH at slot-level with UEs with mini-slot based scheduling. Another approach is to transmit separate group common PDCCH at slot/mini-slot level respectively.

Proposal 4: It is necessary to clarify whether group-common DCI is applied to multiple scheduling unit (e.g. slot and mini-slot), or group-common DCI is introduced for each scheduling unit.

- Impact of URLLC DL transmission on eMBB RS

As well as eMBB PDCCH, when parts of RS is pre-empted by URLLC DL transmission, it can lead eMBB PDCCH/PDSCH performance degradation. For instance, if DMRS is punctured, UE will fail to perform demodulation of PDSCH. Meanwhile, when CSI-RS is punctured, since CSI measurements and feedback would not be accurate, scheduling information such as resource allocation and MCS will not be optimized. Furthermore, if CSI measurement and feedback are jointly applied to eMBB and URLLC, this problem would have impact on both eMBB and URLLC PDSCH detection performances.

Proposal 5: RS including DMRS and CSI-RS needs to be protected by pre-emption of another DL transmission.

- Conclusion

In this contribution, we discuss URLLC-specific aspects on NR-PDCCH design. Our proposals are as follows:

Observation 1: A URLLC UE may be configured with only mini-slot based scheduling or both mini-slot and slot-based scheduling.

Observation 2: When resources for PDCCH transmission is partially pre-empted by another DL transmission, the corresponding PDSCH could be entirely dropped due to PDCCH missing.

Proposal 1: It is necessary to investigate how to configure BD attempts and monitoring occasions for URLLC scheduling considering UE complexity and latency requirements.

Proposal 2: It is necessary to investigate whether localized and/or distributed mapping methods are supported for URLLC PDCCH considering indication of impacted resources and detection performance of PDCCH.

Proposal 3: NR supports control region sharing between DL transmissions with different service requirements and/or scheduling unit at least at the beginning of a slot.

Proposal 4: It is necessary to clarify whether group-common DCI is applied to multiple scheduling unit (e.g. slot and mini-slot), or group-common DCI is introduced for each scheduling unit.

Proposal 5: RS including DMRS and CSI-RS needs to be protected by pre-emption of another DL transmission.

- Reference

RAN1 chairman’s notes, RAN1#88.