3GPP TSG RAN WG1  Meeting #88bis							R1-1705751

Spokane, USA 3rd - 7th April 2017

Source:	NTT DOCOMO, INC.

Title:	Low latency uplink for URLLC

Agenda Item:	8.1.3.2.5

Document for: 	Discussion and Decision

Introduction

At the RAN1 #88 meeting [1], the following agreements regarding UL transmission with/without grant were achieved for UL URLLC:

Agreements in RAN1 #88:

- For UL transmission without grant,

    - The resource configuration includes at least the following

      - Time and frequency resources, FFS: including resources for repetitions, implicitly or explicitly

      - Modulation and coding scheme(s), possibly including RV, implicitly or explicitly

      - Reference signal parameters

        - FFS: Details

      - FFS: The number of repetitions K

        - FFS: Whether multiple number of K can be configured to one UE

      - FFS other parameters

    - FFS: A UE may continue repetitions for a TB until one of the following conditions is met

      - An ACK is successfully received from gNB

      - The number of repetitions for the TB reaches K

- For UE configured with K repetitions for a TB transmission with/without grant, the UE can continue repetitions (FFS can be different RV versions, FFS different MCS) for the TB until one of the following conditions is met

  - If an UL grant is successfully received for a slot/mini-slot for the same TB

    - FFS: How to determine the grant is for the same TB

  - FFS: An acknowledgement/indication of successful receiving of that TB from gNB

  - The number of repetitions for that TB reaches K

  - FFS: Whether it is possible to determine if the grant is for the same TB

  - Note that this does not assume that UL grant is scheduled based on the slot whereas grant free allocation is based on mini-slot (vice versa)

  - Note that other termination condition of repetition may apply

In this contribution, the low latency realizations for URLLC uplink are discussed. We provide our views on UL grant-free transmission and shorter periodicity of scheduling request transmission for URLLC.

UL grant-free transmission for URLLC

Resource configuration for grant-free transmission

Conventionally, dynamic scheduling by gNB provides frequency selective gain and high robust performance. For dynamic scheduling, transmission of scheduling request and/or buffer status report (BSR) would be necessary in order for gNB to acquire enough information of UL buffer. However, transmission of such information demands large latency. Especially for the case UL traffic with small packets where each packet can be transmitted by one transmission, the transmission procedure starting from SR/BSR is not beneficial for latency reduction. To meet the requirement of latency for URLLC, dynamic scheduling without full information on UL buffer, e.g. without SR/BSR at gNB may be needed. For this, gNB needs to schedule a UE onto every TTI (pre-scheduling) since gNB has limited information on UL buffer status of the UE. However, in this case, transmission of scheduling information would result in large signaling overhead.

UL grant-free transmission is beneficial for reduction of both signaling and latency. At the RAN1 #87 NR Ad-Hoc meeting, support of UL grant-free transmission with semi-static resource configuration was agreed for URLLC [2]. To offer frequency diversity in transmission, the resources can be non-contiguous in frequency-domain so that UE can transmit UL grant-free data on different frequencies at different occurrence. In order to meet the latency requirement of URLLC, resources in time-domain for UL grant-free transmission can be configured in sufficiently short periodicity, e.g. every TTI. Inter/intra TTI frequency-hopping can be considered for grant-free UL transmission to further achieve frequency diversity gain.

For a URLLC UE, UE buffer could be empty for most of the TTIs depending on the packet arrival rate of UE, thus it results in low resource utilization. Therefore, uplink skipping where UE can skip its uplink transmission if UE buffer is empty can be considered to avoid unnecessary UL transmission. For semi-static resource configuration for UL grant-free transmission, significant amount of UL resources would be needed if dedicated resources are reserved for each UE. Besides, larger sub-carrier spacing could be used for URLLC to apply shorter processing time, e.g. shorter TTI length. Therefore, large amount of frequency resource needs to be reserved for UL grant-free transmission if traffic load increases. In this case, the limited frequency bandwidth may be bottleneck to achieve high capacity for URLLC. To improve the capacity, shared resource configuration for each UE is beneficial, where collision/contention among UL transmissions is allowed.

Proposal 1: UL SPS with UL transmission skipping should be supported for URLLC UL grant free transmission.

Proposal 2: In addition to dedicated resource configuration, shared resource configuration for grant-free UE can be considered.

Reliable UE identification

In the case of allowing collision/contention among UL transmissions, gNB may fail to decode the UL data of the collided UEs. In order to resolve significant performance degradation due to contention, a retransmission can be scheduled by an UL grant, as agreed in RAN1 #88 meeting. To realize scheduling a retransmission by UL grant, robust UE identification should be ensured. The reliable transmission of UE identification is critical to the delay and reliability requirements of UL grant-free transmission for URLLC. Without reliable UE identification, gNB cannot know who is transmitting UL data and when the retransmission occurs.

Proposal 3: Reliable identification should be ensured for UL grant-free transmission.

UE identification could be realized by reliable RS (or preamble) transmission, e.g. orthogonal RS for different UEs. The reliability of UE identification based on RS is depending on the number of orthogonal RS sequences/codes and the arrival rate of URLLC traffic for a given contention resource. Therefore, increasing amount of RS resources/lengths is beneficial to improve the detection reliability of UE identification. Considering limited frequency resources and co-existence with eMBB, it is difficult to increase amount of RS resources in frequency domain. Therefore, increase of RS resources in time domain can be considered. Examples of increasing amount of RS resources in time domain are given as in Figure 1. Compared to the RS resources used for UL grant-based transmission, the RS resources for UL grant-free resources can be increased in two different options, contiguously spanning or comb-like.

Figure 1: Examples of increasing amount of RS resources for UL grant-free transmission in time-domain

We consider two different alternatives for RS configuration for grant-free UE

- Alt.1: Dedicated RS sequence/code for each UE configured with UL grant-free transmission

- Alt.2: Shared RS sequences/codes for a group of UE configured with UL grant-free transmission

For Alt. 1, orthogonal RS can be configured for each UE and sufficient reliability is expected. However, large number of RS shall be needed to ensure the orthogonality of RS. For Alt. 2, shared RS can be configured for UE group and UE needs to selects one of RS sequence/code among possible candidates. In this case, gNB may need to rely on blind detection or additional information to identify the actual UE-ID. In this case, there is possibility that multi-user selecting the same RS sequence/code, which is called RS collision. Although there is unavoidable RS collision, the necessary number of orthogonal RS can be smaller.

Proposal 4: UE identification based on RS transmission can be considered for UL grant-free transmission.

Switching grant-free transmission to grant-based transmission

As discussed above, if gNB successfully detect UE identification, an UL grant can be transmitted by gNB to schedule a retransmission if gNB fails to decode the data. This is aimed to resolve contention among UL grant-free transmission and avoid significant performance loss. Similarly, gNB can schedule also further UL transmission by UL grant, with the purpose of reducing reserved resources for UL grant-free transmission. As less reserved resources needed for UEs configured with UL grant-free transmission by doing this, collision probability among potential UL grant-free transmissions can be lower. Besides, less reserved resources for UL grant-free transmission is also beneficial to ensure performance of eMBB traffic in the case that the carrier is shared between eMBB and URLLC. Therefore, we have following proposal.

Proposal 5: If gNB successfully decode UL data, gNB can schedule also further UL transmission by UL grant.

Repetition

As agreed at the RAN1 #87 Ad-Hoc meeting, K repetitions including initial transmission and retransmission can be configured for UL transmission with/without grant, where K >=1. The repetitions are targeting for increasing number of transmissions within URLLC latency bound so that to improve the received SINR to fulfill the reliability requirement. For periodic traffic with low traffic load, K repetitions can be finished within packet arrival interval. However, for high traffic load or for the sporadic traffic, a new packet may arrive during the repetitions. For the new packet transmission, there are two options

- Option 1: new data will be transmitted after the current K repetitions

- Option 2: new data can be transmitted during the current K repetitions

For option 1, the new UL transmission needs to wait for the completion of repetitions for the previous UL data. It may result in latency increase for the new arrived traffic. For option 2, a new packet does not need to wait for completion of current repetition but to start fast after packet arrival. Allowing fast start of new UL transmission for new data is beneficial to reduce the latency. However, for option 2, multiplexing between new data transmission and current data repetitions needs to be considered.

Proposal 6: Further study method of transmitting new UL data during repetitions

Shorter periodicity of scheduling request

RAN1 agreed that a time interval between SR resources configured for a UE can be smaller than a slot. Assuming that if a one-symbol SR is configured per 2-symbol periodicity, the delay from the scheduling request to data delivery could be; 1 symbol (FA) + 1 symbol (SR) + 1 symbol (gNB processing) + 1 symbol (UL grant) + 1 symbol (UE processing) + 2 symbol (UL data) + 2 symbol (gNB processing) = 9 symbols < 0.5 ms, if SCS = 30 kHz. As such, shorter periodicity of SR can also meet the latency requirement of URLLC. Considering that the target use-case of SR shorter periodicity than a slot is for URLLC using mini-slot configuration, shorter periodicity of SR and its related UE scheduling request procedure should be specified taking into account the scheduling mechanisms for mini-slot operation.

Proposal 7: SR periodicity shorter than a slot and related UE procedure are specified taking into account the mini-slot operation for URLLC low latency.

Conclusion

In this contribution, we discussed the UL transmission and retransmission for grant-free for URLLC. Observations and proposals are summarized below.

Proposal 1:  UL SPS with UL transmission skipping could be the baseline for URLLC UL grant free transmission.

Proposal 2:  In addition to dedicated resource configuration, shared resource configuration for grant-free UE needs to be considered.

Proposal 3: Reliable identification should be ensured for UL grant-free transmission.

Proposal 4: UE identification based on RS transmission can be considered for UL grant-free transmission.

Proposal 5: If gNB successfully decode UL data, gNB can schedule also further UL transmission by UL grant.

Proposal 6: Further study method of transmitting new UL data during repetition

Proposal 7: SR periodicity shorter than a slot and related UE procedure are specified taking into account the mini-slot operation for URLLC low latency.

- References

- RAN1 Chairman’s note, RAN1#88.

- RAN1 Chairman’s note, RAN1#87 NR Ad-Hoc Jan. meeting