3GPP TSG RAN WG1 Meeting #88bis		R1-1705850

Spokane, USA 3rd – 7th April 2017

Source:	WILUS Inc.

Title:	Discussion on PUCCH in short duration

Agenda item:	8.1.3.2.1

Document for:	Discussion/Decision

- Introduction

At the previous RAN1#88 meeting, the following has been agreed on PUCCH with short duration [1].

Agreements at RAN1 #88: - For 2-symbol PUCCH, consider following options - Option 1: RS and UCI are multiplexed by FDM manner in each symbol. - Option 2: RS and UCI are multiplexed by TDM manner. - Option 3: RS and UCI are multiplexed by FDM manner in one symbol and only UCI is carried on another symbol without RS - Option 4: Sequence based design without RS only for small payload size case - Option 5: Sequence based design with RS only for small payload size case - Option 6: Pre-DFT multiplexing in one or both symbol(s) - Combination of above options are not precluded - RAN1 will definitely down select above options in the next meeting Agreements at RAN1 #88: - For 1-symbol PUCCH, consider following options - Option 1: RS and UCI of one UE are multiplexed by FDM manner in each symbol. - Already agreed. - Option 4: Sequence based design without RS only for small (1~2) payload size case - Information is delivered by which sequence/code is transmitted - Sequence is mapped over contiguous or non-contiguous REs - UCI sequence can be CDMed with DMRS sequence of other UEs - Option 5: Sequence based design with RS only for small (1~2) payload size case - Information is delivered by which/what sequence/code is transmitted - RS and UCI are multiplexed by CDM manner - Option 6: Pre-DFT multiplexing of RS and UCI - Consider for both small and large UCI payload size cases - Possibility 1: {CP + Pilot} + {CP + Data} to avoid MPI b/w pilot and data - Possibility 2: CP + {Pilot + Data} as current DFT-s-OFDM - Other possibilities are not precluded - RAN1 will definitely down select above options in the next meeting

In this contribution, we discuss PUCCH structure in a short duration and provide our view regarding PUCCH with short duration.

Discussion on PUCCH structure with short duration

For PUCCH in 1-symbol duration, FDM between RS and UCI has been already agreed at the RAN1 NR-Adhoc#1 meeting as following:

Agreements at RAN1#AH_NR:

- For PUCCH in short-duration,

  - At least following is supported for PUCCH in 1-symbol duration:

    - UCI and RS are multiplexed in the given OFDM symbol in FDM manner if RS is multiplexed.

      - Same SCS between DL/UL data and PUCCH in short-duration in the same slot.

At the previous RAN1#88 meetings, however, several options have been discussed for PUCCH structure with short duration in terms of PUCCH payload size for 1- or 2-symbol PUCCH structures in addition to the 1-symbol PUCCH structure with RS and UCI multiplexed by FDM manner [1]. In this section, we compare options on some aspects such as PAPR/CM, UE multiplexing capability for HARQ-ACK/SR, frequency diversity, PUCCH DMRS flexibility, commonality between 1 and 2-symbol PUCCH and applicability with payload size. In Table 1 and Table 2, the comparison among options for 1-symbol PUCCH and 2-symbol is addressed as follows:

Table 1. Comparison on some aspects among options for 1-symbol PUCCH

Table 2. Comparison on some aspects among options for 2-symbol PUCCH

From the perspective of the PUCCH PAPR/CM aspects, UE multiplexing capability on each UCI type, and PUCCH cell coverage, it would not be possible to accommodate the different required aspects with various UCI payload sizes into a single unified PUCCH format. Therefore, we propose that CP-OFDM based FDM between RS and UCI is supported at least for the large payload size for PUCCH in NR. And for small payload size, we prefer to select one option between FDM-based option and sequence-based option without RS in terms of some aspects such as cell coverage, UE multiplexing capacity for HARQ-ACK/SR and commonality between 1 and 2-symbol PUCCH structure.

- Proposal: we propose that CP-OFDM based FDM between RS and UCI is supported at least for the large payload size for PUCCH in NR and for small payload size, we prefer to select only one option between FDM-based option and sequence-based option without RS in terms of some other aspects such as cell coverage, UE multiplexing capacity for HARQ-ACK/SR, and commonality with 1-symbol and 2-symbol PUCCH.

- Conclusion

In this contribution, we have compared several options for PUCCH structure with 1-symbol and 2-symbol in a short duration and provide our view regarding PUCCH with short duration as following:

- Proposal: we propose that CP-OFDM based FDM between RS and UCI is supported at least for the large payload size for PUCCH in NR and for small payload size, we prefer to select only one option between FDM-based option and sequence-based option without RS in terms of some other aspects such as cell coverage, UE multiplexing capacity for HARQ-ACK/SR, and commonality with 1-symbol and 2-symbol PUCCH.

References

[1] Draft Report of 3GPP TSG RAN WG1 #88 v0.1.0