3GPP TSG RAN WG1 Meeting #89	R1-1707064

Hangzhou, P.R. China 15th – 19th May 2017

Source:	Ericsson

Title:	Study of Number of CRC Bits for Code Blocks

Agenda Item:	7.1.4.1.1.1

Document for:	Discussion and Decision

# Introduction

In RAN1#88bis, the following agreements on CRC attachment were reached [1] :

Agreements:

- Number of bits for TB-level CRC is: LTB,CRC =24 bits, at least for TBs larger than a threshold (e.g. around 512 bits)

- FFS the value of LTB,CRC for TBs smaller than the threshold, and the value of the threshold (0 is not precluded)

- If a TB is segmented into 2 or more CBs after code block (CB) segmentation,

  - CB-level CRC is applied, i.e., CRC bits are attached to each code block individually (as in LTE)

  - Number bits for CB-level CRC is: 0 < LCB,CRC <= 24 bits

    - Exact value(s) LCB,CRC are to be agreed after base graph(s) are agreed, taking into account inherent LDPC PC capability

- FFS whether for a code block group (CBG) containing 2 or more CBs but not all CBs of the TB, any additional CRC bits are attached to the CBG

To be decide after decision on the value(s) of LCB,CRC

In this contribution, we consider CRC attachment on code block level for the case where a TB is divided into 2 or more CBs after code block segmentation. We present simulation results that show the probability of undetected error at CB-level and discuss the number of CRC bits that should be attached to each CB.

# Inherent error detection capability combined with CRC

According to the agreements, each CB is appended with a CRC sequence of length LCB, where 0<LCB≤24 bits after code block segmentation. The value of LCB should be selected while taking the inherent error detection capability of the LDPC code into account. However, the inherent error detection capability depends on the block length and code rate of the LDPC code. In this contribution, we evaluate the probability of undetected error for a code rate of 8/9, which corresponds to the worst case in terms of undetected errors. Furthermore, we consider the two extremes when it comes to CB info block length, i.e. K=4224 and K=8448, which is the smallest and the largest possible information block size after code block segmentation.

## Simulations

From RAN1#88bis, we have the working assumption that the largest info block size supported by LDPC encoder Kmax and the largest shift size Zmax defined is {8448, 384}, which implies that Kb,max=22. Since no LDPC code with these parameters is currently available, we have used the high #2 family with Kb,max=20 from [2] for the simulations shown in this contribution. Since the set of parameters for this code is close to the parameters stated in the working assumption, we believe that results for the LDPC code that is finally selected will not deviate much from the results presented here.

### Simulation assumptions

Simulations has been performed for the high #2 family presented in [2] using both the sum-product algorithm and the min-sum algorithm. A maximum of 50 decoding iterations with flooding scheduling is allowed. The highest possible code rate, R=8/9, is considered. This corresponds to the fewest parity check equations for a given info block size K, hence the worst case in terms of undetected errors when applying the inherent parity checksum.

The probability of undetected error labeled “No CRC” corresponds to the inherent error detection capability of the LDPC code and an undetected error occurs when the decoder finds a valid codeword within the maximum number of decoding iterations, but the codeword found is different from the codeword transmitted.

When CRC is applied, then for all valid codewords that pass the LDPC parity checks, the CRC is checked. If also the CRC checks, this corresponds to an undetected error for the given CRC length. We have used the CRC generator polynomials given in Table 1. For longer CRCs that we have considered, only very few errors within the simulated code blocks were detected and no statistically reliable results can be shown.

Table 1: CRC polynomials

| CRC length | Generator polynomial | Comment |
| --- | --- | --- |
| 4 | X4 + X + 1 | Has highest order among all degree-4 CRC polynomials |
| 8 | X8 + X7 + X4 + X3 + X + 1 | From 36.212 V8.8.0 p.8 Sec 5.1.1 |

### Probability of undetected error for K=8448

The probability of undetected error for the longest possible code block length K=8448, with R=8/9, is shown in Figure 1 and Figure 2. At this block length, the probability of undetected error with no CRC is Pud≤9·10-6 (as shown in Figure 1) when the sum-product algorithm is used, and Pud≤3·10-6 (as shown in Figure 2) when the min-sum algorithm is used. This shows that the probability of undetected error is lower with the min-sum algorithm than with the sum-product algorithm.

In addition to the “No CRC” cases, Figure 1 and Figure 2 also illustrate the undetected error probability when 4 CRC bits are checked in addition to the inherent parity checksum. As expected, the CRC bits can further lower Pud-CB. Although the natural length of 4-bit CRC is only 15, far lower than K=8448, it still lowered the undetected error probability by the factor of 2-LCB=2-4, despite the minimum distance of 2 for the repeated CRC code.

The inherent error detection capability of the LDPC code gives a probability of undetected error Pud-CB≤3·10-6 for the longest block length K=8448 when the min-sum algorithm is used.

Decoding with the min-sum algorithm gives lower probability of undetected error than the sum-product algorithm.

Figure 1: Probability of undetected error for {K=8448, R=8/9} with the sum-product algorithm

Figure 2: Probability of undetected error for {K=8448, R=8/9} with the min-sum algorithm

### Probability of undetected error for K=4224

CRC bits are appended on code block level only if code block segmentation is performed. The maximum TB size, including TB-level CRC, that can be transmitted without code block segmentation is 8448 bits. The smallest possible code block after segmentation is therefore K≈4224 bits. The probability of undetected error for this block length is shown in Figure 3 and Figure 4. At this block length, the probability of undetected error with no CRC is Pud≤6·10-6 when the sum-product algorithm is used and Pud≤2·10-6 when the min-sum algorithm is used.

It can be noted that Pud for the shortest possible code block length is slightly lower than Pud for the longest possible code block length. It is speculated that this is related to the properties of the actual LDPC code design, for example, how fast the actual LDPC code dmin and its multiplicities grows with the info block size K.

The inherent error detection capability of the LDPC code is similar for the shortest and longest block length after code block segmentation.

Figure 3: Probability of undetected error for {K=4224, R=8/9} with the sum-product algorithm

Figure 4: Probability of undetected error for {K=4224, R=8/9} with the min-sum algorithm

## Estimation

Monte Carlo simulations are only useful for estimation of the probability of undetected error with short CRC lengths. Because of the very good inherent error detection capability of long LDPC codes, simulation of Pud for the combination of LDPC and CRC becomes almost impossible for medium and long CRC lengths. In this contribution, we simply estimate the combined Pud by assuming that

Pud=Pud-LDPC·Pud-CRC

where Pud-LDPC is the probability of undetected error considering only the inherent error detection capability of the LDPC code and Pud-CRC is estimated by 2-LCB.

Figure 5 shows Pud estimated from the simulated Pud-LDPC shown in Figure 2. From these results it is clear that the number of CRC bits attached on CB-level can be reduced to fewer than 8 bits, as compared to 24 bits in LTE, while still achieving a sufficiently low probability of undetected error at CB-level, due to the inherent error detection capability of the LDPC code at long block length.

To reach the Pud=2-24=5.96×10-8 of LTE due to 24-bit CB-level CRC, the number of CB-level CRC bits required to attach to the LDPC code, LCB, is:

LCB=-log22-24Pud-LDPC

For different levels of LDPC inherent undetected error probability Pud-LDPC, Table 1 shows the corresponding value of LCB for reaching Pud=2-24.  As shown in Table 1, for the range of Pud-LDPC values shown in Section 2.1, no more than 8 CB-level CRC bits is necessary.

The number of CRC bits attached on CB-level can be reduced to LCB≤8 bits.

Table 1: Required number of CB-level CRC bits (LCB) for reaching Pud=2-24

| LDPC Inherent Pud-LDPC | Required number of CB-level CRC bits (LCB) |
| --- | --- |
| 10-5 | 8 |
| 7×10-6 | 7 |
| 3×10-6 | 6 |
| 1×10-6 | 5 |

Figure 5: Estimated probability of undetected error for the combination of LDPC and CRC

# Conclusion

In this contribution, we made the following observations:

Observation 1	The inherent error detection capability of the LDPC code gives a probability of undetected error Pud-CB≤3·10-6 for the longest block length K=8448 when the min-sum algorithm is used.

Observation 2	Decoding with the min-sum algorithm gives lower probability of undetected error than the sum-product algorithm.

Observation 3	The inherent error detection capability of the LDPC code is similar for the shortest and longest block length after code block segmentation.

Observation 4	The number of CRC bits attached on CB-level can be reduced to LCB≤8 bits.

# References

RAN1, Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #88bis, April 2017.

R1-1705627, LDPC code design for larger lift sizes, Qualcomm Incorporated, RAN1#88bis, April 2017.