3GPP TSG RAN WG1 Meeting #89	R1-1707080

Hangzhou, P.R. China 15th – 19th May 2017

Source:	Ericsson

Title:	Coding Techniques for NR-PBCH

Agenda Item:	7.1.4.3

Document for:	Discussion and Decision

# Introduction

In RAN1#88bis, the following agreement was reached on PBCH carrying the Time Index indication:

- Time index indication: PBCH conditioned that mobility and HO related requirements can be met

The size of Time Index is determined by the maximum number of SS-blocks, L, within a SS burst set. The agreed value range of L is:

- The considered maximum number of SS-blocks, L, within SS burst set for different frequency ranges are

  - For frequency range up to 3 GHz, the maximum number of SS-blocks, L,  within SS burst set is [1, 2, 4]

  - For frequency range from 3GHz to 6 GHz, the maximum number of SS-blocks, L,  within SS burst set is [4, 8]

  - For frequency range from 6 GHz to 52.6 GHz, the maximum number of SS-blocks, L,  within SS burst set is [64]

In this contribution, we study how to carry Time Index indication with PBCH. Polar codes is used to compare the different methods.

# Methods for Carrying Time Index via NR-PBCH

There are two types of methods for carrying the Time Index indication on NR-PBCH.

- One type of methods is information bit based, i.e., carry the Time Index by the encoder input.

- The other type of methods is coded bit based, i.e., carry the Time Index by the encoder output.

The methods to carry Time Index need to consider two receiver scenarios:

- Only one instance of NR-PBCH is received, and the UE performs direct decoding of the one instance;

- Multiple instances of NR-PBCH are received by the UE. The UE performs soft combining of the multiple instances of NR-PBCH, then the UE performs polar decoding of the combined channel LLR vector;

## Info Bit Based

For the information bit based methods, 3 alternatives are possible:

- Time Index explicitly carried as a part of info bits

- Time Index carried by frozen bits

- Time Index carried by color coding the CRC bits

### Alternative 1-1: Time Index explicitly carried as a part of info bits

This method is illustrated in Figure 1.  Time Index is explicitly carried as a part of info bits, and the CRC bits is generated based on the Time Index and the rest of MIB bits.

When decoding one instance of NR-PBCH (i.e., Scenario A), CRC protects the Time Index. When try to soft combine multiple instances of NR-PBCH (i.e., Scenario B), the cover code removes the Time Index and the CRC vector corresponding to it. In this scenario, CRC bits do not protect the Time Index.

After Time Index field is removed (i.e., made all-zero) by applying a cover code, then the bit location occupied by Time Index are effectively frozen bits. This helps with the Polar decoder.

Figure 1.	Alternative 1-1: Time Index explicitly carried as part of the information bits

### Alternative 1-2: Time Index carried by frozen bits

This method is illustrated in Figure 2. Since the Time Index is carried by frozen bits, the Time Index is not part of input to CRC encoding, and the CRC bits are not a function of the Time Index.

Figure 2.	Alternative 1-2: Time Index carried by frozen bits

### Alternative 1-3: Time Index carried by color coding the CRC bits

This method is illustrated in Figure 3. In Alternative 1-3, the Time Index bits are used to “color code” the CRC bits, i.e., the Time Index bits are XOR-ed on top of the CRC bits. Since there are 16 CRC bits, but at most 6 Time Index bits, only a fraction of the CRC bits are color coded.

Alternative 1-3 has the benefit of relatively lower code rate in Polar decoding.  On the other hand, hypothesis testing of Time Index value is necessary even when the receiver only attempts to decode one instance of NR-PBCH (i.e., Scenario A).

Figure 3.	Alternative 1-3: Color code CRC bits by Time Index

## Coded bit based

One coded bit based method is to carry Time Index by RV indices.  This method is illustrated in Figure 4. In this Alternative 2, Time Index is not part of the MIB, and the Time Index is conveyed via the RV indices in the circular buffer of the Polar code word.

Figure 4.	Alternative 2: Time Index is conveyed via the RV indices

# Simulation Results

## Simulation Setup

The following simulation assumptions are applied in evaluating the methods:

- Information block size K = 31 or 61 bits, respectively, where CRC bits are excluded.

  - If 16+3 CRC bits are attached, then the number of information bits from Polar encoder perspective is Kencoder= 50 or 80 bits, respectively.

- Number of coded bits is: M = 480 bits [1].

- Polar code size: N= 512 bits.

- SCL list size L = 8;

## Simulation Results

Based on the simulation results below, the following observations can be made:

- Hypothesis testing on Time Index affects NR-PBCH code performance.

- When Polar code is used for NR-PBCH, consider increasing the number of CRC bits attached to account for the hypothesis testing of Time Index.

### Alternative 1-1: Time Index explicitly carried as a part of info bits

The simulation results of Alternative 1-1 are shown in Figure 5 – Figure 7.

Figure 5 and Figure 6 show that with (16+3) CRC bits, if more than 4 instances of NR-PBCH are soft combined before Polar decoding, then the code performance deteriorates at higher SNR.

Figure 7 shows that with (16+5) CRC bits, then code performance does not deteriorate at higher SNR even when more than 4 instances of NR-PBCH are soft combined.

Thus, it is speculated that this deterioration is due to the hypothesis testing of Time Index value carried by the first NR-PBCH instance.

Figure 5.	Alternative 1-1: K=31 bits, 16+3 CRC bits.

Figure 6.	Alternative 1-1: K=61 bits, 16+3 CRC bits.

Figure 7.	Alternative 1-1: K=31 bits, 16+5 CRC bits.

### Alternative 1-3: Time Index carried by color coding the CRC bits

The simulation results of Alternative 1-3 are shown in Figure 8 – Figure 9.

Figure 8 and Figure 9 show that Alternative 1-3 exhibit similar performance trend as Alternative 1-1 when (16+3) CRC bits are attached. That is, if multiple instances of NR-PBCH are soft combined before Polar decoding, then the code performance deteriorates at higher SNR.

Figure 8.	Alternative 1-3: K=31 bits, 16+3 CRC bits.

Figure 9.	Alternative 1-3: K=61 bits, 16+3 CRC bits.

# Conclusions

In this contribution, methods for carrying Time Index indication with PBCH are studied. The following observations are made:

- Hypothesis testing on Time Index affects NR-PBCH code performance.

- When Polar code is used for NR-PBCH, consider increasing the number of CRC bits attached to account for the hypothesis testing of Time Index.

# References

R1-1707079,	“Study of Number of CRC Bits for PBCH,” Ericsson.  3GPP TSG RAN WG1 Meeting #89, Hangzhou, P.R. China 15th – 19th May 2017.