3GPP TSG RAN WG1 89 Meeting		R1-1707240

Hangzhou, China, 15th - 19th May 2017

Source:	vivo

Title:	NR HARQ Process Number

Agenda Item:	7.1.3.3.4

Document for:	Discussion and Decision

- Introduction & Background

Some discussion and email discussion was occurred in Spokane RAN1 meeting in 2017 April. And some concrete proposals are proposed for eMBB Harq Process Number [1][2][3]. This contribution further discusses the design principle for HARQ process number design from LTE design history and provide a concrete proposals for the NR eMBB downlink and uplink HARQ process number.

- Design Philosophy

The contribution [4] reviews the design principle for HARQ process number design from LTE design history and considers the following aspects,

- Processing timing for NR shall be no less than that of LTE.

- Very flexible uplink/downlink configuration for every slot shall be taken into account for NR frame structure. Data and its corresponding acknowledgement shall follow the rules in the Table 1 in [4]

- Similar subframe-specific HARQ timing shall be kept as LTE.

- Slot direction restriction with LTE-TDD on adjacent carrier shall be considered. Specific TDD UL/DL asymmetry and SCS (subcarrier spacing) shall be further studied.

  - From practical deployment perspective, TDD configuration 1 and configuration 2 is most often used for consideration.

- Results

Symmetric processing time

Accoding to the computer searching, if symmetric processing time is considered for gNB and UE, the HARQ process number is as follows,

|  | 1x LTE SCS |  | 2x LTE SCS |  |
| --- | --- | --- | --- | --- |
| gNB/UE processing time(symmetric) | # DL HARQ | # UL HARQ | # DL HARQ | # UL HARQ |
| 1.5ms (0.5x LTE) | 6 (A1-2) | 3 | 13 | 5 |
| 1ms (0.3x LTE) | 5 (A1-1) | 3 | 9 (A1-3) | 5 |

Some of the results are attached in Annex A1-1~A1-3.

Asymmetric processing time

|  | 1x LTE SCS |  | 2x LTE SCS |  |
| --- | --- | --- | --- | --- |
| gNB/UE processing time(symmetric) | # DL HARQ | # UL HARQ | # DL HARQ | # UL HARQ |
| UE: 1ms(0.3xLTE), gNB 3ms(1xLTE) | 7 | 4 | 13 | 6 |
| UE: 1ms(0.3xLTE), gNB 2ms(0.6xLTE) | 6 | 3 | 11 | 5 |

>=4x LTE SCS

For SCS larger or equal to 4x LTE SCS, no LTE/NR coexistence is considered. Therefore, it is up to gNB/UE processing time to determine the HARQ process number.  A starting point is to follow the case from 2xLTE SCS HARQ process number to see if it satisfies the processing timing.

- Conclusion

Proposal : HARQ process number are picked up from the following table by considering processing timing.

For symmetric processing time

|  | 1x LTE SCS |  | 2x LTE SCS |  |
| --- | --- | --- | --- | --- |
| gNB/UE processing time(symmetric) | # DL HARQ | # UL HARQ | # DL HARQ | # UL HARQ |
| 1.5ms (0.5x LTE) | 6 (A1-2) | 3 | 13 | 5 |
| 1ms (0.3x LTE) | 5 (A1-1) | 3 | 9 (A1-3) | 5 |

For asymmetric processing time

|  | 1x LTE SCS |  | 2x LTE SCS |  |
| --- | --- | --- | --- | --- |
| gNB/UE processing time(symmetric) | # DL HARQ | # UL HARQ | # DL HARQ | # UL HARQ |
| UE: 1ms(0.3xLTE), gNB 3ms(1xLTE) | 7 | 4 | 13 | 6 |
| UE: 1ms(0.3xLTE), gNB 2ms(0.6xLTE) | 6 | 3 | 11 | 5 |

References

- R1-1706815, “WF on the number of HARQ processes”, Qualcomm, vivo, OPPO, ZTE, RAN1#88bis, Spokane, USA, 3rd – 7th April 2017

- R1-1704015, “WF on Number of HARQ Processes”, Huawei, HiSilicon, CATT, NTT DoCoMo, OPPO, Nokia, ASB, Intel, Xinwei, Xiaomi, CATR, RAN1#88bis, Spokane, USA, 3rd – 7th April 2017

- R1-1706625, “WF on HARQ process”, Huawei, RAN1#88bis, Spokane, USA, 3rd – 7th April 2017

- R1-1704498, “Discussion on maximum HARQ process number for high round trip time”, vivo, RAN1#88bis, Spokane, USA, 3rd – 7th April 2017

- R1-081124, “Way forward for TDD HARQ process”, Huawei, CMCC, Nokia, Nokia Siemens Networks, Ericsson, CATT, RITT

- R1-081384, “TDD Uplink HARQ Timing Design”, CMCC, Huawei

Annex 1 HARQ timing for NR/LTE Coexistence

According to the aformentioned principles, the maximum HARQ process number for each configuration can be obtained by computer searching.

For example:

A1-1: Coexistence with LTE TDD configuration 2

NR downlink, number of process =5, required slots for processing >1.5ms, 1xSCS (15kHz)

A1-2: Coexistence with LTE TDD configuration 2

NR downlink, number of process =6, required slots for processing >1ms, 1xSCS (15kHz)

A1-3: Coexistence with LTE TDD configuration 2

NR downlink, number of process =9, required slots for processing >1ms, 2xSCS (30kHz)

Note :

D : DL only	D+ :DL centric	U : UL only	U+ : UL centric